3GPP RAN WG4 Meeting #99e		R4-2109433

Electronic meeting, 19 – 27th May 2021

Agenda item:	9.15.2

Source:	Apple Inc.

Title:	Overview of the regulatory parameters the 52.6 GHz to 71 GHz frequency range

WI/SI:	NR_ext_to_71GHz

Release:	Rel-17

Document for:	Discussion & Decision

# 1	Introduction

As a first step for enabling even higher frequency ranges for the NR radio system, 3GPP RAN was studying frequency bands above 52.6GHz and up to 114.25GHz [1], for which the corresponding TR 38.807 [2] captured global spectrum availability, regulatory requirements, and potential use cases. After RAN#86 meeting, a new SI was agreed that aims at studying more specific technical aspects on how the NR technology can be adopted to the 60GHz frequency range [3]. Referring to the corresponding SI description, the scope has been narrowed down to the operation between 52.6 GHz and 71 GHz. Finally, 3GPP RAN agreed the corresponding WI [4] to enable support of the 52.6-71GHz frequency range.

During the RAN4#98bis meeting several issues were raised on which UE form factor should be assumed, how many antenna elements can be considered as well as which power classes should be introduced. In addition to that, RAN WG1 also sent an LS asking RAN WG4 view on maximum UE conducted power and maximum UE EIRP [5]. In this discussion paper we present an overview of the regulator related parameters in several countries/regions. This information can facilitate further discussions on assumptions for the UE form factors, associated number of antenna elements and power classes.

# 2	Overview of the regulatory parameters

It is worth noting that 3GPP specifications already define limits for FR2 power classes, such as maximum TRP, minimum peak EIRP, and the maximum EIRP. The table below presents the corresponding parameters for the existing FR2 power classes that are already captured in TS 38.101-2.

Table 2.1: 3GPP UE limits for FR2 power classes.

| Operating band | Max TRP (dBm) | Min peak EIRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- | --- |
| Power class 1 (Fixed wireless access UE) |  |  |  |
| n257 | 35 | 40.0 | 55 |
| n258 | 35 | 40.0 | 55 |
| n260 | 35 | 38.0 | 55 |
| n261 | 35 | 40.0 | 55 |
| Power class 2 (Vehicular UE) |  |  |  |
| n257 | 23 | 29 | 43 |
| n258 | 23 | 29 | 43 |
| n261 | 23 | 29 | 43 |
| Power class 3 (Handheld UE) |  |  |  |
| n257 | 23 | 22.4 | 43 |
| n258 | 23 | 22.4 | 43 |
| n259 | 23 | 18.7 | 43 |
| n260 | 23 | 20.6 | 43 |
| n261 | 23 | 22.4 | 43 |
| n262 | 23 | 16.0 | 43 |
| Power class 4 (High power non-handheld UE) |  |  |  |
| n257 | 23 | 34 | 43 |
| n258 | 23 | 34 | 43 |
| n260 | 23 | 31 | 43 |
| n261 | 23 | 34 | 43 |
| Power class 5 (Fixed wireless access UE) |  |  |  |
| n257 | 23 | 30 | 43 |
| n258 | 23 | 30.4 | 43 |

Referring back to Table 2.1 it is worth noting that existing FR2 bands are licensed and, for instance, minimum peak EIRP is not the regulatory parameter but rather the 3GPP performance requirement. Similarly, maximum TRP is usually limited not by regulations but rather by design constraints, e.g. for handheld device. The table 2.2 below presents an overview of similar parameters for the unlicensed operation within the 52.6-71GHz range as defined by different countries/regions. As can be seen from the table, most countries put restrictions on the maximum TRP and EIRP values, and some countries even limit the maximum EIRP density.

Table 2.2: Overview of regulatory parameters for 52.6-71GHz range in different countries/regions.

| Country | Max TRP (dBm) | Max EIRP (dBm) | Max EIRP density (dBm/MHz) | Reference |
| --- | --- | --- | --- | --- |
| USA/Australia | 27 | 40 (average) 43 (peak) |  | Title 47 Part 15 |
| Canada | 27 | 40 (average) 43 (peak) |  | RSS-210 |
| EU/CEPT |  | 40 | 13 | ETSI EN 302 567 |
|  | 27 | 40 | 23 | ERC Recommendation 70-03 ETSI EN 303 753 (draft) |
| South Korea | 27 | 43 | 13 |  |
| South Africa | [27] | 40 | [13] | Use EN 302 567 |
| Brazil | 27 |  |  | Anatel Resolution 680/2017 (act 14448) |
| Japan | 24 |  |  |  |

Based on information presented in Table 2.2, the following general observations can be made:

-	Most of the countries limit maximum TRP value to 27dBm;

-	The maximum EIRP is usually limited 40/43dBm. Some countries allow 43dBm as a peak value with an exception that the average one should stay at 40dBm;

-	There are several countries that also define the maximum EIRP density.

Proposal:	Consider presented regulatory parameters in further technical discussions for the 52.6-71GHz frequency range.

# 3	Conclusions

In this discussion paper we have presented a summary of the key regulatory parameters for the 52-71GHz frequency range, such as maximum TRP, maximum EIRP and, when applicable, EIRP density. This information can be used to define the corresponding 3GPP UE power classes with associated minimum performance requirements.

Proposal:	Consider presented regulatory parameters in further technical discussions for the 52.6-71GHz frequency range.

# 4	References

RP-182861, "Study on NR design above 52.6GHz (spectrum & regulatory)", Intel

TR 38.807, "Study on requirements for NR beyond 52.6 GHz"

RP-193259, "New SID: Study on supporting NR from 52.6GHz to 71 GHz", Intel

RP-201843, "New WID on Extending current NR operation to 71 GHz", Qualcomm

R4-2107608, "LS to RAN4 on maximum UE conducted power and maximum UE EIRP for operation in the 52.6 – 71 GHz band", RAN WG1