3GPP RAN WG4 Meeting #99e		R4-2109436

Electronic meeting, 19th – 27th May 2021

Agenda item:	9.2.3

Source:	Apple, Ericsson, Skyworks Solutions Inc.

Title:	TP on using overlapping channels from the network perspective solution

WI/SI:	FS_NR_eff_BW_util

Release:	Rel-17

Document for:	Approval

# 1	Introduction

During previous RAN TSG and WG4 meetings, several operators expressed an interest in enabling more efficient utilization of "non-standard" channel bandwidths, i.e. the ones which are not present now in TS 38.101 specifications. Referring to the corresponding operator requests, the following channel bandwidths were suggested by operators: 6, 7, 11, 12, 13, 33, 35, 45. After the RAN#88 meeting, a new WI was agreed to add explicitly 35 and 45MHz channel bandwidth into the 3GPP specifications [1]. At the same time, for "non-standard" channel bandwidths, which are not multiple of 5MHz, a new SI was agreed at the RAN#89 meeting aiming to study further which existing solutions can be used and whether new mechanism should be devised [2].

Most solutions and methods can be coarsely classified into the ones that require introduction of new channel bandwidths (either to the BS side only, or both to the UE and BS specifications) and the ones that leverage existing mechanism. In this paper we provide a text proposal for the "overlapping channels from the network perspective" solution.

# 2	Text proposal

## 6.2	Study of overlapping UE channel bandwidths

## 6.2.1 General

One way to utilise the whole chunk of irregular spectrum of a particular size is to combine several overlapping channels of next lower standard channel bandwidth. As an example, Figure X-1 shows a case when two overlapping 5MHz carriers cover 7MHz channel bandwidth. From an individual UE perspective, each UE is configured with existing immediately lower channel bandwidth following legacy procedures and signalling: one UE can use the first 5MHz carrier, while another UE can use another carrier. In fact, both UEs can use overlapping part of the spectrum provided that the BS takes care that the overlapping region is allocated to one particular carrier at a time. From the network perspective, the BS will/can use the irregular channel bandwidth. It should be also noted that from the UE perspective, an existing immediately lower channel bandwidth will be always used, either as an initial or the dedicated bandwidth part. However, this approach may require 3GPP to specify the new bandwidths for the BS side.  To limit specification efforts and impacts one approach is to apply a subset of requirements (e.g. regulatory) and ensure that the gNB has also met all requirements above and below the irregular bandwidth.  In the example below this would mean that the gNB has met all requirements for 10 MHz and 15 MHz while only a subset of requirements tested for 13 MHz.

Figure X-1: Using overlapping carriers (example for 13MHz).

It is worth noting that overall capacity of the cell will be according to the irregular channel bandwidth because the BS can use the full bandwidth. However, since a particular UE will use only one carrier of a smaller bandwidth within the irregular channel bandwidth, the maximum throughput for a single UE will be less than the theoretically possible within the spectrum in case there is only a single UE in the cell. Nevertheless, since there will be multiple UEs in the cell the overall system throughput will not decrease.

## 6.2.2 Detailed description

One of the challenges associated with configuring overlapping carriers for the same spectrum is that both carriers should have aligned grid so that the BS can perform same FFT and schedule resources in the overlapping region.  Moreover for alignment the initial BWP would be benefitical from BS coordination efforts to keep aligned between the UE dedicated BWPs.

Another challenge while aligning RB grids is not an issue for bands above 3GHz that have the SCS based raster, it becomes more challenging for the sub-3GHz band that have 100kHz raster. As a result, carriers can be configured on raster points that correspond to the least common multiple of the channel raster and the RB size. As an example, the least common multiple will be 900kHz in case of the 15kHz SCS, which corresponds to 5RBs. It effectively means that overlapping carriers will not be able to address efficiently any irregular spectrum size and in some case maybe will not be applicable at all. Figure X-2 presents an example for the 6MHz channel comprising two 5MHz channels. As can be seen from the figure, centre frequency distance between carriers is 900kHz, which is a multiple of 100kHz channel raster and 180kHz RB size. From an individual UE perspective, it is just a normal 5MHz carrier comprising 25RBs. From the BS perspective, it is a 6MHz channel with 30RBs and 300kHz guard bands. Similarly, Figure X-3 exemplifies how this approach can be used to support the 7MHz irregular channel bandwidth, in which the distance between the carriers is 2RBs i.e. 1800kHz. Finally, Figure X-4 shows the 11MHz channel that is supported with two 10MHz channels.

Figure X-2: Detailed overview of overlapping carriers (6MHz channel with 5MHz carriers).

Figure X-3: Detailed overview of overlapping carriers (7MHz channel with 5MHz carriers).

Figure X-4: Detailed overview of overlapping carriers (11MHz channel with 10MHz carriers).

Table X-1 below summarises potential number of schedulable RBs for a scenario when the next smaller overlapping channels are used. To calculate them, it is assumed that distance between individual carriers is a multiple of 900kHz and that the resulting guard bands must meet at least next smaller channel requirements. So, "Channel Nrb", "Channel guard bands", and "Utilisation" represent the network view, while from the UE perspective all the parameters are the same as for the next smaller channel.

Table X-1: Exemplary number of RBs based on the next smaller overlapping channel (15kHz SCS).

| Channel (MHz) | Next smaller channel (MHz) | Next smaller channel guard band (kHz) | Next smaller channel Nrb | Channel Nrb | Channel guard bands (kHz) | Utilisation (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 6 | 5 | 242,5 | 25 | 30 | 292,5 | 90 |
| 7 | 5 | 242,5 | 25 | 35 | 342,5 | 90 |
| 11 | 10 | 312,5 | 52 | 57 | 362,5 | 93,3 |
| 12 | 10 | 312,5 | 52 | 62 | 412,5 | 93 |
| 13 | 10 | 312,5 | 52 | 67 | 462,5 | 92,8 |

Table X-2 presents similar calculations for the number of available RBs with overlapping carriers, but for the 30kHz SCS. As can be seen from the table, a solution based on the 30kHz SCS overlapping carriers does not provide a good spectral utilisation for certain non-standard channel bandwidths due to the reason that the "distance" between carriers must be a multiple of 1800kHz. Because of that, channel bandwidths such as 7 and 12MHz have more or less good utilisation, whereas 6 and 11MHz do not provide any benefit at all.

Table X-2: Exemplary number of RBs based on the next smaller overlapping channel (30kHz SCS).

| Channel (MHz) | Next smaller channel (MHz) | Next smaller channel guard band (kHz) | Next smaller channel Nrb | Channel Nrb | Channel guard bands (kHz) | Utilisation (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 6 | 5 | 505 | 11 | 11 | 1005 | 66 |
| 7 | 5 | 505 | 11 | 16 | 605 | 82,3 |
| 11 | 10 | 665 | 24 | 24 | 1165 | 78,5 |
| 12 | 10 | 665 | 24 | 29 | 765 | 87 |
| 13 | 10 | 665 | 24 | 29 | 1265 | 80,3 |

To suppport two overlapping carriers, the network will most likely need to broadcast two separate SSBs, one for each carrier. In case of irregular channels of a small size, e.g. less than 10MHz, it may create the challenge of aligning SSBs in the time and frequency domain so that they do not overlap. To be more precise, the overlapping CORESET#0 (4.32 MHz) means that this approach complexity increases to coordinate the overlapping SSB for different UEs.As an example, if a particular irregular channel bandwidth does not allow for placing two SSBs in the same time slots, then the network will have two ensure that they are "multiplexed" accordingly in the time domain.

This approach works with all the legacy UEs. As mentioned earlier, from an individual UE perspective, this is just a standard Rel-15 channel and no special UE side enhancements are needed. Thus an operator can use this solution with the whole ecosystem of available devices.

# 3	Conclusions

In this discussion paper we have presented the TP for TR 38.844 describing the key details of the "overlapping channels from the network view" solution.

# 4	References

RP-201722, "Introduction of channel bandwidths 35MHz and 45MHz for NR FR1", Huawei

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson