3GPP TSG-RAN WG4 Meeting # 99-e                	R4-2109549

Electronic Meeting, May.19 - 27, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the interruption requirements at SRS antenna port switching

Agenda item:	9.9.2.1

Document for:	Discussion

Introduction

In RAN4#98bis-e meeting, the requirements at SRS antenna port switching were further discussed and some general principles for defining the interruption requirements were agreed as below [1]. In this paper we start from the discussion of different scenarios of SRS antenna switching. Afterwards we clarify the UE behavior based on existing specifications and discuss how to define the interruption requirements due to SRS antenna port switching.

- Do not define SRS antenna port switching delay requirement in RRM.

- Define the RRM requirements at SRS antenna switching only for FR1 unless the transient period in FR2 gets clarified in RF session (the scope of “RRM requirements” here depends on the conclusions from issue 1-1-1)

- The interruption requirement should be defined based on the band combination capability reported by UE, i.e., txSwitchImpactToRx or txSwitchWithAnotherBand

- Interruption requirement is based on the aggressor CC and victim CC SCS

Applicable Scenarios

In NR, the SRS antenna port switching is defined to enable channel sounding for DL CSI acquisition. The UE may be configured with SRS antenna switching depending on the indicated UE capability supportedSRS-TxPortSwitch, when the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching'.

The principles of switching the SRS transmission over respective antenna ports are specified in 3GPP TS 38.214 section 6.2.1.2 [2]. In particular, a guard period of Y symbols is defined, in which the UE does not transmit any other signal as cited below. However, the guard period is conditionally defined “in the case the SRS resources of a set are transmitted in the same slot”, and the guard period is “in-between” the SRS resources. In order to identify the requirements applicability, the respective scenarios are illustrated in Fig.1 based on the SRS resource configuration (taking 1T2R as example).

The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set.

The value of Y is defined by Table 6.2.1.2-1.

Table 6.2.1.2-1: The minimum guard period between two SRS resources of an SRS resource set for antenna switching

|  |  | Y [symbol] |
| --- | --- | --- |
| 0 | 15 | 1 |
| 1 | 30 | 1 |
| 2 | 60 | 1 |
| 3 | 120 | 2 |

- The SRS resources of a set are transmitted NOT in the same slot

- The SRS resources of a set are transmitted in the same slot - Consecutive transmission

- The SRS resources of a set are transmitted in the same slot – non-Consecutive transmission

Figure 1. Respective scenarios of SRS resources of a set at SRS antenna switching

In Fig.1(a), the SRS resources of a set are not configured in a slot. According to RAN1, the guard period does not apply in this scenario. The RAN4 requirements shall be defined for this scenario but not considering the guard period in RAN1.

Observation 1:  For the scenario in Fig.1 (a), RAN4 requirements shall be defined not considering the guard period in RAN1.

In Fig.1 (b), the SRS resources of a set are configured in a slot and the UE is expected to perform SRS antenna switching in consecutive manner. The SRS resources in a slot are therefore separated by exactly the minimum guard period. In this case, the UE is supposed to transmit SRS on different ports consecutively, and the minimum guard period e.g. one OFDM symbol in FR1, is used for switching the antenna to the other ports. The UE behavior is clear in RAN1 specification and the requirements shall be defined for the scenario in Fig.1(b) taking into account the guard period in-between the SRS resources.

Observation #2: For the scenario in Fig.1 (b), RAN4 requirements shall be defined taking into account the guard period in-between the SRS resources.

In Fig.1(c), the SRS resources of a set in a slot may also be configured being separated by a number of OFDM symbols more than the guard period of Y symbols, e.g. three OFDMs symbols in-between the SRS resources. While such SRS resource configuration is possible in current spec, it is not stated which of the three OFDM symbols is considered as the guard period, assuming the guard period is of Y symbol as defined in RAN1. Some vendors had a different view that all the OFDM symbols “in-between” the SRS resources can be counted within the guard period; hence the length of guard period may be extended to more than Y symbols up to the SRS configuration. We understood the SRS antenna switching is being discussed in Rel17 FeMIMO WI in RAN1 where this scenario is concerned. Given different understanding, it is not feasible to define the requirements before the guard period in this scenario gets clarified by RAN1. Therefore, we would propose not defining the requirements before the guard period in this scenario gets clarified by RAN1. If there is strong view to study the scenario in Rel17, we may further send LS asking for RAN1 clarification on the guard period in this case.

Observation #3: For the scenario in Fig.1 (c), RAN4 requirements shall be not defined before the guard period gets clarified by RAN1.

Given the above observations, it is proposed to define the RAN4 requirements starting from the scenarios in Fig.1 (a) and (b) in Rel17. If RAN4 concludes to also define the requirements for scenario in Fig. 1(c), we need to send LS to RAN1 asking for the clarification on the guard period.

Proposal #1: RAN4 shall define the requirements for the following scenarios in Rel17 where

- The SRS resources of a set are transmitted NOT in the same slot, or

- The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission

Proposal #2: RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period gets clarified in RAN1 in this scenario.

Interruption requirements

In last RAN4#98bis-e meeting, some general principles were agreed on defining the interruption requirements. Based on the applicable scenarios in section 2, we will discuss how to define the interruption requirements for the scenarios in Fig.1 (a) and Fig. 1 (b) respectively.

- The interruption requirement should be defined based on the band combination capability reported by UE, i.e., txSwitchImpactToRx or txSwitchWithAnotherBand.

  - FFS: SRS antenna switching interruptions on both DL and UL applies to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand.

- Interruption requirement is based on the aggressor CC and victim CC SCS.

When the SRS resources of a set are not configured in a slot, as in Fig.1 (a), the interruption may occur within the SRS transmission period and the transient period before or after or both before and after each SRS transmission. According to TS 38.101 defined in RF session, the transient period would take 10us when the PUCCH/PUSCH transmission is on the same antenna of SRS transmission, or 15us if they are on the different antennas. In the worst case, the transient period of 15us may occupy nearly one OFDM symbol i.e. 17.84us in case of 60kHz SCS. While at 15kHz and 30kHz SCS, only partial of one OFDM symbol is affected and it is unclear if the whole OFDM symbol will be interrupted. Therefore, the transient period due to antenna switching may bring an interruption of up to 1 OFDM symbol, dependent on the SCS.

Observation #4: For the scenario in Fig.1 (a), the transient period due to SRS antenna switching may bring an interruption of up to 1 OFDM, dependent on the SCS.

Figure 2 Uplink interruption when the SRS resources of a set are not configured in a slot

Another aspect which may cause interruption is the SRS transmission. Based on the rules cited from TS 38.214 as below, the SRS transmission on one carrier may bring interruption on the uplink transmission on other carriers, if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. Therefore, the UE is allowed an uplink interruption on any of the active serving cells, if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. And the uplink interruption comprises the SRS transmission period and the transient period dependent on the SCS.

Proposal #3: When the SRS resources of a set are not configured in a slot, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. And the uplink interruption comprises the SRS transmission period and the transient period dependent on the SCS.

In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol.

In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

When the SRS resources of a set are configured in a slot with consecutive SRS transmission, as in Fig.1 (b), there is a guard period of Y symbols in-between the SRS resources. As the UE does not transmit any other signals within the guard period as defined by RAN1, the interruption would occur during both the SRS transmission periods and the guard period. There was also proposal to take into account the transient periods before the first SRS transmission and after the last SRS transmission in one slot. As discussed above, these transient periods may also be counted as interruption dependent on the SCS.

Observation #5: For the scenario in Fig.1 (b), the transient periods before the first SRS transmission and after the last SRS transmission in one slot may also be counted as interruption dependent on the SCS.

Therefore, in the scenario as in Fig1(b), the UE is allowed an uplink interruption on any of the active serving cells, if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. And the uplink interruption comprises the SRS transmission periods, the guard period in-between and the transient periods dependent on the SCS.

Proposal #4: When the SRS resources of a set are configured in a slot with consecutive SRS transmission, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. The uplink interruption comprises the SRS transmission periods, the guard period in-between and the transient periods dependent on the SCS.

Figure 3 Uplink interruption when the SRS resources of a set are configured in a slot with consecutive Tx

In both scenarios, the transient periods may cause interruption due to antenna switching, however, the exact position of transient period depends on where the UE switches the antenna port, which may or may not happen on the OFDM symbols adjacent to the SRS resources. The position of the transient period needs to be clarified before defining the interruption and the scheduling restriction in RRM session.

Proposal #5: The position of the transient period needs to be clarified before defining the interruption and the scheduling restriction in RRM spec.

In addition to the uplink interruption as discussed above, the DL transmission on the carrier(s) may also be affected based on the UE capability of srs-Txswitch indicated in the BandCombinationList IE. In particular, if the IE txSwitchImpactToRx is configured, the DL may be “affected” by the SRS switching performed on the band(s) indicated in the IE hence the DL performance may be degraded due to less number of Rx antennas. But it is not clear if the DL is fully interrupted or may still survive despite of the degraded performance. From RAN4 point of view, we propose adding one note indicating the DL may be affected due to SRS antenna switching if txSwitchImpactToRx is configured. The text proposal for the note can be found in the end of this paper.

Proposal #6: Add one note indicating the DL may be affected due to SRS antenna switching if txSwitchImpactToRx is configured.

BandParameters-v1540 ::=            SEQUENCE {

srs-CarrierSwitch                   CHOICE {

nr                                  SEQUENCE {

srs-SwitchingTimesListNR            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR

},

eutra                               SEQUENCE {

srs-SwitchingTimesListEUTRA         SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA

}

}                                                                              OPTIONAL,

srs-TxSwitch                    SEQUENCE {

supportedSRS-TxPortSwitch       ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported},

txSwitchImpactToRx              INTEGER (1..32)                            OPTIONAL,

txSwitchWithAnotherBand         INTEGER (1..32)                            OPTIONAL

}                                                                              OPTIONAL

}

-	txSwitchImpactToRx indicates the entry number of the first-listed band with UL (see NOTE) in the band combination that affects this DL, which is mandatory with capability signaling;

-	txSwitchWithAnotherBand indicates the entry number of the first-listed band with UL (see NOTE) in the band combination that switches together with this UL, which is mandatory with capability signaling.

Conclusion

This contribution discussed the applicable scenarios of SRS antenna switching and the interruption requirements in respective scenarios. The observations and proposals are summarized as below:

Observation 1:  For the scenario in Fig.1 (a), RAN4 requirements shall be defined not considering the guard period in RAN1.

Observation #2: For the scenario in Fig.1 (b), RAN4 requirements shall be defined taking into account the guard period in-between the SRS resources.

Observation #3: For the scenario in Fig.1 (c), RAN4 requirements shall be not defined before the guard period gets clarified by RAN1.

Proposal #1: RAN4 shall define the requirements for the following scenarios in Rel17 where

- The SRS resources of a set are transmitted NOT in the same slot, or

- The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission

Proposal #2: RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period gets clarified in RAN1 in this scenario.

Observation #4: For the scenario in Fig.1 (a), the transient period due to SRS antenna switching may bring an interruption of up to 1 OFDM, dependent on the SCS.

Proposal #3: When the SRS resources of a set are not configured in a slot, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. And the uplink interruption comprises the SRS transmission period and the transient period dependent on the SCS.

Observation #5: For the scenario in Fig.1 (b), the transient periods before the first SRS transmission and after the last SRS transmission in one slot may also be counted as interruption dependent on the SCS.

Proposal #4: When the SRS resources of a set are configured in a slot with consecutive SRS transmission, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. The uplink interruption comprises the SRS transmission periods, the guard period in-between and the transient periods dependent on the SCS.

Proposal #5: The position of the transient period needs to be clarified before defining the interruption and the scheduling restriction in RRM spec.

Proposal #6: Add one note indicating the DL may be affected due to SRS antenna switching if txSwitchImpactToRx is configured.

References

- R4-2105786, WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching, Apple

- 3GPP TS 38.214 v16.3.0

Text Proposals

Note: The downlink may be affected due to SRS antenna switching if txSwitchImpactToRx is configured