3GPP TSG-RAN WG4 Meeting #99-e	R4-2109867

Online Meeting, May 19 – 27, 2021

Agenda item:	8.27

Source: 	Qualcomm Incorporated

Title: 	Introducing NR-U 100 MHz carrier bandwidth in bands n46 and n96

Document for:	Discussion

# Introduction

In RAN#90-e meeting a WF (R4-2017835) proposing the continuation of leftover NR-U items in R-16 was endorsed and captured in the agreement below [1]. This agreement includes the introduction of NR-U 100MHz carrier bandwidth (CBW) in bands n46 and n96 [2]. In this brief contribution we will discuss channel rasters for the new CBW in both bands and the spectral emission mask (SEM).

RAN endorses the RAN4 recommendations in R4-2017835 with the following additional details

- For the introduction of 100 MHz channel BW

  - The NR_bands_R17_BWs WID should be modified to add this new objective

  - Papers and discussion related to 100 MHz NR-U shall not be treated by block approval within this work item

# Channel raster for 100 MHz carrier bandwidth

Coexistence between NR-U and other technologies such as Wifi should be a key consideration in the formulation of a channel raster for 100 MHz CBW. Similar to the channel raster for NR-U 60 MHz CBW (see Appendix), the channel raster for NR-U 100 MHz CBW should ensure that each channel overlaps with no more than five 20 MHz Wifi channels. In some cases, it may be possible to place channels adjacent to the band edge, so that they overlap with only four 20 MHz Wifi channels. Otherwise, inevitably, a 100 MHz NR-U carrier will have partial overlap with some 40 MHz and 80 MHz Wifi channels. The number of impacted Wifi carriers impacted by a 100 MHz NR-U carrier should be limited to three 40 MHz, two 80 MHz and one 160 MHz carrier.

Even though partial overlap between channels cannot be avoided always, that does not necessarily imply a coexistence issue. Note that by using type A multi-channel access, NR-U transceivers are capable of accessing only 20 MHz sub-channels that are not occupied by other users, thereby ensuring fair access to the media by other wireless technologies such as Wifi.

Channel rasters for NR-U 100 MHz CBW in band n46 have been proposed in previous RAN4 meetings [3,4,5]. Our proposed channel raster for n46, shown in Table 1, is closer to the proposals in [3,4].

In [5], a rather limited channel raster for NR-U was proposed to try to avoid partial overlap with Wifi channels with CBW larger than 20 MHz. However, this was accomplished by relegating NR-U 100 MHz to band-edge channels. In our view, such an approach would not be fair since band-edge channels are burdened with more strict emissions requirements.

To reiterate, coexistence issues should be addressed effectively by means of efficient/flexible channel access procedures and equitable spectral emissions requirements. As noted before, NR-U supports multi-channel access procedures to facilitate coexistence with other wireless technologies. Additionally, in some deployment scenarios of interest for NR-U, e.g. private networks, coexistence considerations would be non-existent or easily avoided. E.g. NR-U supports semi-static channel access procedures “intended for environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies” [6]. Adopting an overly conservative approach that statically limits access to unlicensed spectrum by NR-U would be even less justifiable in such scenarios.

Figure 1: Proposed channels for NR-U 100 MHz in U-NII-1 and U-NII-2A.

Figure 2: Proposed channels for NR-U 100 MHz in U-NII-2C.

Figure 3: Proposed channels for NR-U 100 MHz in U-NII-3 and U-NII-4.

| Nominal channel freq (MHz) | NREF (NR-ARFCN) | FREF (MHz) |
| --- | --- | --- |
| 5200 | 746668 | 5200.02 |
| 5220 | 748000 | 5220.00 |
| 5240 | 749332 | 5239.98 |
| 5260 | 750668 | 5260.02 |
| 5280 | 752000 | 5280.00 |
| 5300 | 753332 | 5299.98 |
| 5520 | 768000 | 5520.00 |
| 5540 | 769332 | 5539.98 |
| 5560 | 770668 | 5560.02 |
| 5580 | 772000 | 5580.00 |
| 5600 | 773332 | 5599.98 |
| 5620 | 774668 | 5620.02 |
| 6540 | 776000 | 5640.00 |
| 5660 | 777332 | 5659.98 |
| 5680 | 778668 | 5680.02 |
| 5785 | 785668 | 5785.02 |
| 5805 | 787000 | 5805.00 |
| 5825 | 788332 | 5824.98 |
| 5845 | 789668 | 5845.02 |
| 5865 | 791000 | 5865.00 |

Table 1: Channel raster for NRU 100 MHz in band n46

Proposal 1: Adopt the channel raster in Table 1 for NR-U 100 MHz CBW in band n46.

For band n96 we propose a channel raster with points uniformly spaced by 20 MHz (nominal) ranging from 5985 MHz to 6665 MHz.

Proposal 2: Adopt the following channel raster for NR-U 100 MHz CBW in band n96, with raster points uniformly spaced by 20 MHz (nominal) ranging from 5985 MHz to 6665 MHz.

- Nominal channel frequency (MHz): Fk=5985+20∙k, for 0≤k≤34.

- Allowed NREF: NREFk=NREF,basek mod 3+4000∙k3, where NREF,base0=799000, NREF,base1=800332 and NREF,base2=801668.

- FREF  (MHz): FREFk=FREF,basek mod 3+60∙k3, where FREF,base0=5985.00, FREF,base1=6004.98 and FREF,base2=6025.02.

# Spectral emission mask

We support extending the general spectral emission mask (SEM) agreed for NR-U as proposed in [3]. The proposal is reproduced here for convenience. The general mask in Figure 4 also applies for 100MHz CBW, for the punctured emission mask, similar requirement should be approved:

- for single punctured channel in the middle, the emission mask of the puncture center is limited at -23dBr

- for multiple punctured channels in the middle, the emission mask in the middle is floored at -25dBr

- for punctured channel(s) at the edge, the emission mask edge is floored at -28dBr.

Figure 4: General spectral emission mask for NR-U

Proposal 3: The spectrum emission mask for 100MHz channel bandwidths in NRU should be supported as below:

- for full bandwidth transmission, the general spectrum emission mask in NRU is applied.

- for single punctured channel in the middle, the emission mask of the puncture center is limited at -23dBr.

- for multiple punctured channels in the middle, the emission mask in the middle is floored at -25dBr.

- for punctured channel(s) at the edge, the emission mask edge is floored at -28dBr.

# Conclusions

Proposal 1: Adopt the channel raster in Table 1 for NR-U 100 MHz CBW in band n46.

Proposal 2: Adopt the following channel raster for NR-U 100 MHz CBW in band n96, with raster points uniformly spaced by 20 MHz (nominal) ranging from 5985 MHz to 6665 MHz.

- Nominal channel frequency (MHz): Fk=5985+20∙k, for 0≤k≤34.

- Allowed NREF: NREFk=NREF,basek mod 3+4000∙k3, where NREF,base0=799000, NREF,base1=800332 and NREF,base2=801668.

- FREF  (MHz): FREFk=FREF,basek mod 3+60∙k3, where FREF,base0=5985.00, FREF,base1=6004.98 and FREF,base2=6025.02.

Proposal 3: The spectrum emission mask for 100MHz channel bandwidths in NRU should be supported as below:

- for full bandwidth transmission, the general spectrum emission mask in NRU is applied.

- for single punctured channel in the middle, the emission mask of the puncture center is limited at -23dBr.

- for multiple punctured channels in the middle, the emission mask in the middle is floored at -25dBr.

- for punctured channel(s) at the edge, the emission mask edge is floored at -28dBr.

# Appendix

Excerpt from TS 38.101-1 clause 5.4.2 for reference.

Table 5.4.2.3-2: Allowed NREF (NR-ARFCN) for operation in Band n46

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 10 MHz | 782000, 788668 |
| 20 MHz | 744000, 745332, 746668, 748000, 749332, 750668, 752000, 753332, 754668, 756000, 765332, 766668, 768000, 769332, 770668, 772000, 773332, 774668, 776000, 777332, 778668, 780000, 781332, 783000, 784332, 785668, 787000, 788332, 789668, 791000, 792332, 793668 |
| 40 MHz | 744668, 746000, 748668, 751332, 754000, 755332, 766000, 767332, 770000, 772668, 775332, 778000, 780668, 783668, 786332, 787668, 790332, 793000 |
| 60 MHz | 745332, 746668, 748000, 752000, 753332, 754668, 766668, 768000, 769332, 773332, 774668, 778668, 780000, 784332, 785668, 787000, 791000, 792332 |
| 80 MHz | 746000, 747332, 752668, 754000, 767332, 768668, 774000, 779332, 785000, 786332, 791668 |
| NOTE: 10 MHz channel bandwidth shall only apply in certain regions where the absence of non 3GPP technologies can be guaranteed on a long-term basis in this version of specification. |  |

# References

[1] RP-202890, Moderator’s summary for email discussion [90E][18][RAN4_NR-U_Leftovers]

[2] R4-2017835, WF on NR-U continuation work

[3] R4-2007323, Further considerations on 100MHz CBW in NR-U, Huawei/HiSilicon

[4] R4-20074717, Discussion on 100MHz for NR-U, ZTE

[5] R4-2104484, Co-existence challenges with NR-U 100MHz channel bandwidth and other technologies, Charter Communications, Cable Labs.

[6] 3GPP TS 37.213, clause 4.3