3GPP TSG-RAN WG4 Meeting # 99-e                  	R4-2110013

# Electronic Meeting, May. 19-27, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On PRS-RSRP measurement period definition

Agenda item:	6.5.1.2

Document for:	Discussion

Introduction

At RAN4 #98bis-e, the discussion on PRS-RSRP measurement period definition was continued. Further remaining issues are captured in WF [1]. This contribution treats some of the open issues.

Discussion

RSRP measurement period with RSTD or UE Rx-Tx

- PRS-RSRP requirements when configured for DL-TDOA or multi-RTT - Option 1 (QC) - According to a prior agreement, RAN4 may revise the number of samples (Nsample) in the requirement if more samples are needed to meet PRS-RSRP accuracy requirements. - Option 2a (vivo, HW, CATT, OPPO, Intel) - UE behaviour is not defined when PRS-RSRP is configured additionally to RSTD or UE Rx-Tx measurement. - Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT. - Option 2b (Nokia, Intel) - Regarding UE behavior, a UE continues to measure PRS-RSRP over the entire RSTD/UE Rx-Tx measurement period, when PRS-RSRP is configured together with RSTD/UE Rx-Tx. - RSRP requirements in clause 9.9.3 also apply for the case when configuring PRS-RSRP with RSTD or UE Rx-Tx. - Option 3 (Ericsson, Nokia) - UE behavior when PRS-RSRP is configured together with RSTD/UE Rx-Tx and the required PRS-RSRP measurement period is shorter than that for RSTD/UE Rx-Tx (configured without PRS-RSRP), then the PRS-RSRP measurement continues over the entire RSTD/UE Rx-Tx measurement period.

We need to investigate RSRP value usage in localization algorithm. LMF can request various positioning measurements for specific localization algorithm, the RSRP values are very fundamental values to set up positioning measurements such as beam correspondence and to evaluate measurement reliability.

Observation 1 : RSRP values are very fundamental values to set up positioning measurements such as beam correspondence and to evaluate measurement reliability.

Regarding UE behavior, the LPP spec TS 37.355 defines a UE capability report on DL TDOA measurement with DL-PRS RSRP measurements “supportofDL-PRS-RSRP-Meas” field.

RAN1/2 define separate UE capability on PRS-RSRP measurement combined for other positioning methods for respectively FR1 and FR2. If a UE is capable to measure PRS-RSRP for the OTDOA method, it is reasonable to report the additional measurement element as same period of RSTD report.

Observation 2 : RAN1/2 define separate UE capability on PRS-RSRP measurement when it is combined for other positioning methods (i.e. DL-ToA or RTT).

For other DL positioning such as UE RX-TX TD and DL-AoD, the same additional positioning measurements are found. PRS-RSRP measurement can be reported as an additional measurement report.

Proposal 1 :  When PRS-RSRP measurement is required when configured for DL-TDOA or multi-RTT, the measurement period is supposed to be same as DL-TDOA or multi-RTT measurement period respectively.
   - Accordingly, the DL-TDOA or multi-RTT measurement period are respectively applied to PRS-RSRP in this case, which means that PRS-RSRP measurement period is same as  DL-TDOA or multi-RTT measurement period.

In other words, if PRS-RSRP is an additional measurement for DL-TDOA or multi-RTT, the measurement period of PRS-RSRP is supposed to be same as DL-TDOA or multi-RTT measurement period.

Conclusion

This contribution has provided our view on some open issues regarding PRS-RSRP measurement period requirement :

Observation 1 : RSRP values are very fundamental values to set up positioning measurements such as beam correspondence and to evaluate measurement reliability.

Observation 2 : RAN1/2 define separate UE capability on PRS-RSRP measurement when it is combined for other positioning methods (i.e. DL-ToA or RTT).

Proposal 1 :  When PRS-RSRP measurement is required when configured for DL-TDOA or multi-RTT, the measurement period is supposed to be same as DL-TDOA or multi-RTT measurement period respectively.
   - Accordingly, the DL-TDOA or multi-RTT measurement period are respectively applied to PRS-RSRP in this case, which means that PRS-RSRP measurement period is same as  DL-TDOA or multi-RTT measurement period.

References

- R4-2105743, "WF on UE PRS measurement requirements", source: Huawei, HiSilicon