# 3GPP TSG-RAN WG4 Meeting # 99-e 						      		 R4-2110020

# Electronic Meeting, May 19-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0xxx | rev |  | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME | X | Radio Access Network |  |  |  |  |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | CR for TS 38.101-3 switching period for V2X con-current operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  |  |  |  |  | Date: |  |  | 2021-4-25 |  |  |  |
| Category: | F |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-16 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |  |
| Reason for change: |  | The switching period of V2X con-current operation has not been added in the specification. This CR is to complete this part. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, A Reference to TS 38.213 is added. 2, The switching period requirement is added. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for switching period of V2X con-current operation is not completed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.3E.1, 6.3E.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  | X |  |  | Test specifications |  |  |  |  |  |  | TS 38.521-3 |  |  |  |  |  |
| (show related CRs) |  |  |  |  | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<Start of 1st Change>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[4]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

[5]	3GPP TS 38.521-3: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[6]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[7]	3GPP TS 36.211: "E-UTRA; Physical channels and modulation"

[8]	3GPP TS 36.331: " Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification"

[9]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[10]	3GPP TS 38.213: "NR; Physical layer procedures for control"

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities"

[12]	3GPP TS 38.133: "NR; Requirements for support of radio resource management"

[13]	3GPP TS 38.211: "NR; Physical channels and modulation".

[14]	3GPP TS 38.214: "NR; Physical layer procedures for data"

[15]	3GPP TS 38.133: "NR; Requirements for support of radio resource management"

[16]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management"

[X]	3GPP TS 38.213: "NR; Physical layer procedures for control"

<End of 1st Change>

<Start of 2nd Change>

6.3E	Output power dynamics for V2X operation in FR1

6.3E.1	General

The E-UTRA SL and NR SL switching time mask defines the observation period between E-UTRA subframe and NR slot/mini-slot boundary. Both E-UTRA subframe and NR slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA subframe or NR slot/mini-slot having OFF power transmission, the general time mask for E-UTRA or NR shall apply.

For intra-band V2X operation, the output power dynamics specified in clause 6.3.2G, 6.3.3G, 6.3.4G and 6.3.5G [4] shall be applied for E-UTRA sidelink transmission or the output power dynamics specified in clause 6.3E [2] shall be applied for NR SL transmission, respectively.

For the inter-band con-current NR V2X operation, output power dynamics requirement specified in subclause 6.3 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.3E of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

6.3E.2	Output power dynamics for intra-band V2X operation

The switching periods described in Figure 6.3E.2-1 and Figure 6.3E.2-2 are located in the lower priority carrier of the NR sidelink and LTE sidelink while the priority is determined by the SCI formats scheduling the transmissions of the channel(s)/signal(s), or, is indicated by higher layers in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, or, is equal to the priority of the corresponding PSSCH in case of PSFCH as defined in [X]. The length of the switching period is not defined based on the scheduling restriction of RRM requirement as specified in [12].

Figure 6.3E.2-1: Time mask for switching between NR V2X SL and E-UTRA V2X SL, where the switching period is located in NR V2X SL

Figure 6.3E.2-2: Time mask for switching between E-UTRA V2X SL and NR V2X SL, where the switching period is located in E-UTRA V2X SL

<End of Change>