# 3GPP TSG-RAN WG4 Meeting # 99-e												 R4-2110030

# Electronic Meeting, May. 19-27, 2021

Title: 	on UE TX requirement and regulation requirements

Source: 	Xiaomi

Agenda item:	9.15.4.1

Document for:	Approval

Background

The UE TX requirement and regulation requirements have been discussed during the last meeting. The EU regulation of ECC and the US regulation have been mentioned however the source and detail are not stated clearly. In this paper we would like to further clarify the issue.

Discussion

During the last meeting, a WF [1] on the NR extend to 71GHz with multiple issues has been agreed and the regulation baseline is agreed as below:

- Proposals

  - Option 1: ETSI EN 303 753 harmonized standard as baseline (Nokia, Xiaomi, LGE)

  - Option 2: ETSI EN 303 753 is one of considerations and cannot be baseline (Qualcomm, Ericsson)

      - WF (GTW on Apr. 14): :

  - RAN4 to consider EU harmonized standards as starting point, not precluding other available regulatory requirements

During the meeting, it has been provided to the group with ECC regulation the ERC Recommendation 70-03 [2] on short range devices and corresponding wideband data transmission systems. The regulation parameter as shown below. Furthermore, the harmonized standards captured in the recommendation 70-03 as also shown in the following figure 2.

Figure 1 Wideband data transmission systems parameter

Figure 2 Harmonised standards

In this case, it can be concluded the corresponding harmonized standards to the specific frequency band and the use case scenarios as below. For C2 and C3 bands draft standard is on-going within the corresponding ETSI group as also captured in table 1.

Table 1 EU harmonised standard and scope

| Frequency band | Harmonised standard | Scope |
| --- | --- | --- |
| C1 57-71GHz | EN 302 567 for sub-band c1 as 57-66GHz |  |
| C2 57-71GHz | Draft EN 303 753 | Mobile and Fixed Radio Equipment |
| C3 57-71GHz | Draft EN 303 722 | Fixed Network Radio Equipment |

From the table 1 above, it seems the on-going draft EN 303 753 which covers band c2 includes both mobile and fixed radio equipment and will be more suitable to be set as a reference of UE RF requirement for 3GPP.

For the FCC regulations for 57—71GHz, it has been captured in the federal regulations part 15.255: “Operation within the band 57-71 GHz”. The FCC part 15.255 describes the operation requirements for intentional radiators which operate within the 57-71GHz band. The E.I.R.P for average and peak power, spurious emission and the human body exposure requirements are specified. Hence from UE RF requirement perspective, at least both the EN 303 753 and the FCC prat 15.255 should be both considered.

Observation 1: From UE RF requirement perspective, at least the FCC prat 15.255 should be also considered.

The corresponding UE TX requirements which both captured in ETSI and FCC standard are listed below as table 2:

Table 2: RF requirements for EU and FCC regulation

| Requirement | EU Regulation | FCC part 15.255 |
| --- | --- | --- |
| RF output power | 40dBm EIRP 27dBm at antenna port | EIRP: 40dBm Average 43dBm Peak Conducted: 27dBm (500mW) |
| Spurious emission | The table is from EN 302 567 as current draft EN 303 753 hasn’t finished the spurious emission part yet. The requirement is similar to the 2.4GHz WIFI requirement as only extends the highest frequency to 142GHz. | <40GHz: Specified in 15.209 40—200GHz: Emissions shall not exceed 90 pW/cm2 at a distance of 3 meters. |
| RF exposure | Not specified | As specified in federal rule part 1.1307(b), 1.1310, 2.1091, and 2.1093 which is same as legacy UE requirements. |

Based on the above comparison, we can see for RF output power, the 27dBm conducted requirement and 40dBm EIRP can be set as the starting point of UE RF output power and to further consider the feasibility and the UE power class definition as well as the use case scenario.

Proposal 1: 27dBm conducted requirement and 40dBm EIRP to be set as the starting point of UE RF output power

For the spurious emission requirement, for below 40GHz, the requirement of EU and FCC are the same to their own WIFI requirement respectively as EN 300 328 and FCC prat 15.247. These requirements are comparable to UE FR1 spurious emission. However, for current UE FR2 spurious emission as captured in TS 38.101-2, the requirement is -13dBm for 12.75GHz above comparing to -30dBm for above 1GHz of EU regulation, which is more stringent for regulations. In this case, further study of the feasibility of spurious emission for UE operating in 57-71GHz is needed.

Observation 2: Current regulation spurious emission is more stringent comparing to UE FR2 requirement.

Conclusions

In this contribution, we give discussion on the TRP and TRS work plan and the observations are shown as below：

Observation 1: From UE RF requirement perspective, at least the FCC prat 15.255 should be also considered.

Proposal 1: 27dBm conducted requirement and 40dBm EIRP to be set as the starting point of UE RF output power

Observation 2: Current regulation spurious emission is more stringent comparing to UE FR2 requirement.

Reference

- R4-2105410	WF on [137] NR_ext_to_71GHz_Part1, Intel (Moderator)