3GPP TSG-RAN WG4 Meeting # 99-e																R4-2110064

Electronic Meeting, May. 19-27, 2021

Agenda Item:	9.10.2.1

Source:	OPPO

Title:	On pre-configured MG pattern(s) for NR_MG_enh

Document for:	Approval

- Introduction

A WF on pre-configured MG pattern was approved in last RAN4#98bis meeting [1]. In this contribution, we provide our views on the remaining issues for pre-configured MG pattern for RRM measurement gap enhancement in R17.

- Discussion

Using scenarios:

- Focus on the scenarios with the pre-configured MG for single CC BWP switching scenario.

- FFS whether to define pre-configured MG for multiple CC BWP switching scenario for NR-CA case

  - Consider this in 2nd stage.

- Whether the pre-configured gaps shall be considered as a part of multiple concurrent gap patterns framework

  - Consider this in 2nd stage.

- Pre-configured MG for SSB measurements shall be configured.

- FFS whether the pre-configured MG shall be configured for PRS measurement:

  - Option 1. Yes

  - Option 2. No

- FFS whether the pre-configured MG shall be configured for CSI-RS L3 measurement:

  - Option 1. Yes

  - Option 2. No

For using scenarios, it has been agreed that SSB based measurement shall be configured with per-configured MG. In our view, it should be also feasible for PRS measurement and CSI-RS L3 measurement, or other dedicated measurements. However, to be simplified, pre-configured MG for PRS measurement should be hold until RAN4 scope of R17 positioning WID has been decided. Therefore, pre-configured MG shall be also allowed for CSI-RS L3 measurement. How to ensure such MG can be used for SSB or CSI-RS based measurements separately, can be further discussed and even ask RAN2 for potential configuration design.

Proposal 1: Pre-configured MG shall be also allowed to be configured for CSI-RS L3 measurement.

- Configuration procedure

- Pre-configured MG(s) are configured per UE or per FR

  - FFS if pre-configured MGs can be additionally configured per BWP

- How pre-configured MGs can be configured:

  - The common configuration parameters of pre-configured MG (e.g. MGRP, MGL, etc) which are same as these of Rel16 legacy MG can be configured by the similar way as the Rel16 legacy MGs

  - FFS on the specific configuration parameters for the new aspects of pre-configured MG to be introduced,

    - One single bit (or flag) per BWP to indicate whether the pre-configured MG is ON or OFF when that BWP becomes active

    - Others are not precluded.

- FFS on whether MG configuration can be changed after BWP switching

  - Option 1: Yes

  - Option 2: No

Since RAN4 agreed to start pre-configured MG for single CC BWP switching scenario, it is still possible to configure different per-configured MG for at most 4 BWPs in this CC. Regarding the common configuration parameters of pre-configured MG (e.g. MGRP, MGL, etc) which are same as these of Rel16 legacy MG, two possible configurations can be allowed for per-configured MGs.

- Option 1: all BWPs in one CC are associated with 1 set of MG configuration (i.e., 1 MG pattern)

- Option 2: each BWP　or subset of BWPs in one CC are associated with 1 set of MG configuration (i.e., 1 MG pattern), with specific configuration parameters per BWP, e.g., MGRP, MGL, MG offset

In our view, both options are valid, and option 1 seems a simple one. A specific configuration parameters per BWP can be additional configured for this kind of pre-configured MG, e.g., one single bit (or flag) per BWP to indicate whether the pre-configured MG is ON or OFF when that BWP becomes active. In this case, network signals the status (activated or deactivated) when pre-configured MG is configured.

However, option 2 also seems flexible for both network and UE, which can enable per-configured MG for more scenarios of measurements. Different gap pattern is allowed to be configured for different UE BWP. It leads to multiple MG patterns can be configured for one single CC or a UE. And in this case, MG configuration can be changed after BWP switching. As agreed, whether pre-configured gaps shall be considered as a part of multiple concurrent gap patterns framework, should be considered in 2nd stage, we can compromise to keep option 2 as an open issue at this stage. Once considering pre-configured gaps as a part of multiple concurrent gap patterns, we can further discussion option 2 as a candidate solution.

Therefore, at this stage, RAN4 should focus on the scenario with BWP switch on single CC cases firstly. Anyway, for either options, pre-configured MGs can be additionally configured per BWP.

Observation 1: Assuming pre-configured MGs can be configured per BWP, two options are suggested:

- Option 1: all BWPs in one CC are associated with 1 set of MG configuration (i.e., 1 MG pattern), when per-configured MG is not considered for multiple concurrent gaps.

- Option 2: each or subset of BWPs in one CC are associated with 1 set of MG configuration (i.e., 1 MG pattern), when per-configured MG is considered as part of multiple concurrent gaps.

Observation 2: MG configuration can be changed after BWP switching, if per-configured MG is considered as part of multiple concurrent gaps.

Proposal 2：Pre-configured MGs can be additionally configured per BWP.

- FFS whether the pre-configured MG is activated or not after configuration completed

  - Option 1: Activated by default

  - Option 2: Not activated by default until being activated.

  - Option 3: Status of pre-configured MG is not fixed (no default status):

  - Option 3a: Whether pre-configured MG activated or not depends on whether reference signal to measure is within the active BWP or not.

  - Option 3b: Network signals the status (activated or deactivated) when pre-configured MG is configured

- FFS on how pre-configured MGs can be activated/deactivated:

  - Option 1: Autonomously/implicitly triggered by condition change

  - Option1a: Autonomously triggered by BWP switching DCI/timer but explicitly directed according to the pre-configured ON/OFF state/bit for the target BWP

  - Option 2: If the use case of MG pattern change following BWP switch is considered, to activate/deactivate the pre-configured MG by the network indication

In our view, if per-configured MG is not considered for multiple concurrent gaps, whether per-configured MG is activated or not for one BWP can be also implicitly indicated via BWP switching, under the condition that per-configured MG is “not activated by default” until being activated along with BWP switch/activation. In this case, such configuration procedure is also workable, and can avoid increases signalling overhead.

Observation 3: If per-configured MG is not considered for multiple concurrent gaps, prefer whether per-configured MG is activated or not for one BWP can be implicitly triggered by BWP switching,

- e.g., per-configured MG is “not activated by default” until being activated along with BWP switch/activation.

Proposal 3: Prefer default status of ‘Not activated’ for pre-configured MG.

- The conditions to activate/de-activate the Pre-configured MG are:

  - BWP switching

  - For activation decision, MO configured can perform gapless measurement before BWP switching but not after BWP switching. For deactivation decision, MO configured can perform gapwith measurement before BWP switching but can not after BWP switching.

  - Other conditions are not precluded

- FFS on whether NW can fully control the pre-configured MG being activated/deactivated

- FFS on evaluation on MG activation/deactivation mechanism

- FFS on UE behavior after deactivation of pre-configured MG

As agreed in last meeting, UE behavior after deactivation of pre-configured MG needs to be further discussed. When UE’s active BWP was not per-configured with any gap pattern, it still triggers the deactivation of per-configured MG of previous BWP after switching to this BWP. Whether gap is still needed should be clarified:

- Option 1: UE shall perform measurement without gap.

- Option 2: UE shall perform measurement with legacy gaps.

- Option 3: UE shall perform measurement with a default gap through immediate activation.

Observation 4: UE behavior should be clarified after deactivation of pre-configured MG and switching to a new BWP without any per-configured gap.

Proposal 4: For UE behavior after deactivation of pre-configured MG,

- Option 1: UE shall perform measurement without gap.

- Option 2: UE shall perform measurement with legacy gaps.

- Option 3: UE shall perform measurement with a default gap through immediate activation.

On applicability of pre-configured MG pattern(s), as NR UE does not support per-CC gap, the current types of gap, i.e., per UE and per FR gap, can be reused for pre-configured MG. If the gap patterns which can be used as the pre-configured gap are reused from Rel16, the same applicability(per-UE/per-FR) shall follow the rules defined in Rel16.

- Number of pre-configured MG patterns:

  - A single pre-configured MG is considered for the case of non-concurrent MG scenarios.

  - FFS if more pre-configured MGs shall be considered for the multiple concurrent MG scenarios

- FFS on MG patterns used for the pre-configured MG mechanism

  - Option 1: The existing gap patterns (0~23) in Rel16 can be reused for the pre-configured MG.

  - Option 2: The existing gap patterns (0~25) in Rel16 can be reused for the pre-configured MG.

In our view，the existing gap patterns (0~23) in Rel16 can be reused for the pre-configured MG. For gap pattern 24，25 for positioning, they can be discussed at 2nd stage when considering multiple concurrent gaps or in R17 positioning WID.

Proposal 5: The existing gap patterns (0~23) in Rel16 can be reused for the pre-configured MG.

- FFS on activation/deactivation delay

  - Option 1: No separated activation delay for the pre-configured MG activation/deactivation

  - Option 2: some transition time (T) shall be included in the pre-configured MG activation/deactivation time.

  - Option 2a

    - MG deactivation delay of [Y]ms is defined from the DCI of BWP switch is received to the earliest time instant when MG can be aborted.

    - MG activation delay of [X]ms is defined from the completion of target BWP activation to the earliest time instant when a MG can be entered.

    - No requirements shall be defined for the measurements within the gaps during [Y] and/or [X]ms

  - Option 2b The pre-configured delay includes BWP switch delay with additional validation time. The validation time is 20ms.

  - Others

- FFS on measurement period for the measurements with the pre-configured MGs

- FFS on transitions between gapless and gap-based measurement procedures during ongoing measurements

  - Option 1:

    - RAN4 is to investigate the maximum number of transitions (N1,max) allowed for switching from gapless measurement procedure to gap-based measurement procedure during the ongoing measurement.

    - RAN4 is to investigate the maximum number of transitions (N2,max) allowed for switching from gap-based measurement procedure to gapless measurement procedure during the ongoing measurement

  - Option2 : No need

In our view, assuming RF tuning time has been already considered in MGL, BWP switching delay would be not extended owing to additional parameters of MG configuration adjustment and applying. To avoid misalignment of network and UE, MG configuration is expected to become valid after BWP switching delay. But the exact starting time of MG depends on MG offset, MGTA, etc.

Proposal 6: No separated activation delay for the pre-configured MG activation/deactivation.

- Summary

In this contribution, we provided work plan for WI NR RRM measurement gap enhancement for approval.

Proposal 1: Pre-configured MG shall be also allowed to be configured for CSI-RS L3 measurement.

Observation 1: Assuming pre-configured MGs can be configured per BWP, two options are suggested:

- Option 1: all BWPs in one CC are associated with 1 set of MG configuration (i.e., 1 MG pattern), when per-configured MG is not considered for multiple concurrent gaps.

- Option 2: each or subset of BWPs in one CC are associated with 1 set of MG configuration (i.e., 1 MG pattern), when per-configured MG is considered as part of multiple concurrent gaps.

Observation 2: MG configuration can be changed after BWP switching, if per-configured MG is considered as part of multiple concurrent gaps.

Proposal 2：Pre-configured MGs can be additionally configured per BWP.

Observation 3: If per-configured MG is not considered for multiple concurrent gaps, prefer whether per-configured MG is activated or not for one BWP can be implicitly triggered by BWP switching,

- e.g., per-configured MG is “not activated by default” until being activated along with BWP switch/activation.

Proposal 3: Prefer default status of ‘Not activated’ for pre-configured MG.

Observation 4: UE behavior should be clarified after deactivation of pre-configured MG and switching to a new BWP without any per-configured gap.

Proposal 4: For UE behavior after deactivation of pre-configured MG,

- Option 1: UE shall perform measurement without gap.

- Option 2: UE shall perform measurement with legacy gaps.

- Option 3: UE shall perform measurement with a default gap through immediate activation.

Proposal 5: The existing gap patterns (0~23) in Rel16 can be reused for the pre-configured MG.

Proposal 6: No separated activation delay for the pre-configured MG activation/deactivation.

References

[1] R4-2105791, WF on R17 NR MG enhancements – Pre-configured MG patterns and NCSG, Intel Corporation

[2]	R4-2106535, On pre-configured MG pattern(s) for NR_MG_enh, OPPO