3GPP TSG-RAN WG4 #99-e	R4-2110220

Electronic meeting, 19th – 27th May 2021

Source: 	Ericsson

Title:  	On SCell intra-frequency measurements for NR HST in FR1

Agenda Item:	9.7.2.1.2

Document for:	Discussion

# Introduction

According to work plan [1] on enhancement for NR high speed train scenario in FR1, we shall:

- Continue to discuss and decide the solution for the RRM enhancement

- Provide draft CR on TS38.133 for the RRM enhancement

- Discuss and agree on the possible signaling impact (e.g. UE capabilities, network flag) and send LS to RAN2

In this contribution, we discuss further open issues about intra-frequency measurements on above items.

# Discussion

In [2], there are agreements on L1-SINR:

L1-SINR - Option 1 (MTK): For L1-SINR measurement in Rel-17 HST, the upper bound of the side condition SSB Ês/Iot ≤5 dB should be introduced, for CMR only case at least - Other options are not precluded

Recalling SS-SINR issue, the problem is ‘intra-SSB’ ICI may exist with assumption of HST-SFN joint transmission and multiple taps HST channel mode.

Considering that Doppler shift also happens when beams from 2 or more RRHs belonging to one cell, UE gets L1-SINR with interference which is similar with SS-SINR topic discussed before.

Proposal 1: L1-SINR was introduced in Rel-16 eMIMO, we should wait conclusion in Rel-16 HST. We’re OK to use Ês/Iot ≤5 dB from SS-SINR in L1-SINR for CMR case.

# Conclusion

Proposal 1: L1-SINR was introduced in Rel-16 eMIMO, we should wait conclusion in Rel-16 HST. We’re OK to use Ês/Iot ≤5 dB from SS-SINR in L1-SINR for CMR case.

# References

[1] R4-2017267, Work plan for enhancement for NR high speed train scenario in FR1, CMCC

[2] R4-2105793, WF on FR1 HST RRM, CMCC