3GPP TSG-RAN WG4 Meeting #99-e	R4-2110547

# Electronic Meeting, 19th - 27th May, 2021

Title: 	Discussion and simulation results on NR UE demodulation for 35MHz and 45MHz bandwidth

Source: 	Huawei, HiSilicon

Agenda item:	8.28.5

Document for:	Discussion

Background

At the RAN4#98-bis-e meeting, the Way forward [1] on UE demodulation and CQI reporting for channel bandwidths 35MHz and 45MHz for NR FR1 was approved. In this contribution, we share our views about the demodulation requirements for NR UE 35MHz and 45MHz bandwidth.

Discussion

Subband size for CA CQI reporting

- Introduction of CA CQI requirements for 35MHz and 45MHz bandwidth (FDD) - Add the subband size in TS38.101-4 Table 6.2A.3.1.1-2. - It is suggested to revise WID to define CA CQI reporting requirements for channel bandwidths 35MHz and 45MHz.

The only remained issue is the subband size for CA CQI reporting. As per TS 38.214, the subband size can be 16 PRBs or 32 PRBs for the bandwidth part within the range of 145-275. Considering that 32 PRBs is selected for the existing 30MHz, 40MHz and 50MHz requirements, it is reasonable to select the same value for 35MHz and 45MHz bandwidth.

Select subband size 32 for both 35MHz and 45MHz bandwidth requirements.

Simulations

PDSCH CA

The simulation results for PDSCH CA is shown as Table 3.1-1 and Figure 3.1-1 below, as per simulation assumption in [1].

Table 3.1-1 Ideal simulation results for PDSCH CA

| Case number | Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 35 | R.PDSCH.1-10.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 10.86 |
| 2 | 45 | R.PDSCH.1-10.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.38 |
| 3 | 35 | R.PDSCH.1-10.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 5.22 |
| 4 | 45 | R.PDSCH.1-10.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 5.48 |

Figure 3.1-1 Ideal simulation results for PDSCH CA

Proposals

In this contribution, we share our views on demodulation performance for NR UE demodulation for 35MHz and 45MHz bandwidth. Our observations and proposals are:

- Select subband size 32 for both 35MHz and 45MHz bandwidth requirements.

Reference

R4-2106090, Way forward on UE demodulation and CQI reporting for channel bandwidths 35MHz and 45MHz for NR FR1, RAN4#98-bis-e, Ericsson