3GPP TSG-RAN WG4 Meeting #99-e	R4-2110571

# Electronic Meeting,  May. 19th – 27th, 2021

Title: 	Discussion on Intel-cell CRS-IM

Source: 	Huawei, HiSilicon

Agenda item:	9.11.2.3

Document for:	Discussion

Background

In RAN #91-e meeting, the work item on Rel-17 UE and BS demodulation performance enhancement was approved and it was updated with small modification in [1]. For the UE demodulation part, the objectives are as follows:

UE demodulation and CSI reporting requirements:

- Evaluate techniques to cope with CRS interference in scenarios with overlapping spectrum for LTE and NR

    - Candidate reference receiver to enable neighbouring cell CRS-IM

    - The performance benefit of neighboring cell LTE CRS-IM over the existing rate matching solutions specified in Rel-15 and Rel-16 shall be evaluated.

    - Feasibility of the considered solution regarding NR PDSCH processing timeline need to be checked.

    - Priority will be given to solutions not having RAN1 specification impact.

    - Synchronous network scenario is prioritized. As second priority, RAN4 could evaluate the feasibility and usefulness of the asynchronous network scenario and specify if feasible and useful.

    - 15 kHz SCS for NR is prioritized. RAN4 should evaluate the feasibility and usefulness of 30 kHz SCS for scenarios with LTE and NR deployed in neighboring BSs/areas and specify if feasible and useful.

Note: The work can be started from May 2021 meeting.

In this paper, some parameters for CRS-IM are discussed and proposed.

Discussion

As companies agreed in the WID that the performance benefit of inter-cell LTE CRS-IM over the existing rate matching solution should be evaluated, the evaluation results are provided in this paper. Meanwhile, as the link level simulation assumptions will be discussed in RAN4 #99 e-meeting according to the work plan, so the parameters for both of NR cell and LTE cell are proposed here.

Parameters:

In NR, the minimum requirements for LTE-NR coexistence with solution of rate-matching has been defined. We propose to reuse the parameters as much as possible.

SCS and bandwidth:

As agreed in the RAN #91-e meeting, 15 kHz is prioritized. By considering this is the NR-LTE coexistence scenario, we propose to only define requirements with 15 kHz and 10 MHz for both of NR and LTE cells.

Proposal 1: We propose to define the requirements for 10 MHz/15 kHz for FDD and TDD.

Propagation condition:

In this WID, the CRS-IM is defined for the inter-cell interference. So, the cell-edge users will be mainly considered. Thus, a propagation condition with low time delay and non-frequency selective channel could be defined to achieve a better performance improvement. Thus, we propose to define TDLA30-10 as the propagation condition.

Proposal 2: We propose to use TDLA30-10 as the propagation condition.

Other parameters for NR cell:

Proposal 3: For NR serving cell, we propose the parameters in table below:

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD/TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Frequency allocation |  |  | Full bandwidth |
| PMI precoding model |  |  | Random |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Other parameters for LTE cell:

For LTE interference cell, we propose the parameters in table below:

Proposal 4: We propose the following parameters for LTE cell:

| Parameter | Value |
| --- | --- |
| Number of interfering cell | 1 interfering cell |
| Signal level for interfering cells CRS (interference over Noc) | 5dB, 10dB, 15dB, 20dB |
| Network synchronization | All cells are synchronous |
| CRS configuration | 4 CRS ports per cell with planning |
| HARQ | 8 HARQ processes and max 4 transmissions |
| Resource allocation | Full bandwidth |
| PMI | Random PMI |

Simulation results:

To evaluate the benefit of CRS-IM over rate-matching solution, the performance of CRS-IM is simulated with proposed parameters and the performance of rate-matching is simulated with Rel-16 exiting parameters.

- Rank 1:

For rank 1, INR values of 5dB and 15dB are used, the results are shown in figure below:

With the help of CRS-IM, the demodulation performance at 70% maximum throughput is improved about 2dB for INR value of 5dB and 2.8dB for INR value of 15dB.

- Rank 2:

For rank 2, INR values of 10dB and 20dB are used, the results are shown in figure below:

Comparing the SNR values at 70% maximum throughput, the gain is about1.8dB for INR value of 10dB and 2dB for INR value of 20dB.

The CRS-IM improves the demodulation performance compared with rate matching solution.

Conclusion

In this paper, we have the following proposals:

Proposal 1: We propose to define the requirements for 10 MHz/15 kHz for FDD and TDD.

Proposal 2: We propose to use TDLA30-10 as the propagation condition.

Proposal 3: For NR serving cell, we propose the parameters in table below:

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD/TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Frequency allocation |  |  | Full bandwidth |
| PMI precoding model |  |  | Random |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Proposal 4: We propose the following parameters for LTE cell:

| Parameter | Value |
| --- | --- |
| Number of interfering cell | 1 interfering cell |
| Signal level for interfering cells CRS (interference over Noc) | 5dB, 10dB, 15dB, 20dB |
| Network synchronization | All cells are synchronous |
| CRS configuration | 4 CRS ports per cell with planning |
| HARQ | 8 HARQ processes and max 4 transmissions |
| Resource allocation | Full bandwidth |
| PMI | Random PMI |

Reference

[1] RP-210920 “Revised WID: Further enhancement on NR demodulation performance”, China Telecom, RAN #91-e, March 2021.