3GPP TSG-RAN WG4 meeting #99-e	R4-2110638

Electronic Meeting, 19 May – 27 May 2021

Agenda Item:	5.1.1.2.1

Source:	Ericsson

Title:	Update of simulation results of PDSCH with multi-TRP transmission

Document for:	Discussion

# 1	Introduction

This contribution updates our simulation results of PDSCH for multi-TRP transmission scheme [1].

# 2	Simulation results

## 2.1	Multi-DCI based transmission

Table 1 is the update of our simulation results for the SNR levels to achieve 70% of maximum throughput for multi-DCI based PDSCH transmission.

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition (Note 1) | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | FDD | 10MHz / 15kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 2a: 200Hz / -0.5us | 70% of max Tput |
| 1-2 | FDD | 10MHz / 15kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 2a: 200Hz / -0.5us | 70% of max Tput |
| 2-1 | TDD | 40MHz / 30kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 2a: 300Hz / -0.25us | 70% of max Tput |
| 2-2 | TDD | 40MHz / 30kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 2a: 300Hz / -0.25us | 70% of max Tput |
| Note 1: MIMO faders are independent for each TRP transmission. |  |  |  |  |  |  |  |

Table 1	Summary of simulation results for multi-DCI based transmission (SNR to achieve 70% of maximum throughput (dB)).

|  | FDD SCS=15kHz 2Rx | FDD SCS=15kHz 4Rx | TDD SCS=30kHz 2Rx | TDD SCS=30kHz 4Rx |
| --- | --- | --- | --- | --- |
| Alignment results | 16.4 | 10.5 | 17.1 | 10.8 |
| Impairment results | 18.4 | 12.5 | 19.1 | 12.8 |

## 2.2	Single-DCI based SDM transmission

Table 2 and Table 3 are the updates of our simulation results of the SNR levels to achieve 70% of maximum throughput for the single-DCI based SDM transmission.

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | FDD | 10MHz / 15kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1a: 200Hz / -0.5us Test 1b: 0Hz / 2us | 70% of max Tput |
| 3-2 | FDD | 10MHz / 15kHz | MCS19 (64QAM 0.5) MCS able 1 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1a: 200Hz / -0.5us Test 1b: 0Hz / 2us | 70% of max Tput |
| 4-1 | TDD | 40MHz / 30kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1a: 300Hz / -0.25us Test 1b: 0Hz / 1us | 70% of max Tput |
| 4-2 | TDD | 40MHz / 30kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1a: 300Hz / -0.25us Test 1b: 0Hz / 1us | 70% of max Tput |

Table 2	Summary of simulation results for single-DCI based SDM transmission (Test 1a, SNR to achieve 70% of maximum throughput (dB)).

|  | FDD SCS=15kHz 2Rx | FDD SCS=15kHz 4Rx | TDD SCS=30kHz 2Rx | TDD SCS=30kHz 4Rx |
| --- | --- | --- | --- | --- |
|  | TO = -0.5us FO = 200Hz | TO = -0.5us FO = 200Hz | TO = -0.25us FO = 300Hz | TO = -0.25us FO = 300Hz |
| Alignment results | 18.1 | 12.1 | 19.2 | 12.1 |
| Impairment results | 20.1 | 14.1 | 21.2 | 14.1 |

Table 3	Summary of simulation results for single-DCI based SDM transmission (Test 1b, SNR to achieve 70% of maximum throughput (dB)).

|  | FDD SCS=15kHz 2Rx | FDD SCS=15kHz 4Rx | TDD SCS=30kHz 2Rx | TDD SCS=30kHz 4Rx |
| --- | --- | --- | --- | --- |
|  | TO = 2us FO = 0Hz | TO = 2us FO = 0Hz | TO = 1us FO = 0Hz | TO = 1us FO = 0Hz |
| Alignment results | 16.5 | 10.4 | 17.2 | 10.6 |
| Impairment results | 18.5 | 12.4 | 19.2 | 12.6 |

## 2.3	Single-DCI based FDM Scheme A

Table 4 is the update of the update of our simulation results of the SNR levels to achieve 70% of maximum throughput for the single-DCI based FDM Scheme A.

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5-1 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 200Hz / -0.5us | 70% of max Tput |
| 5-2 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 200Hz / -0.5us | 70% of max Tput |
| 6-1 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 300Hz / -0.25us | 70% of max Tput |
| 6-2 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 300Hz / -0.25us | 70% of max Tput |

Table 4	Summary of simulation results for single-DCI based FDM Scheme A (SNR to achieve 70% of maximum throughput (dB)).

|  | FDD SCS=15kHz 2Rx | FDD SCS=15kHz 4Rx | TDD SCS=30kHz 2Rx | TDD SCS=30kHz 4Rx |
| --- | --- | --- | --- | --- |
| Alignment results | 13.7 | 7.1 | 13.6 | 6.8 |
| Impairment results | 15.7 | 9.1 | 15.6 | 8.8 |

## 2.4	Single-DCI based Inter-slot TDM

Table 5 is the update of our simulation results of the SNR levels to achieve 1% of PDSCH BLER for the single-DCI based Inter-slot TDM. Note the results for inter-slot TDM are same as [2].

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7-1 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 200Hz / 2us | 1% BLER |
| 7-2 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 200Hz / 2us | 1% BLER |
| 8-1 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 300Hz / 1us | 1% BLER |
| 8-2 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 300Hz / 1us | 1% BLER |

Table 5	Summary of simulation results for single-DCI based inter-slot TDM (SNR to achieve 1% of PDSCH BLER (dB)).

|  | FDD SCS=15kHz 2Rx | FDD SCS=15kHz 4Rx | TDD SCS=30kHz 2Rx | TDD SCS=30kHz 4Rx |
| --- | --- | --- | --- | --- |
| Alignment results | -0.6 | -5.0 | -1.1 | -5.7 |
| Impairment results | 1.4 | -3.0 | 0.9 | -3.7 |

# 3	Summary

This contribution provided our alignment and impairment simulation results for PDSCH with multi-TRP transmission. We would like to consider these results for the final PDSCH demodulation requirements.

# References

- R4-2101449, “Simulation assumption for PDSCH requirement with mDCI/sDCI-based transmission schemes”, Ericsson.

- R4-2101452, “Simulation results of sDCI-based FDM/TDM transmission”, Ericsson.