3GPP TSG-RAN WG4 Meeting#98bis             	R4-2110702

E-meeting, 19th – 27th May, 2021

Title:	TP to TR 38.717-03-01 Addition of CA_n14A-n66A-n77A

Source:	Nokia, AT&T

Agenda item:	8.10.2

Document for:	Approval

# 1	Introduction

This contribution is a TP to TR 38.717-03-01 to introduce CA_n14A-n66A-n77A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

### 6.x	CA_n14-n66-n77

#### 6.x.1		Operating bands for CA

Table 6.x.1-1: CA band combination of CA_n14 + n66 + n77

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n14A-n66A-n77A | n14 | 778 MHz | – | 788 MHz | 758 MHz | – | 768 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 6.x.2		Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported bandwidths per CA band combination of CA_n14 + n66 + n77

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n14A-n66A-n77A | - | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.x.3  Co-existence studies

Table 6.x.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL

Table 6.x.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n14 | 778 | 788 | 1556 | 1576 | 2334 | 2364 | 3112 | 3152 | 3890 | 3940 |
| n66 | 1710 | 1780 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 | 8550 | 8900 |
| n77 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table the 5th harmonic of band n14 as well as the 2nd harmonic of band n66 falls inside band n77. Both are found in the fall back mode to this combination.

Table 6.x.3-2 gives harmonic mixing issue for CA with Band n14, n66 and n77. No harmonic mixing issue is identified for this band combination.

Table 6.x.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n14 | 778 | 788 | 758 | 768 | 1516 | 1536 | 2274 | 2304 | 3032 | 3072 |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

### 6.x.4	∆TIB,c and ∆RIB,c values

For three DLs of Band n14, n66 and n77, the same TIB,c and RIB,c values specified for CA_n5-n66-n77 are used as below.

Table 6.x.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n14-n66-n77 | n14 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n14-n66-n77 | n14 | 0.2 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |

#### 6.x.5	REFSENS requirements

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

*********************** End of the TP ***************************************************

# 3	References