3GPP TSG-RAN WG4 Meeting #99-e	R4-2110718

Online, May 19-27, 2021

Source: 	Qualcomm Incorporated

Title: 	Simulation Setup and Results for NR-U CQI Performance Tests

Agenda item:	5.1.4.3

Document for:	Discussion

# Introduction

Some aspects of the test setup for NR-U CQI Performance testing are addressed in [2], while in this contribution we will address the simulations parameters and results.

# Test Setup Details

## CSI Reporting Setup Parameters Used

For parameters not specified in the simulation assumptions in [1], the following parameters have been used, in line with CQI tests for NR licensed already included in 38.101-4, section 6.2:

- CQI table: Table 2;

- CodebookSubsetRestriction: 010000;

Proposals 1: For NR-U CQI Performance tests, use CQI Table 2.

Proposals 2: For NR-U CQI Performance tests, use CodebookSubsetRestriction 010000.

## Simulation results

The simulation results included show the median CQI delta across transmission with different power boost [0, +6] dB with:

- SNR [8,9] dB for 2 RX;

- SNR [5,6] dB for 4 RX;

| ## CBW | ## NRX | ## CINR ## Including Power boost | ## Median CQI Delta Between different Power Boost |
| --- | --- | --- | --- |
| ## 80MHz | ## 2 | ## [8dB, 14dB] | ## 3 |
|  |  | ## [9db, 15dB] | ## 2 |
|  | ## 4 | ## [5dB, 11dB] | ## 3 |
|  |  | ## [6db, 12dB] | ## 2 |

Proposal 3: For NR-U CQI Performance tests, define requirements for 2RX using SNR [8, 9] dB, with the applicability rule that satisfying the requirement for one SNR point is sufficient.

Proposal 4: For NR-U CQI Performance tests, define requirements for 4RX using SNR [5, 6] dB, with the applicability rule that satisfying the requirement for one SNR point is sufficient.

Proposal 4: For NR-U CQI Performance tests, use the applicability rule to test UEs only for the largest supported number of RX.

Proposal 4: Regarding the passing criteria for NR-U CQI Performance tests, the Median CQI delta across reporting based on different power boost should be larger than 2, for both 2RX and 4RX.

# Conclusions

Proposals 1: For NR-U CQI Performance tests, use CQI Table 2.

Proposals 2: For NR-U CQI Performance tests, use CodebookSubsetRestriction 010000.

Proposal 3: For NR-U CQI Performance tests, define requirements for SNR=[8, 9] dB, with the applicability rule that satisfying the requirement for one SNR point is sufficient.

Proposal 4: Regarding the passing criteria for NR-U CQI Performance tests, the Median CQI delta across reporting based on different power boost should be larger than 2.

# References

[1] R4-2106173, “Way Forward on NR-U UE demodulation requirements”, 3GPP TSG RAN WG4 Meeting #98-bis-e, e-Meeting, April 12 – 20, 2021;

[3] R4-2110766_Discussion on NR-U UE PDSCH and CQI Requirements, “Discussion on NR-U UE PDSCH and CQI Requirements”, 3GPP TSG RAN WG4 Meeting #99-e, Online, May 19-27, 2021