3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2110747

Electronic Meeting, 19– 27 May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0332 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.104: introduction of channel bandwidths 35MHz and 45MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_35MHz_45MHz_BW-Core |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of channel bandwidths 35MHz and 45MHz for BS TX and general part |  |  |  |  |  |  |  |  |
| Summary of change: |  | RF requirements for 35MHz and 45MHz CBW are added in relavent clauses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RF requirements on BS TX and general part for 35 MHz and 45 MHz are not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2, 5.3.3, 5.3.5, 6.3.3.2, 6.3.4.2, 6.6.3.2, 7.2.2, 7.3.2, 7.4.1, 7.7.2, 7.8.2, 10.3.2, 10.4.2, 10.5.1, 10.5.2, 10.8.2, 10.9.2, B.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.141-1, TS 38.141-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

---Start of changes for TS 38.104 ---

5.3.2	Transmission bandwidth configuration

The transmission bandwidth configuration NRB for each BS channel bandwidth and subcarrier spacing is specified in table 5.3.2.-1 for FR1 and table 5.3.2-2 for FR2.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

---Next modified section ---

5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each BS channel bandwidth and SCS is specified in table 5.3.3-1 for FR1 and in table 5.3.3-2 for FR2.

Table 5.3.3-1: Minimum guardband (kHz) (FR1)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

Table: 5.3.3-2: Minimum guardband (kHz) (FR2)

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N/A |
| 120 | 1900 | 2420 | 4900 | 9860 |

---Next modified section ---

5.3.5	BS channel bandwidth per operating band

The requirements in this specification apply to the combination of BS channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1 and in table 5.3.5-2 for FR2. The transmission bandwidth configuration in table 5.3.2-1 and table 5.3.2-2 shall be supported for each of the BS channel bandwidths within the BS capability. The BS channel bandwidths are specified for both the Tx and Rx path.

Table 5.3.5-1: BS channel bandwidths and SCS per operating band in FR1

|  |  |  | NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band |  | SCS kHz |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
| n1 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
| n2 |  | 30 |  |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n3 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n5 |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |
| n7 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
| n8 |  | 30 |  |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 |  | 30 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N13 |  | 15 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 |  | 30 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| n18 |  | 30 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n20 |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
| n24 |  | 15 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n25 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n26 |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |
| n28 |  | 30 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n29 |  | 30 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 |  | 30 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 |  | 30 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
| n38 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
| n39 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes4 | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |
| n40 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes |
|  |  | 15 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |
| n41 |  | 30 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 60 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 15 |  |  | Yes6 |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |
| n46 |  | 30 |  |  | Yes6 |  | Yes |  |  |  | Yes |  |  | Yes |  | Yes |  |  |
|  |  | 60 |  |  | Yes6 |  | Yes |  |  |  | Yes |  |  | Yes |  | Yes |  |  |
|  |  | 15 |  | Yes2 | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes1 |  |  |  |  |  |
| n48 |  | 30 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes1 | Yes1 | Yes1 | Yes1 | Yes1 | Yes1 |
|  |  | 60 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes1 | Yes1 | Yes1 | Yes1 | Yes1 | Yes1 |
|  |  | 15 |  | Yes2 | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |
| n50 |  | 30 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  |  |
|  |  | 15 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n51 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 |  | 30 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| n65 |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n66 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| n70 |  | 30 |  |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
| n71 |  | 30 |  |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n74 |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |
| n75 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |
|  |  | 15 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n76 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |
| n77 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |
| n78 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 15 |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |
| n79 |  | 30 |  |  |  |  |  |  |  |  | Yes |  | Yes | Yes |  | Yes |  | Yes |
|  |  | 60 |  |  |  |  |  |  |  |  | Yes |  | Yes | Yes |  | Yes |  | Yes |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
| n80 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n81 |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n82 |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |
| n83 |  | 30 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |
| n84 |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |  |  |  |
| n86 |  | 30 |  |  | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n89 |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |
| n90 |  | 30 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 60 |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 15 |  | Yes | Yes3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n91 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n92 |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n93 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n94 |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| n95 |  | 30 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |
| n96 |  | 30 |  |  |  |  | Yes |  |  |  | Yes |  |  | Yes |  | Yes |  |  |
|  |  | 60 |  |  |  |  | Yes |  |  |  | Yes |  |  | Yes |  | Yes |  |  |
| N97 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes |
| N98 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
| n99 |  | 15 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NOTE 1: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. NOTE 2: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 3: For this bandwidth, it only applies for UL transmission. NOTE 4: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 5: Void. NOTE 6: This bandwidth can only be applied in certain regions where the absence of non 3GPP technologies can be guaranteed on a long term basis in this version of specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-2: BS channel bandwidths and SCS per operating band in FR2

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n258 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n259 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n260 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n261 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |

---Next modified section ---

6.3.3.2	Minimum requirement for BS type 1-C and BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.3.3.2-1.

Table 6.3.3.2-1: Total power dynamic range

| BS channel bandwidth (MHz) | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.9 | 10.4 | N/A |
| 10 | 17.1 | 13.8 | 10.4 |
| 15 | 18.9 | 15.7 | 12.5 |
| 20 | 20.2 | 17 | 13.8 |
| 25 | 21.2 | 18.1 | 14.9 |
| 30 | 22 | 18.9 | 15.7 |
| 35 | 22.7 | 19.6 | 16.4 |
| 40 | 23.3 | 20.2 | 17 |
| 45 | 23.8 | 20.7 | 17.6 |
| 50 | 24.3 | 21.2 | 18.1 |
| 60 | N/A | 22 | 18.9 |
| 70 | N/A | 22.7 | 19.6 |
| 80 | N/A | 23.3 | 20.2 |
| 90 | N/A | 23.8 | 20.8 |
| 100 | N/A | 24.3 | 21.3 |

---Next modified section ---

6.3.4.2	Minimum Requirement

NB-IoT RB power dynamic range for NB-IoT operation in NR in-band shall be larger than or equal to the level specified in Table 6.3.4.2-1. This power dynamic range level is only required for one NB-IoT RB.

Table 6.3.4.2-1: NB-IoT RB power dynamic range for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 10 | Any | +6 |
| 15 | Within center 77*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 20 | Within center 102*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Within center 90% of BS channel bandwidth | +6 |
|  | Other | +3 |

---Next modified section ---

6.6.3.2	Limits and Basic limits

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

For operation in paired and unpaired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.2-1 in any operating band except for band n46 and n96.

Table 6.6.3.2-1: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 45 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 45 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 45 dB (Note 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 45 dB (Note 3) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |

For band n46 and n96, the ACLR shall be higher than the value specified in Table 6.6.3.2-1a.

Table 6.6.3.2-1a: Base station ACLR limit for band n46 and n96

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 10, 20, 40, 60, 80 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 35 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 40 dB |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

The ACLR absolute basic limit is specified in table 6.6.3.2-2.

Table 6.6.3.2-2: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

For operation in non-contiguous spectrum or multiple bands, the ACLR shall be higher than the value specified in Table 6.6.3.2-2a in any operating band except for band n46 and n96.

Table 6.6.3.2-2a: Base Station ACLR limit in non-contiguous spectrum or multiple bands

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
|  | Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
|  | Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |

For operation in non-contiguous spectrum for band n46 and n96, the ACLR shall be higher than the value specified in Table 6.6.3.2-2b.

Table 6.6.3.2-2b: Base Station ACLR limit in non-contiguous spectrum for band n46 and n96

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 10, 20, 40, 60, 80 | Wgap ≥ 60 | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 35 dB |
|  | Wgap ≥ 80 | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 40 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |  |

The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in a sub-block gap or the Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in table 6.6.3.2-3 and the filters on the assigned channels are defined in table 6.6.3.2-4.

For operation in non-contiguous spectrum or multiple bands, the CACLR for NR carriers located on either side of the sub-block gap or the Inter RF Bandwidth gap shall be higher than the value specified in table 6.6.3.2-3.

The CACLR requirements in Table 6.6.3.2-3 apply to BS that supports NR, in any operating band except for band n46 and n96. The CACLR requirements for band n46 and n96 are in Table 6.6.3.2-3aa.

Table 6.6.3.2-3: Base Station CACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | 5 ≤Wgap< 15 (Note 3) 5 ≤Wgap< 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
|  | 10 < Wgap< 20 (Note 3) 10 ≤Wgap< 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
| 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 | 20 ≤Wgap< 60 (Note 4) 20 ≤Wgap< 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
|  | 40 < Wgap< 80 (Note 4) 40 ≤Wgap< 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |

For operation in non-contiguous spectrum for band n46 and n96, the CACLR for NR carriers located on either side of the sub-block gap shall be higher than the value specified in Table 6.6.3.2-3aa.

Table 6.6.3.2-3aa: Base Station CACLR limit for band n46 and n96

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- |
| 10, 20, 40, 60, 80 | 20 ≤Wgap< 60 | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 35 dB |
|  | 40 < Wgap< 80 | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 40 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |  |

The CACLR absolute basic limit is specified in table 6.6.3.2-3a.

Table 6.6.3.2-3a: Base station CACLR absolute basic limit

| BS category / BS class | CACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Table 6.6.3.2-4: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| NR | NR of same BW with SCS that provides largest transmission bandwidth configuration |

---Next modified section ---

7.2.2	Minimum requirements for BS type 1-C and BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.2-1 for Wide Area BS, in table 7.2.2-2 for Medium Range BS and in table 7.2.2-3 for Local Area BS in any operating band except for band n46 and n96.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.2-2a for Medium Range BS and in table 7.2.2-3a for Local Area BS, for band n46.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.2-2b for Medium Range BS and in table 7.2.2-3b for Local Area BS, for band n96.

The reference sensitivity level requirements for NB-IoT are specified in TS 36.104 [13] clause 7.2.

Table 7.2.2-1: NR Wide Area BS reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -101.7 |
|  |  | G-FR1-A1-10 (Note 3) | -101.7 (Note 2) |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -101.8 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -98.9 |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 (Note 1) | -95.3 |
|  |  | G-FR1-A1-11 (Note 4) | -95.3 (Note 2) |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 (Note 1) | -95.6 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 (Note 1) | -95.7 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The requirements apply to BS that supports NB-IoT operation in NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. NOTE 5: Void. |  |  |  |

Table 7.2.2-2: NR Medium Range BS reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (NOTE 5) | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -96.7 |
|  |  | G-FR1-A1-10 (Note 3) | -96.7 (Note 2) |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -96.8 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -93.9 |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 (Note 1) | -90.3 |
|  |  | G-FR1-A1-11 (Note 4) | -90.3 (Note 2) |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 (Note 1) | -90.6 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 (Note 1) | -90.7 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The requirements apply to BS that supports NB-IoT operation in NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. NOTE 5: These reference measurement channels are not applied for band n46 and n96. |  |  |  |

Table 7.2.2-2a: NR Medium Range BS reference sensitivity levels for band n46

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 10 | 15 | G-FR1-A1-12 (NOTE 2) | -103.0 |
|  | 30 | G-FR1-A1-13 (NOTE 2) | -100.7 |
|  | 60 | G-FR1-A1-3 (NOTE 1) | -93.9 |
| 20 | 15 | G-FR1-A1-14 (NOTE 2) | -100.1 |
|  | 30 | G-FR1-A1-15 (NOTE 2) | -97.1 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -90.7 |
| 40 | 15 | G-FR1-A1-16 (NOTE 2) | -97.0 |
|  | 30 | G-FR1-A1-17 (NOTE 2) | -94.0 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -90.7 |
| 60 | 30 | G-FR1-A1-18 (NOTE 2) | -92.4 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -90.7 |
| 80 | 30 | G-FR1-A1-19 (NOTE 2) | -91.1 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -90.7 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each single interlace of FRC G-FR1-A1-12 and G-FR1-A1-19, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 7.2.2-2b: NR Medium Range BS reference sensitivity levels for band n96

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 20 | 15 | G-FR1-A1-14 (NOTE 2) | -99.1 |
|  | 30 | G-FR1-A1-15 (NOTE 2) | -96.1 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -89.7 |
| 40 | 15 | G-FR1-A1-16 (NOTE 2) | -96.0 |
|  | 30 | G-FR1-A1-17 (NOTE 2) | -93.0 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -89.7 |
| 60 | 30 | G-FR1-A1-18 (NOTE 2) | -91.4 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -89.7 |
| 80 | 30 | G-FR1-A1-19 (NOTE 2) | -90.1 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -89.7 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each single interlace of FRC G-FR1-A1-14 and G-FR1-A1-19, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 7.2.2-3: NR Local Area BS reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (NOTE 5) | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -93.7 |
|  |  | G-FR1-A1-10 (Note 3) | -93.7 (Note 2) |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -93.8 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -90.9 |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 (Note 1) | -87.3 |
|  |  | G-FR1-A1-11 (Note 4) | -87.3 (Note 2) |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 (Note 1) | -87.6 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 (Note 1) | -87.7 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The requirements apply to BS that supports NB-IoT operation in NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. NOTE 5: These reference measurement channels are not applied for band n46 and n96. |  |  |  |

Table 7.2.2-3a: NR Local Area BS reference sensitivity levels for band n46

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 10 | 15 | G-FR1-A1-12 (NOTE 2) | -100.0 |
|  | 30 | G-FR1-A1-13 (NOTE 2) | -97.7 |
|  | 60 | G-FR1-A1-3 (NOTE 1) | -90.9 |
| 20 | 15 | G-FR1-A1-14 (NOTE 2) | -97.1 |
|  | 30 | G-FR1-A1-15 (NOTE 2) | -94.1 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -87.7 |
| 40 | 15 | G-FR1-A1-16 (NOTE 2) | -94.0 |
|  | 30 | G-FR1-A1-17 (NOTE 2) | -91.0 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -87.7 |
| 60 | 30 | G-FR1-A1-18 (NOTE 2) | -89.4 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -87.7 |
| 80 | 30 | G-FR1-A1-19 (NOTE 2) | -88.1 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -87.7 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each single interlace of FRC G-FR1-A1-12 and G-FR1-A1-19, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 7.2.2-3b: NR Local Area BS reference sensitivity levels for band n96

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 20 | 15 | G-FR1-A1-14 (NOTE 2) | -96.1 |
|  | 30 | G-FR1-A1-15 (NOTE 2) | -93.1 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -86.7 |
| 40 | 15 | G-FR1-A1-16 (NOTE 2) | -93.0 |
|  | 30 | G-FR1-A1-17 (NOTE 2) | -90.0 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -86.7 |
| 60 | 30 | G-FR1-A1-18 (NOTE 2) | -88.4 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -86.7 |
| 80 | 30 | G-FR1-A1-19 (NOTE 2) | -87.1 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -86.7 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each single interlace of FRC G-FR1-A1-12 and G-FR1-A1-19, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

---Next modified section ---

7.3.2	Minimum requirement for BS type 1-C and BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in table 7.3.2-1 for Wide Area BS, in table 7.3.2-2 for Medium Range BS and in table 7.3.2-3 for Local Area BS.

For NB-IoT operation in NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A of TS 36.104 [13] with parameters specified in table 7.3.2-1a for Wide Area BS, in table 7.3.2-2a for Medium Range BS and in table 7.3.2-3a for Local Area BS.

Table 7.3.2-1: Wide Area BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -70.7 | -82.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
| 10 | 15 | G-FR1-A2-1 | -70.7 | -79.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.7 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |
| 20 | 15 | G-FR1-A2-4 | -64.5 | -76.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5 |  |  |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 25 | 15 | G-FR1-A2-4 | -64.5 | -75.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5 |  |  |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 30 | 15 | G-FR1-A2-4 | -64.5 | -74.4 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5 |  |  |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 35 | 15 | G-FR1-A2-4 | -64.5 | -73.7 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5 |  |  |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 40 | 15 | G-FR1-A2-4 | -64.5 | -73.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5 |  |  |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 45 | 15 | G-FR1-A2-4 | -64.5 | -72.6 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5 |  |  |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 50 | 15 | G-FR1-A2-4 | -64.5 | -72.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5 |  |  |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 60 | 30 | G-FR1-A2-5 | -64.5 | -71.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 70 | 30 | G-FR1-A2-5 | -64.5 | -70.7 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 80 | 30 | G-FR1-A2-5 | -64.5 | -70.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 90 | 30 | G-FR1-A2-5 | -64.5 | -69.5 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| 100 | 30 | G-FR1-A2-5 | -64.5 | -69.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.8 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.2-1a: Wide Area BS dynamic range for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 |  |  | -82.5 |  |
| 10 |  |  | -79.3 |  |
| 15 |  |  | -77.5 |  |
| 20 | FRC A15-1 in |  | -76.2 |  |
| 25 | Annex A.15 in | -99.7 | -75.2 | AWGN |
| 30 | TS 36.104 [13] |  | -74.4 |  |
| 35 |  |  | -73.7 |  |
| 40 |  |  | -73.1 |  |
| 45 |  |  | -72.6 |  |
| 50 |  |  | -72.1 |  |
| 5 |  |  | -82.5 |  |
| 10 |  |  | -79.3 |  |
| 15 |  |  | -77.5 |  |
| 20 | FRC A15-2 in |  | -76.2 |  |
| 25 | Annex A.15 in | -105.6 | -75.2 | AWGN |
| 30 | TS 36.104 [13] |  | -74.4 |  |
| 35 |  |  | -73.7 |  |
| 40 |  |  | -73.1 |  |
| 45 |  |  | -72.6 |  |
| 50 |  |  | -72.1 |  |

Table 7.3.2-2: Medium Range BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -65.7 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4 |  |  |
| 10 | 15 | G-FR1-A2-1 | -65.7 | -74.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4 |  |  |
|  | 60 | G-FR1-A2-3 | -63.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.7 | -72.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4 |  |  |
|  | 60 | G-FR1-A2-3 | -63.4 |  |  |
| 20 | 15 | G-FR1-A2-4 | -59.5 | -71.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5 |  |  |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 25 | 15 | G-FR1-A2-4 | -59.5 | -70.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5 |  |  |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 30 | 15 | G-FR1-A2-4 | -59.5 | -69.4 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5 |  |  |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 35 | 15 | G-FR1-A2-4 | -59.5 | -68.7 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5 |  |  |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 40 | 15 | G-FR1-A2-4 | -59.5 | -68.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5 |  |  |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 45 | 15 | G-FR1-A2-4 | -59.5 | -67.6 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5 |  |  |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 50 | 15 | G-FR1-A2-4 | -59.5 | -67.1 | AWGN |
|  | 30 | G-FR1-A2-5 | --59.5 |  |  |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 60 | 30 | G-FR1-A2-5 | -59.5 | -66.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 70 | 30 | G-FR1-A2-5 | -59.5 | -65.7 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 80 | 30 | G-FR1-A2-5 | -59.5 | -65.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 90 | 30 | G-FR1-A2-5 | -59.5 | -64.5 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 100 | 30 | G-FR1-A2-5 | -59.5 | -64.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| NOTE 1: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: These reference measurement channels are not applied for band n46 and n96. |  |  |  |  |  |

Table 7.3.2-2a: Medium Range BS dynamic range for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 |  |  | -77.5 |  |
| 10 |  |  | -74.3 |  |
| 15 | FRC A15-1 in |  | -72.5 |  |
| 20 | Annex A.15 in | -94.7 | -71.2 | AWGN |
| 25 | TS 36.104 [13] |  | -70.2 |  |
| 30 |  |  | -69.4 |  |
| 35 |  |  | -68.7 |  |
| 40 |  |  | -68.1 |  |
| 45 |  |  | -67.6 |  |
| 50 |  |  | -67.1 |  |
| 5 |  |  | -77.5 |  |
| 10 |  |  | -74.3 |  |
| 15 | FRC A15-2 in |  | -72.5 |  |
| 20 | Annex A.15 in | -100.6 | -71.2 | AWGN |
| 25 | TS 36.104 [13] |  | -70.2 |  |
| 30 |  |  | -69.4 |  |
| 35 |  |  | -68.7 |  |
| 40 |  |  | -68.1 |  |
| 45 |  |  | -67.6 |  |
| 50 |  |  | -67.1 |  |

Table 7.3.2-2b: Medium Range BS dynamic range for band n46

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 10 | 15 | G-FR1-A2-7 | -72.8 | -74.3 | AWGN |
|  | 30 | G-FR1-A2-8 | -70.6 |  |  |
|  | 60 | G-FR1-A2-3 | -63.4 |  |  |
| 20 | 15 | G-FR1-A2-9 | -69.8 | -71.2 | AWGN |
|  | 30 | G-FR1-A2-10 | -66.8 |  |  |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 40 | 15 | G-FR1-A2-11 | -66.7 | -68.1 | AWGN |
|  | 30 | G-FR1-A2-12 | -63.7 |  |  |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 60 | 30 | G-FR1-A2-13 | -61.9 | -66.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| 80 | 30 | G-FR1-A2-14 | -60.7 | -65.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.2-2c: Medium Range BS dynamic range for band n96

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20 | 15 | G-FR1-A2-9 | -68.8 | -70.2 | AWGN |
|  | 30 | G-FR1-A2-10 | -65.8 |  |  |
|  | 60 | G-FR1-A2-6 | -58.8 |  |  |
| 40 | 15 | G-FR1-A2-11 | -65.7 | -67.1 | AWGN |
|  | 30 | G-FR1-A2-12 | -62.7 |  |  |
|  | 60 | G-FR1-A2-6 | -58.8 |  |  |
| 60 | 30 | G-FR1-A2-13 | -60.9 | -65.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -58.8 |  |  |
| 80 | 30 | G-FR1-A2-14 | -59.7 | -64.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -58.8 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.2-3: Local Area BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -62.7 | -74.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.4 |  |  |
| 10 | 15 | G-FR1-A2-1 | -62.7 | -71.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.4 |  |  |
|  | 60 | G-FR1-A2-3 | -60.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.7 | -69.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.4 |  |  |
|  | 60 | G-FR1-A2-3 | -60.4 |  |  |
| 20 | 15 | G-FR1-A2-4 | -56.5 | -68.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5 |  |  |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 25 | 15 | G-FR1-A2-4 | -56.5 | -67.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5 |  |  |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 30 | 15 | G-FR1-A2-4 | -56.5 | -66.4 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5 |  |  |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 35 | 15 | G-FR1-A2-4 | -56.5 | -65.7 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5 |  |  |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 40 | 15 | G-FR1-A2-4 | -56.5 | -65.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5 |  |  |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 45 | 15 | G-FR1-A2-4 | -56.5 | -64.6 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5 |  |  |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 50 | 15 | G-FR1-A2-4 | -56.5 | -64.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5 |  |  |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 60 | 30 | G-FR1-A2-5 | -56.5 | -63.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 70 | 30 | G-FR1-A2-5 | -56.5 | -62.7 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 80 | 30 | G-FR1-A2-5 | -56.5 | -62.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 90 | 30 | G-FR1-A2-5 | -56.5 | -61.5 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 100 | 30 | G-FR1-A2-5 | -56.5 | -61.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| NOTE 1: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: These reference measurement channels are not applied for band n46 and n96. |  |  |  |  |  |

Table 7.3.2-3a: Local Area BS dynamic range for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 |  |  | -74.5 |  |
| 10 |  |  | -71.3 |  |
| 15 | FRC A15-1 in |  | -69.5 |  |
| 20 | Annex A.15 in | -91.7 | -68.2 | AWGN |
| 25 | TS 36.104 [13] |  | -67.2 |  |
| 30 |  |  | -66.4 |  |
| 35 |  |  | -65.7 |  |
| 40 |  |  | -65.1 |  |
| 45 |  |  | -64.6 |  |
| 50 |  |  | -64.1 |  |
| 5 |  |  | -74.5 |  |
| 10 |  |  | -71.3 |  |
| 15 | FRC A15-2 in |  | -69.5 |  |
| 20 | Annex A.15 in | -97.6 | -68.2 | AWGN |
| 25 | TS 36.104 [13] |  | -67.2 |  |
| 30 |  |  | -66.4 |  |
| 35 |  |  | -65.7 |  |
| 40 |  |  | -65.1 |  |
| 45 |  |  | -64.6 |  |
| 50 |  |  | -64.1 |  |

Table 7.3.2-3b: Local Area BS dynamic range for band n46

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 10 | 15 | G-FR1-A2-7 | -69.8 | -71.3 | AWGN |
|  | 30 | G-FR1-A2-8 | -67.6 |  |  |
|  | 60 | G-FR1-A2-3 | -60.4 |  |  |
| 20 | 15 | G-FR1-A2-9 | -66.8 | -68.2 | AWGN |
|  | 30 | G-FR1-A2-19 | -63.8 |  |  |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 40 | 15 | G-FR1-A2-11 | -63.7 | -65.1 | AWGN |
|  | 30 | G-FR1-A2-12 | -60.7 |  |  |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 60 | 30 | G-FR1-A2-13 | -58.9 | -63.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| 80 | 30 | G-FR1-A2-14 | -57.7 | -62.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each. |  |  |  |  |  |

Table 7.3.2-3c: Local area BS dynamic range for band n96

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20 | 15 | G-FR1-A2-9 | -65.8 | -67.2 | AWGN |
|  | 30 | G-FR1-A2-19 | -62.8 |  |  |
| 40 | 15 | G-FR1-A2-11 | -62.7 | -64.1 | AWGN |
|  | 30 | G-FR1-A2-12 | -59.7 |  |  |
| 60 | 30 | G-FR1-A2-13 | -57.9 | -62.3 | AWGN |
| 80 | 30 | G-FR1-A2-14 | -56.7 | -61.1 | AWGN |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each. |  |  |  |  |  |

---Next modified section ---

7.4.1	Adjacent Channel Selectivity (ACS)

7.4.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

7.4.1.2	Minimum requirement for BS type 1-C and BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For BS, the wanted and the interfering signal coupled to the BS type 1-C antenna connector or BS type 1-H TAB connector are specified in table 7.4.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

For BS operating in band n46 and n96, the wanted and the interfering signal coupled to the BS type 1-C antenna connector or BS type 1-H TAB connector are specified in table 7.4.1.2-1a and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2a for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-2a and 7.2.2-3a for each BS channel bandwidth and further specified in annex A.1a. The characteristics of the interfering signal is further specified in annex D.

For BS supporting NB-IoT operation in NR in-band, the wanted and the interfering signal coupled to the BS type 1-C antenna connector are specified in table 7.4.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2 for ACS. The reference measurement channel for the NB-IoT wanted signal is identified in clause 7.2.1 of TS 36.104 [13]. The characteristics of the interfering signal is further specified in annex D.

The ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

For a BS operating in non-contiguous spectrum within any operating band, the ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.4.1.2-2. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the ACS requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 7.4.1.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C and at the TAB connector for BS type 1-H.

Table 7.4.1.2-1: Base station ACS requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area BS: -52 Medium Range BS: -47 Local Area BS: -44 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2, 7.2.2-3. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5, 7.2.1-5a and 7.2.1-5c of TS 36.104 [13]. |  |  |

Table 7.4.1.2-1a: Base station ACS requirement for band n46 and n96

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 10, 20, 40, 60, 80 (Note 1) | PREFSENS + 6 dB | Medium Range BS: -47 Local Area BS: -44 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-2a, 7.2.2-3a. NOTE 3: Void. |  |  |

Table 7.4.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 |  |
| 10 | ±2.5075 | 5 MHz DFT-s-OFDM NR signal |
| 15 | ±2.5125 | 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 |  |
| 30 | ±9.4725 |  |
| 35 | ±9.4625 |  |
| 40 | ±9.4675 |  |
| 45 | ±9.4725 |  |
| 50 | ±9.4625 | 20 MHz DFT-s-OFDM NR signal |
| 60 | ±9.4725 | 15 kHz SCS, 100 RBs |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

Table 7.4.1.2-2: Base Station ACS interferer frequency offset values for band n46 and n96

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 10 | ±9.4675 |  |
| 20 | ±9.4625 | 20 MHz DFT-s-OFDM NR signal |
| 40 | ±9.4675 | 15 kHz SCS, 100 RBs |
| 60 | ±9.4725 |  |
| 80 | ±9.4625 |  |

---Next modified section ---

7.4.2.2	Minimum requirement for BS type 1-C and BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-C antenna connector or BS type 1-H TAB connector using the parameters in tables 7.4.2.2-1, 7.4.2.2-2 and 7.4.2.2-3 for general blocking and narrowband blocking requirements. Narrowband blocking requirements are not applied for band n46 and n96. The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

For NB-IoT operation in NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-C antenna connector using the parameters in tables 7.4.2.2-1, 7.4.2.2-2a and 7.4.2.2-3 for general blocking and narrowband blocking requirements. The reference measurement channel for the NB-IoT wanted signal is identified in clause 7.2.1 of TS 36.104 [13]. The characteristics of the interfering signal is further specified in annex D.

The in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

The in-band blocking requirement shall apply from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the FDD operating band. The ΔfOOB for BS type 1-C and BS type 1-H is defined in table 7.4.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C and at the TAB connector for BS type 1-H.

Table 7.4.2.2-0: ΔfOOB offset for NR operating bands

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
|  | FUL,high – FUL,low ≤ 200 MHz | 20 |
| BS type 1-C | 200 MHz < FUL,high – FUL,low ≤ 900 MHz | 60 |
|  | FUL,high – FUL,low < 100 MHz | 20 |
| BS type 1-H | 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz | 60 |

For band n46 and n96, ΔfOOB for BS type 1-C and BS type 1-H is defined in table 7.4.2.2-0a.

Table 7.4.2.2-0a: ΔfOOB offset for NR operating bands for band n46 and n96

| Operating band | ΔfOOB (MHz) |
| --- | --- |
| n46 | 60 |
| n96 | 70 |

For a BS operating in non-contiguous spectrum within any operating band, the in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in tables 7.4.2.2-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 7.4.2.2-1.

For a BS operating in non-contiguous spectrum within any operating band, the narrowband blocking requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the channel bandwidth of the NR interfering signal in Table 7.4.2.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the narrowband blocking requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in Table 7.4.2.2-3. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 7.4.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | PREFSENS + 6 dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±30 | 20 MHz DFT-s-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5, 7.2.1-5a and 7.2.1-5c of TS 36.104 [13]. |  |  |  |  |

Table 7.4.2.2-1a: Base station general blocking requirement for n46

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 10, 20, 40, 60, 80 | PREFSENS + 6 dB | Medium Range BS: -38 Local Area BS: -35 | ±30 | 20 MHz DFT-s-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-2a and 7.2.2-3a. |  |  |  |  |

Table 7.4.2.2-1b: Base station general blocking requirement for n96

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 40, 60, 80 | PREFSENS + 6 dB | Medium Range BS: -38 Local Area BS: -35 | ±30 | 20 MHz DFT-s-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE 1: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-2a and 7.2.2-3a. |  |  |  |  |

Table 7.4.2.2-2: Base Station narrowband blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area BS: -49 Medium Range BS: -44 Local Area BS: -41 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that BS channel bandwidth NOTE 2: PREFSENS depends on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. NOTE 3: 7.5 kHz shift is not applied to the wanted signal. |  |  |

Table 7.4.2.2-2a: Base Station narrowband blocking requirement for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | PREFSENS + x dB (Note 2) | Wide Area: -49 Medium Range: -44 Local Area: -41 |
| NOTE 1: PREFSENS depends on the sub-carrier spacing as specified in tables 7.2.1-5, 7.2.1-5a and 7.2.1-5c of TS 36.104 [13]. NOTE 2: “x” is equal to 8 in case of 5 MHz channel bandwidth and equal to 6 otherwise. |  |  |

Table 7.4.2.2-3: Base Station narrowband blocking interferer frequency offsets

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 10 | ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | ±(360+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 30 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 35 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 40 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 45 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 50 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 60 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 70 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 80 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 90 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 100 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

---Next modified section ---

7.7.2	Minimum requirement for BS type 1-C and BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals coupled to the BS type 1-C antenna connector or BS type 1-H TAB connector, with the conditions specified in tables 7.7.2-1 and 7.7.2-2 for intermodulation performance and in tables 7.7.2-3, and 7.7.2-4 for narrowband intermodulation performance. Narrowband intermodulation requirements are not applied for band n46 and n96. The reference measurement channel for the wanted signal is identified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

For NB-IoT operation in NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals coupled to the BS type 1-C antenna connector, with the conditions specified in tables 7.7.2-1 and 7.7.2-2 for intermodulation performance and in tables 7.7.2-3, and 7.7.2-4 for narrowband intermodulation performance. The reference measurement channel for the NB-IoT wanted signal is identified in clause 7.2.1 of TS 36.104 [13]. The characteristics of the interfering signal is further specified in annex D.

The subcarrier spacing for the modulated interfering signal shall in general be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60 kHz and BS channel bandwidth <=20MHz, for which the subcarrier spacing of the interfering signal shall be 30 kHz.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For a BS operating in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement shall apply in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the NR interfering signal in table 7.7.2-2 or 7.7.2-4. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the NR interfering signal centre frequency offset from the Base Station RF Bandwidth edge.

For a multi-band connector, the narrowband intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the NR interfering signal in tables 7.7.2-2 and 7.7.2-4. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 7.7.2-1: General intermodulation requirement

| Base Station Type | Wanted Signal mean power (dBm) | Mean power of interfering signals (dBm) | Type of interfering signals |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS +6 dB | -52 |  |
| Medium Range BS | PREFSENS +6 dB | -47 | See Table 7.7.2-2 |
| Local Area BS | PREFSENS +6 dB | -44 |  |
| NOTE 1: PREFSENS depends on the RAT and the BS class. For NR, PREFSENS depends also on the BS channel bandwidth, see clause 7.2. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5, 7.2.1-5a and 7.2.1-5c of TS 36.104 [13]. |  |  |  |

Table 7.7.2-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 10 | ±7.465 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 15 | ±7.43 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 20 | ±7.395 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 25 | ±7.465 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 30 | ±7.43 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 35 | ±7.44 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 40 | ±7.45 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 45 | ±7.37 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 50 | ±7.35 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 60 | ±7.49 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 70 | ±7.42 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 80 | ±7.44 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 90 | ±7.46 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 100 | ±7.48 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| NOTE 1: Number of RBs is 25 for 15 kHz subcarrier spacing and 10 for 30 kHz subcarrier spacing. NOTE 2: Number of RBs is 100 for 15 kHz subcarrier spacing, 50 for 30 kHz subcarrier spacing and 24 for 60 kHz subcarrier spacing. NOTE 3: The RBs shall be placed adjacent to the transmission bandwidth configuration edge which is closer to the Base Station RF Bandwidth edge. |  |  |

Table 7.7.2-2a: Interfering signals for intermodulation requirement for n46 and n96

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 2) |
| --- | --- | --- |
| 10 | ±7.57 | CW (Note 3) |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 1, 3) |
| 20 | ±7.50 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 1) |
| 40 | ±7.45 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 1) |
| 60 | ±7.49 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 1) |
| 80 | ±7.44 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 1) |
| NOTE 1: Number of RBs is 100 for 15 kHz subcarrier spacing and 50 for 30 kHz subcarrier spacing. NOTE 2: The RBs shall be placed adjacent to the transmission bandwidth configuration edge which is closer to the Base Station RF Bandwidth edge. NOTE 3: This type of interfering signal is only applied for band n46. |  |  |

Table 7.7.2-3: Narrowband intermodulation performance requirement in FR1

| BS type | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signals |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS + 6dB (Note 1) | -52 |  |
| Medium Range BS | PREFSENS + 6dB (Note 2) | -47 | See Table 7.7.2-4 |
| Local Area BS | PREFSENS + 6dB (Note 3) | -44 |  |
| NOTE 1: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in table 7.2.2-1. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5 of TS 36.104 [13]. NOTE 2: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in table 7.2.2-2. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5c of TS 36.104 [13]. NOTE 3: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in table 7.2.2-3. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5a of TS 36.104 [13]. NOTE 4: For NB-IoT, the requirement shall apply only for a FRC A1-3 of TS 36.104 [13] mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 5: For NB-IoT, the frequency offset shall be adjusted to accommodate the IMD product to fall in the NB-IoT RB for NB-IoT operation in NR in-band. NOTE 6: For NB-IoT, if a BS RF receiver fails the test of the requirement, the test shall be performed with the CW interfering signal frequency shifted away from the wanted signal by 180 kHz and the NR interfering signal frequency shifted away from the wanted signal by 360 kHz. If the BS RF receiver still fails the test after the frequency shift, then the BS RF receiver shall be deemed to fail the requirement. |  |  |  |

Table 7.7.2-4: Interfering signals for narrowband intermodulation requirement in FR1

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±360 | CW |
|  | ±1420 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 10 | ±370 | CW |
|  | ±1960 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 15 (Note 2) | ±380 | CW |
|  | ±1960 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 20 (Note 2) | ±390 | CW |
|  | ±2320 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 25 (Note 2) | ±325 | CW |
|  | ±2350 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 30 (Note 2) | ±335 | CW |
|  | ±2350 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 35 (Note 2) | ±345 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 40 (Note 2) | ±355 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 45 (Note 2) | ±365 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 50 (Note 2) | ±375 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 60 (Note 2) | ±395 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 70 (Note 2) | ±415 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 80 (Note 2) | ±435 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 90 (Note 2) | ±365 | CW |
|  | ±2530 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 100 (Note 2) | ±385 | CW |
|  | ±2530 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the BS channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 3: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

---Next modified section ---

7.8.2	Minimum requirement for BS type 1-C and BS type 1-H

For BS type 1-C and BS type 1-H, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.8.2-1 for Wide Area BS, in table 7.8.2-2 for Medium Range BS, in table 7.8.2-2b for Medium Range BS for band n46, in table 7.8.2-2c for Medium Range BS for band n96, in table 7.8.2-3 for Local Area BS, in table 7.8.2-3b for Local Area BS for band n46, and in table 7.8.2-3c for Local Area BS for band n96. The characteristics of the interfering signal is further specified in annex D.

For NB-IoT operation in NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the NB-IoT reference measurement channel as specified in Annex A of TS 36.104 [13] with parameters specified in table 7.8.2-1a for Wide Area BS, in table 7.8.2-2a for Medium Range BS and in table 7.8.2-3a for Local Area BS.

Table 7.8.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -92.3 | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -101.3 | -81.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -98.8 | -78.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -92.6 | -71.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -98.2 | -78.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -92.7 | -71.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 7.8.2-1a: Wide Area BS in-channel selectivity for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 |  |  | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | FRC A14-1 in Annex A.14 in TS 36.104 [13] | -124.3 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 |  |  | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 |  |  | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | FRC A14-2 in Annex A.14 in TS 36.104 [13] | -130.2 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 |  |  | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| NOTE: Interfering signal is placed in one side of the Fc, while the NB-IoT PRB is placed on the other side. Both interfering signal and NB-IoT PRB are placed at the middle of the available PRB locations. The wanted NB-IoT tone is placed at the centre of this NB-IoT PRB. |  |  |  |  |

Table 7.8.2-2: Medium Range BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -95.6 | -76.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -93.7 | -72.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -87.3 | -66.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -96.3 | -76.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -93.8 | -73.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -87.6 | -66.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -93.2 | -73.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -87.7 | -66.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 7.8.2-2a: Medium Range BS in-channel selectivity for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 |  |  | -76.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | FRC A14-1 in Annex A.14 in TS 36.104 [13] | -119.3 | -72.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 |  |  | -66.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 |  |  | -76.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | FRC A14-2 in Annex A.14 in TS 36.104 [13] | -125.2 | -72.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 |  |  | -66.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| NOTE: Interfering signal is placed in one side of the Fc, while the NB-IoT PRB is placed on the other side. Both interfering signal and NB-IoT PRB are placed at the middle of the available PRB locations. The wanted NB-IoT tone is placed at the centre of this NB-IoT PRB. |  |  |  |  |

Table 7.8.2-2b: Medium Range BS in-channel selectivity for band n46

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 10 | 15 | G-FR1-A1-12 | -100.0 | -79.5 | CP-OFDM NR signal, 15 kHz SCS, 10 RBs |
|  | 30 | G-FR1-A1-13 | -97.7 | -77.4 | CP-OFDM NR signal, 30 kHz SCS, 10 RBs |
|  | 60 | G-FR1-A1-9 | -93.2 | -73.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 20 | 15 | G-FR1-A1-14 | -97.1 | -76.4 | CP-OFDM NR signal, 15 kHz SCS, 10 RBs |
|  | 30 | G-FR1-A1-15 | -94.1 | -73.4 | CP-OFDM NR signal, 30 kHz SCS, 10 RBs |
|  | 60 | G-FR1-A1-9 | -93.2 | -73.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40 | 15 | G-FR1-A1-16 | -94.0 | -73.2 | CP-OFDM NR signal, 15 kHz SCS, 20 RBs |
|  | 30 | G-FR1-A1-17 | -91.0 | -70.2 | CP-OFDM NR signal, 30 kHz SCS, 10 RBs |
|  | 60 | G-FR1-A1-6 | -87.7 | -66.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| 60 | 30 | G-FR1-A1-18 | -89.4 | -68.4 | CP-OFDM NR signal, 30 kHz SCS, 20 RBs |
|  | 60 | G-FR1-A1-6 | -87.7 | -66.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| 80 | 30 | G-FR1-A1-19 | -88.1 | -67.1 | CP-OFDM NR signal, 30 kHz SCS, 20 RBs |
|  | 60 | G-FR1-A1-6 | -87.7 | -66.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 7.8.2-2c: Medium Range BS in-channel selectivity for band n96

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20 | 15 | G-FR1-A1-14 | -96.1 | -75.4 | CP-OFDM NR signal, 15 kHz SCS, 10 RBs |
|  | 30 | G-FR1-A1-15 | -93.1 | -72.4 | CP-OFDM NR signal, 30 kHz SCS, 10 RBs |
|  | 60 | G-FR1-A1-9 | -93.2 | -73.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40 | 15 | G-FR1-A1-16 | -93.0 | -72.2 | CP-OFDM NR signal, 15 kHz SCS, 20 RBs |
|  | 30 | G-FR1-A1-17 | -90.0 | -69.2 | CP-OFDM NR signal, 30 kHz SCS, 10 RBs |
|  | 60 | G-FR1-A1-6 | -87.7 | -66.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| 60 | 30 | G-FR1-A1-18 | -88.4 | -67.4 | CP-OFDM NR signal, 30 kHz SCS, 20 RBs |
|  | 60 | G-FR1-A1-6 | -87.7 | -66.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| 80 | 30 | G-FR1-A1-19 | -87.1 | -66.1 | CP-OFDM NR signal, 30 kHz SCS, 20 RBs |
|  | 60 | G-FR1-A1-6 | -87.7 | -66.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 7.8.2-3: Local area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -92.6 | -73.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -90.7 | -69.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -84.3 | -63.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -93.3 | -73.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -90.8 | -70.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -84.6 | -63.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -90.2 | -70.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -84.7 | -63.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 7.8.2-3a: Local Area BS in-channel selectivity for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 |  |  | -73.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | FRC A14-1 in Annex A.14 in TS 36.104 [13] | -116.3 | -69.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 |  |  | -63.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 |  |  | -73.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | FRC A14-2 in Annex A.14 in TS 36.104 [13] | -122.2 | -69.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 |  |  | -63.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| NOTE: Interfering signal is placed in one side of the Fc, while the NB-IoT PRB is placed on the other side. Both interfering signal and NB-IoT PRB are placed at the middle of the available PRB locations. The wanted NB-IoT tone is placed at the centre of this NB-IoT PRB. |  |  |  |  |

Table 7.8.2-3b: Local Area BS in-channel selectivity for band n46

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 10 | 15 | G-FR1-A1-12 | -97.0 | -76.5 | CP-OFDM NR signal, 15 kHz SCS, 10 RBs |
|  | 30 | G-FR1-A1-13 | -94.7 | -74.4 | CP-OFDM NR signal, 30 kHz SCS, 10 RBs |
|  | 60 | G-FR1-A1-9 | -90.2 | -70.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 20 | 15 | G-FR1-A1-14 | -94.1 | -73.4 | CP-OFDM NR signal, 15 kHz SCS, 10 RBs |
|  | 30 | G-FR1-A1-15 | -91.1 | -70.4 | CP-OFDM NR signal, 30 kHz SCS, 10 RBs |
|  | 60 | G-FR1-A1-9 | -90.2 | -70.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40 | 15 | G-FR1-A1-16 | -91.0 | -70.2 | CP-OFDM NR signal, 15 kHz SCS, 20 RBs |
|  | 30 | G-FR1-A1-17 | -88.0 | -67.2 | CP-OFDM NR signal, 30 kHz SCS, 10 RBs |
|  | 60 | G-FR1-A1-6 | -84.7 | -63.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| 60 | 30 | G-FR1-A1-18 | -86.4 | -65.4 | CP-OFDM NR signal, 30 kHz SCS, 20 RBs |
|  | 60 | G-FR1-A1-6 | -84.7 | -63.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| 80 | 30 | G-FR1-A1-19 | -85.1 | -64.1 | CP-OFDM NR signal, 30 kHz SCS, 20 RBs |
|  | 60 | G-FR1-A1-6 | -84.7 | -63.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 7.8.2-3c: Local Area BS in-channel selectivity for band n96

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20 | 15 | G-FR1-A1-14 | -93.1 | [-73.4] | CP-OFDM NR signal, 15 kHz SCS, 10 RBs |
|  | 30 | G-FR1-A1-15 | -90.1 | [-70.4] | CP-OFDM NR signal, 30 kHz SCS, 10 RBs |
|  | 60 | G-FR1-A1-9 | -90.2 | -70.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40 | 15 | G-FR1-A1-16 | -90.0 | [-70.2] | CP-OFDM NR signal, 15 kHz SCS, 20 RBs |
|  | 30 | G-FR1-A1-17 | -87.0 | [-67.2] | CP-OFDM NR signal, 30 kHz SCS, 10 RBs |
|  | 60 | G-FR1-A1-6 | -84.7 | -63.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| 60 | 30 | G-FR1-A1-18 | -85.4 | [-65.4] | CP-OFDM NR signal, 30 kHz SCS, 20 RBs |
|  | 60 | G-FR1-A1-6 | -84.7 | -63.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| 80 | 30 | G-FR1-A1-19 | -84.1 | [-64.1] | CP-OFDM NR signal, 30 kHz SCS, 20 RBs |
|  | 60 | G-FR1-A1-6 | -84.7 | -63.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

---Next modified section ---

10.3.2	Minimum requirement for BS type 1-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-1: Wide Area BS reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -101.7 - ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -101.8 - ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -98.9 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -95.3 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -95.6 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -95.7 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 10.3.2-2: Medium Range BS reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -96.7 - ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -96.8 - ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -93.9 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -90.3 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -90.6 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -90.7 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 10.3.2-3: Local Area BS reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -93.7 - ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -93.8 - ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -90.9 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -87.3 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -87.6 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -87.7 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

---Next modified section ---

10.4.2	Minimum requirement for BS type 1-O

For NR, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

Table 10.4.2-1: Wide Area BS OTA dynamic range for NR carrier

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -82.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -79.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.4- ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -77.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.4- ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -64.5- ΔOTAREFSENS | -76.2- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -64.5- ΔOTAREFSENS | -75.2- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -64.5- ΔOTAREFSENS | -74.4- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -64.5- ΔOTAREFSENS | -73.7- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -64.5- ΔOTAREFSENS | -73.1- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -64.5- ΔOTAREFSENS | -72.6- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -64.5- ΔOTAREFSENS | -72.1- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS | -71.3- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS | -70.7- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS | -70.1- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS | -69.5- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -64.5- ΔOTAREFSENS | -69.1- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.8- ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 10.4.2-2: Medium Range BS OTA dynamic range for NR carrier

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -77.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4- ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -74.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.4- ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -72.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.4- ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -59.5- ΔOTAREFSENS | -71.2- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -59.5- ΔOTAREFSENS | -70.2- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -59.5- ΔOTAREFSENS | -69.4- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -59.5- ΔOTAREFSENS | -68.7- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -59.5- ΔOTAREFSENS | -68.1- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -59.5- ΔOTAREFSENS | -67.6- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -59.5- ΔOTAREFSENS | -67.1- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS | -66.3- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS | -65.7- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS | -65.1- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS | -64.5- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -59.5- ΔOTAREFSENS | -64.1- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.8- ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 10.4.2-3: Local Area BS OTA dynamic range for NR carrier

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -74.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.4- ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -71.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.4- ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -69.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.4- ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -56.5- ΔOTAREFSENS | -68.2- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -56.5- ΔOTAREFSENS | -67.2- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -56.5- ΔOTAREFSENS | -66.4- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -56.5- ΔOTAREFSENS | -65.7- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -56.5- ΔOTAREFSENS | -65.1- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -56.5- ΔOTAREFSENS | -64.6- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -56.5- ΔOTAREFSENS | -64.1- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS | -63.3- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS | -62.7- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS | -62.1- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS | -61.5- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -56.5- ΔOTAREFSENS | -61.1- ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.8- ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

---Next modified section ---

10.5.1	OTA adjacent channel selectivity

10.5.1.1	General

OTA Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

10.5.1.2	Minimum requirement for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1, the OTA wanted and the interfering signal are specified in table 10.5.1.2-1 and table 10.5.1.2-2 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 10.5.1.2-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the OTA ACS requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 10.5.1.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 10.5.1.2-1: OTA ACS requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 (Note 1) | EISminSENS + 6 dB | Wide Area BS: -52 – ΔminSENS Medium Range BS: -47– ΔminSENS Local Area BS: -44– ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth NOTE 2: EISminSENS depends on the BS channel bandwidth |  |  |

Table 10.5.1.2-2: OTA ACS interferer frequency offset for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 |  |
| 10 | ±2.5075 | 5 MHz DFT-s-OFDM NR signal, |
| 15 | ±2.5125 | 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 |  |
| 30 | ±9.4725 |  |
| 35 | ±9.4625 |  |
| 40 | ±9.4675 |  |
| 45 | ±9.4725 |  |
| 50 | ±9.4625 | 20 MHz DFT-s-OFDM NR signal, |
| 60 | ±9.4725 | 15 kHz SCS, 100 RBs |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

---Next modified section ---

10.5.2	OTA in-band blocking

10.5.2.1	General

The OTA in-band blocking characteristics is a measure of the receiver's ability to receive a OTA wanted signal at its assigned channel in the presence of an unwanted OTA interferer, which is an NR signal for general blocking or an NR signal with one RB for narrowband blocking.

10.5.2.2	Minimum requirement for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted and OTA interfering signal specified in tables 10.5.2.2-1, table 10.5.2.2-2 and table 10.5.2.2-3 for general OTA and narrowband OTA blocking requirements. The reference measurement channel for the OTA wanted signal is identified in clause 10.3.2 and are further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For BS type 1-O the OTA in-band blocking requirement shall apply in the in-band blocking frequency range, which is from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the FDD operating band. The ΔfOOB for BS type 1-O is defined in table 10.5.2.2-0.

Table 10.5.2.2-0: ΔfOOB offset for NR operating bands in FR1

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| BS type 1-O | FUL,high – FUL,low < 100 MHz | 20 |
|  | 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz | 60 |

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in table 10.5.2.2-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the OTA in-band blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 10.5.2.2-1 and 10.5.2.2-3.

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA narrowband blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the interfering signal minimum offset in table 10.5.2.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band RIBs, the OTA narrowband blocking requirements apply in the narrowband blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the interfering signal minimum offset in table 10.5.2.2-3.

Table 10.5.2.2-1: General OTA blocking requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
|  | EISREFSENS + 6 dB | Wide Area BS: -43 - ΔOTAREFSENS Medium Range BS: -38 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS | ±7.5 |  |
| 5, 10, 15, 20 | EISminSENS + 6 dB | Wide Area BS: -43 – ΔminSENS Medium Range BS: -38 – ΔminSENS Local Area BS: -35 – ΔminSENS | ±7.5 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
|  | EISREFSENS + 6 dB | Wide Area BS: -43 - ΔOTAREFSENS Medium Range BS: -38 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS | ±30 |  |
| 25 ,30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISminSENS + 6 dB | Wide Area BS: -43 – ΔminSENS Medium Range BS: -38 – ΔminSENS Local Area BS: -35 – ΔminSENS | ±30 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |

Table 10.5.2.2-2: OTA narrowband blocking requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA Wanted signal mean power (dBm) | OTA Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20 | EISREFSENS + 6 dB | Wide Area BS: -49 - ΔOTAREFSENS Medium Range BS: -44 - ΔOTAREFSENS Local Area BS: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB | Wide Area BS: -49 – ΔminSENS Medium Range BS: -44 – ΔminSENS Local Area BS: -41 – ΔminSENS |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB | Wide Area BS: -49 - ΔOTAREFSENS Medium Range BS: -44 - ΔOTAREFSENS Local Area BS: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB | Wide Area BS: -49 – ΔminSENS Medium Range BS: -44 – ΔminSENS Local Area BS: -41 – ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: 7.5 kHz shift is not applied to the wanted signal. |  |  |

Table 10.5.2.2-3: OTA narrowband blocking interferer frequency offsets for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±(350 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 10 | ±(355 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | ±(360 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | ±(350 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 30 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 35 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 40 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 45 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 50 | ±(560 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 60 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 70 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 80 | ±(560 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 90 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 100 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| NOTE 1: Interfering signal consisting of one resource block is positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

---Next modified section ---

10.8.2	Minimum requirement for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals at the RIB with the conditions specified in tables 10.8.2-1 and 10.8.2-2 for intermodulation performance and in tables 10.8.2-3 and 10.8.2-4 for narrowband intermodulation performance.

The reference measurement channel for the wanted signal is identified in table 10.3.2-1, table 10.3.2-2 and table 10.3.2-3 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The subcarrier spacing for the modulated interfering signal shall be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60kHz and BS channel bandwidth <=20MHz, for which the subcarrier spacing of the interfering signal shall be 30kHz.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement shall apply in addition inside any sub-block gap in case the sub-block gap is at least as wide as the BS channel bandwidth of the NR interfering signal in tables 10.8.2-2 and 10.8.2-4. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the NR interfering signal centre frequency offset from the Base Station RF Bandwidth edge.

For multi-band RIBs, the narrowband intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the NR interfering signal in tables 10.8.2-2 and 10.8.2-4. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 10.8.2-1: General intermodulation requirement

| BS class | Wanted Signal mean power (dBm) | Mean power of the interfering signals (dBm) | Type of interfering signals |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB | -52 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB | -52 - ΔminSENS |  |
| Medium Range | EISREFSENS + 6 dB | -47 - ΔOTAREFSENS | See Table 10.8.2-2 |
| BS | EISminSENS + 6 dB | -47 - ΔminSENS |  |
| Local Area BS | EISREFSENS + 6 dB | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB | -44 - ΔminSENS |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the BS class and on the BS channel bandwidth, see clause 10.3 and 10.2. |  |  |  |

Table 10.8.2-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper base station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 10 | ±7.465 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 15 | ±7.43 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 20 | ±7.395 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 25 | ±7.465 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 30 | ±7.43 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 35 | ±7.44 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 40 | ±7.45 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 45 | ±7.37 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 50 | ±7.35 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 60 | ±7.49 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 70 | ±7.42 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 80 | ±7.44 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 90 | ±7.46 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 100 | ±7.48 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| NOTE 1: Number of RBs is 25 for 15 kHz subcarrier spacing and 10 for 30 kHz subcarrier spacing. NOTE 2: Number of RBs is 100 for 15 kHz subcarrier spacing, 50 for 30 kHz subcarrier spacing and 24 for 60 kHz subcarrier spacing. NOTE 3: The RBs shall be placed adjacent to the transmission bandwidth configuration edge which is closer to the Base Station RF Bandwidth edge. |  |  |

Table 10.8.2-3: Narrowband intermodulation performance requirement in FR1

| BS class | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signals |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB (Note 1) | -52 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (Note 1) | -52 - ΔminSENS |  |
| Medium Range BS | EISREFSENS + 6 dB (Note 1) | -47 - ΔOTAREFSENS | See Table 10.8.2-4 |
|  | EISminSENS + 6 dB (Note 1) | -47 - ΔminSENS |  |
| Local Area BS | EISREFSENS + 6 dB (Note 1) | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (Note 1) | -44 - ΔminSENS |  |
| NOTE 1: EISREFSENS / EISminSENS depends on the BS channel bandwidth, see clause 10.3 and 10.2. |  |  |  |

Table 10.8.2-4: Interfering signals for narrowband intermodulation requirement in FR1

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±360 | CW |
|  | ±1420 | 5 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 10 | ±370 | CW |
|  | ±1960 | 5 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 15 (NOTE 2) | ±380 | CW |
|  | ±1960 | 5 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 20 (NOTE 2) | ±390 | CW |
|  | ±2320 | 5 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 25 (NOTE 2) | ±325 | CW |
|  | ±2350 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 30 (NOTE 2) | ±335 | CW |
|  | ±2350 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 35 (NOTE 2) | ±345 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 40 (NOTE 2) | ±355 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 45 (NOTE 2) | ±365 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 50 (NOTE 2) | ±375 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 60 (NOTE 2) | ±395 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 70 (NOTE 2) | ±415 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 80 (NOTE 2) | ±435 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 90 (NOTE 2) | ±365 | CW |
|  | ±2530 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 100 (NOTE 2) | ±385 | CW |
|  | ±2530 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the BS channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 3: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

---Next modified section ---

10.9.2	Minimum requirement for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA

The wanted and interfering signals applies to each supported polarization, under the assumption of polarization match.

For a wanted and an interfering signal coupled to the RIB, the following requirements shall be met:

-	For BS type 1-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 10.9.2-1 for Wide Area BS, in table 10.9.2-2 for Medium Range BS and in table 10.9.2-3 for Local Area BS. The characteristics of the interfering signal is further specified in annex D.

Table 10.9.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -100.6-ΔminSENS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7-ΔminSENS | -77.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -92.3-ΔminSENS | -71.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -101.3-ΔminSENS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -98.8-ΔminSENS | -78.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -92.6-ΔminSENS | -71.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -98.2-ΔminSENS | -78.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -92.7-ΔminSENS | -71.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 . The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 10.9.2-2:  Medium Range BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -95.6-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -93.7-ΔminSENS | -72.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -87.3-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -96.3-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -93.8-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -87.6-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -93.2-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 50, 60, 70, 80, 90,100 | 60 | G-FR1-A1-6 | -87.7-ΔminSENS | -66.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 10.9.2-3: Local area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -92.6-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -90.7-ΔminSENS | -69.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -84.3-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -93.3-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -90.8-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -84.6-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -90.2-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -84.7-ΔminSENS | -63.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

---Next modified section ---

B.5.2	Window length

Table B.5.2-1, B.5.2-2, B.5.2-3 specify the EVM window length (W) for normal CP.

Table B.5.2-1: EVM window length for normal CP, FR1, 15 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 14 | 40 |
| 10 | 1024 | 72 | 28 | 40 |
| 15 | 1536 | 108 | 44 | 40 |
| 20 | 2048 | 144 | 58 | 40 |
| 25 | 2048 | 144 | 72 | 50 |
| 30 | 3072 | 216 | 108 | 50 |
| 35 | 3072 | 216 | 108 | 50 |
| 40 | 4096 | 288 | 144 | 50 |
| 45 | 4096 | 288 | 144 | 50 |
| 50 | 4096 | 288 | 144 | 50 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 to 6 and 8 to 13. Symbols 0 and 7 have a longer CP and therefore a lower percentage. |  |  |  |  |

Table B.5.2-2: EVM window length for normal CP, FR1, 30 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 256 | 18 | 8 | 40 |
| 10 | 512 | 36 | 14 | 40 |
| 15 | 768 | 54 | 22 | 40 |
| 20 | 1024 | 72 | 28 | 40 |
| 25 | 1024 | 72 | 36 | 50 |
| 30 | 1536 | 108 | 54 | 50 |
| 35 | 1536 | 108 | 54 | 50 |
| 40 | 2048 | 144 | 72 | 50 |
| 45 | 2048 | 144 | 72 | 50 |
| 50 | 2048 | 144 | 72 | 50 |
| 60 | 3072 | 216 | 130 | 60 |
| 70 | 3072 | 216 | 130 | 60 |
| 80 | 4096 | 288 | 172 | 60 |
| 90 | 4096 | 288 | 172 | 60 |
| 100 | 4096 | 288 | 172 | 60 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 through 13. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |  |

Table B.5.2-3: EVM window length for normal CP, FR1, 60 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) |
| --- | --- | --- | --- | --- |
| 10 | 256 | 18 | 8 | 40 |
| 15 | 384 | 27 | 11 | 40 |
| 20 | 512 | 36 | 14 | 40 |
| 25 | 512 | 36 | 18 | 50 |
| 30 | 768 | 54 | 26 | 50 |
| 35 | 768 | 54 | 26 | 50 |
| 40 | 1024 | 72 | 36 | 50 |
| 45 | 1024 | 72 | 36 | 50 |
| 50 | 1024 | 72 | 36 | 50 |
| 60 | 1536 | 108 | 64 | 60 |
| 70 | 1536 | 108 | 64 | 60 |
| 80 | 2048 | 144 | 86 | 60 |
| 90 | 2048 | 144 | 86 | 60 |
| 100 | 2048 | 144 | 86 | 60 |
| NOTE: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 2. Symbol 0 of slot 0 and slot 2 may have a longer CP and therefore a lower percentage. |  |  |  |  |

Table B.5.2-4 below specifies the EVM window length (W) for extended CP. The number of CP samples excluded from the EVM window is the same as for normal CP length.

Table B.5.2-4: EVM window length for extended CP, FR1, 60 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) |
| --- | --- | --- | --- | --- |
| 10 | 256 | 64 | 54 | 84 |
| 15 | 384 | 96 | 80 | 83 |
| 20 | 512 | 128 | 106 | 83 |
| 25 | 512 | 128 | 110 | 85.9 |
| 30 | 768 | 192 | 164 | 85.9 |
| 35 | 768 | 192 | 164 | 85.9 |
| 40 | 1024 | 256 | 220 | 85.9 |
| 45 | 1024 | 256 | 220 | 85.9 |
| 50 | 1024 | 256 | 220 | 85.9 |
| 60 | 1536 | 384 | 340 | 88.6 |
| 70 | 1536 | 384 | 340 | 88.7 |
| 80 | 2048 | 512 | 454 | 88.7 |
| 90 | 2048 | 512 | 454 | 88.7 |
| 100 | 2048 | 512 | 454 | 88.7 |
| NOTE: These percentages are informative. |  |  |  |  |  |

------------------------------ End of modified section ---------------------------