3GPP TSG-RAN WG4 Meeting #99-e	R4-2110780

Electronic Meeting, 19 May – 27 May 2021

Agenda Item:	6.1.5.1

Source:	Ericsson

Title:	SSB monitoring capability for CBD

Document for:	Discussion

# 1	Introduction

RAN4#98bis-e agreed with the way forward on terminology update due to CSSF as a part of NR-U RRM core requirements [1], and the remaining open issue is the determination of the availability of SSB occasions for CBD.

General: Terminology updates due to CSSF - For RLM: The UE is not required to determine the availability of SSB occasions more frequent than once per L1 indication interval. - For CBD - FFS whether the UE is not required to determine the availability of SSB occasions more frequent than once per P*DRX cycle length when configured with DRX.

# 2	Discussion

For the terminology for SSB-based RLM, it was agreed that the UE is not required to determine the availability of SSB occasions more frequent than once per L1 indication interval, where L1 indication interval with DRX is specified as follows (TS 38.133 8.1A.5):

- TIndication_interval,CCA = Max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) if DRX_cycle_length ≤ 320ms

- TIndication_interval,CCA = DRX_cycle_length if DRX_cycle_length > 320ms.

In our understanding, L1 indication interval for RLM is derived from the RLM evaluation period as shown in Table 1. For DRX with DRX_cycle_length ≤ 320ms, the L1 indication depends on the configuration of TSSB and TDRX, but the relaxation factor 1.5 applies according to Table 1. For DRX with DRX_cycle_length > 320ms, the L1 indication interval is same as TDRX and no relaxation factor is applied. Here we should potin out the L1 indication interval does not include P and number of measurement samples.

Table 1	Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA (TS 38.133 Table 8.1A.2.2-1)

| Configuration | TEvaluate_out_SSB,CCA (ms) |  | TEvaluate_in_SSB,CCA (ms) |
| --- | --- | --- | --- |
|  | RLM-RS SSB Es/IotNote4 ≥-7 dB | RLM-RS SSB Es/Iot Note4 <-7 dB |  |
| no DRX | Max(200, Ceil(17*P)*TSSB) | Max(200, Ceil(24*P)*TSSB) | Max(100, Ceil((5+Lin)*P)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*15*P)*Max(TDRX,TSSB)) | Max(200, Ceil(1.5*20*P)*Max(TDRX,TSSB)) | Max(100, Ceil(1.5*(5+Lin)*P)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil(13*P)*TDRX | Ceil(16*P)*TDRX | Ceil((5+Lin)*P)*TDRX |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: Lin is the number of RLM-RS SSB occasions which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. [The UE is not required to determine the availability of SSB occasions more frequent than once per DRX cycle length, when configured with DRX.] NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. NOTE 4: RLM-RS SSB Es/Iot is the averaged Es/Iot over the most recent previous out-of-sync evaluation period. |  |  |  |

Table 2	Evaluation period TEvaluate_CBD_SSB_CCA (TS 38.133 Table 8.5A.5.2-1)

| Configuration | TEvaluate_CBD_SSB_CCA (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil((3 + LCBD) P) TSSB) |
| DRX cycle > 320ms | Ceil((3 + LCBD) P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q1. TDRX is the DRX cycle length. Note 2: LCBD is the number of CBD-RS SSB occasions not available at the UE during TEvaluate_CBD_SSB_CCA where LCBD ≤ LCBD,max. [The UE is not required to determine the availability of SSB occasions more frequent than once per DRX cycle length, when configured with DRX.] Note 3: LCBD,max=7 for Max(TDRX, TSSB) ≤ 40 assuming TDRX=0 for non-DRX, LCBD,max=5 for 40 < Max(TDRX, TSSB) ≤ 320, LCBD,max=3 for TDRX > 320. Note 4 If LCBD>LCBD,max, the UE shall assume no new candidate beams are found for this evaluation period. |  |

Since CBD has no L1 indication interval, we can derive the determination of availability of SSB occasions from the CBD evaluation period in Table 2. Since the relaxation factor of 1.5 is not applicable for CBD even with DRX cycle length ≤ 320ms, we propose the determination of availability of SSB occasions for CBD as follows.

Proposal: For CBD, the UE is not required to determine the availability of SSB occasions more frequent than:

- Once per Max(25ms, DRX_cycle_length, TSSB),	if DRX_cycle_length ≤ 320ms

- Once per DRX_cycle_length,							if DRX_cycle_length > 320ms.

# 3	Summary

Proposal: For CBD, the UE is not required to determine the availability of SSB occasions more frequent than:

- Once per Max(25ms, DRX_cycle_length, TSSB),	if DRX_cycle_length ≤ 320ms

- Once per DRX_cycle_length,							if DRX_cycle_length > 320ms.

# 4	References

- R4-2105700, “WF on NR-U RRM Core Requirements”, Ericsson.