3GPP TSG-RAN WG4 Meeting #99e	R4-2110932

# Electronic Meeting, May. 19-27, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0828 | rev | - | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | R15 CR on simultaneous Tx-Rx for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The simultaneous RxTx supported by band combinations in the spec is unclear with some of the band combinations explicitly mandatory to support or not support simultaneous RxTx. It is unclear for band combinations which neither have mandatory support nor not support notes in the spec. For these band combinations it should be interpretated as the simultaneous Rx/Tx is optionally supported. This is also aligned with the UE capability below in 38.306. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification sentence in section 5.2A.0 to clarify that the minimum requirements apply for UEs with or without simultaneous Tx/Rx capability unless otherwise stated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is still unclear whether a band combinations is mandatory or optional support simultaneous Tx/Rx for band combinations without explicit notes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

5.2A	Operating bands for CA

5.2A.0	General

Unless otherwise stated, the minimum requirements apply for UEs with or without simultaneous Tx/Rx capability for TDD-TDD and TDD-FDD band combinations defined in this clause.

Unless otherwise stated, the minimum requirements apply for UEs with simultaneous Tx/Rx for FDD-FDD band combinations defined in this clause.

5.2A.1	Intra-band CA

NR intra-band contiguous carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1, where all operating bands are within FR1.

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n411 | n41 |
| CA_n771 | n77 |
| CA_n781 | n78 |
| CA_n791 | n79 |
| NOTE 1: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |

5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, where all operating bands are within FR1.

Table 5.2A.2-1: Void

5.2A.2.1	Inter-band CA (two bands)

Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3-n771 | n3, n77 |
| CA_n3-n781 | n3, n78 |
| CA_n3-n791 | n3, n79 |
| CA_n8-n751 | n8, n75 |
| CA n8-n781 | n8, n78 |
| CA_n8-n791 | n8, n79 |
| CA_n28-n752 | n28, n75 |
| CA_n28-n781 | n28, n78 |
| CA_n41-n78 | n41, n78 |
| CA_n75-n781 | n75, n78 |
| CA_n76-n781 | n76, n78 |
| CA_n77-n79 | n77, n79 |
| CA_n78-n793 | n78, n79 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 3: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 4: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. |  |

5.2B	Void

5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, where all operating bands are within FR1.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 | n78, n81 |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 | n78, n83 |
| SUL_n78-n842 | n78, n84 |
| SUL_n78-n862 | n78, n86 |
| SUL_n79-n802 | n79, n80 |
| SUL_n79-n812 | n79, n81 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: For UE supporting SUL band combination, UL MIMO is not configured on SUL carrier. |  |

## <<End of change>>