3GPP TSG- RAN WG4 Meeting #99-e	R4-2110938

# Electronic Meeting, May 19th – May 27th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draft | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.101-4, PDSCH requirements for standalone NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Demodulation performance requirement for NR-U needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The requirements for demodulation performance for NR-U are introduced in section 5.2.2.2.15 and 5.2.3.2.15 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demodulation performance requirements for NR-U are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 5.2.2.2.15 and 5.2.3.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

##### 5.2.2.2.15	Minimum requirements for PDSCH under CCA

The performance requirements are specified in Table 5.2.2.2.15-3, with the addition of test parameters in Table 5.2.2.2.15-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.15-1.

Table 5.2.2.2.15-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under CCA when CSI-RS-ValidationWith-DCI-r16 IE [17] is configured | 1-1, 1-2, 1-3, 1-4 |

Table 5.2.2.2.15-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| DL transmission model |  |  |  | [As specified in x.x] |
| LBT failure probability |  |  |  | 0.25 |
| Common serving cell parameters |  | Physical Cell ID |  | 0 |
|  |  | SSB position in burst |  | The first SSB |
|  |  | SSB Q factor |  | 8 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | The number of slots set in a burst |  |  | {2,4,6,7} |
|  | Occupied OFDM symbols set other than the last slot |  |  | 12 |
|  | Occupied OFDM symbols set in the last slot |  |  | {4,7,10,12} (Note 1) |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity |  | Slots | 40 |
|  | CSI-RS offset |  | Slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| TX EVM |  |  |  | 6% |
| Note 1: If DL Transmission duration is 2 Slot, the occupied OFDM symbols for PDSCH in the last slot is 12. |  |  |  |  |

Table 5.2.2.2.15-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [TBD] | 20 / 30 | 16QAM, 0.48 | [TBD] | TDLA30-10 | 2x2, ULA Low | 70 | [TBD] |
| 1-2 | [TBD] | 40 / 30 | 16QAM, 0.48 | [TBD] | TDLA30-10 | 2x2, ULA Low | 70 | [TBD] |
| 1-3 | [TBD] | 60 / 30 | 16QAM, 0.48 | [TBD] | TDLA30-10 | 2x2, ULA Low | 70 | [TBD] |
| 1-4 | [TBD] | 80 / 30 | 16QAM, 0.48 | [TBD] | TDLA30-10 | 2x2, ULA Low | 70 | [TBD] |

END OF CHANGE 1

START OF CHANGE 1

##### 5.2.3.2.15	Minimum requirements for PDSCH under CCA

The performance requirements are specified in Table 5.2.3.2.15-3, with the addition of test parameters in Table 5.2.3.2.15-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.15-1.

Table 5.2.3.2.15-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under CCA when CSI-RS-ValidationWith-DCI-r16 IE [17] is configured | 1-1, 1-2, 1-3, 1-4 |

Table 5.2.3.2.15-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| DL transmission model |  |  |  | [As specified in x.x] |
| LBT failure probability |  |  |  | 0.25 |
| Common serving cell parameters |  | Physical Cell ID |  | 0 |
|  |  | SSB position in burst |  | The first SSB |
|  |  | SSB Q factor |  | 8 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | The number of slots set in a burst |  |  | {2,4,6,7} |
|  | Occupied OFDM symbols set other than the last slot |  |  | 12 |
|  | Occupied OFDM symbols set in the last slot |  |  | {4,7,10,12} (Note 1) |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity |  | Slots | 40 |
|  | CSI-RS offset |  | Slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| TX EVM |  |  |  | 6% |
| Note 1: If DL Transmission duration is 2 Slot, the occupied OFDM symbols for PDSCH in the last slot is 12. |  |  |  |  |

Table 5.2.3.2.15-3: Minimum performance for Rank 4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [TBD] | 20 / 30 | 16QAM, 0.48 | [TBD] | TDLA30-10 | 2x4, ULA Low | 70 | [TBD] |
| 1-2 | [TBD] | 40 / 30 | 16QAM, 0.48 | [TBD] | TDLA30-10 | 2x4, ULA Low | 70 | [TBD] |
| 1-3 | [TBD] | 60 / 30 | 16QAM, 0.48 | [TBD] | TDLA30-10 | 2x4, ULA Low | 70 | [TBD] |
| 1-4 | [TBD] | 80 / 30 | 16QAM, 0.48 | [TBD] | TDLA30-10 | 2x4, ULA Low | 70 | [TBD] |

END OF CHANGE 2