3GPP TR 38.717-02-01 V0.5.0 (2021-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2)

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	25

1	Scope		26

2	References		26

3	Definitions, symbols and abbreviations	26

3.1	Definitions		26

3.2	Symbols		27

3.3	Abbreviations		27

4	Background		27

4.1	TR Maintenance	27

5	NR Inter-Band Combination: General Part	27

6	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part	28

6.1	 CA_n5-n25		28

6.1.1	Common for 1 band UL and 2 bands UL CA	28

6.1.1.1 Operating bands for CA	28

6.1.1.2	Channel bandwidths per operating band for CA	28

6.1.1.3	Co-existence studies	28

6.1.1.4	∆TIB and ∆RIB values	29

6.1.1.5	REFSENs requirements	29

6.1.1.6	OOB blocking exception requirements	29

6.1.2	 Specific for 2 bands UL CA	29

6.1.2.1	Maximum output power for inter-band CA	29

6.1.2.2	 UE co-existence	30

6.1.2.3	 REFSENS requirements	31

6.2	 CA_n71-n78		31

6.2.1	Common for 1 band UL and 2 bands UL CA	31

6.2.1.1 Operating bands for CA	31

6.2.1.2	Channel bandwidths per operating band for CA	31

6.2.1.3	Co-existence studies	32

6.2.1.4	∆TIB and ∆RIB values	32

6.2.1.5	REFSENs requirements	32

6.2.1.6	OOB blocking exception requirements	32

6.2.2	 Specific for 2 bands UL CA	32

6.2.2.1	Maximum output power for inter-band CA	32

6.2.2.2	 UE co-existence	32

6.2.2.3	 REFSENS requirements	33

6.3	 CA_n7-n66		33

6.3.1	Common for 1 band UL and 2 bands UL CA	33

6.3.1.1	Operating bands for CA	33

6.3.1.2	Channel bandwidths per operating band for CA	34

6.3.1.3	UE co-existence studies	34

6.3.1.4	∆TIB and ∆RIB values	34

6.3.1.5	REFSENS requirements	35

6.3.1.6	OOB blocking exception requirements	35

6.3.2	Specific for 2 bands UL CA	35

6.3.2.1	Maximum output power for inter-band CA	35

6.3.2.2	UE co-existence studies	35

6.3.2.3	REFSENS requirements	36

6.4	 CA_n25-38		37

6.4.1	 Common for 1 band UL and 2 bands UL CA	37

6.4.1.2	Channel bandwidths per operating band for CA	37

6.4.1.2	Channel bandwidths per operating band for CA	37

6.4.1.3	UE co-existence studies	37

6.4.1.4	∆TIB and ∆RIB values	38

6.4.1.5	REFSENS requirements	38

6.4.1.6	OOB blocking exception requirements	39

6.4.2	Specific for 2 bands UL CA	39

6.4.2.1	Maximum output power for inter-band CA	39

6.4.2.2	UE co-existence studies	39

6.4.2.3	REFSENS requirements	40

6.5	 CA_n1-n77		41

6.5.1	 Common for 1 band UL and 2 bands UL CA	41

6.5.1.1	  Operating bands for CA	41

6.5.1.2	  Channel bandwidths per operating band for CA	41

6.5.1.3	  UE co-existence studies	43

6.5.1.4	  ∆TIB and ∆RIB values	43

6.5.1.5	  REFSENS requirements	43

6.5.1.6	  OOB blocking exception requirements	43

6.5.2	 Specific for 2 bands UL CA	43

6.5.2.1	  Maximum output power for inter-band CA	43

6.5.2.2	  UE co-existence studies	43

6.5.2.3	  REFSENS requirements	45

6.6	CA_n77-n79		45

6.6.1	 Common for 1 band UL and 2 bands UL CA	45

6.6.1.1	  Operating bands for CA	45

6.6.1.2	  Channel bandwidths per operating band for CA	46

6.6.1.3	  UE co-existence studies	48

6.6.1.4	  ∆TIB and ∆RIB values	48

6.6.1.5	  REFSENS requirements	48

6.6.1.6	  OOB blocking exception requirements	48

6.6.2	 Specific for 2 bands UL CA	48

6.6.2.1	  Maximum output power for inter-band CA	48

6.6.2.2	  UE co-existence studies	48

6.7	CA_n78-n79		51

6.7.1	 Common for 1 band UL and 2 bands UL CA	51

6.7.1.1	 Operating bands for CA	51

6.7.1.2	 Channel bandwidths per operating band for CA	51

6.7.1.3	 UE co-existence studies	53

6.7.1.4	 ∆TIB and ∆RIB values	53

6.7.1.5	 REFSENS requirements	53

6.7.1.6	 OOB blocking exception requirements	54

6.7.2	 Specific for 2 bands UL CA	54

6.7.2.1	 Maximum output power for inter-band CA	54

6.7.2.2	 UE co-existence studies	54

6.7.2.3	 REFSENS requirements	56

6.8	CA_n25-n48		56

6.8.1	Common for 1 band UL and 2 bands UL CA	56

6.8.1.1	 Operating bands for CA	56

6.8.1.2	 Channel bandwidths per operating band for CA	56

6.8.1.3	 UE co-existence studies	57

6.8.1.4	 ∆TIB and ∆RIB values	57

6.8.1.5 	 REFSENS requirements	57

6.8.1.6 	 OOB blocking exception requirements	58

6.8.2	Specific for 2 bands UL CA	58

6.8.2.1	Maximum output power for inter-band CA	58

6.8.2.2	UE co-existence studies	58

6.8.2.3	REFSENS requirements	60

6.9	CA_n28-n79		60

6.9.1	Common for 1 band UL and 2 bands UL CA	60

6.9.1.1 Operating bands for CA	60

6.9.1.2	Channel bandwidths per operating band for CA	61

6.9.1.3	UE Co-existence studies	61

6.9.1.4	∆TIB and ∆RIB values	61

6.9.1.5	REFSENs requirements	62

6.9.1.6	OOB blocking exception requirements	62

6.9.2	 Specific for 2 bands UL CA	62

6.9.2.1	Maximum output power for inter-band CA	62

6.9.2.2	 UE co-existence studies	62

6.9.2.3	 REFSENS requirements	64

6.12	 CA_n28-n77		67

6.12.1	 Common for 1 band UL and 2 bands UL CA	67

6.12.1.1	 Operating bands for CA	67

6.12.1.2	 Channel bandwidths per operating band for CA	67

6.12.1.4	 ∆TIB and ∆RIB values	67

6.12.1.5	 REFSENS requirements	68

6.12.1.6	    OOB blocking exception requirements	68

6.13	 CA_n28-n78		68

6.13.1	 Common for 1 band UL and 2 bands UL CA	68

6.13.1.1	 Operating bands for CA	68

6.13.1.2	 Channel bandwidths per operating band for CA	68

6.13.1.4	 ∆TIB and ∆RIB values	69

6.13.1.5	 REFSENS requirements	69

6.13.1.6	    OOB blocking exception requirements	69

6.14.1.4	 ∆TIB and ∆RIB values	70

6.15.1.4	 ∆TIB and ∆RIB values	72

6.15.2	 Specific for 2 bands UL CA	73

6.15.2.1	Maximum output power for inter-band CA	73

6.15.2.2	 UE co-existence studies	73

6.15.2.3	 REFSENS requirements	74

6.16	CA_n41-n77		74

6.16.1	Common for 1 band UL and 2 bands UL CA	74

6.16.1.1 Operating bands for CA	74

6.16.1.2	Channel bandwidths per operating band for CA	75

6.16.1.3	Co-existence studies	75

6.16.1.4	∆TIB and ∆RIB values	76

6.16.1.5	REFSENs requirements	76

6.16.1.6	OOB blocking exception requirements	76

6.16.2	 Specific for 2 bands UL CA	76

6.16.2.1	Maximum output power for inter-band CA	76

6.16.2.2	 UE co-existence	76

6.16.2.3	 REFSENS requirements	78

6.17	 CA_n71-n77		79

6.17.1	Common for 1 band UL and 2 bands UL CA	79

6.17.1.1 Operating bands for CA	79

6.17.1.2	Channel bandwidths per operating band for CA	79

6.17.1.3	Co-existence studies	79

6.17.1.4	∆TIB and ∆RIB values	80

6.17.1.5	REFSENs requirements	80

6.17.1.6	OOB blocking exception requirements	80

6.17.2	 Specific for 2 bands UL CA	80

6.17.2.1	Maximum output power for inter-band CA	80

6.17.2.2	 UE co-existence	80

6.17.2.3	 REFSENS requirements	82

6.18	CA_n5-n48		82

6.18.1	Common for 1 band UL and 2 bands UL CA	82

6.18.1.1	 Operating bands for CA	82

6.18.1.2	 Channel bandwidths per operating band for CA	82

6.18.1.3	 UE co-existence studies	83

6.18.1.4	 ∆TIB and ∆RIB values	83

6.18.1.5	 REFSENS requirements	84

6.18.1.6	 OOB blocking exception requirements	84

6.18.2	 Specific for 2 bands UL CA	84

6.18.2.1	Maximum output power for inter-band CA	84

6.18.2.2	 UE co-existence	84

6.18.2.3  REFSENs requirements	85

6.19	 CA_n34-n79		85

6.19.1	 Common for 1 band UL and 2 bands UL CA	85

6.19.1.1	 Operating bands for CA	85

6.19.1.2	 Channel bandwidths per operating band for CA	86

6.19.1.3	 Co-existence studies	86

6.19.1.4	 ∆TIB and ∆RIB values	86

6.19.1.5	 REFSENS requirements	87

6.19.1.6	 OOB blocking exception requirements	87

6.19.2	 Specific for 2 bands UL CA	87

6.19.2.1	 UE co-existence studies	87

6.19.2.2	 REFSENS requirements	88

6.20	 CA_n5-n25		88

6.20.1	Common for 1 band UL and 2 bands UL CA	88

6.20.1.1	Operating bands for CA	88

6.20.1.2	Channel bandwidths per operating band for CA	89

6.20.1.3	UE co-existence studies	89

6.20.1.4	∆TIB and ∆RIB values	89

6.20.1.5	REFSENS requirements	90

6.20.1.6	OOB blocking exception requirements	90

6.20.2	Specific for 2 bands UL CA	90

6.20.2.1	Maximum output power for inter-band CA	90

6.20.2.2	UE co-existence studies	90

6.20.2.3	REFSENS requirements	91

6.21	 CA_n25-n77		91

6.21.1	Common for 1 band UL and 2 bands UL CA	91

6.21.1.1	Operating bands for CA	91

6.21.1.2	Channel bandwidths per operating band for CA	91

6.21.1.3	UE co-existence studies	92

6.21.1.4	∆TIB and ∆RIB values	92

6.21.1.5	REFSENS requirements	92

6.21.1.6	OOB blocking exception requirements	93

6.21.2	Specific for 2 bands UL CA	94

6.21.2.1	Maximum output power for inter-band CA	94

6.21.2.2	UE co-existence studies	94

6.21.2.3	REFSENS requirements	95

6.22	CA_n8-n28		95

6.22.1	Common for 1 band UL and 2 bands UL CA	95

6.22.1.1 Operating bands for CA	95

6.22.1.2	Channel bandwidths per operating band for CA	96

6.22.1.3	UE Co-existence studies	96

6.22.1.4	∆TIB and ∆RIB values	96

6.22.1.5	REFSENs requirements	97

6.22.1.6	OOB blocking exception requirements	97

6.23	CA_n3-n7		97

6.23.1	Common for 1 band UL and 2 bands UL CA	97

6.23.1.1 Operating bands for CA	97

6.23.1.2	Channel bandwidths per operating band for CA	97

6.23.1.3	UE Co-existence studies	97

6.23.1.4	∆TIB and ∆RIB values	98

6.23.1.5	REFSENs requirements	98

6.23.1.6	OOB blocking exception requirements	99

6.24.1.4	 ∆TIB and ∆RIB values	100

6.24.1.5 REFSENS requirements	100

6.24.1.6	OOB blocking exception requirements	100

6.24.2	Specific for 2 bands UL CA	100

6.24.2.1	Maximum output power for inter-band CA	100

6.24.2.2	UE co-existence studies	100

6.24.2.3	REFSENS requirements	101

6.25.1.4	 ∆TIB and ∆RIB values	103

6.25.2	Specific for 2 bands UL CA	104

6.25.2.1	Maximum output power for inter-band CA	104

6.25.2.2	UE co-existence studies	104

6.25.2.3	REFSENS requirements	105

6.26.1.4	 ∆TIB and ∆RIB values	106

6.26.1.5 REFSENS requirements	107

6.26.1.6	OOB blocking exception requirements	107

6.27.1.3	 UE co-existence studies	107

6.27.1.4	 ∆TIB and ∆RIB values	108

6.27.1.5 	 REFSENS requirements	108

6.27.2	Specific for 2 bands UL CA	109

6.27.2.1	 UE co-existence studies	109

6.27.2.2	 REFSENS requirements	111

6.28.1.4	 ∆TIB and ∆RIB values	112

6.28.1.5 REFSENS requirements	112

6.28.1.6	OOB blocking exception requirements	112

6.28.2	Specific for 2 bands UL CA	112

6.28.2.1	Maximum output power for inter-band CA	112

6.28.2.2	UE co-existence studies	113

6.28.2.3	REFSENS requirements	114

6.29.1.4	 ∆TIB and ∆RIB values	115

6.29.1.5 REFSENS requirements	115

6.29.1.6	OOB blocking exception requirements	115

6.29.2	Specific for 2 bands UL CA	115

6.29.2.1	Maximum output power for inter-band CA	115

6.29.2.2	UE co-existence studies	116

6.29.2.3	REFSENS requirements	117

6.30	CA_n8-n20		117

6.30.1	Common for 1 band UL and 2 bands UL CA	117

6.30.1.1 Operating bands for CA	117

6.30.1.2	Channel bandwidths per operating band for CA	117

6.30.1.3	UE Co-existence studies	117

6.30.1.4	∆TIB and ∆RIB values	118

6.30.1.5	REFSENs requirements	118

6.30.1.6	OOB blocking exception requirements	118

6.31	CA_n7-n77		119

6.31.1	Common for 1 band UL and 2 bands UL CA	119

6.31.1.1	 Operating bands for CA	119

6.31.1.2	 Channel bandwidths per operating band for CA	119

6.31.1.3	 UE co-existence studies	119

6.31.1.4	 ∆TIB and ∆RIB values	119

6.31.1.5 	 REFSENS requirements	120

6.31.1.6 	 OOB blocking exception requirements	120

6.31.2	 Specific for 2 bands UL CA	120

6.31.2.1	Maximum output power for inter-band CA	120

6.31.2.2	 UE co-existence studies	120

6.31.2.3	 REFSENS requirements	121

6.32	 CA_n12-n77		122

6.32.1	Common for 1 band UL and 2 bands UL CA	122

6.32.1.1 Operating bands for CA	122

6.32.1.2	Channel bandwidths per operating band for CA	122

6.32.1.3	Co-existence studies	122

6.32.1.4	∆TIB and ∆RIB values	123

6.32.1.5	REFSENs requirements	123

6.32.1.6	OOB blocking exception requirements	124

6.32.2	 Specific for 2 bands UL CA	124

6.32.2.1	Maximum output power for inter-band CA	124

6.32.2.2	 UE co-existence	124

6.32.2.3	 REFSENS requirements	125

6.33	 CA_n14-n77		126

6.33.1	Common for 1 band UL and 2 bands UL CA	126

6.33.1.1 Operating bands for CA	126

6.33.1.2	Channel bandwidths per operating band for CA	126

6.33.1.3	Co-existence studies	126

6.33.1.4	∆TIB and ∆RIB values	127

6.33.1.5	REFSENs requirements	127

6.33.1.6	OOB blocking exception requirements	128

6.33.2	 Specific for 2 bands UL CA	128

6.33.2.1	Maximum output power for inter-band CA	128

6.33.2.2	 UE co-existence	128

6.33.2.3	 REFSENS requirements	129

6.34	 CA_n30-n77		130

6.34.1	Common for 1 band UL and 2 bands UL CA	130

6.34.1.1 Operating bands for CA	130

6.34.1.2	Channel bandwidths per operating band for CA	130

6.34.1.3	Co-existence studies	130

6.34.1.4	∆TIB and ∆RIB values	130

6.34.1.5	REFSENs requirements	131

6.34.1.6	OOB blocking exception requirements	131

6.34.2	 Specific for 2 bands UL CA	131

6.34.2.1	Maximum output power for inter-band CA	131

6.34.2.2	 UE co-existence	131

6.34.2.3	 REFSENS requirements	132

6.35	 CA_n41-n66		133

6.35.1	Common for 1 band UL and 2 bands UL CA	133

6.35.1.1 Operating bands for CA	133

6.35.1.2	Channel bandwidths per operating band for CA	133

6.35.1.3	Co-existence studies	133

6.35.1.4	∆TIB and ∆RIB values	133

6.35.1.5	REFSENs requirements	134

6.35.1.6	OOB blocking exception requirements	134

6.35.2	 Specific for 2 bands UL CA	134

6.35.2.1	Maximum output power for inter-band CA	134

6.35.2.2	 UE co-existence	134

6.35.2.3	 REFSENS requirements	134

6.36.1.4	 ∆TIB and ∆RIB values	135

6.36.1.5 REFSENS requirements	136

6.36.1.6	OOB blocking exception requirements	136

6.37.1.4	 ∆TIB and ∆RIB values	137

6.37.1.5 REFSENS requirements	137

6.37.1.6	OOB blocking exception requirements	137

6.38.1.4	 ∆TIB and ∆RIB values	138

6.38.1.5    REFSENS requirements	138

6.38.1.6	OOB blocking exception requirements	139

6.38.2	Specific for 2 bands UL CA	139

6.38.2.1	Maximum output power for inter-band CA	139

6.38.2.2	UE co-existence studies	139

6.38.2.3	REFSENS requirements	140

6.39.1.4	 ∆TIB and ∆RIB values	141

6.39.1.5    REFSENS requirements	141

6.39.1.6	OOB blocking exception requirements	141

6.39.2	Specific for 2 bands UL CA	141

6.39.2.1	Maximum output power for inter-band CA	141

6.39.2.2	UE co-existence studies	142

6.39.2.3	REFSENS requirements	143

6.40.1.4	 ∆TIB and ∆RIB values	144

6.40.1.5     REFSENS requirements	145

6.40.1.6	OOB blocking exception requirements	145

6.40.2	Specific for 2 bands UL CA	145

6.40.2.1	Maximum output power for inter-band CA	145

6.40.2.2	UE co-existence studies	145

6.40.2.3	REFSENS requirements	146

6.41.1.4	 ∆TIB and ∆RIB values	148

6.41.1.5 REFSENS requirements	148

6.41.1.6	OOB blocking exception requirements	148

6.41.2	Specific for 2 bands UL CA	148

6.41.2.1	Maximum output power for inter-band CA	148

6.41.2.2	UE co-existence studies	149

6.41.2.3	REFSENS requirements	150

6.42.2	Specific for 2 bands UL CA	152

6.42.2.1	Maximum output power for inter-band CA	152

6.42.2.2	UE co-existence studies	152

6.42.2.3	REFSENS requirements	153

6.43.1.4	 ∆TIB and ∆RIB values	154

6.43.1.5    REFSENS requirements	154

6.43.1.6    OOB blocking exception requirements	154

6.43.2	Specific for 2 bands UL CA	154

6.43.2.1	Maximum output power for inter-band CA	154

6.43.2.2	UE co-existence studies	155

6.43.2.3	REFSENS requirements	156

6.44.1.4	 ∆TIB and ∆RIB values	157

6.44.1.5    REFSENS requirements	157

6.44.1.6	OOB blocking exception requirements	158

6.44.2	Specific for 2 bands UL CA	158

6.44.2.1	Maximum output power for inter-band CA	158

6.44.2.2	UE co-existence studies	158

6.44.2.3	REFSENS requirements	159

6.45.1.4	 ∆TIB and ∆RIB values	160

6.45.1.5    REFSENS requirements	160

6.45.1.6	OOB blocking exception requirements	161

6.45.2	Specific for 2 bands UL CA	161

6.45.2.1	Maximum output power for inter-band CA	161

6.45.2.2	UE co-existence studies	161

6.45.2.3	REFSENS requirements	163

6.46.1.4	 ∆TIB and ∆RIB values	164

6.46.1.5     REFSENS requirements	164

6.46.1.6	OOB blocking exception requirements	164

6.46.2	Specific for 2 bands UL CA	164

6.46.2.1	Maximum output power for inter-band CA	164

6.46.2.2	UE co-existence studies	164

6.46.2.3	REFSENS requirements	165

6.47.1	 Common for 1 band UL and 2 bands UL CA	165

6.47.1.5	 REFSEN requirements	167

6.47.2	 Specific for 2 bands UL CA	167

6.47.2.1	Maximum output power for inter-band CA	167

6.47.2.2	 UE co-existence studies	167

6.47.2.3	 REFSENS requirements	167

6.48	CA_n13-n77	168

6.48.1	Common for 1 band UL and 2 bands UL CA	168

6.48.1.1 Operating bands for CA	168

6.48.1.2	Channel bandwidths per operating band for CA	168

6.48.1.3	Co-existence studies	168

6.48.1.4	∆TIB and ∆RIB values	169

6.48.1.5	REFSENs requirements	169

6.48.1.6	OOB blocking exception requirements	170

6.48.2	 Specific for 2 bands UL CA	170

6.48.2.1	Maximum output power for inter-band CA	170

6.48.2.2	 UE co-existence	170

6.48.2.3	 REFSENS requirements	171

6.49	CA_n2-n7		172

6.49.1	Common for 1 band UL and 2 bands UL CA	172

6.49.1.1 Operating bands for CA	172

6.49.1.2	Channel bandwidths per operating band for CA	172

6.49.1.3	Co-existence studies	172

6.49.1.4	∆TIB and ∆RIB values	173

6.49.1.5	REFSENs requirements	173

6.49.1.6	OOB blocking exception requirements	173

6.49.2	 Specific for 2 bands UL CA	173

6.49.2.1	Maximum output power for inter-band CA	173

6.49.2.2	 UE co-existence	173

6.49.2.3	 REFSENS requirements	175

6.50	CA_n24-n41		175

6.50.1	 Common for 1 band UL and 2 bands UL CA	175

6.50.1.1	 Operating bands for CA	175

6.50.1.2	 Channel bandwidths per operating band for CA	175

6.50.1.3	 Co-existence studies	175

6.50.1.4	 ∆TIB and ∆RIB values	176

6.50.1.5	 REFSENS requirements	177

6.50.1.6	 OOB blocking exception requirements	177

6.50.2	 Specific for 2 bands UL CA	177

6.50.2.1	Maximum output power for inter-band CA	177

6.50.2.2	 UE co-existence studies	177

6.50.2.3	 REFSENS requirements	178

6.51	CA_n24-n48		178

6.51.1.1	 Operating bands for CA	178

6.51.1.2	 Channel bandwidths per operating band for CA	179

6.51.1.3	 Co-existence studies	179

6.51.1.4	 ∆TIB and ∆RIB values	180

6.51.1.5	    REFSENS requirements	180

6.51.1.6	   OOB blocking exception requirements	180

6.51.2	 Specific for 2 bands UL CA	180

6.51.2.1	   Maximum output power for inter-band CA	180

6.51.2.2	   UE co-existence studies	181

6.51.2.3	   REFSENS requirements	182

6.52	CA_n24-n77		182

6.52.1.1	 Operating bands for CA	182

6.52.1.2	 Channel bandwidths per operating band for CA	182

6.52.1.3	 Co-existence studies	183

6.52.1.4	 ∆TIB and ∆RIB values	183

6.52.1.5	 REFSENS requirements	184

6.52.1.6	 OOB blocking exception requirements	184

6.52.2	 Specific for 2 bands UL CA	185

6.52.2.1	Maximum output power for inter-band CA	185

6.52.2.2	 UE co-existence studies	185

6.52.2.3	 REFSENS requirements	186

6.53	CA_n28-n71		186

6.53.1	Common for 1 band UL and 2 bands UL CA	186

6.53.1.1 Operating bands for CA	186

6.53.1.2	Channel bandwidths per operating band for CA	187

6.53.1.3	UE Co-existence studies	187

6.53.1.4	∆TIB and ∆RIB values	187

6.53.1.5	REFSENs requirements	188

6.53.1.6	OOB blocking exception requirements	188

6.54	CA_n2-n30		188

6.54.1	Common for 1 band UL and 2 bands UL CA	188

6.54.1.1	 Operating bands for CA	188

6.54.1.2	 Channel bandwidths per operating band for CA	188

6.54.1.3	 UE co-existence studies	188

6.54.1.4	 ∆TIB and ∆RIB values	189

6.54.1.5 	 REFSENS requirements	189

6.54.1.6 	 OOB blocking exception requirements	189

6.54.2	 Specific for 2 bands UL CA	189

6.54.2.1	Maximum output power for inter-band CA	189

6.54.2.2	 UE co-existence studies	189

6.54.2.3	 REFSENS requirements	190

6.55	CA_n5-n30		191

6.55.1	Common for 1 band UL and 2 bands UL CA	191

6.55.1.1	 Operating bands for CA	191

6.55.1.2	 Channel bandwidths per operating band for CA	191

6.55.1.3	 UE co-existence studies	191

6.55.1.4	 ∆TIB and ∆RIB values	191

6.55.1.5 	 REFSENS requirements	192

6.55.1.6 	 OOB blocking exception requirements	192

6.55.2	 Specific for 2 bands UL CA	192

6.55.2.1	Maximum output power for inter-band CA	192

6.55.2.2	 UE co-existence studies	192

6.55.2.3	 REFSENS requirements	193

6.56	CA_n30-n66		194

6.56.1	Common for 1 band UL and 2 bands UL CA	194

6.56.1.1	 Operating bands for CA	194

6.56.1.2	 Channel bandwidths per operating band for CA	194

6.56.1.3	 UE co-existence studies	194

6.56.1.4	 ∆TIB and ∆RIB values	194

6.56.1.5 	 REFSENS requirements	195

6.56.1.6 	 OOB blocking exception requirements	195

6.56.2	 Specific for 2 bands UL CA	195

6.56.2.1	Maximum output power for inter-band CA	195

6.56.2.2	 UE co-existence studies	195

6.56.2.3	 REFSENS requirements	196

6.57	CA_n48-n53		197

6.57.1	Common for 1 band UL and 2 bands UL CA	197

6.57.1.1	 Operating bands for CA	197

6.57.1.2	 Channel bandwidths per operating band for CA	197

6.57.1.3	 UE co-existence studies	197

6.57.1.4	 ∆TIB and ∆RIB values	198

6.57.1.5 	 REFSENS requirements	198

6.57.1.6 	 OOB blocking exception requirements	198

6.58	 CA_n2-n12		198

6.58.1	Common for 1 band UL and 2 bands UL CA	198

6.58.1.1 Operating bands for CA	198

6.58.1.2	Channel bandwidths per operating band for CA	198

6.58.1.3	Co-existence studies	199

6.58.1.4	∆TIB and ∆RIB values	199

6.58.1.5	REFSENS requirements	199

6.58.1.6 	 OOB blocking exception requirements	199

6.58.2	 Specific for 2 bands UL CA	200

6.58.2.1	Maximum output power for inter-band CA	200

6.58.2.2	 UE co-existence	200

6.58.2.3	 REFSENS requirements	201

6.59	 CA_n2-n14		201

6.59.1	Common for 1 band UL and 2 bands UL CA	201

6.59.1.1 Operating bands for CA	201

6.59.1.2	Channel bandwidths per operating band for CA	201

6.59.1.3	Co-existence studies	201

6.59.1.4	∆TIB and ∆RIB values	202

6.59.1.5	REFSENS requirements	202

6.59.1.6 	 OOB blocking exception requirements	202

6.59.2	 Specific for 2 bands UL CA	203

6.59.2.1	Maximum output power for inter-band CA	203

6.59.2.2	 UE co-existence	203

6.59.2.3	 REFSENS requirements	204

6.60	 CA_n12-n30		204

6.60.1	Common for 1 band UL and 2 bands UL CA	204

6.60.1.1 Operating bands for CA	204

6.60.1.2	Channel bandwidths per operating band for CA	204

6.60.1.3	Co-existence studies	204

6.60.1.4	∆TIB and ∆RIB values	205

6.60.1.5	REFSENS requirements	205

6.60.1.6 	 OOB blocking exception requirements	205

6.60.2	 Specific for 2 bands UL CA	206

6.60.2.1	Maximum output power for inter-band CA	206

6.60.2.2	 UE co-existence	206

6.60.2.3	 REFSENS requirements	207

6.61	 CA_n12-n66		207

6.61.1	Common for 1 band UL and 2 bands UL CA	207

6.61.1.1 Operating bands for CA	207

6.61.1.2	Channel bandwidths per operating band for CA	207

6.61.1.3	Co-existence studies	207

6.61.1.4	∆TIB and ∆RIB values	208

6.61.1.5	REFSENS requirements	208

6.61.1.6 	 OOB blocking exception requirements	209

6.61.2	 Specific for 2 bands UL CA	209

6.61.2.1	Maximum output power for inter-band CA	209

6.61.2.2	 UE co-existence	209

6.61.2.3	 REFSENS requirements	211

6.62	 CA_n14-n30		211

6.62.1	Common for 1 band UL and 2 bands UL CA	211

6.62.1.1 Operating bands for CA	211

6.62.1.2	Channel bandwidths per operating band for CA	211

6.62.1.3	Co-existence studies	211

6.62.1.4	∆TIB and ∆RIB values	212

6.62.1.5	REFSENS requirements	212

6.62.1.6 	 OOB blocking exception requirements	212

6.62.2	 Specific for 2 bands UL CA	212

6.62.2.1	Maximum output power for inter-band CA	212

6.62.2.2	 UE co-existence	212

6.62.2.3	 REFSENS requirements	214

6.63	 CA_n14-n66		214

6.63.1	Common for 1 band UL and 2 bands UL CA	214

6.63.1.1 Operating bands for CA	214

6.63.1.2	Channel bandwidths per operating band for CA	214

6.63.1.3	Co-existence studies	214

6.63.1.4	∆TIB and ∆RIB values	215

6.63.1.5	REFSENS requirements	215

6.63.1.6 	 OOB blocking exception requirements	215

6.63.2	 Specific for 2 bands UL CA	215

6.63.2.1	Maximum output power for inter-band CA	215

6.63.2.2	 UE co-existence	215

6.63.2.3	 REFSENS requirements	217

6.64	CA_n28-n74		217

6.64.1	Common for 1 band UL and 2 bands UL CA	217

6.64.1.1 Operating bands for CA	217

6.64.1.2	Channel bandwidths per operating band for CA	217

6.64.1.3	UE Co-existence studies	217

6.64.1.4	∆TIB and ∆RIB values	218

6.64.1.5	REFSENs requirements	218

6.64.1.6	OOB blocking exception requirements	218

6.64.2	Specific for 2 bands UL CA	219

6.64.2.1	Maximum output power for inter-band CA	219

6.64.2.2	UE co-existence studies	219

6.64.2.3	REFSENS requirements	220

6.65	CA_n74-n77		221

6.65.1	Common for 1 band UL and 2 bands UL CA	221

6.65.1.1 Operating bands for CA	221

6.65.1.2	Channel bandwidths per operating band for CA	221

6.65.1.3	UE Co-existence studies	221

6.65.1.4	∆TIB and ∆RIB values	222

6.65.1.5	REFSENs requirements	222

6.65.1.6	OOB blocking exception requirements	222

6.65.2	Specific for 2 bands UL CA	222

6.65.2.1	Maximum output power for inter-band CA	222

6.65.2.2	UE co-existence studies	222

6.65.2.3	REFSENS requirements	224

6.66	CA_n3-n34		224

6.66.1	 Common for 1 band UL and 2 bands UL CA	224

6.66.1.5	 REFSEN requirements	225

6.66.1.6	OOB blocking exception requirements	225

6.66.2	 Specific for 2 bands UL CA	226

6.66.2.1	Maximum output power for inter-band CA	226

6.66.2.2	 UE co-existence studies	226

6.66.2.3	 REFSENS requirements	227

6.67	CA_n8-n34		227

6.67.1	 Common for 1 band UL and 2 bands UL CA	227

6.67.1.5	 REFSEN requirements	228

6.67.2	 Specific for 2 bands UL CA	229

6.67.2.1	Maximum output power for inter-band CA	229

6.67.2.2	 UE co-existence studies	229

6.67.2.3	 REFSENS requirements	230

6.66	CA_n5-n12		230

6.66.1	 Common for 1 band UL and 2 bands UL CA	230

6.66.1.5	 REFSEN requirements	231

6.66.1.6	OOB blocking exception requirements	231

6.66.2	 Specific for 2 bands UL CA	232

6.66.2.1	Maximum output power for inter-band CA	233

6.66.2.2	 UE co-existence studies	233

6.66.2.3	 REFSENS requirements	234

6.67	CA_n5-n14		235

6.67.1	 Common for 1 band UL and 2 bands UL CA	235

6.67.1.5	 REFSEN requirements	236

6.67.2	 Specific for 2 bands UL CA	237

6.67.2.1	Maximum output power for inter-band CA	238

6.67.2.2	 UE co-existence studies	240

6.67.2.3	 REFSENS requirements	241

7	Both bands within FR2 Carrier Aggregation: Specific Band Combination Part	230

8	FR1+FR2 Carrier Aggregation: Specific Band Combination Part	230

8.1	CA_n41-n257		230

8.1.1	Common for 1 band UL and 2 bands UL CA	230

8.1.1.1	Operating bands for CA	230

8.1.1.2	Channel bandwidths per operating band for CA	231

8.1.1.3	∆TIB and ∆RIB values	231

8.1.1.4	REFSENS requirements	231

8.2	CA_n3-n258		232

8.2.1	Common for 1 band UL and 2 bands DL CA	232

8.2.1.1	 Operating bands for CA	232

8.2.1.2	 Channel bandwidths per operating band for CA	232

8.2.1.3	 Co-existence studies	233

8.2.1.4	 ∆TIB and ∆RIB values	234

8.2.1.5	 REFSENS requirements	234

8.2.2	Specific for 2 bands UL CA	234

8.2.2.1	 UE co-existence studies	234

8.2.2.2	 REFSENS requirements	235

9	2 bands Dual Connectivity: Specific Band Combination Part	235

9.X	DC_nX-nY		235

9.x.1	Operating bands for DC_nX-nY	235

9.x.2   Configurations for DC_nX-nY	235

9.x.3   Maximum output power for NR-DC	236

9.1	DC_n5-n48		236

9.1.1	Operating bands for DC_n5-n48	236

9.1.2   Configurations for DC_n5-n48	236

9.2	DC_n41-n257		236

9.2.1	Operating bands for DC_n41-n257	236

9.2.2   Configurations for DC_n41-n257	236

Annex A: Change history	237

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for power class 3 NR inter-band CA and DC for 2 bands DL with up to 2 bands UL under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address NR inter-band CA and DC for 2 bands DL with up to 2 bands UL for the Rel-17 band combinations.

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BS	Base Station

CA	Carrier Aggregation

DC	Dual Connectivity

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

4	Background

The present document is a technical report for NR inter-band CA and DC for 2 bands DL with up to 2 bands UL under Rel-17 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	NR Inter-Band Combination: General Part

The general issue for NR inter-band combination including:

1: common issues for both 1 band UL and 2 bands UL NR CA, including the impact of UL/DL harmonic/ harmonic mixing associated with REFSEN,  delta Tib and delta Rib, and OOB blocking exception, etc.

2: 2 bands UL NR CA specific issues, including MSD caused by IMD issue, etc.

3: No new issue for inter-band NR DC combination, the 2 bands UL NR CA specific issues shall be re-used.

6	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part

## 6.1		CA_n5-n25

6.1.1	Common for 1 band UL and 2 bands UL CA

6.1.1.1 Operating bands for CA

Table 6.1.1-1: CA band combination of band n5 + n25

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n25 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |

6.1.1.2	Channel bandwidths per operating band for CA

Table 8.x.2-1: Supported bandwidths per CA band combination of band n2+n78

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n5A-n25A | CA_n5A-n25A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| CA_n5A-n25(2A) | CA_n5A-n25A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.1.1.3	Co-existence studies

Table 6.1.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n25.

Table 6.1.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |

Based on above table, there is no harmonic issue for CA_n5-n25.

Table 6.1.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 |

Based on above table, there is no harmonics mixing issue for CA_n5-n25.

6.1.1.4	∆TIB and ∆RIB values

For CA_n5-n25, the TIB,c and RIB values are given in the tables below.

Table 6.1.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n25 | n5 | 0.3 |
|  | n25 | 0.3 |

Table 6.1.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n5-n25 | n5 | 0 |
|  | n25 | 0 |

6.1.1.5	REFSENs requirements

There is no harmonic issue for the CA combination.

6.1.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

6.1.2		Specific for 2 bands UL CA

6.1.2.1	Maximum output power for inter-band CA

Table 6.1.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n25A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.1.2.2		UE co-existence

Table 6.1.2.2-1 gives IMD interference analysis for CA_ n5-n25 with 2 ULs.

Table 6.1.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1850 | 1915 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1001 | 1091 | 2674 | 2764 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 267 | 152 | 2851 | 3006 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3498 | 3613 | 4524 | 4679 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 557 | 697 | 4701 | 4921 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4322 | 4462 | 6374 | 6569 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 2002 | 2182 | 5348 | 5528 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1381 | 1546 | 6551 | 6836 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 5146 | 5311 | 8224 | 8509 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 1358 | 1153 | 3852 | 4097 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 7198 | 7443 | 6172 | 6377 |

Based on the table 6.x.2.2-1, there is no IMD issue for CA_n5-n25.

Table 6.1.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.1.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n25 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.1.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.2.3		REFSENS requirements

There is no MSD issue for the CA combination.

## 6.2		CA_n71-n78

6.2.1	Common for 1 band UL and 2 bands UL CA

6.2.1.1 Operating bands for CA

Table 6.2.1-1: CA band combination of band n38 + n66

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n71-n78 | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.2.1.2	Channel bandwidths per operating band for CA

Table 8.x.2-1: Supported bandwidths per CA band combination of band n2+n78

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n71A-n78A | CA_n71A-n78A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n71A-n78(2A) | CA_n71A-n78A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2.1.3	Co-existence studies

DC_71A_n78A is defined in Rel-16. Co-existence studies for DC_71A_n78A can be reused for CA_n71-n78.

6.2.1.4	∆TIB and ∆RIB values

For CA_n71-n78, the TIB,c and RIB values are given in the tables below.

Table 6.2.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n71-n78 | n71 | 0.5 |
|  | n78 | 0.8 |

Table 6.2.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n71-n78 | n71 | 0.2 |
|  | n78 | 0.5 |

6.2.1.5	REFSENs requirements

There is no harmonic issue for the CA combination.

6.2.1.6	OOB blocking exception requirements

Table 6.2.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n71-n78

6.2.2		Specific for 2 bands UL CA

6.2.2.1	Maximum output power for inter-band CA

Table 6.2.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n71A-n78A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.2.2.2		UE co-existence

Table 6.2.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.2.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n71-n78 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.2.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.2.2.3		REFSENS requirements

DC_71A_n78A is defined in existing Rel-16 TS 38.101-3. Two-tone 5th order IMD products may fall into the own Rx Band of Band n71. The MSD value can be reused from DC_71A_n78A.

Table 6.2.2.3: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71A-n78A CA_n71A-n78(2A) | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |

6.3		CA_n7-n66

6.3.1	Common for 1 band UL and 2 bands UL CA

6.3.1.1	Operating bands for CA

Table 6.3.1.1-1: CA band combination of band n7+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.3.1.2	Channel bandwidths per operating band for CA

Table 6.3.1.2-1: Supported bandwidths per CA band combination of band n7+n66

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n7A-n66A | CA_n7A-n66A | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 1 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| CA_n7A-n66(2A) | CA_n7A-n66A | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n66 | CA_n7A-n66A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| CA_n7(2A)-n66(2A) | CA_n7A-n66A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.3.1.3	UE co-existence studies

Table 6.3.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n7-n66.

Table 6.3.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |  |  |

Table 6.3.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6660 |  |  |

Based on above table, there is no harmonic issue for the band combination of n7 and n66.

Based on above table, there is no harmonic mixing issue for the band combination of n7 and n66.

6.3.1.4	∆TIB and ∆RIB values

For CA_n7-n66 , the TIB,c and RIB,c values are given in the tables below.

Table 6.3.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |

Table 6.1.x.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |

6.3.1.5	REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.3.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.3.2	Specific for 2 bands UL CA

6.3.2.1	Maximum output power for inter-band CA

Table 6.3.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n66A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.3.2.2	UE co-existence studies

Table 6.3.2.1-1 lists Band n7 +Band n66 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.3.2.2-1: Band n7 and Band n66 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 2500 | 2570 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 720 | 860 | 4210 | 4350 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 850 | 1060 | 3220 | 3430 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5920 | 6130 | 6710 | 6920 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 2560 | 2840 | 5720 | 6000 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7630 | 7910 | 9210 | 9490 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -1720 | -1440 | 8420 | 8700 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -8570 | -8220 | -4620 | -4270 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 11710 | 12060 | 9340 | 9690 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -4290 | -3940 | -340 | 10 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10920 | 11270 | 10130 | 10480 |

Based on the table above, the 4th order IMD may fall into Rx frequencies of band n7.

Table 6.3.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.3.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7-n66 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 5 | 15, 21 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

6.3.2.3	REFSENS requirements

Table 6.3.2.3-1 lists the MSD required due to the 4th IMD for the dual uplink configuration The MSD value is reused from E-UTRA CA_4-7.

Table 6.3.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7A-n66A | n7 | 2535 | 5 | 25 | 2655 | 15 | FDD | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | TDD | N/A |

6.4		CA_n25-38

6.4.1		Common for 1 band UL and 2 bands UL CA

6.4.1.2	Channel bandwidths per operating band for CA

Table 6.4.1.1-1: CA band combination of band n25+n38

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

6.4.1.2	Channel bandwidths per operating band for CA

Table 6.4.1.2-1: Supported bandwidths per CA band combination of band n25+n38

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n25A-n38A | CA_n25A-n38A | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| CA_n25(2A)-n38A | CA_n25A-n38A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.4.1.3	UE co-existence studies

Table 6.4.1.3-1 summarizes frequency ranges where harmonics occur for CA_n25-n38.

Table 6.4.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |  |  |
| n38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 |  |  |

Based on above table, there is no harmonic issue for the band combination of n25 and n38.

Table 6.4.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n25-n38.

Table 6.4.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |  |  |
| n38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n25 and n38.

6.4.1.4	∆TIB and ∆RIB values

For CA_n25-n38 , the ∆TIB,c and ∆RIB,c values are given according to the general framework for the high-high band combination.

Table 6.4.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n38 | n25 | 0.5 |
|  | n38 | 0.5 |

Table 6.4.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n38 | n25 | 0 |
|  | n38 | 0 |

6.4.1.5	REFSENS requirements

Since the UE RF architecture for CA_n25-n38 is similar to CA_n25-n41, MSD due to cross band isolation defined for CA_n25-n41 is assumed.

Table 6.4.2.3-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n38 | n25 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |

Table 6.4.2.3-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n38 | n25 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |

6.4.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.4.2	Specific for 2 bands UL CA

6.4.2.1	Maximum output power for inter-band CA

Table 6.4.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_25A-n38A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.4.2.2	UE co-existence studies

Table 6.4.2.2-1 lists Band n25 +Band n38 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.4.2.2-1: Band n25 and Band n38 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 2570 | 2620 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 655 | 770 | 4420 | 4535 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1080 | 1260 | 3225 | 3390 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6270 | 6450 | 6990 | 7155 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 2930 | 3175 | 5795 | 6010 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8120 | 8365 | 9560 | 9775 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -1540 | -1310 | 8840 | 9070 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -8630 | -8365 | -5090 | -4780 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 12130 | 12395 | 9970 | 10280 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -4160 | -3880 | -605 | -310 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 11410 | 11690 | 10690 | 10985 |

Based on the table above, no IMD issue is identified for this band combination.

Table 6.4.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.4.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n25-n38 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 27, 29, 30, 42, 48, 66, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, 48 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 21 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

6.4.2.3	REFSENS requirements

There is no specific REFSENS requirement for 2 band UL.

6.5		CA_n1-n77

6.5.1		Common for 1 band UL and 2 bands UL CA

6.5.1.1			Operating bands for CA

Table 6.5.1.1-1: CA band combination of band n1+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.5.1.2			Channel bandwidths per operating band for CA

Table 6.5.1.2-1: Supported bandwidths per CA band combination of band n1+n77

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n77A | CA_n1A-n77A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.5.1.3			UE co-existence studies

This section is skipped since CA_n1-n77 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been studied in TR 38.716-02-00.

6.5.1.4			∆TIB and ∆RIB values

For CA_n1-n77 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The same values are shown as what are specified in TS 38.101-1.

Table 6.5.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n77 | n1 | 0.6 |
|  | n77 | 0.8 |

Table 6.5.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n77 | n1 | 0.2 |
|  | n77 | 0.5 |

6.5.1.5			REFSENS requirements

This section is skipped since CA_n1-n77 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been studied in TR 38.716-02-00.

6.5.1.6			OOB blocking exception requirements

This section is skipped since CA_n1-n77 without 2UL CA has been already specified.

6.5.2		Specific for 2 bands UL CA

6.5.2.1			Maximum output power for inter-band CA

Table 6.5.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.5.2.2			UE co-existence studies

Table 6.5.2.2-1 lists Band n1 +Band n77 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.5.2.2-1: Band n1 and Band n77 UL IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1320 – 2280 |  |  | 5220 – 6180 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 360 – 660 |  |  | 4620 – 6480 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1920 – 1980 |  |  | 3300 – 4200 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1820 – 2080 |  |  | 3280 – 4220 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1560 – 2640 |  |  | 7920 – 10680 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2640 – 4560 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 – 10140 |  |  | 11820 – 14580 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10440 – 12360 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220 – 14880 |  |  | 3480 – 4620 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940 – 8760 |  |  | 660 – 2640 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 – 18780 |  |  | 10980 – 12120 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 – 16560 |  |  | 12360 – 14340 |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |  |

Based on above Table , it can be seen that

- 2nd order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 46, 47, 50, 51, 65, 66, 70, 74, 75, 76, n91, n92, n93, n94

- 3rd order IMD products may fall into Rx frequencies of bands  2, 3, 9, 22, 25, 31, 33, 34, 35, 36, 37, 39, 42, 43, 46, 47, 48, 49, 52, 70, 71, 72, 73, n77, n78, n79, n87, n88

- 4th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 22, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 48, 49, 52, 53, 65, 66, 69, 70, n77, n78, n79, n90

- 5th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 48, 49, 50, 51, 53, 65, 66, 67, 68, 69, 70, 74, 75, 76, n77, n78, n79, 85, n90, n91, n92 ,n93, n94

Self-interference to own Rx frequencies are shown as red.

Table 6.5.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.5.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n77 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

6.5.2.3			REFSENS requirements

MSD for harmonic issues is already covered by the study for fallback combinations of 1UL/2DL. Therefore, here we focus on MSD for IMD issues.

Table 6.5.2.3-1 lists the MSD required due to 2nd and 4th  IMD for the dual uplink configuration. We use same MSD as DC_1_n77 specified in TS 38.101-3. Note that no MSD for 3rd, 4th and 5th IMD to n77 is specified in TS 38.101-3, and then we follow the result.

Table 6.5.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1A-n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24 |
|  |  |  |  |  |  | 32.5 5 |  |  |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
| CA_n1A-n77A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |

6.6	CA_n77-n79

6.6.1		Common for 1 band UL and 2 bands UL CA

6.6.1.1			Operating bands for CA

Table 6.6.1.1-1: CA band combination of band n77+n79

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.6.1.2			Channel bandwidths per operating band for CA

Table 6.6.1.2-1: Supported bandwidths per CA band combination of band n77+n79

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n77A-n79A | CA_n77A-n79A | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.6.1.3			UE co-existence studies

This section is skipped since CA_n77-n79 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been discussed in TR 37.865-01-01, where n77 and n79 is assumed as synchronous operation.

6.6.1.4			∆TIB and ∆RIB values

For CA_n77-n79 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The same values are shown as what are specified in TS 38.101-1.

Table 6.6.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n77-n79 | n77 | 0.5 |
|  | n79 | 0.5 |

Table 6.6.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n77-n79 | n77 | 0 |
|  | n79 | 0 |

6.6.1.5			REFSENS requirements

This section is skipped since CA_n77-n79 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been discussed in TR 37.865-01-01, where n77 and n79 is assumed as synchronous operation.

6.6.1.6			OOB blocking exception requirements

This section is skipped since CA_n77-n79 without 2UL CA has been already specified.

6.6.2		Specific for 2 bands UL CA

6.6.2.1			Maximum output power for inter-band CA

Table 6.6.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n77A-n79A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.6.2.2			UE co-existence studies

Table 6.6.2.2-1 lists Band n77 +Band n79 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.6.2.2-1: Band n77 and Band n79 UL IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 200 – 1700 |  |  | 7700 – 9200 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1600 – 4000 |  |  | 4600 – 6700 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 11000 – 13400 |  |  | 12100 – 14200 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 3200 – 4300 |  |  | 4300 – 5100 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4900 – 8200 |  |  | 9000 – 11700 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 400 – 3400 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 14300 – 17600 |  |  | 16500 – 19200 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 15400 – 18400 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13400 – 16700 |  |  | 8200 – 12400 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4800 – 8400 |  |  | 100 – 3800 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20900 – 24200 |  |  | 17600 – 21800 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19800 – 23400 |  |  | 18700 – 22600 |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |  |

Based on above Table, it can be seen that

- 2nd order IMD products may fall into Rx frequencies of bands  5, 6, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 32, 44, 45, 50, 51, 67, 68, 71, 72, 73, 74, 75, 76, 85, n91, n92, n93, n94

- 3rd order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 22, 23, 25, 30, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 46, 47, 48, 49, 52, 53, 65, 66, 69, 70, n77, n78, n79, n90

- 4th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 44, 45, 46, 47, 50, 51, 52, 53, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, n77, n78, n79, 85, 87, 88, n90, n91, n92, n93, n94

- 5th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, n77, n78, n79, 85, 87, 88, n90, n91, n92, n93, n94

Table 6.6.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.6.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n77-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

6.6.2.3			REFSENS requirements

Considering both n77 and n79 are TDD bands and synchronous operation for CA_n77-n79 is assumed, no MSD needed for dual UL of CA_n77-n79.

6.7	CA_n78-n79

6.7.1		Common for 1 band UL and 2 bands UL CA

6.7.1.1		Operating bands for CA

Table 6.7.1.1-1: CA band combination of band n78+n79

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.7.1.2		Channel bandwidths per operating band for CA

Table 6.7.1.2-1: Supported bandwidths per CA band combination of band n78+n79

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz |  |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | 25 MHz |  | 30 MHz | 40 MHz |  |  | 50 MHz |  | 60 MHz |  | 70 MHz |  | 80 MHz |  | 90 MHz |  | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78A-n79AX | CA_n78A-n79AY | n78 | 15 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |
|  |  | n78 | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |
| CA_ n78(2A)-n79A | CA_n78A-n79A | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |
| NOTE X: Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE Y: Simultaneous Rx/Tx capability does not apply to uplink CA transmission, and only apply to single uplink transmission. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.7.1.3		UE co-existence studies

This section is skipped since CA_n78-n79 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been discussed in TR 37.865-01-01.

Furthermore, ΔTIB and ΔRIB and MSD requirements have been specified for asynchronous operation in TS 38.101-1.

6.7.1.4		∆TIB and ∆RIB values

For CA_n77-n79 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The same values are shown as what are specified in TS 38.101-1.

Table 6.7.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n78-n79 | n78 | 0.5 |
|  |  | 1.58 |
|  | n79 | 0.5 |
|  |  | 1.58 |
| NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

Table 6.7.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n78-n79 | n78 | 0 |
|  | n79 | 0 |

6.7.1.5		REFSENS requirements

The MSD due to cross band isolation are specified below for n78 25MHz and 30MHz.

Table 6.7.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n781 | n79 |  |  |  |  |  |  | 2 | 2 | 2 |  | 2 |  | 2 |
| n79 | n781 |  | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |  | 2.6 | 2.6 | 2.6 |
| NOTE 1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n783 | n79 | 30 |  |  |  |  |  | 270 | 270 | 270 | 270 |  | 270 |  | 270 |
| n79 | n783 | 30 |  | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  | 270 | 270 | 270 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. NOTE 2: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

This section is skipped since CA_n78-n79 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been discussed in TR 37.865-01-01.

Furthermore, ΔTIB and ΔRIB and MSD requirements have been specified for asynchronous operation in TS 38.101-1.

6.7.1.6		OOB blocking exception requirements

This section is skipped since CA_n78-n79 without 2UL CA has been already specified.

6.7.2		Specific for 2 bands UL CA

6.7.2.1		Maximum output power for inter-band CA

Table 6.7.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n78A-n79A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.7.2.2		UE co-existence studies

Table 6.7.2.2-1 lists Band n78 +Band n79 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.7.2.2-1: Band n78 and Band n79 UL IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 600 – 1700 |  |  | 7700 – 8800 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1600 – 3200 |  |  | 5000 – 6700 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 11000 – 12600 |  |  | 12100 – 13800 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 3200 – 3900 |  |  | 4300 – 5100 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4900 – 7000 |  |  | 9400 – 11700 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1200 – 3400 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 14300 – 16400 |  |  | 16500 – 18800 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 15400 – 17600 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13800 – 16700 |  |  | 8200 – 10800 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5600 – 8400 |  |  | 100 – 2600 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20900 – 23800 |  |  | 17600 – 20200 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19800 – 22600 |  |  | 18700 – 21400 |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |  |

Based on above Table, it can be seen that

- 2nd order IMD products may fall into Rx frequencies of bands  5, 6, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 32, 44, 45, 50, 51, 67, 68, 71, 74, 75, 76, 85, n91, n92, n93, n94

- 3rd order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 22, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 46, 47, 48, 49, 52, 53, 65, 66, 69, 70, n77, n78, n79, n90

- 4th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 45, 46, 47, 50, 51, 52, 53, 65, 66, 69, 70, 74, 75, 76, n77, n78, n79, n90, n91, n92, n93, n94

- 5th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 44, 45, 46, 47, 50, 51, 53, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, 85, n87, n88, n90, n91, n92, n93, n94

Table 6.7.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n78-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

6.7.2.3		REFSENS requirements

For harmonic, MSD studies can be omitted since MSD studies have been already conducted in 1UL/2DL NR CA fallback combinations.

For IMD, Considering both n78 and n79 are TDD bands and IMD occurs when UE transmit in both n77 and n79 and receive in neither n77 and n79,  no MSD are needed for dual UL of CA_n78-n79.

6.8	CA_n25-n48

6.8.1	Common for 1 band UL and 2 bands UL CA

6.8.1.1		Operating bands for CA

Table 6.8.1.1-1: CA band combination of band n25 and n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

6.8.1.2		Channel bandwidths per operating band for CA

Table 6.8.1.2-1: Supported bandwidths per CA band combination of band n25 and n48

| NR CA configuration | UL | NR Band | SCS (kHz) |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | 25 MHz |  | 30 MHz |  | 40 MHz |  | 50 MHz |  | 60 MHz |  | 70 MHz |  | 80 MHz |  | 90 MHz |  | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n48A | CA_n25A-n48A | n25 | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |
|  |  | n25 |  |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 |  |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n25A-n48(2A) | CA_n25A-n48A | n25 | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 |  |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 |  |  | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n48C | CA_n25A-n48A | n25 | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.8.1.3		UE co-existence studies

Table 6.8.1.3-1 lists up to 7th harmonics for n25A-n48A which shows that there are 2nd harmonics issues.

Table 6.8.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 | 9250 | 9575 | 11100 | 11490 | 12950 | 13405 |
| n48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 | 21300 | 22200 | 24850 | 25900 |

Table 6.8.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.8.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7400 | 10650 | 11100 | 14200 | 14800 | 7400 |

6.8.1.4		∆TIB and ∆RIB values

For CA_n25-n48, the TIB,c and RIB,c values are derived from CA_2-48 and CA_n2-n48 and are given in the tables below.

Table 6.8.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n48 | n25 | 0.6 |
|  | n48 | 0.8 |

Table 6.8.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n48 | n25 | 0.2 |
|  | n48 | 0.5 |

6.8.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 6.8.1.3 there are 2nd harmonics issues from n25 UL into the n48 DL. MSD values are same as for CA_n2A-n48A and need to be inserted into the 38.101-1 tables below.

CA_n25-n48 need to be added in NOTE 1 of Table 7.3A.4-1.

Table 7.3A.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n25 | n481, 2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 | 14.812 | 14.312 | 13.812 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n48, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 12: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n25 | n48 | 25 | 50 | 50 | 50 |  |  | 50 | 50 | 50 | 50 | 50 | 50 |

6.8.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n25-n48 since it belongs to H-H configuration-

Table 6.8.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.8.2	Specific for 2 bands UL CA

6.8.2.1	Maximum output power for inter-band CA

Table 6.8.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_25A-n48A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.8.2.2	UE co-existence studies

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th, 5th, 6th and 7th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented below.

Table 6.8.2.2-1: Band n25 and Band n48 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1850 | 1915 | 3550 | 3700 |
| DL Frequency [MHz] | 1930 | 1995 | 3550 | 3700 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3830 | 7100 | 7400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5745 | 10650 | 11100 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7660 | 14200 | 14800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9575 | 17750 | 18500 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 11100 | 11490 | 21300 | 22200 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 12950 | 13405 | 24850 | 25900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1635 | 1850 | 5400 | 5615 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 0 | 280 | 5185 | 5550 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7250 | 7530 | 8950 | 9315 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1850 | 2195 | 8735 | 9250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3700 | 3270 | 10800 | 11230 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9100 | 9445 | 12500 | 13015 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12950 | 12285 | 4110 | 3700 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7400 | 6820 | 1355 | 1850 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16050 | 16715 | 10950 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14350 | 14930 | 12650 | 13145 |

Based on Table 6.8.2.2-1, there is 4th order IMD issue for CA_n25-n48

Table 6.8.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.8.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n25-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, , 50, 51, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, NR band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

6.8.2.3	REFSENS requirements

As seen in the coexistence study the 4th IMD might fall in own Rx band why MSD is needed for 2 band UL. Values from CA_n2A-n48A is reused due to similar frequency range.

Table 6.8.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n48 | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |

6.9	CA_n28-n79

6.9.1	Common for 1 band UL and 2 bands UL CA

6.9.1.1 Operating bands for CA

Table 6.9.1.1-1: CA band combination CA_n28A-n79A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n79 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.9.1.2	Channel bandwidths per operating band for CA

Table 6.9.1.2-1: Supported bandwidths per CA band combination CA_n28A-n79A

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n28A-n79A |  | CA_n28A-n79A | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.9.1.3	UE Co-existence studies

Table 6.9.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n28-n79.

Table 6.9.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |

Based on above table, there is no harmonic interference.

Table 6.9.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |

Based on above table, there is no harmonic mixing issue.

6.9.1.4	∆TIB and ∆RIB values

For CA_n28A-n79A, the TIB,c and RIB are given in the tables below.

Table 6.9.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n79 | n28 | 0.5 |
|  | n79 | 0.8 |

Table 6.9.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n28-n79 | n28 | 0.2 |
|  | n79 | 0.5 |

6.9.1.5	REFSENs requirements

There is no MSD exception requirement.

6.9.1.6	OOB blocking exception requirements

Since band n28 is a low band and n79 is a wide band, the OOBB exception is needed.

Table 6.9.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n28-n79

6.9.2		Specific for 2 bands UL CA

6.9.2.1	Maximum output power for inter-band CA

Table 6.9.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28A-n79A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.9.2.2		UE co-existence studies

Table 6.9.2.2-1 gives IMD interference analysis for CA_n28-n79 with 2 ULs.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 4400 | 5000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3652 | 4297 | 5103 | 5748 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3594 | 2904 | 8052 | 9297 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5806 | 6496 | 9503 | 10748 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2891 | 2156 | 12452 | 14297 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6509 | 7244 | 13903 | 15748 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 8594 | 7304 | 10206 | 11496 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 19297 | 16852 | 1408 | 2188 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 13594 | 11704 | 6556 | 7891 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18303 | 20748 | 7212 | 7992 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14606 | 16496 | 10909 | 12244 |

For band combination CA_n28-n79, no IMD interference will fall into Rx.

Table 6.9.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.9.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n28-n79 | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 10, 11 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 10: This requirement applies when the NR carrier is confined within 2545 - 2575 MHz or 2595 – 2645vMHz and the channel bandwidth is 10 or 20 MHz. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

6.9.2.3		REFSENS requirements

There is no additional MSD requirements for two UL CA_n28A-n79A

## 6.10		CA_n3-n77

### 6.10.1		Common for 1 band UL and 2 bands UL CA

#### 6.10.1.1		Operating bands for CA

Table 6.10.1.1-1: CA band combination of band n3+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 6.10.1.2		Channel bandwidths per operating band for CA

Table 6.10.1.2-1: Supported bandwidths per CA band combination of band n3+n77

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n3A-n77(2A) | CA_n77(2A) | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.10.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n3 + n77 have been already completed and captured into TR 37.865-01-01, there is no IMD issue for band n3 + n77.

The IMD5 of n77(2A) may fall into band n3 DL when the two UL carriers are at the same time assigned within the frequency range of 3300MHz-3400MHz and the frequency range of 4000MHz-4200MHz separately.

#### 6.10.1.4		∆TIB and ∆RIB values

For CA_n3-n77, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

#### 6.10.1.5		REFSENS requirements

Table 6.10.1.5-1 shows the MSD requirement for CA_n3-n77(2A) with UL CA_n77(2A).Table 6.10.1.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3A-n77(2A) | n3 | N/A | N/A | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). |  |  |  |  |  |  |  |  |

#### 6.10.1.6	    OOB blocking exception requirements

Table 6.10.1.6-1: CA band combination with exceptions allowed

CA band combination

No exception

## 6.11		CA_n3-n78

### 6.11.1		Common for 1 band UL and 2 bands UL CA

#### 6.11.1.1		Operating bands for CA

Table 6.11.1.1-1: CA band combination of band n3+n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

#### 6.11.1.2		Channel bandwidths per operating band for CA

Table 6.11.1.2-1: Supported bandwidths per CA band combination of band n3+n78

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n3A-n78(2A) | CA_n78(2A) | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.11.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n3 + n78 have been already completed and captured into TR 37.865-01-01, there is no IMD issue for band n3 + n78.

#### 6.11.1.4		∆TIB and ∆RIB values

For CA_n3-n78, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

#### 6.11.1.5		REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration..

#### 6.11.1.6	    OOB blocking exception requirements

Table 6.11.1.6-1: CA band combination with exceptions allowed

CA band combination

No exception

6.12		CA_n28-n77

6.12.1		Common for 1 band UL and 2 bands UL CA

6.12.1.1		Operating bands for CA

Table 6.12.1.1-1: CA band combination of band n28+n77

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n77 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.12.1.2		Channel bandwidths per operating band for CA

Table 6.12.1.2-1: Supported bandwidths per CA band combination of band n28+n77

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n77(2A) | CA_n77(2A)3 | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.12.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n28 + n78 have been already completed and captured into TR 38.716-02-00.

The IMD2 of n77(2A) may fall into band n28 DL when the two UL carriers are at the same time assigned within the frequency range of 3300MHz-3400MHz and the frequency range of 4000MHz-4200MHz separately.

6.12.1.4		∆TIB and ∆RIB values

For CA_n28-n77, the TIB,c and RIB,c values are already specified in TR38.716-02-00.

6.12.1.5		REFSENS requirements

Table 6.12.1.5-1 shows the MSD requirement for CA_n28-n77(2A) with UL CA_n77(2A).

Table 6.12.1.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28A-n77(2A) | n28 | N/A | N/A | N/A | N/A | N/A7 | FDD | IMD2 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| NOTE 7: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD2 result will fall in Rx frequency range of band n28. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A).. |

6.12.1.6	    OOB blocking exception requirements

Table 6.12.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n28-n77

6.13		CA_n28-n78

6.13.1		Common for 1 band UL and 2 bands UL CA

6.13.1.1		Operating bands for CA

Table 6.13.1.1-1: CA band combination of band n28+n78

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n78 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.13.1.2		Channel bandwidths per operating band for CA

Table 6.13.1.2-1: Supported bandwidths per CA band combination of band n28+n78

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n78(2A) | CA_n78(2A) | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.13.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n28 + n78 have been already completed and captured into TR 37.865-01-01, there is no IMD issue for band n28 + n78.

6.13.1.4		∆TIB and ∆RIB values

For CA_n28-n78, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

6.13.1.5		REFSENS requirements

No additional MSD requirement need to be defined for this band combs configuration.

#### 6.13.1.6	    OOB blocking exception requirements

Table 6.13.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n28-n78

## 6.14	CA_n41-n78

### 6.14.1		Common for 1 band UL and 2 bands UL CA

#### 6.14.1.1		Operating bands for CA

Table 6.14.1.1-1: CA band combination of band n41+n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

#### 6.14.1.2		Channel bandwidths per operating band for CA

Table 6.14.1.2-1: Supported bandwidths per CA band combination of band n41+n78

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n41A-n78(2A) |  | - | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.14.1.3		Co-existence studies

Table 6.14.1.3-1 summarizes frequency ranges where harmonics occur for CA_n41-n78.

Table 6.14.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8077 |  |  |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 |  |  |

Based on above table, there is no harmonic issue for the band combination of n41 and n78.

Table 6.14.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n41-n78.

Table 6.14.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8077 |  |  |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n41 and n78.

6.14.1.4		∆TIB and ∆RIB values

For CA_n41-n78 , the ∆TIB,c and ∆RIB,c values have been defined in TS 38.101-1.

#### 6.14.1.5		REFSENS requirements

The MSD requirements due to cross band isolation defined as Table 6.14.1.5-1 and Table 6.14.1.5-2:

Table 6.14.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | n78 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |
| n78 | n411 |  | 4.5 | 4.5 | 4.5 |  | 4.5 | 4.5 | 4.5 | 4.5 |  | 4.5 | 4.5 | 4.5 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.14.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n78 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n41 | 30 |  | 270 | 270 | 270 |  | 270 | 270 | 270 | 270 |  | 270 | 270 | 270 |

#### 6.14.1.6	    OOB blocking exception requirements

Table 6.14.1.6-1: CA band combination with exceptions allowed

CA band combination

No exception

## 6.15	CA_n2-n66

### 6.15.1		Common for 1 band UL and 2 bands UL CA

#### 6.15.1.1		Operating bands for CA

Table 6.15.1.1-1: CA band combination of band n2+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 6.15.1.2		Channel bandwidths per operating band for CA

Table 6.15.1.2-1: Supported bandwidths per CA band combination of band n2+n66

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n2A-n66A | CA_n2A-n66A | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

#### 6.15.1.3		Co-existence studies

Table 6.15.1.3-1 summarizes frequency ranges where harmonics occur for CA_n2-n66.

Table 6.15.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |  |  |

Based on above table, there is no harmonic issue for the band combination of n2 and n66.

Table 6.15.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n2-n66.

Table 6.15.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6660 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n2 and n66.

6.15.1.4		∆TIB and ∆RIB values

For CA_n2-n66 , the ∆TIB,c and ∆RIB,c values are given according to DC_2_n66 relaxation values.

Table 6.15.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n66 | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.15.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2-n66 | n2 | 0.3 |
|  | n66 | 0.3 |

#### 6.15.1.5		REFSENS requirements

#### No additional MSD requirement need to be defined for this band combs.6.15.1.6	    OOB blocking exception requirements

Table 6.15.1.6-1: CA band combination with exceptions allowed

CA band combination

No exception

6.15.2		Specific for 2 bands UL CA

6.15.2.1	Maximum output power for inter-band CA

Table 6.15.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n66A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.15.2.2		UE co-existence studies

Table 6.15.2.2-1 lists Band n2 +Band n66 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.15.2.2-1: Band n2 and Band n66 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 1710 | 1780 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 70 | 200 | 3560 | 3690 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1920 | 2110 | 1510 | 1710 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5410 | 5600 | 5270 | 5470 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3770 | 4020 | 3220 | 3490 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7260 | 7510 | 6980 | 7250 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 140 | 400 | 7120 | 7380 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 4930 | 5270 | 5620 | 5930 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 8690 | 9030 | 9110 | 9420 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 1310 | 1640 | 1990 | 2310 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8830 | 9160 | 8970 | 9290 |

Based on the table above:

- The 3rd order IMD may fall into Rx frequencies of band n2.

- The 5th order IMD may fall into Rx frequencies of band n66

Table 6.15.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.15.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n2-n66 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.15.2.3		REFSENS requirements

Table 6.15.2.3-1 lists the MSD required due to the 3rd and 5th IMD for the dual uplink configuration The MSD value is reused from DC_2_n66.

Table 6.15.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  |
| CA_n2A-n66A | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |

6.16	CA_n41-n77

6.16.1	Common for 1 band UL and 2 bands UL CA

6.16.1.1 Operating bands for CA

Table 6.16.1.1-1: CA band combination of band n41 + n77

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n41-n771 | n41 | 2495 MHz | – | 2690 MHz | 2495 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |  |  |  |  |  |

6.16.1.2	Channel bandwidths per operating band for CA

Table 6.16.1.2-1: Supported bandwidths per CA band combination of band n41+n77

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n41A-n77A | CA_n41A-n77A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n41(2A)-n77A | CA_n41A-n77A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n41C-n77A | CA_n41A-n77A CA_n41C | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n41A-n77(2A) | CA_n41A-n77A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.16.1.3	Co-existence studies

Table 6.16.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41-n77.

Table 6.16.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Based on above table no harmonics occur.

Table 6.16.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Based on above table 2nd UL harmonic of n77 is same as n41 DL 3rd harmonic.

6.16.1.4	∆TIB and ∆RIB values

For CA_n41-n77, it is proposed to re-use the TIB,c and RIB values from CA_n41-n78.

Table 6.16.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n771 | n41 | 0.3 |
|  | n77 | 0.8 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. |  |  |

Table 6.16.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n41-n771 | n77 | 0.5 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. |  |  |

6.16.1.5	REFSENs requirements

No additional requirements are necessary.

6.16.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

6.16.2		Specific for 2 bands UL CA

6.16.2.1	Maximum output power for inter-band CA

Table 6.16.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n41A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.16.2.2		UE co-existence

Table 6.16.2.2-1 gives IMD interference analysis for CA_ n41-n77 with 2 ULs.

Table 6.16.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 4200 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 | 1704 | 5796 | 6890 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 792 | 2080 | 3910 | 5904 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9580 | 9096 | 11090 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 3288 | 4770 | 7210 | 10104 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 12270 | 12396 | 15290 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3408 | 1220 | 11592 | 13780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14304 | 10510 | 7460 | 5784 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 19490 | 13284 | 14960 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7608 | 4520 | 1470 | 912 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 17980 | 14088 | 16470 |

Based on the table 6.16.2.2-1, there is 3rd and 4th order IMD are an issue for n77, however IMD3 cannot interfere own downlink channel even it hits the band.

Table 6.16.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.16.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  |  | MBW (MHz) | NOTE |  |
| CA_n41-n77 |  | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 44, 45, 50, 51, 53, 65, 66, 70, 71, 73, 74, 85, |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | 1915.7 |  | -41 | 0.3 |  |  | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.1601 NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

6.16.2.3		REFSENS requirements

There is MSD defined for CA_41A-78A due to cross band isolation, it is proposed to define same requirement for CA_41A-77A.

Table 7.3A.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | n77 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |

Table 7.3A.6.2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n77 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |

There is also harmonic receiver mixing issue that needs MSD.

Table 7.3A.4-4: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 412 |  | 10.4 | 10.4 | 10.4 |  | 10.4 | 10.4 | 10.4 | 10.4 |  | 10.4 | 10.4 | 10.4 |
| NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | 41 | 15 |  | 25 | 36 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.17		CA_n71-n77

6.17.1	Common for 1 band UL and 2 bands UL CA

6.17.1.1 Operating bands for CA

Table 6.17.1-1: CA band combination of band n71 + n77

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n71-n77 | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.17.1.2	Channel bandwidths per operating band for CA

Table 6.17.2-1: Supported bandwidths per CA band combination of band n71+n77

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n71A-n77A | CA_n71A-n77A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.17.1.3	Co-existence studies

Table 6.17.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n71-n77.

Table 6.17.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| Note: The combination is subject to LB 5th order harmonic impact to n77 but does not happen in the US |  |  |  |  |  |  |  |  |  |  |  |  |

Based on above table no harmonics occur.

Table 6.17.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Based on above table no harmonic mixing occur.

6.17.1.4	∆TIB and ∆RIB values

For CA_n71-n77, it is proposed to re-use the TIB,c and RIB values from CA_n71-n78.

Table 6.17.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n71-n77 | n71 | 0.5 |
|  | n77 | 0.8 |

Table 6.17.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n71-n77 | n71 | 0.2 |
|  | n77 | 0.5 |

6.17.1.5	REFSENs requirements

No additional requirements are necessary.

6.17.1.6	OOB blocking exception requirements

Table 6.17.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n71-n77

6.17.2		Specific for 2 bands UL CA

6.17.2.1	Maximum output power for inter-band CA

Table 6.17.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n71A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.17.2.2		UE co-existence

Table 6.17.2.2-1 gives IMD interference analysis for CA_ n71-n77 with 2 ULs.

Table 6.17.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 3300 | 4200 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 | 3537 | 3963 | 4898 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 | 1904 | 5902 | 7737 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 | 5596 | 7263 | 9098 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 2211 | 1206 | 9202 | 11937 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5289 | 6294 | 10563 | 13298 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 7074 | 5204 | 7926 | 9796 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 | 12502 | 508 | 1548 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 | 17498 | 5952 | 6992 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 | 8504 | 4506 | 6411 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 | 13996 | 8589 | 10494 |

Based on the table 6.17.2.2-1, there is 2nd and 5th order IMD are an issue for n77, however IMD2 cannot interfere own downlink channel even it hits the band.

Table 6.17.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.17.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  |  | MBW (MHz) | NOTE |  |
| CA_n71-n77 |  | E-UTRA Band 1, 3, 4, 5, 7, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 30, 34, 39, 40, 44, 45, 50, 51, 53, 65, 66, 71, 73, 74, 85, |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | 1915.7 |  | -41 | 0.3 |  |  | 3 |
|  |  | E-UTRA Band 2, 25, 41, 70 |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  | 2 |
|  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high |  | -38 | 1 |  |  | 4 |
|  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  | 4 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |

6.17.2.3		REFSENS requirements

It is proposed to use IMD5 test point in Table 6.17.2.3-1 which is based on similar CA_n28A-n77A.

Table 6.17.2.3-1

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71A-n77A | n71 | 671 | 5 | 25 | 625 | 5.5 | FDD | IMD5 |
|  | n77 | 3300 | 10 | 50 | 3300 | N/A | TDD | N/A |

6.18	CA_n5-n48

6.18.1	Common for 1 band UL and 2 bands UL CA

6.18.1.1		Operating bands for CA

Table 6.18.1.1-1: CA band combination of band n5+n48

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n48 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

6.18.1.2		Channel bandwidths per operating band for CA

Table 6.18.1.2-1: Supported bandwidths per CA configuration of band n5+n48

|  |  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL CA Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| CA_n5A-n48A | CA_n5A-n48A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
| CA_n5A-n48(2A) | CA_n5A-n48A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_48(2A) in Table 5.5A.2-1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48C | CA_n5A-n48A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48C in Table 5.5A.1-1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.18.1.3		UE co-existence studies

Table 6.18.1.3-1 lists up to 5th harmonics for CA_n5A-n48A. As can be seen there are no harmonic issue.

Table 6.18.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14800 | 14800 | 17750 | 18500 |

Table 6.18.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.18.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4245 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |

Based on study above, there is no harmonic issue for this band combination.

6.18.1.4		∆TIB and ∆RIB values

For CA_n5-n48, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.18.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n48 | n5 | 0.3 |
|  | n48 | 0.3 |

Table 6.18.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n48 | n5 | 0 |
|  | n48 | 0 |

6.18.1.5		REFSENS requirements

As can be seen in the co-existence studies in 6.18.1.3 there are no harmonics issues.

6.18.1.6		OOB blocking exception requirements

There is no OOB exception for this CA combination.

6.18.2		Specific for 2 bands UL CA

6.18.2.1	Maximum output power for inter-band CA

Table 6.18.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n48A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.18.2.2		UE co-existence

Table 6.18.2.2-1 lists Band n5+Band n48 2UL CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.18.2.2-1: Band n5 and Band n48 2 UL bands IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 3550 | 3700 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 7100 | 7400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 10650 | 11100 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 14200 | 14800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 17750 | 18500 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2701 | 2876 | 4374 | 4549 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2052 | 1852 | 6251 | 6576 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5198 | 5398 | 7924 | 8249 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1228 | 1003 | 9801 | 10276 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 6022 | 6247 | 11474 | 11949 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 5752 | 5402 | 8748 | 9098 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13976 | 13351 | 154 | 404 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15024 | 15649 | 6846 | 7096 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9452 | 8952 | 4553 | 4928 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12298 | 12798 | 9572 | 9947 |

Based on the table 6.18.2.2-1, there is no IMD issue for CA_n5-n48.

Table 6.18.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5A-n48A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 65, 66, 70, 71, 73 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |  |

6.18.2.3  REFSENs requirements

Based on above coexistence study, there is no MSD issue for the CA combination.

  - 6.19		CA_n34-n79

    - 6.19.1		Common for 1 band UL and 2 bands UL CA

      - 6.19.1.1		Operating bands for CA

Table 6.19.1.1-1: CA band combination of band n34+n79

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | FDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |  |  |  |  |

      - 6.19.1.2		Channel bandwidths per operating band for CA

Table 6.19.2-1: Supported bandwidths per CA band combination of band n34+n79

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n34A-n79A | CA_n34A-n79A | n34 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |

      - 6.19.1.3		Co-existence studies

Table 6.19.1.3-1 summarizes frequency ranges where harmonics occur for CA_n34-n79.

Table 6.19.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 |  |  |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 |  |  |

Based on above table, there is no harmonic issue for the band combination of n34 and n79.

Table 6.19.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n34-n79.

Table 6.19.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 |  |  |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n34 and n79.

      - 6.19.1.4		∆TIB and ∆RIB values

For CA_n34-n79 , the ∆TIB,c and ∆RIB,c values are given in the tables below.

Table 6.1.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n34-n79 | n34 | 0.3 |
|  | n79 | 0.8 |

Table 6.1.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n34-n79 | n34 | 0 |
|  | n79 | 0.5 |

      - 6.19.1.5		REFSENS requirements

No specific REFSENS requirements need to be specified.

      - 6.19.1.6		OOB blocking exception requirements

No OOB blocking exception requirements for CA_n34-n79 specified.

    - 6.19.2		Specific for 2 bands UL CA

      - 6.19.2.1		UE co-existence studies

Table 6.19.2.1-1 lists Band n34 +Band n79 2UL CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.19.2.1-1: Band n34 and Band n79 UL IMD products

| DL frequency | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 2010 | 2025 | 4400 | 5000 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 2375 | 2990 | 6410 | 7025 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 980 | 350 | 6775 | 7990 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8420 | 9050 | 10810 | 12025 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5980 | 4750 | 12820 | 14050 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 1675 | 11175 | 12990 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10430 | 11075 | 15210 | 17010 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17990 | 15575 | 3700 | 3040 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19610 | 22025 | 12440 | 13100 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10980 | 9150 | 2725 | 3970 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |

Based on Table 6.19.2.1-1, although 4th order IMD may also fall into own Rx of band n79, both n34 and n79 are TDD band which means the UL/DL will not simultaneous work at the same time. Therefore, no need to consider the IMD issue.

Table 6.19.2.1-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.19.2.1-2: Protected bands for the 2UL bands CA configuration

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n34-n79 | E-UTRA Band 1, 3, 8, 11, 18, 19, 21, 28, 39, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

      - 6.19.2.2		REFSENS requirements

According to the co-existent analysis in table 6.19.2.1-1,  no IMD issues are expected for this CA configuration with dual uplink carrier.

6.20		CA_n5-n25

6.20.1	Common for 1 band UL and 2 bands UL CA

6.20.1.1	Operating bands for CA

Table 6.20.1.1-1: CA band combination of band n5+n25

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |

6.20.1.2	Channel bandwidths per operating band for CA

Table 6.20.1.2-1: Supported bandwidths per CA band combination of band n5+n25

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n5A-n25A | CA_n5A-n25A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.20.1.3	UE co-existence studies

Table 6.20.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n25.

Table 6.20.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 |  |  |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |  |  |

Table 6.20.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 |  |  |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 |  |  |

Based on above table, there is no harmonic relations for n5+n25.

6.20.1.4	∆TIB and ∆RIB values

For CA_n5-n25, the TIB,c and RIB,c values are reused from the similar combination E-UTR CA_2-5 and are given in the tables below.

Table 6.20.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n25 | n5 | 0.3 |
|  | n25 | 0.3 |

Table 6.20.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n25 | n5 | 0 |
|  | n25 | 0 |

6.20.1.5	REFSENS requirements

There is no REFSENS exception for 1UL/2DL of for CA_n5-n25.

6.20.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.20.2	Specific for 2 bands UL CA

6.20.2.1	Maximum output power for inter-band CA

Table 6.20.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n25A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.20.2.2	UE co-existence studies

Table 6.20.2.1-1 lists Band n5 +Band n25 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 1850 | 1915 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1001 | 1091 | 2674 | 2764 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 267 | 152 | 2851 | 3006 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3498 | 3613 | 4524 | 4679 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 557 | 697 | 4701 | 4921 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4322 | 4462 | 6374 | 6594 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 2182 | 2002 | 5348 | 5528 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 6836 | 6551 | 1546 | 1381 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 8224 | 8509 | 5146 | 5311 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 4097 | 3852 | 1153 | 1358 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 7198 | 7443 | 6172 | 6377 |

Based on the table above, there is no IMD issue for 2UL n5+n25.

Table 6.20.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.20.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n25 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 28, 29, 30, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.20.2.3	REFSENS requirements

There is no REFSENS exception for 2UL/2DL of for CA_n5-n25.

6.21		CA_n25-n77

6.21.1	Common for 1 band UL and 2 bands UL CA

6.21.1.1	Operating bands for CA

Table 6.21.1.1-1: CA band combination of band n25+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.21.1.2	Channel bandwidths per operating band for CA

Table 6.21.1.2-1: Supported bandwidths per CA band combination of band n25+n77

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n25A-n77A |  | CA_n25A-n77A | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.21.1.3	UE co-existence studies

Table 6.21.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n25-n77.

Table 6.21.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |  |  |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 |  |  |

Table 6.21.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 |  |  |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 |  |  |

Based on above table, the 2nd harmonic of band n25 uplink may fall into band n77.

Based on above table, the 2nd harmonic of band n25 downlink may fall into band 77 thus there can be harmonic mixing issue for the band combination of n25 and n77.

6.21.1.4	∆TIB and ∆RIB values

For CA_n25-n77 , the TIB,c and RIB,c values are reused from the similar combination CA_n2-n77 and are given in the tables below.

Table 6.21.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n77 | n25 | 0.6 |
|  | n77 | 0.8 |

Table 6.21.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n77 | n25 | 0.2 |
|  | n77 | 0.5 |

6.21.1.5	REFSENS requirements

The MSD due to the harmonic issue is the same as the one for CA_n2-n77, which is reused for CA_n25-n77.

Table 6.21.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n25 | n771, 2 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n77, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.21.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n2 | n77 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |

The MSD due to the harmonic mixing issue is the same as the one for CA_n2-n77, which is reused for CA_n25-n77.

Table 6.21.1.5-3: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n25 | 6.7 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |  |  |

Table 6.21.1.5-4: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | n25 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |

6.21.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.21.2	Specific for 2 bands UL CA

6.21.2.1	Maximum output power for inter-band CA

Table 6.21.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n25A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.21.2.2	UE co-existence studies

Table 6.3.2.1-1 lists Band n25 +Band n77 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.21.2.2-1: Band n25 and Band n77 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 3300 | 4200 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1385 | 2350 | 5150 | 6115 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 500 | 530 | 4685 | 6550 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 8030 | 8450 | 10315 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1350 | 2445 | 7985 | 10750 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9945 | 11750 | 14515 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 4700 | 2770 | 10300 | 12230 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 14950 | 11285 | 4360 | 3200 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 18715 | 10700 | 11860 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 8900 | 6070 | 855 | 2850 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13600 | 16430 | 12150 | 14145 |

Based on the table above, the 2nd and 4th order IMDs may fall into Rx frequencies of band n25.

Further, the 4th order and 5th order IMDs may fall into Rx frequencies of band n77.

Table 6.3.2.2-2 lists the protected bands required for the 2UL bands CA configuration (the same as CA_n2-n77).

Table 6.21.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n25-n77 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.21.2.3	REFSENS requirements

Table 6.21.2.3-1 lists the MSD required due to the 2nd, 3rd, and 5th IMD for the dual uplink configuration The MSD value is reused from CA_n2-n77.

Table 6.21.2.3-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25A-n77A | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 8.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |

6.22	CA_n8-n28

6.22.1	Common for 1 band UL and 2 bands UL CA

6.22.1.1 Operating bands for CA

Table 6.22.1.1-1: CA band combination CA_n8A-n28A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n28 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.22.1.2	Channel bandwidths per operating band for CA

Table 6.22.1.2-1: Supported bandwidths per CA band combination CA_n8A-n28A

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n8A-n28A |  | - | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.22.1.3	UE Co-existence studies

Table 6.22.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n8-n28.

Table 6.22.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |

Based on above table, there is no harmonic interference.

Table 6.22.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |

Based on above table, there is no harmonic mixing issue.

6.22.1.4	∆TIB and ∆RIB values

For CA_n8A-n28A, the TIB,c and RIB are given in the tables below reusing the values of DC_8_n28.

Table 6.22.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n8-n28 | n8 | 0.6 |
|  | n28 | 0.5 |

Table 6.22.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n8-n28 | n8 | 0.2 |
|  | n28 | 0.2 |

6.22.1.5	REFSENs requirements

There is no MSD exception requirement for CA_n8-n28.

6.22.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirement for CA_n8-n28.

6.23	CA_n3-n7

6.23.1	Common for 1 band UL and 2 bands UL CA

6.23.1.1 Operating bands for CA

Table 6.23.1.1-1: CA band combination CA_n3A-n7A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n7 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

6.23.1.2	Channel bandwidths per operating band for CA

Table 6.23.1.2-1: Supported bandwidths per CA band combination CA_n3A-n7A

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n3A-n7A |  | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 1 |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.23.1.3	UE Co-existence studies

Table 6.23.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n7.

Table 6.23.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 |

Based on above table, there is no harmonic interference.

Table 6.23.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 |

Based on above table, there is no harmonic mixing issue.

6.23.1.4	∆TIB and ∆RIB values

For CA_n3A-n7A, the TIB,c and RIB are given in the tables below reusing the values of DC_3_n7.

Table 6.23.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n7 | n3 | 0.5 |
|  | n7 | 0.5 |

Table 6.23.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n3-n7 | n3 | 0 |
|  | n7 | 0 |

6.23.1.5	REFSENs requirements

The MSD due to cross band isolation for CA_n3-n7 is shown below, referring to DC_3_n41.

Table 6.23.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n7 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |

Table 6.23.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n7 | n3 | 15 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |

6.23.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirement for CA_n3-n7.

## 6.24	CA_n3-n18

### 6.24.1		Common for 1 band UL and 2 bands UL CA

#### 6.24.1.1		Operating bands for CA

Table 6.24.1.1-1: CA band combination of band n3+n18

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |

#### 6.24.1.2		Channel bandwidths per operating band for CA

Table 6.24.1.2-1: Supported bandwidths per CA band combination of band n3+n18

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n18A | CA_n3A-n18A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |

#### 6.24.1.3		Co-existence studies

Table 6.24.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n18.

Table 6.24.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 |

Based on above table, there is no harmonic issue for the band combination of n3 and n18.

Table 6.24.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 |

Based on above table, there is no harmonic mixing issue for the band combination of n3 and n18.

6.24.1.4		∆TIB and ∆RIB values

For CA_n3-n18 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 36.101 CA_3-18 relaxation values.

Table 6.24.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n18 | n3 | 0.3 |
|  | n18 | 0.3 |

Table 6.24.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n18 | n3 | 0 |
|  | n18 | 0 |

6.24.1.5 REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.24.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.24.2	Specific for 2 bands UL CA

6.24.2.1	Maximum output power for inter-band CA

Table 6.24.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n18A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.24.2.2	UE co-existence studies

Table 6.24.2.1-1 lists Band n3 +Band n18 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.24.2.2-1: Band n3 and Band n18 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 815 | 830 | 1710 | 1785 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 880 | 970 | 2525 | 2615 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 50 | 155 | 2590 | 2755 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 3340 | 3445 | 4235 | 4400 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 660 | 780 | 4300 | 4540 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 4155 | 4275 | 5945 | 6185 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 1760 | 1940 | 5050 | 5230 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 6010 | 6325 | 1475 | 1610 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 7655 | 7970 | 4970 | 5105 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 3470 | 3725 | 930 | 1125 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 6760 | 7015 | 5865 | 6060 |

Based on the table above, the 4th order IMD may fall into Rx frequencies of band n3.

Table 6.24.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.24.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n18 | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz |  |  |  |  |  |  |  |

6.24.2.3	REFSENS requirements

Table 6.24.2.3-1 lists the MSD required due to the 4th IMD for the dual uplink configuration The MSD value is reused from TS 38.101-3 DC_18A_n3A.

Table 6.24.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3A-n18A | n18 | 818 | 5 | 25 | 863 | N/A | FDD | N/A |
|  | n3 | 1731 | 5 | 25 | 1826 | 4 | FDD | IMD4 |

## 6.25	CA_n18-n41

### 6.25.1		Common for 1 band UL and 2 bands UL CA

#### 6.25.1.1		Operating bands for CA

Table 6.25.1.1-1: CA band combination of band n18+n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| n411 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |

NOTE 1: 	The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz.

#### 6.25.1.2		Channel bandwidths per operating band for CA

Table 6.25.1.2-1: Supported bandwidths per CA band combination of band n18+n41

| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n18A-n41A | CA_n18A-n41A | n18 | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | 15 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

#### 6.25.1.3		Co-existence studies

Table 6.25.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA _ n18-n41.

Table 6.25.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 |
| n41 | 2595 | 2645 | 2595 | 2645 | 5190 | 5290 | 7785 | 7935 | 10380 | 10580 |

Based on above table, there is no harmonic issue for the band combination of n18 and n41.

Table 6.25.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 |
| n41 | 2595 | 2645 | 2595 | 2645 | 5190 | 5290 | 7785 | 7935 | 10380 | 10580 |

Based on above table, there is n18 3rd harmonic mixing issue to n41.

6.25.1.4		∆TIB and ∆RIB values

For CA_n18-n41 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 36.101 CA_8-41 relaxation values.

Table 6.25.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n18-n41 | n18 | 0.3 |
|  | n41 | 0.3 |

Table 6.25.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n18-n41 | n18 | 0 |
|  | n41 | 0 |

#### 6.25.1.5		REFSENS requirements

IMD2 may fall in n18 but due to spectrum holding considerations, this IMD2 is not applicable.

The MSD for n18 3rd harmonic mixing issue to n41 can refer to 36.101 CA_20-41 value.

Table 6.25.1.5-1: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n413,4 | n18 | [24.3] | [24.3] | [22.5] |  |  |  |  |  |  |  |  |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.25.1.5-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n18 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.25.1.6	    OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.25.2	Specific for 2 bands UL CA

6.25.2.1	Maximum output power for inter-band CA

Table 6.25.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n18A-n41A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.25.2.2	UE co-existence studies

Table 6.25.2.2-1 lists Band n18 +Band n41 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.25.2.2-1: Band n18 and Band n41 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 815 | 830 | 2595 | 2645 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 1765 | 1830 | 3410 | 3475 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 935 | 1015 | 4360 | 4475 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 4225 | 4305 | 6005 | 6120 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 105 | 200 | 6955 | 7120 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 5040 | 5135 | 8600 | 8765 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 3530 | 3660 | 6820 | 6950 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 9550 | 9765 | 615 | 725 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 11195 | 11410 | 5855 | 5965 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 6125 | 6305 | 2700 | 2845 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 9415 | 9595 | 7635 | 7780 |

Based on the table above, there is no IMD issue for this band combination.

Table 6.25.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n18-n41 | E-UTRA Band 1, 3, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.25.2.3	REFSENS requirements

No additional MSD requirement need to be defined for this band combs since no IMD issue as analysis.

## 6.26	CA_n25-n29

### 6.26.1		Common for 1 band UL and 2 bands UL CA

#### 6.26.1.1		Operating bands for CA

Table 6.26.1.1-1: CA band combination of band n25+n29

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n29 | N/A |  |  | 717 MHz | – | 728 MHz | SDL |

#### 6.26.1.2		Channel bandwidths per operating band for CA

Table 6.26.1.2-1: Supported bandwidths per CA band combination of band n25+n29

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n25A-n29A |  | - | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n29 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.26.1.3		Co-existence studies

Table 6.26.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n25-n29.

Table 6.26.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 |
| n29 | 0 | 0 | 717 | 728 | 0 | 0 | 0 | 0 | 0 | 0 |

Based on above table, no harmonic for CA_n25-n29

Table 6.26.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 |
| n29 | 0 | 0 | 717 | 728 | 1434 | 1456 | 2151 | 2184 | 2868 | 2912 |

Based on above table, no harmonic for CA_n25-n29.

6.26.1.4		∆TIB and ∆RIB values

For CA_n25-n29 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The values same as CA_n29-n70 what are specified in TS 38.101-1.

Table 6.26.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n29 | n25 | 0.3 |
|  | n29 | 0 |

Table 6.26.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n29 | n25 | 0 |
|  | n29 | 0 |

6.26.1.5 REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.26.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

## 6.27	CA_n25-n71

### 6.27.1		Common for 1 band UL and 2 bands UL CA

#### 6.27.1.1		Operating bands for CA

Table 6.27.1.1-1: CA band combination of band n25+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

#### 6.27.1.2		Channel bandwidths per operating band for CA

Table 6.27.1.2-1: Supported bandwidths per CA band combination of band n25+n71

| NR CA Configuration | UL Configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n71A | CA_n25A-n71A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n71(2A) | CA_n25A-n71A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.27.1.3		UE co-existence studies

Table 6.27.1.3-1 lists up to 7th harmonics for n25A-n71A. As can be seen, 3rd harmonic from n71 UL might fall into n25 DL.

Table 6.27.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 | 9250 | 9575 | 11100 | 11490 | 12950 | 13405 |
| n71 | 663 | 698 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 | 3978 | 4188 | 4641 | 4886 |

Table 6.27.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen, 3rd harmonic mixing from n71 DL might affect n25 UL.

Table 6.27.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 |

6.27.1.4		∆TIB and ∆RIB values

For CA_n25A-n71, the TIB,c and RIB,c values are derived from LTE CA_4-17 which use a trap filter.

Table 6.27.1.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n71 | 25 | 0.3 |
|  | n71 | 0.6 |

Table 6.27.1.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n71 | 25 | 0 |
|  | n71 | 0.3 |

6.27.1.5 		REFSENS requirements

Due to identified harmonic issues MSD is derived from CA_4-17 and need to be defined in 38.101-1 as defined below.

Table 6.27.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n71 | n256 | 10 | 7.5 | 6 | 5.1 | 4.1 | 3.0 | 2.1 |  |  |  |  |  |
| NOTE 6: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 7: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.27.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n71 | n25 | 83 | 83 | 83 | 83 | 83 | 83 | 83 |  |  |  |  |  |
| NOTE 3: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.27.1.5-3: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n25 | n713,4 | 26.5 | 23.3 | 20.9 | 15.3 |  |  |  |  |  |  |  |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.27.1.5-4: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n25 | n71 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.27.2	Specific for 2 bands UL CA

6.27.2.1		UE co-existence studies

Table 6.27.2.1-1 lists Band n25 + Band n71 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.27.2.1-1: Band n25 and Band n71 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1252 | 1152 | 2513 | 2613 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 589 | 454 | 3002 | 3167 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3311 | 4363 | 4528 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2504 | 2304 | 5026 | 5226 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 244 | 4852 | 5082 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4009 | 6213 | 6443 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6997 | 6702 | 942 | 737 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8358 | 4502 | 4707 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4419 | 4154 | 1606 | 1841 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7141 | 5689 | 5924 |

Based on Table 6.27.2.1-1 there are no IMD issues affecting own Rx frequencies.

Table 6.27.2.1-2 lists the protected bands required for the 2UL bands CA configuration as to be used in Table 6.5A.3.2.3-1 of TS 38.101-1.

Table 6.27.2.1-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n25-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.27.2.2		REFSENS requirements

Based on the co-existence studies for CA_n25-n71 there are no further MSD needed to be defined.

## 6.28	CA_n13-n25

### 6.28.1		Common for 1 band UL and 2 bands UL CA

#### 6.28.1.1		Operating bands for CA

Table 6.28.1.1-1: CA band combination of band n13+n25

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |

#### 6.28.1.2		Channel bandwidths per operating band for CA

Table 6.28.1.2-1: Supported bandwidths per CA band combination of band n13+n25

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n13A-n25A | CA_n13A-n25A | n13 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

#### 6.28.1.3		Co-existence studies

Table 6.28.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n13-n25.

Table 6.28.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1554 | 1574 | 2331 | 2361 |  |  |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |  |  |

Table 6.28.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1492 | 1512 | 2238 | 2268 |  |  |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 |  |  |

Based on above table, there is no harmonic and harmonic mixing issue for the band combination of n13 and n25.

6.28.1.4		∆TIB and ∆RIB values

For CA_n13-n25 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The same values are shown as what are specified in TR 38.101-3 DC_13_n2.

Table 6.28.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n13-n25 | n13 | 0.3 |
|  | n25 | 0.3 |

Table 6.28.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n13-n25 | n13 | 0 |
|  | n25 | 0 |

6.28.1.5 REFSENS requirements

There is neither harmonic issue nor harmonics mixing issue for the band combination of n13 and n25.

6.28.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.28.2	Specific for 2 bands UL CA

6.28.2.1	Maximum output power for inter-band CA

Table 6.28.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n13A-n25A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.28.2.2	UE co-existence studies

Table 6.28.2.2-1 lists band n13+band n25 2UL CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.28.2.2-1: Band n13 and Band n25 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 777 | 787 | 1850 | 1915 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 1063 | 1138 | 2627 | 2702 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 276 | 361 | 2913 | 3053 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 3404 | 3489 | 4477 | 4617 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 416 | 511 | 4763 | 4968 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 4181 | 4276 | 6327 | 6532 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 2126 | 2276 | 5254 | 5404 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 6613 | 6883 | 1193 | 1298 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 8177 | 8447 | 4958 | 5063 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 3976 | 4191 | 1339 | 1499 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 7104 | 7319 | 6031 | 6191 |

Based on the table above, there is no IMD for the band combination of n13 and n25.

Table 6.28.2.2-2 lists the protected bands required for the 2UL bands CA configuration refer to DC_13_n2 in 38.101-3.

Table 6.28.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n13-n25 | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2,14, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.28.2.3	REFSENS requirements

There is no MSD issue due to IMD for this combination.

## 6.29	CA_n13-n66

### 6.29.1		Common for 1 band UL and 2 bands UL CA

#### 6.29.1.1		Operating bands for CA

Table 6.29.1.1-1: CA band combination of band n13+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 6.29.1.2		Channel bandwidths per operating band for CA

Table 6.29.1.2-1: Supported bandwidths per CA band combination of band n13+n66

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n13A-n66A | CA_n13A-n66A | n13 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

#### 6.29.1.3		Co-existence studies

Table 6.29.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n13-n66.

Table 6.29.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1554 | 1574 | 2331 | 2361 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |  |  |

Table 6.29.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1492 | 1512 | 2238 | 2268 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 |  |  |

Based on above table, there is no harmonic and harmonic mixing issue for the band combination of n13 and n66.

6.29.1.4		∆TIB and ∆RIB values

For CA_n13-n66 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The same values are shown as what are specified in TR 38.101-3 DC_13_n66.

Table 6.29.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n13-n66 | n13 | 0.3 |
|  | n66 | 0.3 |

Table 6.29.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n13-n66 | n13 | 0 |
|  | n66 | 0 |

6.29.1.5 REFSENS requirements

There is neither harmonic issue nor harmonics mixing issue for the band combination of n13 and n66.

6.29.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.29.2	Specific for 2 bands UL CA

6.29.2.1	Maximum output power for inter-band CA

Table 6.29.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n13A-n66A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.29.2.2	UE co-existence studies

Table 6.29.2.2-1 lists band n13+band n66 2UL CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.29.2.2-1: Band n13 and Band n66 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 923 | 1003 | 2487 | 2567 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 136 | 226 | 2633 | 2783 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 3264 | 3354 | 4197 | 4347 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 551 | 651 | 4343 | 4563 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 4041 | 4141 | 5907 | 6127 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 1846 | 2006 | 4974 | 5134 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 6053 | 6343 | 1328 | 1438 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 7617 | 7907 | 4818 | 4928 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 3556 | 3786 | 1059 | 1229 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 6684 | 6914 | 5751 | 5921 |

Based on the table above, there is no IMD for the band combination of n13 and n66.

Table 6.29.2.2-2 lists the protected bands required for the 2UL bands CA configuration refer to DC_13_n66 in 38.101-3.

Table 6.29.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n13-n66 | Bands 2, 4, 5, 7, 12, 13, 17, 25, 26, 27, 29, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 24, 30, 46, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.29.2.3	REFSENS requirements

There is no MSD issue due to IMD for this combination.

6.30	CA_n8-n20

6.30.1	Common for 1 band UL and 2 bands UL CA

6.30.1.1 Operating bands for CA

Table 6.30.1.1-1: CA band combination CA_n8A-n20A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n20 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |

6.30.1.2	Channel bandwidths per operating band for CA

Table 6.30.1.2-1: Supported bandwidths per CA band combination CA_n8A-n20A

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n8A-n20A | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

6.30.1.3	UE Co-existence studies

Table 6.30.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n8-n20.

Table 6.30.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |
| n20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 |

Based on above table, there is no harmonic interference.

Table 6.30.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 |
| n20 | 832 | 862 | 791 | 821 | 1582 | 1642 | 2373 | 2463 | 3146 | 3284 |

Based on above table, there is no harmonic mixing issue.

6.30.1.4	∆TIB and ∆RIB values

For CA_n8A-n20A, the TIB,c and RIB are given in the tables below reusing the values of DC_8_n20.

Table 6.30.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n8-n20 | n8 | 0.4 |
|  | n20 | 0.4 |

Table 6.30.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n8-n20 | n8 | 0 |
|  | n20 | 0 |

6.30.1.5	REFSENs requirements

There is no MSD exception requirement for CA_n8-n20.

6.30.1.6	OOB blocking exception requirements

There is no MSD exception requirement for CA_n8-n20.

6.31	CA_n7-n77

6.31.1	Common for 1 band UL and 2 bands UL CA

6.31.1.1		Operating bands for CA

Table 6.31.1.1-1: CA band combination of band n7 and n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.31.1.2		Channel bandwidths per operating band for CA

Table 6.31.1.2-1: Supported bandwidths per CA band combination of band n7 and n77

| NR CA configuration | UL | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n77A | CA_n7A-n77A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7(2A)-n77A | CA_n7A-n77A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n77(2A) | CA_n7A-n77A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n77(2A) | CA_n7A-n77A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.31.1.3		UE co-existence studies

Table 6.31.1.3-1 lists up to 7th harmonics for n7A-n77A which shows that there are no harmonics issues.

Table 6.31.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 | 15000 | 15420 | 17500 | 17990 |
| n77 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 | 19800 | 25200 | 23100 | 29400 |

Table 6.31.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.31.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

6.31.1.4		∆TIB and ∆RIB values

For CA_n7-n77, the TIB,c and RIB,c values are same as for DC_7_n77 and are given in the tables below.

Table 6.31.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n7-n77 | n7 | 0.5 |
|  | n77 | 0.8 |

Table 6.31.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n7-n77 | n7 | 0 |
|  | n77 | 0.5 |

6.31.1.5 		REFSENS requirements

There is no harmonic issue for the CA combination.

6.31.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n7-n77.

Table 6.31.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.31.2		Specific for 2 bands UL CA

6.31.2.1	Maximum output power for inter-band CA

Table 6.31.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.31.2.2		UE co-existence studies

Table 6.31.2.2-1 lists Band n7 +Band n77 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.31.2.2-1: Band n7 and Band n77 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 2500 | 2570 | 3300 | 4200 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1700 | 730 | 5800 | 6770 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 1840 | 4030 | 5900 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 9340 | 9100 | 10970 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3400 | 1460 | 11600 | 13540 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3300 | 4410 | 7330 | 10100 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11910 | 12400 | 15170 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14300 | 10630 | 6980 | 5800 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 19370 | 13300 | 14480 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7600 | 4760 | 1110 | 900 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 17740 | 14100 | 16110 |

Based on the table above:

- The 4th order IMD may fall into Rx frequencies of band n7.

- The 3rd and 4th order IMD may fall into Rx frequencies of band n77

Table 6.31.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.31.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7-n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 6 |
| NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

6.31.2.3		REFSENS requirements

Table 6.31.2.3-1 lists the MSD required. Same values as for DC_7A_n77A are used.

Table 6.31.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  |
| CA_n7A-n77A | n7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |

6.32		CA_n12-n77

6.32.1	Common for 1 band UL and 2 bands UL CA

6.32.1.1 Operating bands for CA

Table 6.32.1-1: CA band combination of band CA_n12 + n77

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n12-n77 | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.32.1.2	Channel bandwidths per operating band for CA

Table 6.32.1.2-1: Supported bandwidths per CA band combination of band CA_n12 + n77

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n12A-n77A | CA_n12A-n77A | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n12A-n77(2A) | CA_n12A-n77A | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.32.1.3	Co-existence studies

Table 6.32.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n12-n77.

Table 6.32.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 2796 | 2864 | 3495 | 3580 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is no UL 5th harmonic issue for CA_n12-n77.

Table 6.32.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 | 2916 | 2984 | 3645 | 3730 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is 5th order harmonic mixing issue for CA_n12-n77.

6.32.1.4	∆TIB and ∆RIB values

Table 6.32.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n12-n77 | n12 | 0.5 |
|  | n77 | 0.8 |

Table 6.32.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n12-n77 | n12 | 0.2 |
|  | n77 | 0.5 |

6.32.1.5	REFSENs requirements

It is proposed to reuse UL harmonic MSD from DC_12_n78.

Table 7.3B.2.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n12 | n774,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n12 | n77 | 15 |  | 10 | 15 | 20 |  |  | 20 | 20 | 20 |  | 20 | 20 | 20 |

It is proposed to reuse DL harmonic mixing MSD from DC_13_n77

Table 7.3B.2.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n122 | 31 | 28 | 26.2 |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | n12 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |

6.32.1.6	OOB blocking exception requirements

Table 6.2.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n12-n77

6.32.2		Specific for 2 bands UL CA

6.32.2.1	Maximum output power for inter-band CA

Table 6.32.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n12A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.32.2.2		UE co-existence

Table 6.32.2.2-1 gives IMD interference analysis for CA_ n12-n77 with 2 ULs.

Table 6.32.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 4200 |
| DL frequency (MHz) | 729 | 746 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 16500 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 16500 | 21000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2584 | 3501 | 3999 | 4916 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2802 | 1868 | 5884 | 7701 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4698 | 5632 | 7299 | 9116 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 2103 | 1152 | 9184 | 11901 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5397 | 6348 | 10599 | 13316 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 7002 | 5168 | 7998 | 9832 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16101 | 12484 | 436 | 1404 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13899 | 17516 | 6096 | 7064 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11202 | 8468 | 4452 | 6303 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11298 | 14032 | 8697 | 10548 |

Based on the table 6.32.2.2-1, there IMD2 potentially interfering n77 and IMD5 interfering n12.

Table 6.32.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.32.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n12-n77 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 66, 70, | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.32.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.32.2.3		REFSENS requirements

IMD5 interfering n12 own DL needs MSD requirement. MSD is re-used from CA_n28-n77 which is very similar combination.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n12-n77 | n12 | 705.5 | 5 | 20 | 760.5 | 5.5 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |

6.33		CA_n14-n77

6.33.1	Common for 1 band UL and 2 bands UL CA

6.33.1.1 Operating bands for CA

Table 6.33.1-1: CA band combination of band CA_n14-n77

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n14-n77 | n14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.33.1.2	Channel bandwidths per operating band for CA

Table 6.33.1.2-1: Supported bandwidths per CA band combination of band CA_n14-n77

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n14A-n77A | CA_n14A-n77A | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n14A-n77(2A) | CA_n14A-n77A | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.33.1.3	Co-existence studies

Table 6.33.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n14-n77.

Table 6.33.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 14 | 788 | 798 | 758 | 768 | 1576 | 1596 | 2364 | 2394 | 3152 | 3192 | 3940 | 3990 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is no UL 5th harmonic issue for CA_n14-n77.

Table 6.33.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 14 | 788 | 798 | 758 | 768 | 1516 | 1536 | 2274 | 2304 | 3032 | 3072 | 3790 | 3840 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is 5th order harmonic mixing issue for CA_n14-n77.

6.33.1.4	∆TIB and ∆RIB values

Table 6.33.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n14-n77 | n14 | 0.5 |
|  | n77 | 0.8 |

Table 6.33.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n14-n77 | n14 | 0.2 |
|  | n77 | 0.5 |

6.33.1.5	REFSENs requirements

It is proposed to reuse UL harmonic MSD from DC_12_n78.

Table 7.3B.2.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n14 | n774,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n14 | n77 | 15 |  | 10 | 15 | 20 |  |  | 20 | 20 | 20 |  | 20 | 20 | 20 |

It is proposed to reuse DL harmonic mixing MSD from DC_13_n77

Table 7.3B.2.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n142 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | n14 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |

6.33.1.6	OOB blocking exception requirements

Table 6.2.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n14-n77

6.33.2		Specific for 2 bands UL CA

6.33.2.1	Maximum output power for inter-band CA

Table 6.33.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n14A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.33.2.2		UE co-existence

Table 6.33.2.2-1 gives IMD interference analysis for CA_ n14-n77 with 2 ULs.

Table 6.33.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 788 | 798 | 3300 | 4200 |
| DL frequency (MHz) | 758 | 768 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1576 | 1596 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2364 | 2394 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3152 | 3192 | 16500 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3940 | 3990 | 16500 | 21000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2502 | 3412 | 4088 | 4998 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2624 | 1704 | 5802 | 7612 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4876 | 5796 | 7388 | 9198 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 1836 | 906 | 9102 | 11812 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5664 | 6594 | 10688 | 13398 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 6824 | 5004 | 8176 | 9996 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16012 | 12402 | 108 | 1048 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13988 | 17598 | 6452 | 7392 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11024 | 8304 | 4206 | 6036 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11476 | 14196 | 8964 | 10794 |

Based on the table 6.33.2.2-1, there IMD2 potentially interfering n77 and IMD5 interfering n14.

Table 6.33.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.33.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n14-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4, 20 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4, 20 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.33.3.1-1 from the edge of the channel bandwidth. NOTE 20: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB |  |  |  |  |  |  |  |

6.33.2.3		REFSENS requirements

IMD5 interfering n14 own DL needs MSD requirement. MSD is re-used from CA_n28-n77 which is very similar combination.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n14-n77 | n14 | 793 | 5 | 20 | 763 | 5.5 | FDD | IMD5 |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | TDD | N/A |

6.34		CA_n30-n77

6.34.1	Common for 1 band UL and 2 bands UL CA

6.34.1.1 Operating bands for CA

Table 6.34.1-1: CA band combination of band CA_n30-n77

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n30-n77 | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.34.1.2	Channel bandwidths per operating band for CA

Table 6.34.1.2-1: Supported bandwidths per CA band combination of band CA_n30-n77

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n30A-n77A | CA_n30A-n77A | n30 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n30A-n77(2A) | CA_n77(2A) CA_n30A-n77A | n30 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.34.1.3	Co-existence studies

Table 6.34.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n30-n77.

Table 6.34.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 30 | 2305 | 2315 | 2350 | 2360 | 4610 | 4630 | 6915 | 6945 | 9220 | 9260 | 11525 | 11575 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is no UL harmonic issue for CA_n30-n77.

Table 6.34.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 30 | 2305 | 2315 | 2350 | 2360 | 4700 | 4720 | 7050 | 7080 | 9400 | 9440 | 11750 | 11800 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

UE receiver may suffer receiver 3rd order harmonic mixing from 2*TX of n77

6.34.1.4	∆TIB and ∆RIB values

It is proposed to re- use relaxation values from DC_41_n77,

Table 6.34.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n30-n77 | n30 | 0.3 |
|  | n77 | 0.8 |

Table 6.34.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n30-n77 | n30 | 0 |
|  | n77 | 0.5 |

6.34.1.5	REFSENs requirements

There is no REFSENS exceptions needed.

6.34.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

6.34.2		Specific for 2 bands UL CA

6.34.2.1	Maximum output power for inter-band CA

Table 6.34.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n30A-n77A | 23 | +2/-3 |

6.34.2.2		UE co-existence

Table 6.34.2.2-1 gives IMD interference analysis for CA_ n30-n77 with 2 ULs.

Table 6.34.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2305 | 2315 | 3300 | 4200 |
| DL frequency (MHz) | 2350 | 2360 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4610 | 4630 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6915 | 6945 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9220 | 9260 | 16500 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11525 | 11575 | 16500 | 21000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 985 | 1895 | 5605 | 6515 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 410 | 1330 | 4285 | 6095 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7910 | 8830 | 8905 | 10715 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 2715 | 3645 | 7585 | 10295 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10215 | 11145 | 12205 | 14915 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3790 | 1970 | 11210 | 13030 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14495 | 10885 | 5960 | 5020 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15505 | 19115 | 12520 | 13460 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7990 | 5270 | 345 | 1485 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14510 | 17230 | 13515 | 15345 |

Based on the table 6.34.2.2-1, there is IMD4 interfering n30.

Table 6.34.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.34.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n30-n77 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |

6.34.2.3		REFSENS requirements

IMD5 interfering n30 own DL needs MSD requirement. MSD is re-used from CA_n1-n77 which is very similar combination.

Table 7.3A.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 8.0 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |

It is proposed to use MSD values from DC_40_n78.

Table 7.3B.2.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 308 | 10.4 | 10.4 |  |  |  |  |  |  |  |  |  |
| NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 30 | 15 | 12 | 25 |  |  |  |  |  |  |  |  |  |

6.35		CA_n41-n66

6.35.1	Common for 1 band UL and 2 bands UL CA

6.35.1.1 Operating bands for CA

Table 6.35.1-1: CA band combination of band CA_n41-n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2469 MHz | – | 2690 MHz | 2469 MHz | – | 2690 MHz | TDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.35.1.2	Channel bandwidths per operating band for CA

Table 8.x.2-1: Supported bandwidths per CA band combination of band CA_n41-n66

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41(2A)-n66A | CA_n41A-n66A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

6.35.1.3	Co-existence studies

Co-existence studies for CA_n41-n66 are captured into 38.716-02-00 and from there we can see that there are no harmonic issue.

6.35.1.4	∆TIB and ∆RIB values

Already in specification.

6.35.1.5	REFSENs requirements

Reference sensitivity exceptions due to cross band isolation are already specified for CA_n41-n66 but as CA_n25A-n71(2A) CA_n41(2A)-n66A BCS1 introduced new channel bandwidths for n66 MSD table need updating.

Table 6.35.1.5-1: MSD for the CA configuration for asynchronous operation and cross band isolation for CA

| NR CA Configuration CA_n41-n66 | NR UL band | NR DL band | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
|  | n411 | n66 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied.. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.35.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for CA

| Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR UL band | NR DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n66 | 30 | 128 | 128 | 128 | 128 | 128 | 128 | 128 |  |  |  |  |  |

6.35.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

6.35.2		Specific for 2 bands UL CA

6.35.2.1	Maximum output power for inter-band CA

Already in specification.

6.35.2.2		UE co-existence

Two UL co-ex analysis is captured into 38.716-02-00 and from there we can see that there are IMD4 issues affecting own Rx frequencies of band n41.

Protected bands list is already in specification.

6.35.2.3		REFSENS requirements

Based on the co-existence studies for CA_n41-n66 MSD does not need to be defined.

## 6.36	CA_n1-n20

### 6.36.1		Common for 1 band UL and 2 bands UL CA

#### 6.36.1.1		Operating bands for CA

Table 6.36.1.1-1: CA band combination of band n1+n20

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |

#### 6.36.1.2		Channel bandwidths per operating band for CA

Table 6.36.1.2-1: Supported bandwidths per CA band combination of band n1+n20

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n20A | - | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

#### 6.36.1.3		Co-existence studies

Table 6.36.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1-n20.

Table 6.36.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |
| n20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 |

Based on above table, no harmonic for CA_n1-n20

Table 6.36.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n20 | 832 | 862 | 791 | 821 | 1582 | 1456 | 2151 | 2184 | 2868 | 2912 |

Based on above table, no harmonic for CA_n1-n20.

6.36.1.4		∆TIB and ∆RIB values

For CA_n1-n20 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The values same as CA_1-20 specified in TS 36.101.

Table 6.36.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n20 | n1 | 0.3 |
|  | n20 | 0.3 |

Table 6.36.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n20 | n1 | 0 |
|  | n20 | 0 |

6.36.1.5 REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.36.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

## 6.37	CA_n3-n20

### 6.37.1		Common for 1 band UL and 2 bands UL CA

#### 6.37.1.1		Operating bands for CA

Table 6.37.1.1-1: CA band combination of band n3+n20

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |

#### 6.37.1.2		Channel bandwidths per operating band for CA

Table 6.37.1.2-1: Supported bandwidths per CA band combination of band n3+n20

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n20A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

#### 6.37.1.3		Co-existence studies

Table 6.37.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n20.

Table 6.37.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |
| n20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 |

Based on above table, no harmonic for CA_n3-n20

Table 6.37.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| n20 | 832 | 862 | 791 | 821 | 1582 | 1456 | 2151 | 2184 | 2868 | 2912 |

Based on above table, no harmonic for CA_n3-n20.

6.37.1.4		∆TIB and ∆RIB values

For CA_n3-n20 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The values same as CA_3-20 specified in TS 36.101.

Table 6.37.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n20 | n3 | 0.3 |
|  | n20 | 0.3 |

Table 6.37.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n20 | n3 | 0 |
|  | n20 | 0 |

6.37.1.5 REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.37.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

## 6.38	CA_n1-n18

### 6.38.1		Common for 1 band UL and 2 bands UL CA

#### 6.38.1.1		Operating bands for CA

Table 6.38.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n1-n18 | n1, n18 |  |

#### 6.38.1.2		Channel bandwidths per operating band for CA

Table 6.38.1.2-1: Supported bandwidths per CA band combination of band n1+n18

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n18A | CA_n1A-n18A | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |

#### 6.38.1.3		Co-existence studies

Table 6.38.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1-n18.

Table 6.38.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 |

Based on above table, there is no harmonic issue for the band combination of n1 and n18.

Table 6.38.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 |

Based on above table, there is no harmonic mixing issue for the band combination of n1 and n18.

6.38.1.4		∆TIB and ∆RIB values

For CA_n1-n18 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 36.101 CA_1-18 relaxation values.

Table 6.38.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n18 | n1 | 0.3 |
|  | n18 | 0.3 |

Table 6.38.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n18 | n1 | 0 |
|  | n18 | 0 |

6.38.1.5    REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.38.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.38.2	Specific for 2 bands UL CA

6.38.2.1	Maximum output power for inter-band CA

Table 6.38.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n18A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.38.2.2	UE co-existence studies

Table 6.38.2.1-1 lists Band n1 +Band n18 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.38.2.2-1: Band n1 and Band n18 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 815 | 830 | 1920 | 1980 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 1090 | 1165 | 2735 | 2810 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 260 | 350 | 3010 | 3145 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 3550 | 3640 | 4655 | 4790 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 465 | 570 | 4930 | 5125 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 4365 | 4470 | 6575 | 6770 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 2180 | 2330 | 5470 | 5620 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 6850 | 7105 | 1280 | 1400 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 8495 | 8750 | 5180 | 5300 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 4100 | 4310 | 1350 | 1515 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 7390 | 7600 | 6285 | 6450 |

Based on above, there is no IMD issue of this band combination.

Table 6.38.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.38.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n18 | E-UTRA Band 1, 11, 21, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.38.2.3	REFSENS requirements

Based on Table 6.38.2.2-1, there is no MSD exception requirement.

## 6.39	CA_n1-n74

### 6.39.1		Common for 1 band UL and 2 bands UL CA

#### 6.39.1.1		Operating bands for CA

Table 6.39.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n1-n74 | n1, n74 |  |

#### 6.39.1.2		Channel bandwidths per operating band for CA

Table 6.39.1.2-1: Supported bandwidths per CA band combination of band n1+n74

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n74A | CA_n1A-n74A | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n74 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

#### 6.39.1.3		Co-existence studies

Table 6.39.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1-n74.

Table 6.39.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2854 | 2940 | 4281 | 4410 | 5708 | 5880 |

Based on above table, there is no harmonic issue for the band combination of n1 and n74.

Table 6.39.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2950 | 3036 | 4425 | 4554 | 5900 | 6072 |

Based on above table, there is no harmonic mixing issue for the band combination of n1 and n74.

6.39.1.4		∆TIB and ∆RIB values

For CA_n1-n74 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 36.101 CA_1-11 and CA_1-21 relaxation values.

Table 6.39.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n74 | n1 | 0.3 |
|  | n74 | 0.3 |

Table 6.39.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n74 | n1 | 0 |
|  | n74 | 0 |

6.39.1.5    REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.39.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.39.2	Specific for 2 bands UL CA

6.39.2.1	Maximum output power for inter-band CA

Table 6.39.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n74A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.39.2.2	UE co-existence studies

Table 6.39.2.1-1 lists Band n1 +Band n74 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.39.2.2-1: Band n1 and Band n74 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1920 | 1980 | 1427 | 1470 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 450 | 553 | 3347 | 3450 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 2370 | 2533 | 874 | 1020 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 5267 | 5430 | 4774 | 4920 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 4290 | 4513 | 2301 | 2490 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 7187 | 7410 | 6201 | 6390 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 900 | 1106 | 6694 | 6900 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 3728 | 3960 | 6210 | 6493 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 7628 | 7860 | 9107 | 9390 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 321 | 570 | 2820 | 3086 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 8121 | 8370 | 8614 | 8880 |

Based on above, there is no IMD issue for the band combination of n1 and n74.

Table 6.39.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.39.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n74 | E-UTRA Band 1, 5, 7, 8, 18, 19, 20, 26, 28, 31, 38, 40, 41, 42, 43, 52, 65, 67, 68 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 21 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for channel bandwidths up to 20 MHz within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 21: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth |  |  |  |  |  |  |  |

6.39.2.3	REFSENS requirements

Based on Table 6.39.2.2-1, there are no MSD exception requirements.

## 6.40	CA_n18-n74

### 6.40.1		Common for 1 band UL and 2 bands UL CA

#### 6.40.1.1		Operating bands for CA

Table 6.40.1.1-1: CA band combination of band n18+n74

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| n74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |

#### 6.40.1.2		Channel bandwidths per operating band for CA

Table 6.40.1.2-1: Supported bandwidths per CA band combination of band n18+n74

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n18A-n74A | CA_n18A-n74A | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n74 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

6.40.1.3		Co-existence studies

Table 6.40.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n18-n74.

Table 6.40.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2854 | 2940 | 4281 | 4410 | 5708 | 5880 |

Based on above table, there is no harmonic issue for the band combination of n18 and n74.

Table 6.40.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2950 | 3036 | 4425 | 4554 | 5900 | 6072 |

Based on above table, there is no harmonic mixing issue for the band combination of n18 and n74.

6.40.1.4		∆TIB and ∆RIB values

For CA_n18-n74 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to 36.101 CA_11-18.

Table 6.40.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n18-n74 | n18 | 0.3 |
|  | n74 | 0.3 |

Table 6.40.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n18-n74 | n18 | 0 |
|  | n74 | 0 |

6.40.1.5     REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.40.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.40.2	Specific for 2 bands UL CA

6.40.2.1	Maximum output power for inter-band CA

Table 6.40.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n18A-n74A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.40.2.2	UE co-existence studies

Table 6.40.2.1-1 lists Band n18 +Band n74 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.40.2.2-1: Band n18 and Band n74 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 815 | 830 | 1427 | 1470 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 597 | 655 | 2242 | 2300 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 160 | 233 | 2024 | 2125 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 3057 | 3130 | 3669 | 3770 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 975 | 1063 | 3451 | 3595 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 3872 | 3960 | 5096 | 5240 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 1194 | 1310 | 4484 | 4600 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 4878 | 5065 | 1790 | 1893 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 6523 | 6710 | 4687 | 4790 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 2621 | 2780 | 364 | 495 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 5911 | 6070 | 5299 | 5430 |

Based on above, there is no IMD issue.

Table 6.40.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.40.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n18-n74 | E-UTRA Band 1, 3, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 21 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 21: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth |  |  |  |  |  |  |  |

6.40.2.3	REFSENS requirements

Based on Table 6.40.2.2-1, there is no MSD exception requirement.

## 6.41	CA_n18-n77

### 6.41.1		Common for 1 band UL and 2 bands UL CA

#### 6.41.1.1		Operating bands for CA

Table 6.41.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n18-n779 | n18, n77 |  |
| NOTE 9: The frequency range in band n77 is restricted for this band combination to 3520-3560 MHz，3700-3800 MHz，4000-4100 MHz. |  |  |

#### 6.41.1.2		Channel bandwidths per operating band for CA

Table 6.41.1.2-1: Supported bandwidths per CA band combination of band n18+n77

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n18A-n77A | CA_n18A-n77A | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

#### 6.41.1.3		Co-existence studies

Table 6.41.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n18-n77.

Table 6.41.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 | 4075 | 4150 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, 4th, 5th harmonic of n18 UL falls into n77 DL.

Table 6.41.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 | 4300 | 4375 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |  |  |

Based on above table, 4th harmonic mixing of n18 DL falls into n77 UL.

6.41.1.4		∆TIB and ∆RIB values

For CA_n18-n77, the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 38.101-3 DC_18_n77 relaxation values.

Table 6.41.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n18-n77 | n18 | 0.3 |
|  | n77 | 0.8 |

Table 6.41.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n18-n77 | n18 | 0 |
|  | n77 | 0.5 |

6.41.1.5 REFSENS requirements

Based on the analysis above, reference sensitivity exceptions is specified below considering actual spectrum allocation.

Table 6.41.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB |  | dB | dB | dB |
| n18 | n776,7 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.41.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n18 | n77 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |

6.41.1.6	OOB blocking exception requirements

Table 6.41.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n18-n77

6.41.2	Specific for 2 bands UL CA

6.41.2.1	Maximum output power for inter-band CA

Table 6.41.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n18A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.41.2.2	UE co-existence studies

Table 6.41.2.1-1 lists Band n18 +Band n77 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.41.2.2-1: Band n18 and Band n77 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 815 | 830 | 3300 | 4200 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 2470 | 3385 | 4115 | 5030 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 1640 | 2570 | 5770 | 7585 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 4930 | 5860 | 7415 | 9230 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 810 | 1755 | 9070 | 11785 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 5745 | 6690 | 10715 | 13430 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 4940 | 6770 | 8230 | 10060 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 12370 | 15985 | 20 | 940 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 14015 | 17630 | 6560 | 7520 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 8240 | 10970 | 4110 | 5955 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 11530 | 14260 | 9045 | 10890 |

Based on above, the 4th, 5th IMD of n18 UL and n77 UL fall into n18 DL.

Table 6.41.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.41.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n18-n77 | E-UTRA Band 1, 3, 11, 21, 34, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |

6.41.2.3	REFSENS requirements

Although table 6.41.2.2-1 shows that 4th, 5th IMD may falls into own Rx of band 18, there is no IMD issue for own Rx range in this combination taking actual spectrum into account.

Table 6.41.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  |
| CA_n18A-n77A8 | n18 | N/A | N/A | N/A | N/A | N/A | IMD4/5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE8: There is no IMD4/5 products in band n18 downlink for n77 operating in 3520 – 3560 MHz, 3700 – 3800MHz and 4000 - 4100MHz frequency range. |  |  |  |  |  |  |  |

## 6.42	CA_n18-n78

### 6.42.1		Common for 1 band UL and 2 bands UL CA

#### 6.42.1.1		Operating bands for CA

Table 6.42.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n18-n7810 | n18, n78 |  |
| NOTE 10: The frequency range in band n78 is restricted for this band combination to 3520 -3560 MHz and 3700– 3800 MHz. |  |  |

#### 6.42.1.2		Channel bandwidths per operating band for CA

Table 6.42.1.2-1: Supported bandwidths per CA band combination of band n18+n78

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n18A-n78A | CA_n18A-n78A | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

#### 6.42.1.3		Co-existence studies

Table 6.42.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n18-n78.

Table 6.42.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 | 4075 | 4150 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |

Based on above table, 4th, harmonic of n18 UL falls into n78 DL.

Table 6.42.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |  |  |

Based on above table, 4th harmonic mixing of n18 DL falls into n78 UL.

#### 6.42.1.4		∆TIB and ∆RIB values

For CA_n18-n78, the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TR 37.863-01-01 DC_18_n78 relaxation values.

Table 6.42.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n18-n78 | n18 | 0.3 |
|  | n78 | 0.8 |

Table 6.42.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n18-n78 | n18 | 0 |
|  | n78 | 0.5 |

#### 6.42.1.5     REFSENS requirements

There is no additional MSD issue taking actual spectrum into account.

#### 6.42.1.6	    OOB blocking exception requirements

Table 6.42.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n18-n78

6.42.2	Specific for 2 bands UL CA

6.42.2.1	Maximum output power for inter-band CA

Table 6.42.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n18A-n78A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.42.2.2	UE co-existence studies

Table 6.42.2.1-1 lists Band n18 +Band n78 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.42.2.2-1: Band n18 and Band n78 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 815 | 830 | 3300 | 3800 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 2470 | 2985 | 4115 | 4630 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 1640 | 2170 | 5770 | 6785 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 4930 | 5460 | 7415 | 8430 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 810 | 1355 | 9070 | 10585 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 5745 | 6290 | 10715 | 12230 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 4940 | 5970 | 8230 | 9260 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 12370 | 14385 | 20 | 540 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 14015 | 16030 | 6560 | 7120 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 8240 | 9770 | 4110 | 5155 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 11530 | 13060 | 9045 | 10090 |

The 4th IMD of band18 UL and band 78 UL fall into own RX of band 18.

Table 6.42.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.42.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n18-n78 | E-UTRA Band 1, 3, 11, 21, 34, 65, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |

6.42.2.3	REFSENS requirements

Although table 6.42.2.2-1 shows that 4th IMD may falls into own Rx of band 18, there is no IMD issue for own Rx range in this combination taking actual spectrum into account.

Table 6.42.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  |
| CA_n18A-n78A9 | n18 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE9: There is no IMD4 product in band n18 downlink for n78 operating in 3520 – 3560MHz and 3700-3800MHz frequency range. |  |  |  |  |  |  |  |

## 6.43	CA_n74-n78

### 6.43.1		Common for 1 band UL and 2 bands UL CA

#### 6.43.1.1		Operating bands for CA

Table 6.43.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n74-n78 | n74, n78 |  |

#### 6.43.1.2		Channel bandwidths per operating band for CA

Table 6.43.1.2-1: Supported bandwidths per CA band combination of band n74+n78

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n74A-n78A | CA_n74A-n78A | n74 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

#### 6.43.1.3		Co-existence studies

Table 6.43.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n74-n78.

Table 6.43.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n74 | 1427 | 1470 | 1475 | 1518 | 2854 | 2940 | 4281 | 4410 | 5708 | 5880 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

Based on above table, there is no harmonic issue of the band combination.

Table 6.43.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n74 | 1427 | 1470 | 1475 | 1518 | 2950 | 3036 | 4425 | 4554 | 5900 | 6072 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |  |  |

Based on above table, there is no harmonic mixing issue of the band combination.

6.43.1.4		∆TIB and ∆RIB values

For CA_n74-n78, the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 38.101-3 DC_11_n78 and DC_21_n78 relaxation values.

Table 6.43.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n74-n78 | n74 | 0.4 |
|  | n78 | 0.8 |

Table 6.43.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n74-n78 | n74 | 0 |
|  | n78 | 0.5 |

6.43.1.5    REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.43.1.6    OOB blocking exception requirements

There are no OOB blocking exception requirements.

6.43.2	Specific for 2 bands UL CA

6.43.2.1	Maximum output power for inter-band CA

Table 6.43.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n74A-n78A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.43.2.2	UE co-existence studies

Table 6.43.2.1-1 lists Band n74 +Band n78 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.43.2.2-1: Band n18 and Band n78 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1427 | 1470 | 3300 | 3800 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 1830 | 2373 | 4727 | 5270 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 360 | 946 | 5130 | 6173 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 6154 | 6740 | 8027 | 9070 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 481 | 1110 | 8430 | 9973 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 7581 | 8210 | 11327 | 12870 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 3660 | 4746 | 9454 | 10540 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 11730 | 13773 | 1908 | 2580 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 14627 | 16670 | 9008 | 9680 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 6960 | 8546 | 2190 | 3319 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 12754 | 14340 | 10881 | 12010 |

Based on above analysis, there is no IMD issue.

Table 6.43.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.43.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n74-n78 | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 26, 28, 34, 39, 40, 41, 65, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 21 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 21: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth |  |  |  |  |  |  |  |

6.43.2.3	REFSENS requirements

No additional MSD requirements need to be defined for this band combs configuration.

## 6.44	CA_n3-n74

### 6.44.1		Common for 1 band UL and 2 bands UL CA

#### 6.44.1.1		Operating bands for CA

Table 6.44.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n3-n74 | n3, n74 |  |

#### 6.44.1.2		Channel bandwidths per operating band for CA

Table 6.44.1.2-1: Supported bandwidths per CA band combination of band n3+n74

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n74A | CA_n3A-n74A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n74 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

#### 6.44.1.3		Co-existence studies

Table 6.44.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n74.

Table 6.44.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2854 | 2940 | 4281 | 4410 | 5708 | 5880 |

Based on above table, there is no harmonic issue for the band combination of n3 and n74.

Table 6.44.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2950 | 3036 | 4425 | 4554 | 5900 | 6072 |

Based on above table, there is no harmonic mixing issue for the band combination of n3 and n74.

6.44.1.4		∆TIB and ∆RIB values

For CA_n3-n74 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 38.101-3 DC_11-n3 and TS36.101 CA_3-11 ,CA_3-21 relaxation values.

Table 6.44.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n74 | n3 | 0.8 |
|  | n74 | 0.9 |

Table 6.44.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n74 | n3 | 0.3 |
|  | n74 | 0.5 |

6.44.1.5    REFSENS requirements

For this combinations, sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same CA configuration due to cross band isolation issues. Reference sensitivity exceptions are specified below.

Table 6.44.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n3 | n74 | 2.6 | 2.6 | 2.6 | 2.6 |  |  |  |  |  |  |  |  |  |

Table 6.44.1.5-2: Table 7.3A.6.2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n3 | n74 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |

6.44.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.44.2	Specific for 2 bands UL CA

6.44.2.1	Maximum output power for inter-band CA

Table 6.44.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n74A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.44.2.2	UE co-existence studies

Table 6.44.2.1-1 lists Band n3 +Band n74 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.44.2.2-1: Band n3 and Band n74 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1785 | 1427 | 1470 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 240 | 358 | 3137 | 3255 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 1950 | 2143 | 1069 | 1230 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 4847 | 5040 | 4564 | 4725 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 3660 | 3928 | 2496 | 2700 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 6557 | 6825 | 5991 | 6195 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 480 | 716 | 6274 | 6510 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 3923 | 4170 | 5370 | 5713 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 7418 | 7665 | 8267 | 8610 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 711 | 990 | 2190 | 2501 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 7701 | 7980 | 7984 | 8295 |

Based on above, there is no IMD issue.

Table 6.44.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.44.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n74 | E-UTRA Band 1, 5, 7, 8, 18, 19, 20, 26, 28, 31, 34, 38, 39, 40, 41, 43, 65, 67, 68 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 21 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 21: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth |  |  |  |  |  |  |  |

6.44.2.3	REFSENS requirements

Based on Table 6.44.2.2-1, there is no MSD exception requirement.

## 6.45	CA_n18-n28

### 6.45.1		Common for 1 band UL and 2 bands UL CA

#### 6.45.1.1		Operating bands for CA

Table 6.45.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n18-n28 | n18, n28 |  |

#### 6.45.1.2		Channel bandwidths per operating band for CA

Table 6.45.1.2-1: Supported bandwidths per CA band combination of band n18+n28

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n18A-n28A | CA_n18A-n28A | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.45.1.3		Co-existence studies

Table 6.45.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n18-n28.

Table 6.45.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |

Based on above table, there is no harmonic issue for the band combination of n18 and n28.

Table 6.45.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |

Based on above table, there is no harmonic mixing issue for the band combination of n18 and n28.

6.45.1.4		∆TIB and ∆RIB values

For CA_n18-n28 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 36.101 CA_18-28 and TS 38.101-3 DC_18_n28 relaxation values.

Table 6.45.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n18-n28 | n18 | 0.5 |
|  | n28 | 0.5 |

Table 6.45.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n18-n28 | n18 | 0 |
|  | n28 | 0 |

6.45.1.5    REFSENS requirements

MSD due to cross band isolation defined for CA_n18-n28 is assumed below.

Table 6.45.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n18 | n28 | [4.5] | [3] |  |  |  |  |  |  |  |  |  |  |  |

Table 6.45.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n18 | n28 | 15 | 18 | 18 |  |  |  |  |  |  |  |  |  |  |  |

6.45.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.45.2	Specific for 2 bands UL CA

6.45.2.1	Maximum output power for inter-band CA

Table 6.45.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n18A-n28A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.45.2.2	UE co-existence studies

Table 6.45.2.1-1 lists Band n18 +Band n28 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.45.2.2-1: Band n18 and Band n28 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 815 | 830 | 703 | 748 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 67 | 127 | 1518 | 1578 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 882 | 957 | 576 | 681 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 2333 | 2408 | 2221 | 2326 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 1697 | 1787 | 1279 | 1429 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 3148 | 3238 | 2924 | 3074 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 134 | 254 | 3036 | 3156 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 1982 | 2177 | 2512 | 2617 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 3627 | 3822 | 3963 | 4068 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 449 | 614 | 949 | 1084 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 3739 | 3904 | 3851 | 3986 |

Based on above, there is no IMD issue.

Table 6.45.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.45.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n18-n28 | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2,11, 15 |
|  | E-UTRA Band 42, 65 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 11: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

6.45.2.3	REFSENS requirements

Based on Table 6.45.2.2-1, there is no MSD exception requirement.

## 6.46	CA_n41-n74

### 6.46.1		Common for 1 band UL and 2 bands UL CA

#### 6.46.1.1		Operating bands for CA

Table 6.46.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n41-n74 | n41, n74 |  |

#### 6.46.1.2		Channel bandwidths per operating band for CA

Table 6.46.1.2-1: Supported bandwidths per CA band combination of band n41+n74

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n74A | CA_n41A-n74A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n74 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

#### 6.46.1.3		Co-existence studies

Table 6.46.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41-n74.

Table 6.46.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2854 | 2940 | 4281 | 4410 | 5708 | 5880 |

Based on above table, there is no harmonic issue for the band combination of n41 and n74.

Table 6.46.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2950 | 3036 | 4425 | 4554 | 5900 | 6072 |

Based on above table, there is no harmonic mixing issue for the band combination of n41 and n74.

6.46.1.4		∆TIB and ∆RIB values

For CA_n41-n74 , the ∆TIB,c and ∆RIB,c values are specified in the table below which refer to TS 36.101 CA_11-41.

Table 6.46.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n74 | n41 | 0.3 |
|  | n74 | 0.3 |

Table 6.46.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41-n74 | n41 | 0 |
|  | n74 | 0 |

6.46.1.5     REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.46.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.46.2	Specific for 2 bands UL CA

6.46.2.1	Maximum output power for inter-band CA

Table 6.46.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n41A-n74A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.46.2.2	UE co-existence studies

Table 6.46.2.1-1 lists Band n41 +Band n74 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.46.2.2-1: Band n41 and Band n74 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 2496 | 2690 | 1427 | 1470 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 1026 | 1263 | 3923 | 4160 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 3522 | 3953 | 164 | 444 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 6419 | 6850 | 5350 | 5630 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 6018 | 6643 | 1591 | 1914 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 8915 | 9540 | 6777 | 7100 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 2052 | 2526 | 7846 | 8320 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 3018 | 3384 | 8514 | 9333 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 8204 | 8570 | 11411 | 12230 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 582 | 1099 | 4548 | 5216 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 9273 | 9790 | 10342 | 11010 |

Based on above, there is no IMD issue for band n41 is TDD.

Table 6.46.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.46.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n41-n74 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 17, 18, 19, 26, 28, 29, 34, 39, 42, 48, 52, 65, 66, 85 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 21 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 21: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth |  |  |  |  |  |  |  |

6.46.2.3	REFSENS requirements

Based on Table 6.46.2.2-1, there is no MSD exception requirement.

## 6.47	CA_n34-n40

- 6.47.1		Common for 1 band UL and 2 bands UL CA

#### 6.47.1.1		Operating bands for CA

Table 6.47.1.1-1: CA band combination of band n34+n40

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n34-n40x | n34 | 2010 MHz | – | 2025 MHz | 2010MHz | – | 2025 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| Note x: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |  |  |  |  |  |  |

#### 6.47.1.2		Channel bandwidths per operating band for CA

Table 6.47.2-1: Supported bandwidths per CA band combination of band n34+n40

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n34A-n40A | CA_n34A-n40A | n34 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 |  |  |  |

#### 6.47.1.3		Co-existence studies

Table 6.47.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n34-n40.

Table 6.47.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 |  |  |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 |  |  |

Based on above table, there is no harmonic issue for the band combination of n34 and n40.

Table 6.47.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 |  |  |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n34 and n40.

#### 6.47.1.4		∆TIB and ∆RIB values

For CA_n34-n40, it is proposed to use the same TIB,c and RIB values of CA_n39-n40, which are given in the tables below.

Table 6.47.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n34-n40 | n34 | 0 |
|  | n40 | 0 |

Table 6.47.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n34-n40 | n34 | 0.3 |
|  | n40 | 0.3 |

- 6.47.1.5		REFSEN requirements

According to the analysis in subclause 6.47.1.3, there are no harmonic issues for this combination. Hence it is no need to define the MSD values caused by harmonic issues for NR CA n34+n40 band combination.

- 6.47.2		Specific for 2 bands UL CA

      - 6.47.2.1	Maximum output power for inter-band CA

Table 6.24.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n34A-n40A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

- 6.47.2.2		UE co-existence studies

Due to both bands n34 and n40 are TDD bands, and only non-simultaneous Rx/Tx is supported for band n34+n40 inter-band NR CA band combinations. Therefore no IMDs issues for NR CA n34+n40 band combination.

Table 6.47.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.47.2.1-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n34-n40 | E-UTRA Band 1, 3, 7, 8, 20, 22, 26, 28, 31, 32, 33, 38, 39, 41, 42, 43, 44, 45, 50, 51, 65, 67, 69, 72, 73, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz.. |  |  |  |  |  |  |  |

- 6.47.2.3		REFSENS requirements

No MSD issues for NR CA n34+n40 band combination.

6.48	CA_n13-n77

6.48.1	Common for 1 band UL and 2 bands UL CA

6.48.1.1 Operating bands for CA

Table 6.48.1.1-1: CA band combination of band n13 + n77

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n13-n77 | n13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.48.1.2	Channel bandwidths per operating band for CA

Table 6.48.1.2-1: Supported bandwidths per CA band combination of band n13+n77

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n13A-n77A | CA_n13A-n77A | n13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.48.1.3	Co-existence studies

Table 6.48.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n13-n77.

Table 6.48.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1554 | 1574 | 2331 | 2361 | 3108 | 3148 | 3885 | 3935 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is a 5th harmonic issue.

Table 6.48.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1492 | 1512 | 2238 | 2268 | 2984 | 3024 | 3730 | 3780 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is a 5th harmonic mixing issue for CA_n13-n77.

6.48.1.4	∆TIB and ∆RIB values

For CA_n13-n77, the TIB,c and RIB values are given in the tables below.

Table 6.48.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n13-n77 | n71 | 0.5 |
|  | n78 | 0.8 |

Table 6.48.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n13-n77 | n71 | 0.2 |
|  | n78 | 0.5 |

6.48.1.5	REFSENs requirements

Based on the table Table 6.48.1.3-1, the n13 5th harmonics may fall in to the own RX band of band n77. The MSD value can refer to CA_n28-n77 and DC_13_n77.

Table 6.48.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB |  | dB | dB | dB |
| n13 | n776,7 |  | 10.4 | 8.9 | 7.8 | 6.7 | 6 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 7: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.48.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n13 | n77 |  | 10 | 15 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |

Based on the table Table 6.48.1.3-2, there is a 5th harmonic mixing issue. The MSD value can refer to DC_13_n77.

Table 6.48.1.5-3: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n135 | 31 | 28 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.48.1.5-4: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | n13 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |

6.48.1.6	OOB blocking exception requirements

Table 6.48.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n13-n77

6.48.2		Specific for 2 bands UL CA

6.48.2.1	Maximum output power for inter-band CA

Table 6.48.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n13A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.48.2.2		UE co-existence

Table 6.48.2.2-1 gives IMD interference analysis for CA_ n13-n77 with 2 ULs.

Table 6.48.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 3300 | 4200 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2513 | 3423 | 4077 | 4987 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2646 | 1726 | 5813 | 7623 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4854 | 5774 | 7377 | 9187 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1869 | 939 | 9113 | 11823 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 5631 | 6561 | 10677 | 13377 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 5026 | 6846 | 8154 | 9974 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1092 | 152 | 12413 | 16023 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 6408 | 7348 | 13977 | 17587 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 6069 | 4239 | 8326 | 11046 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 11454 | 14174 | 8931 | 10761 |

Table 6.48.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.48.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n13-n77 | E-UTRA Band 2, 5, 7, 12, 13, 25, 26, 41, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.48.2.3		REFSENS requirements

Based on the table 6.48.2.2-1, two-tone 5th order IMD products may fall into the own Rx Band of Band n13. The MSD value can be reused from CA_71A-n78A.

Table 6.48.2.3: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n13A-n77A | n13 | 782 | 5 | 20 | 751 | 5.5 | FDD | IMD5 |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |

6.49	CA_n2-n7

6.49.1	Common for 1 band UL and 2 bands UL CA

6.49.1.1 Operating bands for CA

Table 6.49.1.1-1: CA band combination of band n38 + n66

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2-n7 | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

6.49.1.2	Channel bandwidths per operating band for CA

Table 6.49.1.2-1: Supported bandwidths per CA band combination of band n2+n78

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n2A-n7A | CA_n2A-n7A | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| CA_n2A-n7(2A) | CA_n2A-n7A | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.49.1.3	Co-existence studies

Table 6.49.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n2-n7.

Table 6.49.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 |

Based on above table, there is no harmonic interference.

Table 6.49.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 |

Based on above table, there is no harmonic mixing issue.

6.49.1.4	∆TIB and ∆RIB values

For CA_n2-n7, the TIB,c and RIB values are given in the tables below.

Table 6.49.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n7 | n2 | 0.5 |
|  | n7 | 0.5 |

Table 6.49.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n2-n7 | n2 | 0 |
|  | n7 | 0 |

6.49.1.5	REFSENs requirements

There is no harmonic issue for the CA combination.

6.49.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.49.2		Specific for 2 bands UL CA

6.49.2.1	Maximum output power for inter-band CA

Table 6.49.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n7A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.49.2.2		UE co-existence

Table 6.49.2.2-1 gives IMD interference analysis for CA_ n2-n7 with 2 ULs.

Table 6.49.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2500 | 2570 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 590 | 720 | 4350 | 4480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 1130 | 1320 | 3090 | 3290 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6200 | 6390 | 6850 | 7050 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 2980 | 3230 | 5590 | 5860 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8050 | 8300 | 9350 | 9560 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1180 | 1440 | 8700 | 8960 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 4830 | 5140 | 8090 | 8430 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9900 | 10210 | 11850 | 12190 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 410 | 730 | 3680 | 4010 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 11200 | 11530 | 10550 | 10870 |

Table 6.49.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.49.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n2-n7 | E-UTRA Band 2, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 18: This requirement is applicable for any channel bandwidths within the range 2500 – 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 – 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.49.2.3		REFSENS requirements

There is no MSD issue for the CA combination.

6.50	CA_n24-n41

6.50.1		Common for 1 band UL and 2 bands UL CA

6.50.1.1		Operating bands for CA

Table 6.50.1.1-1: CA band combination of band n24+n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.50.1.2		Channel bandwidths per operating band for CA

Table 6.50.1.2-1: Supported bandwidths per CA band combination of band n24+n41

| NR CA Configuration | UL Configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n24A-n41A | CA_n24A-n41A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n24A-n41(2A) | CA_n24A-n41A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) BCS1 in Table 5.5A.2-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.50.1.3		Co-existence studies

Table 6.50.1.3-1 summarizes frequency ranges where harmonics occur for CA_n24-n41.

Table 6.50.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |  |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3253 | 3321 | 4879.5 | 4981.5 | 6506 | 6642 | 8132.5 | 8302.5 |  |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |  |

Based on above table, there is no harmonic issue for the band combination of n24 and n41.

Table 6.50.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n24-n41.

Table 6.50.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3050 | 3118 | 4575 | 4677 | 6100 | 6236 | 7625 | 7795 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

Based on above table, there is no harmonic mixing issue for the band combination of n24 and n41.

6.50.1.4		∆TIB and ∆RIB values

For CA_n24-n41 , the ∆TIB,c and ∆RIB,c values are given according to CA_n25-n41 relaxation values.

Table 6.50.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n24-n41 | n24 | 0.3 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.50.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n24-n41 | n24 | 0 |
|  | n41 | 0 |

6.50.1.5		REFSENS requirements

No additional MSD requirement need to be defined for this CA band combination.

6.50.1.6		OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.50.2		Specific for 2 bands UL CA

6.50.2.1	Maximum output power for inter-band CA

Table 6.50.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n24A-n41A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.50.2.2		UE co-existence studies

Table 6.50.2.2-1: Impact of Intermodulations

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1626.5 | 1660.5 | 2496 | 2690 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 835.5 | 1063.5 | 4122.5 | 4350.5 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 563 | 825 | 3331.5 | 3753.5 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5749 | 6011 | 6618.5 | 7040.5 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 2189.5 | 2485.5 | 5827.5 | 6443.5 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 7375.5 | 7671.5 | 9114.5 | 9730.5 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1671 | 2127 | 8245 | 8701 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 3816 | 4146 | 8323.5 | 9133.5 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9002 | 9332 | 11610.5 | 12420.5 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 500.5 | 10.5 | 4167 | 4817 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 10741 | 11391 | 9871.5 | 10361.5 |

Based on the table above, there are no IMDs falling on top of n24 or n41 DL, and therefore no additional MSD needs to be specified.

Table 6.50.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.50.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n24-n41 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.50.2.3		REFSENS requirements

No MSD needs to be specified.

6.51	CA_n24-n48

### 6.51.1		Common for 1 band UL and 2 bands UL CA

6.51.1.1		Operating bands for CA

Table 6.51.1.1-1: CA band combination of band n24+n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

6.51.1.2		Channel bandwidths per operating band for CA

Table 6.51.1.2-1: Supported bandwidths per CA band combination of band n24+n48

| NR CA Configuration | UL Configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n24A-n48A | CA_n24A-n48A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n24A-n48B | CA_n24A-n48A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B BCS1 in Table 5.5A.1-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n24A-n48(2A) | CA_n24A-n48A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) BCS0 in Table 5.5A.2-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n24A-n48(3A) | CA_n24A-n48A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(3A) BCS0 in Table 5.5A.2-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.51.1.3		Co-existence studies

Table 6.51.1.3-1 summarizes frequency ranges where harmonics occur for CA_n24-n48.

Table 6.51.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |  |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3253 | 3321 | 4879.5 | 4981.5 | 6506 | 6642 | 8132.5 | 8302.5 |  |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |  |

Based on above table, there is no harmonic issue for the band combination of n24 and n48.

Table 6.51.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n24-n48.

Table 6.51.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3050 | 3118 | 4575 | 4677 | 6100 | 6236 | 7625 | 7795 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |

Based on above table, there is no harmonic mixing issue for the band combination of n24 and n48.

6.51.1.4		∆TIB and ∆RIB values

For CA_n24-n48 , the ∆TIB,c and ∆RIB,c values are according to CA_n48-n66 relaxation values.

Table 6.51.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n24-n48 | n24 | 0.6 |
|  | n48 | 0.8 |

Table 6.51.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n24-n48 | n24 | 0.2 |
|  | n48 | 0.5 |

6.51.1.5	   	REFSENS requirements

No additional MSD requirement need to be defined for this band combination.

6.51.1.6	   OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.51.2		Specific for 2 bands UL CA

6.51.2.1	   Maximum output power for inter-band CA

Table 6.51.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n24A-n48A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.51.2.2	   UE co-existence studies

Table 6.51.2.2-1: Impact of Intermodulations

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1626.5 | 1660.5 | 3550 | 3700 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1889.5 | 2073.5 | 5176.5 | 5360.5 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 447 | 229 | 5439.5 | 5773.5 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6803 | 7021 | 8726.5 | 9060.5 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1179.5 | 1431.5 | 8989.5 | 9473.5 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8429.5 | 8681.5 | 12276.5 | 12760.5 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3779 | 4147 | 10353 | 10721 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 2806 | 3092 | 12539.5 | 13173.5 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 10056 | 10342 | 15826.5 | 16460.5 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 2520.5 | 2118.5 | 7329 | 7847 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 13903 | 14421 | 11979.5 | 12381.5 |

Based on the table above, there are no IMDs falling on top of n24 or n48 DL, and therefore no additional MSD needs to be specified.

Table 6.51.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.51.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n24-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 |  |  |  |  |  |  |  |

6.51.2.3	   REFSENS requirements

No MSD needs to be specified.

6.52	CA_n24-n77

### 6.52.1		Common for 1 band UL and 2 bands UL CA

6.52.1.1		Operating bands for CA

Table 6.52.1.1-1: CA band combination of band n24+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.52.1.2		Channel bandwidths per operating band for CA

Table 6.52.1.2-1: Supported bandwidths per CA band combination of band n24+n77

| NR CA Configuration | UL Configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n24A-n77A | CA_n24A-n77A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 25 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n24A-n77C | CA_n24A-n77A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77C BCS1 in Table 5.5A.1-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n24A-n77(2A) | CA_n24A-n77A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) BCS0 in Table 5.5A.2-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.52.1.3		Co-existence studies

Table 6.52.1.3-1 summarizes frequency ranges where harmonics occur for CA_n24-n77.

Table 6.52.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |  |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3253 | 3321 | 4879.5 | 4981.5 | 6506 | 6642 | 8132.5 | 8302.5 |  |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |  |

Based on above table, the 2nd harmonic of n24 UL may fall into the lower DL frequency range of n77.

Table 6.52.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n24-n77.

Table 6.52.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3050 | 3118 | 4575 | 4677 | 6100 | 6236 | 7625 | 7795 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is no harmonic mixing issue for the band combination of n24 and n77.

6.52.1.4		∆TIB and ∆RIB values

For CA_n24-n77, the ∆TIB,c and ∆RIB,c values are given according to CA_n66-n77 relaxation values.

Table 6.52.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n24-n77 | n24 | 0.6 |
|  | n77 | 0.8 |

Table 6.52.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n24-n77 | n24 | 0.2 |
|  | n77 | 0.5 |

6.52.1.5		REFSENS requirements

MSD values for Band n77 due to 2nd harmonic of Band n24 in CA_n24A-n77A are captured in Table 6.52.1.5-1 according to CA_n66-n77 values which also had a similar 2nd harmonic of n66 falling in n77 DL frequeny range.

Table 6.52.1.5-1: MSD due to UL harmonic issue for CA_n24-n77

|  | NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n24 | n771, 2 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.3 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n78, CA_n48-n66, CA_n66-n78, CA_n66-n77, n24-n77. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The uplink configuration for reference sensitivity exceptions due to UL harmonic interference for CA_n24A-n77A are captured in Table 6.52.1.5-2.

Table 6.52.1.5-2 Uplink configuration due to UL harmonic interference

|  |  | NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n24 | n77 | 12 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |

6.52.1.6		OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.52.2		Specific for 2 bands UL CA

6.52.2.1	Maximum output power for inter-band CA

Table 6.52.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n24A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.52.2.2		UE co-existence studies

Table 6.52.2.2-1: Impact of Intermodulations

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1626.5 | 1660.5 | 3300 | 4200 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1639.5 | 2573.5 | 4926.5 | 5860.5 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 947 | 21 | 4939.5 | 6773.5 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6553 | 7521 | 8226.5 | 10060.5 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 679.5 | 1681.5 | 8239.5 | 10973.5 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8179.5 | 9181.5 | 11526.5 | 14260.5 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3279 | 5147 | 9853 | 11721 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 2306 | 3342 | 11539.5 | 15173.5 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9806 | 10842 | 14826.5 | 18460.5 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 3520.5 | 1618.5 | 6579 | 9347 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 13153 | 15921 | 11479.5 | 13381.5 |

No IMD impact to the n77 is expected since it is TDD band. Based on the table above, the 4th order IMDs may fall into Rx frequencies of band n24.  However, within USA, the frequency of operation of n77 is going to be above 3450 MHz.  Moreover, n24 uplink operation is restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz.  For this combination of n77 and n24 UL operating frequency ranges, there are no IMDs falling on top of n24 Rx frequencies, and therefore no additional MSD needs to be specified for n24.

Table 6.52.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.52.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n24-n77 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 |  |  |  |  |  |  |  |

6.52.2.3		REFSENS requirements

Table 6.52.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  |
| CA_n24A-n77A1 | n24 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| Note 1: There is no IMD4 product in band n24 downlink for n77 operating in 3450 – 3980 MHz and n24 uplink restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz. |  |  |  |  |  |  |  |

6.53	CA_n28-n71

6.53.1	Common for 1 band UL and 2 bands UL CA

6.53.1.1 Operating bands for CA

Table 6.53.1.1-1: CA band combination CA_n28-n71

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n71x | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| NOTE X: The implementation with 4 antennas is targeted for FWA form factor for this band combination. |  |  |  |  |  |  |  |  |

6.53.1.2	Channel bandwidths per operating band for CA

Table 6.53.1.2-1: Supported bandwidths per CA band combination CA_n28-n71

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n71A | - | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

6.53.1.3	UE Co-existence studies

Table 6.53.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n28-n71.

Table 6.53.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 |

Based on above table, there is no harmonic interference.

Table 6.53.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 |

Based on above table, there is no harmonic mixing issue.

6.53.1.4	∆TIB and ∆RIB values

For CA_n28A-n71A, the TIB,c and RIB are given in the tables below.

Table 6.53.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n71 | n28 | 1.1 |
|  | n71 | 1.1 |

Table 6.53.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n28-n71 | n28 | 0.7 |
|  | n71 | 0.7 |

6.53.1.5	REFSENs requirements

There is no MSD exception requirement for CA_n28-n71.

6.53.1.6	OOB blocking exception requirements

There is no MSD exception requirement for CA_n28-n71.

6.54	CA_n2-n30

6.54.1	Common for 1 band UL and 2 bands UL CA

6.54.1.1		Operating bands for CA

Table 6.54.1.1-1: CA band combination of band n2 and n30

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |

6.54.1.2		Channel bandwidths per operating band for CA

Table 6.54.1.2-1: Supported bandwidths per CA band combination of band n2 and n30

| NR CA configuration | UL | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n30A | CA_n2A-n30A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n30A | CA_n2A-n30A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

6.54.1.3		UE co-existence studies

Table 6.54.1.3-1 lists up to 7th harmonics for n2A-n30A which shows that there are no harmonics issues.

Table 6.54.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 | 9250 | 9550 | 11100 | 11460 | 12950 | 13370 |
| n30 | 2305 | 2315 | 4610 | 4630 | 6915 | 6945 | 9220 | 9260 | 11525 | 11575 | 13830 | 13890 | 16135 | 16205 |

Table 6.54.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.54.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 |
| n30 | 2305 | 2315 | 2350 | 2360 | 4700 | 4720 | 7050 | 7080 | 9400 | 9440 |

6.54.1.4		∆TIB and ∆RIB values

For CA_n2-n30, the TIB,c and RIB,c values are same as for DC_30_n2 and are given in the tables below.

Table 6.54.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n30 | n2 | 0.5 |
|  | n30 | 0.3 |

Table 6.54.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2-n30 | n2 | 0 |
|  | n30 | 0 |

6.54.1.5 		REFSENS requirements

There is no harmonic issue for the CA combination.

6.54.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n2-n30.

Table 6.54.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.54.2		Specific for 2 bands UL CA

6.54.2.1	Maximum output power for inter-band CA

Table 6.54.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n30A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.54.2.2		UE co-existence studies

Table 6.54.2.2-1 lists Band n2 + Band n30 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.54.2.2-1: Band n2 and Band n30 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 2305 | 2315 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 465 | 395 | 4155 | 4225 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1385 | 1515 | 2700 | 2780 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6005 | 6135 | 6460 | 6540 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 930 | 790 | 8310 | 8450 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3235 | 3425 | 5005 | 5095 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7855 | 8045 | 8765 | 8855 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7410 | 7310 | 5335 | 5085 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11070 | 11170 | 9705 | 9955 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3245 | 3095 | 1120 | 920 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10615 | 10765 | 10160 | 10360 |

Based on the table above:

- The are no IMD that fall into Rx frequencies of band n2.

- The are no IMD that fall into Rx frequencies of band n30

Table 6.54.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.54.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n2-n30 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 41, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85 NR band n30 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | NR Band n2 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.54.2.3		REFSENS requirements

There is no need to defined MSD requirements.

6.55	CA_n5-n30

6.55.1	Common for 1 band UL and 2 bands UL CA

6.55.1.1		Operating bands for CA

Table 6.55.1.1-1: CA band combination of band n5 and n30

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |

6.55.1.2		Channel bandwidths per operating band for CA

Table 6.55.1.2-1: Supported bandwidths per CA band combination of band n5 and n30

| NR CA configuration | UL | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n30A | CA_n5A-n30A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

6.55.1.3		UE co-existence studies

Table 6.55.1.3-1 lists up to 7th harmonics for n5A-n30A which shows that there are no harmonics issues.

Table 6.55.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 | 4944 | 5094 | 5768 | 5943 |
| n30 | 2305 | 2315 | 4610 | 4630 | 6915 | 6945 | 9220 | 9260 | 11525 | 11575 | 13830 | 13890 | 16135 | 16205 |

Table 6.55.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.55.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 |
| n30 | 2305 | 2315 | 2350 | 2360 | 4700 | 4720 | 7050 | 7080 | 9400 | 9440 |

6.55.1.4		∆TIB and ∆RIB values

For CA_n5-n30, the TIB,c and RIB,c values are same as for DC_30_n5 and are given in the tables below.

Table 6.55.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n30 | n5 | 0.3 |
|  | n30 | 0.3 |

Table 6.55.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n30 | n5 | 0 |
|  | n30 | 0 |

6.55.1.5 		REFSENS requirements

There is no harmonic issue for the CA combination.

6.55.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n5-n30.

Table 6.55.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.55.2		Specific for 2 bands UL CA

6.55.2.1	Maximum output power for inter-band CA

Table 6.55.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n30A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.55.2.2		UE co-existence studies

Table 6.55.2.2-1 lists Band n5 + Band n30 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.55.2.2-1: Band n5 and Band n30 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 2305 | 2315 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1491 | 1456 | 3129 | 3164 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 667 | 607 | 3761 | 3806 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3953 | 4013 | 5434 | 5479 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2982 | 2912 | 6258 | 6328 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 157 | 242 | 6066 | 6121 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4777 | 4862 | 7739 | 7794 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8436 | 8371 | 1091 | 981 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10044 | 10109 | 5601 | 5711 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5297 | 5217 | 2063 | 2158 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8563 | 8643 | 7082 | 7177 |

Based on the table above:

- The are no IMD that fall into Rx frequencies of band n5.

- The are no IMD that fall into Rx frequencies of band n30

Table 6.55.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.55.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n30 | E-UTRA Band 2, 4, 7, 12, 13, 14, 17, 24, 25, 26, 29, 38, 48, 66, 70, 71, 85 NR band n5, 30 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.55.2.3		REFSENS requirements

There is no need to defined MSD requirements.

6.56	CA_n30-n66

6.56.1	Common for 1 band UL and 2 bands UL CA

6.56.1.1		Operating bands for CA

Table 6.56.1.1-1: CA band combination of band n30 and n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.56.1.2		Channel bandwidths per operating band for CA

Table 6.56.1.2-1: Supported bandwidths per CA band combination of band n30 and n66

| NR CA configuration | UL | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n30A-n66A | CA_n30A-n66A | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n30A-n66(2A) | CA_n30A-n66A | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n30A-n66(3A) | CA_n30A-n66A | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(3A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.56.1.3		UE co-existence studies

Table 6.56.1.3-1 lists up to 7th harmonics for n30A-n66A which shows that there are no harmonics issues.

Table 6.56.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n30 | 2305 | 2315 | 4610 | 4630 | 6915 | 6945 | 9220 | 9260 | 11525 | 11575 | 13830 | 13890 | 16135 | 16205 |
| n66 | 1710 | 1780 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 | 8550 | 8900 | 10260 | 10680 | 11970 | 12460 |

Table 6.56.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.56.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n30 | 2305 | 2315 | 2350 | 2360 | 4700 | 4720 | 7050 | 7080 | 9400 | 9440 |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 |

6.56.1.4		∆TIB and ∆RIB values

For CA_n30-n66, the TIB,c and RIB,c values are same as for DC_30_n66 and are given in the tables below.

Table 6.56.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n30-n66 | n30 | 0.5 |
|  | n66 | 0.8 |

Table 6.56.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n30-n66 | n30 | 0.5 |
|  | n66 | 0.4 |

6.56.1.5 		REFSENS requirements

There is no harmonic issue for the CA combination.

6.56.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n30-n66.

Table 6.56.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.56.2		Specific for 2 bands UL CA

6.56.2.1	Maximum output power for inter-band CA

Table 6.56.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n30A-n66A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.56.2.2		UE co-existence studies

Table 6.56.2.2-1 lists Band n30 + Band n66 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.56.2.2-1: Band n30 and Band n66 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 2305 | 2315 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 605 | 525 | 4015 | 4095 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1105 | 1255 | 2830 | 2920 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5725 | 5875 | 6320 | 6410 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1210 | 1050 | 8030 | 8190 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2815 | 3035 | 5135 | 5235 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7435 | 7655 | 8625 | 8725 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7550 | 7440 | 4815 | 4525 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10930 | 11040 | 9145 | 9435 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3525 | 3355 | 730 | 500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10335 | 10505 | 9740 | 9970 |

Based on the table above:

- The are no IMD that fall into Rx frequencies of band n30.

- The are no IMD that fall into Rx frequencies of band n66

Table 6.56.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.56.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n30-n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 38, 41, 70, 71 NR band n30, n66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.56.2.3		REFSENS requirements

There is no need to defined MSD requirements.

6.57	CA_n48-n53

6.57.1	Common for 1 band UL and 2 bands UL CA

6.57.1.1		Operating bands for CA

Table 6.57.1.1-1: CA band combination of band n48 and n53

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| n53 | 2483.5 MHz | – | 2495 MHz | 2483.5 MHz | – | 2495 MHz | TDD |

6.57.1.2		Channel bandwidths per operating band for CA

Table 6.57.1.2-1: Supported bandwidths per CA band combination of band n48 and n53

| NR CA configuration | UL | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48A-n53A | - | n48 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n53 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n53A |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n53 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |

6.57.1.3		UE co-existence studies

Table 6.57.1.3-1 lists up to 7th harmonics for n48A-n53A which shows that there are no harmonics issues.

Table 6.57.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 | 21300 | 22200 | 24850 | 25900 |
| n53 | 2483.5 | 2495 | 4967 | 4990 | 7450.5 | 7485 | 9934 | 9980 | 12417.5 | 12475 | 14901 | 14970 | 17384.5 | 17465 |

Table 6.57.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.57.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 |
| n53 | 2483.5 | 2495 | 2483.5 | 4967 | 4990 | 7450.5 | 7485 | 9934 | 9980 | 4967 |

6.57.1.4		∆TIB and ∆RIB values

For CA_n48-n53, the TIB,c and RIB,c values are derived from CA_41-48 and are given in the tables below.

Table 6.57.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n48-n53 | n48 | 0.53 |
|  | n53 | 0.33 |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

Table 6.57.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n48-n53 | n48 | 0.53 |
|  | n53 | 0 |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

6.57.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 6.57.1.3 there are no harmonics issues.

6.57.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n48-n53 since it belongs to H-H configuration-

Table 6.57.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.58		CA_n2-n12

6.58.1	Common for 1 band UL and 2 bands UL CA

6.58.1.1 Operating bands for CA

Table 6.58.1-1: CA band combination of band CA_n2 + n12

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2-n12 | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |

6.58.1.2	Channel bandwidths per operating band for CA

Table 6.58.1.2-1: Supported bandwidths per CA band combination of band CA_n2 + n12

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n12A | CA_n2A-n12A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |

6.58.1.3	Co-existence studies

Table 6.58.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n2-n12.

Table 6.58.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 | 9250 | 9550 |
| n12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 2796 | 2864 | 3495 | 3580 |

Based on above table, there is no harmonic issue for the band combination CA_n2-n12.

Table 6.58.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 | 9650 | 9550 |
| n12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 | 2916 | 2984 | 3645 | 3730 |

Based on above table, there is no harmonic mixing issue for the band combination CA_n2-n12.

6.58.1.4	∆TIB and ∆RIB values

For CA_n2-n12, the TIB,c and RIB,c values are reused from the EN-DC combination DC_12_n2, and are given in the tables below.

Table 6.58.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n12 | n2 | 0.3 |
|  | n12 | 0.3 |

Table 6.58.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n2-n12 | n2 | 0 |
|  | n12 | 0 |

6.58.1.5	REFSENS requirements

There is no harmonic issue for the CA combination.

6.58.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n2-n12.

Table 6.58.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.58.2		Specific for 2 bands UL CA

6.58.2.1	Maximum output power for inter-band CA

Table 6.58.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n12A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.58.2.2		UE co-existence

Table 6.58.2.2-1 gives IMD interference analysis for CA_ n2-n12 with 2 ULs.

Table 6.58.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1850 | 1910 |
| DL frequency (MHz) | 729 | 746 | 1930 | 1990 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 9250 | 9550 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1134 | 1211 | 2549 | 2626 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 512 | 418 | 2984 | 3121 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3248 | 3342 | 4399 | 4536 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 187 | 298 | 4834 | 5031 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3947 | 4058 | 6249 | 6446 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2422 | 2268 | 5098 | 5252 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6941 | 6684 | 1014 | 886 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8099 | 8356 | 4646 | 4774 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4332 | 4118 | 1552 | 1723 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6948 | 7162 | 5797 | 5968 |

Based on the table above:

- There are no IMD products that fall into Rx frequencies of band n2.

- There are no IMD products that fall into Rx frequencies of band n12.

Table 6.58.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.58.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n2-n12 | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 50, 53, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 12, 25, 85 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 4, 10, 51, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.58.2.3		REFSENS requirements

Based on the co-existence studies for CA_n2-n12, there is no need to have MSD added.

6.59		CA_n2-n14

6.59.1	Common for 1 band UL and 2 bands UL CA

6.59.1.1 Operating bands for CA

Table 6.59.1-1: CA band combination of band CA_n2 + n14

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2-n14 | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |

6.59.1.2	Channel bandwidths per operating band for CA

Table 6.59.1.2-1: Supported bandwidths per CA band combination of band CA_n2 + n14

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n14A | CA_n2A-n14A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

6.59.1.3	Co-existence studies

Table 6.59.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n2-n14.

Table 6.59.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 | 9250 | 9550 |
| n14 | 788 | 798 | 758 | 768 | 1576 | 1596 | 2364 | 2394 | 3152 | 3192 | 3940 | 3990 |

Based on above table, there is no harmonic issue for the band combination CA_n2-n14.

Table 6.59.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 | 9650 | 9550 |
| n14 | 788 | 798 | 758 | 768 | 1516 | 1536 | 2274 | 2304 | 3032 | 3072 | 3790 | 3840 |

Based on above table, there is no harmonic mixing issue for the band combination CA_n2-n14.

6.59.1.4	∆TIB and ∆RIB values

For CA_n2-n14, the TIB,c and RIB,c values are reused from the EN-DC combination DC_14_n2, and are given in the tables below.

Table 6.59.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n14 | n2 | 0.3 |
|  | n14 | 0.3 |

Table 6.59.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n2-n14 | n2 | 0 |
|  | n14 | 0 |

6.59.1.5	REFSENS requirements

There is no harmonic issue for the CA combination.

6.59.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n2-n14.

Table 6.59.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.59.2		Specific for 2 bands UL CA

6.59.2.1	Maximum output power for inter-band CA

Table 6.59.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n14A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.59.2.2		UE co-existence

Table 6.59.2.2-1 gives IMD interference analysis for CA_ n2-n14 with 2 ULs.

Table 6.59.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 788 | 798 | 1850 | 1910 |
| DL frequency (MHz) | 758 | 768 | 1930 | 1990 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1576 | 1596 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2364 | 2394 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3152 | 3192 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3940 | 3990 | 9250 | 9550 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1052 | 1122 | 2638 | 2708 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 334 | 254 | 2902 | 3032 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3426 | 3506 | 4488 | 4618 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 454 | 544 | 4752 | 4942 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4214 | 4304 | 6338 | 6528 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2244 | 2104 | 5276 | 5416 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6852 | 6602 | 1342 | 1242 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8188 | 8438 | 5002 | 5102 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4154 | 3954 | 1306 | 1456 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7126 | 7326 | 6064 | 6214 |

Based on the table above:

- There are no IMD products that fall into Rx frequencies of band n2.

- There are no IMD products that fall into Rx frequencies of band n14.

Table 6.59.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.59.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n2-n14 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.59.2.3		REFSENS requirements

Based on the co-existence studies for CA_n2-n14, there is no need to have MSD added.

6.60		CA_n12-n30

6.60.1	Common for 1 band UL and 2 bands UL CA

6.60.1.1 Operating bands for CA

Table 6.60.1-1: CA band combination of band CA_n12 + n30

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n12-n30 | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |

6.60.1.2	Channel bandwidths per operating band for CA

Table 6.60.1.2-1: Supported bandwidths per CA band combination of band CA_n12 + n30

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n12A-n30A | CA_n12A-n30A | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

6.60.1.3	Co-existence studies

Table 6.60.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n12-n30.

Table 6.60.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 2796 | 2864 | 3495 | 3580 |
| n30 | 2305 | 2315 | 2350 | 2360 | 4610 | 4630 | 6915 | 6945 | 9220 | 9260 | 11525 | 11575 |

Based on above table, there is no harmonic issue for the band combination CA_n12-n30.

Table 6.60.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 | 2916 | 2984 | 3645 | 3730 |
| n30 | 2305 | 2315 | 2350 | 2360 | 4700 | 4720 | 7050 | 7080 | 9400 | 9440 | 11750 | 11800 |

Based on above table, there is no harmonic mixing issue for the band combination CA_n12-n30.

6.60.1.4	∆TIB and ∆RIB values

For CA_n12-n30, the TIB,c and RIB,c values are reused from the EN-DC combination DC_12_n30, and are given in the tables below.

Table 6.60.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n12-n30 | n12 | 0.3 |
|  | n30 | 0.3 |

Table 6.60.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n12-n30 | n12 | 0 |
|  | n30 | 0 |

6.60.1.5	REFSENS requirements

There is no harmonic issue for the CA combination.

6.60.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n12-n30.

Table 6.60.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.60.2		Specific for 2 bands UL CA

6.60.2.1	Maximum output power for inter-band CA

Table 6.60.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n12A-n30A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.60.2.2		UE co-existence

Table 6.60.2.2-1 gives IMD interference analysis for CA_ n12-n30 with 2 ULs.

Table 6.60.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 2305 | 2315 |
| DL frequency (MHz) | 729 | 746 | 2350 | 2360 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 6915 | 6945 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 11525 | 9260 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 11525 | 11575 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1589 | 1616 | 3004 | 3031 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 917 | 873 | 3894 | 3931 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3703 | 3747 | 5309 | 5346 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 218 | 157 | 6199 | 6246 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4402 | 4463 | 7614 | 7661 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3232 | 3178 | 6008 | 6062 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8561 | 8504 | 559 | 481 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9919 | 9976 | 5101 | 5179 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5547 | 5483 | 2462 | 2533 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8313 | 8377 | 6707 | 6778 |

Based on the table above:

- There are no IMD products that fall into Rx frequencies of band n12.

- There are no IMD products that fall into Rx frequencies of band n30.

Table 6.60.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.60.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n12-n30 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.60.2.3		REFSENS requirements

Based on the co-existence studies for CA_n12-n30, there is no need to have MSD added.

6.61		CA_n12-n66

6.61.1	Common for 1 band UL and 2 bands UL CA

6.61.1.1 Operating bands for CA

Table 6.61.1-1: CA band combination of band CA_n12 + n66

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n12-n66 | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.61.1.2	Channel bandwidths per operating band for CA

Table 6.61.1.2-1: Supported bandwidths per CA band combination of band CA_n12 + n66

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n12A-n66A | CA_n12A-n66A | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |

6.61.1.3	Co-existence studies

Table 6.61.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n12-n66.

Table 6.61.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 2796 | 2864 | 3495 | 3580 |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 | 8550 | 8900 |

Based on above table, the 3rd harmonic of n12 UL may fall into the lower DL frequency range of n66.

Table 6.61.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 | 2916 | 2984 | 3645 | 3730 |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 | 10550 | 11000 |

Based on above table, there is no harmonic mixing issue for the band combination CA_n12-n66.

6.61.1.4	∆TIB and ∆RIB values

For CA_n12-n66, the TIB,c and RIB,c values are reused from the EN-DC combination DC_12_n66, and are given in the tables below.

Table 6.61.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n12-n66 | n12 | 0.8 |
|  | n66 | 0.3 |

Table 6.61.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n12-n66 | n12 | 0.5 |
|  | n66 | 0 |

6.61.1.5	REFSENS requirements

Due to the identified harmonic issues, MSD is derived by reusing the values specified for DC_12A-n66A and needs to be defined in 38.101-1 as defined below.

Table 6.61.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n12 | n668,9 | 10 | 7.5 | 6.2 | 5.5 |  |  | 2.4 |  |  |  |  |  |
| NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.61.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n12 | n66 | 8 | 16 | 20 | 20 |  |  | 20 |  |  |  |  |  |

6.61.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n12-n66.

Table 6.61.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.61.2		Specific for 2 bands UL CA

6.61.2.1	Maximum output power for inter-band CA

Table 6.61.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n12A-n66A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.61.2.2		UE co-existence

Table 6.61.2.2-1 gives IMD interference analysis for CA_ n12-n66 with 2 ULs.

Table 6.61.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1710 | 1780 |
| DL frequency (MHz) | 729 | 746 | 2110 | 2200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 8550 | 8900 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 994 | 1081 | 2409 | 2496 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 382 | 278 | 2704 | 2861 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3108 | 3212 | 4119 | 4276 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 317 | 438 | 4414 | 4641 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3807 | 3928 | 5829 | 6056 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2162 | 1988 | 4818 | 4992 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6421 | 6124 | 1154 | 1016 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7539 | 7836 | 4506 | 4644 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3942 | 3698 | 1272 | 1463 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6528 | 6772 | 5517 | 5708 |

Based on the table above:

- There are no IMD products that fall into Rx frequencies of band n12.

- There is a 4th order IMD product that may fall into Rx frequencies of band n66.

Table 6.61.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.61.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n12-n66 | E-UTRA Band 2, 5, 13, 14, 17, 25, 26, 27, 30, 41, 53, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.61.2.3		REFSENS requirements

Based on the co-existence studies for CA_n12-n66, MSD due to IMD4 is required. The MSD values can be reused from CA_n66-n71.

Table 6.61.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  |
| CA_n12A-n66A | n12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n66 | 1765 | 5 | 25 | 2115 | 5.0 | IMD4 |

6.62		CA_n14-n30

6.62.1	Common for 1 band UL and 2 bands UL CA

6.62.1.1 Operating bands for CA

Table 6.62.1-1: CA band combination of band CA_n14 + n30

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n14-n30 | n14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
|  | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |

6.62.1.2	Channel bandwidths per operating band for CA

Table 6.62.1.2-1: Supported bandwidths per CA band combination of band CA_n14 + n30

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n14A-n30A | CA_n14A-n30A | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

6.62.1.3	Co-existence studies

Table 6.62.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n14-n30.

Table 6.62.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n14 | 788 | 798 | 758 | 768 | 1576 | 1596 | 2364 | 2394 | 3152 | 3192 | 3940 | 3990 |
| n30 | 2305 | 2315 | 2350 | 2360 | 4610 | 4630 | 6915 | 6945 | 9220 | 9260 | 11525 | 11575 |

Based on above table, there is no harmonic issue for the band combination CA_n14-n30.

Table 6.62.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n14 | 788 | 798 | 758 | 768 | 1516 | 1536 | 2274 | 2304 | 3032 | 3072 | 3790 | 3840 |
| n30 | 2305 | 2315 | 2350 | 2360 | 4700 | 4720 | 7050 | 7080 | 9400 | 9440 | 11750 | 11800 |

Based on above table, there is no harmonic mixing issue for the band combination CA_n14-n30.

6.62.1.4	∆TIB and ∆RIB values

For CA_n14-n30, the TIB,c and RIB,c values are reused from the EN-DC combination DC_14_n30, and are given in the tables below.

Table 6.62.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n14-n30 | n14 | 0.3 |
|  | n30 | 0.3 |

Table 6.62.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n14-n30 | n14 | 0 |
|  | n30 | 0 |

6.62.1.5	REFSENS requirements

There is no harmonic issue for the CA combination.

6.62.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n14-n30.

Table 6.62.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.62.2		Specific for 2 bands UL CA

6.62.2.1	Maximum output power for inter-band CA

Table 6.62.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n14A-n30A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.62.2.2		UE co-existence

Table 6.62.2.2-1 gives IMD interference analysis for CA_ n14-n30 with 2 ULs.

Table 6.62.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 788 | 798 | 2305 | 2315 |
| DL frequency (MHz) | 758 | 768 | 2350 | 2360 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1576 | 1596 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2364 | 2394 | 6915 | 6945 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3152 | 3192 | 9220 | 9260 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3940 | 3990 | 11525 | 11575 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1507 | 1527 | 3093 | 3113 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 739 | 709 | 3812 | 3842 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3881 | 3911 | 5398 | 5428 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 49 | 89 | 6117 | 6157 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4669 | 4709 | 7703 | 7743 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3054 | 3014 | 6186 | 6226 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8472 | 8422 | 887 | 837 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10008 | 10058 | 5457 | 5507 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5369 | 5319 | 2216 | 2266 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8491 | 8541 | 6974 | 7024 |

Based on the table above:

- There are no IMD products that fall into Rx frequencies of band n14.

- There are no IMD products that fall into Rx frequencies of band n30.

Table 6.62.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.62.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n14-n30 | E-UTRA Band 2, 4, 5,12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.62.2.3		REFSENS requirements

Based on the co-existence studies for CA_n14-n30, there is no need to have MSD added.

6.63		CA_n14-n66

6.63.1	Common for 1 band UL and 2 bands UL CA

6.63.1.1 Operating bands for CA

Table 6.63.1-1: CA band combination of band CA_n14 + n66

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n14-n66 | n14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.63.1.2	Channel bandwidths per operating band for CA

Table 6.63.1.2-1: Supported bandwidths per CA band combination of band CA_n14 + n66

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n14A-n66A | CA_n14A-n66A | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

6.63.1.3	Co-existence studies

Table 6.63.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n14-n66.

Table 6.63.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n14 | 788 | 798 | 758 | 768 | 1576 | 1596 | 2364 | 2394 | 3152 | 3192 | 3940 | 3990 |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 | 8550 | 8900 |

Based on above table, there is no harmonic issue for the band combination CA_n14-n66.

Table 6.63.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n14 | 788 | 798 | 758 | 768 | 1516 | 1536 | 2274 | 2304 | 3032 | 3072 | 3790 | 3840 |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 | 10550 | 11000 |

Based on above table, there is no harmonic mixing issue for the band combination CA_n14-n66.

6.63.1.4	∆TIB and ∆RIB values

For CA_n14-n66, the TIB,c and RIB,c values are reused from the EN-DC combination DC_14_n66, and are given in the tables below.

Table 6.63.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n14-n66 | n14 | 0.3 |
|  | n66 | 0.3 |

Table 6.63.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n14-n66 | n14 | 0 |
|  | n66 | 0 |

6.63.1.5	REFSENS requirements

There is no harmonic issue for the CA combination.

6.63.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n14-n66.

Table 6.63.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.63.2		Specific for 2 bands UL CA

6.63.2.1	Maximum output power for inter-band CA

Table 6.63.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n14A-n66A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.63.2.2		UE co-existence

Table 6.63.2.2-1 gives IMD interference analysis for CA_ n14-n66 with 2 ULs.

Table 6.63.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 788 | 798 | 1710 | 1780 |
| DL frequency (MHz) | 758 | 768 | 2110 | 2200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1576 | 1596 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2364 | 2394 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3152 | 3192 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3940 | 3990 | 8550 | 8900 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 912 | 992 | 2498 | 2578 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 204 | 114 | 2622 | 2772 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3286 | 3376 | 4208 | 4358 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 584 | 684 | 4332 | 4552 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4074 | 4174 | 5918 | 6138 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1984 | 1824 | 4996 | 5156 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6332 | 6042 | 1482 | 1372 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7628 | 7918 | 4862 | 4972 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3764 | 3534 | 1026 | 1196 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6706 | 6936 | 5784 | 5954 |

Based on the table above:

- There are no IMD products that fall into Rx frequencies of band n14.

- There are no IMD products that fall into Rx frequencies of band n66.

Table 6.63.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.63.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n14-n66 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.63.2.3		REFSENS requirements

Based on the co-existence studies for CA_n14-n66, there is no need to have MSD added.

6.64	CA_n28-n74

6.64.1	Common for 1 band UL and 2 bands UL CA

6.64.1.1 Operating bands for CA

Table 6.64.1.1-1: CA band combination CA_n28A-n74A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n74 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |

6.64.1.2	Channel bandwidths per operating band for CA

Table 6.64.1.2-1: Supported bandwidths per CA band combination CA_n28A-n74A

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n74A | - | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n74 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

6.64.1.3	UE Co-existence studies

Table 6.64.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n28-n74.

Table 6.64.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2854 | 2940 | 4281 | 4410 | 5708 | 5880 |

Based on above table, 2nd harmonic of band n28 UL fall into band n74 DL.

Table 6.64.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2950 | 3036 | 4425 | 4554 | 5900 | 6072 |

Based on above table, there is no harmonic mixing issue.

6.64.1.4	∆TIB and ∆RIB values

For CA_n28-n74 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 38.101-3 DC_11-n28 and TS 36.101 CA_11-28, CA_21-28 relaxation values.

Table 6.64.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n74 | n28 | 0.6 |
|  | n74 | 0.4 |

Table 6.64.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n28-n74 | n28 | 0.2 |
|  | n74 | 0 |

6.64.1.5	REFSENs requirements

Regarding MSD due to harmonics of n28 UL fall into n74 DL, the exceptions are needed. The reference sensitivity exceptions are proposed in Table 6.64.1.5-1 with uplink configuration specified in Table 6.64.1.5-2.

Table 6.64.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n28 | n741,2 | 23.1 | 19.8 | 18 | 16.8 |  |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n48, CA_n25-n78, CA_n28-n74, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.64.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n28 | n74 | 12 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |

6.64.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirement for CA_n28-n74.

6.64.2	Specific for 2 bands UL CA

6.64.2.1	Maximum output power for inter-band CA

Table 6.64.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28A-n74A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.64.2.2	UE co-existence studies

Table 6.64.2.1-1 lists Band n28 +Band n74 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.64.2.2-1: Band n28 and Band n74 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 703 | 748 | 1427 | 1470 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 679 | 767 | 2130 | 2218 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 64 | 69 | 2106 | 2237 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 2833 | 2966 | 3557 | 3688 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 639 | 817 | 3533 | 3707 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 3536 | 3714 | 4984 | 5158 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 1358 | 1534 | 4260 | 4436 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 4960 | 5177 | 1342 | 1565 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 6411 | 6628 | 4239 | 4462 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 2785 | 3004 | 610 | 831 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 5687 | 5906 | 4963 | 5184 |

Based on above, 2nd,4th,5th IMD of n28 UL and n74 UL fall into n28 DL, and 4th,5th IMD of n28 UL and n74 UL fall into n74 DL.

Table 6.64.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.64.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n28-n74 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 26, 31, 34, 38, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 15 |
|  | E-UTRA Band 4, 42, 43, 52, 65, 66 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20, 2 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 21, 2 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4, 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 21: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth |  |  |  |  |  |  |  |

6.64.2.3	REFSENS requirements

Based on Table 6.64.2.2-1, MSD due to IMD is specified below:

Table 6.64.2.3-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28A-n74A | n28 | 705.5 | 5 | 25 | 760.5 | 24.6 | FDD | IMD2 |
|  | n74 | 1466 | 5 | 25 | 1514 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 11.3 | FDD | IMD4X |
|  | n74 | 1431 | 5 | 25 | 1479 | N/A | FDD | N/A |
|  | n28 | 709 | 5 | 25 | 764 | N/A | FDD | N/A |
|  | n74 | 1466 | 5 | 25 | 1514 | 14.6 | FDD | IMD4 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | FDD | N/A |
|  | n74 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | FDD | IMD5 |
| NOTE X: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

6.65	CA_n74-n77

6.65.1	Common for 1 band UL and 2 bands UL CA

6.65.1.1 Operating bands for CA

Table 6.65.1.1-1: CA band combination CA_n74A-n77A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n74-n77 | n74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.65.1.2	Channel bandwidths per operating band for CA

Table 6.65.1.2-1: Supported bandwidths per CA band combination CA_n74A-n77A

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n74A-n77A | CA_n74A-n77A | n74 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

6.65.1.3	UE Co-existence studies

Table 6.65.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n74-n77.

Table 6.65.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n74 | 1427 | 1470 | 1475 | 1518 | 2854 | 2940 | 4281 | 4410 | 5708 | 5880 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Based on above table, there is no harmonic issue.

Table 6.65.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n74 | 1427 | 1470 | 1475 | 1518 | 2950 | 3036 | 4425 | 4554 | 5900 | 6072 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Based on above table, there is no harmonic mixing issue.

6.65.1.4	∆TIB and ∆RIB values

For CA_n74-n77 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 38.101-3 DC_11_n77 and DC_21_n77 relaxation values.

Table 6.65.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n74-n77 | n74 | 0.4 |
|  | n77 | 0.8 |

Table 6.65.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n74-n77 | n74 | 0 |
|  | n77 | 0.5 |

6.65.1.5	REFSENs requirements

There is no MSD requriements.

6.65.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirement for CA_n74-n77.

6.65.2	Specific for 2 bands UL CA

6.65.2.1	Maximum output power for inter-band CA

Table 6.65.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n74A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.65.2.2	UE co-existence studies

Table 6.65.2.1-1 lists Band n77 +Band n74 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.65.2.2-1: Band n77 and Band n74 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427 | 1470 | 3300 | 4200 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1830 | 2773 | 4727 | 5670 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1346 | 360 | 5130 | 6973 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6154 | 7140 | 8027 | 9870 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 81 | 1110 | 8430 | 11173 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7581 | 8610 | 11327 | 14070 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5546 | 3660 | 9454 | 11340 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15373 | 11730 | 2580 | 1508 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9746 | 6960 | 2190 | 4119 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14627 | 18270 | 9008 | 10080 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12754 | 15540 | 10881 | 12810 |

The 5th IMD of n74 UL and n77 UL fall into n74 DL.

Table 6.65.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.65.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n74-n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 34, 39, 40, 41, 65, 66,85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 21 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 21: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth |  |  |  |  |  |  |  |

6.65.2.3	REFSENS requirements

Even if we choose the UL Fc 1427 and DL Fc 1475 for band n74, we still can’t find the feasible MSD test configuration. So there is no need to specify the MSD due IMD5 of Tx band n74 and n77.

## 6.66			CA_n3-n34

- 6.66.1		Common for 1 band UL and 2 bands UL CA

#### 6.66.1.1		Operating bands for CA

Table 6.66.1.1-1: CA band combination of band n3+n34

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n34 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |

#### 6.66.1.2		Channel bandwidths per operating band for CA

Table 6.66.2-1: Supported bandwidths per CA band combination of band n3+n34

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n34A | CA_n3A-n34A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n34 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |

#### 6.66.1.3		Co-existence studies

Table 6.66.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n34.

Table 6.66.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 | 8550 | 8925 |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 | 8020 | 8100 | 10050 | 10125 |

Based on above table, there is no harmonic issue for the band combination of n3 and n34.

Table 6.66.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 | 8550 | 8925 |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 | 8020 | 8100 | 10050 | 10125 |

Based on above table, there is no harmonic mixing issue for the band combination of n3 and n34.

#### 6.66.1.4		∆TIB and ∆RIB values

For CA_n3-n34, it is proposed to use the same TIB,c and RIB values of DC_n3-n34, which are given in the tables below.

Table 6.66.1.4-1: ΔTIB,c

| NR CA Combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n34 | n3 | 0.5 |
|  | n34 | 0.5 |

Table 6.66.1.4-2: ΔRIB,c

| NR CA Combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n34 | n3 | 0 |
|  | n34 | 0 |

- 6.66.1.5		REFSEN requirements

According to the analysis in subclause 6.66.1.3, there are no harmonic issues for this combination. However, MSD due to cross band isolation on n3 RX due to insufficient isolation to n34 UL are defined below.

Table 6.66.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n34 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 |  |  |  |  |  |  |  |

Table 6.66.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n34 | n3 | 15 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |

6.66.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

- 6.66.2		Specific for 2 bands UL CA

      - 6.66.2.1	Maximum output power for inter-band CA

Table 6.24.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n34A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

- 6.66.2.2		UE co-existence studies

For 2UL/2DL UE coexistence study  2nd, 3rd, 4th and 5th order intermodulation products  for NR CA n3+n34 band combination are calculated and presented in Table 6.66.2.2-1.

Table 6.66.2.2-1: IMD analysis

| DL frequency | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 1710 | 1785 | 2010 | 2025 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 225 | 315 | 3720 | 3810 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1395 | 1560 | 2235 | 2340 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5430 | 5595 | 5730 | 5835 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 630 | 450 | 7440 | 7620 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3105 | 3345 | 4245 | 4365 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7140 | 7380 | 7740 | 7785 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6390 | 6255 | 5130 | 4815 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9750 | 9885 | 8850 | 9165 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2655 | 2460 | 1335 | 1080 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9450 | 9645 | 9150 | 9405 |

Based on the above table, there are  no harmonic and IMD issue.

Table 6.66.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.66.2.1-2: Protected bands for the 2UL bands CA configuration

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n34 | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 69,72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 52 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

- 6.66.2.3		REFSENS requirements

No MSD issues for NR CA n3+n34 band combination.

## 6.67			CA_n8-n34

- 6.67.1		Common for 1 band UL and 2 bands UL CA

#### 6.67.1.1		Operating bands for CA

Table 6.67.1.1-1: CA band combination of band n8+n34

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n34 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |

#### 6.67.1.2		Channel bandwidths per operating band for CA

Table 6.67.2-1: Supported bandwidths per CA band combination of band n8+n34

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n8A-n34A | CA_n8A-n34A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n34 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |

#### 6.67.1.3		Co-existence studies

Table 6.67.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n8-n34.

Table 6.67.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 | 4400 | 4575 |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 | 8020 | 8100 | 10050 | 10125 |

Based on above table, there is no harmonic issue for the band combination of n8 and n34.

Table 6.67.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 | 4400 | 4575 |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 | 8020 | 8100 | 10050 | 10125 |

Based on above table, there is no harmonic mixing issue for the band combination of n8 and n34.

#### 6.67.1.4		∆TIB and ∆RIB values

For CA_n8-n34, it is proposed to use the same TIB,c and RIB values of DC_n8-n34, which are given in the tables below.

Table 6.67.1.4-1: ΔTIB,c

| NR CA Combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n34 | n3 | 0.3 |
|  | n34 | 0.3 |

Table 6.67.1.4-2: ΔRIB,c

| NR CA Combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n34 | n3 | 0 |
|  | n34 | 0 |

- 6.67.1.5		REFSEN requirements

According to the analysis in subclause 6.67.1.3, there are no harmonic issues for this combination. Hence it is no need to define the MSD values caused by harmonic issues for NR CA n8+n34 band combination.

- 6.67.2		Specific for 2 bands UL CA

      - 6.67.2.1	Maximum output power for inter-band CA

Table 6.24.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n8A-n34A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

- 6.67.2.2		UE co-existence studies

For 2UL/2DL UE coexistence study  2nd, 3rd, 4th and 5th order intermodulation products  for NR CA n8+n34 band combination are calculated and presented in Table 6.67.2.2-1.

Table 6.67.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 880 | 915 | 2010 | 2025 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1095 | 1145 | 2890 | 2940 |
| Two-tone 3rd order IMD products | ¦2*fx_low -fy_high ¦ | ¦ 2*fx_high –fy_low ¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 265 | 180 | 3105 | 3170 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3770 | 3855 | 4900 | 4965 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2290 | 2190 | 5780 | 5880 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 615 | 735 | 5115 | 5195 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4650 | 4770 | 6910 | 6990 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7220 | 7125 | 1650 | 1495 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8920 | 9015 | 5530 | 5685 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4315 | 4200 | 1275 | 1410 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7790 | 7905 | 6660 | 6795 |

Based on the above table, there are IMD issues.

Table 6.67.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.67.2.1-2: Protected bands for the 2UL bands CA configuration

| NR CA Combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n8-n34 | E-UTRA Band 1, 20, 28, 31, 32, 33, 38, 39, 40, 45, 50, 51, 65, 67, 69,72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

- 6.67.2.3		REFSENS requirements

No MSD issues for NR CA n8+n34 band combination.

6.68		CA_n5-n12

6.68.1	Common for 1 band UL and 2 bands UL CA

6.68.1.1 Operating bands for CA

Table 6.68.1-1: CA band combination of band CA_n5 + n12

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n12 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |

6.68.1.2	Channel bandwidths per operating band for CA

Table 6.68.1.2-1: Supported bandwidths per CA band combination of band CA_n5 + n12

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n12A | CA_n5A-n12A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |

6.68.1.3	Co-existence studies

Table 6.68.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n12.

Table 6.68.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 2796 | 2864 | 3495 | 3580 |

Based on above table, there is no harmonic issue for the band combination CA_n5-n12.

Table 6.68.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4245 |
| n12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 | 2916 | 2984 | 3645 | 3730 |

Based on above table, there is no harmonic mixing issue for the band combination CA_n5-n12.

6.68.1.4	∆TIB and ∆RIB values

For CA_n5-n12, the TIB,c and RIB,c values are reused from the EN-DC combination DC_12_n5, and are given in the tables below.

Table 6.68.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n12 | n5 | 0.8 |
|  | n12 | 0.4 |

Table 6.68.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n5-n12 | n5 | 0.5 |
|  | n12 | 0.3 |

6.68.1.5	REFSENS requirements

There is no harmonic issue for the CA combination.

6.68.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n5-n12.

Table 6.68.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.68.2		Specific for 2 bands UL CA

6.68.2.1	Maximum output power for inter-band CA

Table 6.68.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n12A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.68.2.2		UE co-existence

Table 6.68.2.2-1 gives IMD interference analysis for CA_ n5-n12 with 2 ULs.

Table 6.68.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 824 | 849 |
| DL frequency (MHz) | 729 | 746 | 869 | 894 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 4120 | 4245 |
| 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 108 | 150 | 1523 | 1565 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 549 | 608 | 932 | 999 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2222 | 2281 | 2347 | 2414 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 1248 | 1324 | 1756 | 1848 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2921 | 2997 | 3171 | 3263 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 300 | 216 | 3046 | 3130 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2697 | 2580 | 2040 | 1947 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3995 | 4112 | 3620 | 3713 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1149 | 1040 | 500 | 399 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3870 | 3979 | 3745 | 3846 |

Based on the table above:

- There are no IMD products that fall into Rx frequencies of band n5.

- There are no IMD products that fall into Rx frequencies of band n12.

Table 6.68.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.68.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 42, 43 50, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 41, 42, 48, 51, 66, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.68.2.3		REFSENS requirements

Based on the co-existence studies for CA_n5-n12, there is no need to have MSD added.

6.69		CA_n5-n14

6.69.1	Common for 1 band UL and 2 bands UL CA

6.69.1.1 Operating bands for CA

Table 6.69.1-1: CA band combination of band CA_n5 + n14

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n14 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |

6.69.1.2	Channel bandwidths per operating band for CA

Table 6.69.1.2-1: Supported bandwidths per CA band combination of band CA_n5 + n14

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n14A | CA_n5A-n14A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

6.69.1.3	Co-existence studies

Table 6.69.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n14.

Table 6.69.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n14 | 788 | 798 | 758 | 768 | 1576 | 1596 | 2364 | 2394 | 3152 | 3192 | 3940 | 3990 |

Based on above table, there is no harmonic issue for the band combination CA_n5-n14.

Table 6.69.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4245 |
| n14 | 788 | 798 | 758 | 768 | 1516 | 1536 | 2274 | 2304 | 3032 | 3072 | 3790 | 3840 |

Based on above table, there is no harmonic mixing issue for the band combination CA_n5-n14.

6.69.1.4	∆TIB and ∆RIB values

For CA_n5-n14, the TIB,c and RIB,c values are reused from CA_5_13 due to similarity in frequency range, and are given in the tables below.

Table 6.69.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n14 | n5 | 0.5 |
|  | n14 | 0.5 |

Table 6.69.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n5-n14 | n5 | 0 |
|  | n14 | 0 |

6.69.1.5	REFSENS requirements

There is no harmonic issue for the CA combination.

6.69.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n5-n14.

Table 6.69.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.69.2		Specific for 2 bands UL CA

6.69.2.1	Maximum output power for inter-band CA

Table 6.69.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n14A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.69.2.2		UE co-existence

Table 6.69.2.2-1 gives IMD interference analysis for CA_ n5-n14 with 2 ULs.

Table 6.69.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 788 | 798 | 824 | 849 |
| DL frequency (MHz) | 758 | 768 | 869 | 894 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1576 | 1596 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2364 | 2394 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3152 | 3192 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3940 | 3990 | 4120 | 4245 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 26 | 61 | 1612 | 1647 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 727 | 772 | 850 | 910 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2400 | 2445 | 2436 | 2496 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1570 | 1674 | 1759 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 122 | 52 | 3224 | 3294 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3188 | 3243 | 3260 | 3345 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2608 | 2498 | 2368 | 2303 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 971 | 876 | 746 | 666 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4084 | 4194 | 3976 | 4041 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4048 | 4143 | 4012 | 4092 |

Based on the table above:

- 3rd and 5th IMD products might fall into Rx frequencies of band n5.

- 3rd IMD products might fall into Rx frequencies of band n14.

Table 6.69.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.69.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n14 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 41, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.69.2.3		REFSENS requirements

Based on the co-existence studies for CA_n5-n14, MSD is needed. DC_8A_n20A is used as baseline due to similarity in frequency.

Table 6.69.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n14 | n5 | 836 | 5 | 25 | 881 | 25 | FDD | IMD34 |
|  | n14 | 791 | 5 | 25 | 761 | N/A | FDD | N/A |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n14 | 795.5 | 5 | 25 | 765.5 | 25 | FDD | IMD3 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

7	Both bands within FR2 Carrier Aggregation: Specific Band Combination Part

< Editor's note: This section is reserved for future used>

8	FR1+FR2 Carrier Aggregation: Specific Band Combination Part

8.1	CA_n41-n257

8.1.1	Common for 1 band UL and 2 bands UL CA

8.1.1.1	Operating bands for CA

Table 8.1.1.1-1: CA band combination of band n41+n257

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.1.1.2	Channel bandwidths per operating band for CA

Table 8.1.1.2-1: Supported bandwidths per CA band combination of band n41+n257

< Editor's note: see Table 5.5A.1-1 in TS38.101-3 for the table format>

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 |  |
| CA_n41A-n257A | CA_n41A-n257A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |
| CA_n41A-n257G | CA_n41A-n257A, CA_n41A-n257G | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 | See CA_n257G Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n257H | CA_n41A-n257A, CA_n41A-n257G, CA_n41A-n257H | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 | See CA_n257H Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n257I | CA_n41A-n257A, CA_n41A-n257G, CA_n41A-n257H, CA_n41A-n257I | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 | See CA_n257I Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.1.1.3	∆TIB and ∆RIB values

Assuming separate antenna architecture between FR1 and FR2, TIB,c and RIB values for this CA configuration are given in the tables below.

Table 8.1.1.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n257 | n41 | 0 |
|  | n257 | 0 |

Table 8.1.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41-n257 | n41 | 0 |
|  | n257 | 0 |

8.1.1.4	REFSENS requirements

No IMD issues are expected in this CA configuration as these are TDD bands. These TDD bands do not transmit and receive simultaneously

8.2	CA_n3-n258

8.2.1	Common for 1 band UL and 2 bands DL CA

8.2.1.1		Operating bands for CA

Table 8.2.1-1: CA band combination of band n3 + n258

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n258 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

8.2.1.2		Channel bandwidths per operating band for CA

Table 8.2.2-1: Supported bandwidths per CA band combination of band n3+n258

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n258A | CA_n3A-n258A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n258B | CA_n3A-n258A CA_n3A-n258B | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258C | CA_n3A-n258A CA_n3A-n258B CA_n3A-n258C | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258D | CA_n3A-n258A CA_n3A-n258D | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258E | CA_n3A-n258A CA_n3A-n258D CA_n3A-n258E | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258F | CA_n3A-n258A CA_n3A-n258D CA_n3A-n258E CA_n3A-n258F | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258G | CA_n3A-n258A CA_n3A-n258G | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258H | CA_n3A-n258A CA_n3A-n258G CA_n3A-n258H | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258I | CA_n3A-n258A CA_n3A-n258G CA_n3A-n258H CA_n3A-n258I | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258J | CA_n3A-n258A CA_n3A-n258G CA_n3A-n258H CA_n3A-n258I | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258J |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258K | CA_n3A-n258A CA_n3A-n258G CA_n3A-n258H CA_n3A-n258I | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258K |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258L | CA_n3A-n258A CA_n3A-n258G CA_n3A-n258H CA_n3A-n258I | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n258M | CA_n3A-n258A CA_n3A-n258G CA_n3A-n258H CA_n3A-n258I | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n258 | CA_n258M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.2.1.3		Co-existence studies

Table 8.2.3-1 gives the UL 2nd, 3rd, 4th, 5th, 6th, 7th harmonic for CA_n3A-n258A. As can ben there are no harmonic produced by UL that may fall into own DL

Table 8.2.1.3-1: Band n3 and Band n258 UL harmonics products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency(MHz) | 1710 | 1785 | 24250 | 27500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits(MHz) | 3420 | 3570 | 48500 | 55000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits(MHz) | 5130 | 5355 | 72750 | 82500 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits(MHz) | 6840 | 7140 | 97000 | 110000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits(MHz) | 8550 | 8925 | 121250 | 137500 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6*fy_low | 6*fy_high |
| 6th harmonics frequency limits(MHz) | 10260 | 10710 | 145500 | 165000 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7*fy_low | 7*fy_high |
| 7th harmonics frequency limits(MHz) | 11970 | 12495 | 169750 | 192500 |

8.2.1.4		∆TIB and ∆RIB values

For CA_n3A-n258A, the TIB,c and RIB values are given in the tables below.

Table 8.2.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n258 | n3 | 0 |
|  | n258 | 0 |

Table 8.2.4-2: ΔRIB

| Inter-band CA Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n3-n258 | n3 | 0 |
|  | n258 | 0 |

8.2.1.5		REFSENS requirements

There is no specific REFSENS requirement.

8.2.2	Specific for 2 bands UL CA

8.2.2.1		UE co-existence studies

Table 8.2.2.1-1 lists Band n3 + Band n258 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 8.2.2.1-1: Band n3 and Band n258 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25790 | 22465 | 25960 | 29285 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 24080 | 20680 | 46715 | 53290 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27670 | 31070 | 50210 | 56785 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 51580 | 44930 | 51920 | 58570 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 22370 | 18895 | 70965 | 80790 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 29380 | 32855 | 74460 | 84285 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 108290 | 95215 | 17110 | 20660 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 98710 | 111785 | 31090 | 34640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 79080 | 69180 | 43145 | 49870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 76170 | 86070 | 53630 | 60355 |

Based on Table 8.2.2.1-1 there are no IMD issues affecting own Rx frequencies.

8.2.2.2		REFSENS requirements

Based on the co-existence studies in 8.2.2.1, there is no need to define MSD for CA_n3-n258.

9	2 bands Dual Connectivity: Specific Band Combination Part

9.X	DC_nX-nY

< Editor's note: The texts for NR DC can only be added associated with the texts for the corresponding inter-band 2 bands UL CA above, which means contribution only adding  pure NR DC texts is not allowed.>

9.x.1	Operating bands for DC_nX-nY

Table 9.X.1-1: Inter-band NR DC operating bands within FR1

| NR DC Band | NR Band |
| --- | --- |
| DC_nX-nY | nX, nY |

Table 9.X.1-2: Inter-band NR DC operating bands for FR1+FR2

| NR DC Band | NR Band |
| --- | --- |
| DC_nX-nY | nX, nY |

9.x.2   Configurations for DC_nX-nY

Table 9.x.2-1: Inter-band NR DC configurations within FR1

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA | DC_nXA-nYA |

Table 9.x.2-2: Inter-band NR DC configurations for FR1+FR2

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA | DC_nXA-nYA |

9.x.3   Maximum output power for NR-DC

< Editor's note: Text will be added, the examples is given as follows. More information can be referred to Table 6.2B.1.3-1 in TS38.101-1. It shall be noted that no need to specify the UE power class for FR1+FR2 NR DC combination. For FR2-FR2 NR DC combination, it is FFS.>

Table 9.x.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_nXA-nYA | 23 | +x/-y1 |
| NOTE 1: refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

9.1	DC_n5-n48

9.1.1	Operating bands for DC_n5-n48

Table 9.1.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n5-n48 | N5, n48 |

9.1.2   Configurations for DC_n5-n48

Table 9.1.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n5A-n48A | DC_n5A-n48A |
| DC_n5A-n48(2A) | DC_n5A-n48A |
| DC_n5A-n48C | DC_n5A-n48A |

9.2	DC_n41-n257

9.2.1	Operating bands for DC_n41-n257

Table 9.2.1-2: Inter-band NR DC operating bands for FR1+FR2

| NR DC Band | NR Band |
| --- | --- |
| DC_n41-n257 | n41, n257 |

9.2.2   Configurations for DC_n41-n257

Table 9.2.2-2: Inter-band NR DC configurations for FR1+FR2

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I | DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | RAN4#96-e | R4-2011626 |  |  |  | TR skeleton | 0.0.1 |
| 2020-08 | RAN4#96-e | R4-2010792 |  |  |  | The following approved TP in RAN4 #96-e meeting are included: - R4-2011637 TP for TR 38.717-02-01: CA_n5-n25, Huawei, HiSilicon, Bell Mobility, Telus - R4-2011638 TP for TR 38.717-02-01: CA_n71-n78, Huawei, HiSilicon, Bell Mobility, Telus - R4-2011639 TP to TR 38.717-02-01: CA_n7-n66, Nokia, Bell Mobility - R4-2011640 TP to TR 38.717-02-00: CA_n25-n38, Nokia, Bell Mobility - R4-2010539 TP for CA 2DL2UL n1-n77 for TR 38.717-02-01, NTT DOCOMO INC. - R4-2010560 TP for CA 2DL2UL n77-n79 for TR 38.717-02-01, NTT DOCOMO INC. - R4-2011643 TP for CA 2DL2UL n78-n79 for TR 38.717-02-01, NTT DOCOMO INC. - R4-2011645 TP for TR 38.717-02-00 to include CA_n25A-n48A, CA_n25A-n48(2A), CA_n25A-n48C, Ericsson, T-Mobile US - R4-2011646 TP for TR 38.717-02-01: CA_n28A-n79A, Huawei, HiSilicon,CBN - R4-2011628 TP for TR 38.717-02-01 CA_n3-n77(2A) _UL_n77(2A),Samsung, KDDI - R4-2011629 TP for TR 38.717-02-01 CA_n3-n78(2A) _UL_n78(2A), Samsung, KDDI - R4-2011630 TP for TR 38.717-02-01 CA_n28-n77(2A) _UL_n77(2A), Samsung, KDDI - R4-2011631 TP for TR 38.717-02-01 CA_n28-n78(2A) _UL_n78(2A), Samsung, KDDI | 0.1.0 |
| 2020-11 | RAN4#97-e | R4-2015184 |  |  |  | Including the approved TP in RAN4 #97-e meeting. - R4-2016680 TP for TR 38.717-02-01 CA_n41-n78 - R4-2014131 TP for TR 38.717-02-01 CA_n2-n66 - R4-2016682 TP for TR 38.717-02-01: CA_n41-n77 - R4-2016683 TP for TR 38.717-02-01: CA_n71A-n77A - R4-2016684 TP for TR 37.717-02-01: CA_n5-n48 - R4-2016686 TP to TR 38.717-02-01: CA_n5-n25 - R4-2016687 TP to TR 38.717-02-01: CA_n25-n77 - R4-2015082 TP to TR 38.717-02-01 to correct CA_n7(2A)-n66 BCS - R4-2016689 TP for TR 38.717-02-01: to add UL configuration for CA_n78A-n79A and CA_n78(2A)-n79A_BCS0 - R4-2016690 TP for TR 38.717-02-01: CA_n8A-n28A - R4-2016691 TP for TR 38.717-02-01: CA_n3A-n7A | V0.2.0 |
| 2021-02 | RAN4#98-e | R4-2102304 |  |  |  | Including the approved TP in RAN4 #98-e meeting. - R4-2102210 TP for TR38.717-02-01_Removal of the sub-clauses under clause 7 ZTE Corporation - R4-2103037 TP for TR 38.717-02-01: CA_n3-n18 Samsung, KDDI - R4-2103038 TP for TR 38.717-02-01: CA_n18-n41 Samsung, KDDI - R4-2103039 TP for TR 38.717-02-01: DC_n3-n41 Samsung, KDDI - R4-2100973 TP for TR 38.717-02-01: CA_n25-n29 Samsung, TELUS, Bell mobility - R4-2103042 TP for TR 38.717-02-01: CA_n13-n25 Samsung, TELUS, Bell mobility - R4-2103043 TP for TR 38.717-02-01: CA_n13-n66 Samsung, TELUS, Bell mobility - R4-2101599 TP for TR 38.717-02-01: CA_n8A-n20A Huawei, HiSilicon - R4-2103047 TP for TR 38.717-02-01 to include CA_n7-n77 Ericsson, Bell Mobility - R4-2103049 TP for TR 38.717-02-01: CA_n41-n257 Samsung, KDDI | V0.3.0 |
| 2021-04 | RAN4#98bis-e | R4-2104913 |  |  |  | Including the approved TP in RAN4 #98-e meeting. - R4-2105236 TP to TR 38.717-02-01: CA_n12-n77，Nokia, AT&T - R4-2105237 TP to TR 38.717-02-01: CA_n14-n77，Nokia, AT&T - R4-2105238 TP to TR 38.717-02-01: CA_n30-n77，Nokia, AT&T - R4-2104418 TP to TR 38.717-02-01: CA_n25A-n71(2A)， Nokia, T-Mobile USA - R4-2104419 TP to TR 38.717-02-01: CA_n41(2A)-n66A，Nokia, T-Mobile USA - R4-2104423 TP to TR 38.717-02-01: CA_n25-n48 combinations，Nokia, T-Mobile USA - R4-2104640 TP for TR 38.717-02-01: CA_n1-n20，VODAFONE Group Plc - R4-2104641 TP for TR 38.717-02-01: CA_n3-n20，VODAFONE Group Plc - R4-2105055 TP for TR 38.717-02-01: CA_n1-n18， samsung,KDDI - R4-2105056 TP for TR 38.717-02-01: CA_n1-n74，samsung,KDDI - R4-2105057 TP for TR 38.717-02-01: CA_n18-n74，samsung,KDDI - R4-2105247 TP for TR 38.717-02-01: CA_n18-n77，samsung,KDDI - R4-2105248 TP for TR 38.717-02-01: CA_n18-n78，samsung,KDDI - R4-2105061 TP for TR 38.717-02-01: CA_n74-n78，samsung,KDDI - R4-2105250 TP for TR 38.717-02-01: CA_n1-n41， samsung,KDDI - R4-2105252 TP for TR 38.717-02-01: CA_n3-n74，samsung,KDDI - R4-2105253 TP for TR 38.717-02-01: CA_n18-n28，samsung,KDDI - R4-2105067 TP for TR 38.717-02-01: CA_n41-n74， samsung,KDDI - R4-2105324 TP for TR 38.717-02-01: CA_n34A-n40A，ZTE Corporation - R4-2105256 TP for TR 38.717-02-01: CA_n13-n77，Huawei, HiSilicon, Bell Mobility, Telus - R4-2105257 TP for TR 38.717-02-01: CA_n2-n7，Huawei, HiSilicon, Bell Mobility, Telus - R4-2105258 TP for TR 38.717-02-01: CA_n24-n41， Ligado Networks - R4-2105259 TP for TR 38.717-02-01: CA_n24-n48，Ligado Networks - R4-2105260 TP for TR 38.717-02-01: CA_n24-n77，Ligado Networks - R4-2105262 TP for TR 38.717-02-01: CA_n28A-n71A， Huawei, HiSilicon - R4-2106715 TP for TR 38.717-02-01 to include CA_n2A-n30A, CA_n2(2A)-n30A， Ericsson, AT&T - R4-2105264 TP for TR 38.717-02-01 to include CA_n5A-n30A, UL CA_n2A-n30A，Ericsson, AT&T - R4-2106717 TP for TR 38.717-02-01 to include CA_n30A-n66A, CA_n30A-n66(2A), CA_n30A-n66(3A), UL CA_n30A-n66A，Ericsson, AT&T - R4-2105266 TP for TR 38.717-02-01 to include new BCS's and UL configurations n25-n71，Ericsson, T-Mobile US - R4-2105267 TP for TR 38.717-02-01 to include new BCS's and UL configurations n41-n66，Ericsson, T-Mobile US - R4-2105268 TP for TR 38.717-02-01 to include new BCS's and UL configurationsn41A-n71，Ericsson, T-Mobile US - R4-2105269 TP for TR 38.717-02-01 to include new BCS's and UL configurations n66-n71， Ericsson, T-Mobile US - R4-2105270 TP to TR 38.717-02-01 Addition of CA_48-53，Nokia, Globalstar | V0.4.0 |
| 2021-05 | RAN4#99-e | R4-2110999 |  |  |  | Including the approved TP in RAN4 #99-e meeting. - R4-2109397 TP to TR 38.717-02-01 Addition of CA_n2A-n12A AT&T, Nokia - R4-2109398 TP to TR 38.717-02-01 Addition of CA_n2A-n14A AT&T, Nokia - R4-2107694 TP to TR 38.717-02-01 Addition of CA_n12A-n30A AT&T, Nokia - R4-2107695 TP to TR 38.717-02-01 Addition of CA_n12A-n66A AT&T, Nokia - R4-2109402 TP to TR 38.717-02-01 Addition of CA_n14A-n30A AT&T, Nokia - R4-2109403 TP to TR 38.717-02-01 Addition of CA_n14A-n66A AT&T, Nokia - R4-2107697 TP for TR 38.717-02-01: CA_n28-n74 HiSilicon, KDDI - R4-2107698 TP for TR 38.717-02-01: CA_n74-n77 Huawei, HiSilicon, KDDI - R4-2107699 TP for TR 38.717-02-01: CA_n3A-n34A ZTE Corporation - R4-2110453 TP for TR 38.717-02-01: CA_n8A-n34A ZTE Corporation - R4-2107700 TP to TR 38.717-02-01 Addition of CA_n25-n48 Nokia, T-Mobile USA - R4-2107812 TP to TR 38.717-02-01 Addition of CA_n5A-n12A AT&T, Nokia - R4-2107978 TP to TR 38.717-02-01 Addition of CA_n5A-n14A Nokia, AT&T | V0.5.0 |