3GPP TSG-RAN WG4 Meeting #99-e                                          R4-2111444

Electronic Meeting, May 19-27, 2021

Source: 	Huawei, HiSilicon

Title: 	On requirements for n40 supporting PC2 UL MIMO

Agenda Item:	8.42.2

Document for:	Approval

# Introduction

A WF on PC2 UL MIMO for Band n40 was agreed in last RAN4 meeting. The main focus on the WF is to address the questions raised during the discussion, i.e. for MPR requirements. However, there is no conclusion on A-MPR. To make the analysis complete, in this contribution we provide additional consideration on A-MPR for a band supporting UL MIMO, which is not subjected to band n40.

# Discussion

## A-MPR requirement in general

The A-MPR requirements for UL MIMO is also considered in the specification, which is defined in a generic manner for all possible bands as below.

### 6.2D.3 UE additional maximum output power reduction for UL MIMO For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used. For UE support uplink full power transmission (ULFPTx) for UL MIMO, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode. For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2D.4 apply. If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.4 apply for the power class as indicated by the ue-PowerClass field in capability signaling.

Currently there are several bands already support UL MIMO with power class other than PC3. Since there are no specific additional requirements for band n77, n78 and n79, the way we adopted for n41 can be considered as reference to define the A-MPR requirements for HPUE supporting UL MIMO.

Table 6.2D.1-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | +2/-3 |  |  |
| n2 |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  | 23 | +2/-31 |  |  |
| n25 |  |  |  |  | 23 | +2/-31 |  |  |
| n30 |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  |  |  | 23 | +2/-3 |  |  |
| n38 |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  |  |  | 23 | +2/-3 |  |  |
| n40 |  |  |  |  | 23 | +2/-3 |  |  |
| n41 | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  |  |  | 23 | +2/-3 |  |  |
| n70 |  |  |  |  | 23 | +2/-3 |  |  |
| n71 |  |  |  |  | 23 | +2/-3 |  |  |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

According to requirements specified in 6.2D.3, the A-MPR values specified in clause 6.2.3 shall apply, which means that this specific requirement can be considered in the clause for general requirements as well. Since not all bands supporting UL MIMO need to consider A-MPR, we think it is easier to follow the exiting manner we adopted for PC1.5 for n41 to just consider additional supplement of A-MPR for UL MIMO in clause 6.2.3 if needed.

Observation 1: A-MPR requirement should be considered for NR bands supporting UL MIMO for power classes other than PC3, which is studied band by band

Observation 2: Band specific A-MPR requirements for UL MIMO are considered additionally in clause 6.2.3 with A-MPR applicable to non-MIMO mode

Proposal 1: A-MPR for UL MIMO can be specified additionally with other A-MPR requirements for the operating bands in the same sub-clause if needed.

## A-MPR consideration for band n40

So far, the bands supporting UL MIMO are listed in the table below (recopied from the spec):

Table 5.2D-1: NR operating bands for UL MIMO in FR1

NR operating band

n1

n2

n3

n7

n25

n301

n34

n38

n39

n40

n41

n46

n48

n66

n70

n712

n77

n78

n79

n84

n95

n96

n97

n98

NOTE 1: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 2: UL MIMO is targeted for FWA form factor.

Now for FDD bands, only PC3 is supported.

For a specific operating band to support UL MIMO for a power class, we need to consider whether the power class is already supported for non-MIMO operation. The table is copied from the existing specification. Specifically, for n40, we see both PC 2 and PC3 are already supported.

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  | 23 | ±2 |
| n2 |  |  |  |  |  |  | 23 | ±23 |
| n3 |  |  |  |  |  |  | 23 | ±23 |
| n7 |  |  |  |  |  |  | 23 | ±23 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n34 |  |  |  |  |  |  | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  |  |  |  |  | 23 | ±2 |
| n40 |  |  |  |  | 26 | +2/-33 | 23 | ±2 |
| n41 |  |  | 295 | _2/-33 | 26 | +2/-33 | 23 | ±23 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n66 |  |  |  |  |  |  | 23 | ±2 |
| n70 |  |  |  |  |  |  | 23 | ±2 |
| n71 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n77 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n84 |  |  |  |  |  |  | 23 | ±2 |
| n95 |  |  |  |  |  |  | 23 | ±2 |
| n97 |  |  |  |  |  |  | 23 | ±2 |
| n98 |  |  |  |  |  |  | 23 | ±2 |

As agreed in the WF, the MPR requirements are general for all operating bands, which is not covered in the basket WI. However, A-MPR, as a band specific requirement, should be considered in the WI if the corresponding requirement is not defined yet.

Not all operating bands have A-MPR requirements, which depends on whether there are some specific additional requirements are imposed for the band.

Specifically, from the table below, there is no A-MPR requirement needed for band n40. But for other bands, A-MPR may need to be considered.

Table 6.2.3.1-1A: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n77 | NS_01 |  |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n97 | NS_01 |  |  |  |  |  |  |  |
| n98 | NS_01 | NS_50 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Observation 2: Specifically for band n40, there is no A-MPR requirements.

# Conclusion

This contribution provides consideration the requirements for NR bands supporting UL MIMO other than MPR.

Observation 1: A-MPR requirement should be considered for NR bands supporting UL MIMO for power classes other than PC3, which is studied band by band

Observation 2: Band specific A-MPR requirements for UL MIMO are considered additionally in clause 6.2.3 with A-MPR applicable to non-MIMO mode

Proposal 1: A-MPR for UL MIMO can be specified additionally with other A-MPR requirements for the operating bands in the same sub-clause if needed.

Observation 3: Specifically for band n40, there is no A-MPR requirements.

# References

[1] R4-2105374, WF on PC2 UL MIMO for Band n40, Huawei, HiSilicon