3GPP TSG-RAN WG4 Meeting # 99-e 												R4-210XXXX

Electronic Meeting, 19th – 27th May, 2021

Agenda item:			9.3.1

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [98-bis-e][135]NR_RF_FR1_enh_Part_1

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Thread [135] includes following topics:

- Topic #1: intra-band contiguous UL CA for FR1 power class 2 which is for agenda 9.3.2.4

- Topic #2: intra-band NC UL CA for FR1 power class 2 which is for agenda 9.3.2.5

- Topic #3: Intra-band UL contiguous CA for UL MIMO which is for agenda 9.3.2.6

Topic #1: PC2 intra-band contiguous UL CA

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109979 | Ericsson | Proposal 1: the PCMAX,f,c for serving cell c should be based on the MPR for the total signal. For PH reporting the following exception: if the UE is configured with multiple uplink serving cells, the power PCMAX,c used for the purpose of PH reporting on first serving cell c1 may not consider for computation of the said PH report transmissions on a second serving cell c2 as exempted in subclause 7.7.1 of 38.213. Proposal 2: when secondary cells or serving cells with transmissions of low priority are significantly reduced in power (dropped), the MPRc for the remaining serving cell should apply for the total signal, that is, MPR = MPRc, at least for inner allocations on the remaining cell. Proposal 3: preventing SCell power reductions and “equal PSD” in conformance tests should be achieved by specifying configurable limits relative to the configured power for the serving cells. This would account for the actual power back-off (up to MPR and same for all serving cells) that is applied by the UE. The UE-specific limits should configured by RRC and activated/deactivated by a MAC-CE. |
| R4-2111351 | Huawei, HiSilicon | Big CR for PC2 intra-band UL contiguous CA |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: Pcmax,c definition for intra-band UL CA

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1-1: Pcmax,c definition for intra-band (contiguous and non-contiguous) UL CA

- Proposals：

- Option 1: (From R4-2109979)

The PCMAX,f,c for serving cell c should be based on the MPR for the total signal. For PH reporting the following exception: if the UE is configured with multiple uplink serving cells, the power PCMAX,c  used for the purpose of PH reporting on first serving cell c1 may not consider for computation of the said PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 of 38.213.

- Option 2: (From R4-2109979)

When secondary cells or serving cells with transmissions of low priority are significantly reduced in power (dropped), the MPRc for the remaining serving cell should apply for the total signal, that is, MPR = MPRc, at least for inner allocations on the remaining cell.

- Recommended WF

  - TBA

Sub-topic 2-2: MAC-CE enabling/disabling the UE-specific limits

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2-1: Are discussions on “MAC-CE enabling/disabling the UE-specific limits” in the current scope of Rel-17 FR1 RF enh WID?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBA

Issue 2-2-2: If ‘no’ of issue 2-2-1, do we need to add “specifying configurable limits relative to the configured power for the serving cells” into the WID?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBA

Sub-topic 2-3: CR for PC2 intra-band UL contiguous CA

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-3-1: The applicable RF architecture for the MPR requirements agreed in RAN4 #98bis-e (i.e. R4-2105388)

- Proposals

  - Applies for 1PA architecture, and clarifies in the CR:

“The MPR with contiguous RB allocation is specified in Table 6.2A.2.1-1a for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE.”

- Recommended WF

  - TBA

Issue 2-3-2: ΔPpowerclass,CA definition

- Proposals

  - Applies with:

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,(PPowerClass,CA– ΔPPowerClass,CA) – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA– ΔPPowerClass,CA }

ΔPPowerClass,CA = 3 dB for a power class 2 capable UE when 10 log10 ∑ pEMAX,c of 23 dBm or lower is indicated; or when PEMAX,CA  of 23dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame);

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Example 1

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111351 | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #2: PC2 intra-band NC UL CA

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2108799 | Qualcomm | Observation 1: In order to avoid scheduling impacted by PA swap time, the time must be << CP length. Observation 2: PUSCH must always be transmitted by the same antenna as preceding SRS Proposal 1: PA swap time for 26+23 dBm 2 LO architecture is << CP length and no impact to RAN1 for it. Proposal 2: 26+23 dBm 2 LO architecture is not used for MPR analysis but this architecture conforms to the MPRs based on 26+26 dBm architecture. Proposal 3: Remove Carrier leakage and IQ image exceptions from 6.5A.2.2.2 without any relaxations Proposal 4: Remove Carrier leakage and IQ image exceptions from 6.5A.3.1 without any relaxations Proposal 5: Remove Carrier leakage and IQ image exceptions from 6.5A.2.4.1.2 without any relaxations Proposal 6: Delay the work for NC UL CA with Tx diversity architcture until generic Rel-16 Tx diversity work has been completed. |
| R4-2109260 | Nokia | Simulation information based on 1x26 dBm PA + 1LO with 200 MHz BW is provided: - Simulated backoff for 20+40 MHz CA with 20 MHz gap as a function of the allocation bandwidth with (on the left) and without (on the right) the LO included - Simulated backoff for 40+40 MHz CA with 120 MHz gap as a function of the allocation bandwidth with (on the left) and without (on the right) the LO included. |
| R4-2109261 | Nokia | Proposal 1. Define PC2 1PA intra-band UL non-contiguous carrier aggregation MPR with the LO exemption as 14; 0 <= B <= 1.44 14.5 - 0.34 A; 1.44 < B <= 25 6.45 - 0.0182 A; 25 < B <= BWagg Proposal 2. Handle asymmetric allocations whose IQ image falls outside the channels with increased IQ image attenuation. |
| R4-2109965 | LGE | Observation1: The #4 RF architecture with 1x23dBm+1x26dBm PAs can support UL non-contiguous CA with the separate class III. Also support UL-MIMO in single carrier compare to architecture #2 and #3. Obeservation2: Based on MPR results, the delta MPR value is not quite different (up to 1.5dB) between #1 PA architecture and #4 PA architecture. Observation3: The PA swapping time does not need for intra-band non-contiguous CA since each PA per CC operate for intra-band non-contiguous CA operation. Proposal 1: Based on the MPR results, we propose following MPR requirements for PC2 NR intra-band non-contiguous CA UE - MPRIM3 to meet -30dBm/MHz MPR=MA Where MA is defined as follows MA = 16; 0 ≤ B < 1.08 15.0; 1.08 ≤ B < 2.88 14.0; 2.88 ≤ B < 5.40 12.0; 5.40 ≤ B < 9.72 10.5; 9.72 ≤ B < 16.38 9.0; 16.38 ≤ B Where: B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000 - MPRIM5 to meet -13dBm/MHz MPR=MA Where MA is defined as follows MA = 9 ; 0 ≤ B < 0.54 8 ; 0.54 ≤ B < 1.08 7 ; 1.08 ≤ B < 2.16 6.5 ; 2.16 ≤ B < 3.24 5.5 ; 3.24 ≤ B < 5.4 4 ; 5.4 ≤ B Where: B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000 |
| R4-2110820 | OPPO | Observation 1: For intra-band UL NC CA with 1x23dBm+1x26dBm + 2LO architecture, 0us is not achievable. Proposal 1: It is proposed to specify 35us and 140us as the PA swap time for architecture #4. Proposal 2: It is proposed to reuse in-gap exception under some conditions (e.g. Sync) as defined for PC3 for the architecture #2 and #3. |
| R4-2111384 | Huawei, HiSilicon | Observation 1: it was agreed to allow the exception on spurious requirement for intra-band UL NC CA when synchronized across licensees. Proposal 1: For PC2 intra-band UL NC CA, in-gap exception follows the agreement made in Rel-16. For ACLR, 3dB relaxation is proposed to be reused for PC2 intra-band UL NC CA. For SEM requirement, exception refers to the requirement for LO leakage or image requirement applies. Proposal 2: Introduce new UE capability for intra-band UL NC CA, to indicate the network that whether UE can support CA without RF requirement exception. Proposal 3: PA swap time for architecture #4 could be 0us or 35us or 140us, define new UE capability to indicate PA swap time. |
| R4-2111480 | Skyworks | Proposal on swap time for PC3+PC2 architecture: - To provide any benefit versus PC3 and limited degradation versus baseline architecture a maximum swap time of 15us is allowed. - Unless the MPR difference compared to the baseline architecture is less than 1dB for the best allocation, this architecture will use separate MPR values in the specification (table or delta). Proposal on architecture requiring in-gap exceptions: - In-gap exceptions are only allowed for CC configurations where the gap bandwidth is less or equal than the two CC aggregated bandwidth - In-gap exceptions are only allowed for UEs also supporting UL MIMO together with NC UL CA - FFS if this restriction is applicable for the case where the in-gap channels belong to the same operator or to co-located cells with non-simultaneous Tx/Rx - FFS if in-gap SEM exceptions are associated with better image and carrier leakage UE capability - Unless the MPR difference compared to the baseline architecture is less than 1dB for the best allocation, this architecture will use separate MPR values in the specification (table or delta) |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1: MPR comparison among architecture options

Sub-topic description:

Open issues and candidate options before e-meeting:

| Arch | description |
| --- | --- |
| #1 | 2x26dBm PA + 2LO with 100MHz BW |
| #2 | 1x26dBm PA + 1LO with 200MHz BW |
| #3 | 2x23dBm PA + 1LO with 200MHz BW |
| #4 | 1x23dBm+1x26dBm + 2LO with 100MHz BW |

Issue 3-1-1: MPR comparison among architecture options

<Recommend discussion on the analysis provided in R4-2104819>

- Proposals

  - For Architecture #2:

From R4-2109260, Simulated backoff for 40+40 MHz CA with 120 MHz gap as a function of the allocation bandwidth with (on the left) and without (on the right) the LO included.

Define PC2 1PA intra-band UL non-contiguous carrier aggregation MPR with the LO exemption as

14;                 	        0 <= B <= 1.44

14.5 - 0.34 A;         1.44 < B <= 25

6.45 - 0.0182 A; 25 < B <= BWagg

  - For Architecture #1 and #4:

From R4-2108799: 26+23 dBm 2 LO architecture is not used for MPR analysis but this architecture conforms to the MPRs based on 26+26 dBm architecture.

From R4-2109965: Based on MPR results, the delta MPR value is not quite different (up to 1.5dB) between #1 PA architecture and #4 PA architecture.

- MPRIM3 to meet -30dBm/MHz

MPR=MA Where MA is defined as follows

MA = 	16; 	0 ≤ B < 1.08

15.0; 	1.08 ≤ B < 2.88

14.0; 	2.88 ≤ B < 5.40

12.0;  5.40 ≤ B < 9.72

10.5; 	9.72 ≤ B < 16.38

9.0; 	16.38 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

- MPRIM5 to meet -13dBm/MHz

MPR=MA Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

- Recommended WF

  - TBA

Sub-topic 3-2: Architecture options handling

Issue 3-2-1: For 1x26dBm PA + 1LO with 200MHz BW and 2x23dBm PA + 1LO with 200MHz BW, how to handle in-gap requirement when LO or image fall inside?

- Proposals

  - Option 1: remove exceptions for ACLR/SEM/SE when LO or image fall inside the gap

  - Option 2:

•	In-gap exceptions are only allowed for CC configurations where the gap bandwidth is less or equal than the two CC aggregated bandwidth

•	In-gap exceptions are only allowed for UEs also supporting UL MIMO together with NC UL CA

  - Option 3: reuse in-gap exception under some conditions (e.g. Sync) as defined for PC3 for the architecture #2 and #3.

  - Option 4:

For PC2 intra-band UL NC CA, in-gap exception follows the agreement made in Rel-16. For ACLR, 3dB relaxation is proposed to be reused for PC2 intra-band UL NC CA. For SEM requirement, exception refers to the requirement for LO leakage or image requirement applies.

Introduce new UE capability for intra-band UL NC CA, to indicate the network that whether UE can support CA without RF requirement exception.

- Recommended WF

  - TBA

Issue 3-2-2: For 1x23dBm + 1x26dBm PA + 2LO with 100MHz BW, how to handle the swap time between PAs?

- Proposals

  - Option 1: PA swap time for 26+23 dBm 2 LO architecture is << CP length and no impact to RAN1 for it.

  - Option 2: specify 35us and 140us as the PA swap time for architecture #4.

  - Option 3: a maximum swap time of 15us is allowed

  - Option 4: PA swap time for architecture #4 could be 0us or 35us or 140us, define new UE capability to indicate PA swap time.

  - Option 5: The PA swapping time does not need for intra-band non-contiguous CA since each PA per CC operate for intra-band non-contiguous CA operation.

Moderator note: option 5 is generally equal to option 1?

- Recommended WF

  - TBA

Issue 3-2-3: architecture option(s) for intra-band UL NC CA: architecture No. is as in the table

| Arch | description |
| --- | --- |
| #1 | 2x26dBm PA + 2LO with 100MHz BW |
| #2 | 1x26dBm PA + 1LO with 200MHz BW |
| #3 | 2x23dBm PA + 1LO with 200MHz BW |
| #4 | 1x23dBm+1x26dBm + 2LO with 100MHz BW |

- Proposals

  - Option 1: Delay the work for NC UL CA with Tx diversity architcture until generic Rel-16 Tx diversity work has been completed.   (delay architecture #3)

  - Option 2: other

- Recommended WF

  - TBA

Issue 3-2-4: whether MPR requirements are separate defined for different architecture?

- Proposals

  - Option 1:

Unless the MPR difference compared to the baseline architecture is less than 1dB for the best allocation, this architecture will use separate MPR values in the specification

  - Option 2: Other

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Example 1

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #3: Intra-band UL contiguous CA for UL MIMO

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109425 | ZTE | Observation 1: RF requirements for the combination of UL-MIMO and intra-band contiguous CA should be defined for the case where all component carriers in the intra-band contiguous CA should be in UL-MIMO mode and have the same number of layers and antenna ports. Proposal 1: RAN4 include 1-layer-2-port configuration for CA + UL MIMO, but exclude transparent TxD configuration. Proposal 2: RAN4 to define the capability of combining UL-MIMO and intra-band contiguous CA, and also the capability of supported aggregated bandwidth for the combination of UL-MIMO and CA. |
| R4-2109680 | vivo | Observation 1: 1 layer 2 port UL MIMO configuration may not essential to be verified with CA. Observation 2: Transparent TxD may also not essential to be verified with CA and also doubtful if included in the WID scope. Proposal 1: Do not include 1 layer 2 port UL MIMO configuration and Transparent TxD configuration. Proposal 2: Postpone EVM discussion after more stable UL-MIMO requirements. Proposal 3: Further check if UL-MIMO is per-band, if so define new capability signalling for supported aggregated CBW within UL CA+UL MIMO. |
| R4-2110819 | OPPO | 2.1 Capability signalling Observation 1: For intra-band UL CA, both PC3 and PC2, different PA capabilities in supporting the aggregated CBW were supported by the spec. Observation 2: UEs are different in supporting the aggregated CBW under CA+UL MIMO feature and it needs to be clear to NW. Observation 3: Currently ca-BandwidthClassUL-NR is a per-band capability used to report the supported aggregated CBW for intra-band contiguous UL CA, and it doesn’t differentiate with or without MIMO capability. Observation 4: The aggregated CBW capability could be different when UE works under CA mode or under CA+UL MIMO mode, however, with one ca-BandwidthClassUL-NR capability reported this cannot be differentiated. Observation 5: RAN2 didn’t touch the UE aggregated CBW capability limitation in CA+UL MIMO. Observation 6: Intra-band non-contiguous UL CA also has similar problem when it works together with UL MIMO. Proposal 1: It is proposed to report the UE supported aggregated CBW for UL CA+UL MIMO feature to NW. 2.2 Release independent Observation 7: Rel-16 UE can support both UL CA and UL MIMO together when it is implemented with large BW PAs. Proposal 2: It is proposed that CA+UL MIMO feature is release independent from Rel-16. Observation 8: Rel-16 UE cannot support both UL CA and UL MIMO together when it needs two PAs to achieve the aggregated CBW. Proposal 3: It is proposed to inform RAN2 that UE supported aggregated CBW could be different between CA and CA+UL MIMO and it is up to RAN2 how to update Rel-16 UE capabilities. 2.3 Requirements Observation 9: Requirements for UL MIMO only considered 100MHz, requirements for UL CA only considered single layer transmission, both may not be applicable directly to UL CA+UL MIMO. |
| R4-2111023 | Skyworks | Proposal on TxDiv and UL MIMO contiguous UL CA MPR: - The agreed baseline architecture MPR is used for the 38.101-1 requirement for the UEs that do not signal support for TxDiv or UL MIMO to be able to support PC2 - UL MIMO support for ULCA with 1LO+2PC2 PA is supported by the spec and can reuse the baseline MPR thanks to 3dB intrinsic headroom - A separate MPR requirement is developed for class B UL CA with TxDiv or UL MIMO based on 1LO/2PC3 PAs and evaluation is started after corresponding single CC is specified. |
| R4-2111380 | Huawei, HiSilicon | draft CR on contiguous CA with UL MIMO for power class 3 |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 4-1: RF requirements framework

Sub-topic description:

Open issues and candidate options before e-meeting:

Copy the proposal on each RF requirement item from R4-2104956 and also referenced by R4-2109680

| Tx characteristics | UL-MIMO + Intra-band UL C CA |
| --- | --- |
| UE maximum output power | Per-UE (Sum of all Tx and CC) |
| UE maximum output power reduction | [FFS, Per-UE but requirements need study] |
| UE addition maximum output power reduction | Per-UE |
| Configured transmitted power | Per-UE |
| Minimum output power | Per-carrier, sum of 2Tx |
| Transmit OFF power | Per-carrier per connector |
| Transmit ON/OFF time mask | Per-carrier per connector |
| Power control | Per-carrier, sum of 2Tx |
| Frequency error | Per-carrier per connector |
| Transmit modulation quality (EVM, Carrier leakage, IBE and EVM spectrum flatness) | [FFS] Postpone EVM discussion after more stable UL-MIMO requirements |
| Occupied bandwidth | Per-UE |
| Out of band emission | Per-UE |
| Spurious emission | Per-UE |
| Transmit intermodulation | Per connector, 2carreirs active |

Issue 4-1-1: MIMO Configurations for CA+UL MIMO requirements

- Proposals

  - Option 1:  2 layer configuration and 1 layer 2 port configuration are included, transparent TxD configuration is excluded

  - Option 2:  Do not include 1 layer 2 port UL MIMO configuration and Transparent TxD configuration

Moderator comment: Option 2 means only 2 layer configuration is included

  - Option 3:  Other

- Recommended WF

  - TBA

Issue 4-1-2: RF requirement items: MOP

- Proposals

  - Sum of all Tx and CC:

i.e. For UE supporting intra-band UL contiguous CA with UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors.

- Recommended WF

  - TBA

Issue 4-1-3: RF requirement items: AMPR

- Proposals

  - Per UE:

i.e. For UE supporting intra-band UL contiguous CA with UL MIMO, the A-MPR values specified in clause 6.2A.3(i.e. CA AMPR requirement) shall apply to the maximum output power.

- Recommended WF

  - TBA

Issue 4-1-4: RF requirement items: Configured output power

- Proposals

  - Per UE:

i.e.

- Recommended WF

  - TBA

Issue 4-1-5: RF requirement items: Minimum output power

- Proposals

  - Per-carrier, sum of 2Tx

i.e. For UE supporting intra-band UL contiguous CA with UL MIMO, the minimum output power is defined as the sum of the mean power from both transmit connector in one sub-frame (1 ms) per carrier.

- Recommended WF

  - TBA

Issue 4-1-6: RF requirement items: transmit OFF power

- Proposals

  - Per carrier per connector:

i.e. The transmit OFF power is defined as the mean power at each transmit antenna connector on each component carrier in a duration of at least one sub-frame (1 ms) excluding any transient periods.

- Recommended WF

  - TBA

Issue 4-1-7: RF requirement items: Transmit ON/OFF time mask

- Proposals

  - Per-carrier per connector:

i.e. For UE supporting intra-band UL contiguous CA and UL MIMO, the ON/OFF time mask requirements in clause 6.3A.3(per CC) apply at each transmit antenna connector.

- Recommended WF

  - TBA

Issue 4-1-8: RF requirement items: 	Power control tolerance

- Proposals

  - Per-carrier, sum of 2Tx

i.e. For UE supporting intra-band UL contiguous CA and UL MIMO, the power control tolerance applies to the sum of output powers from both transmit antenna connector on each component carrier.

- Recommended WF

  - TBA

Issue 4-1-9: RF requirement items: 	transmit signal quality

- Proposals

  - Option 1: Postpone the discussion

  - Option 2: contiguous CA requirement per connector

i.e. For UE supporting intra-band UL contiguous CA and UL MIMO, the transmit modulation quality requirements are specified at each transmit antenna connector as in subclause 6.4A.2

- Recommended WF

  - TBA

Issue 4-1-10: RF requirement items: Output RF spectrum emissions

- Proposals

  - Per UE:

i.e. The requirements for OBW/Out of band emissions/spurious emissions is defined as the sum of the emissions from both UE transmit antenna connectors.

- Recommended WF

  - TBA

Issue 4-1-11: RF requirement items: Transmit intermodulation

- Proposals

  - Per connector, 2carreirs active

i.e. For UE supporting intra-band UL contiguous CA and UL MIMO, the transmit intermodulation requirements are specified at each transmit antenna connector and the wanted signal is defined as the sum of output powers from both UE transmit antenna connectors on both CC.

- Recommended WF

  - TBA

Sub-topic 4-2: MPR

Issue 4-2-1: MPR requirement for PC3 UL contiguous CA +MIMO with 2 PC3 PA+1LO

- Proposals

  - Option 1: Reuse the MPR value defined for PC3 intra-band UL contiguous CA with 2 PC3 PA+1LO

  - Option 2: A separate MPR requirement is developed for class B UL CA with TxDiv or UL MIMO based on 1LO/2PC3 PAs and evaluation is started after corresponding single CC is specified.

- Recommended WF

  - TBA

Issue 4-2-2: MPR requirement for PC2 UL contiguous CA +MIMO with 2 PC2 PA+1LO or 2 PC3 PA+1LO

- Proposals

  - Option 1: Reuse the MPR value defined for PC2 intra-band UL contiguous CA for 2PC2 PA+1LO

A separate MPR requirement is developed for class B UL CA with TxDiv or UL MIMO based on 1LO/2PC3 PAs and evaluation is started after corresponding single CC is specified.

- Recommended WF

  - TBA

Sub-topic 4-3 signalling

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 4-3-1: Signalling proposals

- Proposals

- Option 1: define the capability of combining UL-MIMO and intra-band contiguous CA, and also the capability of supported aggregated bandwidth for the combination of UL-MIMO and CA.

- Option 2: Further check if UL-MIMO is per-band, if so define new capability signalling for supported aggregated CBW within UL CA+UL MIMO.

- Option 3: Report the UE supported aggregated CBW for UL CA+UL MIMO feature to NW.

- Recommended WF

  - TBA

Issue 4-3-2: LS to RAN2

- Proposals:

Inform RAN2 that UE supported aggregated CBW could be different between CA and CA+UL MIMO and it is up to RAN2 how to update Rel-16 UE capabilities.

- Recommended WF

  - TBA

Sub-topic 4-4 Draft CR

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 4-4: Draft CR for PC3 intra-band UL contiguous CA for UL MIMO

- Proposals

- Option 1: Endorse draft CR R4-2111380

- Option 2: Endorse the draft CR after revision

- Option 3: other

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Example 1

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

Example 2

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX |  |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents