3GPP TSG RAN WG4 Meeting #69 UE RF AH	                   R4-69AH-0010

Austin, TX, U.S., 14th – 16th Jan., 2014

- Agenda Item:	5.2

- Source:	LG Electronics, LG Uplus

- Title:	TP for TR36.853: 3DLs CA_B1_B5_B7

Document for:	Approval

- Introduction

In the last RAN plenary #62 meetings, 3DL CA WI for B1+B5+B7 was approved and there was an email discussion on the bandwidth combination set. The IMD products for 2ULs inter-band CA are not different from those for 1UL inter-band CA, so we analyzed the harmonic products for UE in this paper. And also, we proposed TIB,C and RIB,C values for simultaneous 3DLs and 1UL transmission.

- References

- R4-137152, “Way Forward on TX and Rx relaxations for 3DL/1UL FDD carrier aggregation,” Telecom Italia

- TS 36.101 v12.2.0

- R4-136249, “Tx and Rx relaxations for 3DL/1UL FDD carrier aggregation,” Qualcomm

Text proposal for TR 36.853

********************** Start of text proposal to TR 36.853 Chapter 6 ***********************

- 6.X	LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 1, Band 5 and Band 7

- 6.X.1	Operating bands for CA

Table 6.X.1-1: Inter-band CA operating bands for 3DLs

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-5-7 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |

- 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA with 3DLs

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-5A-7A | 1 |  |  | Yes | Yes |  |  | 40 | 0 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 1 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 7 |  |  |  | Yes | Yes | Yes |  |  |

NOTE: In single or dual CA mode the UE shall support all bandwidths as defined in table 5.6.1-1 and table 5.6A.2-1 in TS 36.101 respectively.

- 6.X.3	Harmonics and IMD study

- 6.X.3.1	For the BS

<Text will be added>

- 6.X.3.2	For the UE

Table 6.X.3.1-1 gives the 2nd and 3rd harmonic for CA with B1+ B5 + B7. None of the products falls into the B1, B5 or B7 UE own receiving bands. For UE coexistence the single UL is already considered for these bands in TS 36.101.

Table 6.X.3.1-1: UE harmonic table for 3DLs/1UL

| UE DL Carriers with B1, B5, B7 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 3840 to 3960 |
| 2*fy_low to 2*fy_high | 1648 to 1698 |
| 2*fz_low to 2*fz_high | 5000 to 5140 |
| 3*fx_low to 3*fx_high | 5760 to 5940 |
| 3*fy_low to 3*fy_high | 2472 to 2547 |
| 3*fz_low to 3*fz_high | 7500 to 7710 |

- 6.X.4	ΔTIB,c and ΔRIB,c

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c is defined for applicable bands in Table 6.X.4-1.

Table 6.X.4-1: ΔTIB,c for 3DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-5A-7A | 1 | FFS |
|  | 5 | 0.3 |
|  | 7 | FFS |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table x (in TS 36.101 for 3DLs ΔTIB,c), truncated to one decimal place for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table x (in TS 36.101 for 3DLs ΔTIB,c), that applies for that operating band among the supported CA configurations |  |  |

For the UE which supports inter-band carrier aggregation configuration in Table 6.X.4-2 with uplink in one E-UTRA band, the minimum requirement for reference sensitivity in Table 7.3.1-1 (TS 36.101) shall be increased by the amount given in ΔRIB,c in Table 6.X.4-2 for the applicable E-UTRA bands.

# Table 6.X.4-2: ΔRIB,c for 3DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-5A-7A | 1 | FFS |
|  | 5 | 0.0 |
|  | 7 | FFS |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table y (in TS 36.101 for 3DL ΔRIB,c),, truncated to one decimal place that would apply for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table y (in TS 36.101 for 3DL ΔRIB,c), that would apply for that operating band among the supported CA configurations |  |  |