3GPP TSG-RAN4 Meeting #73-AH-UE-RF 	R4-73AH-0019

# Oulu, Finland, 13 – 16 January 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.853 | CR | CRNum | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_B25_B41_B41-Core |  |  |  |  |  |  | Date: |  | 2015-01-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | The WI to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 was completed in Rel-12. But the related text proposals were approved into the Rel-13 TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Put the corresponding materials into the Rel-12 TR to avoid unnecessary confusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains and unnecessary confusion may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, New clause |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

# 1	Scope

The present document is a technical report for 3DL Inter-band Carrier Aggregation under Rel-12 time frame. The purpose is to gather the relevant background information and studies in order to address 3DL Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address 3DL Inter-band Carrier Aggregation requirements for the Rel-12 band combinations in table 1-1.

Table 1-1: Release 12 3DL inter-band carrier aggregation combinations

| WI code | WI title |
| --- | --- |
| LTE_CA_B2_B2_B13 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2 and Band 1 |
| LTE_CA_B2_B4_B13 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 4 and Band 13 |
| LTE_CA_B2_B5_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 5 and Band 30 |
| LTE_CA_B2_B12_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 30 |
| LTE_CA_B2_B29_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 29 and Band 30 |
| LTE_CA_B4_B4_B13 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 4, Band 4 and Band 13 |
| LTE_CA_B4_B5_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 5 and Band 30 |
| LTE_CA_B4_B12_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 12 and Band 30 |
| LTE_CA_B4_B29_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 29 and Band 30 |
| LTE_CA_B1_B3_B8 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 3 and Band 8 |
| LTE_CA_B1_B5_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 5 and Band 7 |
| LTE_CA_B2_B4_B5 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 4 and Band 5 |
| LTE_CA_B2_B4_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 4 and Band 12 |
| LTE_CA_B2_B5_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 5 and Band 12 |
| LTE_CA_B2_B12_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 12 |
| LTE_CA_B3_B7_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 3, Band 7 and Band 20 |
| LTE_CA_B3_B8_B27 | LTE Advanced 3 band Carrier Aggregation (3DL/1UL) for Band 3, Band 8 and Band 27 |
| LTE_CA_B4_B5_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 5 and Band 12 |
| LTE_CA_B4_B12_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 12 and Band 12 |
| LTE_CA_B1_B3_B5 | LTE Advanced 3 Band Carrier Aggregation(3DL/1UL) for Band 1, Band 3 and Band 5 |
| LTE_CA_B1_B3_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 3 and Band 20 |
| LTE_CA_B1_B7_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 7 and Band 20 |
| LTE_CA_B7_B8_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 7, Band 8 and Band 20 |
| LTE_CA_B3_B7_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 3, Band 7 and Band 7 |
| LTE_CA_B3_B3_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 3, Band 3 and Band 7 |
| LTE_CA_B1_B42_B42 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 42 and Band 42 |
| LTE_CA_B3_B42_B42 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 3, Band 42 and Band 42 |
| LTE_CA_B1_B3_B19 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 3 and Band 19 |
| LTE_CA_B2_B4_B29 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 4 and Band 29 |
| LTE_CA_B4_B4_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 4 and Band 7 |
| LTE_CA_B4_B7_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 7 and Band 12 |
| LTE_CA_B39_B39_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 39, Band 39 and Band 41 |
| LTE_CA_B39_B41_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 39, Band 41 and Band 41 |
| LTE_CA_B2_B2_B29 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 2 and Band 29 |
| LTE_CA_B1_B41_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 41, and Band 41 |
| LTE_CA_B2_B2_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 2, and Band 12 |
| LTE_CA_B25_B41_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 25, Band 41, and Band 41 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<New clause>

## 6.X	LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 25, Band 41, and Band 41

Table 6.X-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_25-41 | 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 6.X-2: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_25A-41C | 25 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | CA_41C | See table below |  |  |  |  |  |  |  |

Table 6.X-3: Supported E-UTRA bandwidths of CA_41C configuration for 3DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_41C | 10 | 20 | 40 | 0 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |
|  | 5, 10 | 20 | 40 | 1 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 5, 10, 15, 20 |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [4] when operating in single carrier mode

##### 6.X.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of contiguous Band 41 DL carriers can be calculated as shown in Table 6.X.1-1 below:

Table 6.X.1-1: Band 41 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -194 | 194 |
| 3rd order IMD products | (2*f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 2302 | 2884 |
| 3rd order IMD products (with maximum channel bandwidth) | (f-low – BWmax) | (f-high + BWmax) |
| IMD frequency limits (MHz) | 2456 | 2730 |

It can be seen from Table 6.X.1-1 that the 3rd order IMD products of BS transmitting contiguous carriers in Band 41 may fall into the BS receive band of Bands 7, 30, 38, 40 and 41. Note that the calculation in Table 6.X.1-1 (except the last row) assumes the BS is transmitting with the whole 194 MHz DL frequency of Band 41. If the BS is only transmitting an up to 40 MHz DL in Band 41 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of the Band 30 or 40 as shown in the last row in Table 6.X.1-1.

Note that Bands 7 and 38 are not intended for use in the same geographical area as Band 41. Moreover, the Band 41 BS deployed in the same geographical area are assumed to be synchronized so that the Band 41 BS receivers will not be interfered by the Band 41 BS transmission. Therefore, the 3rd IMD products of BS transmitting two contiguous carriers in Band 41will not fall into the BS receive band of any operating band currently defined in the 3GPP standards for use in the same geographical area.

##### 6.X.2	Co-existence studies for 1 UL/3 DL

The 3rd order IMD products caused in the BS by transmitting of Band 25, Band 41 and Band 41 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.X.2-1 below:

Table 6.X.2-1: Band 25, Band 41 and Band 41 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1930 | 1995 | 2496 | 2690 | 2496 | 2690 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 1736 |  |  | 2189 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 1736 |  |  | 2189 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 2997 |  |  | 3450 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 6922 |  |  | 7375 |  |  |

It can be seen from Table 6.X.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (25 + 41 + 41) may fall into the BS receive band of Bands 1, 2, 3, 4, 9, 10, 22, 23, 25, 33, 34, 35, 36, 37, 39 and 42. However, Bands 1, 3, 9, 10, 33, 34 and 39 are not intended for use in the same geographical area as Bands 25 and 41. Moreover, co-location of Band 25 transmitter and Band 35, 36 or 37 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Therefore, the focus here is on the 3rd order IMD products falling into Bands 2, 4, 22, 23, 25 and 42. Note that the 3rd order IMD products will not fall into the BS own (up to 20 MHz) receive block within Band 25.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 4, 22 or 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (25 + 41 + 41) BS transmitter does not share the same antenna with Band 4, 22 or 42 BS receiver. But there is not a large frequency gap between Band 25 DL and Bands 2, 23 and 25 UL, and hence Bands 2, 23 and 25 BS receiver desensitization may still be an issue.

Therefore, it is recommended that Band (25 + 41 + 41) BS transmitter should not share the same antenna with Band 2, 23 or 25 BS receiver to prevent BS receiver desensitization, or with Band 4, 22 or 42 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 4, 22 or 42 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

##### 6.X.3	ΔTIB,c and ΔRIB,c

For the UE which supports inter-band carrier aggregation in Band 25A and Band 41C with uplink assigned to Band 25 the ΔTIB,c and ΔRIB,c  is defined for applicable bands in Table 6.X.3-1 and 6.X.3-2 respectively.

Table 6.X.3-1: ΔTIB,c for 3DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_25A-41C | 25 | 0.5 |
|  | 41 | 0.5 |

Table 6.X.3-2: ΔRIB,c for 3DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_25A-41C | 25 | 0 |
|  | 41 | 0 |

<End of change>