3GPP TSG-RAN WG4 NR AH Meeting	R4-1700002

Spokane, Washington, USA, 17 - 19 January, 2017

Agenda Item:	3.1.1.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Consideration on dense urban BS noise figure for coexistence study at 30 GHz for WP5D on new radio access technology

Document for:	Approval

# 1.	Introduction

The revised study item on New Radio Access Technology was approved at TSG RAN#72 [1]. The objectives of this study item include identifying relevant RF parameters to be used for sharing and co-existence studies. Also a LS was received at TSG RAN#72 [2] from ITU-R WP5D asking for characteristics of terrestrial IMT systems for frequency sharing/interference analysis in 24.25 - 86 GHz. ITU-R WP5D sent another LS on “Updated characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz” in [3]. On the other hand, a LS has been sent from ITU-R WP5D to 3GPP describing detailed modelling and simulation of IMT networks for use in sharing and compatibility studies [4]. This topic has been discussed at previous RAN4 meetings, and the way forwards on simulation assumptions were approved in RAN4#80bis [5-7].

This contribution provides the simulation results using the agreed assumptions (with wrap around technique as agreed in RAN4#81 [8]), and proposes consideration on the BS Noise Figure (NF) in the dense urban scenario at 30 GHz for the coexistence study for WP5D in order to facilitate the final output of the study.

# 2.	Discussion

The following BS and UE NF were assumed in the way forward [5]:

Table 1: Assumed BS and UE noise figures

Note 1: At least the following NF are used for simulation assumptions for the RAN4#81 meeting for both UE and BS. 30GHz: 9 and 11 dB, 45GHz: 11 and 13 dB, 70GHz: 13 and 15 dB.

The simulation results on throughput loss of the victim BS at 30 GHz for both 9 and 11 dB BS NF with UL ACIR between 5 and 40 dB are provided in Table 1 below.

Table 1: Throughput loss of victim BS with interfering UE (at 30 GHz)

(a) 9 dB BS NF

| UL ACIR (dB) | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (%) | 1.85 | 0.86 | 0.37 | 0.15 | 0.06 | 0.02 | 0.01 | 0.00 |
| 5%-tile throughput loss (%) | 8.46 | 4.03 | 1.83 | 0.92 | 0.68 | 0.18 | 0.00 | 0.00 |

(b) 11 dB BS NF

| UL ACIR (dB) | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (%) | 1.50 | 0.68 | 0.29 | 0.11 | 0.04 | 0.01 | 0.00 | 0.00 |
| 5%-tile throughput loss (%) | NA | NA | NA | NA | NA | NA | NA | NA |

It is shown in Table 1 that for 11 dB BS NF, the 5%-tile throughput loss are Not Applicable (NA)for the simulated ACIR range between 5 and 40 dB. This is because in these cases, more than 5% of the victim UE have lower than -10 dB UL SINR and thus have zero 5%-tile throughput. This is shown in Figure 1 below where the simulation results on the UL SINR of the victim BS at 30 GHz for BS NF of 9 dB with ACIR of 15 dB are shown.

Figure 1: UL SINR of victim UE at 30 GHz with and without interfering UE (11 dB BS NF)

On the other hand, it can be seen in Table 1 that the average throughput losses of the victim BS are of the same order of magnitude for the simulated UL ACIR range (between 5 and 40 dB) for 9 and 11 dB BS NF. Hence it is proposed to consider the simulation results for 9 dB BS NF at 30GHz (for both average and 5%-tile throughput losses) as a base for deciding the required UL ACIR value.

# 3.	Conclusion and proposal

The simulation results in this contribution have shown that the average throughput losses of the victim BS at 30 GHz are of the same order of magnitude for 9 and 11 dB BS NF, while more than 5% of the victim UE have lower than -10 dB DL SINR and thus have zero 5%-tile throughput for 11 dB BS NF; hence it is proposed to:

Proposal: Consider the simulation results for 9 dB BS NF at 30 GHz (for both average and 5%-tile throughput losses) as a base for deciding the required UL ACIR value.

# References

[1]	RP-161214, “Revision of SI: Study on New Radio Access Technology”, NTT DOCOMO, INC.

[2]	RP-160508, “LS on Characteristics of terrestrial IMT systems for frequency sharing/interf. analysis in 24.25 - 86 GHz”, ITU-R WP5D.

[3]	R4-165140, Updated characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz, ITU-R WP5D

[4]	R4-165141, “Modelling and simulation of IMT networks for use in sharing and compatibility studies”, ITU-R WP5D.

[5]	R4-168953, “WF on Simulation assumptions of Co-existence study for WP5D”, NTT DOCOMO, INC.

[6]	R4-168794, “Further Way forward on Dense urban Layout and BS Beamforming Model”, Huawei, HiSilicon.

[7]	R4-167750, “Clarifications about UE antenna modeling for NR coexistence study”, Qualcomm Incorporated.

[8]	R4-1610153, “Proposal on dense urban network layout for coexistence study for WP5D on new radio access technology”, Nokia, Alcatel-Lucent Shanghai Bell.