3GPP TSG-RAN WG4 meeting #81-NR-AH  	R4-1700067

# Spokane, Washington, USA, 17-19 January 2017

Source: 	Ericsson

Title:  	UE OTA sensitivity for mm-wave frequencies

Agenda Item:	3.1.2.1

Document for:	Approval

# Introduction

As requested ITU-R compatibility parameter list contain sensitivity, this paper proposes a simplified approach to define UE OTA sensitivity even though in [1], there is a discussion on sensitivity being not essential for compatibility studies.

As the RAN4#81AH is the last meeting to conclude on ITU-R parameters, a simplified and pragmatic approach with proposed UE OTA sensitivity would allow for submitting a complete list of compatibility parameters.

It should be noted that, the UE OTA sensitivity estimates in this paper should not be seen as an agreement of what the final NR parameters and characteristics will be, rather that more work is required during the WI phase.

# Discussion

Considering the other related ITU-R parameters such as bandwidth and the simulation assumptions on number of antennas which is also used in this paper, the following additional aspects should be considered.

- There is no need to develop any specific NR FRCs at this stage.

- SINR can be chosen from the SINR range which is also a parameter requested by ITU-R. For simplification, we would propose SINR = 0 dB.

- The noise figure values of 10 dB, 12 dB and 14 dB can be used 30 GHz, 45 GHz and 70 GHz respectively.

- An implementation margin of ~1.5 dB can be used to calculate the UE OTA sensitivity at this stage.

Thus the UE OTA sensitivity can be expressed as:

OTA sensitivity= -174 + NF + 10 log BW + SINR - 10 log (n) - 5 (element gain) + implementation margin

Table 1	UE OTA sensitivity estimations

| Frequency range | UE OTA sensitivity |
| --- | --- |
| 30 GHz | -93,5 dBm |
| 45 GHz | -91.5 dBm |
| 70 GHz | -89.5 dBm |

These estimated values can be used for the ITU-R related work.

# Conclusion

In this paper, as sensitivity is a parameter requested by ITU-R, a simplified and pragmatic levels for UE OTA sensitivity is presented. The estimated values are based on other related ITU-R parameters such as bandwidth, noise figure and also simulation assumptions in terms of number of antennas etc.

We thus propose to use these estimated values for the ITU-R response should RAN4 decide to submit explicit values.

# References

[1]  	R4-1610616, “Way forward on IMT parameters WP5D”, Ericsson