3GPP TSG RAN WG R5 Meeting #39	R5-081570

- Kansas City, USA, 5th - 9th May 2008

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | CRNum |  rev | - |  Current version: | 8.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Corrections to timers in GPS Notification test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Spirent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | R5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI8_Test |  |  |  |  |  |  | Date:  |  | 28/04/2008 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8 Rel-9 (Release 9) |  |
| Reason for change:  |  | The timer T(LCSN) is a Network timer (defined in TS 24.030 - Section 4.1.1) and has a suggested range (defined in TS 24.080 in a comment in the ASN.1) of 10s to 20s. The UE also has an internal timer with a similar function, but this is undefined in any core specification. For the call flows tested in the Notification test cases 17.2.4.6 to 17.2.4.9, T(LCSN) must expire before the timer in the UE. The UE LCS Notification timer should be added to the test cases so that T(LCSN) will always be set to a value that is dependent on this and not to 20 seconds. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | PIXIT added to provide the value of the UE LCS Notification timeout timer in the UE. PIXIT will allow the conformance test case to be carried out in the true spirit of the test without compromising the purpose of the test. T(LCSN) set to 90% of the value of the UE LCS Notification timeout timer in the UE. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Test cases 17.2.4.6 to 17.2.4.9 may unfairly fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 17.2.4.6.4, 17.2.4.7.4, 17.2.4.8.4, 17.2.4.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 34.123-3 |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

#### 17.2.4.6	LCS Mobile terminated location request/ UE-Based GPS/ Privacy Verification/ Location Allowed if No Response

17.2.4.6.1	Definition

This test case applies to all UEs supporting UE-Based GPS Location Service capabilities.

17.2.4.6.2	Conformance requirements

1)	The network invokes a location notification procedure by sending a REGISTER message containing a LCS-LocationNotification invoke component to the UE. This may be sent either to request verification for MT-LR or to notify about already authorized MT-LR.

2) 	In case of privacy verification the MS shall respond to the request by sending a RELEASE COMPLETE message containing the mobile subscriber's response in a return result component.

3) 	If the timer expires in the network before any response from the MS (e.g. due to no response from the user), the network shall interpret this by applying the default treatment defined in GSM 03.71 for GSM and TS 23.171 for UMTS (i.e. disallow location if barred by subscription and allow location if allowed by subscription).

4)	if the IE "Measurement command" has the value "setup":

2>	store this measurement in the variable MEASUREMENT_IDENTITY according to the IE "measurement identity", first releasing any previously stored measurement with that identity if that exists;

…

2>	for any other measurement type:

3>	if the measurement is valid in the current RRC state of the UE:

4>	begin measurements according to the stored control information for this measurement identity.

5)	if the IE "Measurement command" has the value "modify":

2>	for all IEs present in the MEASUREMENT CONTROL message:

3>	if a measurement was stored in the variable MEASUREMENT_IDENTITY associated to the identity by the IE "measurement identity":

4>	if measurement type is set to "UE positioning measurement" and the IE "UE positioning GPS assistance data" is present, for any of the optional IEs "UE positioning GPS reference time", "UE positioning GPS reference UE position", "UE positioning GPS DGPS corrections", "UE positioning GPS ionospheric model", "UE positioning GPS UTC model", "UE positioning GPS acquisition assistance", "UE positioning GPS real-time integrity" that are present in the MEASUREMENT CONTROL message:

5>	replace all instances of the IEs listed above (and all their children) stored in variable MEASUREMENT_IDENTITY associated to the identity indicated by the IE "measurement identity" with the IEs received in the MEASUREMENT CONTROL message;

5>	leave all other stored information elements unchanged in the variable MEASUREMENT_IDENTITY.

6)	If the IE "UE positioning GPS Navigation Model" is included, for each satellite, the UE shall:

1>	use IE "Satellite Status" to determine if an update of IE "UE positioning GPS Ephemeris and Clock Correction parameters" has been provided for the satellite indicated by the IE "SatID";

1>	if an update has been provided for this satellite:

2>	act as specified in subclause 8.6.7.19.3.4 of TS 25.331.

7)	If the IE "UE positioning GPS Ephemeris and Clock Correction parameters" is included, for each satellite, the UE shall:

1>	update the variable UE_POSITIONING_GPS_DATA as follows:

2>	store this IE at the position indicated by the IE "Sat ID" in the IE "UE positioning GPS Navigation Model" in the variable UE_POSITIONING_GPS_DATA, possibly overwriting any existing information in this position.

1>	act on these GPS ephemeris and clock correction parameters in a manner similar to that specified in ICD-GPS-200.

8)	If the IE "UE positioning GPS reference time" is included, the UE shall:

1>	store the IE "GPS Week" in "UE positioning GPS reference time" in variable UE_POSITIONING_GPS_DATA and use it as the current GPS week;

1>	store the IE "GPS TOW msec" in the IE "UE positioning GPS reference time" in variable UE_POSITIONING_GPS_DATA and use it as an estimate of the GPS Time-of-Week at the time of reception of the complete message containing the IE "GPS TOW msec";

NOTE:	The UE does not need to apply any compensation on the GPS Time-of-Week.

9)	If the IE "UE positioning GPS reference UE position" is included, the UE shall:

1>	store this IE in the IE "UE positioning GPS reference UE position" in variable UE_POSITIONING_GPS_DATA; and

1>	use it as a priori knowledge of the approximate location of the UE.

10)	The UE shall when a measurement report is triggered:

2>	if the UE has been able to calculate a position after performing measurements on the cells included in the variable UE_POSITIONING_OTDOA_DATA_UE_BASED in case of OTDOA or on the list of satellites included in the variable UE_POSITIONING_GPS_DATA in case of GPS positioning:

3>	include IE "UE positioning Position Estimate Info" in the MEASUREMENT REPORT and set the contents of the IE as follows:

4>	if the UE does not support the capability to perform the UE GPS timing of cell frames measurement; or

4>	if the IE "GPS timing of Cell wanted" is set to FALSE:

5>	include the IE "GPS TOW msec".

4>	if IE "Vertical Accuracy" has been included in IE "UE positioning reporting quantity":

5>	if the IE "Vertical Accuracy" has been assigned to a value unequal to "0":

6>	if the UE has been able to calculate a 3-dimensional position:

7>	include IE "Ellipsoid point with altitude and uncertainty ellipsoid" as the position estimate.

6>	if the UE has not been able to calculate a 3-dimensional position:

7>	act as if IE "Vertical Accuracy" has not been included in IE "UE positioning reporting quantity".

4>	if IE "Vertical Accuracy" has not been included in IE "UE positioning reporting quantity":

5>	if IE "Horizontal Accuracy" in IE "UE positioning reporting quantity" has been assigned to value "0":

6>	may include IE "Ellipsoid point".

5>	if IE "Horizontal Accuracy" in IE "UE positioning reporting quantity" has been assigned to a value unequal to 0:

6> include either IE "Ellipsoid point with uncertainty circle" or IE "Ellipsoid point with uncertainty ellipse" or IE "Ellipsoid point with altitude and uncertainty ellipsoid" as the position estimate.

References

-	Conformance requirement 1, 2 and 3: TS 24.030, clause 4.1.1.

--	Conformance requirements 4 and 5: TS 25.331, subclause 8.4.1.3

-	Conformance requirement 6: TS 25.331, clause 8.6.7.19.3.3a.

-	Conformance requirement 7: TS 25.331, clause 8.6.7.19.3.4.

-	Conformance requirement 8: TS 25.331, clause 8.6.7.19.3.7.

-	Conformance requirement 9: TS 25.331, clause 8.6.7.19.3.8.

-	Conformance requirement 10: TS 25.331, clause 8.6.7.19.1b.

17.2.4.6.3	Test Purpose

To verify that when the UE receives a REGISTER message, containing a LCS Location Notification Invoke component set to notifyAndVerify-LocationAllowedIfNoResponse, the UE notifies the user of the request and indicates that the default response is location allowed if no response and providing the opportunity to accept or deny the request and sends a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionDenied or permissionGranted as appropriate.

17.2.4.6.4	Method of Test

Initial Conditions

System Simulator (SS):

-	1 cell, default parameters

-	Satellites: As specified in 17.2.1.2

UE:

-	State “CS-CELL DCH Initial (State 6-1)” as specified in clause 7.4.1 of TS 34.108.

Related PICS/PIXIT Statements

-	UE Based Network Assisted GPS

-	px_UeLcsNotification: value for UE LCS Notification timeout timer

Test Procedure

The SS initiates authentication and ciphering and sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse.

The LCS Client Name contained in the USSD text string of the lcs-LocationNotification should be displayed with the option to accept or deny the request and an indication that location will be allowed if no user response is received.

The user accepts the location request. The UE responds with a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionGranted.

The SS orders an A-GPS positioning measurement using MEASUREMENT CONTROL messages.

The UE sends a MEASUREMENT REPORT message including a location estimate.

The SS sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse.

The user denies the location request. The UE responds with a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionDenied.

The SS sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse.

The user ignores the location request by taking no action.

The SS orders an A-GPS positioning measurement using MEASUREMENT CONTROL messages.

The UE then sends a MEASUREMENT REPORT message including a location estimate.

Expected Sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | AUTHENTICATION REQUEST |  |
| 2 | --> |  | AUTHENTICATION RESPONSE |  |
| 3 | SS |  |  | SS starts security procedure |
| 4 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse |
| 5 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 6 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be allowed in the absence of a response |
| 7 | UE |  |  | The user accepts the location request before timer T(LCSN) expires |
| 8 | -> |  | RELEASE COMPLETE | Containing a LocationNotification return result with verificationResponse set to permissionGranted |
| 9 | <- |  | MEASUREMENT CONTROL |  |
| 10 | <- |  | MEASUREMENT CONTROL |  |
| 11 | -> |  | MEASUREMENT REPORT |  |
| 12 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse |
| 13 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 14 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be allowed in the absence of a response |
| 15 | UE |  |  | The user denies the location request before timer T(LCSN) expires |
| 16 | -> |  | RELEASE COMPLETE | Containing a LocationNotification return result with verificationResponse set to permissionDenied |
| 17 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse |
| 18 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 19 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be allowed in the absence of a response |
| 20 | UE |  |  | The user does not reply |
| 21 | SS |  |  | SS waits until T(LCSN) expires to ensure that the UE does not send a RELEASE COMPLETE message. |
| 22 | <- |  | RELEASE COMPLETE | SS terminates the dialogue |
| 23 | <- |  | MEASUREMENT CONTROL |  |
| 24 | <- |  | MEASUREMENT CONTROL |  |
| 25 | -> |  | MEASUREMENT REPORT |  |
| 26 | SS |  |  | SS releases the connection and the test case ends |

Specific Message Contents

REGISTER (Step 4)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 8)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (xx10 1010) |
| Facility | Return result = LCS-LocationNotification LocationNotificationRes verificationResponse -> permissionGranted |

MEASUREMENT CONTROL (Step 9):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Setup |
| Measurement Reporting Mode |  |
| - Measurement report transfer mode | Acknowledged mode RLC |
| - Periodical reporting / Event trigger reporting mode | Periodical reporting |
| Additional Measurements List | Not present |
| CHOICE Measurement type | UE positioning measurement |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE based |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | FALSE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria |  |
| - No reporting |  |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified for the first MEASUREMENT CONTROL message for “Adequate assistance data for UE-based A-GPS” in 17.2.1.3.1 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

MEASUREMENT CONTROL (Step 10):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Modify |
| Measurement Reporting Mode |  |
| - Measurement report transfer mode | Acknowledged mode RLC |
| - Periodical reporting / Event trigger reporting mode | Periodical reporting |
| Additional Measurements List | Not present |
| CHOICE Measurement type | UE positioning measurement |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE based |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | FALSE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria | Periodical reporting criteria |
| - Amount of reporting | 1 |
| - Reporting interval | 64000 |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified for the second MEASUREMENT CONTROL message for “Adequate assistance data for UE-based A-GPS” in 17.2.1.3.1 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

MEASUREMENT REPORT (Step 11)

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measured Results |  |
| - CHOICE Measurement |  |
| - UE positioning measured results |  |
| - UE positioning OTDOA measured results | Not present |
| - UE positioning position estimate info |  |
| - CHOICE Reference time |  |
| - GPS reference time only |  |
| - GPS TOW msec | Not checked |
| - CHOICE Position estimate | One of ‘Ellipsoid point with uncertainty Circle’ or ‘Ellipsoid point with uncertainty Ellipse’ or ‘Ellipsoid point with altitude and uncertainty Ellipsoid’ |
| - UE positioning GPS measured results | Not present |
| - UE positioning error | Not present |
| Measured Results on RACH | Not present |
| Additional Measured Results | Not present |
| Event Results | Not present |

REGISTER (Step 12)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 16)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (xx10 1010) |
| Facility | Return result = LCS-LocationNotification LocationNotificationRes verificationResponse -> permissionDenied |

REGISTER (Step 17)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 22)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (0010 1010) |

MEASUREMENT CONTROL (Step 23):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Setup |
| Measurement Reporting Mode |  |
| - Measurement report transfer mode | Acknowledged mode RLC |
| - Periodical reporting / Event trigger reporting mode | Periodical reporting |
| Additional Measurements List | Not present |
| CHOICE Measurement type | UE positioning measurement |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE based |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | FALSE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria |  |
| - No reporting |  |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified for the first MEASUREMENT CONTROL message for “Adequate assistance data for UE-based A-GPS” in 17.2.1.3.1 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

MEASUREMENT CONTROL (Step 24):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Modify |
| Measurement Reporting Mode | Not present |
| Additional Measurements List | Not present |
| CHOICE Measurement type |  |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE based |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | Set according to 17.2.1.2 (unequal to 0) |
| - Vertical accuracy | Set according to 17.2.1.2 (unequal to 0) |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | FALSE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria | Periodical reporting criteria |
| - Amount of reporting | 1 |
| - Reporting interval | 64000 |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified for the second MEASUREMENT CONTROL message for “Adequate assistance data for UE-based A-GPS” in 17.2.1.3.1 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

MEASUREMENT REPORT (Step 25)

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measured Results |  |
| - CHOICE Measurement |  |
| - UE positioning measured results |  |
| - UE positioning OTDOA measured results | Not present |
| - UE positioning position estimate info |  |
| - CHOICE Reference time |  |
| - GPS reference time only |  |
| - GPS TOW msec | Not checked |
| - CHOICE Position estimate | One of ‘Ellipsoid point with uncertainty Circle’ or ‘Ellipsoid point with uncertainty Ellipse’ or ‘Ellipsoid point with altitude and uncertainty Ellipsoid’ |
| - UE positioning GPS measured results | Not present |
| - UE positioning error | Not present |
| Measured Results on RACH | Not present |
| Additional Measured Results | Not present |
| Event Results | Not present |

17.2.4.6.5	Test requirements

After step 7 the UE shall send a RELEASE COMPLETE message with verificationResponse set to permissionGranted.

After step 10 the UE shall respond with a MEASUREMENT REPORT message containing a UE position estimate.

After step 15 the UE shall send a RELEASE COMPLETE message with verificationResponse set to permissionDenied.

After step 24 the UE shall respond with a MEASUREMENT REPORT message containing a UE position estimate.

#### 17.2.4.7	LCS Mobile terminated location request/ UE-Based GPS/ Privacy Verification/ Location Not Allowed if No Response

17.2.4.7.1	Definition

This test case applies to all UEs supporting UE-Based GPS Location Service capabilities.

17.2.4.7.2	Conformance requirements

1)	The network invokes a location notification procedure by sending a REGISTER message containing a LCS-LocationNotification invoke component to the UE. This may be sent either to request verification for MT-LR or to notify about already authorized MT-LR.

2) 	In case of privacy verification the MS shall respond to the request by sending a RELEASE COMPLETE message containing the mobile subscriber's response in a return result component.

3) 	If the timer expires in the network before any response from the MS (e.g. due to no response from the user), the network shall interpret this by applying the default treatment defined in GSM 03.71 for GSM and TS 23.171 for UMTS (i.e. disallow location if barred by subscription and allow location if allowed by subscription).

4)	if the IE "Measurement command" has the value "setup":

2>	store this measurement in the variable MEASUREMENT_IDENTITY according to the IE "measurement identity", first releasing any previously stored measurement with that identity if that exists;

…

2>	for any other measurement type:

3>	if the measurement is valid in the current RRC state of the UE:

4>	begin measurements according to the stored control information for this measurement identity.

5)	if the IE "Measurement command" has the value "modify":

2>	for all IEs present in the MEASUREMENT CONTROL message:

3>	if a measurement was stored in the variable MEASUREMENT_IDENTITY associated to the identity by the IE "measurement identity":

4>	if measurement type is set to "UE positioning measurement" and the IE "UE positioning GPS assistance data" is present, for any of the optional IEs "UE positioning GPS reference time", "UE positioning GPS reference UE position", "UE positioning GPS DGPS corrections", "UE positioning GPS ionospheric model", "UE positioning GPS UTC model", "UE positioning GPS acquisition assistance", "UE positioning GPS real-time integrity" that are present in the MEASUREMENT CONTROL message:

5>	replace all instances of the IEs listed above (and all their children) stored in variable MEASUREMENT_IDENTITY associated to the identity indicated by the IE "measurement identity" with the IEs received in the MEASUREMENT CONTROL message;

5>	leave all other stored information elements unchanged in the variable MEASUREMENT_IDENTITY.

6)	If the IE "UE positioning GPS Navigation Model" is included, for each satellite, the UE shall:

1>	use IE "Satellite Status" to determine if an update of IE "UE positioning GPS Ephemeris and Clock Correction parameters" has been provided for the satellite indicated by the IE "SatID";

1>	if an update has been provided for this satellite:

2>	act as specified in subclause 8.6.7.19.3.4 of TS 25.331.

7)	If the IE "UE positioning GPS Ephemeris and Clock Correction parameters" is included, for each satellite, the UE shall:

1>	update the variable UE_POSITIONING_GPS_DATA as follows:

2>	store this IE at the position indicated by the IE "Sat ID" in the IE "UE positioning GPS Navigation Model" in the variable UE_POSITIONING_GPS_DATA, possibly overwriting any existing information in this position.

1>	act on these GPS ephemeris and clock correction parameters in a manner similar to that specified in ICD-GPS-200.

8)	If the IE "UE positioning GPS reference time" is included, the UE shall:

1>	store the IE "GPS Week" in "UE positioning GPS reference time" in variable UE_POSITIONING_GPS_DATA and use it as the current GPS week;

1>	store the IE "GPS TOW msec" in the IE "UE positioning GPS reference time" in variable UE_POSITIONING_GPS_DATA and use it as an estimate of the GPS Time-of-Week at the time of reception of the complete message containing the IE "GPS TOW msec";

NOTE:	The UE does not need to apply any compensation on the GPS Time-of-Week.

9)	If the IE "UE positioning GPS reference UE position" is included, the UE shall:

1>	store this IE in the IE "UE positioning GPS reference UE position" in variable UE_POSITIONING_GPS_DATA; and

1>	use it as a priori knowledge of the approximate location of the UE.

10)	The UE shall when a measurement report is triggered:

2>	if the UE has been able to calculate a position after performing measurements on the cells included in the variable UE_POSITIONING_OTDOA_DATA_UE_BASED in case of OTDOA or on the list of satellites included in the variable UE_POSITIONING_GPS_DATA in case of GPS positioning:

3>	include IE "UE positioning Position Estimate Info" in the MEASUREMENT REPORT and set the contents of the IE as follows:

4>	if the UE does not support the capability to perform the UE GPS timing of cell frames measurement; or

4>	if the IE "GPS timing of Cell wanted" is set to FALSE:

5>	include the IE "GPS TOW msec".

4>	if IE "Vertical Accuracy" has been included in IE "UE positioning reporting quantity":

5>	if the IE "Vertical Accuracy" has been assigned to a value unequal to "0":

6>	if the UE has been able to calculate a 3-dimensional position:

7>	include IE "Ellipsoid point with altitude and uncertainty ellipsoid" as the position estimate.

6>	if the UE has not been able to calculate a 3-dimensional position:

7>	act as if IE "Vertical Accuracy" has not been included in IE "UE positioning reporting quantity".

4>	if IE "Vertical Accuracy" has not been included in IE "UE positioning reporting quantity":

5>	if IE "Horizontal Accuracy" in IE "UE positioning reporting quantity" has been assigned to value "0":

6>	may include IE "Ellipsoid point".

5>	if IE "Horizontal Accuracy" in IE "UE positioning reporting quantity" has been assigned to a value unequal to 0:

6> include either IE "Ellipsoid point with uncertainty circle" or IE "Ellipsoid point with uncertainty ellipse" or IE "Ellipsoid point with altitude and uncertainty ellipsoid" as the position estimate.

References

-	Conformance requirement 1, 2 and 3: TS 24.030, clause 4.1.1.

-	Conformance requirements 4 and 5: TS 25.331, clause 8.4.1.3.

-	Conformance requirement 6: TS 25.331, clause 8.6.7.19.3.3a.

-	Conformance requirement 7: TS 25.331, clause 8.6.7.19.3.4.

-	Conformance requirement 8: TS 25.331, clause 8.6.7.19.3.7.

-	Conformance requirement 9: TS 25.331, clause 8.6.7.19.3.8.

-	Conformance requirement 10: TS 25.331, clause 8.6.7.19.1b.

17.2.4.7.3	Test Purpose

To verify that when the UE receives a REGISTER message, containing a LCS Location Notification Invoke component set to notifyAndVerify-LocationNotAllowedIfNoResponse, the UE notifies the user of the request and indicates that the default response is location not allowed if no response and providing the opportunity to accept or deny the request and sends a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionDenied or permissionGranted as appropriate.

17.2.4.7.4	Method of Test

Initial Conditions

System Simulator (SS):

-	1 cell, default parameters

-	Satellites: As specified in 17.2.1.2

UE:

-	State “CS-CELL DCH Initial (State 6-1)” as specified in clause 7.4.1 of TS 34.108.

Related PICS/PIXIT Statements

-	UE Based Network Assisted GPS

-	px_UeLcsNotification: value for UE LCS Notification timeout timer

Test Procedure

The SS initiates authentication and ciphering and sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse.

The LCS Client Name contained in the USSD text string of the lcs-LocationNotification should be displayed with the option to accept or deny the request and an indication that location will be not allowed if no user response is received.

The user accepts the location request. The UE responds with a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionGranted.

The SS orders an A-GPS positioning measurement using MEASUREMENT CONTROL messages.

The UE sends a MEASUREMENT REPORT message including a location estimate.

The SS sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse.

The user denies the location request. The UE responds with a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionDenied.

The SS sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse.

The user ignores the location request by taking no action. If the timer expires in the SS before any response from the UE is received, the SS interprets this by applying the default treatment LocationNotAllowed.

Expected Sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | AUTHENTICATION REQUEST |  |
| 2 | --> |  | AUTHENTICATION RESPONSE |  |
| 3 | SS |  |  | SS starts security procedure |
| 4 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse |
| 5 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 6 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be not allowed in the absence of a response |
| 7 | UE |  |  | The user accepts the location request before timer T(LCSN) expires |
| 8 | -> |  | RELEASE COMPLETE | Containing a LocationNotification return result with verificationResponse set to permissionGranted |
| 9 | <- |  | MEASUREMENT CONTROL |  |
| 10 | <- |  | MEASUREMENT CONTROL |  |
| 11 | -> |  | MEASUREMENT REPORT |  |
| 12 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse |
| 13 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 14 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be not allowed in the absence of a response |
| 15 | UE |  |  | The user denies the location request before timer T(LCSN) expires |
| 16 | -> |  | RELEASE COMPLETE | Containing a LocationNotification return result with verificationResponse set to permissionDenied |
| 17 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse |
| 18 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 19 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be not allowed in the absence of a response |
| 20 | UE |  |  | The user does not reply |
| 21 | SS |  |  | SS waits until T(LCSN) expires to verify that the UE does not send a RELEASE COMPLETE message. |
| 22 | <- |  | RELEASE COMPLETE | SS terminates the dialogue |
| 23 | SS |  |  | SS releases the connection and the test case ends |

Specific Message Contents

REGISTER (Step 4)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationNotAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 8)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (xx10 1010) |
| Facility | Return result = LCS-LocationNotification LocationNotificationRes verificationResponse -> permissionGranted |

MEASUREMENT CONTROL (Step 9):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Setup |
| Measurement Reporting Mode |  |
| - Measurement report transfer mode | Acknowledged mode RLC |
| - Periodical reporting / Event trigger reporting mode | Periodical reporting |
| Additional Measurements List | Not present |
| CHOICE Measurement type | UE positioning measurement |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE based |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | FALSE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria |  |
| - No reporting |  |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified for the first MEASUREMENT CONTROL message for “Adequate assistance data for UE-based A-GPS” in 17.2.1.3.1 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

MEASUREMENT CONTROL (Step 10):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Modify |
| Measurement Reporting Mode | Not present |
| Additional Measurements List | Not present |
| CHOICE Measurement type |  |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE based |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | FALSE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria | Periodical reporting criteria |
| - Amount of reporting | 1 |
| - Reporting interval | 64000 |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified for the second MEASUREMENT CONTROL message for “Adequate assistance data for UE-based A-GPS” in 17.2.1.3.1 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

MEASUREMENT REPORT (Step 11)

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measured Results |  |
| - CHOICE Measurement |  |
| - UE positioning measured results |  |
| - UE positioning OTDOA measured results | Not present |
| - UE positioning position estimate info |  |
| - CHOICE Reference time |  |
| - GPS reference time only |  |
| - GPS TOW msec | Not checked |
| - CHOICE Position estimate | One of ‘Ellipsoid point with uncertainty Circle’ or ‘Ellipsoid point with uncertainty Ellipse’ or ‘Ellipsoid point with altitude and uncertainty Ellipsoid’ |
| - UE positioning GPS measured results | Not present |
| - UE positioning error | Not present |
| Measured Results on RACH | Not present |
| Additional Measured Results | Not present |
| Event Results | Not present |

REGISTER (Step 12)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationNotAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 16)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (xx10 1010) |
| Facility | Return result = LCS-LocationNotification LocationNotificationRes verificationResponse -> permissionDenied |

REGISTER (Step 17)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationNotAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 22)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (0010 1010) |

17.2.4.7.5	Test requirements

After step 7 the UE shall send a RELEASE COMPLETE message with verificationResponse set to permissionGranted.

After step 10 the UE shall respond with a MEASUREMENT REPORT message containing a UE position estimate.

After step 15 the UE shall send a RELEASE COMPLETE message with verificationResponse set to permissionDenied.

During step 21 the UE shall not send any RELEASE COMPLETE message.

#### 17.2.4.8	LCS Mobile terminated location request/ UE-Assisted GPS/ Privacy Verification/ Location Allowed if No Response

17.2.4.8.1	Definition

This test case applies to all UEs supporting UE-Assisted GPS Location Service capabilities.

17.2.4.8.2	Conformance requirements

1)	The network invokes a location notification procedure by sending a REGISTER message containing a LCS-LocationNotification invoke component to the UE. This may be sent either to request verification for MT-LR or to notify about already authorized MT-LR.

2) 	In case of privacy verification the MS shall respond to the request by sending a RELEASE COMPLETE message containing the mobile subscriber's response in a return result component.

3) 	If the timer expires in the network before any response from the MS (e.g. due to no response from the user), the network shall interpret this by applying the default treatment defined in GSM 03.71 for GSM and TS 23.171 for UMTS (i.e. disallow location if barred by subscription and allow location if allowed by subscription).

4)	if the IE "Measurement command" has the value "setup":

2>	store this measurement in the variable MEASUREMENT_IDENTITY according to the IE "measurement identity", first releasing any previously stored measurement with that identity if that exists;

…

2>	for any other measurement type:

3>	if the measurement is valid in the current RRC state of the UE:

4>	begin measurements according to the stored control information for this measurement identity.

5)	The UE shall:

1> when a measurement report is triggered:

2> if the UE was able to perform measurements on at least one neighbour cell included in the variable  UE_POSITIONING_OTDOA_DATA_UE_ASSISTED in case of OTDOA or one satellite included in the variable UE_POSITIONING_GPS_DATA in case of GPS positioning or one cell from the active set in case of CELL ID:

3>	if the IE "Vertical Accuracy" is included:

4>	interpret the presence of this IE to indicate that the UTRAN desires to compute a 3-dimensional position estimate.

3>	if the IE "Positioning Methods" is set to "GPS":

4>	include the IE "UE positioning GPS measured results" in the measurement report and set the contents of the IE as follows:

5>	if the UE supports the capability to provide the GPS timing of the cell frames measurement:

6>	if the IE "GPS timing of Cell wanted" is set to TRUE:

7>	perform the UE GPS timing of cell frames measurement on the serving cell or on one cell of the active set.

7>	include the IE "Primary CPICH Info" for FDD or the IE "cell parameters id" for TDD; and

7>	include the IE "Reference SFN" and the IE "UE GPS timing of cell frames".

6>	if the IE "GPS timing of Cell wanted" is set to FALSE:

7>	include the IE "GPS TOW msec".

5>	if the UE does not support the capability to provide the GPS timing of the cell:

6>	include the IE "GPS TOW msec".

References

-	Conformance requirement 1, 2 and 3: TS 24.030, clause 4.1.1.

-	Conformance requirement 4: TS 25.331, clause 8.4.1.3.

-	Conformance requirement 5: TS 25.331, clause 8.6.7.19.3.3b.

17.2.4.8.3	Test Purpose

To verify that when the UE receives a REGISTER message, containing a LCS Location Notification Invoke component set to notifyAndVerify-LocationAllowedIfNoResponse, the UE notifies the user of the request and indicates that the default response is location allowed if no response and providing the opportunity to accept or deny the request and sends a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionDenied or permissionGranted as appropriate.

17.2.4.8.4	Method of Test

Initial Conditions

System Simulator (SS):

-	1 cell, default parameters

-	Satellites: As specified in 17.2.1.2

UE:

-	State "CS-CELL DCH Initial (State 6-1)" as specified in clause 7.4.1 of TS 34.108.

Related PICS/PIXIT Statements

-	UE Assisted Network Assisted GPS

-	px_UeLcsNotification: value for UE LCS Notification timeout timer

Test Procedure

The SS initiates authentication and ciphering and sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse.

The LCS Client Name contained in the USSD text string of the lcs-LocationNotification should be displayed with the option to accept or deny the request and an indication that location will be allowed if no user response is received.

The user accepts the location request. The UE responds with a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionGranted.

The SS orders an A-GPS positioning measurement using a MEASUREMENT CONTROL message, including assistance data as specified in section 17.2.1.3.3.    The UE may request additional assistance data by sending a MEASUREMENT REPORT message containing a positioning error indication with the IE “Error reason” set to “Assistance Data Missing”.  If the UE requests additional assistance data, the SS provides the requested assistance data in one or more MEASUREMENT CONTROL messages.

The UE sends a MEASUREMENT REPORT message including IE “UE positioning GPS measured results”.

The SS sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse.

The SS sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse.

The user denies the location request. The UE responds with a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionDenied.

The SS sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse.

The user ignores the location request by taking no action. If the timer expires in the SS before any response from the UE is received, the SS interprets this by applying the default treatment LocationAllowed.

The SS orders an A-GPS positioning measurement using a MEASUREMENT CONTROL message, including assistance data as specified in section 17.2.1.3.3.  The UE may request additional assistance data by sending a MEASUREMENT REPORT message containing a positioning error indication with the IE “Error reason” set to “Assistance Data Missing”.  If the UE requests additional assistance data, the SS provides the requested assistance data in one or more MEASUREMENT CONTROL messages.

The UE then sends a MEASUREMENT REPORT message including IE “UE positioning GPS measured results”.

Expected Sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | AUTHENTICATION REQUEST |  |
| 2 | --> |  | AUTHENTICATION RESPONSE |  |
| 3 | SS |  |  | SS starts security procedure |
| 4 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse |
| 5 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 6 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be allowed in the absence of a response |
| 7 | UE |  |  | The user accepts the location request before timer T(LCSN) expires |
| 8 | -> |  | RELEASE COMPLETE | Containing a LocationNotification return result with verificationResponse set to permissionGranted |
| 9 | <- |  | MEASUREMENT CONTROL | Assistance data set as specified for “Adequate assistance data for UE-assisted A-GPS” in 17.2.1.3.3 |
| 10 | -> |  | MEASUREMENT REPORT | UE reports positioning measurement results (Option 1) or requests additional assistance data (Option 2). |
| 10a | <- |  | MEASUREMENT CONTROL | If UE requested additional assistance data in step 10, SS provides the requested data in one or more MEASUREMENT CONTROL messages as specified in section 17.2.1.3.5. |
| 10b | -> |  | MEASUREMENT REPORT | If UE requested additional assistance data in step 10, this message contains the IE “UE positioning GPS measured results”. |
| 11 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse |
| 12 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 13 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be allowed in the absence of a response |
| 14 | UE |  |  | The user denies the location request before timer T(LCSN) expires |
| 15 | -> |  | RELEASE COMPLETE | Containing a LocationNotification return result with verificationResponse set to permissionDenied |
| 16 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationAllowedIfNoResponse |
| 17 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 18 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be allowed in the absence of a response |
| 19 | UE |  |  | The user does not reply |
| 20 | SS |  |  | SS waits until T(LCSN) expires to verify that the UE does not send a RELEASE COMPLETE message. |
| 21 | <- |  | RELEASE COMPLETE | SS terminates the dialogue |
| 22 | <- |  | MEASUREMENT CONTROL | Assistance data set as specified for “Adequate assistance data for UE-assisted A-GPS” in 17.2.1.3.5 |
| 23 | -> |  | MEASUREMENT REPORT | UE reports positioning measurement results (Option 1) or requests additional assistance data (Option 2). |
| 23a | <- |  | MEASUREMENT CONTROL | If UE requested additional assistance data in step 23, SS provides the requested data in one or more MEASUREMENT CONTROL messages as specified in section 17.2.1.3.5. |
| 23b | -> |  | MEASUREMENT REPORT | If UE requested additional assistance data in step 23, this message contains the IE “UE positioning GPS measured results”. |
| 24 | SS |  |  | SS releases the connection and the test case ends |

Specific Message Contents

REGISTER (Step 4)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 8)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (xx10 1010) |
| Facility | Return result = LCS-LocationNotification LocationNotificationRes verificationResponse -> permissionGranted |

MEASUREMENT CONTROL (Step 9):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Setup |
| Measurement Reporting Mode |  |
| - Measurement report transfer mode | Acknowledged mode RLC |
| - Periodical reporting / Event trigger reporting mode | Periodical reporting |
| Additional Measurements List | Not present |
| CHOICE Measurement type | UE positioning measurement |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE assisted |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | TRUE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria | Periodical reporting criteria |
| - Amount of reporting | 1 |
| - Reporting interval | 64000 |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified for “Adequate assistance data for UE-assisted A-GPS” in 17.2.1.3.3 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

MEASUREMENT REPORT (Steps 10 (Option 1) or 10b (Option 2))

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measured Results |  |
| - CHOICE Measurement |  |
| - UE positioning measured results |  |
| - UE positioning OTDOA measured results | Not present |
| - UE positioning position estimate info | Not present |
| - UE positioning GPS measured results | Present |
| - UE positioning error | Not present |
| Measured Results on RACH | Not present |
| Additional Measured Results | Not present |
| Event Results | Not present |

MEASUREMENT REPORT (Step 10 (Option 2)):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measured Results |  |
| - CHOICE Measurement |  |
| - UE positioning measured results |  |
| - UE positioning OTDOA measured results | Not present |
| - UE positioning position estimate info | Not present |
| - UE positioning GPS measured results | Not present |
| - UE positioning error |  |
| - Error reason | Assistance Data Missing |
| - GPS additional assistance data request |  |
| - Almanac | Not checked |
| - UTC model | Not checked |
| - Ionospheric model | Not checked |
| - Navigation model | Not checked |
| - DGPS corrections | Not checked |
| - Reference location | Not checked |
| - Reference time | Not checked |
| - Acquisition assistance | Not checked |
| - Real-time integrity | Not checked |
| - Navigation model additional data | Not checked |
| Measured Results on RACH | Not present |
| Additional Measured Results | Not present |
| Event Results | Not present |

MEASUREMENT CONTROL (Step 10a (Option 2)):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Modify |
| Measurement Reporting Mode |  |
| - Measurement report transfer mode | Acknowledged mode RLC |
| - Periodical reporting / Event trigger reporting mode | Periodical reporting |
| Additional Measurements List | Not present |
| CHOICE Measurement type | UE positioning measurement |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE assisted |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | FALSE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria | Periodical reporting criteria |
| - Amount of reporting | 1 |
| - Reporting interval | 64000 |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified in 17.2.1.3.5 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

REGISTER (Step 11)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 15)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (xx10 1010) |
| Facility | Return result = LCS-LocationNotification LocationNotificationRes verificationResponse -> permissionDenied |

REGISTER (Step 16)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 21)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (0010 1010) |

MEASUREMENT CONTROL (Step 22):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Setup |
| Measurement Reporting Mode |  |
| - Measurement report transfer mode | Acknowledged mode RLC |
| - Periodical reporting / Event trigger reporting mode | Periodical reporting |
| Additional Measurements List | Not present |
| CHOICE Measurement type | UE positioning measurement |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE assisted |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | TRUE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria | Periodical reporting criteria |
| - Amount of reporting | 1 |
| - Reporting interval | 64000 |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified for “Adequate assistance data for UE-assisted A-GPS” in 17.2.1.3.3 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

MEASUREMENT REPORT (Steps 23 (Option 1) or 23b (Option 2))

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measured Results |  |
| - CHOICE Measurement |  |
| - UE positioning measured results |  |
| - UE positioning OTDOA measured results | Not present |
| - UE positioning position estimate info | Not present |
| - UE positioning GPS measured results | Present |
| - UE positioning error | Not present |
| Measured Results on RACH | Not present |
| Additional Measured Results | Not present |
| Event Results | Not present |

MEASUREMENT REPORT (Step 23 (Option 2)):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measured Results |  |
| - CHOICE Measurement |  |
| - UE positioning measured results |  |
| - UE positioning OTDOA measured results | Not present |
| - UE positioning position estimate info | Not present |
| - UE positioning GPS measured results | Not present |
| - UE positioning error |  |
| - Error reason | Assistance Data Missing |
| - GPS additional assistance data request |  |
| - Almanac | Not checked |
| - UTC model | Not checked |
| - Ionospheric model | Not checked |
| - Navigation model | Not checked |
| - DGPS corrections | Not checked |
| - Reference location | Not checked |
| - Reference time | Not checked |
| - Acquisition assistance | Not checked |
| - Real-time integrity | Not checked |
| - Navigation model additional data | Not checked |
| Measured Results on RACH | Not present |
| Additional Measured Results | Not present |
| Event Results | Not present |

MEASUREMENT CONTROL (Step 23a (Option 2)):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Modify |
| Measurement Reporting Mode |  |
| - Measurement report transfer mode | Acknowledged mode RLC |
| - Periodical reporting / Event trigger reporting mode | Periodical reporting |
| Additional Measurements List | Not present |
| CHOICE Measurement type | UE positioning measurement |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE assisted |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | FALSE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria | Periodical reporting criteria |
| - Amount of reporting | 1 |
| - Reporting interval | 64000 |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified in 17.2.1.3.5 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

17.2.4.8.5	Test requirements

After step 7 the UE shall send a RELEASE COMPLETE message with verificationResponse set to permissionGranted.

After step 9 the UE shall respond with a MEASUREMENT REPORT message containing the IE “UE positioning GPS measured results”.

After step 14 the UE shall send a RELEASE COMPLETE message with verificationResponse set to permissionDenied.

After step 22 the UE shall respond with a MEASUREMENT REPORT message containing the IE “UE positioning GPS measured results”.

#### 17.2.4.9	LCS Mobile terminated location request/ UE-Assisted GPS/ Privacy Verification/ Location Not Allowed if No Response

17.2.4.9.1	Definition

This test case applies to all UEs supporting UE-Assisted GPS Location Service capabilities.

17.2.4.9.2	Conformance requirements

1)	The network invokes a location notification procedure by sending a REGISTER message containing a LCS-LocationNotification invoke component to the UE. This may be sent either to request verification for MT-LR or to notify about already authorized MT-LR.

2) 	In case of privacy verification the MS shall respond to the request by sending a RELEASE COMPLETE message containing the mobile subscriber's response in a return result component.

3) 	If the timer expires in the network before any response from the MS (e.g. due to no response from the user), the network shall interpret this by applying the default treatment defined in GSM 03.71 for GSM and TS 23.171 for UMTS (i.e. disallow location if barred by subscription and allow location if allowed by subscription).

4)	if the IE "Measurement command" has the value "setup":

2>	store this measurement in the variable MEASUREMENT_IDENTITY according to the IE "measurement identity", first releasing any previously stored measurement with that identity if that exists;

…

2>	for any other measurement type:

3>	if the measurement is valid in the current RRC state of the UE:

4>	begin measurements according to the stored control information for this measurement identity.

5)	The UE shall:

1> when a measurement report is triggered:

2> if the UE was able to perform measurements on at least one neighbour cell included in the variable  UE_POSITIONING_OTDOA_DATA_UE_ASSISTED in case of OTDOA or one satellite included in the variable UE_POSITIONING_GPS_DATA in case of GPS positioning or one cell from the active set in case of CELL ID:

3>	if the IE "Vertical Accuracy" is included:

4>	interpret the presence of this IE to indicate that the UTRAN desires to compute a 3-dimensional position estimate.

3>	if the IE "Positioning Methods" is set to "GPS":

4>	include the IE "UE positioning GPS measured results" in the measurement report and set the contents of the IE as follows:

5>	if the UE supports the capability to provide the GPS timing of the cell frames measurement:

6>	if the IE "GPS timing of Cell wanted" is set to TRUE:

7>	perform the UE GPS timing of cell frames measurement on the serving cell or on one cell of the active set.

7>	include the IE "Primary CPICH Info" for FDD or the IE "cell parameters id" for TDD; and

7>	include the IE "Reference SFN" and the IE "UE GPS timing of cell frames".

6>	if the IE "GPS timing of Cell wanted" is set to FALSE:

7>	include the IE "GPS TOW msec".

5>	if the UE does not support the capability to provide the GPS timing of the cell:

6>	include the IE "GPS TOW msec".

References

-	Conformance requirement 1, 2 and 3: TS 24.030, clause 4.1.1.

-	Conformance requirement 4: TS 25.331, clause 8.4.1.3.

-	Conformance requirement 5: TS 25.331, clause 8.6.7.19.3.3b.

17.2.4.9.3	Test Purpose

To verify that when the UE receives a REGISTER message, containing a LCS Location Notification Invoke component set to notifyAndVerify-LocationNotAllowedIfNoResponse, the UE notifies the user of the request and indicates that the default response is location not allowed if no response and providing the opportunity to accept or deny the request and sends a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionDenied or permissionGranted as appropriate.

17.2.4.9.4	Method of Test

Initial Conditions

System Simulator (SS):

-	1 cell, default parameters

-	Satellites: As specified in 17.2.1.2

UE:

-	State “CS-CELL DCH Initial (State 6-1)” as specified in clause 7.4.1 of TS 34.108.

Related PICS/PIXIT Statements

-	UE Assisted Network Assisted GPS

-	px_UeLcsNotification: value for UE LCS Notification timeout timer

Test Procedure

The SS initiates authentication and ciphering and sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse.

The LCS Client Name contained in the USSD text string of the lcs-LocationNotification should be displayed with the option to accept or deny the request and an indication that location will be allowed if no user response is received.

The user accepts the location request. The UE responds with a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionGranted.

The SS orders an A-GPS positioning measurement using a MEASUREMENT CONTROL message, including assistance data as specified in section 17.2.1.3.3..  The UE may request additional assistance data by sending a MEASUREMENT REPORT message containing a positioning error indication with the IE “Error reason” set to “Assistance Data Missing”.  If the UE requests additional assistance data, the SS provides the requested assistance data in one or more MEASUREMENT CONTROL messages.

The UE sends a MEASUREMENT REPORT message including IE “UE positioning GPS measured results”.

The SS sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse.

The user denies the location request. The UE responds with a RELEASE COMPLETE message containing a LocationNotification return result with verificationResponse set to permissionDenied.

The SS sends a REGISTER message containing a Facility IE containing a LCS Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse.

The user ignores the location request by taking no action. If the timer expires in the SS before any response from the UE is received, the SS interprets this by applying the default treatment LocationNotAllowed.

Expected Sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | AUTHENTICATION REQUEST |  |
| 2 | --> |  | AUTHENTICATION RESPONSE |  |
| 3 | SS |  |  | SS starts security procedure |
| 4 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse |
| 5 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 6 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be not allowed in the absence of a response |
| 7 | UE |  |  | The user accepts the location request before timer T(LCSN) expires |
| 8 | -> |  | RELEASE COMPLETE | Containing a LocationNotification return result with verificationResponse set to permissionGranted |
| 9 | <- |  | MEASUREMENT CONTROL |  |
| 10 | -> |  | MEASUREMENT REPORT | UE reports positioning measurement results (Option 1) or requests additional assistance data (Option 2). |
| 10a | <- |  | MEASUREMENT CONTROL | If UE requested additional assistance data in step 10, SS provides the requested data in one or more MEASUREMENT CONTROL messages as specified in section 17.2.1.3.5. |
| 10b | -> |  | MEASUREMENT REPORT | If UE requested additional assistance data in step 10, this message contains the IE “UE positioning GPS measured results”. |
| 11 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse |
| 12 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 13 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be not allowed in the absence of a response |
| 14 | UE |  |  | The user denies the location request before timer T(LCSN) expires |
| 15 | -> |  | RELEASE COMPLETE | Containing a LocationNotification return result with verificationResponse set to permissionDenied |
| 16 | <- |  | REGISTER | Call Independent SS containing Facility IE Location Notification Invoke message set to notifyAndVerify-LocationNotAllowedIfNoResponse |
| 17 | SS |  |  | SS starts timer T(LCSN) set to 90% of px_UeLcsNotification |
| 18 | UE |  |  | The UE notifies the user of the location request and indicates to the user that location will be not allowed in the absence of a response |
| 19 | UE |  |  | The user does not reply |
| 20 | SS |  |  | SS waits until T(LCSN) expires to verify that the UE does not send a RELEASE COMPLETE message. |
| 21 | <- |  | RELEASE COMPLETE | SS terminates the dialogue |
| 22 | SS |  |  | SS releases the connection and the test case ends |

Specific Message Contents

REGISTER (Step 4)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationNotAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 8)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (xx10 1010) |
| Facility | Return result = LCS-LocationNotification LocationNotificationRes verificationResponse -> permissionGranted |

MEASUREMENT CONTROL (Step 9):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Setup |
| Measurement Reporting Mode |  |
| - Measurement report transfer mode | Acknowledged mode RLC |
| - Periodical reporting / Event trigger reporting mode | Periodical reporting |
| Additional Measurements List | Not present |
| CHOICE Measurement type | UE positioning measurement |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE assisted |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | TRUE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria | Periodical reporting criteria |
| - Amount of reporting | 1 |
| - Reporting interval | 64000 |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified for “Adequate assistance data for UE-assisted A-GPS” in 17.2.1.3.3 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

MEASUREMENT REPORT (Steps 10 (Option 1) or 10b (Option 2))

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measured Results |  |
| - CHOICE Measurement |  |
| - UE positioning measured results |  |
| - UE positioning OTDOA measured results | Not present |
| - UE positioning position estimate info | Not present |
| - UE positioning GPS measured results | Present |
| - UE positioning error | Not present |
| Measured Results on RACH | Not present |
| Additional Measured Results | Not present |
| Event Results | Not present |

MEASUREMENT REPORT (Step 10 (Option 2)):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measured Results |  |
| - CHOICE Measurement |  |
| - UE positioning measured results |  |
| - UE positioning OTDOA measured results | Not present |
| - UE positioning position estimate info | Not present |
| - UE positioning GPS measured results | Not present |
| - UE positioning error |  |
| - Error reason | Assistance Data Missing |
| - GPS additional assistance data request |  |
| - Almanac | Not checked |
| - UTC model | Not checked |
| - Ionospheric model | Not checked |
| - Navigation model | Not checked |
| - DGPS corrections | Not checked |
| - Reference location | Not checked |
| - Reference time | Not checked |
| - Acquisition assistance | Not checked |
| - Real-time integrity | Not checked |
| - Navigation model additional data | Not checked |
| Measured Results on RACH | Not present |
| Additional Measured Results | Not present |
| Event Results | Not present |

MEASUREMENT CONTROL (Step 10a (Option 2)):

| Information element | Value/remark |
| --- | --- |
| Measurement Information Elements |  |
| Measurement Identity | 10 |
| Measurement Command | Modify |
| Measurement Reporting Mode |  |
| - Measurement report transfer mode | Acknowledged mode RLC |
| - Periodical reporting / Event trigger reporting mode | Periodical reporting |
| Additional Measurements List | Not present |
| CHOICE Measurement type | UE positioning measurement |
| - UE positioning measurement |  |
| - UE positioning reporting quantity |  |
| - Method type | UE assisted |
| - Positioning methods | GPS |
| - Response time | 128 |
| - Horizontal accuracy | 127 |
| - Vertical accuracy | 127 |
| - GPS timing of cell wanted | FALSE |
| - Multiple sets | FALSE |
| - Additional assistance data request | FALSE |
| - Environmental characterization | Not present |
| - Measurement validity |  |
| - UE state | All states |
| - CHOICE Reporting criteria | Periodical reporting criteria |
| - Amount of reporting | 1 |
| - Reporting interval | 64000 |
| - UE pos OTDOA assistance data for UE-assisted | Not present |
| - UE pos OTDOA assistance data for UE-based | Not present |
| - UE positioning GPS assistance data | Set as specified in 17.2.1.3.5 |
| Physical Channel Information Elements |  |
| DPCH compressed mode status info | Not present |

REGISTER (Step 11)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | REGISTER (0011 1011) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationNotAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 15)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (xx10 1010) |
| Facility | Return result = LCS-LocationNotification LocationNotificationRes verificationResponse -> permissionDenied |

REGISTER (Step 16)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (0010 1010) |
| Facility | Invoke = LCS-LocationNotification LocationNotificationArg notificationType -> notifyAndVerify-LocationNotAllowedIfNoResponse locationType -> current Location lcsClientExternalID -> externalAddress lcsClientName ->dataCodingScheme nameString |

RELEASE COMPLETE (Step 21)

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | Call Independent SS message (1011) |
| Transaction identifier |  |
| Message type | RELEASE COMPLETE (0x10 1010) |

17.2.4.9.5	Test requirements

After step 7 the UE shall send a RELEASE COMPLETE message with verificationResponse set to permissionGranted.

After step 9 the UE shall respond with a MEASUREMENT REPORT message containing the IE “UE positioning GPS measured results”.

After step 14 the UE shall send a RELEASE COMPLETE message with verificationResponse set to permissionDenied.

During step 20 the UE shall not send any RELEASE COMPLETE message.