3GPP TSG RAN WG R5 Meeting #41-bis	R5-086213r1

- Munich, Germany, 15 – 18 December 2008

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | CRNum |  rev | - |  Current version: | 8.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Update of generic setup procedure for state 4 (Loopback Activated) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | R5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | LTE-UEConTest_SIG |  |  |  |  |  |  | Date:  |  | 11/12/2008 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8 Rel-9 (Release 9) |  |
| Reason for change:  |  | Current generic test procedure for State 4 “Loopback Activated” is not consistent with 36.509 regarding the order to activate the radio bearer test mode and establishing radio bearers (the radio bearer is established before the activating the radio bearer test mode). |  |  |  |  |  |  |  |  |
| Summary of change:  |  | - Changed initial condition to refer to Idle Mode state (State 2) instead of Generic RB Establishment state (State 3). - Added steps for establish radio bearer (default or other as specified in the actual test case) after activation of radio bearer test mode. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Test procedure incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | 1st revision R5-086213 to R5-08xxxx |  |  |  |  |  |  |  |  |

## 4.5	Generic procedures

This clause describes UE test states which can be used in the initial condition of many test cases defined in TS 36.521-1 [21], TS 36.523-1 [18] and TS 36.523-3 [34] or other procedures defined in this specification. This section also defines a set of procedures to bring the UE into these states.

### 4.5.1	UE test states

NOTE:	The need to have a procedure for the transition from State 4/State 3 to State 2 is for further study and it can added if the technical motivation for this procedure can be justified.

Figure 4.5.1-1: UE Test States for Basic Generic Procedures

In order that the UE can set up a call or session in E-UTRAN, there are a number of procedures to be undertaken in a hierachical sequence to move between known states. The sequences are shown in figure 4.5.1-1 and the status of the relevant protocols in the UE in the different states are given in table 4.5.1-1.

Table 4.5.1-1: The UE states

|  |  | RRC | ECM | EMM | ESM |
| --- | --- | --- | --- | --- | --- |
| State 1 | Switched OFF | ----- | ----- | ----- | ----- |
| State 2 | Registered, Idle Mode | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 1 default EPS bearer context active |
| State 3 | Generic RB Established | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active |
| State 4 | Loopback Activated | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active |

NOTE: Refer to TS 24.301 [28] subclause 5.5.1.1 for more details on the ESM state.

### 4.5.2	UE Registration (State 2)

Editor’s Note:

- It needs to be defined what the default bearer is in terms of its characteristics.(step 14).

- According to latest 36.331 specification, it is not possible to encapsulate the ATTACH COMPLETE message in the RRC CONNECTION RECONFIGURATION COMPLETE message, hence two separate messages need to be sent by the UE, one for RRC CONNECTION RECONFIGURATION COMPLETE message and one for ATTACH COMPLETE message.

- The default parameters and system information will depend on the progress in Clause 4.4, once it is finalised RAN5 can refer to them accordingly.

#### 4.5.2.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The Test USIM shall be inserted.

#### 4.5.2.2	Definition of system information messages

The default system information messages are used.

#### 4.5.2.3	Procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 |  | <-- | RRC: SYSTEM INFORMATION (BCCH) |
| 2 | UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 3 | SS transmit a RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits a RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message. The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST | --> | RRC: RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST |
| 5 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | RRC: DLInformationTransfer NAS: AUTHENTICATION REQUEST |
| 6 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | RRC: ULInformationTransfer NAS: AUTHENTICATION RESPONSE |
| 7 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | RRC: DLInformationTransfer NAS: SECURITY MODE COMMAND |
| 8 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer NAS: SECURITY MODE COMPLETE |
| 9 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 10 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 11 | The SS transmits an UECapabilityEnquiry message to initiate the UE radio access capability transfer procedure. | <-- | RRC: UECapabilityEnquiry |
| 12 | The UE transmits an UECapabilityInformation message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation |
| - | EXCEPTION: Steps 13a1 to 13a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred after SECURITY MODE COMPLETE message. | - | - |
| 13a1 | IF the UE sets the ESM information transfer flag in the last PDN CONNECTIVITY REQUEST message THEN the SS transmits an ESM INFORMATION REQUEST message to initiate exchange of protocol configuration options. | <-- | RRC: DLInformationTransfer NAS: ESM INFORMATION REQUEST |
| 13a2 | The UE transmits an ESM INFORMATION RESPONSE message to transfer protocol configuration options. | --> | RRC: ULInformationTransfer NAS: ESM INFORMATION RESPONSE |
| 14 | The SS transmits a RRCConnectionReconfiguration message to establish the default bearer. This message includes the ATTACH ACCEPT message. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. | <-- | RRC: RRCConnectionReconfiguration NAS: ATTACH ACCEPT NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| 15 | The UE transmits a RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 16 | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 17 | The SS transmits a RRCConnectionRelease message to release RRC connection and move to RRC_IDLE (State 2). | <-- | RRC: RRCConnectionRelease |

#### 4.5.2.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5.3	Generic Radio Bearer Establishment (State 3)

Editor note: The default parameter and system information will depend on progress in Clause 4.4, once it is finalised RAN5 can refer to them accordingly.

#### 4.5.3.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5.3.2	Definition of system information messages

The default system information messages are used.

#### 4.5.3.3	Procedure

The establishment of generic radio bearer connection is assumed to always be mobile terminated.

Editor’s note: the need for a mobile originated procedure is FFS (use case needed)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 |  | <-- | RRC: SYSTEM INFORMATION (BCCH) |
| 2 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) |
| 3 | UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 4 | SS transmit a RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 5 | The UE transmits a RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. (State3) | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 6 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 7 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 8 | The SS configures a new data radio bearer, associated with the default EPS bearer context. | <-- | RRC: RRCConnectionReconfiguration |
| 9 | The UE transmits a RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |

#### 4.5.3.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5.4	Loopback Activation (State 4)

Editor’s Note: This section will be completed when message for loopback activation is defined in TS 36.509. The table below is just an example and should be alighed with TS 36.509.

#### 4.5.4.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

4.5.4.2	Definition of system information messages

The default system information messages are used.

#### 4.5.4.3	Procedure

The establishment of generic radio bearer connection is assumed to always be mobile terminated.

Editor’s note: the need for a mobile originated procedure is FFS (use case needed)

| Step | Procedure | Direction |  | Message Sequence |
| --- | --- | --- | --- | --- |
|  |  | UE - SS |  | Message |
| 1 |  | <-- | RRC: SYSTEM INFORMATION (BCCH) |  |
| 2 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) |  |
| 3 | UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |  |
| 4 | SS transmit a RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |  |
| 5 | The UE transmits a RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. (State3) | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |  |
| 6 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |  |
| 7 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |  |
| 8 | The SS transmits an ACTIVATE RB TEST MODE message to activate UE radio bearer test mode procedure. | <-- |  | RRC: DLInformationTransfer TC: ACTIVATE RB TEST MODE |
| 9 | The UE transmits an ACTIVATE RB TEST MODE COMPLETE message. | --> |  | RRC: ULInformationTransfer TC: ACTIVATE RB TEST MODE COMPLETE |
| 10 | The SS configures a new data radio bearer, associated with the default EPS bearer context (Note 1) | <-- | RRC: RRCConnectionReconfiguration |  |
| 11 | The UE transmits a RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |  |
| 12 | The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. | <-- |  | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP |
| 13 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated (State 4). | --> |  | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE |
| Note 1: The establishment of the default EPS bearer context in step 10 may be replaced with establishment of a combination of EPS bearers as specified in the specific test case using the procedure. |  |  |  |  |

#### 4.5.4.4	Specific message contents

All specific message contents shall be referred to clause 4.6.