3GPP TSG-RAN WG5 Meeting #46 	R5-101085

# San Francisco, USA, 22 - 26 Feb 2010

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | CRNum | rev | - | Current version: | 8.4.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | GCF Priority 1 - Update of Acknowledged Mode Procedures part TC 7.2.3.15 AM RLC / Re-ordering of RLC PDU segments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-UEConTest_SIG |  |  |  |  |  |  | Date: |  | 22/02/2010 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change: |  | Conformance Requirements have been updated in TS 36.322 version 8.7.0 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes are the following: - Update conformance requirements according to the new modification of TS36.331 - Correct the wrong word at step 2 in test procedure sequence |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test remains unaligned with latest version of the core spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 7.2.3.15	AM RLC / Re-ordering of RLC PDU segments

7.2.3.15.1	Test Purpose (TP)

(1)

with { UE in E-UTRAN RRC_CONNECTED state }

ensure that {

when { UE receives RLC AM PDU segments }

then { UE reorders RLC AMD PDU segments received out of sequence }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { t-Reordering expires }

then { Set VR(MS) to SN of the first AMD PDU with SN >= VR(X) for which not all byte segments have been received }

}

7.2.3.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.322 clauses 4.2.1.3.3, 5.1.3.2.1, 5.1.3.2.2, 5.1.2.3.3 and 5.1.2.3.4.

[TS 36.322, clause 4.2.1.3.3]

When the receiving side of an AM RLC entity receives RLC data PDUs, it shall:

...

-	detect the loss of RLC data PDUs at lower layers and request retransmissions to its peer AM RLC entity;

-	reassemble RLC SDUs from the reordered RLC data PDUs and deliver the RLC SDUs to upper layer in sequence.

...

[TS 36.322, clause 5.1.3.2.1]

The receiving side of an AM RLC entity shall maintain a receiving window according to state variables VR(R) and VR(MR) as follows:

-	a SN falls within the receiving window if VR(R) <= SN < VR(MR);

-	a SN falls outside of the receiving window otherwise.

When receiving a RLC data PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received RLC data PDU or place it in the reception buffer (see sub clause 5.1.3.2.2);

-	if the received RLC data PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reordering as needed (see sub clause 5.1.3.2.3).

When t-Reordering expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reordering as needed (see sub clause 5.1.3.2.4).

[TS 36.322, clause 5.1.3.2.2]

When a RLC data PDU is received from lower layer, where the RLC data PDU contains byte segment numbers y to z of an AMD PDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the AMD PDU with SN = x have been received before:

-	discard the received RLC data PDU;

-	else:

-	place the received RLC data PDU in the reception buffer;

-	if some byte segments of the AMD PDU contained in the RLC data PDU have been received before:

-	discard the duplicate byte segments.

[TS 36.322, clause 5.1.3.2.3]

When a RLC data PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if all byte segments of the AMD PDU with SN = VR(MS) are received:

-	update VR(MS) to the SN of the first AMD PDU with SN > current VR(MS) for which not all byte segments have been received;

-	if x = VR(R):

-	if all byte segments of the AMD PDU with SN = VR(R) are received:

-	update VR(R) to the SN of the first AMD PDU with SN > current VR(R) for which not all byte segments have been received;

-	update VR(MR) to the updated VR(R) + AM_Window_Size;

-	reassemble RLC SDUs from any byte segments of AMD PDUs with SN that falls outside of the receiving window and in-sequence byte segments of the AMD PDU with SN = VR(R), remove RLC headers when doing so and deliver the reassembled RLC SDUs to upper layer in sequence if not delivered before;

-	if x >= VR(H)

-	update VR(H) to x+ 1;

-	if t-Reordering is running:

-	if VR(X) = VR(R); or

-	if VR(X) falls outside of the receiving window and VR(X) is not equal to VR(MR):

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case t-Reordering is stopped due to actions above):

-	if VR (H) > VR(R):

-	start t-Reordering;

-	set VR(X) to VR(H).

[TS 36.322, clause 5.1.3.2.4]

When t-Reordering expires, the receiving side of an AM RLC entity shall:

-	update VR(MS) to the SN of the first AMD PDU with SN >= VR(X) for which not all byte segments have been received;

-	if VR(H) > VR(MS):

-	start t-Reordering;

-	set VR(X) to VR(H).

7.2.3.15.3	Test description

7.2.3.15.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18] with a loop back size of 98 bytes.

7.2.3.15.3.2	Test procedure sequence

Table 7.2.3.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The SS transmits one AMD PDU containing SDU#8 (100 bytes) in its data field to the UE. SN=7 indicates the loss of 7 PDUs. | <-- | AMD PDU#8 (SN=7) | - | - |
| 2 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#1 in its data field to the UE. This AMD PDU segment carries part 1 of AMD PDU#1, which contained SDU#1 (100 bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#1 (SN=0) segment 1 | - | - |
| 3 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#2 in its data field to the UE. This AMD PDU segment carries part 2 of AMD PDU#2, which contained SDU#2 (100 bytes) in its data field. SO=50 and LSF=1. | <-- | AMD PDU#2 (SN=1) segment 2 | - | - |
| 4 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#3 in its data field to the UE. This AMD PDU segment carries part 1 of AMD PDU#3, which contained SDU#3 (100 bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#3 (SN=2) segment 1 | - | - |
| 5 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#4 in its data field to the UE. This AMD PDU segment carries part 2 of AMD PDU#4, which contained SDU#4 (100 bytes) in its data field. SO=50 and LSF=1. | <-- | AMD PDU#4 (SN=3) segment 2 | - | - |
| 6 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#4 in its data field to the UE. This AMD PDU segment carries part 1of AMD PDU#4, which contained SDU#4 (100 bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#4 (SN=3) segment 1 | - | - |
| 7 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#1 in its data field to the UE. This AMD PDU segment carries part 2 of AMD PDU#1, which contained SDU#1 (100 bytes) in its data field. SO=50 and LSF=1. | <-- | AMD PDU#1 (SN=0) segment 2 | - | - |
| 8 | Check: Does the UE transmit an RLC SDU containing SDU#1 in its data field? | --> | (RLC SDU#1) | 1 | P |
| 9 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#2 in its data field to the UE. This AMD PDU segment carries part 1 of AMD PDU#2, which contained SDU#2 (100 bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#2 (SN=1) segment 1 | - | - |
| 10 | Check: Does the UE transmit an RLC SDU containing SDU#2 in its data field? | --> | (RLC SDU#2) | 1 | P |
| 11 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#3 in its data field to the UE. This AMD PDU segment carries part 2 of PDU#3, which contained SDU#3 (100 bytes) in its data field. SO=50 and LSF=1. | <-- | AMD PDU#3 (SN=2) segment 2 | - | - |
| 12 | Check: Does the UE transmit an RLC SDU containing SDU#3 in its data field? | --> | (RLC SDU#3) | 1 | P |
| 13 | Check: Does the UE transmit an RLC SDU containing SDU#4 in its data field? | --> | (RLC SDU#4) | 1 | P |
| 14 | The SS transmits an RLC STATUS PDU to the UE. This PDU acks PDUs up to those including SDU#4. ACK_SN=4. | <-- | STATUS PDU | - | - |
| 15 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#7 in its data field to the UE. This AMD PDU segment carries part 1 of PDU #7, which contained SDU#7 (100 bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#7 (SN=6) segment 1 | - | - |
| 16 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#6 in its data field to the UE. This AMD PDU segment carries segment 2 of AMD PDU#6, which contained SDU#6 (100 bytes) in its data field. SO=50 and LSF=1. | <-- | AMD PDU#6 (SN=5) segment 2 | - | - |
| 17 | Wait for t-Reordering to run out at the UE side. | - | - | - | - |
| 18 | Check: Does the UE transmit a Status Report with NACK_SN=4, NACK_SN=5 with SOStart=0 and SOEnd=49, and NACK_SN=6 with SOStart=50 and SOEnd=32767 (special SOEnd value), and ACK_SN=8? | --> | STATUS PDU | 2 | P |
| 19 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#7 in its data field to the UE. This AMD PDU segment carries part 2 of AMD PDU#7, which contained SDU#7 (100 bytes) in its data field. SO=50 and LSF=1. | <-- | AMD PDU#7 (SN=6) segment 2 | - | - |
| 20 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#6 in its data field to the UE. This AMD PDU segment carries part 1 of AMD PDU#6, which contained SDU#6 (100 bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#6 (SN=5) segment 1 | - | - |
| 21 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#5 in its data field to the UE. This AMD PDU segment carries part 1 of AMD PDU#5, which contained SDU#5 (100 bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#5 (SN=4) segment 1 | - | - |
| 22 | Wait for t-Reordering to run out at the UE side. | - | - | - | - |
| 23 | Check: Does the UE transmit a Status Report with NACK_SN=4 with SOStart=50 and SOEnd=32767 (special SOEnd value), and ACK_SN=8? | --> | STATUS PDU | 2 | P |
| 24 | The SS transmits one AMD PDU segment containing 50 bytes of SDU#5 in its data field to the UE. This AMD PDU segment carries part 2 of AMD PDU#5, which contained SDU#5 (100 bytes) in its data field. SO=50 and LSF=1. | <-- | AMD PDU#5 (SN=4) segment 2 | - | - |
| 25 | Check: Does the UE transmit an RLC SDU containing SDU#5 in its data field? | --> | (RLC SDU#5) | 1 | P |
| 26 | Check: Does the UE transmit an RLC SDU containing SDU#6 in its data field? | --> | (RLC SDU#6) | 1 | P |
| 27 | Check: Does the UE transmit an RLC SDU containing SDU#7 in its data field? | --> | (RLC SDU#7) | 1 | P |
| 28 | Check: Does the UE transmit an RLC SDU containing SDU#8 in its data field? | --> | (RLC SDU#8) | 1 | P |
| 29 | The SS transmits an RLC STATUS PDU to the UE. This PDU acks PDUs up to those including SDU#7. ACK_SN=8. | <-- | STATUS PDU | - | - |

7.2.3.15.3.3	Specific message contents

None.