3GPP TSG-RAN WG5 Meeting #50 	R5-110657

# Taipei, Taiwan, 21st -25th Feb 2011

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.121-1 | CR | CRNum | rev | - | Current version: | 9.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Annexure A to Annexure F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RANimp-DC_MIMO_UEConTest |  |  |  |  |  |  |  |  | 6/1/2011 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | The core specifications for Rel-9 have been enhanced by introduction of the DC-HSDPA with MIMO performance requirements . . This CR is a corresponding CR updating Annexure of TS 34.121-1 for test cases 9.2.4C and 9.2.4D. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated annexure A to Annexure F with relevant info for DC- HSDPA with MIMO for TC 9.2.4C and 9.2.4D |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Annexure wouldn’t be up to date for TC 9.2.4C and 9.2.4D |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annexure A to Annexure F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | 34.121-2 |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR applies from Rel-9 and later releases. |  |  |  |  |  |  |  |  |

# [Many Sections are skipped here]

# C.8	DL reference channel parameters for HSDPA tests

# C.8.1	Fixed Reference Channel (FRC)

[Skipping many sections]

### C.8.1.9	Fixed Reference Channel Definition H-Set 9/9A

Table C.8.1.9: Fixed Reference Channel H-Set 9/9A

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Transport block |  | Primary | Secondary |
| Combined Nominal Avg. Inf. Bit Rate |  | 13652 |  |
| Nominal Avg. Inf. Bit Rate | kbps | 8784 | 4868 |
| Inter-TTI Distance | TTI’s | 1 | 1 |
| Number of HARQ Processes | Processes | 6 | 6 |
| Information Bit Payload () | Bits | 17568 | 9736 |
| Number Code Blocks | Blocks | 4 | 2 |
| Binary Channel Bits Per TTI | Bits | 28800 | 14400 |
| Total available SML's in UE | Bits | 345600 |  |
| Number of SML’s per HARQ Proc. | SML’s | 28800 | 28800 |
| Coding Rate |  | 0.61 | 0.68 |
| Number of Physical Channel Codes | Codes | 15 | 15 |
| Modulation |  | 16QAM | QPSK |
| Note : The values in the table defines H-Set 9. H-Set 9A for DC-HSDPA is formed by applying H-Set 9 to each of the carriers available in DC-HSDPA mode. |  |  |  |

Figure C.8.13: Coding rate for Fixed Reference Channel H-Set 9/9A Primary Transport Block

Figure C.8.14: Coding rate for Fixed Reference Channel H-Set 9/9A Secondary Transport Block

### C.8.1.10	Fixed Reference Channel Definition H-Set 10/10A

Table C.8.1.10: Fixed Reference Channel H-Set 10

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Nominal Avg. Inf. Bit Rate | Kbps | 8774 | 4860 |
| Inter-TTI Distance | TTI’s | 1 | 1 |
| Number of HARQ Processes | Processes | 6 | 6 |
| Information Bit Payload | Bits | 17548 | 9719 |
| Number Code Blocks | Blocks | 4 | 2 |
| Binary Channel Bits Per TTI | Bits | 28800 | 14400 |
| Number of SML’s per HARQ Proc. | SML’s | 28800 | 28800 |
| Coding Rate |  | 0.6 | 0.67 |
| Number of Physical Channel Codes | Codes | 15 | 15 |
| Modulation |  | 16QAM | QPSK |
| Nominal Avg. Inf. Bit Rate | Kbps | 8774 | 4860 |
| Note: The values in the table define H-Set 10. H-Set 10A for DC-HSDPA is formed by applying H-Set 10 to each of the carriers available in DC-HSDPA mode. |  |  |  |

Figure C.8.15: Coding rate for Fixed Reference Channel H-Set 10 (16QAM)

Figure C.8.16: Coding rate for Fixed Reference Channel H-Set 10 (QPSK)

### C.8.1.11	Fixed Reference Channel Definition H-Set 11/11A

Table C.8.1.11: Fixed Reference Channel H-Set 11/11A

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Transport block |  | Primary | Secondary |
| Combined Nominal Avg. Inf. Bit Rate |  | 22074 |  |
| Nominal Avg. Inf. Bit Rate | kbps | 13300 | 8774 |
| Inter-TTI Distance | TTI’s | 1 | 1 |
| Number of HARQ Processes | Processes | 6 | 6 |
| Information Bit Payload () | Bits | 26504 | 17568 |
| Number Code Blocks | Blocks | 6 | 4 |
| Binary Channel Bits Per TTI | Bits | 43200 | 28800 |
| Total available SML's in UE | Bits | 518400 |  |
| Number of SML’s per HARQ Proc. | SML’s | 43200 | 43200 |
| Coding Rate |  | 0.61 | 0.6 |
| Number of Physical Channel Codes | Codes | 15 | 15 |
| Modulation |  | 64QAM | 16QAM |
| Note : The values in the table defines H-Set 11. H-Set 11A for DC-HSDPA is formed by applying H-Set 11 to each of the carriers available in DC-HSDPA mode. |  |  |  |

Figure C.8.17: Coding rate for Fixed Reference Channel H-Set 11/11A Primary Transport Block

Figure C.8.18: Coding rate for Fixed Reference Channel H-Set 11/11A Secondary Transport Block

[Skipping many Sections]

## F.1.6	Performance requirement (HSDPA)

Table F.1.6: Maximum Test System Uncertainty for Performance Requirements (HSDPA)

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 9.2.1A to 9.2.1KA Single Link Performance | ±0.6 dB ±1.0 dB ±0.1 dB | 0.1 dB uncertainty in Ec/Ior ratio Worst case gain uncertainty due to the fader from the calibrated static profile is ±0.5 dB per output In addition the same ±0.3 dB ratio error as 7.2. These are uncorrelated so can be RSS. Overall error in is (0.52 + 0.32) 0.5 = 0.6 dB |
| 9.2.1L Single Link Enhanced Performance Type 3i | Wanted signal ±0.6 dB First interferer ±0.6 dB Second interferer ±0.6 dB ±1.0 dB ±0.1 dB | 0.1 dB uncertainty in Ec/Ior ratio For wanted signal and each interferer, worst case gain uncertainty due to the fader from the calibrated static profile is ±0.5 dB per output In addition the same ±0.3 dB uncertainty as 7.2. These are uncorrelated so can be combined RSS. Overall uncertainty in ,for each signal: (0.52 + 0.32) 0.5 = 0.6 dB. |
| 9.2.1LA Enhanced Performance Type 3i for DC-HSDPA | Same as 9.2.1L Uncertainties apply for each carrier | Same as 9.2.1L Uncertainties apply for each carrier |
| 9.2.2A to 9.2.2E Open loop diversity performance | ±0.8 dB ±1.0 dB ±0.1 dB | Worst case gain uncertainty due to the fader from the calibrated static profile is ±0.5 dB per output In addition the same ±0.3 dB ratio error as 7.2. These are uncorrelated so can be RSS. Overall error in is (0.52 + 0.52 + 0.32) 0.5 = 0.768 dB. Round up to 0.8 dB |
| 9.2.3A to 9.2.3E Closed loop diversity performance | Same as 9.2.2A | Same as 9.2.2A |
| 9.2.4A to 9.2.4D MIMO performance | Same as 9.2.2A | Same as 9.2.2A |
| 9.3.1 Single Link Performance - AWGN propagation conditions | ±0.3 dB ±1.0 dB ±0.1 dB | 0.1 dB uncertainty in DPCH_Ec ratio 0.3 dB uncertainty in based on power meter measurement after the combiner Overall error is the sum of the ratio error and the DPCH_Ec/Ior ratio but is not RSS for simplicity. The absolute error of the AWGN Ioc is not important for any tests in clause 7 but is specified as 1.0 dB. |
| 9.3.1A Single Link Performance - AWGN propagation conditions, 64QAM | Same as 9.3.1 | Same as 9.3.1 |
| 9.3.1B Single Link Performance - AWGN Propagation Conditions, DC-HSDPA requirements | ±0.3 dB ±0.3 dB ±1.0 dB ±0.1 dB | 0.1 dB uncertainty in DPCH_Ec ratio 0.3 dB uncertainty in and based on power meter measurement after the combiner Overall error is the sum of the ratio error and the DPCH_Ec/Ior ratio but is not RSS for simplicity. The absolute error of the AWGN Ioc is not important for any tests in clause 7 but is specified as 1.0 dB. |
| 9.3.1C Single Link Performance - AWGN Propagation Conditions, Periodically Varying Radio Conditions | , ±0.3 dB , ±1.0 dB ±0.1 dB Linearity within the applicable range of 10dB 1.0 dB | 0.1 dB uncertainty in DPCH_Ec ratio 0.3 dB uncertainty in based on power meter measurement after the combiner Overall error is the sum of the ratio error and the DPCH_Ec/Ior ratio but is not RSS for simplicity. The absolute error of the AWGN Ioc is specified as 1.0 dB. The linearity of of the AWGN Ioc is specified as 1.0 dB (±0.5dB) |

[Skipping many sections]

## F.2.5	Performance requirements (HSDPA)

Table F.2.5: Test Tolerances for Performance Requirements (HSDPA)

| Clause | Test Tolerance |
| --- | --- |
| 9.2.1A to 9.2.1KA Single Link Performance | 0.6 dB for 0.1 dB for Ec/Ior |
| 9.2.1L Single Link Enhanced Performance Type 3i | 0.76 dB for 0.17 dB for DIP1, DIP2 0.1 dB for Ec/Ior |
| 9.2.1LA Enhanced Performance Type 3i for DC-HSDPA | Same as 9.2.1L Test Tolerances apply for each carrier |
| 9.2.2A to 9.2.2E Open loop diversity performance | 0.8 dB for 0.1 dB for Ec/Ior |
| 9.2.3A to 9.2.3E Closed loop diversity performance | Same as 9.2.2A |
| 9.2.4A to 9.2.4D MIMO performance | Same as 9.2.2A |
| 9.3.1 Single Link Performance - AWGN propagation conditions | No test tolerances applied |
| 9.3.1A Single Link Performance - AWGN propagation conditions, 64QAM | No test tolerances applied |
| 9.3.1B Single Link Performance - AWGN Propagation Conditions, DC-HSDPA requirements | No test tolerances applied |
| 9.3.1C Single Link Performance - AWGN Propagation Conditions, Periodically Varying Radio Conditions | No test tolerances applied for test step 7 and 8 TT for M-difference is 1 for test step 9 (difference in medians M1 and M2) |
| 9.3.2 Single Link Performance - Fading propagation conditions | No test tolerances applied |
| 9.3.2A Single Link Performance - Fading Propagation Conditions, DC-HSDPA requirements | No test tolerances applied |
| 9.3.2B Single Link Performance - Fading propagation conditions, 64QAM | No test tolerances applied |
| 9.3.3 Open Loop Diversity Performance - AWGN propagation conditions | No test tolerances applied |
| 9.3.4 Open Loop Diversity Performance - Fading propagation conditions | No test tolerances applied |
| 9.3.5 Closed Loop Diversity Performance - AWGN propagation conditions | No test tolerances applied |
| 9.3.6 Closed Loop Diversity Performance - Fading propagation conditions | No test tolerances applied |
| 9.3.7A, MIMO performance – Reporting of Channel Quality indicator - Single stream fading conditions | No test tolerances applied |
| 9.3.7B MIMO performance – Reporting of Channel Quality indicator - Dual stream fading conditions | No test tolerances applied |
| 9.3.7C MIMO performance – Reporting of Channel Quality indicator - Dual stream fading conditions– UE categories 19-20 | No test tolerances applied |
| 9.3.7D MIMO performance – Reporting of Channel Quality indicator - Dual stream static orthogonal conditions – UE categories 15-20 | No test tolerances applied |
| 9.3.7E MIMO performance –Reporting of Channel Quality indicator - Dual stream static orhogonal conditions – UE categories 19-20 | No test tolerances applied |
| 9.4.1 Single Link Performance | 0.6 dB for 0.1 dB for P-CPICH_Ec/Ior and HS-SCCH_Ec/Ior |
| 9.4.1A Single Link Performance - Enhanced Performance Requirements Type 1 | Same as 9.4.1 |
| 9.4.2 Open loop diversity performance | 0.8 dB for 0.1 dB for P-CPICH_Ec/Ior and HS-SCCH_Ec/Ior |
| 9.4.2A Open loop diversity performance – Enhanced Performance Requirements Type 1 | Same as 9.4.2 |
| 9.4.3 HS-SCCH Type 3 performance | Same as 9.4.2 |
| 9.5.1 HS-SCCH-less demodulation of HS-DSCH | Same as 9.2.1A |
| 9.5.1A HS-SCCH-less demodulation of HS-DSCH, Enhanced Performance Requirements Type 1 | Same as 9.2.1A |
| 9.6.1 Single link HS-DSCH Demodulation performance in CELL_FACH state | Same as 9.2.1A |
| 9.6.2 Single link HS-SCCH Detection performance in CELL_FACH state | Same as 9.2.1A |

[Skipping many sections here]

## F.4.5	Performance requirements (HSDPA)

Table F.4.5: Derivation of Test Requirements (Performance tests HSDPA)

| Test | Minimum Requirement in TS 25.101 | Test Tolerance (TT) | Test Requirement in TS 34.121 |
| --- | --- | --- | --- |
| 9.2.1A to 9.2.1KA Single Link Performance | -12, -9, -6, -3 and -2 dB = -60 dBm = 0, 5, 10, 15 and 18 dB | 0.1 dB for 0.6 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.2.1L Single Link Enhanced Performance Type 3i | -6 and -3 dB = -60 dBm = 0 dB DIP1 = -2.75 dB DIP2 = -7.64 dB | 0.1 dB for 0.76 dB for 0.17 dB for DIP1, DIP2 | Formulas: = ratio + TT = ratio + 1.2dB = ratio + 0.6dB = ratio + 0.6dB This has the effect of increasing by 0.76dB and increasing DIP1 and DIP2 by 0.17dB. and TT is derived by increasing ratio by uncertainty to ensure DIP values are maintaned. TT is derived by combined effect of , and uncertainties in wanted signal, intererfer 1 and intererfer 2 respectively, after applying TT to and . The intererfer uncertainties are scaled according to their effect on , which is determined from the DIP values. unchanged |
| 9.2.1LA Enhanced Performance Type 3i | Same as 9.2.1L Minimum requirements apply for each carrier | Same as 9.2.1L, for each carrier | Same as 9.2.1L Test requirements apply for each carrier |
| 9.2.2A to 9.2.2E Open loop diversity performance | -6 and -3 dB = -60 dBm = 0 and 10 dB | 0.1 dB for 0.8 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.2.3A to 9.2.3E Closed loop diversity performance | Same as 9.2.2A | Same as 9.2.2A | Same as 9.2.2A |
| 9.2.4A MIMO performance | -2 dB = -60 dBm = 6 and 10 dB | 0.1 dB for 0.8 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.2.4B MIMO and 64QAM performance | -1.5 dB = -60 dBm = 18 dB | 0.1 dB for 0.8 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.2.4C MIMO Performance - Fixed Reference Channel (FRC) H-Set 9A | -2 dB = -60 dBm = 6 and 10 dB | 0.1 dB for 0.8 dB for | Formulas: = ratio + TT = ratio + TT unchanged Test requirements should be applied for both the cells |
| 9.2.4D MIMO Performance - Fixed Reference Channel (FRC) H-Set 11A | -1.5 dB = -60 dBm = 18 dB | 0.1 dB for 0.8 dB for | Formulas: = ratio + TT = ratio + TT unchanged Test requirements should be applied for both the cells |
| 9.3.1 Single Link Performance - AWGN propagation conditions |  | No test tolerances applied |  |
| 9.3.1A Single Link Performance - AWGN propagation conditions, 64QAM |  | No test tolerances applied |  |
| 9.3.1B Single Link Performance - AWGN Propagation Conditions, DC-HSDPA requirements |  | No test tolerances applied |  |
| 9.3.1C Single Link Performance - AWGN Propagation Conditions, Periodically Varying Radio Conditions |  | No test tolerances applied for test step 7 and 8. TT for the M-difference is 1 for test step 9. | 10 dB between and maps to 6 between medians M1 and M2 hence 0.6M per dB. 1dB linearity uncertainty maps to 0.6 M. Since M is integer, the M uncertainty is 1: TT for M is defined to 1 Formula: difference in medians M1 and M2 = 6-TT |
| 9.3.2 Single Link Performance - Fading propagation conditions |  | No test tolerances applied |  |
| 9.3.2A Single Link Performance - Fading propagation conditions, DC-HSDPA requirements |  | No test tolerances applied |  |
| 9.3.2B Single Link Performance - Fading propagation conditions, 64QAM |  | No test tolerances applied |  |
| 9.3.3 Open Loop Diversity Performance - AWGN propagation conditions |  | No test tolerances applied |  |
| 9.3.4 Open Loop Diversity Performance - Fading propagation conditions |  | No test tolerances applied |  |
| 9.3.5 Closed Loop Diversity Performance - AWGN propagation conditions |  | No test tolerances applied |  |
| 9.3.6 Closed Loop Diversity Performance - Fading propagation conditions |  | No test tolerances applied |  |
| 9.3.7A, MIMO performance – Reporting of Channel Quality indicator - Single stream fading conditions |  | No test tolerances applied |  |
| 9.3.7B MIMO performance – Reporting of Channel Quality indicator - Dual stream fading conditions |  | No test tolerances applied |  |
| 9.3.7C MIMO performance – Reporting of Channel Quality indicator - Dual stream fading conditions– UE categories 19-20 |  | No test tolerances applied |  |
| 9.3.7D MIMO performance – Reporting of Channel Quality indicator - Dual stream static orthogonal conditions – UE categories 15-20 |  | No test tolerances applied |  |
| 9.3.7E MIMO performance –Reporting of Channel Quality indicator - Dual stream static orhogonal conditions – UE categories 19-20 |  | No test tolerances applied |  |
| 9.4.1 Single Link Performance | -9, -9.9 and -10 dB = -60 dBm = 0 and 5 dB | 0.1 dB for 0.6 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.4.1A Single Link Performance - Enhanced Performance Requirements Type 1 | -12 and -15.6 dB = -60 dBm = 0 dB | 0.1 dB for 0.6 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.4.2 Open loop diversity performance | -11.6, -13.4 and -11.5 dB = -60 dBm = 0 and 5 dB | 0.1 dB for 0.8 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.4.2A Open loop diversity performance – Enhanced Performance Requirements Type 1 | -15.2 and -16.4 dB = -60 dBm = 0 dB | 0.1 dB for 0.8 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.4.3 HS-SCCH Type 3 performance | -14.7, -15.6, -16 and -16.8 dB = -60 dBm = 0 dB | 0.1 dB for 0.8 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.5.1 HS-SCCH-less demodulation of HS-DSCH | -6 dB = -60 dBm = 0 dB | 0.1 dB for 0.6 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.5.1A HS-SCCH-less demodulation of HS-DSCH, Enhanced Performance Requirements Type 1 | -9 dB = -60 dBm = 0 dB | 0.1 dB for 0.6 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.6.1 Single link HS-DSCH Demodulation performance in CELL_FACH state | -6 dB = -60 dBm = 0 dB | 0.1 dB for 0.6 dB for | Formulas: = ratio + TT = ratio + TT unchanged |
| 9.6.2 Single link HS-SCCH Detection performance in CELL_FACH state | -10 dB = -60 dBm = 0 dB | 0.1 dB for 0.6 dB for | Formulas: = ratio + TT = ratio + TT unchanged |

[skipping many sections]

## F.5.5	Performance measurements (HSDPA)

Table F.5.5: Equipment accuracy for performance measurements (HSDPA)

| Clause | Equipment accuracy | Test conditions |
| --- | --- | --- |
| 9.2.1A to 9.2.1I 9.2.1IA,9.2.1JA,9.2.1KA Single Link Performance | ±0.1 dB | -12, -9, -6 and -3 dB |
| 9.2.2A to 9.2.2E Open loop diversity performance | Same as 9.2.1A | Same as 9.2.1A |
| 9.2.3A to 9.2.3E Closed loop diversity performance | Same as 9.2.1A | Same as 9.2.1A |
| 9.2.4A to 9.2.4D MIMO performance | Same as 9.2.1A | -1.5, -2 dB |
| 9.3.1 Single Link Performance - AWGN propagation conditions | Same as 9.2.1A |  |
| 9.3.1A Single Link Performance - AWGN propagation conditions, 64QAM | Same as 9.2.1A |  |
| 9.3.1B Single Link Performance - AWGN propagation conditions, DC HSDPA requirements | Same as 9.2.1A |  |
| 9.3.1C Single Link Performance - AWGN Propagation Conditions, Periodically Varying Radio Conditions | Same as 9.2.1A Additionally Ioc linearity 1dB | -60dBm ≤ Ioc ≤ -50dBm |
| 9.3.2 Single Link Performance - Fading propagation conditions | Same as 9.2.1A |  |
| 9.3.2A Single Link Performance - Fading propagation conditions, DC HSDPA requirements | Same as 9.2.1A |  |
| 9.3.2B Single Link Performance - Fading propagation conditions, 64QAM | Same as 9.2.1A |  |
| 9.3.3 Open Loop Diversity Performance - AWGN propagation conditions | Same as 9.2.1A |  |
| 9.3.4 Open Loop Diversity Performance - Fading propagation conditions | Same as 9.2.1A |  |
| 9.3.5 Closed Loop Diversity Performance - AWGN propagation conditions | Same as 9.2.1A |  |
| 9.3.6 Closed Loop Diversity Performance - Fading propagation conditions | Same as 9.2.1A |  |
| 9.3.7A, MIMO performance – Reporting of Channel Quality indicator - Single stream fading conditions | Same as 9.2.1A |  |
| 9.3.7B MIMO performance – Reporting of Channel Quality indicator - Dual stream fading conditions | Same as 9.2.1A |  |
| 9.3.7C MIMO performance – Reporting of Channel Quality indicator - Dual stream fading conditions– UE categories 19-20 | Same as 9.2.1A |  |
| 9.3.7D MIMO performance – Reporting of Channel Quality indicator - Dual stream static orthogonal conditions – UE categories 15-20 | Same as 9.2.1A |  |
| 9.3.7E MIMO performance –Reporting of Channel Quality indicator - Dual stream static orhogonal conditions – UE categories 19-20 | Same as 9.2.1A |  |
| 9.5.1 HS-SCCH-less demodulation of HS-DSCH | Same as 9.2.1A | -6 dB |
| 9.5.1A HS-SCCH-less demodulation of HS-DSCH, Enhanced Performance Requirements Type 1 | Same as 9.2.1A | -9 dB |
| 9.6.1 Single link HS-DSCH Demodulation performance in CELL_FACH state | Same as 9.2.1A |  |
| 9.6.2 Single link HS-SCCH Detection performance in CELL_FACH state | Same as 9.2.1A |  |

[skipping many sections]

Table F.6.3.5.4.9: HS-DSCH and HS-SCCH reception in CELL-FACH state

| 9.6.1 Single link HS-DSCH demodulation performance in CELL-FACK | Absolute Test requirement SDU ER |  | No of events/No of samples in % Error Ratio(ER) (Success Ratio(SR)) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | ER / SR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 3 |  |  |  |  |  |  |  |
| (= 0 dB) | VA30 | 0.82 | 82% (18%) | 57/382 (m = 0.668) | 382 (>=57) | 30.5s | SR |
| 9.6.2 (HS-DCCH) (= 0 dB) | VA30 | 0.01 | 1% (99%) | 65/5247 (M = 1.5) | 5247 (<=65) | 420s | ER |

Table F.6.3.5.4.10: MIMO Performance for test case 9.2.4A, 9.2.4C  (FRC , H-Set 9/9A)

| MIMO Performance | Absolute Test requirement (kbps) | Relative test requirement (normalized to ideal=13510 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- |
| H-Set 9/9A |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |

| 1 (= 10 dB) | PA3 | 5563 | 55.8% (41.2%) | 68/192* (m = 0.725) | N.A. | 164s (fading) | RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 (= 10 dB) | VA3 | 4347 | 67.8% (32.2%) | 61/225* (m = 0.679) | N.A. | 164s (fading) | RT |
| *)nominator and denominator by its own are irrelevant, only the ratio is relevant. |  |  |  |  |  |  |  |
| MIMO Performance H-Set 9/9A Test number | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=8650 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| 3 (= 6 dB) | PA3 | 3933 | 54.5% (45.5%) | 71/180 (m = 0.739) | N.A. | 164s (fading) | RT |
| 4 (= 6 dB) | VA3 | 3011 | 65.2% (34.8%) | 62/210 (m = 0.705) | N.A. | 164s (fading) | RT |

Table F.6.3.5.2.11: MIMO Performance for test case 9.2.4D  (FRC  H-Set 11A)

| Single link Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=22074 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HSET-11/11A 64 QAM |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 18 dB) | PA3 | 9980 | 54.79% (45.21%) | 71/180 (m = 0.739) | N.A. | 164s (fading) | RT |