3GPP TSG-RAN WG5 Meeting #55 	R5-121181

# Prague, Czech Republic, 21 – 25 May 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | CRNum | rev | - | Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to ICS for inter-mode TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Clearwire, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-UEConTest_SIG |  |  |  |  |  |  | Date: |  | 2012-03-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | 1: RAN#55 reached an agreement that enables inter-mode cell reselection TCs to be “R” from Rel-9 onwards based on their current definition 2: RAN#55 confirmed that FGI-25 is mandatory to be set to one (1) in Rel-9, therefore for TCs applicable to Rel-9 and onwards, there is no need to condition applicability on FGI-25 3: Measurement TCs 8.3.1.12a, 8.3.1.13a and 8.3.1.14a are propsed to accommodate both RRC connection release with redirection and handover as mechanisms for mobility between FDD and TDD cells, therefore FGI-30 set to one (1) is no longer needed as a requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: Cell reselection TCs are changed to applicable from Rel-9 onwards 2: Checking of FGI-25 as part of the applicability is removed 3: Checking of FGI-30 is removed from the three measurement TCs that support both mobility options 4: Condition “C130” is removed, as it is no longer required |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases could be incorrectly applied or not applied due to applicability requiremetns being unaligned with the TC definition and core specification requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Table 4-1: Applicability of tests and additional information for testing

| Clause | TC Title | Release | Applicability |  | Additional Information |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment | Specific ICS |  | Specific IXIT |  | Number of TC Executions |
|  | IDLE MODE |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 6.1.1.1a | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN; Automatic mode; between FDD and TDD | Rel-8 | R | UEs supporting E-UTRA FDD and E-UTRA TDD | pc_eFDD and pc_eTDD |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 6.1.1.3a | Cell reselection of ePLMN in manual mode / between FDD and TDD | Rel-9 | R | UEs supporting E-UTRA FDD and E-UTRA TDD | pc_eFDD and pc_eTDD |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 6.1.1.4a | PLMN selection in shared network environment / Automatic mode / Between FDD and TDD | Rel-8 | R | UEs supporting E-UTRA FDD and E-UTRA TDD | pc_eFDD and pc_eTDD |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 6.1.2.15a | Inter-frequency cell reselection according to cell reselection priority provided by SIBs / Between FDD and TDD | Rel-9 | R | UEs supporting E-UTRA FDD and E-UTRA TDD | pc_eFDD and pc_eTDD |  |  |  |  |
| 6.1.2.16 | Cell reselection / interband operation / Between FDD and TDD | Rel-9 | R | UEs supporting E-UTRA FDD and E-UTRA TDD | pc_eFDD and pc_eTDD |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 8 | RADIO RESOURCE CONTROL |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 8.1.3.11a | RRC connection release / Redirection to another E-UTRAN band / Inter-band / Between FDD and TDD | Rel-9 | R | UEs supporting E-UTRA FDD and E-UTRA TDD | pc_eFDD AND pc_eTDD |  |  |  |  |
| 8.1.3.12a | RRC connection release / Success / With priority information / Inter-band / Between FDD and TDD | Rel-9 | R | UEs supporting E-UTRA FDD and E-UTRA TDD | pc_eFDD AND pc_eTDD |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 8.2.4.10 | RRC connection reconfiguration / Handover / Between FDD and TDD | Rel-8 | C63 | UEs supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicator 30 | pc_eFDD AND pc_eTDD |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 8.2.4.13a | RRC connection reconfiguration / Handover / Success (with measurement) / Inter-band / Between FDD and TDD | Rel-9 | C63 | UEs supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicator 30 | pc_eFDD AND pc_eTDD |  |  |  |  |
| 8.2.4.14a | RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band / Between FDD and TDD | Rel-9 | C63 | UEs supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicator 30 | pc_eFDD AND pc_eTDD |  |  |  |  |
| 8.2.4.15a | RRC connection reconfiguration / Handover / Failure / Re-establishment failure / Inter-band / Between FDD and TDD | Rel-9 | C63 | UEs supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicator 30 | pc_eFDD AND pc_eTDD |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 8.3.1.12a | Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A3 (inter-band measurements) / Between FDD and TDD | Rel-9 | R | UEs supporting E-UTRA FDD and E-UTRA TDD | pc_eFDD AND pc_eTDD |  |  |  |  |
| 8.3.1.13a | Measurement configuration control and reporting / Intra E-UTRAN measurements / Periodic reporting (intra-frequency and inter-band measurements) / Between FDD and TDD | Rel-9 | R | UEs supporting E-UTRA FDD and E-UTRA TDD | pc_eFDD AND pc_eTDD |  |  |  |  |
| 8.3.1.14a | Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A2 and A3 (inter-band measurements) / Between FDD and TDD | Rel-9 | R | UEs supporting E-UTRA FDD and E-UTRA TDD | pc_eFDD AND pc_eTDD |  |  |  |  |
| 8.3.1.15a | Measurement configuration control and reporting / Intra E-UTRAN measurements / Inter-band handover / IE measurement configuration not present / Between FDD and TDD | Rel-9 | C63 | UEs supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicator 30 | pc_eFDD AND pc_eTDD |  |  |  |  |
| 8.3.1.16a | Measurement configuration control and reporting / Intra E-UTRAN measurements / Continuation of the measurements after RRC connection re-establishment / Inter-band / Between FDD and TDD | Rel-9 | C63 | UEs supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicator 30 | pc_eFDD AND pc_eTDD |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 13 | Multi-layer Procedures |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 13.4.1.3 | Intra-system mobility / E-UTRA FDD to E-UTRA TDD to E-UTRA FDD packet | Rel-8 | C63 | UEs supporting E-UTRA FDD and TDD and Feature Group Indicator 30 | pc_eFDD AND pc_eTDD |  |  |  |  |

Table 4-1a: Applicability of tests Conditions

C01 IF A.4.1-1/6 THEN R ELSE N/A

C02 IF A.4.4-2/2 THEN R ELSE N/A

C03 IF A.4.4-1/1 THEN R ELSE N/A

C04 IF A.4.4-2/1 THEN R ELSE N/A

C05 IF A.4.1-1/7 THEN R ELSE N/A

C06 IF A.4.1-1/3 THEN R ELSE N/A

C07 IF A.4.1-1/4 THEN R ELSE N/A

C08 IF A.4.5-1/5 THEN R ELSE N/A

C09 Void

C10 IF A.4.5-1/25 THEN R ELSE N/A

C11 IF A.4.5-1/16 AND A.4.5-1/25 THEN R ELSE N/A

C12 Void

C13 IF A.4.1-1/6 AND A.4.5-1/16 AND A.4.5-1/22 THEN R ELSE N/A

C14 IF A.4.5-1/5 AND A.4.5-1/17 THEN R ELSE N/A

C15 IF A.4.5-1/3 AND A.4.5-1/7 THEN R ELSE N/A

C16 IF A.4.5-1/7 THEN R ELSE N/A

C17 IF A.4.1-1/6 AND A.4.1-1/7 AND A.4.5-1/22 AND A.4.5-1/23 THEN R ELSE N/A

C18 Void

C19 IF A.4.5-1/6 AND A.4.5-1/7 THEN R ELSE N/A

C20 IF A.4.1-1/7 AND A.4.5-1/16 AND A.4.5-1/23 THEN R ELSE N/A

C21 IF A.4.5-1/13 AND A.4.5-1/25 THEN R ELSE N/A

C22 IF A.4.4-1/3 THEN R ELSE N/A

C23 IF A.4.4-1/4 THEN R ELSE N/A

C24 IF A.4.1-1/3 AND A.4.5-1/16 AND A.4.5-1/26 THEN R ELSE N/A

C25 IF A.4.1-1/4 AND A.4.5-1/16 AND A.4.5-1/24 THEN R ELSE N/A

C26 IF A.4.2.1.1-1/1 THEN R ELSE N/A

C27 IF (A.4.1-1/6 OR A.4.1-1/7) AND A.4.4-1/5 THEN R ELSE N/A

C28 IF A.4.5-1/1 THEN R ELSE N/A

C29 IF A.4.5-1/7 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) THEN R ELSE N/A

C30 IF A.4.5-1/20 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) THEN R ELSE N/A

C31 IF (A.4.5-1/7 AND A.4.5-1/20 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5)) THEN R ELSE N/A

C32 IF (A.4.5-1/7 AND A.4.5-1/20) THEN R ELSE N/A

C33 IF A.4.5-1/20 THEN R ELSE N/A

C34 IF A.4.4-1/6 AND A.4.4-1/7 THEN R ELSE N/A

C35 IF A.4.4-1/6 THEN R ELSE N/A

C36 IF A.4.1-1/6 AND A.4.5-1/8 AND A.4.5-1/22 THEN R ELSE N/A

C37 IF A.4.1-1/6 AND (A.4.4-1/8 OR A.4.4-1/53) AND A.4.5-2/2 THEN R ELSE N/A

C38 IF A.4.1-1/7 AND A.4.5-1/10 AND A.4.5-1/23 THEN R ELSE N/A

C39 IF A.4.1-1/6 AND A.4.5-1/5 AND A.4.5-1/19 AND A.4.5.-1/22 THEN R ELSE N/A

C40 IF A.4.1-1/7 AND A.4.5-1/5 AND A.4.5-1/19 AND A.4.5.-1/23 THEN R ELSE N/A

C41 IF A.4.1-1/4 AND A.4.2.1.1-1/3 THEN R ELSE N/A

C42 IF A.4.1-1/3 AND A.4.5-1/12 AND A.4.5-1/26 THEN R ELSE N/A

C44 IF A.4.1-1/3 AND A.4.5-1/5 AND A.4.5-1/19 AND A.4.5-1/26 THEN R ELSE N/A

C45 IF A.4.1-1/4 AND A.4.5-1/5 AND A.4.5-1/19 AND A.4.5-1/24 THEN R ELSE N/A

C46 IF A.4.1-1/1 OR A.4.1-1/2 AND( NOT A.4.4-1/9) THEN R ELSE N/A

C47 IF A.4.4-1/2 AND A.4.4-2/1THEN R ELSE N/A

C48 IF A.4.1-1/6 AND A.4.2.1.1-1/1 AND [8]A.2/1 THEN R ELSE N/A

C49 IF A.4.4-1/6 AND A.4.4-1/10 THEN R ELSE N/A

C50 Void

C51 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/9 AND (A.4.4-1/12 OR A.4.4-1/13 OR A.4.4-1/14 OR A.4.4-1/15) THEN R ELSE N/A

C52 IF A.4.1-1/4 AND A.4.4-1/16 THEN R ELSE N/A

C53 IF A.4.4-1/17 THEN R ELSE N/A

C54 IF A.4.4-1/18 THEN R ELSE N/A

C55 IF A.4.4-1/19 AND A.4.4-1/54 THEN R ELSE N/A

C56 IF (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) THEN R ELSE N/A

C57 IF (A4.1-1/1 OR A.4.1-1/2) AND A4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 THEN R ELSE N/A

C58 IF A.4.5-1/21 THEN R ELSE N/A

C59 IF A.4.1-1/6 AND A.4.4-1/5 AND NOT (A.4.2.1.1-1/1) THEN R ELSE N/A

C60 IF A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 THEN R ELSE N/A

C61 IF A.4.1-1/6 AND A.4.1-1/7 AND A.4.5-1/16 AND A.4.5-1/22 AND A.4.5-1/23 THEN R ELSE N/A

C62 Void

C63 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.5-1/30 THEN R ELSE N/A

C64 IF A.4.4-1/20 THEN R ELSE N/A

C65 Void

C66 IF [8]A.1/4 AND A.4.4-1/21 THEN R ELSE N/A

C67 Void

C68 IF A.4.4-1/6 AND A.4.4-1/22 THEN R ELSE N/A

C69 IF A.4.4-1/6 AND A.4.4-1/23 THEN R ELSE N/A

C70 Void

C71 IF A.4.2.1.1-1/4 THEN R ELSE N/A

C72 Void

C73 Void

C74 IF A.4.4-1/26 THEN R ELSE N/A

C75 IF A.4.1-1/6 AND A.4.4-1/2 THEN R ELSE N/A

C76 IF A.4.1-1/6 AND A.4.4-1/2 AND A.4.4-1/49 THEN R ELSE N/A

C77 IF A.4.1-1/6 AND A.4.5-2/1THEN R ELSE N/A

C78 Void

C79 Void

C80 IF A.4.4-1/2 AND A.4.4-1/49 THEN R ELSE N/A

C81 IF ([8]A.1/1 OR [8]A.1/2) AND A.4.2.1.1-1/1 AND A.4.5-1/8 AND [8]A.2/1 AND [8]A.3/3 THEN R ELSE N/A

C82 IF A.4.1-1/6 AND A.4.4-1/2 AND A.4.5-2/1THEN R ELSE N/A

C83 Void

C84 IF A.4.1-1/6 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND [8]A.3/3 THEN R ELSE N/A

C85 Void

C86 IF A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/4 THEN R ELSE N/A

C87 IF A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/5 THEN R ELSE N/A

C88 IF (A.4.2.1.1-1/2 OR A.4.2.1.1-1/3) AND A.4.2.1.1-1/4 AND (A.4.1-1/6 OR A.4.1-1/7) THEN R ELSE N/A

C89 IF A.4.1-1/7 AND A.4.4-1/29 THEN R ELSE N/A

C90 IF A.4.1-1/7 AND A.4.5-1/23 THEN R ELSE N/A

C91 IF A.4.1-1/6 AND A.4.5-1/22 THEN R ELSE N/A

C92 IF A.4.1-1/3 AND A.4.5-1/26 THEN R ELSE N/A

C93 IF A.4.1-1/4 AND A.4.5-1/24 THEN R ELSE N/A

C94 Void

C95 IF A.4.1-1/7 AND A.4.4-1/2 AND A.4.4-1/49 THEN R ELSE N/A

C96 IF A.4.5-1/10 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 THEN R ELSE N/A

C97 IF A.4.4-1/30 THEN R ELSE N/A

C98 IF (A.4.4-1/18 AND A.4.4-1/30) THEN R ELSE N/A

C99 IF A.4. 4-1/51 AND A.4.5-1/7 THEN R ELSE N/A

C100 IF A.4. 4-1/50 AND A.4.5-1/7 THEN R ELSE N/A

C101 Void

C102 Void

C103 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.2-1/1 THEN R ELSE N/A

C104 IF A.4.1-1/6 AND A.4.2.1.1-1/1 AND A.4.4-1/31 THEN R ELSE N/A

C105 IF A.4.1-1/6 AND A.4.2.1.1-1/1 AND A.4.5-1/8 AND [8]A.2/1 THEN R ELSE N/A

C106 IF A.4.4-1/34 THEN R ELSE N/A

C107 IF A.4.1-1/7 AND A.4.4-1/52 AND A.4.5-1/23 THEN R ELSE N/A

C108 Void

C109 IF A.4.2.1.1-1/4 AND (4.4-1/35 OR 4.4-1/36 OR A.4.4-1/37) THEN R ELSE N/A

C110 IF A.4.4-1/52 AND A.4.5-1/23 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 THEN R ELSE N/A

C111 IF A.4.4-1/38 AND A.4.4-1/52 AND A.4.5-1/23 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 THEN R ELSE N/A

C112 IF A.4.1-1/6 AND A.4.5-1/7 AND A.4.5-1/8 AND A.4.5-1/22 AND A.4.5-1/27 AND A.4.4-1/32 AND A.4.4-1/33 THEN R ELSE N/A

C113 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/5 THEN R ELSE N/A

C114 IF A.4.1-1/7 AND A.4.4-1/39 THEN R ELSE N/A

C115 IF ( A.4.1-1/7 AND NOT A.4.4-2/5 ) THEN R ELSE N/A

C116 IF A.4.1-1/4 AND A.4.2.1.1-1/6 THEN R ELSE N/A

C117 IF A.4.1-1/6 AND (([8]A.18a/14 AND [8]A.18a/18) OR ([8]A.18b/10 AND [8]A.18b/14)) AND A.4.5-1/8 AND A.4.5-1/22 THEN R ELSE N/A

C118 IF A.4.4-1/2 AND A.4.5-1/25 THEN R ELSE N/A

C119 IF A.4.1-1/6 AND A.4.4-1/2 AND A.4.5-1/22 THEN R ELSE N/A

C120 IF A.4.5-1/3 AND A.4.5-1/7 AND A.4.4-1/40 AND A.4.4-1/41 THEN R ELSE N/A

C121 IF A.4.4-2/2 AND A.4.1-1/6 THEN R ELSE N/A

C122 Void

C123 IF A.4.4-1/2 AND A.4.4-2/2THEN R ELSE N/A

C124 IF (A.4.1-1/6 OR A.4.1-1/7) AND A.4.4-1/5 AND A.4.4-2/1 THEN R ELSE N/A

C125 IF A.4.4-2/2 AND (A.4.4-2/5 or (A.4.4-2/4 AND A.4.4-1/33)) THEN R ELSE N/A

C126 IF A.4.1-1/6 AND A.4.4-1/56 THEN R ELSE N/A

C127 IF A.4.1-1/6 AND A.4.4-1/57 THEN R ELSE N/A

C128 IF A.4.4-2/2 AND (A.4.1-1/6 OR A.4.1-1/7) THEN R ELSE N/A

C129 IF A.4.4-1/58 THEN R ELSE N/A

C130 Void

C131 IF A.4.1-1/6 AND (NOT A.4.4-1/59) THEN R ELSE N/A

C132 IF (A.4.1-1/1 OR A.4.1-1/2) AND A4.4-1/61 THEN R ELSE N/A

C133 IF (A.4.1-1/1 OR A.4.1-1/2) AND A4.4-1/61 AND A.4.5-1/25 THEN R ELSE N/A

C134 IF (A.4.1-1/1 OR A.4.1-1/2) AND A4.4-1/61 AND A.4.5-1/25 AND A.4.5-3/111 THEN R ELSE N/A

C135 IF (A.4.1-1/1 OR A.4.1-1/2) AND A4.4-1/61 AND A.4.5-1/13 AND A.4.5-3/25 THEN R ELSE N/A

C136 IF (A.4.1-1/1 OR A.4.1-1/2) AND A4.4-1/61 AND A.4.5-1/13 AND A.4.5-3/25 AND A.4.5-3/112 THEN R ELSE N/A

C137 IF A.4.4-1/62 THEN R ELSE N/A