3GPP TSG RAN WG5 Meeting #57 	R5-125236

# Baltimore, USA, 5th-9th November 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | CRNum | rev | - | Current version: | 10.2.1 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Test Case 8.2.2.71 and 8.2.2.72 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI7_Test |  |  |  |  |  |  | Date: |  | 11/05/2012 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Update UL RB configuration from DCH to E-DCH |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update change in Test Case Text to reflect change made in RB SETUP condition A28a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case specification will remain incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.71, 8.2.2.72 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... 34.108 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |

#### 8.2.2.71	Radio Bearer Reconfiguration for transition from CELL_DCH to CELL_DCH with PCI Restrictions and S-CPICH Power Offset IEs: Success  (16QAM +MIMO)

8.2.2.71.1	Definition and applicability

All UE categories which support MIMO.

UE supports FDD and F-DPCH or Enhanced F-DPCH

8.2.2.71.2	Conformance requirement

If the UE receives an ACTIVE SET UPDATE, CELL UPDATE CONFIRM, or any reconfiguration message:

1>	if the IE "MIMO parameters" is not included:

2>	clear the MIMO_PARAMS variable;

2>	trigger lower layers to stop operation in MIMO mode.

1>	otherwise:

2>	for FDD, if the IE "MIMO N_cqi_type A/M_cqi ratio" is included:

3>	store the value of the IE "MIMO N_cqi_type A/M_cqi ratio" in the MIMO_PARAMS variable.

2>	for FDD, if the IE "MIMO pilot configuration" is included:

3>	store the value of the IE "MIMO pilot configuration" in the MIMO_PARAMS variable.

2>	for 1.28 Mcps TDD, if the IE "MIMO SF Mode for HS-PDSCH dual stream" is included:

3>	store the value of the IE "MIMO SF Mode for HS-PDSCH dual stream" in the MIMO_PARAMS variable.

2>	if the IE "MIMO operation" is set to "start":

3>	for FDD, if the IE "Precoding weight set restriction" is included:

4>	store the value of the IE "Precoding weight set restriction " in the MIMO_PARAMS variable. In addition, it shall be indicated to lower layers that precoding weight set restriction is configured.

3>	else:

4>	clear the value of the IE "Precoding weight set restriction" in the MIMO_PARAMS variable. In addition, it shall be indicated to lower layers that precoding weight set restriction is not configured.

2>	else if the IE "MIMO operation" is set to "continue":

3>	if the IE "Precoding weight set restriction" is included:

4>	if the IE "Precoding weight set restriction" is already stored in the variable MIMO_PARAMS before receiving this message:

5>	continue using "Precoding weight set restriction" for MIMO operation.

4>	else:

5>	UE behaviour is undefined.

3>	else:

4>	if the IE "Precoding weight set restriction" is already stored in the variable MIMO_PARAMS before receiving this message:

5>	continue using "Precoding weight set restriction" for MIMO operation.

4>	else:

5>	continue MIMO operation without using "Precoding weight set restriction".

NOTE:	This subclause applies to FDD and 1.28 Mcps TDD only.

[…]

1>	for FDD, if the IE "Secondary CPICH Info" is included and if  the IE "Channelisation code" in MIMO pilot configuration is different from the IE "Channelisation Code" in Secondary CPICH info:

2>	the UE behavior is undefined.

1>	for FDD, if the IE "Secondary CPICH Info" is included and if the IE "Power Offset For S-CPICH for MIMO" is provided, then pass the offset value received in the IE "Power Offset For S-CPICH for MIMO" to the lower layers.

[…]

If the UE receives an ACTIVE SET UPDATE, CELL UPDATE CONFIRM, or any reconfiguration message, the UE shall:

1>	take actions related to the MIMO_PARAMS variable as specified in subclause 8.5.32;

1>	determine the value of the MIMO_STATUS variable.

The MIMO_STATUS variable shall be set to TRUE only when all the following conditions are met:

1>	The UE is in CELL_DCH state;

1>	the variable HS_DSCH_RECEPTION is set to TRUE;

1>	for FDD, the variable MIMO_PARAMS contains a value for the IE "MIMO N_cqi_type A/M_cqi ratio"; and

1>	for FDD, the variable MIMO_PARAMS contains a value for the IE "MIMO pilot configuration".

1>	for 1.28 Mcps TDD, the variable MIMO_PARAMS contains a value for the IE "MIMO SF Mode for HS-PDSCH dual stream".

If any of the above conditions is not met and the MIMO_STATUS variable is set to TRUE, the UE shall:

1>	set the MIMO_STATUS variable to FALSE;

1>	clear the MIMO_PARAMS variable;

1>	trigger lower layers to stop operation in MIMO mode.

[…]

With the exception of the provisions of subclause 6A.3, the following shall apply when the UE is configured in MIMO mode:

- The UE derives the PCI value as defined in subclause 6A.4 and either a type A or a type B CQI value as

defined in subclause 6A.2.2 depending on which type of CQI shall be reported as defined below.

[…]

If the UE is not configured with precoding weight set restriction by the higher layers, the optimum precoding weight  for antenna 2 is taken from the set:

If the UE is configured with precoding weight set restriction by the higher layers, the optimum precoding vector weight  for antenna 2 is taken from the set as defined in table 7J0.

Table 7J0 : Selection of the precoding weight set when the precoding weight set restriction is configured by higher layers

| Number of Transport Blocks preferred in the composite PCI/CQI report | Precoding weight set from which the optimum precoding weight for antenna 2 is taken. |
| --- | --- |
| 1 |  |
| 2 |  |

The precoding weight is mapped to PCI values as defined in table 7J.

Table 7J: Mapping of preferred precoding weight to PCI values

|  | PCI value |
| --- | --- |
|  | 0 |
|  | 1 |
|  | 2 |
|  | 3 |

Reference

3GPP TS 25.331 clause 8.5.32, 8.5.33,8.6.6.41

3GPP TS 25.214 clause 6A.1.2.2 , 6A.4

8.2.2.71.3	Test purpose

1.	To confirm that the UE configures the lower layers with PCI restriction and S-CPICH Power offset IEs as signalled in RADIO BEARER SETUP message.

2.	To confirm that the UE retains the PCI restriction and  S-CPICH Power offset IEs when MIMO operation is set to “CONTINUE”  according to the received RADIO BEARER RECONFIGURATION message (when PCI restriction and S-CPICH Power offset IEs are excluded).

3.	To confirm that UE does not configure PCI restriction when the IE is not signalled and continue previously configured S-CPICH Power offset when MIMO operation is set to “start” according to the received RADIO BEARER RECONFIGURATION message.

4.	To confirm that UE does not configure PCI restriction and continue previously configured S-CPICH Power offset upon receiving RADIO BEARER RECONFIGURATION message when PCI restriction and S-CPICH Power offset IEs are signaled but MIMO operation is set to “CONTINUE”.

8.2.2.71.4	Method of test

Initial Condition

System Simulator: 1 cell

UE: PS-DCCH E-DCH (state 6-18) under condition A28a as specified in clause  9 of TS 34.108.

Related ICS/IXIT statement(s)

-	UE supports FDD

-	UE category supports MIMO

-	UE supports F-DPCH or Enhanced F-DPCH

Test Procedure

Table 8.2.2.71-1

| Parameter | Unit | Tx0-Rx0 | Tx1-Rx1 |
| --- | --- | --- | --- |
| CPICH Ec (FDD) | dBm/3.84MHz | -60 | -63 |

The UE is in CELL_DCH state and only signalling radio bearers have been established.

The SS transmits a RADIO BEARER SETUP message including the RAB Information setup IE to configure PS  domain RAB, and perform SRB mapping  to E-DCH/HS-DSCH (with MIMO configured for L1with PCI restriction and S-CPICH power offset). The UE transmits a RADIO BEARER SETUP COMPLETE on DCCH. The UE shall configure the lower layers with signalled S-CPICH Power ofsset. UE sends combined CQI/PCI reports of type A/B continuously every 2 ms. The PCI values reported shall be restricted to 0 or 3 for type A CQIs where one transport block is preferred. For type A CQIs where two transport blocks are preferred, restriction shall not apply.

Then, SS transmits a UE CAPABILITY ENQUIRY message on DL SRB2 using the MIMO configuration with single data streams for the SS transmit antennae. The UE shall respond with a UE CAPABILITY INFORMATION message. The SS completes the procedure by transmitting a UE CAPABILITY INFORMATION CONFIRM message to the UE.

The SS transmits a RADIO BEARER RECONFIGURATION message to the UE with MIMO operation set as “CONTINUE” (PCI restriction and S-CPICH Power offset IEs are excluded). The UE shall respond with RADIO BEARER RECONFIGURATION COMPLETE. UE shall continue to use the previously configured S-CPICH power offset IE signaled. UE sends combined CQI/PCI reports of type A/B continuously every 2 ms. The PCI values shall still be restricted to 0 or 3 for type A CQIs where one transport block is preferred. For type A CQIs where two transport blocks are preferred, restriction shall not apply.

Then, SS transmits a UE CAPABILITY ENQUIRY message on DL SRB2 using the MIMO configuration with single data streams for the SS transmit antennae. The UE shall respond with a UE CAPABILITY INFORMATION message. The SS completes the procedure by transmitting a UE CAPABILITY INFORMATION CONFIRM message to the UE.

Then, SS transmits a RADIO BEARER RECONFIGURATION message, using MIMO configuration, to the UE to change the NM ratio and start  MIMO operation with new parameters without PCI restriction and S-CPICH Power offset  IEs  being signalled. The UE shall respond with RADIO BEARER RECONFIGURATION COMPLETE. UE shall continue to use the previously configured S-CPICH power offset IE. UE sends combined CQI/PCI reports of type A/B continuously every 2 ms. The PCI values used for type A CQIs are no longer restricted and can range from values 0-3.

Then, SS transmits a RADIO BEARER RECONFIGURATION message to the UE with MIMO operation set as “continue” (PCI restriction and S-CPICH Power offset IEs are signalled). The UE shall respond with RADIO BEARER RECONFIGURATION COMPLETE. UE shall continue to use the previously configured S-CPICH power offset. UE sends combined CQI/PCI reports of type A/B continuously every 2 ms. The PCI values shall not be restricted and can range from values 0-3.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | RADIO BEARER SETUP | Setup PS RAB on E-DCH/HS-DSCH. PCI restriction and S-CPICH Power offset IEs are signalled. |
| 2 |  |  | RADIO BEARER SETUP COMPLETE |  |
| 3 |  |  |  | UE shall transmit composite CQI/PCI reports of type A or type B. The SS shall check that for 500 consecutive CQI reports, 250 are type A and 250 of type B. PCI values of 0 or 3 shall be used by the UE for type A CQIs where one transport block is preferred. For type A CQIs where two transport blocks are preferred, the restriction does not apply. The UE shall configure the lower layers with signalled S-CPICH Power ofsset. |
| 4 |  |  | UE CAPABILITY ENQUIRY | Use default message. SRB2 is sent using the configured L1 MIMO configuration. i.e. single stream on antenna 1 and antenna 2 with separate modulation patterns on S-CPICH |
| 5 |  |  | UE CAPABILITY INFORMATION |  |
| 6 |  |  | UE CAPABILITY INFORMATION CONFIRM | SRB2 is sent on HS-DSCH using configured L1 MIMO |
| 7 |  |  | RADIO BEARER RECONFIGURATION | SRB2 on HS-DSCH with MIMO operation set to “continue”. PCI restriction and S-CPICH Power offset IEs are excluded. |
| 8 |  |  | RADIO BEARER RECONFIGURATION COMPLETE |  |
| 9 |  |  |  | UE shall transmit composite CQI/PCI reports of type A or type B. The SS shall check that for 500 consecutive CQI reports, 250 are type A and 250 of type B. PCI values of 0 or 3 shall be used by the UE for type A CQIs where one transport block is preferred. For type A CQIs where two transport blocks are preferred, the restriction does not apply. |
| 10 |  |  | UE CAPABILITY ENQUIRY | Use default message. SRB2 is sent using the configured L1 MIMO configuration. i.e. single stream on antenna 1 and antenna 2 with separate modulation patterns on S-CPICH |
| 11 |  |  | UE CAPABILITY INFORMATION |  |
| 12 |  |  | UE CAPABILITY INFORMATION CONFIRM | SRB2 is sent on HS-DSCH using configured L1 MIMO |
| 13 |  |  | RADIO BEARER RECONFIGURATION | SRB2 on HS-DSCH with MIMO operation set to START, NM ratio changed to 4/5 and PCI restriction, S-CPICH Power offset IEs are excluded. |
| 14 |  |  | RADIO BEARER RECONFIGURATION COMPLETE |  |
| 15 |  |  |  | UE shall transmit composite CQI/PCI reports of type A or type B. The SS shall check that for 500 consecutive CQI reports, 400 are type A and 100 of type B. PCI values of 0 - 3 shall be used by the UE for type A CQIs. |
| 16 |  |  | UE CAPABILITY ENQUIRY | Use default message. SRB2 is sent using the configured L1 MIMO configuration. i.e. single stream on antenna 1 and antenna 2 with separate modulation patterns on S-CPICH |
| 17 |  |  | UE CAPABILITY INFORMATION |  |
| 18 |  |  | UE CAPABILITY INFORMATION CONFIRM | SRB2 is sent on HS-DSCH using configured L1 MIMO |
| 19 |  |  | RADIO BEARER RECONFIGURATION | SRB2 on HS-DSCH with MIMO operation set to “continue”, and PCI restriction, S-CPICH Power offset IEs are present. |
| 20 |  |  | RADIO BEARER RECONFIGURATION COMPLETE |  |
| 21 |  |  |  | UE shall transmit composite CQI/PCI reports of type A or type B. The SS shall check that for 500 consecutive CQI reports, 400 are type A and 100 of type B. PCI values of 0 - 3 shall be used by the UE for type A CQIs. |

Specific Message Contents

RADIO BEARER SETUP (Step 1)

Use the same message as specified for "Packet to CELL_DCH / HS-DSCH with enhanced data rate and RLC AM” and SRBs mapped on E-DCH/HS-DSCH(Mac-ehs) under condition A28a as specified in clause 9  of  TS34.108, except for the following

| Information Element | Value/remark |
| --- | --- |
| Added or Reconfigured DL TrCH information | 1 TrCH (HS-DSCH for DTCH and DCCH) |
| - Downlink transport channel type | HS-DSCH |
| - DL Transport channel identity | Not Present |
| - CHOICE DL parameters | HS-DSCH |
| - HARQ Info |  |
| - Number of Processes | 12 |
| - CHOICE Memory Partitioning | Implicit |
| - CHOICE DL MAC header type |  |
| - Added or reconfigured MAC-d flow |  |
| - MAC-hs queue to add or reconfigure list | (two queues) |
| - MAC-ehs queue Id | 0 (for DTCH) |
| - T1 | 50 |
| - MAC-hs window size | 16 |
| - MAC-ehs queue Id | 1 (for DCCH) |
| - MAC-d Flow Identity | 1 |
| - T1 | 50 |
| - MAC-hs window size | 16 |
| - MAC-d PDU size Info |  |
| - MAC-d PDU size | 148 |
| - MAC-d PDU size index | 0 |
| - MAC-hs queue to delete list | Not present |
| - DCH quality target | Not present |
| MIMO Parameters |  |
| - MIMO Operation | Start |
| - CHOICE mode | FDD |
| - MIMO N_cqi_type A/M_cqi ratio | ½ |
| - MIMO pilot configuration |  |
| - CHOICE Second CPICH pattern | Antenna1 S-CPICH |
| - Channelisation code | 13 |
| Downlink HS-PDSCH Information |  |
| - CHOICE mode | FDD |
| - DL Scrambling Code | Not Present |
| - HS-SCCH Channelisation Code Information |  |
| - HS-SCCH Channelisation Code | 7 |
| - Measurement Feedback Info |  |
| - CHOICE mode | FDD |
| - Measurement Power Offset | 12 (6 dB) |
| - CQI Feedback cycle, k | 2 ms |
| - CQI repetition factor | 1 |
| - CQI | 5 |
| - Downlink 64QAM configured | Not Present |
| - HS-DSCH TB size table MIMO-PilotConfiguration-v7f0ext - s-cpich-PowerOffset-Mimo MIMO-Parameters-v7g0ext -precodingWeightSetRestriction | Octet Aligned -3 TRUE |

UE CAPABILITY ENQUIRY (Steps 4, 10,16)

Use the same message sub-type found in TS 34.108 clause 9.1

UE CAPABILITY INFORMATION (Steps 5, 11,17)

Use the same message sub-type found in TS 34.108 clause 9.

UE CAPABILITY INFORMATION CONFIRM (Steps 6, 12,18)

Use the same message sub-type found in TS 34.108 clause 9.1

RADIO BEARER RECONFIGURATION (Step 7) (FDD)

Use the same message as specified for "Packet to CELL_DCH from CELL_DCH in PS" in 34.108 except for the following:

| Information Element | Value/remark |
| --- | --- |
| HARQ Info |  |
| - Number of processes | 12 |
| - CHOICE Memory Partitioning |  |
| - Implicit |  |
| Frequency info | Not present |
| MIMO Parameters - MIMO Operation - CHOICE mode | CONTINUE FDD |
| Maximum allowed UL TX power | Not present |
| Uplink DPCH info | Not present |
| Downlink HS-PDSCH Information |  |
| - HS-SCCH Info |  |
| - CHOICE mode | FDD |
| - DL Scrambling Code | Not present |
| - HS-SCCH Channelisation Code Information |  |
| - HS-SCCH Channelisation Code | 7 |
| - Measurement Feedback Info |  |
| - CHOICE mode | FDD |
| - Measurement Power Offset | 12 (6 dB) |
| - CQI Feedback cycle, k | 2 ms |
| - CQI repetition factor | 1 |
| - CQI | 5 (corresponds to 0dB in relative power offset) |
| - CHOICE mode | FDD (no data) |
| - Downlink 64QAM configured | Not Present |
| - HS-DSCH TB size table | Octet Aligned |
| Downlink information common for all radio links | Not Present |
| Downlink information for each radio link list MIMO-PilotConfiguration-v7f0ext - s-cpich-PowerOffset-Mimo MIMO-Parameters-v7g0ext -precodingWeightSetRestriction | Not Present Not present Not present |

RADIO BEARER RECONFIGURATION (Step 13) (FDD)

Use the same message as specified for "Packet to CELL_DCH from CELL_DCH in PS" in 34.108 except for the following:

| Information Element | Value/remark |
| --- | --- |
| HARQ Info |  |
| - Number of processes | 12 |
| - CHOICE Memory Partitioning |  |
| - Implicit |  |
| Frequency info | Not present |
| MIMO Parameters - MIMO Operation - CHOICE mode - MIMO N_cqi_type A/M_cqi ratio - MIMO pilot configuration - CHOICE Second CPICH pattern Antenna1 - Channelisation code | START FDD 4/5 S-CPICH 13 |
| Maximum allowed UL TX power | Not present |
| Uplink DPCH info | Not present |
| Downlink HS-PDSCH Information |  |
| - HS-SCCH Info |  |
| - CHOICE mode | FDD |
| - DL Scrambling Code | Not present |
| - HS-SCCH Channelisation Code Information |  |
| - HS-SCCH Channelisation Code | 7 |
| - Measurement Feedback Info |  |
| - CHOICE mode | FDD |
| - Measurement Power Offset | 12 (6 dB) |
| - CQI Feedback cycle, k | 2 ms |
| - CQI repetition factor | 1 |
| - CQI | 5 (corresponds to 0dB in relative power offset) |
| - CHOICE mode | FDD (no data) |
| - Downlink 64QAM configured | Not Present |
| - HS-DSCH TB size table | Octet Aligned |
| Downlink information common for all radio links | Not Present |
| Downlink information for each radio link list MIMO-PilotConfiguration-v7f0ext - s-cpich-PowerOffset-Mimo MIMO-Parameters-v7g0ext -precodingWeightSetRestriction | Not Present Not present Not present |

RADIO BEARER RECONFIGURATION (Step 19) (FDD)

Use the same message as specified for "Packet to CELL_DCH from CELL_DCH in PS" in 34.108 except for the following:

| Information Element | Value/remark |
| --- | --- |
| HARQ Info |  |
| - Number of processes | 12 |
| - CHOICE Memory Partitioning |  |
| - Implicit |  |
| Frequency info | Not present |
| MIMO Parameters - MIMO Operation - CHOICE mode | CONTINUE FDD |
| Maximum allowed UL TX power | Not present |
| Uplink DPCH info | Not present |
| Downlink HS-PDSCH Information |  |
| - HS-SCCH Info |  |
| - CHOICE mode | FDD |
| - DL Scrambling Code | Not present |
| - HS-SCCH Channelisation Code Information |  |
| - HS-SCCH Channelisation Code | 7 |
| - Measurement Feedback Info |  |
| - CHOICE mode | FDD |
| - Measurement Power Offset | 12 (6 dB) |
| - CQI Feedback cycle, k | 2 ms |
| - CQI repetition factor | 1 |
| - CQI | 5 (corresponds to 0dB in relative power offset) |
| - CHOICE mode | FDD (no data) |
| - Downlink 64QAM configured | Not Present |
| - HS-DSCH TB size table | Octet Aligned |
| Downlink information common for all radio links | Not Present |
| Downlink information for each radio link list MIMO-PilotConfiguration-v7f0ext - s-cpich-PowerOffset-Mimo MIMO-Parameters-v7g0ext -precodingWeightSetRestriction | Not Present -3 TRUE |

8.2.2.71.5	Test requirements

At step 2, the UE shall transmit a RADIO BEARER SETUP COMPLETE message.

At step 3 and  step 9, the UE shall transmit CQI reports of type A and type B. PCI values of 0 or 3 shall be used by the UE for type A CQIs where one transport block is preferred. For type A CQIs where two transport blocks are preferred, the restriction does not apply. The UE shall configure the lower layers with signalled S-CPICH Power offset.

At step 5, 11 and 17, the UE shall transmit UE CAPABILITY INFORMATION using AM RLC.

At step 8, 14 and 20, the UE shall transmit a RADIO BEARER RECONFIGURATION COMPLETE message

At step 15, the UE shall transmit CQI reports of type A and type B as per the signaled N_M Ratio. PCI values of 0 to 3 shall be used by the UE for type A CQIs being reported. The UE shall configure the lower layers with S-CPICH Power offset signalled previously i.e in RADIO BEARER SETUP message.

At step 21, the UE shall transmit CQI reports of type A and type B. PCI values of 0 to 3 shall be used by the UE for  type A CQIs being reported. The UE shall configure the lower layers with S-CPICH Power offset signalled previously i.e in RADIO BEARER SETUP message.

#### 8.2.2.72	Radio Bearer Reconfiguration for S-CPICH based MIMO with DPCH in STTD: (16QAM +MIMO)

8.2.2.72.1	Definition and applicability

All UE categories which support MIMO.

UE supports FDD and F-DPCH or Enhanced F-DPCH

8.2.2.72.2	Conformance requirement

If the UE receives an ACTIVE SET UPDATE, CELL UPDATE CONFIRM, or any reconfiguration message:

1>	if the IE "MIMO parameters" is not included:

2>	clear the MIMO_PARAMS variable;

2>	trigger lower layers to stop operation in MIMO mode.

1>	otherwise:

2>	for FDD, if the IE "MIMO N_cqi_type A/M_cqi ratio" is included:

3>	store the value of the IE "MIMO N_cqi_type A/M_cqi ratio" in the MIMO_PARAMS variable.

2>	for FDD, if the IE "MIMO pilot configuration" is included:

3>	store the value of the IE "MIMO pilot configuration" in the MIMO_PARAMS variable.

2>	for 1.28 Mcps TDD, if the IE "MIMO SF Mode for HS-PDSCH dual stream" is included:

3>	store the value of the IE "MIMO SF Mode for HS-PDSCH dual stream" in the MIMO_PARAMS variable.

2>	if the IE "MIMO operation" is set to "start":

3>	for FDD, if the IE "Precoding weight set restriction" is included:

4>	store the value of the IE "Precoding weight set restriction” in the MIMO_PARAMS variable. In addition, it shall be indicated to lower layers that precoding weight set restriction is configured.

3>	else:

4>	clear the value of the IE "Precoding weight set restriction" in the MIMO_PARAMS variable. In addition, it shall be indicated to lower layers that precoding weight set restriction is not configured.

2>	else if the IE "MIMO operation" is set to "continue":

3>	if the IE "Precoding weight set restriction" is included:

4>	if the IE "Precoding weight set restriction" is already stored in the variable MIMO_PARAMS before receiving this message:

5>	continue using "Precoding weight set restriction" for MIMO operation.

4>	else:

5>	UE behaviour is undefined.

3>	else:

4>	if the IE "Precoding weight set restriction" is already stored in the variable MIMO_PARAMS before receiving this message:

5>	continue using "Precoding weight set restriction" for MIMO operation.

4>	else:

5>	continue MIMO operation without using "Precoding weight set restriction".

NOTE:	This subclause applies to FDD and 1.28 Mcps TDD only.

[…]

1>	for FDD, if the IE "Secondary CPICH Info" is included and if  the IE "Channelisation code" in MIMO pilot configuration is different from the IE "Channelisation Code" in Secondary CPICH info:

2>	the UE behaviour is undefined.

1>	for FDD, if the IE "Secondary CPICH Info" is included and if the IE "Power Offset For S-CPICH for MIMO" is provided, then pass the offset value received in the IE "Power Offset For S-CPICH for MIMO" to the lower layers.

[…]

If the UE receives an ACTIVE SET UPDATE, CELL UPDATE CONFIRM, or any reconfiguration message, the UE shall:

1>	take actions related to the MIMO_PARAMS variable as specified in subclause 8.5.32;

1>	determine the value of the MIMO_STATUS variable.

The MIMO_STATUS variable shall be set to TRUE only when all the following conditions are met:

1>	The UE is in CELL_DCH state;

1>	the variable HS_DSCH_RECEPTION is set to TRUE;

1>	for FDD, the variable MIMO_PARAMS contains a value for the IE "MIMO N_cqi_type A/M_cqi ratio"; and

1>	for FDD, the variable MIMO_PARAMS contains a value for the IE "MIMO pilot configuration".

1>	for 1.28 Mcps TDD, the variable MIMO_PARAMS contains a value for the IE "MIMO SF Mode for HS-PDSCH dual stream".

If any of the above conditions is not met and the MIMO_STATUS variable is set to TRUE, the UE shall:

1>	set the MIMO_STATUS variable to FALSE;

1>	clear the MIMO_PARAMS variable;

- trigger lower layers to stop operation in MIMO mode.

[…]

If the IE "Tx Diversity Mode" is included the UE shall:

1>	if the value of the IE "Tx Diversity Mode" is STTD:

2>	configure the Layer 1 to use the Tx diversity mode indicated in the IE "Tx Diversity Mode" for the radio links for which the IE "Closed loop timing adjustment mode" is included, ignoring the actual value of IE "Closed loop timing adjustment mode".

1>	if the value of the IE "Tx Diversity Mode" is closed loop mode 1:

2>	configure the Layer 1 to use the Tx diversity mode indicated in the IE "Tx Diversity Mode" for the radio links for which the IE "Closed loop timing adjustment mode" is included, using the actual value of the IE "Closed loop timing adjustment mode".

1>	if the value of the IE "Tx Diversity Mode" is "none":

2>	configure the Layer 1 not to use Tx diversity for all radio links in the active set.

If the IE "Tx Diversity Mode" is not included, the UE shall:

1>	continue to use the already configured Tx diversity mode;

1>	in case no Tx diversity mode has been configured:

2>	do not apply Tx diversity.

For F-DPCH the UE shall:

1>	if the IE "STTD indication" is included in the IE "Downlink F-DPCH info for each RL":

2>	use STTD for F-DPCH on the radio links for which the IE "STTD indication" is set to TRUE

Reference

3GPP TS 25.331 clause 8.5.32, 8.5.33, 8.6.6.41, 8.6.6.24

8.2.2.72.3	Test purpose

1.	To confirm that the UE configures the radio bearer parameters and starts MIMO reception according to the received RADIO BEARER SETUP message and configures layer 1 to use STTD as Tx diversity mode for F-DPCH.

2.	To confirm that the UE configures the radio bearers to stop MIMO reception according to the received RADIO BEARER RECONFIGURATION message and configures layer 1 to use STTD as Tx diversity mode for F-DPCH.

8.2.2.72.4	Method of test

Initial Condition

System Simulator: 1 cell

UE: PS-DCCH E-DCH (state 6-18) under condition A28a as specified in clause  9 of TS 34.108.

Related ICS/IXIT statement(s)

-	UE supports FDD

-	UE category supports MIMO

-	UE supports F-DPCH or Enhanced F-DPCH

Test Procedure

The UE is in CELL_DCH state and only signalling radio bearers have been established.

The SS transmits a RADIO BEARER SETUP message including the RAB Information setup IE to configure PS  domain RAB, and perform SRB mapping  to E-DCH/HS-DSCH (with MIMO configured for L1). The tx-diversity mode for F-DPCH shall be set to STTD.The UE transmits a RADIO BEARER SETUP COMPLETE on DCCH. UE sends combined CQI/PCI reports of type A/B continuously every 2 ms.

Then, SS transmits a UE CAPABILITY ENQUIRY message on DL SRB2 using the MIMO configuration with single data streams for the SS transmit antennae. The UE shall respond with a UE CAPABILITY INFORMATION message. The SS completes the procedure by transmitting a UE CAPABILITY INFORMATION CONFIRM message to the UE.

The SS transmits a RADIO BEARER RECONFIGURATION message,using MIMO configuration to the UE to deactivate MIMO and configure user plane bearers for normal HS-DSCH reception i.e. non MIMO. The tx-diversity mode for F-DPCH shall be set to STTD.The UE shall respond with RADIO BEARER RECONFIGURATION COMPLETE. UE sends combined CQI/PCI reports of type A/B continuously every 8 ms.

Then, SS transmits a UE CAPABILITY ENQUIRY message on DL SRB2 using the MIMO configuration with single data streams for the SS transmit antennae. The UE shall respond with a UE CAPABILITY INFORMATION message. The SS completes the procedure by transmitting a UE CAPABILITY INFORMATION CONFIRM message to the UE.

Then, SS transmits a RADIO BEARER RECONFIGURATION message, using MIMO configuration, to the UE to change the NM ratio and start  MIMO operation. The tx-diversity mode for F-DPCH shall be set to STTD. The UE shall respond with RADIO BEARER RECONFIGURATION COMPLETE. UE sends combined CQI/PCI reports of type A/B continuously every 2 ms.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | RADIO BEARER SETUP | Setup PS RAB on E-DCH/HS-DSCH. Tx-diversity mode for F-DPCH is set to STTD. |
| 2 |  |  | RADIO BEARER SETUP COMPLETE |  |
| 3 |  |  |  | UE shall transmit composite CQI/PCI reports of type A or type B. The SS shall check that for 500 consecutive CQI reports, 250 are type A and 250 of type B. |
| 4 |  |  | UE CAPABILITY ENQUIRY | Use default message. SRB2 is sent using the configured L1 MIMO configuration. i.e. single stream on antenna 1 and antenna 2 with separate modulation patterns on S-CPICH |
| 5 |  |  | UE CAPABILITY INFORMATION |  |
| 6 |  |  | UE CAPABILITY INFORMATION CONFIRM | SRB2 is sent on HS-DSCH using configured L1 MIMO |
| 7 |  |  | RADIO BEARER RECONFIGURATION | SRB2 on HS-DSCH with MIMO parameters absent |
| 8 |  |  | RADIO BEARER RECONFIGURATION COMPLETE |  |
| 9 |  |  |  | UE should transmit CQI reports without PCI values every 8ms. Tx-diversity mode for F-DPCH is set to STTD. |
| 10 |  |  | UE CAPABILITY ENQUIRY | Use default message. SRB2 sent on HS-DSCH |
| 11 |  |  | UE CAPABILITY INFORMATION |  |
| 12 |  |  | UE CAPABILITY INFORMATION CONFIRM | SRB2 on HS-DSCH |
| 13 |  |  | RADIO BEARER RECONFIGURATION | SRB2 on HS-DSCH with MIMO operation set to START, NM ratio changed to 4/5 and Tx-diversity mode for F-DPCH set to STTD. |
| 14 |  |  | RADIO BEARER RECONFIGURATION COMPLETE |  |
| 15 |  |  |  | UE shall transmit composite CQI/PCI reports of type A or type B. The SS shall check that for 500 consecutive CQI reports, 400 are type A and 100 of type B. |
| 16 |  |  | UE CAPABILITY ENQUIRY | Use default message. SRB2 is sent using the configured L1 MIMO configuration. i.e. single stream on antenna 1 and antenna 2 with separate modulation patterns on S-CPICH |
| 17 |  |  | UE CAPABILITY INFORMATION |  |
| 18 |  |  | UE CAPABILITY INFORMATION CONFIRM | SRB2 is sent on HS-DSCH using configured L1 MIMO |

Specific Message Contents

RADIO BEARER SETUP (Step 1)

Use the same message as specified for "Packet to CELL_DCH / HS-DSCH with enhanced data rate and RLC AM” and SRBs mapped on E-DCH/HS-DSCH(Mac-ehs) under condition A28a as specified in clause 9  of  TS34.108, except for the following

| Information Element | Value/remark |
| --- | --- |
| Added or Reconfigured DL TrCH information | 1 TrCH (HS-DSCH for DTCH and DCCH) |
| - Downlink transport channel type | HS-DSCH |
| - DL Transport channel identity | Not Present |
| - CHOICE DL parameters | HS-DSCH |
| - HARQ Info |  |
| - Number of Processes | 12 |
| - CHOICE Memory Partitioning | Implicit |
| - CHOICE DL MAC header type |  |
| - Added or reconfigured MAC-d flow |  |
| - MAC-hs queue to add or reconfigure list | (two queues) |
| - MAC-ehs queue Id | 0 (for DTCH) |
| - T1 | 50 |
| - MAC-hs window size | 16 |
| - MAC-ehs queue Id | 1 (for DCCH) |
| - MAC-d Flow Identity | 1 |
| - T1 | 50 |
| - MAC-hs window size | 16 |
| - MAC-d PDU size Info |  |
| - MAC-d PDU size | 148 |
| - MAC-d PDU size index | 0 |
| - MAC-hs queue to delete list | Not present |
| - DCH quality target | Not present |
| MIMO Parameters |  |
| - MIMO Operation | Start |
| - CHOICE mode | FDD |
| - MIMO N_cqi_type A/M_cqi ratio | ½ |
| - MIMO pilot configuration |  |
| - CHOICE Second CPICH pattern | Antenna1 S-CPICH |
| - Channelisation code | 13 |
| Downlink HS-PDSCH Information |  |
| - CHOICE mode | FDD |
| - DL Scrambling Code | Not Present |
| - HS-SCCH Channelisation Code Information |  |
| - HS-SCCH Channelisation Code | 7 |
| - Measurement Feedback Info |  |
| - CHOICE mode | FDD |
| - Measurement Power Offset | 12 (6 dB) |
| - CQI Feedback cycle, k | 2 ms |
| - CQI repetition factor | 1 |
| - CQI | 5 |
| - Downlink 64QAM configured | Not Present |
| - HS-DSCH TB size table DL-CommonInformation -dl-DPCH-InfoCommon -CHOICE mode -tx-DiversityMode DL-FDPCH-InfoPerRL-r7 -sttdIndication | Octet Aligned FDD STTD TRUE |

UE CAPABILITY ENQUIRY (Steps 4, 10,16)

Use the same message sub-type found in TS 34.108 clause 9.1

UE CAPABILITY INFORMATION (Steps 5, 11,17)

Use the same message sub-type found in TS 34.108 clause 9.

UE CAPABILITY INFORMATION CONFIRM (Steps 6, 12,18)

Use the same message sub-type found in TS 34.108 clause 9.1

RADIO BEARER RECONFIGURATION (Step 7) (FDD)

Use the same message as specified for "Packet to CELL_DCH from CELL_DCH in PS" in 34.108 except for the following:

| Information Element | Value/remark |
| --- | --- |
| HARQ Info |  |
| - Number of processes | 6 |
| - CHOICE Memory Partitioning |  |
| - Implicit |  |
| Frequency info | Not present |
| MIMO Parameters | Not present |
| Maximum allowed UL TX power | Not present |
| Uplink DPCH info | Not present |
| Downlink HS-PDSCH Information |  |
| - HS-SCCH Info |  |
| - CHOICE mode | FDD |
| - DL Scrambling Code | Not present |
| - HS-SCCH Channelisation Code Information |  |
| - HS-SCCH Channelisation Code | 7 |
| - Measurement Feedback Info |  |
| - CHOICE mode | FDD |
| - Measurement Power Offset | 12 (6 dB) |
| - CQI Feedback cycle, k | 8 ms |
| - CQI repetition factor | 1 |
| - CQI | 5 (corresponds to 0dB in relative power offset) |
| - CHOICE mode | FDD (no data) |
| - Downlink 64QAM configured | Not Present |
| - HS-DSCH TB size table | Octet Aligned |
| DL-CommonInformation -dl-DPCH-InfoCommon -CHOICE mode -tx-DiversityMode DL-FDPCH-InfoPerRL-r7 -sttdIndication | FDD STTD TRUE |

RADIO BEARER RECONFIGURATION (Step 13) (FDD)

Use the same message as specified for "Packet to CELL_DCH from CELL_DCH in PS" in 34.108 except for the following:

| Information Element | Value/remark |
| --- | --- |
| HARQ Info |  |
| - Number of processes | 12 |
| - CHOICE Memory Partitioning |  |
| - Implicit |  |
| Frequency info | Not present |
| MIMO Parameters - MIMO Operation - CHOICE mode - MIMO N_cqi_type A/M_cqi ratio - MIMO pilot configuration - CHOICE Second CPICH pattern Antenna1 - Channelisation code | START FDD 4/5 S-CPICH 13 |
| Maximum allowed UL TX power | Not present |
| Uplink DPCH info | Not present |
| Downlink HS-PDSCH Information |  |
| - HS-SCCH Info |  |
| - CHOICE mode | FDD |
| - DL Scrambling Code | Not present |
| - HS-SCCH Channelisation Code Information |  |
| - HS-SCCH Channelisation Code | 7 |
| - Measurement Feedback Info |  |
| - CHOICE mode | FDD |
| - Measurement Power Offset | 12 (6 dB) |
| - CQI Feedback cycle, k | 2 ms |
| - CQI repetition factor | 1 |
| - CQI | 5 (corresponds to 0dB in relative power offset) |
| - CHOICE mode | FDD (no data) |
| - Downlink 64QAM configured | Not Present |
| - HS-DSCH TB size table DL-CommonInformation -dl-DPCH-InfoCommon -CHOICE mode -tx-DiversityMode DL-FDPCH-InfoPerRL-r7 -sttdIndication | Octet Aligned FDD STTD TRUE |

8.2.2.72.5	Test requirements

At step 2, the UE shall transmit a RADIO BEARER SETUP COMPLETE message.

At step 3, the UE shall transmit CQI reports of type A and type B as per the signalled N_M ratio.

At step 5 ,11 and 17  the UE shall transmit UE CAPABILITY INFORMATION using AM RLC.

At step 8 and 14 , the UE shall transmit a RADIO BEARER RECONFIGURATION COMPLETE message

At step 15, the UE shall transmit CQI reports of type A and type B as per the signaled N_M Ratio.