- 3GPP TSG- RAN WG5 Meeting #58	R5-130553

# Malta, MT, 28 January – 01 February 2013

| - CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | - 34.121-1 | - CR | - CRNum | - rev | - - | - Current version: | - 10.5.0 |  |
| - For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| - Proposed change affects: | - UICC apps |  | - ME | - X | - Radio Access Network |  | - Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| - Title: | - Correction of connection diagram for TC 9.2.1Fx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Source to WG: | - Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| - Source to TSG: | - R5 |  |  |  |  |  |  |  |  |  |
| - Work item code: | - TEI10_Test |  |  |  |  |  |  | - Date: |  | - 2013-01-18 |
| - Category: | - F |  |  |  |  |  |  | - Release: |  | - Rel-10 |
|  | - Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) - Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | - Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| - Reason for change: |  | - In RAN5#57, based on RAN4 feedback (CR R4-126018) it was agreed that TC 9.2.1Fx would be used as a reference test case to verify single Rx performance of a Type 3/3i capable UE. Accordingly the connection diagram to be used for a Type 3/3i capable UE should be identical to a Type 2 capable UE. |  |  |  |  |  |  |  |  |
| - Summary of change: |  | Updated connection diagram to verify single Rx performance measurement. |  |  |  |  |  |  |  |  |
| - Consequences if not approved: |  | - Current test spec would not be compliant to RAN 4 recommendations. |  |  |  |  |  |  |  |  |
| - Clauses affected: |  | - 9.2.1F, 9.2.1FA, 9.2.1FB, 9.2.1FC, 9.2.1FD, |  |  |  |  |  |  |  |  |
|  |  | - Y | - N |  |  |  |  |  |  |  |
| - Other specs |  |  | - X | - Other core specifications |  |  | - TS/TR ...CR ... |  |  |  |
| - affected: |  |  | - X | - Test specifications |  |  | - TS/TR …CR ... |  |  |  |
| - (show related CRs) |  |  | - X | - O&M Specifications |  |  | - TS/TR ...CR ... |  |  |  |
| - Other comments: |  |  |  |  |  |  |  |  |  |  |

[Beginning of modifications]

### 9.2.1F	Single Link Performance - Enhanced Performance Requirements Type 2 - QPSK/16QAM, Fixed Reference Channel (FRC) H-Set 6/3

#### 9.2.1F.1	Definition and applicability

The receiver single link performance of the High Speed Physical Downlink Shared Channel (HS-DSCH) in different multi-path fading environments are determined by the information bit throughput R.

The requirements and this test apply for Release 6 and later releases to all types of UTRA for the FDD UE that support: HSDPA UE capability categories 7 - 10 and the optional enhanced performance requirements type 2 or type 3 or type 3i.

The requirements and this test apply also for Release 7 and later releases to all types of UTRA for the FDD UE that support: HSDPA UE capability categories 13-14.

#### 9.2.1F.2	Minimum requirements

The performance requirements for a particular UE belonging to a certain HS-DSCH category and supporting the optional enhanced performance requirements type 2 are determined according to the relevant part of Table 9.2.3.

During the Fixed Reference Channel (FRC) tests the behaviour of the Node-B emulator in response to the ACK/NACK signalling field of the HS-DPCCH is specified in Table 9.2.4.

The requirements are specified in terms of minimum information bit throughput R for the DL reference channels H-set 6/3 specified in Annex C.8.1.6 and C.8.1.3 respectively, with the addition of the relevant parameters in Tables 9.2.1F.1, 9.2.1F.3 and 9.2.1F.5 plus the downlink physical channel setup according to table E.5.1.

Using this configuration the throughput shall meet or exceed the minimum requirements specified in tables 9.2.1F.2, 9.2.1F.4 and 9.2.1F.6.

Table 9.2.1F.1: Test Parameters for Testing QPSK FRCs H-Set 6

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Redundancy and constellation version coding sequence |  | {0,2,5,6} |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |

Table 9.2.1F.2: Minimum requirement Enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 6

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 1494 |
|  |  | -3 | 2153 |
| 2 | PB3 | -6 | 1038 |
|  |  | -3 | 1744 |
| 3 | VA30 | -6 | 1142 |
|  |  | -3 | 1782 |
| 4 | VA120 | -6 | 909 |
|  |  | -3 | 1467 |

Table 9.2.1F.3: Test Parameters for Testing 16QAM FRCs H-Set 6

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |  |
| Redundancy and constellation version coding sequence |  | {6,2,1,5} |  |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |  |

Table 9.2.1F.4: Minimum requirement Enhanced requirement type 2 16QAM,
Fixed Reference Channel (FRC) H-Set 6

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 991 |
|  |  | -3 | 1808 |
| 2 | PB3 | -6 | 465 |
|  |  | -3 | 1370 |
| 3 | VA30 | -6 | 587 |
|  |  | -3 | 1488 |
| 4 | VA120 | -6 | 386 |
|  |  | -3 | 1291 |

Table 9.2.1F.5: Test Parameters for Testing QPSK FRCs H-Set 3

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Redundancy and constellation version coding sequence |  | {0,2,5,6} |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |

Table 9.2.1F.6: Minimum requirement Enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 3

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 0 dB | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 65 | N/A |
|  |  | -3 | N/A | N/A |
| 2 | PB3 | -6 | 23 | N/A |
|  |  | -3 | 138 | N/A |
| 3 | VA30 | -6 | 22 | N/A |
|  |  | -3 | 142 | N/A |
| 4 | VA120 | -6 | 13 | N/A |
|  |  | -3 | 140 | N/A |
| * NOTES: 1) The reference value R is for the Fixed Reference Channel (FRC) H-Set 1, for Fixed Reference Channel (FRC) H-Set 3 the reference values for R should be scaled (multiplied by 3 and rounding to the nearest integer t-put in kbps, where values of i+1/2 are rounded up to i+1, i integer) 2) For UE supporting enhanced performance requirement type 2 and condition = 10 dB this is tested using the Fixed Reference Channel (FRC) H-Set 6. |  |  |  |  |

NOTE:	Table 9.2.1F.6 is based on core requirements for minimum requirement as explained in Table 9.2.3.

The reference for this requirement is TS 25.101 [1] clauses 9.2.1.1, 9.2.1.2, 9.2.1.4, and 9.2.1.5.

#### 9.2.1F.3	Test purpose

To verify the ability of the receiver to receive a predefined test signal, representing a multi-path fading channel with information bit throughput R not falling below a specified value. The test stresses the multicode reception and channel decoding with incremental redundancy.

#### 9.2.1F.4	Method of test

##### 9.2.1F.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

1)	Connect the SS (node B emulator) and fader and AWGN noise source to the UE antenna connector as shown in figure A.10 for UEs that do not support receive diversity and for UEs supporting Type 3 or 3i.

NOTE: For a transition period of two meeting cycles until RAN#60, figure A.21 does not need to be used.

2)	Set up an HSDPA call with looping back 12.2kbps RMC according to TS 34.108 [3] clause 7.3.6 with levels according to table E.5.0.

3)	Set the node B emulator behaviour according to table 9.2.4. Set the test parameters for tests 1-4 according to table  9.2.1F.1, 9.2.1F.3 or 9.2.1F.5 and levels according to tables  9.2.1F.7 to 9.2.1F.12 as appropriate. The configuration of the downlink channels is defined in table E.5.1.

4)	The information bit data shall be pseudo random and not repeated before 10 different information bit payload blocks are processed. (e.g. Fixed reference Channel Definition H-set 6 (16 QAM): The information bit payload block is 9377 bits long. Hence the PRBSequence must be at least 9377 * 10 bits long. ) Use a PRBS from ITU-T O.153 Ref [27]

5)	The SS shall not time the transmission freely. It shall time the transmission strictly according to the  reference measurement channels: i.e. Process number is continued exactly after 6 TTIs.

6)	Setup the fading simulator with fading conditions as described in  table D.2.2.1.A.

##### 9.2.1F.4.2	Procedure

1)	Once the HSDPA connection is setup, change levels according to Tables E.5.6 to E.5.8B as specified by  table E.5.9 and start transmitting HSDPA Data.

2)	For all relevant propagation conditions, for all relevant Ioc levels, for all relevant Ec/Ior, for all relevant Îor/Ioc, for all relevant H-sets in  tables 9.2.1F.7 to 9.2.1F.12 count the number of  NACK, ACK and statDTX on the UL HS-DPCCH during the test interval and  decide pass or fail according to Annex F.6.3 tables F.6.3.5.2.1, F.6.3.5.2.5B and F.6.3.5.2.6B.

#### 9.2.1F.5	Test Requirements

Tables 9.2.1F.7 to 9.2.1F.12 define the primary level settings including test tolerance for all relevant throughput tests. The pass / fail decision for throughput is done according to  Annex F.6.3.

Tables E.5.6 to E.5.8B define the secondary and subsequently ranked level settings including test tolerance. As those level settings are not uniform for the throughput tests in this clause, Table E.5.9 indicates which levels are applied, when the primary level settings (Ec/Ior and Ior/Ioc) and propagation conditions (PA3,PB3, VA30, VA 120) vary.

Table 9.2.1F.7: Test Parameters for Testing QPSK FRCs H-Set 6

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1F.8: Test requirement enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 6

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 1494 |
|  |  | -2.9 | 2153 |
| 2 | PB3 | -5.9 | 1038 |
|  |  | -2.9 | 1744 |
| 3 | VA30 | -5.9 | 1142 |
|  |  | -2.9 | 1782 |
| 4 | VA120 | -5.9 | 909 |
|  |  | -2.9 | 1467 |

Table 9.2.1F.9: Test Parameters for Testing 16QAM FRCs H-Set 6

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1F.10: Test requirement enhanced requirement type 2 16QAM,
Fixed Reference Channel (FRC) H-Set 6

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 991 |
|  |  | -2.9 | 1808 |
| 2 | PB3 | -5.9 | 465 |
|  |  | -2.9 | 1370 |
| 3 | VA30 | -5.9 | 587 |
|  |  | -2.9 | 1488 |
| 4 | VA120 | -5.9 | 386 |
|  |  | -2.9 | 1291 |

Table 9.2.1F.11: Test Parameters for Testing QPSK FRCs H-Set 3

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1F.12: Test requirement QPSK, Fixed Reference Channel (FRC) H-Set 3

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 0.6 dB | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 65 | N/A |
|  |  | -2.9 | N/A | N/A |
| 2 | PB3 | -5.9 | 23 | N/A |
|  |  | -2.9 | 138 | N/A |
| 3 | VA30 | -5.9 | 22 | N/A |
|  |  | -2.9 | 142 | N/A |
| 4 | VA120 | -5.9 | 13 | N/A |
|  |  | -2.9 | 140 | N/A |
| * NOTES: 1) The reference value R is for the Fixed Reference Channel (FRC) H-Set 1, for Fixed Reference Channel (FRC) H-Set 3 the reference values for R should be scaled (multiplied by 3 and rounding to the nearest integer t-put in kbps, where values of i+1/2 are rounded up to i+1, i integer) 2) For UE supporting enhanced performance requirement type 2 and condition = 10 dB this is tested using the Fixed Reference Channel (FRC) H-Set 6. |  |  |  |  |

### 9.2.1FA	Single Link Performance - Enhanced Performance Requirements Type 2 - QPSK/16QAM, Fixed Reference Channel (FRC) H-Set 6A/3A

#### 9.2.1FA.1	Definition and applicability

The receiver single link performance of the High Speed Physical Downlink Shared Channel (HS-DSCH) in different multi-path fading environments are determined by the information bit throughput R.

The requirements and this test apply for Release 8 and later releases to all types of UTRA for the FDD UE that support: HSDPA UE capability categories 21 - 24 but not supporting Dual band operation.

#### 9.2.1FA.2	Minimum requirements

The performance requirements for a particular UE belonging to HS-DSCH categories 21- 24 are determined according to the relevant part of Table 9.2.3C.

During the Fixed Reference Channel (FRC) tests the behaviour of the Node-B emulator in response to the ACK/NACK signalling field of the HS-DPCCH is specified in Table 9.2.4.

The requirements are specified in terms of minimum information bit throughput R for the DL reference channels H-set 6A/3A specified in Annex C.8.1.6 and C.8.1.3 respectively, with the addition of the relevant parameters in Tables 9.2.1FA.1, 9.2.1FA.3 and 9.2.1FA.5 plus the downlink physical channel setup according to table E.5.1.

Using this configuration the throughput shall meet or exceed the minimum requirements specified in tables 9.2.1FA.2, 9.2.1FA.4 and 9.2.1FA.6.

Table 9.2.1FA.1: Test Parameters for Testing QPSK FRCs H-Set 6A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Redundancy and constellation version coding sequence |  | {0,2,5,6} |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |

Table 9.2.1FA.2: Minimum requirement Enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 6A

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 1494 |
|  |  | -3 | 2153 |
| 2 | PB3 | -6 | 1038 |
|  |  | -3 | 1744 |
| 3 | VA30 | -6 | 1142 |
|  |  | -3 | 1782 |
| 4 | VA120 | -6 | 909 |
|  |  | -3 | 1467 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6A the reference values for R should be scaled (multiplied by 2.0) |  |  |  |

Table 9.2.1FA.3: Test Parameters for Testing 16QAM FRCs H-Set 6A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |  |
| Redundancy and constellation version coding sequence |  | {6,2,1,5} |  |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |  |

Table 9.2.1FA.4: Minimum requirement Enhanced requirement type 2 16QAM,
Fixed Reference Channel (FRC) H-Set 6A

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 991 |
|  |  | -3 | 1808 |
| 2 | PB3 | -6 | 465 |
|  |  | -3 | 1370 |
| 3 | VA30 | -6 | 587 |
|  |  | -3 | 1488 |
| 4 | VA120 | -6 | 386 |
|  |  | -3 | 1291 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6A the reference values for R should be scaled (multiplied by 2.0) |  |  |  |

Table 9.2.1FA.5: Test Parameters for Testing QPSK FRCs H-Set 3A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Redundancy and constellation version coding sequence |  | {0,2,5,6} |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |

Table 9.2.1FA.6: Minimum requirement Enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 3A

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 0 dB | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 65 | N/A |
|  |  | -3 | N/A | N/A |
| 2 | PB3 | -6 | 23 | N/A |
|  |  | -3 | 138 | N/A |
| 3 | VA30 | -6 | 22 | N/A |
|  |  | -3 | 142 | N/A |
| 4 | VA120 | -6 | 13 | N/A |
|  |  | -3 | 140 | N/A |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 1, For Fixed Reference Channel (FRC) H-Set 3A the reference values for R should be scaled (multiplied by 6.0) *NOTE 2: For UE supporting enhanced performance requirement type 2 and condition = 10 dB this is tested using the Fixed Reference Channel (FRC) H-Set 6A. |  |  |  |  |

NOTE:	Table 9.2.1FA.6 is based on core requirements for minimum requirement as explained in Table 9.2.3C.

The reference for this requirement is TS 25.101 [1] clauses 9.2.1.1, 9.2.1.2, 9.2.1.4, and 9.2.1.5.

#### 9.2.1FA.3	Test purpose

To verify the ability of the receiver to receive a predefined test signal, representing a multi-path fading channel with information bit throughput R not falling below a specified value. The test stresses the multicode reception and channel decoding with incremental redundancy.

#### 9.2.1FA.4	Method of test

##### 9.2.1FA.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

1)	Connect the SS (node B emulator) and fader and AWGN noise source to the UE antenna connector as shown in figure A.35 for UEs that do not support receive diversity and for UEs supporting Type 3 or 3i.

Note; For a transition period of two meeting cycles until RAN#60, figure A.37 does not need to be used.

2)	Set up an HSDPA call according to TS 34.108 [3] clause 7.3.13 with levels according to table E.5.0.

3)	Set the node B emulator behaviour according to table 9.2.4. Set the test parameters for tests 1-4 according to table  9.2.1FA.1, 9.2.1FA.3 or 9.2.1FA.5 and levels according to tables  9.2.1FA.7 to 9.2.1FA.12 as appropriate for both the serving HS-DSCH cell and secondary serving HS-DSCH cell.. The configuration of the downlink channels is defined in table E.5.1 for the  serving HS-DSCH cell, and for secondary serving HS-DSCH cell set up P-CPICH, HS-PDSCH and HS-SCCH channels only per table E.5.1.

4)	The information bit data shall be pseudo random and not repeated before 10 different information bit payload blocks are processed. (e.g. Fixed reference Channel Definition H-set 6 (16 QAM): The information bit payload block is 9377 bits long. Hence the PRBSequence must be at least 9377 * 10 bits long. ) Use a PRBS from ITU-T O.153 Ref [27]

5)	The SS shall not time the transmission freely. It shall time the transmission strictly according to the  reference measurement channels: i.e. Process number is continued exactly after 6 TTIs.

6)	Setup the fading simulator with fading conditions on each of the serving cells as described in  table D.2.2.1.A.

##### 9.2.1FA.4.2	Procedure

1)	Once the HSDPA connection is setup, change levels according to Tables  E.5.6 to E.5.8B as specified by  table  E.5.9 and start transmitting HSDPA Data on both the serving cells. For secondary serving HS-DSCH cell only P-CPICH, HS-PDSCH and HS-SCCH are configured.

2)	For all relevant propagation conditions, for all relevant Ioc levels, for all relevant Ec/Ior, for all relevant Îor/Ioc, for all relevant H-sets in  tables 9.2.1FA.7 to 9.2.1FA.12 count the number of  NACK, ACK and statDTX on the UL HS-DPCCH during the test interval on both the cells and  decide pass or fail according to Annex F.6.3  tables F.6.3.5.2.1, F.6.3.5.2.5B and F.6.3.5.2.6B. Throughput shall be measured per cell and compared to requirements in these tables.

#### 9.2.1FA.5	Test Requirements

Tables 9.2.1FA.7 to 9.2.1FA.12 define the primary level settings including test tolerance for all relevant throughput tests. The pass / fail decision for throughput is done according to  Annex F.6.3.

Tables  E.5.6 to E.5.8B define the secondary and subsequently ranked level settings including test tolerance to be applied on both the serving cells. As those level settings are not uniform for the throughput tests in this clause, Table  E.5.9 indicates which levels are applied, when the primary level settings (Ec/Ior and Ior/Ioc) and propagation conditions (PA3,PB3, VA30, VA 120) vary.

Table 9.2.1FA.7: Test Parameters for Testing QPSK FRCs H-Set 6A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FA.8: Test requirement enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 6A

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 1494 |
|  |  | -2.9 | 2153 |
| 2 | PB3 | -5.9 | 1038 |
|  |  | -2.9 | 1744 |
| 3 | VA30 | -5.9 | 1142 |
|  |  | -2.9 | 1782 |
| 4 | VA120 | -5.9 | 909 |
|  |  | -2.9 | 1467 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6A the reference values for R should be scaled (multiplied by 2.0). The throughput on each cell should be Reference value R. |  |  |  |

Table 9.2.1FA.9: Test Parameters for Testing 16QAM FRCs H-Set 6A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FA.10: Test requirement enhanced requirement type 2 16QAM,
Fixed Reference Channel (FRC) H-Set 6A

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 991 |
|  |  | -2.9 | 1808 |
| 2 | PB3 | -5.9 | 465 |
|  |  | -2.9 | 1370 |
| 3 | VA30 | -5.9 | 587 |
|  |  | -2.9 | 1488 |
| 4 | VA120 | -5.9 | 386 |
|  |  | -2.9 | 1291 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6A the reference values for R should be scaled (multiplied by 2.0). The throughput on each cell should be Reference value R. |  |  |  |

Table 9.2.1FA.11: Test Parameters for Testing QPSK FRCs H-Set 3A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FA.12: Test requirement QPSK, Fixed Reference Channel (FRC) H-Set 3A

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 0.6 dB | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 65 | N/A |
|  |  | -2.9 | N/A | N/A |
| 2 | PB3 | -5.9 | 23 | N/A |
|  |  | -2.9 | 138 | N/A |
| 3 | VA30 | -5.9 | 22 | N/A |
|  |  | -2.9 | 142 | N/A |
| 4 | VA120 | -5.9 | 13 | N/A |
|  |  | -2.9 | 140 | N/A |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 1, For Fixed Reference Channel (FRC) H-Set 3A the reference values for R should be scaled (multiplied by 6.0). The throughput on each cell should be Reference value R times 3. *NOTE 2: For UE supporting enhanced performance requirement type 2 and condition = 10 dB this is tested using the Fixed Reference Channel (FRC) H-Set 6A. |  |  |  |  |

### 9.2.1FB	Single Link Performance - Enhanced Performance Requirements Type 2 - QPSK/16QAM, Fixed Reference Channel (FRC) H-Set 6A/3A for DB-DC-HSDPA

#### 9.2.1FB.1	Definition and applicability

The receiver single link performance of the High Speed Physical Downlink Shared Channel (HS-DSCH) in different multi-path fading environments are determined by the information bit throughput R.

The requirements and this test apply for Release 9 and later releases to all types of UTRA for the FDD UE that support DB-DC-HSDPA and HSDPA UE capability categories 21-24.

#### 9.2.1FB.2	Minimum requirements

The performance requirements for a particular UE belonging to HS-DSCH categories 21- 24 are determined according to the relevant part of Table 9.2.3C.

During the Fixed Reference Channel (FRC) tests the behaviour of the Node-B emulator in response to the ACK/NACK signalling field of the HS-DPCCH is specified in Table 9.2.4.

The requirements are specified in terms of minimum information bit throughput R for the DL reference channels H-set 6A/3A specified in Annex C.8.1.6 and C.8.1.3 respectively, with the addition of the relevant parameters in Tables 9.2.1FB.1, 9.2.1FB.3 and 9.2.1FB.5 plus the downlink physical channel setup according to table E.5.1.

Using this configuration the throughput shall meet or exceed the minimum requirements specified in tables 9.2.1FB.2, 9.2.1FB.4 and 9.2.1FB.6.

Table 9.2.1FB.1: Test Parameters for Testing QPSK FRCs H-Set 6A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Redundancy and constellation version coding sequence |  | {0,2,5,6} |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |

Table 9.2.1FB.2: Minimum requirement Enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 6A

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 1494 |
|  |  | -3 | 2153 |
| 2 | PB3 | -6 | 1038 |
|  |  | -3 | 1744 |
| 3 | VA30 | -6 | 1142 |
|  |  | -3 | 1782 |
| 4 | VA120 | -6 | 909 |
|  |  | -3 | 1467 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6A the reference values for R should be scaled (multiplied by 2.0) |  |  |  |

Table 9.2.1FB.3: Test Parameters for Testing 16QAM FRCs H-Set 6A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |  |
| Redundancy and constellation version coding sequence |  | {6,2,1,5} |  |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |  |

Table 9.2.1FB.4: Minimum requirement Enhanced requirement type 2 16QAM,
Fixed Reference Channel (FRC) H-Set 6A

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 991 |
|  |  | -3 | 1808 |
| 2 | PB3 | -6 | 465 |
|  |  | -3 | 1370 |
| 3 | VA30 | -6 | 587 |
|  |  | -3 | 1488 |
| 4 | VA120 | -6 | 386 |
|  |  | -3 | 1291 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6A the reference values for R should be scaled (multiplied by 2.0) |  |  |  |

Table 9.2.1FB.5: Test Parameters for Testing QPSK FRCs H-Set 3A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Redundancy and constellation version coding sequence |  | {0,2,5,6} |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |

Table 9.2.1FB.6: Minimum requirement Enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 3A

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 0 dB | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 65 | N/A |
|  |  | -3 | N/A | N/A |
| 2 | PB3 | -6 | 23 | N/A |
|  |  | -3 | 138 | N/A |
| 3 | VA30 | -6 | 22 | N/A |
|  |  | -3 | 142 | N/A |
| 4 | VA120 | -6 | 13 | N/A |
|  |  | -3 | 140 | N/A |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 1, For Fixed Reference Channel (FRC) H-Set 3A the reference values for R should be scaled (multiplied by 6.0) *NOTE 2: For UE supporting enhanced performance requirement type 2 and condition = 10 dB this is tested using the Fixed Reference Channel (FRC) H-Set 6A. |  |  |  |  |

NOTE:	Table 9.2.1FB.6 is based on core requirements for minimum requirement as explained in Table 9.2.3C.

The reference for this requirement is TS 25.101 [1] clauses 9.2.1.1, 9.2.1.2, 9.2.1.4, and 9.2.1.5.

#### 9.2.1FB.3	Test purpose

To verify the ability of the receiver to receive a predefined test signal, representing a multi-path fading channel with information bit throughput R not falling below a specified value. The test stresses the multicode reception and channel decoding with incremental redundancy.

#### 9.2.1FB.4	Method of test

##### 9.2.1FB.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

- Connect the SS (node B emulator) and fader and AWGN noise source to the UE antenna connector as shown in figure A.35 for UEs that do not support receive diversity and for UEs supporting Type 3 or 3i.

Note; For a transition period of two meeting cycles until RAN#60, figure A.37 does not need to be used.

2)	Set up an HSDPA call according to TS 34.108 [3] clause 7.3.13 with levels according to table E.5.0. 3)	Set the node B emulator behaviour according to table 9.2.4. Set the test parameters for tests 1-4 according to table  9.2.1FB.1, 9.2.1FB.3 or 9.2.1FB.5 and levels according to tables  9.2.1FB.7 to 9.2.1FB.12 as appropriate for both the serving HS-DSCH cell and secondary serving HS-DSCH cell. The primary and secondary serving cells are set on different bands according to DB-DC-HSDPA configurations defined in Section 4.2. The configuration of the downlink channels is defined in table E.5.1 for the  serving HS-DSCH cell, and for secondary serving HS-DSCH cell set up P-CPICH, HS-PDSCH and HS-SCCH channels only per table E.5.1.

4)	The information bit data shall be pseudo random and not repeated before 10 different information bit payload blocks are processed. (e.g. Fixed reference Channel Definition H-set 6 (16 QAM): The information bit payload block is 9377 bits long. Hence the PRBSequence must be at least 9377 * 10 bits long. ) Use a PRBS from ITU-T O.153 Ref [27].

5)	The SS shall not time the transmission freely. It shall time the transmission strictly according to the  reference measurement channels: i.e. Process number is continued exactly after 6 TTIs.

6)	Setup the fading simulator with fading conditions on each of the serving cells as described in  table D.2.2.1.A.

##### 9.2.1FB.4.2	Procedure

1)	Once the HSDPA connection is setup, change levels according to Tables  E.5.6 to E.5.8B as specified by  table  E.5.9 and start transmitting HSDPA Data on both the serving cells. For secondary serving HS-DSCH cell only P-CPICH, HS-PDSCH and HS-SCCH are configured.

2)	For all relevant propagation conditions, for all relevant Ioc levels, for all relevant Ec/Ior, for all relevant Îor/Ioc, for all relevant H-sets in  tables 9.2.1FB.7 to 9.2.1FB.12 count the number of  NACK, ACK and statDTX on the UL HS-DPCCH during the test interval on both the cells and  decide pass or fail according to Annex F.6.3  tables F.6.3.5.2.1, F.6.3.5.2.5B and F.6.3.5.2.6B. Throughput shall be measured per cell and compared to requirements in these tables.

#### 9.2.1FB.5	Test Requirements

Tables 9.2.1FB.7 to 9.2.1FB.12 define the primary level settings including test tolerance for all relevant throughput tests. The pass / fail decision for throughput is done according to  Annex F.6.3.

Tables  E.5.6 to E.5.8B define the secondary and subsequently ranked level settings including test tolerance to be applied on both the serving cells. As those level settings are not uniform for the throughput tests in this clause, Table  E.5.9 indicates which levels are applied, when the primary level settings (Ec/Ior and Ior/Ioc) and propagation conditions (PA3,PB3, VA30, VA 120) vary.

Table 9.2.1FB.7: Test Parameters for Testing QPSK FRCs H-Set 6A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FB.8: Test requirement enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 6A

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 1494 |
|  |  | -2.9 | 2153 |
| 2 | PB3 | -5.9 | 1038 |
|  |  | -2.9 | 1744 |
| 3 | VA30 | -5.9 | 1142 |
|  |  | -2.9 | 1782 |
| 4 | VA120 | -5.9 | 909 |
|  |  | -2.9 | 1467 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6A the reference values for R should be scaled (multiplied by 2.0). The throughput on each cell should be Reference value R. |  |  |  |

Table 9.2.1FB.9: Test Parameters for Testing 16QAM FRCs H-Set 6A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FB.10: Test requirement enhanced requirement type 2 16QAM,
Fixed Reference Channel (FRC) H-Set 6A

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 991 |
|  |  | -2.9 | 1808 |
| 2 | PB3 | -5.9 | 465 |
|  |  | -2.9 | 1370 |
| 3 | VA30 | -5.9 | 587 |
|  |  | -2.9 | 1488 |
| 4 | VA120 | -5.9 | 386 |
|  |  | -2.9 | 1291 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6A the reference values for R should be scaled (multiplied by 2.0). The throughput on each cell should be Reference value R. |  |  |  |

Table 9.2.1FB.11: Test Parameters for Testing QPSK FRCs H-Set 3A

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FB.12: Test requirement QPSK, Fixed Reference Channel (FRC) H-Set 3A

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 0.6 dB | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 65 | N/A |
|  |  | -2.9 | N/A | N/A |
| 2 | PB3 | -5.9 | 23 | N/A |
|  |  | -2.9 | 138 | N/A |
| 3 | VA30 | -5.9 | 22 | N/A |
|  |  | -2.9 | 142 | N/A |
| 4 | VA120 | -5.9 | 13 | N/A |
|  |  | -2.9 | 140 | N/A |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 1, For Fixed Reference Channel (FRC) H-Set 3A the reference values for R should be scaled (multiplied by 6.0). The throughput on each cell should be Reference value R times 3. *NOTE 2: For UE supporting enhanced performance requirement type 2 and condition = 10 dB this is tested using the Fixed Reference Channel (FRC) H-Set 6A. |  |  |  |  |

### 9.2.1FC	Single Link Performance - Enhanced Performance Requirements Type 2 - QPSK/16QAM, Fixed Reference Channel (FRC) H-Set 6B/3B

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined

- Message Contents are FFS.

- Test Tolerances are FFS.

- Call Setup procedure is FFS.

- Update of Annexure is FFS

#### 9.2.1FC.1	Definition and applicability

The receiver single link performance of the High Speed Physical Downlink Shared Channel (HS-DSCH) in different multi-path fading environments are determined by the information bit throughput R.

The requirements and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support 4C-HSDPA and HSDPA UE capability category 29.

#### 9.2.1FC.2	Minimum requirements

The performance requirements for a particular UE belonging to HS-DSCH category 29 are determined according to the relevant part of Table 9.2.3F.

During the Fixed Reference Channel (FRC) tests the behaviour of the Node-B emulator in response to the ACK/NACK signalling field of the HS-DPCCH is specified in Table 9.2.4.

The requirements are specified in terms of minimum information bit throughput R for the DL reference channels H-set 6B/3B specified in Annex C.8.1.6 and C.8.1.3 respectively, with the addition of the relevant parameters in Tables 9.2.1FC.1, 9.2.1FC.3 and 9.2.1FC.5 plus the downlink physical channel setup according to table E.5.1.

Using this configuration the throughput shall meet or exceed the minimum requirements specified in tables 9.2.1FC.2, 9.2.1FC.4 and 9.2.1FC.6.

Table 9.2.1FC.1: Test Parameters for Testing QPSK FRCs H-Set 6B

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Redundancy and constellation version coding sequence |  | {0,2,5,6} |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |

Table 9.2.1FC.2: Minimum requirement Enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 6B

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 1494 |
|  |  | -3 | 2153 |
| 2 | PB3 | -6 | 1038 |
|  |  | -3 | 1744 |
| 3 | VA30 | -6 | 1142 |
|  |  | -3 | 1782 |
| 4 | VA120 | -6 | 909 |
|  |  | -3 | 1467 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6B the reference values for R should be scaled (multiplied by 3.0) |  |  |  |

Table 9.2.1FC.3: Test Parameters for Testing 16QAM FRCs H-Set 6B

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |  |
| Redundancy and constellation version coding sequence |  | {6,2,1,5} |  |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |  |

Table 9.2.1FC.4: Minimum requirement Enhanced requirement type 2 16QAM,
Fixed Reference Channel (FRC) H-Set 6B

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 991 |
|  |  | -3 | 1808 |
| 2 | PB3 | -6 | 465 |
|  |  | -3 | 1370 |
| 3 | VA30 | -6 | 587 |
|  |  | -3 | 1488 |
| 4 | VA120 | -6 | 386 |
|  |  | -3 | 1291 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6B the reference values for R should be scaled (multiplied by 3.0) |  |  |  |

Table 9.2.1FC.5: Test Parameters for Testing QPSK FRCs H-Set 3B

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Redundancy and constellation version coding sequence |  | {0,2,5,6} |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |

Table 9.2.1FC.6: Minimum requirement Enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 3B

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 0 dB | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 65 | N/A |
|  |  | -3 | N/A | N/A |
| 2 | PB3 | -6 | 23 | N/A |
|  |  | -3 | 138 | N/A |
| 3 | VA30 | -6 | 22 | N/A |
|  |  | -3 | 142 | N/A |
| 4 | VA120 | -6 | 13 | N/A |
|  |  | -3 | 140 | N/A |
| *NOTE 1: For Fixed Reference Channel (FRC) H-Set 3B the reference values for R should be scaled (multiplied by 9) *NOTE 2: For UE supporting enhanced performance requirement type 2 and condition = 10 dB this is tested using the Fixed Reference Channel (FRC) H-Set 6B. |  |  |  |  |

NOTE:	Table 9.2.1FC.6 is based on core requirements for minimum requirement as explained in Table 9.2.3C.

The reference for this requirement is TS 25.101 [1] clauses 9.2.1.1, 9.2.1.2, 9.2.1.4, and 9.2.1.5.

#### 9.2.1FC.3	Test purpose

To verify the ability of the receiver to receive a predefined test signal, representing a multi-path fading channel with information bit throughput R not falling below a specified value. The test stresses the multicode reception and channel decoding with incremental redundancy.

#### 9.2.1FC.4	Method of test

##### 9.2.1FC.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

1)	Connect the SS (node B emulator) and fader and AWGN noise source to the UE antenna connector as shown in figure A.35 for UEs that do not support receive diversity and for UEs supporting Type 3 or 3i.

NOTE: For a transition period of two meeting cycles until RAN#60, figure A.37 does not need to be used.

2)	Set up an HSDPA call according to TS 34.108 [3] clause FFS with levels according to table E.5.0.

3)	Set the node B emulator behaviour according to table 9.2.4. Set the test parameters for tests 1-4 according to table  9.2.1FC.1, 9.2.1FC.3 or 9.2.1FC.5 and levels according to tables  9.2.1FC.7 to 9.2.1FC.12 as appropriate for both the serving HS-DSCH cell and secondary serving HS-DSCH cell. The configuration of the downlink channels is defined in table E.5.1 for the  serving HS-DSCH cell, and for secondary serving HS-DSCH cell set up P-CPICH, HS-PDSCH and HS-SCCH channels only per table E.5.1.

4)	The information bit data shall be pseudo random and not repeated before 10 different information bit payload blocks are processed. (e.g. Fixed reference Channel Definition H-set 6 (16 QAM): The information bit payload block is 9377 bits long. Hence the PRBSequence must be at least 9377 * 10 bits long. ) Use a PRBS from ITU-T O.153 Ref [27]

5)	The SS shall not time the transmission freely. It shall time the transmission strictly according to the  reference measurement channels: i.e. Process number is continued exactly after 6 TTIs.

6)	Setup the fading simulator with fading conditions on each of the serving cells as described in  table D.2.2.1.A.

##### 9.2.1FC.4.2	Procedure

1)	Once the HSDPA connection is setup, change levels according to Tables  E.5.6 to E.5.8B as specified by  table  E.5.9 and start transmitting HSDPA Data on both the serving cells. For secondary serving HS-DSCH cell only P-CPICH, HS-PDSCH and HS-SCCH are configured.

2)	For all relevant propagation conditions, for all relevant Ioc levels, for all relevant Ec/Ior, for all relevant Îor/Ioc, for all relevant H-sets in  tables 9.2.1FC.7 to 9.2.1FC.12 count the number of  NACK, ACK and statDTX on the UL HS-DPCCH during the test interval on both the cells and  decide pass or fail according to Annex F.6.3  tables F.6.3.5.2.1, F.6.3.5.2.5B and F.6.3.5.2.6B. Throughput shall be measured per cell and compared to requirements in these tables.

#### 9.2.1FC.5	Test Requirements

Tables 9.2.1FC.7 to 9.2.1FC.12 define the primary level settings including test tolerance for all relevant throughput tests. The pass / fail decision for throughput is done according to  Annex F.6.3.

Tables  E.5.6 to E.5.8B define the secondary and subsequently ranked level settings including test tolerance to be applied on both the serving cells. As those level settings are not uniform for the throughput tests in this clause, Table  E.5.9 indicates which levels are applied, when the primary level settings (Ec/Ior and Ior/Ioc) and propagation conditions (PA3,PB3, VA30, VA 120) vary.

Table 9.2.1FC.7: Test Parameters for Testing QPSK FRCs H-Set 6B

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FC.8: Test requirement enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 6B

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 1494 |
|  |  | -2.9 | 2153 |
| 2 | PB3 | -5.9 | 1038 |
|  |  | -2.9 | 1744 |
| 3 | VA30 | -5.9 | 1142 |
|  |  | -2.9 | 1782 |
| 4 | VA120 | -5.9 | 909 |
|  |  | -2.9 | 1467 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6B the reference values for R should be scaled (multiplied by 3.0). The throughput on each cell should be Reference value R. |  |  |  |

Table 9.2.1FC.9: Test Parameters for Testing 16QAM FRCs H-Set 6B

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FC.10: Test requirement enhanced requirement type 2 16QAM,
Fixed Reference Channel (FRC) H-Set 6B

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 991 |
|  |  | -2.9 | 1808 |
| 2 | PB3 | -5.9 | 465 |
|  |  | -2.9 | 1370 |
| 3 | VA30 | -5.9 | 587 |
|  |  | -2.9 | 1488 |
| 4 | VA120 | -5.9 | 386 |
|  |  | -2.9 | 1291 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6B the reference values for R should be scaled (multiplied by 3.0). The throughput on each cell should be Reference value R. |  |  |  |

Table 9.2.1FC.11: Test Parameters for Testing QPSK FRCs H-Set 3B

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FC.12: Test requirement QPSK, Fixed Reference Channel (FRC) H-Set 3B

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 0.6 dB | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 65 | N/A |
|  |  | -2.9 | N/A | N/A |
| 2 | PB3 | -5.9 | 23 | N/A |
|  |  | -2.9 | 138 | N/A |
| 3 | VA30 | -5.9 | 22 | N/A |
|  |  | -2.9 | 142 | N/A |
| 4 | VA120 | -5.9 | 13 | N/A |
|  |  | -2.9 | 140 | N/A |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 1, For Fixed Reference Channel (FRC) H-Set 3B the reference values for R should be scaled (multiplied by 9.0). The throughput on each cell should be Reference value R times 9. |  |  |  |  |

### 9.2.1FD	Single Link Performance - Enhanced Performance Requirements Type 2 - QPSK/16QAM, Fixed Reference Channel (FRC) H-Set 6C/3C

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined

- Message Contents are FFS.

- Test Tolerances are FFS.

- Call Setup procedure is FFS.

- Update of Annexure is FFS

#### 9.2.1FD.1	Definition and applicability

The receiver single link performance of the High Speed Physical Downlink Shared Channel (HS-DSCH) in different multi-path fading environments are determined by the information bit throughput R.

The requirements and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support 4C-HSDPA and HSDPA UE capability category 31.

#### 9.2.1FD.2	Minimum requirements

The performance requirements for a particular UE belonging to HS-DSCH category 31 are determined according to the relevant part of Table 9.2.3F.

During the Fixed Reference Channel (FRC) tests the behaviour of the Node-B emulator in response to the ACK/NACK signalling field of the HS-DPCCH is specified in Table 9.2.4.

The requirements are specified in terms of minimum information bit throughput R for the DL reference channels H-set 6C/3C specified in Annex C.8.1.6 and C.8.1.3 respectively, with the addition of the relevant parameters in Tables 9.2.1FD.1, 9.2.1FD.3 and 9.2.1FD.5 plus the downlink physical channel setup according to table E.5.1.

Using this configuration the throughput shall meet or exceed the minimum requirements specified in tables 9.2.1FD.2, 9.2.1FD.4 and 9.2.1FD.6.

Table 9.2.1FD.1: Test Parameters for Testing QPSK FRCs H-Set 6C

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Redundancy and constellation version coding sequence |  | {0,2,5,6} |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |

Table 9.2.1FD.2: Minimum requirement Enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 6C

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 1494 |
|  |  | -3 | 2153 |
| 2 | PB3 | -6 | 1038 |
|  |  | -3 | 1744 |
| 3 | VA30 | -6 | 1142 |
|  |  | -3 | 1782 |
| 4 | VA120 | -6 | 909 |
|  |  | -3 | 1467 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6C the reference values for R should be scaled (multiplied by 4.0) |  |  |  |

Table 9.2.1FD.3: Test Parameters for Testing 16QAM FRCs H-Set 6C

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |  |
| Redundancy and constellation version coding sequence |  | {6,2,1,5} |  |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |  |

Table 9.2.1FD.4: Minimum requirement Enhanced requirement type 2 16QAM,
Fixed Reference Channel (FRC) H-Set 6C

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 991 |
|  |  | -3 | 1808 |
| 2 | PB3 | -6 | 465 |
|  |  | -3 | 1370 |
| 3 | VA30 | -6 | 587 |
|  |  | -3 | 1488 |
| 4 | VA120 | -6 | 386 |
|  |  | -3 | 1291 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6C the reference values for R should be scaled (multiplied by 4.0) |  |  |  |

Table 9.2.1FD.5: Test Parameters for Testing QPSK FRCs H-Set 3C

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Redundancy and constellation version coding sequence |  | {0,2,5,6} |  |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |  |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for those TTI intended for the UE. |  |  |  |  |  |

Table 9.2.1FD.6: Minimum requirement Enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 3C

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 0 dB | T-put (kbps) * = 10 dB |
| 1 | PA3 | -6 | 65 | N/A |
|  |  | -3 | N/A | N/A |
| 2 | PB3 | -6 | 23 | N/A |
|  |  | -3 | 138 | N/A |
| 3 | VA30 | -6 | 22 | N/A |
|  |  | -3 | 142 | N/A |
| 4 | VA120 | -6 | 13 | N/A |
|  |  | -3 | 140 | N/A |
| *NOTE 1: For Fixed Reference Channel (FRC) H-Set 3C the reference values for R should be scaled (multiplied by 12). *NOTE 2: For UE supporting enhanced performance requirement type 2 and condition = 10 dB this is tested using the Fixed Reference Channel (FRC) H-Set 6C. |  |  |  |  |

NOTE:	Table 9.2.1FD.6 is based on core requirements for minimum requirement as explained in Table 9.2.3C.

The reference for this requirement is TS 25.101 [1] clauses 9.2.1.1, 9.2.1.2, 9.2.1.4, and 9.2.1.5.

#### 9.2.1FD.3	Test purpose

To verify the ability of the receiver to receive a predefined test signal, representing a multi-path fading channel with information bit throughput R not falling below a specified value. The test stresses the multicode reception and channel decoding with incremental redundancy.

#### 9.2.1FD.4	Method of test

##### 9.2.1FD.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

1)	Connect the SS (node B emulator) and fader and AWGN noise source to the UE antenna connector as shown in figure A.35 for UEs that do not support receive diversity and for UEs supporting Type 3 or 3i.

NOTE: For a transition period of two meeting cycles until RAN#60, figure A.37 does not need to be used.

2)	Set up an HSDPA call according to TS 34.108 [3] clause FFS with levels according to table E.5.0.

3)	Set the node B emulator behaviour according to table 9.2.4. Set the test parameters for tests 1-4 according to table  9.2.1FD.1, 9.2.1FD.3 or 9.2.1FD.5 and levels according to tables  9.2.1FD.7 to 9.2.1FD.12 as appropriate for both the serving HS-DSCH cell and secondary serving HS-DSCH cell. The configuration of the downlink channels is defined in table E.5.1 for the  serving HS-DSCH cell, and for secondary serving HS-DSCH cell set up P-CPICH, HS-PDSCH and HS-SCCH channels only per table E.5.1.

4)	The information bit data shall be pseudo random and not repeated before 10 different information bit payload blocks are processed. (e.g. Fixed reference Channel Definition H-set 6 (16 QAM): The information bit payload block is 9377 bits long. Hence the PRBSequence must be at least 9377 * 10 bits long. ) Use a PRBS from ITU-T O.153 Ref [27].

5)	The SS shall not time the transmission freely. It shall time the transmission strictly according to the  reference measurement channels: i.e. Process number is continued exactly after 6 TTIs.

6)	Setup the fading simulator with fading conditions on each of the serving cells as described in  table D.2.2.1.A.

##### 9.2.1FD.4.2	Procedure

1)	Once the HSDPA connection is setup, change levels according to Tables  E.5.6 to E.5.8B as specified by table E.5.9 and start transmitting HSDPA Data on both the serving cells. For secondary serving HS-DSCH cell only P-CPICH, HS-PDSCH and HS-SCCH are configured.

2)	For all relevant propagation conditions, for all relevant Ioc levels, for all relevant Ec/Ior, for all relevant Îor/Ioc, for all relevant H-sets in  tables 9.2.1FD.7 to 9.2.1FD.12 count the number of  NACK, ACK and statDTX on the UL HS-DPCCH during the test interval on both the cells and  decide pass or fail according to Annex F.6.3  tables F.6.3.5.2.1, F.6.3.5.2.5B and F.6.3.5.2.6C. Throughput shall be measured per cell and compared to requirements in these tables.

#### 9.2.1FD.5	Test Requirements

Tables 9.2.1FD.7 to 9.2.1FD.12 define the primary level settings including test tolerance for all relevant throughput tests. The pass / fail decision for throughput is done according to  Annex F.6.3.

Tables  E.5.6 to E.5.8B define the secondary and subsequently ranked level settings including test tolerance to be applied on both the serving cells. As those level settings are not uniform for the throughput tests in this clause, Table  E.5.9 indicates which levels are applied, when the primary level settings (Ec/Ior and Ior/Ioc) and propagation conditions (PA3,PB3, VA30, VA 120) vary.

Table 9.2.1FD.7: Test Parameters for Testing QPSK FRCs H-Set 6C

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FD.8: Test requirement enhanced requirement type 2 QPSK,
Fixed Reference Channel (FRC) H-Set 6C

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 1494 |
|  |  | -2.9 | 2153 |
| 2 | PB3 | -5.9 | 1038 |
|  |  | -2.9 | 1744 |
| 3 | VA30 | -5.9 | 1142 |
|  |  | -2.9 | 1782 |
| 4 | VA120 | -5.9 | 909 |
|  |  | -2.9 | 1467 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6C the reference values for R should be scaled (multiplied by 4.0). The throughput on each cell should be Reference value R. |  |  |  |

Table 9.2.1FD.9: Test Parameters for Testing 16QAM FRCs H-Set 6C

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FD.10: Test requirement enhanced requirement type 2 16QAM,
Fixed Reference Channel (FRC) H-Set 6C

| Test Number | Propagation Conditions | Reference value |  |
| --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 991 |
|  |  | -2.9 | 1808 |
| 2 | PB3 | -5.9 | 465 |
|  |  | -2.9 | 1370 |
| 3 | VA30 | -5.9 | 587 |
|  |  | -2.9 | 1488 |
| 4 | VA120 | -5.9 | 386 |
|  |  | -2.9 | 1291 |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 6 *NOTE 2: For Fixed Reference Channel (FRC) H-Set 6C the reference values for R should be scaled (multiplied by 4.0). The throughput on each cell should be Reference value R. |  |  |  |

Table 9.2.1FD.11: Test Parameters for Testing QPSK FRCs H-Set 3C

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |  |
|  | dBm/3.84 MHz | -60 (no test tolerance applied) |  |  |  |

Table 9.2.1FD.12: Test requirement QPSK, Fixed Reference Channel (FRC) H-Set 3C

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | T-put (kbps) * = 0.6 dB | T-put (kbps) * = 10.6 dB |
| 1 | PA3 | -5.9 | 65 | N/A |
|  |  | -2.9 | N/A | N/A |
| 2 | PB3 | -5.9 | 23 | N/A |
|  |  | -2.9 | 138 | N/A |
| 3 | VA30 | -5.9 | 22 | N/A |
|  |  | -2.9 | 142 | N/A |
| 4 | VA120 | -5.9 | 13 | N/A |
|  |  | -2.9 | 140 | N/A |
| *NOTE 1: The reference value R is for the Fixed Reference Channel (FRC) H-Set 1, For Fixed Reference Channel (FRC) H-Set 3C the reference values for R should be scaled (multiplied by 12.0). The throughput on each cell should be Reference value R times 12. |  |  |  |  |

[End of modifications]