3GPP TSG-RAN WG5 Meeting #69	R5-155187

# Anaheim, USA, 16 - 20 November 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | 3804 | rev | - | Current version: | 12.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Test Case 8.2.1.48 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN5 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_DCH_Enh-UEConTest |  |  |  |  |  |  | Date: |  | 2015-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To add new DCH Enhancements Test Case 8.2.1.48 per Test Plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of new test case: |  |  |  |  |  |  |  |  |
|  |  | 8.2.1.48 Radio Bearer Establishment for transition between CELL_DCH and CELL_DCH with SRBs mapped on DCH: Successful Activation of DCH Enhancements in Full Mode |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Test Coverage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.48 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ... TS 34.123-2 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 8.2.1.48	Radio Bearer Establishment for transition between CELL_DCH and CELL_DCH with SRBs mapped on DCH: Successful Activation of DCH Enhancements in Full Mode

8.2.1.48.1	Definition and applicability

All UEs which supports DCH Enhancements in Full Mode.

UE supports FDD.

8.2.1.48.2	Conformance requirement

The UE shall:

1>	be able to receive any of the following messages:

2>	RADIO BEARER SETUP message; or

….

….

If the UE receives:

-	a RADIO BEARER SETUP message; or

….

it shall:

1>	stop and reset timer T324 if running;

1>	stop monitoring target cell HS-SCCH;

1>	set the variable ORDERED_RECONFIGURATION to TRUE;

….

In case the UE receives a RADIO BEARER SETUP message with the IE "Specification mode" not set to "Complete Specification", the UE behaviour is unspecified.

The UE shall:

….

For FDD: in CELL_DCH state, in case the UE receives a Target cell HS-SCCH order the UE shall:

1>	ignore RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, TRANSPORT CHANNEL RECONFIGURATION or PHYSICAL CHANNEL RECONFIGURATION messages as described in sub clause 8.6.3.11.

If after state transition the UE enters CELL_DCH state, the UE shall, after the state transition:

1>	in FDD; or

….

2>	remove any C-RNTI from MAC;

2>	clear the variable C_RNTI.

1>	determine the value for the READY_FOR_COMMON_EDCH variable and take the corresponding actions as described in sub clause 8.5.47;

1>	determine the value for the COMMON_E_DCH_TRANSMISSION variable and take the corresponding actions as described in sub clause 8.5.46;

1>	for FDD, determine the value for the READY_FOR_COMMON_ERGCH variable and take the corresponding actions as described in sub clause 8.5.75;

1>	if the IE "E-DCH info" was not included in the message:

2>	for FDD:

3>	stop any E-AGCH, E-RGCH and E-HICH reception procedures;

3>	stop any E-DPCCH and E-DPDCH transmission procedures;

3>	act as if the IE "MAC-es/e reset indicator" was received and set to TRUE;

3>	release all E-DCH HARQ resources;

3>	no longer consider any radio link to be the serving E-DCH radio link.

….

….

If the UE was in CELL_DCH state upon reception of the reconfiguration message and remains in CELL_DCH state, the UE shall:

1>	if the IE "Uplink DPCH Info" is absent, not change its current UL Physical channel configuration;

…..

1>	if "DPCH frame offset" is included for one or more RLs in the active set, and the reconfiguration procedure does not request a timing reinitialized hard handover (see subclause 8.3.5.1):

2>	use its value to determine the beginning of the DPCH or F-DPCH frame in accordance with the following:

3>	if the received IE "DPCH frame offset" is across the value range border compared to the DPCH or F-DPCH frame offset currently used by the UE:

4>	consider it to be a request to adjust the timing with 256 chips across the frame border (e.g. if the UE receives value 0 while the value currently used is 38144 consider this as a request to adjust the timing with +256 chips).

3>	if after taking into account value range borders, the received IE "DPCH frame offset" corresponds to a request to adjust the timing with a step exceeding 256 chips:

4>	set the variable INVALID_CONFIGURATION to TRUE.

3>	and the procedure ends.

2>	adjust the radio link timing accordingly.

….

The UE shall transmit a response message as specified in sub clause 8.2.2.4, setting the information elements as specified below. The UE shall:

….

1>	if IE "RRC state indicator" is set to "CELL_DCH":

2>	if the variable DEFERRED_MEASUREMENT_STATUS is set to TRUE:

3>		if System Information Block type 11 is scheduled on the BCCH and the UE has not read nor stored the IEs present in this System Information Block, or

3>	if System Information Block type 11bis is scheduled on the BCCH and the UE has not read nor stored the IEs present in this System Information Block, or

3>	if System Information Block type 11ter is scheduled on the BCCH and the UE has not read nor stored the IEs present in this System Information Block, or

3>	if System Information Block type 12 is scheduled on the BCCH and the UE has not read nor stored the IEs present in this System Information Block:

4>	include IE "Deferred measurement control reading".

If after state transition the UE enters CELL_DCH state, the UE shall, after the state transition:

1>	if the IE "Deferred measurement control reading" was included in the response message:

2>	clear the variable CELL_INFO_LIST.

….

….

In case the procedure was triggered by reception of a RADIO BEARER SETUP message, the UE shall:

1>	transmit a RADIO BEARER SETUP COMPLETE as response message on the uplink DCCH using AM RLC.

….

….

The UE shall:

1>	set the variable ORDERED_RECONFIGURATION to FALSE;

1>	if the received reconfiguration message contained the IE "Ciphering mode info":

2>	resume data transmission on any suspended radio bearer and signalling radio bearer mapped on RLC-AM or RLC-UM;

2>	set the IE "Reconfiguration" in the variable CIPHERING_STATUS to FALSE; and

2>	clear the variable RB_UPLINK_CIPHERING_ACTIVATION_TIME_INFO.

1>	if the received reconfiguration message contained the IE "Integrity protection mode info":

2>	allow the transmission of RRC messages on all signalling radio bearers with any RRC SN;

2>	set "Uplink RRC Message sequence number" for signalling radio bearer RB0 in the variable INTEGRITY_PROTECTION_INFO to a value such that next RRC message to be sent on uplink RB0 will use the new integrity protection configuration;

2>	set the IE "Reconfiguration" in the variable INTEGRITY_PROTECTION_INFO to FALSE; and

2>	clear the variable INTEGRITY_PROTECTION_ACTIVATION_INFO.

1>	clear the variable PDCP_SN_INFO;

1>	clear the variable START_VALUE_TO_TRANSMIT;

1>	clear the variable SECURITY_MODIFICATION.

….

Reference

3GPP TS 25.331 clause 8.2.2.3, 8.2.2.4

8.2.1.48.3	Test purpose

To confirm that the UE establishes radio bearer, mapped to DCH, transits from CELL_DCH to CELL_DCH state, and starts DCH Enhancements in Full Mode according to the received RADIO BEARER SETUP message.

8.2.1.48.4	Method of test

Initial Condition

System Simulator: 1 cell –Cell 1

UE: CS-DCCH+DTCH_DCH (state 6-5) in cell 1 as specified in clause 7.4 of TS 34.108 supported by the UE. [9].

Related ICS/IXIT statement(s)

-	UE supports FDD

-	UE category supports DCH Enhancements in Full Mode

Test Procedure

The UE is in CELL_DCH state. Before step 1, only signalling radio bearers have been established. The SS transmits a RADIO BEARER SETUP message to the UE. This message includes RAB information to configure CS domain RB on DCH (with DCH Enhancements configured for L1). SRB is mapped to DCH/DCH. After the UE receives this message, it configures RAB accordingly and transmits a RADIO BEARER SETUP COMPLETE message using AM RLC.

SS calls for generic procedure C.3 to check that UE is in CELL_DCH state.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
|  |  |  |  | SS configures one cell. |
| 1 |  |  | RADIO BEARER SETUP | Configuration of CS RB on DCH, using DCH Enhancements configuration |
| 2 |  |  | RADIO BEARER SETUP COMPLETE |  |
| 3 |  |  | CALL C.3 | If the test result of C.3 indicates that UE is in CELL_DCH state, the test passes, otherwise it fails. |

Specific Message Contents

RADIO BEARER SETUP (Step 1)

The contents of RADIO BEARER SETUP message in this test case is identical to the message sub-type titled as "Speech in CS" – Condition A2,  as found in clause 9 of TS 34.108, with the following exceptions:

| Information Element | Condition |  | Value/remark |
| --- | --- | --- | --- |
| Added or Reconfigured UL TrCH information |  |  | Not Present |
| CHOICE mode |  |  | Not Present |
| DL Transport channel information common for all transport channel |  |  |  |
| Deleted DL TrCH information |  |  | Not Present |
| Added or Reconfigured DL TrCH information |  |  | Not Present |
| Frequency info |  |  |  |
| DCH Enhancements Info FDD |  |  |  |
| CHOICE Configuration info |  |  |  |
| Continue |  |  | Not Present |
| New Configuration |  |  |  |
| CHOICE UL Transmission Mode |  |  |  |
| 10 ms only |  |  | Not Present |
| 20 ms only |  |  | Not Present |
| 10ms or 20ms |  |  |  |
| Uplink Transmission Mode switching parameters |  |  |  |
| K |  |  | 5 |
| L |  |  | 10 |
| M |  |  | 10 |
| CHOICE DL FET Mode |  |  |  |
| Basic |  |  | Not Present |
| Full |  |  |  |
| Early DCH quality target |  |  |  |
| BLER Quality value |  |  | -1 |
| Early DCH quality target slot |  |  | 14 |
| TrCh Concatenation Info |  |  |  |
| DCH ID |  |  | 6 |
| TrCh Concatenation Info |  |  |  |
| DCH ID |  |  | 7 |
| TrCh Concatenation Info |  |  |  |
| DCH ID |  |  | 8 |
| Maximum allowed UL TX power |  |  | Not Present |
| CHOICE channel requirement |  |  | Uplink DPCH info |
| - Scrambling code |  |  | 1 |
| Downlink information common for all radio links |  |  |  |
| - Downlink DPCH info common for all RL |  |  |  |
| CHOICE cfnhandling |  |  |  |
| Maintain |  |  |  |
| timingmaintainedsyncind |  | FALSE |  |
| Initialize |  | NULL |  |
| - Downlink DPCH power control information |  |  |  |
| - CHOICE mode |  | FDD |  |
| - DPC mode |  | 0 (single TPC) |  |
| - Power offset PPilot-DPDCH |  | 0 |  |
| - DL rate matching restriction information |  | Not Present |  |
| - Spreading factor |  | Reference to clause 6.10 Parameter Set |  |
| - Fixed or Flexible Position |  | Reference to clause 6.10 Parameter Set |  |
| - TFCI existence |  | FALSE |  |
| CHOICE SF |  |  |  |
| SF=128 |  |  |  |
| Number of bits for pilot bits |  | 0 |  |
| Downlink information per radio link list |  |  |  |
| -Downlink information for each radio link |  |  |  |
| - Primary CPICH info |  |  |  |
| - Primary scrambling code |  | Set to same code as used for cell 1 |  |
| -Downlink DPCH info for each RL |  |  |  |
| --Power offset PTPC-DPDCH |  | 3dB |  |
| --Power offset POSRB |  | 3dB |  |
| MBMS PL Service Restriction Information |  | Not Present |  |

8.2.1.48.5	Test requirements

At step 2 the UE shall transmit a RADIO BEARER SETUP COMPLETE message.