3GPP TSG-RAN WG5 Meeting #69 	R5-155250

# Anaheim, USA, 16th – 20th Nov 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 2178 | rev | - | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to test case 6.3.5A.1.2 Power Control Absolute Power Tolerance for CA (inter-band DL CA and UL CA). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tejet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel12_2UL-UEConTest |  |  |  |  |  |  | Date: |  | 2015-10-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | -Currently, initial conditons,test description,test procedure and test requirement of TC 6.3.5A.1.2 Power Control Absolute Power Tolerance for CA are still missing. - Annex F for 2UL CA test cases is still missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Initial conditons,test description,test procedure and test requirement are added. - Annex F for 2UL CA test cases is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI would not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5A.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

6.3.5A.1.2	Power Control Absolute power tolerance for CA (inter-band DL CA and UL CA)

Editor's notes: This test is incomplete.

-The message contents are not yet defined

6.3.5A.1.2.1	Test purpose

To verify the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on each active component carriers larger than 20ms.

6.3.5A.1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

6.3.5A.1.2.3	Minimum conformance requirements

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on each active component carriers larger than 20ms. The requirement can be tested by time aligning any transmission gaps on the component carriers.

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, the absolute power control tolerance is specified on each component carrier exceed the minimum output power as defined in subclause 6.3.2A and the total power is limited by maximum output as defined in subclause 6.2.2A. The requirements defined in Table 6.3.5A.1.2.3-1 shall apply on each component carrier with both component carriers active. The requirements can be tested by time aligning any transmission gaps on both the component carriers.

The requirements apply for one single PUCCH, PUSCH or SRS transmission of contiguous PRB allocation per component carrier.

Table 6.3.5A.1.2.3-1: Absolute power tolerance for inter-band DL CA and UL CA

| Conditions | Tolerance |
| --- | --- |
| Normal conditions | ± 9.0 dB |
| Extreme conditions | ± 12.0 dB |

The normative reference for this requirement is TS 36.101 [2] clause 6.3.5A.1.

6.3.5A.1.2.4	Test description

6.3.5A.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.3.5A.1.2.4.1-1. The details of the uplink reference measurement channel (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.5A.1.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , only the first of those is tested, according to the order on the Test Configuration Table list. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3.5A.1.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.3.5A.1.2.4.3. Any PDCCH DCI format 0 sent to the UE during the configuration should have TPC command 0dB.

6.3.5A.1.2.4.2	Test procedure

Same test procdure as in clause 6.3.5A.1.1.4.2.

-	Instead of Table 6.3.5A.1.1.4.1-1 use Table 6.3.5A.1.2.4.1-1.

-	Instead of clause 6.3.5A.1.1.4.3 use clause 6.3.5A.1.2.4.3.

6.3.5A.1.2.4.3	Message contents

TBD

6.3.5A.1.2.5	Test requirement

The absolute power control tolerance per component carrier measured in step (5) of the test procedure is not to exceed the values specified in Table 6.3.5A.1.2.5-1 and 6.3.5A.1.2.5-2. The test requirement tables are originated from tables 6.3.5.1.5-1 and 6.3.5.1.5-2.

Table 6.3.5A.1.2.5-1: Absolute power tolerance for inter-band CA: test point 1

|  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Expected Measured power Normal conditions | -14.8 dBm | -10.8 dBm | -8.6 dBm | -5.6 dBm | -3.9 dBm | -2.6 dBm |
| Power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB |
| Expected Measured power Extreme conditions | -14.8 dBm | -10.8 dBm | -8.6 dBm | -5.6 dBm | -3.9 dBm | -2.6 dBm |
| Power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.3 |  |  |  |  |  |  |

Table 6.3.5A.1.2.5-2: Absolute power tolerance for inter-band CA: test point 2

|  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Expected Measured power Normal conditions | -2.8 dBm | 1.2 dBm | 3.4 dBm | 6.4 dBm | 8.2 dBm | 9.4 dBm |
| Power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB |
| Expected Measured power Extreme conditions | -2.8 dBm | 1.2 dBm | 3.4 dBm | 6.4 dBm | 8.2 dBm | 9.4 dBm |
| Power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.3 |  |  |  |  |  |  |