3GPP TSG-RAN5 Meeting #71	R5-162166

# Nanjing, China, 23rd May 2016 - 27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 2532 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 6.2.4A.2:AMPR for 2UL inter-band CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel12_2UL-UEConTest |  |  |  |  |  | Date: |  |  | 2016-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - The test case AMPR for 2UL inter-band CA is not yet complete and the test description need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Deleted the editor’s note - Updated test configuration tables, Message contents and test requirements. - Added test case AMPR for 2UL inter-band CA into F.1.2 and F.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will remain incompelete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4A.2, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.521-2 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### <Unchanged sections skipped>

#### 6.2.4A.2	Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA)

##### 6.2.4A.2.1	Test purpose

Additional ACLR, spectrum emission and spurious emission requirements for carrier aggregation can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the CA Power Class as specified in Table 6.2.2A.2.3-1.

##### 6.2.4A.2.2	Test applicability

The requirements of this test apply in test case 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for network signalled value NS_03/NS_03, NS_03/NS_06, NS_03/NS_07, NS_03/NS_01, NS_01/NS_06 to all types of E-UTRA UE release 11 and forward.

The requirements of this test apply in test case 6.6.3.3A.2 Additional spurious emissions for CA (inter-band DL CA and UL CA) for network signalled values NS_05/NS_01, NS_03/NS_07, NS_01/NS_12 and NS_01/NS15 to all types of E-UTRA UE release 11 and forward.

NOTE:	As a result TC 6.2.4A.2 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

##### 6.2.4A.2.3	Minimum conformance requirements

If for inter-band carrier aggregation the UE is configured for transmissions on a single serving cell, then subclauses 6.2.3 and 6.2 4 apply with the Network Signalling value indicated by the field additionalSpectrumEmission.

For inter-band carrier aggregation with the UE configured for transmissions on two serving cells the maximum output power reduction specified in Table 6.2.4-1 is allowed for each serving cell of the applicable uplink CA configuration according to the Network Signalling value indicated by the field additionalSprectrumEmission for the PCC and the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10 for the SCC. The value of additionalSpectrumEmissionSCell-r10 is equal to that of additionalSprectrumEmission configured on the SCC. MPR as specified in subclause 6.2.3A is allowed in addition.

For PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.The normative reference for this requirement is in TS 36.101 [2] clause 6.2.4A.

##### 6.2.4A.2.4	Test description

###### 6.2.4A.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2.4A.2.4.1-1, the power limits specified in subclause 6.2.5A.2.3 apply.

Table 6.2.4A.2.4.1-1 Additional Maximum Power Reduction (A-MPR) for inter-band CA

| Uplink CA Configuration | NS-value per CC (in same order as UL CA Configuration column) |  | Applicable emissions requirements (in same order as UL CA Configuration column) |  |
| --- | --- | --- | --- | --- |
| CA_1A-5A CA_1A-7A CA_1A-8A | NS_05 | NS_01 | Table 6.6.3.3.5.1-1 | N/A |
| CA_2A-4A | NS_03 | NS_03 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_2A-13A, CA_4A-12A, CA_4A-13A, CA_4A-17A | NS_03 | NS_06 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-13A, CA_4A-13A | NS_03 | NS_07 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 Table 6.6.3.3.3.2-1 |
| CA_3A-26A | NS_01 | NS_12 | N/A | Table 6.6.3.3.3.5-1 |
| CA_3A-26A | NS_01 | NS_15 | N/A | Table 6.6.3.3.3.8-1 |
| CA_4A-7A | NS_03 | NS_01 | Table 6.6.2.2.3.1-1 | N/A |
| CA_5A-12A | NS_01 | NS_06 | N/A | Table 6.6.2.2.3.3-1 |
| Note 1: For CCs with NS_01, there are no additional requirements defined in 36.101. Only CCs with NS-value different than NS_01 needs to be tested. Note 2: A-MPR values for NS-values in this table are the same as in section 6.2.4.3. Note 3: Selection of CA configurations and NS-values to test is derived in TR 36.905 [19] Note 4: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.2.4A.2.4.1-2 to 6.2.4A.1.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4A.2.4.1-2: Test Configuration Table (network signalled value NS_05/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low range, Mid range for PCC In case of Low range: - For 5MHz Channel Bandwidth: 1927.2MHz (NUL = 18072) - For 10 MHz Channel Bandwidth: 1934.7 MHz (NUL = 18147) - For 20 MHz Channel Bandwidth: 1930 MHz (NUL = 18100) Mid range for SCC(Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_05) | SCC (NS_01) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 15 | N/A for this test | QPSK | QPSK | 6 | 4 |
| 2 | 25 | 15 |  | 16QAM | 16QAM | 25 | 6 |
| 3 | 25 | 15 |  | 16QAM | QPSK | 25 | 4 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 7 | 25 | 50 |  | QPSK | QPSK | 6 | 15 |
| 8 | 25 | 50 |  | 16QAM | 16QAM | 25 | 25 |
| 9 | 25 | 50 |  | 16QAM | QPSK | 25 | 15 |
| 10 | 50 | 50 |  | QPSK | QPSK | 15 | 15 |
| 11 | 50 | 50 |  | 16QAM | 16QAM | 50 | 25 |
| 12 | 50 | 50 |  | 16QAM | QPSK | 50 | 15 |
| 13 | 100 | 50 |  | QPSK | QPSK | 18 | 15 |
| 14 | 100 | 50 |  | 16QAM | 16QAM | 50 | 25 |
| 15 | 100 | 50 |  | 16QAM | QPSK | 50 | 15 |
| 16 | 100 | 100 |  | QPSK | QPSK | 18 | 18 |
| 17 | 100 | 100 |  | 16QAM | 16QAM | 50 | 25 |
| 18 | 100 | 100 |  | 16QAM | QPSK | 50 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg . only the first of those is tested. according to the order on the Test Configuration Table list. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-3: Test Configuration Table (network signalled value NS_03/NS_03)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_03) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 6 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 6 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 7 | 50 | 50 |  | QPSK | QPSK | 6 | 6 |
| 8 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 9 | 50 | 50 |  | 16QAM | QPSK | 50 | 6 |
| 10 | 100 | 100 |  | QPSK | QPSK | 10 | 10 |
| 11 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 12 | 100 | 100 |  | 16QAM | QPSK | 100 | 10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg . only the first of those is tested. according to the order on the Test Configuration Table list. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-4: Test Configuration Table (network signalled value NS_03/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 8 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 8 |
| 4 | 25 | 15 |  | QPSK | QPSK | 6 | 4 |
| 5 | 25 | 15 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 15 |  | 16QAM | QPSK | 25 | 4 |
| 7 | 25 | 25 |  | QPSK | QPSK | 6 | 8 |
| 8 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 9 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 10 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 11 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 12 | 25 | 50 |  | 16QAM | QPSK | 25 | 12 |
| 13 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 14 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 15 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 16 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 17 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 18 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg . only the first of those is tested. according to the order on the Test Configuration Table list. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-5: Test Configuration Table (network signalled value NS_03/NS_07)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 50 | N/A for this test | QPSK | QPSK | 6 | 4 |
| 2 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 3 | 25 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 4 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 7 | 100 | 50 |  | QPSK | QPSK | 10 | 4 |
| 8 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 9 | 100 | 50 |  | QPSK | 16QAM | 10 | 50 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg . only the first of those is tested. according to the order on the Test Configuration Table list. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-6: Test Configuration Table (network signalled value NS_01/NS_12)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for PCC SCC: (Note 3) For 5 MHz Channel Bandwidth: UL 816.7 MHz (NUL = 26717) For 10 MHz Channel Bandwidth: UL 819.2 MHz (NUL = 26742) For 15 MHz Channel Bandwidth: UL 821.7 MHz (NUL = 26767) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_12) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations (LCRB @ RBstart)(Note 4) |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8@7 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 10 | 8 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 8 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12@16 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 15 | 18 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 18 |
| 7 | 100 | 75 |  | QPSK | QPSK | 15 | 31 |
| 8 | 100 | 75 |  | 16QAM | 16QAM | 20 | 30 |
| 9 | 100 | 75 |  | QPSK | 16QAM | 15 | 30 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg . only the first of those is tested. according to the order on the Test Configuration Table list. Note 3: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1 Note 4: Test point with max A-MPR in B26 selected in region A with largest allocation giving max A-MPR at RBstart0; test point with min A-MPR in B26 selected in region A with same allocation but with shifted RBstart to give A-MPR=0; |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-7: Test Configuration Table (network signalled value NS_01/NS_15)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | High range for both PCC and SCC(Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_15) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations (LCRB @ RBstart)(Note 4) |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 6@14 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 10 | 10@15 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 10@15 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12@25 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 15 | 15@35 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 15@35 |
| 7 | 100 | 75 |  | QPSK | QPSK | 18 | 18@19 |
| 8 | 100 | 75 |  | 16QAM | 16QAM | 20 | 20@55 |
| 9 | 100 | 75 |  | QPSK | 16QAM | 18 | 20@55 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg . only the first of those is tested. according to the order on the Test Configuration Table list. Note 3: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1 Note 4: Test point with max A-MPR in B26 selected in region C with allocation giving max MPR+A-MPR+deltaTc at max RBend; Test point with min A-MPR in B26 selected in region B with max allocation and RBend giving A-MPR=0 |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-8: Test Configuration Table (network signalled value NS_03/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and high range for PCC Mid range for SCC(Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_01) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 4 | 50 | 50 |  | QPSK | QPSK | 15 | 15 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 50 | 50 |  | 16QAM | QPSK | 25 | 15 |
| 7 | 100 | 100 |  | QPSK | QPSK | 18 | 18 |
| 8 | 100 | 100 |  | 16QAM | 16QAM | 25 | 25 |
| 9 | 100 | 100 |  | 16QAM | QPSK | 25 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg . only the first of those is tested. according to the order on the Test Configuration Table list. Note 3: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1 |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-9: Test Configuration Table (network signalled value NS_01/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 15 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 15 | 15 |  | 16QAM | 16QAM | 25 | 9 |
| 3 | 15 | 15 |  | QPSK | 16QAM | 6 | 9 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 15 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 15 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to the applicable table from Table 6.2.4A.2.4.1-2 to 6.2.4A.2.4.1-9.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4A.2.4.3.

###### 6.2.4A.2.4.2	Test procedure

1.	 Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.2.4A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to the applicable from Table 6.2.4A.2.4.1-2 to Table 6.2.4A.2.4.1-9 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

6.	Measure the mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

###### 6.2.4A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.The following exceptions apply for each network signalled value.

6.2.4A.2.4.3.1	Message contents exceptions (network signalled value " NS_03" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_03 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

6.2.4A.2.4.3.2	Message contents exceptions (network signalled value " NS_03" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_03 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.2-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_03 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.2-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_03" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 3 (NS_03) |  |  |

6.2.4A.2.4.3.3	Message contents exceptions (network signalled value " NS_06" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_06 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.3-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_06" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 6 (NS_06) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_06 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.3-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_06" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 6 (NS_06) |  |  |

6.2.4A.2.4.3.4	Message contents exceptions (network signalled value " NS_07" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_07 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.4-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_07" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 7 (NS_07) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_07 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.4-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_07" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 7 (NS_07) |  |  |

6.2.4A.2.4.3.5	Message contents exceptions (network signalled value " NS_05" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_05 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.5-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_05" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_05) |  |  |

6.2.4A.2.4.3.6	Message contents exceptions (network signalled value " NS_12" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_12 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.6-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_12" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 12 (NS_12) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_12 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.6-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_12" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 12 (NS_12) |  |  |

6.2.4A.2.4.3.7	Message contents exceptions (network signalled value " NS_15" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_15 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.7-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_15" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 15 (NS_15) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_15 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.7-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_15" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 15 (NS_15) |  |  |

##### 6.2.4A.2.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4A.2.5-1 to Table 6.2.4A.2.5-8. For the UE maximum output power modified by A-MPR specified in sub-clause 6.2.4A.2.3, the power limits specified in Table 6.2.5A.3.3-1 apply.

Table 6.2.4A.2.5-1: Test requirement (network signalled value NS_05/NS_01)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 13 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 14 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 15 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 16 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 17 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 18 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-2: Test requirement (network signalled value NS_03/NS_03)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 1 | 1.5 | 0 | 18.5 | 20 | 22.3 | 5 | 25.7 | 16.6 |
| 3 | 2 | 0 | 1 | 0 | 1.5 | 0 | 18.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 1 | 1.5 | 0 | 18.5 | 20 | 22.3 | 5 | 25.7 | 16.6 |
| 6 | 2 | 0 | 1 | 0 | 1.5 | 0 | 18.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-3: Test requirement (network signalled value NS_03/NS_06)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 0 | 1.5 | 0 | 18.5 | 21 | 22.9 | 5 | 25.7 | 17.2 |
| 3 | 2 | 0 | 1 | 0 | 1.5 | 1.5 | 18.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 13 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 14 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 15 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 16 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 17 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 18 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-4: Test requirement (network signalled value NS_03/NS_07)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| CA_2A-13A |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 3 | 0 | 2 | 0 | 12 | 1.5 | 0 | 21.5 | 9 | 21.7 | 5 | 25.7 | 16.0 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 6 | 0 | 2 | 0 | 12 | 1.5 | 0 | 21.5 | 9 | 21.7 | 5 | 25.7 | 16.0 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 9 | 0 | 2 | 0 | 12 | 1.5 | 0 | 21.5 | 9 | 21.7 | 5 | 25.7 | 16.0 |
| CA_4A-13A |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 3 | 0 | 2 | 0 | 12 | 0 | 0 | 23 | 9 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 6 | 0 | 2 | 0 | 12 | 0 | 0 | 23 | 9 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 9 | 0 | 2 | 0 | 12 | 0 | 0 | 23 | 9 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-5: Test requirement (network signalled value NS_01/NS_12)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| CA_2A-13A |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 1 | 0 | 5 | 1.5 | 1.5 | 19.5 | 15.5 | 21.0 | 5 | 25.7 | 15.3 |
| 3 | 0 | 1 | 0 | 5 | 1.5 | 1.5 | 21.5 | 15.5 | 22.5 | 5 | 25.7 | 16.8 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 4 | 1.5 | 1.5 | 19.5 | 15.5 | 21.0 | 5 | 25.7 | 15.3 |
| 6 | 0 | 2 | 0 | 4 | 1.5 | 1.5 | 21.5 | 15.5 | 22.5 | 5 | 25.7 | 16.8 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 0 | 4 | 0 | 0 | 21 | 17 | 22.5 | 5 | 25.7 | 16.8 |
| 9 | 0 | 2 | 0 | 4 | 1.5 | 0 | 21.5 | 17 | 22.8 | 5 | 25.7 | 17.1 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-6: Test requirement (network signalled value NS_01/NS_15)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| CA_2A-13A |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 9 | 1.5 | 1.5 | 19.5 | 10.5 | 20.0 | 6 | 25.7 | 13.3 |
| 3 | 0 | 2 | 0 | 9 | 1.5 | 1.5 | 21.5 | 10.5 | 21.8 | 5 | 25.7 | 16.1 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 9 | 1.5 | 1.5 | 19.5 | 10.5 | 20.0 | 6 | 25.7 | 13.3 |
| 6 | 0 | 2 | 0 | 9 | 1.5 | 1.5 | 21.5 | 10.5 | 21.8 | 5 | 25.7 | 16.1 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 0 | 9 | 0 | 0 | 21 | 12 | 21.5 | 5 | 25.7 | 15.8 |
| 9 | 0 | 2 | 0 | 9 | 0 | 0 | 21.5 | 12 | 23 | 5 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-7: Test requirement (network signalled value NS_03/NS_01)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-8: Test requirement (network signalled value NS_01/NS_06)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 1.5 | 23 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 1.5 | 21 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 0 | 2 | 0 | 0 | 0 | 1.5 | 23 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 1.5 | 23 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 0 | 0 | 1.5 | 21 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 0 | 2 | 0 | 0 | 0 | 1.5 | 23 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

<<Unchanged section skipped>>

#### <Unchanged sections skipped>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.2 UE Maximum Output Power | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.2_1 Maximum Output Power for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.2A.1 UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.2 for each CC |  |
| 6.2.2A.2 UE Maximum Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.2 for each CC |  |
| 6.2.2B UE Maximum Output Power for UL-MIMO | Same as 6.2.2 for each antenna |  |
| 6.2.2E UE Maximum Output Power for UE category 0 | Same as 6.2.2 |  |
| 6.2.3 Maximum Power Reduction | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.3_1 Maximum Power Reduction (MPR) for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.1_1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.3 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3B Maximum Power Reduction (MPR) for UL-MIMO | Same as 6.2.3 for each antenna |  |
| 6.2.3_2 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH | Same as 6.2.3 |  |
| 6.2.3E Maximum Power Reduction for UE category 0 | Same as 6.2.3 |  |
| 6.2.3_3 Maximum Power Reduction (MPR) for UL 64QAM | Same as 6.2.3 |  |
| 6.2.3_4 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 64QAM | Same as 6.2.3 |  |
| 6.2.4 UE Maximum Output Power with additional requirements | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.4_1 Additional Maximum Power Reduction (A-MPR) for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.4_2 Additional Maximum Power Reduction (A-MPR) for UL 64QAM | Same as 6.2.4 |  |
| 6.2.4_3 Additional Maximum Power Reduction (A-MPR) with PUSCH frequency hopping | Same as 6.2.4 |  |
| 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.1_1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO | Same as 6.2.4 for each antenna |  |
| 6.2.4E Additional Maximum Power Reduction (A-MPR) for UE category 0 | Same as 6.2.4 |  |
| 6.2.5 Configured UE transmitted Output Power | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.5_1 Configured UE transmitted Output Power for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.5A.1 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.5 for each CC |  |
| 6.2.5A.2 Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.5 for each CC |  |
| 6.2.5A.3 Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.2.5 for each CC |  |
| 6.2.5B Configured UE transmitted output power for UL-MIMO | Same as 6.2.5 for each antenna |  |
| 6.2.5E Configurated Transmitted Power for UE category 0 | Same as 6.2.5 |  |
| 6.3.2 Minimum Output Power | ±1.0 dB, f ≤ 3.0GHz ±1.3 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.2A.1 Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.2 for each CC |  |
| 6.3.2B Minimum Output Power for UL-MIMO | Same as 6.3.2 for each antenna |  |
| 6.3.2E Minimum Output Power for UE category 0 | Same as 6.3.2 |  |
| 6.3.3 Transmission ON/OFF Power | Transmission OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.3A.1 UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3A.2 UE Transmit OFF power for CA (inter-band DL CA and UL CA) | Same as 6.3.3A.1 |  |
| 6.3.3A.3 UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3B UE Transmit OFF power for UL-MIMO | Same as 6.3.3 for each antenna |  |
| 6.3.3E UE Transmit OFF Power for UE category 0 | Same as 6.3.3 |  |
| 6.3.4.1 General ON/OFF time mask | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4.2 PRACH and SRS time mask | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4A.1.2 General ON/OFF time mask for CA (inter-band DL CA and UL CA) | Same as 6.3.4A.1.1 |  |
| 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4B.1 General ON/OFF time mask for UL-MIMO | Same as 6.3.4.1 for each antenna |  |
| 6.3.4E.1 General ON/OFF time mask for UE category 0 | Same as 6.3.4.1 |  |
| 6.3.4E.2 PRACH and SRS time mask for UE category 0 | Same as 6.3.4.2 |  |
| 6.3.5.1 Power Control Absolute power tolerance | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB |
| 6.3.5.2 Power Control Relative power tolerance | ±0.7 dB |  |
| 6.3.5.3 Aggregate power control tolerance | ±0.7 dB |  |
| 6.3.5_1.1 Power Control Absolute power tolerance for HPUE | ±1.0 dB, f ≤ 3.0GHz |  |
| 6.3.5_1.2 Power Control Relative power tolerance for HPUE | ±0.7 dB |  |
| 6.3.5_1.3 Aggregate power control tolerance for HPUE | ±0.7 dB |  |
| 6.3.5A.1.1 Power Control Absolute power tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.2 Power Control Absolute power tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.3 Power Control Absolute power tolerance for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.2.1 Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) | Absolute power ±1dB, f ≤ 4.2GHz Relative power ±0.7 dB Both requirements apply for each CC | Absolute power is used to check when power is near lower and upper limits of the range. Relative power is used to check step sizes. |
| 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.2 Aggregate power control tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.3 Aggregate power control tolerance for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5B.1 Power Control Absolute Power Tolerance for UL- MIMO | Same as 6.3.5.1 for each antenna |  |
| 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO | Same as 6.3.5.2 for each antenna |  |
| 6.3.5E.1 Power Control Absolute power tolerance for UE category 0 | Same as 6.3.5.1 |  |
| 6.3.5E.2 Power Control Relative power tolerance for UE category 0 | Same as 6.3.5.2 |  |
| 6.3.5B.3 Aggregate power control tolerance for UL-MIMO | Same as 6.3.5.3 for each antenna |  |
| 6.3.5E.3 Aggregate power control tolerance for UE category 0 | Same as 6.3.5.3 |  |
| 6.5.1 Frequency Error | ±15 Hz DL Signal level: ±0.7 dB, f ≤ 3.0GHz DL Signal level: ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.5.1A.1 Frequency error for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1A.2 Frequency error for CA (inter-band DL CA and UL CA) | Same as 6.5.1 |  |
| 6.5.1A.3 Frequency error for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1B Frequency Error for UL-MIMO | Same as 6.5.1 for each antenna DL signal level same as 6.5.1 |  |
| 6.5.1E – Frequency Error for UE category 0 | Same as 6.5.1 |  |
| 6.5.2.1 Error Vector Magnitude | PUSCH: ± 2.5% PUCCH: ± 2.5% PRACH: ± 2.5% |  |
| 6.5.2.1_1 Error Vector Magnitude (EVM) for UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.1 Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.3 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.1_1 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2B.1 Error Vector Magnitude (EVM) for UL- MIMO | Same as 6.5.2.1 for each antenna |  |
| 6.5.2.1A PUSCH-EVM with exclusion period | PUSCH : ± 2.5% |  |
| 6.5.2.1E.1 Error Vector Magnitude for UE category 0 | Same as 6.5.2.1 |  |
| 6.5.2.1E.2 PUSCH-EVM with exclusion period for UE category 0 | Same as 6.5.2.1A |  |
| 6.5.2.2 Carrier leakage | ±0.8dB |  |
| 6.5.2A.2.1 Carrier leakage for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.2 |  |
| 6.5.2A.2.2 Carrier leakage for CA (inter-band DL CA and UL CA) | ±0.8dB |  |
| 6.5.2A.2.3 Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA) | ±0.8dB |  |
| 6.5.2.3 In-band emissions for non allocated RB | ±0.8dB |  |
| 6.5.2.4 EVM equalizer Spectrum flatness | ±1.4dB |  |
| 6.5.2A.3.1 In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.3 |  |
| 6.5.2A.3.2 In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2A.3.3 In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO | TBD |  |
| 6.5.2B.2 Carrier leakage for UL-MIMO | Same as 6.5.2.2 for each antenna |  |
| 6.5.2B.3 In-band emissions for non allocated RB for UL-MIMO | Same as 6.5.2.3 for each antenna |  |
| 6.5.2B.4 EVM equalizer spectrum flatness for UL-MIMO | Same as 6.5.2.4 for each antenna |  |
| 6.5.2.2E Carrier Leakage for UE category 0 | Same as 6.5.2.2 |  |
| 6.5.2.3E In-band emissions for non allocated RB for UE category 0 | Same as 6.5.2.3 |  |
| 6.5.2.4E EVM equalizer spectrum flatness for UE category 0 | Same as 6.5.2.4 |  |
| 6.6.1 Occupied bandwidth | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz |  |
| 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz 20MHz < f ≤ 40MHz: 500kHz |  |
| 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) | Same as 6.6.1 for each CC |  |
| 6.6.1A.3 Occupied bandwidth for CA (intra-band non-contiguous DL CA and UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz |  |
| 6.6.1B Occupied bandwidth for UL-MIMO | Same as 6.6.1 for each antenna |  |
| 6.6.1E Occupied bandwidth for UE category 0 | Same as 6.6.1 |  |
| 6.6.2.1 Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH | Same as 6.6.2.1 |  |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.2.1 |  |
| 6.6.2.1A.2 Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.1 and 6.6.2.2 |  |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.1 |  |
| 6.6.2.1B Spectrum Emission Mask for UL-MIMO | Same as 6.6.2.1 for each antenna |  |
| 6.6.2.1E Spectrum Emission Mask for UE category 0 | Same as 6.6.2.1 |  |
| 6.6.2.2 Additional Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.2_1 Additional Spectrum Emission Mask for UL 64QAM | Same as 6.6.2.2 |  |
| 6.6.2.2A.1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.2E Additional Spectrum Emission Mask for UE category 0 | Same as 6.6.2.2 |  |
| 6.6.2.2A.1_1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.2A |  |
| 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.2 |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH | Same as 6.6.2.3 |  |
| 6.6.2.3 Adjacent Channel Leakage power Ratio | ±0.8 dB |  |
| 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE | ±0.8 dB |  |
| 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | ±0.8 dB |  |
| 6.6.2.3A.1_1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) | ±0.8 dB |  |
| 6.6.2.3A.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3B Adjacent Channel Leakage power Ratio for UL-MIMO | Same as 6.6.2.3 for each antenna |  |
| 6.6.2.3E Adjacent Channel Leakage power Ratio for UE category 0 | Same as 6.6.2.3 | Same as 6.6.2.3 |
| 6.6.2.3_3 Adjacent Channel Leakage power Ratio for UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_4 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.4 Additional ACLR requirements | ±0.8 dB |  |
| 6.6.3.1 Transmitter Spurious emissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB |  |
| 6.6.3.1_1 Transmitter Spurious emissions for Multi-Cluster PUSCH | Same as 6.6.3.1 |  |
| 6.6.3.1A.1 Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.2 Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.3 Transmitter Spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3B.1Transmitter Spurious emissions for UL-MIMO | Same as 6.6.3.1, at each antenna used for transmission | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 6.6.3E.1 Transmitter Spurious emissions for UE category 0 | Same as 6.6.3.1 |  |
| 6.6.3.2 Spurious emission band UE co-existence | ± 2.0 dB for results > -60 dBm, f ≤ 3.0GHz ±2.5 dB, 3.0GHz < f ≤ 4.2GHz ± 3.0 dB for results ≤ -60 dBm, f ≤ 3.0GHz ±3.6 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.3.2A.1 Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.2 Spurious emission band UE co-existence for CA (inter-band DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.3 Spurious emission band UE co-existence for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.3 Additional spurious emissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB NS-07 769 ≤ f ≤ 775 MHz: ± 1.5 dB |  |
| 6.6.3.3_1 Additional spurious emissions for UL 64QAM | Same as 6.6.3.3 |  |
| 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) | 9kHz < f ≤ 4 GHz: ± 2.0 dB |  |
| 6.6.3.3A.1_1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.3.3A.1 |  |
| 6.6.3B.2 Spurious emission band UE co-existence for UL-MIMO | Same as 6.6.3.2 at each antenna used for transmission |  |
| 6.6.3B.3 Additional spurious emissions for UL-MIMO | Same as 6.6.3.3, at each antenna used for transmission |  |
| 6.6.3E.2 Spurious emission band UE co-existence for UE category 0 | Same as 6.6.3.2 |  |
| 6.6.3E.3 Additional spurious emissions for UE category 0 | 6.6.3.3 |  |
| 6.7 Transmit intermodulation | ± 2.6 dB, f ≤ 3.0GHz ±3.6 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises four quantities: 1. Wanted signal setting error 2. CW Interferer level error 3. Wanted signal meas. error 4. Intermodulation product measurement error The relative level of the wanted signal and the CW interferer has 2 x effect on the intermodulation product. Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared to provide the combined effect. Test System uncertainty = SQRT [(2 x SQRT (Wanted setting_error2 + CW_level_error2)) 2 + Wanted_level_meas error2 + Intermodulation product measurement error2] f ≤ 3.0GHz Wanted signal setting ± 0.7dB CW Interferer level ± 1.0dB Wanted signal meas ± 0.7dB Intermodulation product measurement error ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal setting ± 1.0dB CW Interferer level ± 1.3dB Wanted signal meas ± 1.0dB Intermodulation product measurement error ± 1.0dB |
| 6.7A.1 Transmit intermodulation for CA (intra-band contiguous DL CA and UL CA) | Same as 6.7, at each antenna used for transmission |  |
| 6.7A.2 Transmit intermodulation for CA (inter-band DL CA and UL CA) | Same as 6.7, at each antenna used for transmission |  |
| 6.7E Transmit intermodulation for UE category 0 | Same as 6.7 |  |
| 6.8B Time alignment error for UL-MIMO | ±25 ns |  |

<<Unchanged section skipped>>

#### <Unchanged sections skipped>

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Test | Minimum Requirement in TS 36.101 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 6.2.2 UE Maximum Output Power | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-3 dB | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-4.0 dB |
| 6.2.2_1 UE Maximum Output Power for HPUE | f ≤ 3.0GHz Power class 1: 31dBm +2/-3dB | 0.7dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: 31dBm +2.7/-3.7dB |
| 6.2.2A.1 UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.2 | Same as 6.2.2 | Same as 6.2.2 |
| 6.2.2A.2 UE Maximum Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.2 | Same as 6.2.2 | Same as 6.2.2 |
| 6.2.2B UE Maximum Output Power for UL-MIMO | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm +2/-3 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-4 dB | Same as 6.2.2 | Formula: Upper limit +TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm +2.7/-3.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-5.0 dB Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2.2E UE Maximum Output Power UE category 0 | Same as 6.2.2 | Same as 6.2.2 | Same as 6.2.2 |
| 6.2.3 Maximum Power Reduction | Power class 3 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: QPSK: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 4.7dB QPSK: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 6.0dB |
| 6.2.3_1 Maximum Power Reduction for HPUE | Power class 1 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 1: QPSK: 31dBm +2.7 / - 4.7dB 16QAM: 31dBm +2.7 / - 4.7dB 16QAM: 31dBm +2.7 / - 5.7dB |
| 6.2.3_2 UE Maximum Output Power for Multi-Cluster PUSCH | Same as 6.2.2 | Same as 6.2.2 | Same as 6.2.2 |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB 3.0GHz < f ≤ 4.2GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 2.7dB 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 6.2dB 23dBm +2.7 / - 14.2dB 23dBm +3.0 / - 4.0dB (+2/-3 limit for B42) 23dBm +3.0 / - 4.0dB 23dBm +3.0 / - 5.0dB 23dBm +3.0 / - 6.5dB 23dBm +3.0 / - 14.5dB |
| 6.2.3A.2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB |
| 6.2.3A.3 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·TC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |
| 6.2.3B Maximum Power Reduction (MPR) for UL-MIMO | Same as 6.2.3 | Same as 6.2.3 | Same as 6.2.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2.3E Maximum Power Reduction for UE category 0 | Same as 6.2.3E | Same as 6.2.3E | Same as 6.2.3E |
| 6.2.4 UE Maximum Output Power with additional requirements | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 3: QPSK: MPR ≤ 1dB 16QAM: Depending on the number RB allocated: 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB For network signalled value NS_03 , NS_04 (5MHz only), NS_05, to NS_06: A-MPR ≤ 1dB For network signalled value NS-04; Depending on the RB_start and RB allocation (10MHz, 15MHz and 20MHz): For 10MHz Region A with RB_start=0 – 12: A-MPR ≤ 3dB. Region B with RB_start=13 – 36 : A-MPR ≤ 2dB. Region C with RB_start=37 – 49 : A-MPR ≤ 3dB. For 15MHz Region A with RB_start=0 – 18: A-MPR ≤ 3dB. Region B with RB_start=19 – 55 : A-MPR ≤ 2dB. Region C with RB_start=56 – 74 : A-MPR ≤ 3dB. For 20MHz Region A with RB_start=0 – 24: A-MPR ≤ 3dB. Region B with RB_start=25 – 74 : A-MPR ≤ 2dB. Region C with RB_start=75 – 99 : A-MPR ≤ 3dB. For network signalled value NS-11; Depending on the RB_start and RB allocation (15MHz and 20MHz): For 15MHz Fc < 2012.5 Region A with RB_start=0 – 4: A-MPR ≤ 15dB. Region B with RB_start=5 – 21: A-MPR ≤ 7dB (LCRB=7-50) or ≤ 10dB (LCRB=0-6 & ≥50). Region C with RB_start=22 – 56: A-MPR ≤ 0dB (LCRB=≤25) or ≤ 6dB (LCRB>25). Region D with RB_start=57 – 74: A-MPR ≤ 15dB. For 15MHz Fc = 2012.5 Region A with RB_start=0 – 12: A-MPR ≤ 10dB. Region B with RB_start=13 – 39: A-MPR ≤ 6dB (LCRB=≥ 30) or ≤ 0dB (LCRB=< 30). Region C with RB_start=40 – 65: A-MPR ≤ 2dB. Region D with RB_start=66 – 74: A-MPR ≤ 6.5dB. For 20MHz Region A with RB_start=0 – 12: A-MPR ≤ 15dB. Region B with RB_start=13 – 29: A-MPR ≤ 7dB (LCRB=10-60) or ≤ 10dB (LCRB=1-9 & > 60). Region C with RB_start=30 – 68: A-MPR ≤ 0dB (LCRB=1-24) or ≤ 7dB (LCRB= ≥ 25). Region D with RB_start=69 – 99: A-MPR ≤ 15dB. For network signalled value NS-20; Depending on the RB_start and RB allocation (5MHZ, 10MHz, 15MHz and 20MHz): For 5MHz Region A with RB_start=≤24: A-MPR ≤ 17dB. Region B with RB_start=0 – 3: A-MPR ≤ 1dB (LCRB=15-19) or ≤ 4dB (LCRB= ≥ 20). Region C with RB_start=4 – 6: A-MPR ≤ 2dB. Region D with RB_start=≤24: A-MPR ≤ 0dB. For 10MHz Fc = 2005 Region A with RB_start=0 – 25: A-MPR ≤ 16dB. Region B with RB_start=26 – 34: A-MPR ≤ 2dB (LCRB=8-15) or ≤ 5dB (LCRB= > 15). Region C with RB_start=35 – 49: A-MPR ≤ 6dB. For 10MHz Fc = 2015 Region A with RB_start=0 – 5: A-MPR ≤ 4dB. Region B with RB_start=6 – 10: A-MPR ≤ 2dB. For 15MHz Region A with RB_start=0 – 14: A-MPR ≤ 11dB (LCRB=1-9 & 40-5) or ≤ 6dB (LCRB= 10-39). Region B with RB_start=15 – 24: A-MPR ≤ 1dB (LCRB=24-29) or ≤ 7dB (LCRB=≥30). Region C with RB_start=25 – 39: A-MPR ≤ 5dB. Region D with RB_start=61 – 74: A-MPR ≤ 6dB. For 20MHz Region A with RB_start=0 – 21: A-MPR ≤ 17dB. Region B with RB_start=22 – 31: A-MPR ≤ 12dB (LCRB=1-9 & 31-75) or ≤ 6dB (LCRB=10-30). Region C with RB_start=32 – 38: A-MPR ≤ 9dB. Region D with RB_start=39 – 49: A-MPR ≤ 7dB. Region E with RB_start=50 – 69: A-MPR ≤ 5dB. Region F with RB_start=70 – 99: A-MPR ≤ 16dB. | 0.7 dB | Formula: Upper limit + TT, A: Lower limit – TT, B: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – MPR – TT, C: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – TT, D: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – MPR – TT Power class 3: Test Requirement Configuration ID versus Formula Above Network signalled value NS_03: [A]: 2, 5, 10, 15, 20, 25 [B]: 1, 3, 7 [C]: 9, 14, 19, 24 [D]: 4, 6, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 Network signalled value NS_04 (5, 10, 15, 20MHz): [A] 3 [B] 10, 11, 19, 20, 28, 29 [C] 2, 6, 7, 13, 14, 15, 16, 23, 24, 25, 32 [D] 1, 4, 5, 8, 9, 12, 17, 18, 21, 22, 26, 27, 30, 31 Network signalled value NS_05: [A] 1, 3, 4, 8, 9, 14, 15, [B] 2, 5, 10, 11, 16, 17 [C] None [D] 6, 7, 12, 13, 18, 19 Network signalled value NS_06: [A]: 2, 5, 8, 11, 14, 17 [B]: 1, 3, 4, 6, 7, 9, 10, 12, 13, 15, 16, 18 [C]: None [D]: None Network signalled value NS_07: [A]:3, 8, 12 [B]: 7, 9 [C]: 1, 2, 5, 13, 15 [D]: 4, 6, 10, 11, 14, 16 Network signalled value NS_08: [A]:1, 2, 4, 5, 12 [B]: 3, 6, 11, 13 [C]: None [D]: 7, 8, 9, 10, 14, 15, 16, 17 Network signalled value NS_11: [A]: 5c, 6b, 10c, 11c, 25 [B]: 8b, 12c, 13c, 14c, 17a, 17b, 21a, 21b, 28 [C]: 5a, 5b, 6a, 6c, 10a, 10b, 11, 11b, 15a, 15b, 16a, 16b, 24 [D]: 1a, 1b, 2a, 2b, 3a, 3b, 4a, 4b ,, 7a,7b,7c, 8a, 8c, 9a, 9b,9c, 12a, 12b, 13a, 13b, 14a, 14b, 18a, 18b, 19a, 19b, 20a, 20b, 22a, 22b, 23a, 23b, 26, 27, 29, 30 Network signalled value NS_12: [A]: 3, 8, 13 [B]: None [C]: 1, 4, 6, 11 [D]: 2, 5, 7, 9, 10, 12, 14, 15 Network signalled value NS_13: [A]: None [B]: 3, 4 [C]: 1 [D]: 2, 5 Network signalled value NS_14: [A]: None [B]: 2, 4, 7, 9 [C]: 1, 6 [D]: 3, 5, 8, 10 Network signalled value NS_15: [A]: 1, 7, 14 [B]: 3, 6, 10, 15, 19, 21, 22, 25 [C]: 12, 13 [D]: 2, 4, 5, 8, 9, 11, 16, 17, 18, 20, 23, 24, 26 Network signalled value NS_20: [A]: 1b, 1c, 1d, 6b, 7b [B]: 2d, 3d, 4d, 5d, 9b [C]: 1a, 6a, 7a, 11, 18, 19 [D]: 2a, 2b, 2c, 3a, 3b, 3c, 4a, 4b, 4c, 5a, 5b, 5c, 8a, 8b, 9a, 10a, 11b, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26 |
| 6.2.4_1 Additional Maximum Power Reduction (A-MPR) for HPUE | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 1: QPSK: MPR ≤ 1dB 16QAM: Depending on the number RB allocated: 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB Power class 1: For network signalled value NS_06: A-MPR 0dB | 0.7dB | Formula: Upper limit + TT, (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – MPR – TT |
| 6.2.4_3 Additional Maximum Power Reduction (A-MPR) with PUSCH frequency hopping | Same as 6.2.4 | Same as 6.2.4 | Same as 6.2.4 |
| 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) | A-MPR values as detailed in core requirement | Same as 6.2.4 | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |
| 6.2.4A.2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) | Same as 6.2.4 | Same as 6.2.4 | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -·TC |
| 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO | Same as 6.2.4 | Same as 6.2.4 | Same as 6.2.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2.4E Additional Maximum Power Reduction (A-MPR) for UE category 0 | Same as 6.2.4 | Same as 6.2.4 | Same as 6.2.4 |
| 6.2.5 Configured UE transmitted Output Power | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 13 ≤ PCMAX < 18 ± 6.0 8 ≤ PCMAX < 13 ± 7.0 -40 ≤ PCMAX < 8 ± 8.0 |
| 6.2.5_1 Configured UE transmitted Output Power for HPUE | f ≤ 3.0GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit + TT, Lower limit – TT 23 ≤ PCMAX < 33 ± 2.7 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 |
| 6.2.5A.1 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 13 ≤ PCMAX < 18 ± 6.0 8 ≤ PCMAX < 13 ± 7.0 -40 ≤ PCMAX < 8 ± 8.0 |
| 6.2.5A.2 Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) | f ≤ 3.0GHz 16 ≤ PCMAX < 20 ± 5.0 11≤ PCMAX < 16 ± 6.0 -40 ≤ PCMAX < 11 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit + TT, Lower limit – TT 16 ≤ PCMAX < 20 ± 5.7 11 ≤ PCMAX < 16 ± 6.7 -40 ≤ PCMAX < 11 ± 7.7 |
| 6.2.5A.3 Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 |
| 6.2.5B Configured UE transmitted output power for UL-MIMO | f ≤ 3.0GHz [16] ≤ PCMAX < [20] ± [5.0] [11] ≤ PCMAX < [16] ± [6.0] [-40] ≤ PCMAX < [11] ± [7.0] 3.0GHz < f ≤ 4.2GHz [16] ≤ PCMAX < [20] ± [5.0] [11] ≤ PCMAX < [16] ± [6.0] [-40] ≤ PCMAX < [11] ± [7.0] | Same as 6.2.5 | Formula: Upper limit + TT, Lower limit – TT [16] ≤ PCMAX < [20] ± [5.7] [11] ≤ PCMAX < [16] ± [6.7] [-40] ≤ PCMAX < [11] ± [7.7] [16] ≤ PCMAX < [20] ± [6.0] [11] ≤ PCMAX < [16] ± [7.0] [-40] ≤ PCMAX < [11] ± [8.0] Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2.5E Configurated Transmitted Power for UE category 0 | Same as 6.2.5 | Same as 6.2.5 | Same as 6.2.5 |
| 6.3.2 Minimum Output Power | f ≤ 3.0GHz -40 dBm 3.0GHz < f ≤ 4.2GHz -40 dBm | 1.0 dB 1.3 dB | Formula: Minimum Requirement + TT UE min. output power =–39 dBm UE min. output power =–38.7 dBm |
| 6.3.2A.1 Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.2 | Same as 6.3.2 | [Same as 6.3.2 |
| 6.3.2B Minimum Output Power for UL-MIMO | Same as 6.3.2 | Same as 6.3.2 | Same as 6.3.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3.2E Minimum Output Power for UE category 0 | Same as 6.3.2 | Same as 6.3.2 | Same as 6.3.2 |
| 6.3.3 Transmission ON/OFF Power | f ≤ 3.0GHz -50 dBm 3.0GHz < f ≤ 4.2GHz -50 dBm | 1.5 dB 1.8 dB | Formula: Minimum Requirement + TT UE OFF Power -48.5 dBm UE OFF Power -48.2 dBm |
| 6.3.3A.1 UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.3 | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.3A.2 Transmit OFF Power for CA (inter-band DL CA and UL CA) | Same as 6.3.3 | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.3A.3 UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.3 | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.3B UE Transmit OFF power for UL-MIMO | Same as 6.3.3 | Same as 6.3.3 | Same as 6.3.3 Uplink power measurement applies to each Tx antenna connector |
| 6.3.3E UE Transmit OFF Power for UE category 0 | Same as 6.3.3 | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.4.1 General ON/OFF time mask | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB | 1.5 dB 1.5 dB 1.8 dB 1.8 dB | Formulae: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB |
| 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC | Same as 6.3.4.1 | Same as 6.3.4.1 for each CC |
| 6.3.4A.1.2 General ON/OFF time mask for CA (inter-band DL CA and UL CA) | Same as 6.3.4.1 | Same as 6.3.4.1 | Same as 6.3.4.1 |
| 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC | Same as 6.3.4.1 | Same as 6.3.4.1 for each CC |
| 6.3.4B.1 General ON/OFF time mask for UL-MIMO | Same as 6.3.4.1 | Same as 6.3.4.1 | Same as 6.3.4.1 Uplink OFF power measurement applies to each Tx antenna connector Uplink ON power measurement applies to [FFS] |
| 6.3.4E.1 General ON/OFF time mask for UE category 0 | Same as 6.3.4.1 | Same as 6.3.4.1 | Same as 6.3.4.1 |
| 6.3.4E.2 PRACH and SRS time mask for UE category 0 | Same as 6.3.4.2 | Same as 6.3.4.2 | Same as 6.3.4.2 |
| 6.3.4.2 PRACH and SRS time mask | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB | 1.5 dB 1.5 dB 1.8 dB 1.8 dB | Formulae: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB |
| 6.3.5.1 Power Control Absolute power tolerance | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB | 1.0 dB 1.0 dB 1.4 dB 1.4 dB | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB |
| 6.3.5.2 Power Control Relative power tolerance | TS 36.101 [2] clause 6.3.5.1 All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±2.5 dB 2 ≤ ΔP < 3; ±3.0 dB 3 ≤ ΔP < 4; ±3.5 dB 4 ≤ ΔP ≤ 10; ±4.0 dB 10 ≤ ΔP < 15; ±5.0 dB 15 ≤ ΔP; ±6.0 dB | 0.7 dB | Formula: Upper limit + TT, Lower limit – TT All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±3.2 dB 2 ≤ ΔP < 3; ±3.7 dB 3 ≤ ΔP < 4; ±4.2 dB 4 ≤ ΔP < 10; ±4.7 dB 10 ≤ ΔP < 15; ±5.7 dB 15 ≤ ΔP; ±6.7 dB |
| 6.3.5.3 Aggregate power control tolerance | Aggregate power control tolerance within 21 ms: PUCCH = ±2.5 dB PUSCH = ±3.5 dB | 0.7 dB | Formula: Upper limit + TT, Lower limit - TT PUCCH = ±3.2 dB PUSCH = ±4.2 dB |
| 6.3.5_1.1 Power Control Absolute power tolerance for HPUE | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB | 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB |
| 6.3.5_1.2 Power Control Relative power tolerance for HPUE | Same as 6.3.5.2 | Same as 6.3.5.2 | Same as 6.3.5.2 |
| 6.3.5_1.3 Aggregate power control tolerance for HPUE | Same as 6.3.5.3 | Same as 6.3.5.3 | Same as 6.3.5.3 |
| 6.3.5A.1.1 Power Control Absolute power tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.1 | Same as 6.3.5.1 | Same as 6.3.5.1 |
| 6.3.5A.1.2 Power Control Absolute power tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.1 | Same as 6.3.5.1 | Same as 6.3.5.1 |
| 6.3.5A.1.3 Power Control Absolute power tolerance for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.5.1 | Same as 6.3.5.1 | Same as 6.3.5.1 |
| 6.3.5A.2.1 Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) | TS 36.101 [2] clause 6.3.5A.2 refers to Table 6.3.5.2.1-1. All combinations of PUSCH and PUCCH transitions: SCC 4 ≤ ΔP ≤ 10: Normal ±4.0dB Exceptions: ±6.0dB PCC ΔP ≤ 1dB: Normal ±1.0dB Exceptions: ±6.0dB Each CC can have exceptions | Normal ΔP = 9dB +4.7dB -4.9dB Exceptions ΔP = 9dB +6.7dB -6.9dB Normal ΔP = 0dB ±1.7dB Exceptions ΔP = 0dB +6.7dB -6.9dB | Relative power measurement uncertainty comprises two quantities: 1. Uplink Relative level measurement uncertainty of Test system 2. IQ image Leakage onto weaker CC at -25dBc is allowed for the UE, so is added arithmetically. It can only increase the measured power of the weaker CC. UL relative meas’t uncert ±0.7dB Leakage +0.17dB for 11dB CC power difference, based on -25dBc. Upper limit +0.7dB Lower limit (–0.7dB -0.17dB) When setting SCC power for the next step, three factors are relevant: 1. Uplink Relative level measurement uncertainty 2. IQ image Leakage onto weaker CC at -25dBc is allowed for the UE 3) TPC step 1dB ±1dB is allowed for the UE These are added arithmetically, giving an extra margin of (1+0.7+0.17) = 1.87dB (2dB is allowed) to avoid the same power step exception being counted twice. |
| 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.3 | Same as 6.3.5.3 | Same as 6.3.5.3 |
| 6.3.5A.3.2 Aggregate power control tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.3 | Same as 6.3.5.3 | Same as 6.3.5.3 |
| 6.3.5A.3.3 Aggregate power control tolerance for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.5.3 | Same as 6.3.5.3 | Same as 6.3.5.3 |
| 6.3.5B.1 Power Control Absolute Power Tolerance for UL- MIMO | Same as 6.3.5.1 | Same as 6.3.5.1 | Same as 6.3.5.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO | Same as 6.3.5.2 | Same as 6.3.5.2 | Same as 6.3.5.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3.5B.3 Aggregate power control tolerance for UL-MIMO | Same as 6.3.5.3 | Same as 6.3.5.3 | Same as 6.3.5.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3.5E.1 Power Control Absolute power tolerance for UE category 0 | Same as 6.3.5.1 | Same as 6.3.5.1 | Same as 6.3.5.1 |
| 6.3.5E.2 Power Control Relative power tolerance for UE category 0 | Same as 6.3.5.2 | Same as 6.3.5.2 | Same as 6.3.5.2 |
| 6.3.5E.3 Aggregate power control tolerance for UE category 0 | Same as 6.3.5.3 | Same as 6.3.5.3 | Same as 6.3.5.3 |
| 6.5.1 Frequency Error | Modulated carrier, f ≤ 4.2GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens 3.0GHz < f ≤ 4.2GHz DL power: Refsens | 15 Hz 0.7 dB 1.0 dB | Formulae: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB Refsens +1.0dB |
| 6.5.1A.1 Frequency error for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.1 | Same as 6.5.1 | Same as 6.5.1 |
| 6.5.1A.2 Frequency Error for CA (inter-band DL CA and UL CA) | Same as 6.5.1 per CC | Same as 6.5.1 per CC | Same as 6.5.1 per CC |
| 6.5.1A.3 Frequency Error for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.1 per CC | Same as 6.5.1 per CC | Same as 6.5.1 per CC |
| 6.5.1B Frequency Error for UL-MIMO | Same as 6.5.1 | Same as 6.5.1 | Same as 6.5.1 |
| 6.5.1E – Frequency Error for UE category 0 | Same as 6.5.1 | Same as 6.5.1 | Same as 6.5.1 |
| 6.5.2.1 Error Vector Magnitude | EVM limit: BPSK :17.5 % QPSK: 17.5 % 16QAM: 12.5 % | 0% | Formula: Minimum Requirement + TT |
| 6.5.2.1A PUSCH-EVM with exclusion period | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % | 0% | Formula: Minimum Requirement + TT |
| 6.5.2A.1.1 Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.1 | Same as 6.5.2.1 | Same as 6.5.2.1 |
| 6.5.2A.1.2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) | Same as 6.5.2.1 | Same as 6.5.2.1 | Same as 6.5.2.1 |
| 6.5.2A.1.3 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.2.1 | Same as 6.5.2.1 | Same as 6.5.2.1 |
| 6.5.2B.1 Error Vector Magnitude (EVM) for UL- MIMO | Same as 6.5.2.1 | Same as 6.5.2.1 | Same as 6.5.2.1 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5.2.1E.1 Error Vector Magnitude for UE category 0 | Same as 6.5.2.1 | Same as 6.5.2.1 | Same as 6.5.2.1 |
| 6.5.2.1E.2 PUSCH-EVM with exclusion period for UE category 0 | Same as 6.5.2.1A | Same as 6.5.2.1A |  |
| 6.5.2.2 Carrier leakage | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc | 0.8dB | Formula: Minimum Requirement + TT |
| 6.5.2.3 In-band emissions for non allocated RB | For general emissions: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc -40 dBm ≤ Output power < -30 dBm -10dBc For each evaluated RB, the test requirement is calculated as the higher of PRB – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) | 0.8dB | Formula: Minimum Requirement + TT |
| 6.5.2.4 EVM equalizer Spectrum flatness | Normal conditions. If (F-FUL_low ≥ [3MHz])&(FUL_high-F≥ [3MHz]) 4 dB else 8 dB maximum coefficient in Range 1 - the minimum coefficient in Range 2 5 dB the maximum coefficient in Range 2 - the minimum coefficient in Range 7 dB Extreme conditions: If (F-FUL_low ≥ [5MHz])&(FUL_high-F≥ [5MHz]) 4 dB else 12 dB maximum coefficient in Range 1 - the minimum coefficient in Range 2 6 dB the maximum coefficient in Range 2 - the minimum coefficient in Range 10 dB | 1.4dB | Formula: Minimum Requirement + TT |
| 6.5.2A.2.1 Carrier leakage for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.2 | Same as 6.5.2.2 | Same as 6.5.2.2 |
| 6.5.2A.2.2 Carrier leakage for CA (inter-band DL CA and UL CA) | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc | 0.8dB | Formula: Minimum Requirement + TT |
| 6.5.2A.2.3 Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA) | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc | 0.8dB | Formula: Minimum Requirement + TT |
| 6.5.2A.3.1 In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.3 | Same as 6.5.2.3 | Same as 6.5.2.3 |
| 6.5.2A.3.2 In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA) | Same as 6.5.2.3 | Same as 6.5.2.3 | Same as 6.5.2.3 |
| 6.5.2A.3.3 In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.2.3 | Same as 6.5.2.3 | Same as 6.5.2.3 |
| 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO | TBD | TBD | TBD |
| 6.5.2B.2 Carrier leakage for UL-MIMO | Same as 6.5.2.2 | Same as 6.5.2.2 | Same as 6.5.2.2 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5.2B.3 In-band emissions for non allocated RB for UL-MIMO | Same as 6.5.2.3 | Same as 6.5.2.3 | Same as 6.5.2.3 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5.2B.4 EVM equalizer spectrum flatness for UL-MIMO | Same as 6.5.2.4 | Same as 6.5.2.4 | Same as 6.5.2.4 |
| 6.5.2.2E Carrier Leakage for UE category 0 | Same as 6.5.2.2 | Same as 6.5.2.2 | Same as 6.5.2.2 |
| 6.5.2.3E In-band emissions for non allocated RB for UE category 0 | Same as 6.5.2.3 | Same as 6.5.2.3 | Same as 6.5.2.3 |
| 6.5.2.4E EVM equalizer spectrum flatness for UE category 0 | Same as 6.5.2.4 | Same as 6.5.2.4 | Same as 6.5.2.4 |
| 6.6.1 Occupied bandwidth | For 1.4 MHz channel bandwidth: Occupied channel bandwidth = 1.4 MHz For 3.0 MHz channel bandwidth: Occupied channel bandwidth = 3.0 MHz For 5 MHz channel bandwidth: Occupied channel bandwidth = 5 MHz For 10 MHz channel bandwidth: Occupied channel bandwidth = 10 MHz For 15 MHz channel bandwidth: Occupied channel bandwidth = 15 MHz For 20 MHz channel bandwidth: Occupied channel bandwidth = 20 MHz | 0kHz | Formula: Minimum Requirement + TT |
| 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) | For 25(20+5) MHz channel bandwidth: Occupied channel bandwidth = 24.95 MHz For 30(20+10) MHz channel bandwidth: Occupied channel bandwidth = 29.90 MHz For 30(15+15) MHz channel bandwidth: Occupied channel bandwidth = 30.00 MHz For 35(20+15)MHz channel bandwidth: Occupied channel bandwidth = 34.85 MHz For 40(20+20) MHz channel bandwidth: Occupied channel bandwidth = 39.80 MHz | Same as 6.6.1 | Same as 6.6.1 |
| 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) | Same as 6.6.1 per CC | Same as 6.6.1 per CC | Same as 6.6.1 per CC |
| 6.6.1A.3 | Same as 6.6.1 for each CC | Same as 6.6.1 | Same as 6.6.1 for each CC |
| 6.6.1B Occupied bandwidth for UL-MIMO | Same as 6.6.1 | Same as 6.6.1 | Same as 6.6.1 |
| 6.6.1E Occupied bandwidth for UE category 0 | Same as 6.6.1 | Same as 6.6.1 | Same as 6.6.1 |
| 6.6.2.1 Spectrum Emission Mask | For 1.4 MHz BW: -10 dBm / 30kHz -25dBm to -10dBm / 1MHz For 3 MHz BW: -13 dBm / 30kHz -25dBm to -10dBm / 1MHz For 5 MHz BW: -15dBm / 30kHz -25dBm to -10dBm / 1MHz For 10 MHz BW: -18dBm / 30kHz -25dBm to -10dBm / 1MHz For 15 MHz BW: -20dBm / 30kHz -25dBm to -10dBm / 1MHz For 20 MHz BW: -21dBm / 30kHz -25dBm to -10dBm / 1MHz | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) | For 24.95 MHz BW: -22 dBm / 30kHz -25 dBm to - 10 dBm / 1 MHz For 29.9 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 30 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 34.85 MHz BW: -23.5dBm / 30kHz -25dBm to -10dBm / 1MHz For 39.8 MHz BW: -24dBm / 30kHz -25dBm to -10dBm / 1MHz | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.1A.2 Spectrum emission mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.1 per CC | Same as 6.6.2.1 per CC | Same as 6.6.2.1 per CC |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.1 for each CC | Same as 6.6.2.1 | Same as 6.6.2.1 for each CC |
| 6.6.2.1B Spectrum Emission Mask for UL-MIMO | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.1E Spectrum Emission Mask for UE category 0 | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.2 Additional Spectrum Emission Mask | For 1.4 MHz BW: NS_03, NS_04 -10 dBm / 30 kHz -25 dBm to -13 dBm / 1MHz NS_06 or NS_07 -13 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to -13 dBm / 1MHz For 3 MHz BW: NS_03, NS_04 -13 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz NS_06 or NS_07 -13 dBm / 30 kHz -13 dBm / 100kHz -25 dBm to -13 dBm / 1 MHz For 5 MHz BW: NS_03, NS_04 -15 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz NS_06 or NS_07 -15 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to -13 dBm / 1 MHz For 10 MHz BW: NS_03, NS_04, -18 dBm / 30 kHz -25 dBm to - 13dBm / 1 MHz NS_06 or NS_07 -18 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to - 13dBm / 1 MHz For 15 MHz BW: NS_03, NS_04 -20 dBm / 30kHz -25 dBm to -13 dBm / 1 MHz For 20 MHz BW: NS_03, NS_04 -21 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |
| 6.6.2.2A-1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) | For network signalled value CA_NS_04: -21dBm/30kHz to -22.5dBm/kHz, BW-dependent, for 0-1MHz -13dBm/1MHz for 1MHz-5.5MHz -25dBm/1MHz beyond 5.5MHz | Same as 6.6.2.2 | Same as 6.6.2.2 |
| 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.2 | Same as 6.6.2.2 | Same as 6.6.2.2 |
| 6.6.2.2B Additional Spectrum Emission Mask for UL-MIMO | Same as 6.6.2.2 | Same as 6.6.2.2 | Same as 6.6.2.2 |
| 6.6.2.2E Additional Spectrum Emission Mask for UE category 0 | Same as 6.6.2.2 | Same as 6.6.2.2 | Same as 6.6.2.2 |
| 6.6.2.3 Adjacent Channel Leakage power Ratio | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 30 dB UTRA ACLR: 33 dB for UTRA ACLR 1 36 dB for UTRA ACLR 2 | 0 dB 0.8 dB 0.8 dB 0.8 dB | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR: 29.2 dB UTRA ACLR: 32.2 dB for UTRA ACLR 1 35.2 dB for UTRA ACLR 2 |
| 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 37 dB | 0dB 0.8dB | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR: 36.2 dB |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH | Same as 6.6.2.3 | Same as 6.6.2.3 | Same as 6.6.2.3 |
| 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. UTRA ACLR: 33 dB for UTRA ACLR 1 36 dB for UTRA ACLR 2 E-UTRA ACLR: 30 dB | 0 dB 0.8 dB 0.8 dB 0.8 dB | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT UTRA ACLR: 32.2 dB for UTRA ACLR 1 35.2 dB for UTRA ACLR 2 E-UTRA ACLR: 29.2 dB |
| 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.2.3 per component carrier | Same as 6.6.2.3 per component carrier | Same as 6.6.2.3 per component carrier |
| 6.6.2.3A.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.3 | Same as 6.6.2.3 | Same as 6.6.2.3 |
| 6.6.2.3B Adjacent Channel Leakage power Ratio for UL-MIMO | Same as 6.6.2.3 | Same as 6.6.2.3 | Same as 6.6.2.3 |
| 6.6.2.3E Adjacent Channel Leakage power Ratio for UE category 0 | Same as 6.6.2.3 | Same as 6.6.2.3 | Same as 6.6.2.3 |
| 6.6.2.4 Additional ACLR requirements | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 43 dB for UTRA ACLR 2 | 0 dB 0.8 dB | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement – TT E-UTRA ACLR: 42.2 dB for UTRA ACLR 2 |
| 6.6.3.1 Transmitter Spurious emissions | 9 kHz f < 150 kHz: -36dBm / 1kHz 150 kHz f < 30 MHz: -36dBm / 10kHz 30 MHz f < 1 GHz: -36dBm / 100kHz 1 GHz f < 12.75 GHz: -30dBm / 1MHz 12.75 GHz ≤ f < 19 GHz: -30dBm / 1MHz | 0 dB | Formula: Minimum Requirement + TT |
| 6.6.3.1_1Transmitter Spurious emissions for Multi-Cluster PUSCH | Same as 6.6.3.1 | Same as 6.6.3.1 | Formula: Minimum Requirement + TT |
| 6.6.3.1A.1 Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.3.1 | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.3.1A.2 Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC |
| 6.6.3.1A.3 Transmitter Spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC |
| 6.6.3B.1Transmitter Spurious emissions for UL-MIMO | Same as 6.6.3.1 | Same as 6.6.3.1 | Formula: Minimum Requirement + TT |
| 6.6.3E.1 Transmitter Spurious emissions for UE category 0 | Same as 6.6.3.1 | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.3.2 Spurious emission band UE co-existence | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz Frequencies as detailed in core requirement | 0 dB | Formula: Minimum Requirement + TT |
| 6.6.3.2A.1 Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz -15.5 dBm / 5MHz +1.6 dBm /5MHz Frequencies as detailed in core requirement | 0 dB | Formula: Minimum Requirement + TT |
| 6.6.3.2A.3 Spurious emission band UE co-existence for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.3.2 | Same as 6.6.3.2 | Formula: Minimum Requirement + TT |
| 6.6.3.3 Additional spurious emissions | NS_05 1884.5MHz f 1915.7MHz: -41dBm / 300kHz NS_07 769MHz ≤ f ≤ 775MHz -57dBm / 6.25kHz NS_08 860MHz ≤ f ≤ 895MHz -40dBm / 1MHz NS_09 1475.9MHz ≤ f ≤ 1510.9MHz -35dBm / 1MHz | 0 dB 1.5dB 0 dB 0 dB | Formula: Minimum Requirement + TT -41dBm / 300kHz -55.5 dBm / 6.25kHz -40dBm / 1MHz -35dBm / 1MHz |
| 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) | CA_NS_01 E-UTRA band 34: -50dBm / 1MHz 1884.5MHz f 1915.7MHz: -41dBm / 300kHz CA_NS_02 E-UTRA band 33: -50dBm / 1MHz E-UTRA band 34: -50dBm / 1MHz CA_NS_03 E-UTRA band 34: -50dBm / 1MHz E-UTRA band 39: -50dBm / 1MHz | 0 dB 0 dB 0 dB 0 dB 0 dB 0 dB | Formula: Minimum Requirement + TT -50 MHz / 1MHz -41 dBm / 300kHz -50 MHz / 1MHz -50 MHz / 1MHz -50 MHz / 1MHz -50 MHz / 1MHz |
| 6.6.3B.2 Spurious emission band UE co-existence for UL-MIMO | Same as 6.6.3.2 | Same as 6.6.3.2 | Formula: Minimum Requirement + TT |
| 6.6.3B.3 Additional spurious emissions for UL-MIMO | Same as 6.6.3.3 | Same as 6.6.3.3 | Formula: Minimum Requirement + TT |
| 6.6.3E.2 Spurious emission band UE co-existence for UE category 0 | Same as 6.6.3.2 | Same as 6.6.3.2 |  |
| 6.6.3E.3 Additional spurious emissions for UE category 0 | Same as 6.6.3.3 | Same as 6.6.3.3 | Same as 6.6.3.3 |
| 6.7 Transmit intermodulation | Intermodulation Product 5MHz -29 dBc 10MHz -35 dBc CW Interferer level = -40 dBc | 0 dB | Formula: CW interferer Minimum Requirement– TT Intermod Products limits remain unchanged. CW interferer level = -40 dBc |
| 6.7A.1 Transmit intermodulation for CA (intra-band contiguous DL CA and UL CA) | Same as 6.7 | Same as 6.7 | Same as 6.7 |
| 6.7A.2 Transmit intermodulation for CA (inter-band DL CA and UL CA) | Same as 6.7 | Same as 6.7 | Same as 6.7 |
| 6.7B Transmit intermodulation for UL-MIMO | Same as 6.7 | Same as 6.7 | Same as 6.7 |
| 6.7E Transmit intermodulation for UE category 0 | Same as 6.7 | Same as 6.7 | Same as 6.7 |
| 6.8B Time alignment error for UL-MIMO | The Time Alignment Error (TAE) shall not exceed 130 ns | 25 ns | Formula: Minimum Requirement+ TT |

#### <Unchanged sections skipped>