3GPP TSG-RAN WG5 Meeting #72 	R5-165330

# Gothenburg, 22-26 August 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 0912 | rev |  | Current version: | 12.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to C.11 MTSI Speech call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  |  | Date: |  | 2016-08-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | [As per IR.92 and section 3.3], The UE and the IMS core network must support DTMF events as defined in Annex G of 3GPP TS 26.114. [26.114 and Annex G], This annex describes a method for sending DTMF events in the same RTP media stream as the speech. - MTSI clients offering speech communication shall support the below described method in the transmitting direction and should support it in the receiving direction. - MTSI media gateways offering speech communication shall support the below described method in both the transmitting direction and in the receiving direction. For MTSI media gateways, the described method applies only to the PS session between the gateway and an MTSI client in terminal. This method was designed to send DTMF events in the same RTP streams as the speech. Based on the above, DTMF support is mandatory. This will be indicated in INVITE message. [24.229] In addition, the UE may indicate support of the in-band DTMF codec, as described in subclause 6.1.1. Based on the above, UE may indicate support DTMF codec. UE may indicate in ‘183 session progress’ during MT call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added telephone-event to INVITE as DTMF support is mandatory. - Optionally, UE may indicate in ‘183 session progress’. - SS shall indicate DTMF support if UE indicates in 183 session progress |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test coverage for DTMF support in MT call. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# C.11	Generic test procedure for setting up MTSI MT speech call

The generic test procedure for setting up MTSI MT speech call may be performed after successful IMS or early IMS registration.

Test procedure

1)	SS sends an INVITE request to the UE.

2)	Void.

3)	SS may receive 100 Trying from the UE.

4)	SS expects and receives 183 Session Progress from the UE.

5)	SS sends PRACK to the UE to acknowledge the 183 Session Progress.

6)	SS expects and receives 200 OK for PRACK from the UE.

7)	SS sends UPDATE to the UE, with SDP indicating that precondition is met on the server side.

8)	SS expects and receives 200 OK for UPDATE from the UE, with proper SDP as answer.

9)	SS may receive 180 Ringing from the UE.

10)	SS may send PRACK to the UE to acknowledge the 180 Ringing.

11)	SS may receive 200 OK for PRACK from the UE.

11A)	The UE accepts the session invite.

12)	SS expects and receives 200 OK for INVITE from the UE.

13)	SS sends ACK to the UE.

14)	SS sends BYE to the UE.

15)	SS expects and receives 200 OK for BYE from the UE.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends INVITE with the first SDP offer. |
| 2 |  |  |  | Void |
| 3 |  |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response |
| 4 |  |  | 183 Session Progress | The UE sends 183 response reliably with the SDP answer to the offer in INVITE |
| 5 |  |  | PRACK | SS acknowledges the receipt of 183 response from the UE. |
| 6 |  |  | 200 OK | The UE responds to PRACK with 200 OK. |
| 7 |  |  | UPDATE | SS sends an UPDATE with SDP offer indicating SS reserved resources. |
| 8 |  |  | 200 OK | The UE acknowledges the UPDATE with 200 OK and includes SDP answer to acknowledge its current precondition status. |
| 9 |  |  | 180 Ringing | (Optional) The UE responds to INVITE with 180 Ringing. |
| 10 |  |  | PRACK | (Optional) SS shall send PRACK only if the 180 response contains 100rel option tag within the Require header. |
| 11 |  |  | 200 OK | (Optional) The UE acknowledges the PRACK with 200 OK. |
| 11A |  |  |  | Make UE accept the speech AMR offer. |
| 12 |  |  | 200 OK | The UE responds to INVITE with a 200 OK final response after the user answers the call. |
| 13 |  |  | ACK | The SS acknowledges the receipt of 200 OK for INVITE. |
| 14 |  |  | BYE | The SS sends BYE to release the call. |
| 15 |  |  | 200 OK | The UE sends 200 OK for the BYE request and ends the call. |

NOTE:	The default messages contents in annex A are used with condition “IMS security” or “early IMS security” when applicable

Specific Message Content

INVITE (Step 1)

Use the default message “INVITE for MT Call” in annex A.2.9 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97100 - b=AS:37 - b=RS:0 - b=RR:2500 Attributes for media: - a=rtpmap:97 AMR/8000/1 - a=fmtp:97 mode-change-capability=2; max-red=220 - a=rtpmap:100 telephone-event/8000 - a=fmtp:100 0-15 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(user-name) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 3] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR/8000 [Note 3] - a=fmtp:(format) [Note 3, 4] - a=rtpmap:(payload type) telephone-event [Note 5] - a=fmtp:(format)[Note 3] Attributes for preconditions: - a=curr:qos local none or a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv - a=conf:qos remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: Void Note 3: The value for fmt, payload type and format is not checked Note 4: Parameters for the AMR codec are not checked Note 5: a rate may be added to the “telephone-event” seperated by “/” (e.g. “telephone-event/8000”) |

UPDATE (step 7)

Use the default message "UPDATE" in annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111112 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 100 [Note 3] - b=AS:37 - b=RS:0 - b=RR:2500 Attributes for media: - a=rtpmap:97 AMR/8000 [Note 2] - a=fmtp:97 mode-change-capability=2; max-red=220 - a=rtpmap:100 telephone-event/8000 [Note 3] - a=fmtp:100 0-15 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none or curr:qos remote sendrecv [Note 1] - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: Use the value (none/sendrecv) received from 183 Session Progress and attribute a=curr:qos local. Note 2: The AMR channel number shall be “/1” or omitted. Note 3: Codec with telephone-event(DTMF) shall be sent only if UE sends telephone-event(DTMF) in 183 session progress |

200 OK (step 8)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(user-name) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 4] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR/8000 [Note 2] [Note 4] - a=fmtp:(format) [Note 2, 3][Note 4] - a=rtpmap:(payload type) telephone-event [Note 3] - a=fmtp:(format)[Note 3] Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The value for fmt, payload type and format is not checked Note 3: Parameters for the AMR codec are not checked Note 4: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 3: Codec with telephone-event(DTMF) shall be sent only if UE sends telephone-event(DTMF) in UPDATE |

180 Ringing (Step 9)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header not present |
| media-type |  |
| Content-Length | 0 |
| value |  |
| Message-body | Not present |