3GPP TSG-RAN5 Meeting #76	R5-174113

# Berlin, Germany, 21st August 2017 – 25th August 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | xxxx | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New TC: 7.3.69 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-UEConTest ? |  |  |  |  |  | Date: |  |  | 2017-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding new NBIoT HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new NBIoT RRM TC 7.3.69 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This test case will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (New) 7.3.69 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

7.3.69	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage

Editor’s note: This Test case is incomplete

-	The Test Tolerances and Test Requirements applicable are undefined

7.3.69.1	Test purpose

The purpose of this test is to verify that the HD-FDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT HD-FDD radio link monitoring requirements in clause TS36.133 clause 7.23.

7.3.69.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 13 and forward of category NB-1.

7.3.69.3	Minimum conformance requirements

The requirements defined in this subclause for performing radio link monitoring are applicable for Category NB1 UE defined in Section 3.1 of TS36.133.

The UE shall meet all applicable requirements specified in this clause under the following condition:

-	at least 1 DL subframe per radio frame of serving NB-IoT cell is available at the UE during Qout_NB-IoT and Qin_NB-IoT evaluation periods.

The UE shall estimate the downlink radio link quality and compare it to the thresholds Qout_NB-IoT and Qin_NB-IoT for the purpose of monitoring downlink radio link quality of the NB-IoT cell.

The threshold Qout_NB-IoT is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to 10% block error rate of a hypothetical NPDCCH transmission with transmission parameters specified in Table 7.3.69.3-1.

The threshold Qin_NB-IoT is defined as the level at which the downlink radio link quality can be significantly more reliably received than at Qout_NB-IoT and shall correspond to 2% block error rate of a hypothetical NPDCCH transmission with transmission parameters specified in Table 7.3.69.3-1.

Table 7.3.69.3-1: NPDCCH transmission parameters for out-of-sync and in-sync for Category NB1 UE

| Attribute | Out-of-sync | In-sync |
| --- | --- | --- |
| DCI format | Format N1 | Format N1 |
| Number of information bits | 22 bits | 22 bits |
| System Bandwidth | 200kHz | 200kHz |
| Antenna configuration | 2x1 | 2x1 |
| Maximum NPDCCH Repetition level | RmaxNote1 | Rmax/4 Note1 |
| Aggregation level | 2 | 2 |
| DRX | OFF | OFF |
| Deployment mode | Standalone | Standalone |
| NOTE 1: Rmax is a configurable parameter defined in TS 36.331[5]. |  |  |

When DRX is used for HD-FDD Category NB1 UE UEs, the Qout_NB-IoT evaluation period (TEvaluate_Qout_DRX_NB-IoT) and the Qin_NB-IoT evaluation period (TEvaluate_Qin_DRX_NB-IoT) is specified in Table 7.3.69.3-2 will be used.

When the downlink radio link quality of the NB-IoT cell estimated over the last TEvaluate_Qout_DRX_NB-IoT [s] period becomes worse than the threshold Qout_NB-IoT, Layer 1 of the UE shall send out-of-sync indication for the NB-IoT cell to the higher layers within TEvaluate_Qout_DRX_NB-IoT [s] evaluation period. A Layer 3 filter shall be applied to the out-of-sync indications as specified in TS 36.331 [5].

When the downlink radio link quality of the NB-IoT cell estimated over the last TEvaluate_Qin_DRX_NB-IoT [s] period becomes better than the threshold Qin_NB-IoT, Layer 1 of the UE shall send in-sync indications for the NB-IoT cell to the higher layers within TEvaluate_Qin_DRX_NB-IoT [s] evaluation period. A L3 filter shall be applied to the in-sync indications as specified in TS 36.331 [5].

The out-of-sync and in-sync evaluations of the NB-IoT cell shall be performed as specified in TS36.213[8], clause 4.2.1. Two successive indications from Layer 1 shall be separated by at least max([10]ms, DRX_cycle_length).

Upon start of T310 timer as specified in clause 5.3.11 in TS 36.331 [5], the UE shall monitor the link for recovery using the evaluation period and Layer 1 indication interval corresponding to the non-DRX mode until the expiry or stop of T310 timer.

The transmitter power of the UE shall be turned off within 40 ms after expiry of T310 timer as specified in clause 5.3.11 in TS 36.331 [5].

Table 7.3.69.3-2: Qout and Qin Evaluation Period in DRX for HD-FDD Category NB1 UE

| DRX cycle length (s) | TEvaluate_Qout_DRX_NB-IoT and TEvaluate_Qin_DRX_NB-IoT (s) |  |  |
| --- | --- | --- | --- |
|  | DRX cycles for Rmax<=64 | DRX cycles for Rmax>64 |
| 0.256 < DRX cycle ≤ 1.024 | Note 1 (20) | Note 1 (40) |  |
| 1.024 < DRX cycle ≤ 3.072 | Note 1 (10) | Note 1 (20) |  |
| 4.096 < DRX cycle ≤ 9.216 | Note 1 (5) | Note 1 (10) |  |
| NOTE 1: Evaluation period length in time depends on the length of the DRX cycle in use |  |  |  |

The normative reference for this requirement is TS 36.133 [4] clause 7.23 and A.7.3.69.

7.3.69.4	Test description

The test consists of one NB-IOT cell (Ncell 1)  configured. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure 7.3.1.4-1 shows the different time durations and the corresponding variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure:

-	Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 within dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH

NOTE 1:	The UE is expected to decode the NPDCCH and complete the UL transmission during T2 according to the UL grant.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 within dT

-	During T3, the SNR is kept as SNR3

NOTE 2:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

NOTE 3:	The UE is not expected to decode the UL grant and conduct any UL transmission during T4, since the UE is expected to declare RLF during T3.

In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode the NPDCCH and complete the UL transmission when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Figure 7.3.69.4-1: SNR variation for out-of-sync testing in DRX for NB-IoT HD-FDD out-of-sync testing for UE category NB1 configured under enhanced coverage

7.3.69.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table [E-1] and TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: As specified in Table 7.3.69.5-1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A figure A.51 using only main Tx/Rx antenna and without using the faders.

2.	The general test parameter settings are set up according to Table 7.3.69.4-1.

3.	Propagation conditions are set according to Annex B clauses B.0.

4.	Message contents are defined in clause 7.3.69.4.3

5.	There are two cells specified in this test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

Table 7.3.69.4.1-1: General test parameters for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Ncell 1 |  |
| CP length |  | Normal |  |
| Deployment Mode |  | Standalone |  |
| NPDCCH transmission parameters Rmax |  | 16 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.3.69.3-1 |
| DRX cycle | ms | 256 | See Table 7.3.69.5-2 |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 100 | T311 is enabled |
| T1 | s | 5.12 |  |
| T2 | s | 10.24 |  |
| T3 | s | 5.12 |  |
| dT | ms | 1000 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331[5]. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

7.3.69.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully be synchronized to Ncell 1. The UE is scheduled in every possible uplink subframe to transmit NPUSCH, which is received by the test equipment.

1.	Ensure the UE is in State 2A-NB according to TS 36.508 [7] clause 8.1.5.

2.	Set the parameters according to T1 in Table 7.3.69.5-1. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	When T1 expires, the 1st time window dT starts. During dT the SS shall change the SNR towards the SNR value as specified in Table 7.3.69.5-1 for T2, with SNR level changing by 1 dB in 0.1 second.

4.	When dT expires the SS shall change the SNR value to T2 (SNR2) as specified in Table 7.3.69.5-1. T2 starts.

5.	At the start of T2 the UE shall be provided with a UL grant on NPDCCH. The UE shall decode NPDCCH and complete the UL transmission, according to the UL grant, before T2 expires. If the UL transmission is not completed the number of failed test is increased by one, if so restart test from step 1.

6.	When T2 expires, the time 2nd window dT starts. During dT the SS shall change the SNR towards the SNR value as specified in Table 7.3.69.5-1 for T3, with SNR level changing by 1 dB in 0.1 second.

7.	When dT expires the SS shall change the SNR value to T3 (SNR3) as specified in Table 7.3.69.5-1. T3 starts.

8.	During T3 the UE is expected to detect OOS and declare RLM. T4 starts.

9.	At the start of T4 the UE shall be provided with another UL grant on NPDCCH. If the SS detects uplink power equal to or higher than -39 dBm in the On-duration part of every DRX cycle in the uplink subframe scheduled to transmit NPUSCH during the period T3 the number of failed tests is increased by one.

Otherwise the number of successful tests is increased by one.

10.	Repeat steps 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.3.69.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions:

Table 7.3.69.4.3-1: Common Exception messages for E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync with DRX for UE Category NB1 Standalone mode in enhanced coverage

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.9-1 |
| Default RRC messages and information elements contents exceptions |  |

7.3.69.5	Test requirement

Table 7.3.69.5-1: NCell specific test parameters for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage

| Parameter | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| NB-IoT RF Channel Number |  | 1 |  |  |  |
| BWchannel | KHz | 180 |  |  |  |
| NPDSCH parameters as defined in A.3.1.5.1 Note 2 |  | R.19 HD-FDD |  |  |  |
| NPDCCH parameters as defined in A.3.1.6.1 Note 2 |  | R.33 HD-FDD |  |  |  |
| NOCNG Patterns defined in A.3.2.3.3 |  | NOP.3 FDD |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |
| NPBCH_RB | dB |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98+TT |  |  |  |
| SNR Note 4, 5 | dB | -6.3+TT | -11.4+TT | -17.4+TT | -6.3+TT |
| Propagation condition |  | AWGN |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.61.1-1. Note 6: During dT between the defined Time Intervals, the Test system shall gradually increase or decrease the SNR until the target SNR is achieved. |  |  |  |  |  |

Table 7.3.69.5-2: DRX-Configuration for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.7.3 in TS 36.331 |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| drx-StartOffset | 0 |  |

Table 7.3.69.5-3: TimeAlignmentTimer -Configuration for NB-IoT HD-FDD out-of-sync testing for UE category NB1 configured under enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |

The UE behaviours in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4.

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

< End of changes >