3GPP TSG-RAN5 Meeting #80	R5-184352

# Gothenburg, Sweden, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 0021 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Test frequency for EN-DC B1_n78 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To include EN-DC Test frequency combination of DC_1A_n78A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update to TS 38.508-1 -Test frequency section to include EN-DC combination of DC_1A_n78A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to test EN-DC signalling test cases for DC_1A_n78A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1 and 4.3.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |

#### 4.3.1.1	Test frequencies for NR operating bands in FR1

##### 4.3.1.1.1	Operating bands in FR1

<Unchanged Contents Omitted >

Table 4.3.1.1.1.71.2-4: UL Test frequencies for NR operating band n71

| Test Frequency ID | Bandwidth [MHz] | Bandwidth [RBs] | offset pointA to carrier [RBs] |
| --- | --- | --- | --- |
| Low Range | 10 | 24 | 0 |
|  | 15 | 38 | 0 |
|  | 20 | 51 | 0 |
| Mid Range | 10 | 24 | 35 |
|  | 15 | 38 | 28 |
|  | 20 | 51 | 22 |
| High Range | 10 | 24 | 72 |
|  | 15 | 38 | 58 |
|  | 20 | 51 | 45 |

<Change1 Starts >

4.3.1.1.1.42	Reference test frequencies for NR operating band n78

4.3.1.1.1.42.1	SCS 15 kHz

Table 4.3.1.1.1.42.1-1: Common parameters for SCS 15 kHz in NR operating band n78 for Low and Mid Range

|  | point A [MHz] | point A [ARFCN] | k0 |
| --- | --- | --- | --- |
| DL | 3300.015 | 620001 | [0] |
| UL | 3300.015 | 620001 | [0] |

Table 4.3.1.1.1.42.1-2: Common parameters for SCS 15 kHz in NR operating band n78 for High Range

|  | point A [MHz] | point A [ARFCN] | k0 |
| --- | --- | --- | --- |
| DL | 3541.38 | 636092 | [0] |
| UL | 3541.38 | 636092 | [0] |

Table 4.3.1.1.1.42.1-3: SSB for SCS 15 kHz in NR operating band n78

| SSB ID | GSCN [N] | GSCN [MHz] | SSB [ARFCN] | kSSB | Bandwidth [MHz] | Bandwidth [RBs] | offset point A to SSB midpoint [RBs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
|  | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Mid Range | 7881 | 3550.56 | 636704 | FFS | 10 | 52 | FFS |
|  | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| High Range | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
|  | FFS | FFS | FFS | FFS | FFS | FFS | FFS |

Table 4.3.1.1.1.42.1-4: Test frequencies for NR operating band n78

| Test Frequency ID | Bandwidth [MHz] | Bandwidth [RBs] | offset pointA to carrier [RBs] |
| --- | --- | --- | --- |
| Low Range | 10 | 52 | 2 |
|  | 20 | 106 | 3 |
|  | 40 | 216 | 4 |
|  | 50 | 270 | 4 |
| Mid Range | 10 | 52 | 1365 |
|  | 20 | 106 | 1338 |
|  | 40 | 216 | 1283 |
|  | 50 | 270 | 1256 |
| High Range | 10 | 52 | 1382 |
|  | 20 | 106 | 1328 |
|  | 40 | 216 | 1217 |
|  | 50 | 270 | 1163 |

4.3.1.1.1.42.2	SCS 30 kHz

Table 4.3.1.1.1.42.2-1: Common parameters for SCS 30 kHz in NR operating band n78

|  | point A [MHz] | point A [ARFCN] | k0 |
| --- | --- | --- | --- |
| DL | 3300.03 | 620002 | [0] |
| UL | 3300.03 | 620002 | [0] |

Table 4.3.1.1.1.42.2-2: Common parameters for SCS 15 kHz in NR operating band n78 for High Range

|  | point A [MHz] | point A [ARFCN] | k0 |
| --- | --- | --- | --- |
| DL | 3541.38 | 636092 | [0] |
| UL | 3541.38 | 636092 | [0] |

Table 4.3.1.1.1.42.2-3: SSB for SCS 30 kHz in NR operating band n78

| SSB ID | GSCN [N] | GSCN [MHz] | SSB [ARFCN] | kSSB | Bandwidth [MHz] | Bandwidth [RBs] | offset point A to SSB midpoint [RBs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
|  | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Mid Range | 7881 | 3550.56 | 636704 | FFS | FFS | FFS | FFS |
|  | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| High Range | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
|  | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Note 1: kSSB is expressed in terms of 15 kHz subcarrier spacing |  |  |  |  |  |  |  |

Table 4.3.1.1.1.42.2-4: Test frequencies for NR operating band n78

| Test Frequency ID | Bandwidth [MHz] | Bandwidth [RBs] | offset pointA to carrier [RBs] |
| --- | --- | --- | --- |
| Low Range | 10 | 24 | 2 |
|  | 20 | 51 | 2 |
|  | 40 | 106 | 3 |
|  | 50 | 133 | 3 |
|  | 60 | 162 | 3 |
|  | 80 | 217 | 3 |
|  | 100 | 273 | 3 |
| Mid Range | 10 | 24 | 683 |
|  | 20 | 51 | 670 |
|  | 40 | 106 | 642 |
|  | 50 | 133 | 629 |
|  | 60 | 162 | 614 |
|  | 80 | 217 | 587 |
|  | 100 | 273 | 559 |
| High Range | 10 | 24 | 692 |
|  | 20 | 51 | 665 |
|  | 40 | 106 | 610 |
|  | 50 | 133 | 582 |
|  | 60 | 162 | 554 |
|  | 80 | 217 | 499 |
|  | 100 | 273 | 443 |

4.3.1.1.1.42.3	SCS 60 kHz

Table 4.3.1.1.1.42.3-1: Common parameters for SCS 60 kHz in NR operating band n78

|  | point A [MHz] | point A [ARFCN] | k0 |
| --- | --- | --- | --- |
| DL | 3300.045 | 620003 | [0] |
| UL | 3300.045 | 620003 | [0] |

Table 4.3.1.1.1.42.3-2: Common parameters for SCS 15 kHz in NR operating band n78 for High Range

|  | point A [MHz] | point A [ARFCN] | k0 |
| --- | --- | --- | --- |
| DL | 3541.38 | 636092 | [0] |
| UL | 3541.38 | 636092 | [0] |

Table 4.3.1.1.1.42.3-3: SSB for SCS 60 kHz in NR operating band n78

| SSB ID | GSCN [N] | GSCN [MHz] | SSB [ARFCN] | kSSB | Bandwidth [MHz] | Bandwidth [RBs] | offset point A to SSB midpoint [RBs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
|  | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Mid Range | 7881 | 3550.56 | 636704 | FFS | FFS | FFS | FFS |
|  | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| High Range | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
|  | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Note 1: is expressed in terms of 15 kHz subcarrier spacing |  |  |  |  |  |  |  |

Table 4.3.1.1.1.42.3-4: Test frequencies for NR operating band n78

| Test Frequency ID | Bandwidth [MHz] | Bandwidth [RBs] | offset pointA to carrier [RBs] |
| --- | --- | --- | --- |
| Low Range | 10 | 11 | 2 |
|  | 20 | 24 | 2 |
|  | 40 | 51 | 2 |
|  | 50 | 65 | 2 |
|  | 60 | 79 | 2 |
|  | 80 | 107 | 2 |
|  | 100 | 135 | 2 |
| Mid Range | 10 | 11 | 342 |
|  | 20 | 24 | 335 |
|  | 40 | 51 | 322 |
|  | 50 | 65 | 315 |
|  | 60 | 79 | 308 |
|  | 80 | 107 | 294 |
|  | 100 | 135 | 280 |
| High Range | 10 | 11 | 347 |
|  | 20 | 24 | 333 |
|  | 40 | 51 | 306 |
|  | 50 | 65 | 292 |
|  | 60 | 79 | 278 |
|  | 80 | 107 | 250 |
|  | 100 | 135 | 222 |

<Change1 Ends >

#### 4.3.1.3	Test frequencies for NR operation with other radios

<Unchanged Contents Omitted >

<Change2 Starts >

4.3.1.3.2.26	Reference test frequencies for EN-DC combinations beginning with B1

4.3.1.3.2.26.1	Reference test frequencies for EN-DC combination DC_1A_n78A / DC_1A_n78A

The test frequencies for B1A are found in TS 36.508 [2] Table 4.3.1.1.1-1.

The test frequencies for n78A are found in sub clause 4.3.1.1.1.42:

<Change2 ends >