3GPP TSG RAN WG5#81 	R5-186763

# Spokane, USA, 12 - 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1210 | rev | <Rev> | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SRB4 configuration to RRC connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_QMC_Streaming-UEConTest |  |  |  |  |  | Date: |  |  | 2018-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The LTE QMC signal is transmitted on SRB4, therefore it needs to setup SRB4 in the RRC connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Addition of condition SRB4 in RRCConnectionReconfiguration; 2 Addition of SRB4-ToAddMod-DEFAULT in RadioResourceConfigDedicated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LTE QMC signal testing cases are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1, 4.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<The Unchanged Contents Omitted>

### 4.6.1	Contents of RRC messages

<The Unchanged Contents Omitted>

<Change 1 starts>

#### –	RRCConnectionReconfiguration

| Table 4.6.1-8: RRCConnectionReconfigurationDerivation Path: 36.331 clause 6.2.2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier |  |  | RRC-TransactionIdentifier-DL |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |  |
| measConfig |  |  | Not present |  |  |  |  |
|  |  |  | MeasConfig-DEFAULT |  |  |  | MEAS |
| mobilityControlInfo |  |  | Not present |  |  |  |  |
|  |  |  | MobilityControlInfo-HO |  |  |  | HO, HO-TO-EUTRA |
| dedicatedInfoNASList |  |  | Not present |  |  |  |  |
|  |  |  | Set according to specific message content |  |  |  | SRB2-DRB(n, m) DRB(n, m) SRB1-SRB2-DRB(n,m) AM-DRB-ADD(bid) UM-DRB-ADD(bid) DRB-REL(bid) DRB-Mod MCG_and_split EN-DC_SRB2-DRB |
| radioResourceConfigDedicated |  |  | Not present |  |  |  |  |
|  |  |  | RadioResourceConfigDedicated-SRB2-DRB(n, m) |  |  |  | SRB2-DRB(n, m) |
|  |  |  | RadioResourceConfigDedicated-DRB(n, m) |  |  |  | DRB(n, m) |
|  |  |  | RadioResourceConfigDedicated-AM-DRB-ADD(bid) |  |  |  | AM-DRB-ADD(bid) |
|  |  |  | RadioResourceConfigDedicated-UM-DRB-ADD(bid) |  |  |  | UM-DRB-ADD(bid) |
|  |  |  | RadioResourceConfigDedicated-DRB-REL(bid) |  |  |  | DRB-REL(bid) |
|  |  |  | RadioResourceConfigDedicated-HO |  |  |  | HO |
|  |  |  | RadioResourceConfigDedicated-HO-TO-EUTRA(n, m) |  |  |  | HO-TO-EUTRA(n,m) |
|  |  |  | RadioResourceConfigDedicated-DRB-Mod |  |  |  | DRB-Mod |
|  |  |  | RadioResourceConfigDedicated-SCell_AddMod |  |  |  | SCell_AddMod |
|  |  |  | RadioResourceConfigDedicated-PCell-PATTERN |  |  |  | PCell-PATTERN |
|  |  |  | RadioResourceConfigDedicated-EN-DC-Split |  |  |  | MCG_and_split |
|  |  |  | RadioResourceConfigDedicated-EN-DC_Split_DRB_Rel(bid) |  |  |  | EN-DC_Split_Rel(bid) |
|  |  |  | RadioResourceConfigDedicated-SRB4 |  |  |  | SRB4 |
|  |  |  | RadioResourceConfigDedicated-EN-DC-SRB2-DRB |  |  |  | EN-DC_SRB2-DRB |
| securityConfigHO |  |  | Not present |  |  |  |  |
|  |  |  | SecurityConfigHO-DEFAULT |  |  |  | HO, HO-TO-EUTRA |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |  |
| lateNonCriticalExtension ::= SEQUENCE { |  |  |  |  | 4Rx_PCC |  |
| lateNonCriticalExtension | Not present |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| antennaInfoDedicatedPCell-v10i0 ::= SEQUENCE { |  |  |  |  |  |  |
| maxLayersMIMO-r10 | fourLayers |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| otherConfig-r9 | OtherConfig-r9 |  |  |  | OtherConfig |  |
| fullConfig-r9 | Not present |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |  |  |  |
| sCellToAddModList-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF | The same number of entries as the configured SCell(s) |  | n denotes the index of the entry |  | SCell_AddMod |  |
| sCellToAddMod-r10[n] | SCellToAddMod-r10-DEFAULT |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| systemInfomationBlockType1Dedicated-r11 | Not present |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| wlan-OffloadInfo-r12 CHOICE {} | Not present |  |  |  |  |  |
| wlan-OffloadInfo-r12 CHOICE { |  |  |  |  |  |  |
| release | NULL |  |  |  | WLAN-OffloadRelease |  |
| setup ::= SEQUENCE { |  |  |  |  | WLAN-OffloadSetup |  |
| wlan-OffloadConfigDedicated-r12 | WLAN-OffloadConfig-DEFAULT |  |  |  |  |  |
| t350-r12 | min10 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| scg-Configuration-r12 | Not present |  |  |  |  |  |
| scg-Configuration-r12 | SCG-Configuration-r12-DEFAULT |  |  |  | PSCell_Release, PSCell_Mod, PSCell_Add_Split_DRB, PSCell_Add_SCG_DRB |  |
| sl-SyncTxControl-r12 | Not present |  |  |  |  |  |
| sl-DiscConfig-r12 | Not present |  |  |  |  |  |
| sl-CommConfig-r12 | Not present |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE {} | Not present |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  | MCG_and_SCG, MCG_and _split, SCG-DRB(n,m), MCG_NR_PDCP, EN-DC_PSCell_Rel, SRB_NR_PDCP |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |  |  |  |
| release |  |  |  |  | EN-DC_PSCell_Rel |  |
| setup SEQUENCE { |  |  |  |  | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m), MCG_NR_PDCP |  |
| endc-ReleaseAndAdd-r15 | false |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-3 with condition EN-DC. |  |  |  | MCG_and_SCG, |  |
|  | OCTET STRING including the RRCReconfiguration message, the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-3 with condition EN-DC and the IE PDCP-Config according TS 38.508-1 [67], table 4.6.3-74 with condition Split. |  |  |  | MCG_and_split |  |
|  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table [4.8.1-n]. |  |  |  | SCG-DRB(n,m) |  |
| p-MaxEUTRA-r15 | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| sk-Counter-r15 | 0 |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], [table 4.6.3-n] with condition EN-DC. |  |  |  | MCG_and_SCG, MCG_and_split |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table [4.8.1-n]. |  |  |  | SCG-DRB(n,m) |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table [4.8.1-n] with condition MCG_NR_PDCP |  |  |  | MCG_NR_PDCP |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-100 with condition SRB_NR_PDCP |  |  |  | SRB_NR_PDCP |  |
| nr-RadioBearerConfig2-r15 | Not present |  |  |  |  |  |
| tdm-PatternConfig-r15 | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB2-DRB(n,m) | Establishment of a SRB and DRB combination with n x AM DRB and m x UM DRB (including establishment of SRB2) |
| DRB(n,m) | Establishment of additional n x AM DRB:s and m x UM DRB:s (SRB2 should already be established) |
| AM-DRB-ADD(bid) | Establishment of a single additional AM DRB with bearer identity bid (SRB2 should already be established) |
| UM-DRB-ADD(bid) | Establishment of a single additional UM DRB with bearer identity bid (SRB2 should already be established) |
| DRB-REL(bid) | Release of the DRB with bearer identity bid |
| HO | Intra LTE handover |
| MEAS | A measurement is configured |
| HO-TO-EUTRA(n,m) | Inter-RAT handover to E-UTRA including the establishment of a SRB1, SRB2 and n x AM DRB plus m x UM DRB |
| nonFullConfig | The field is not present in case of handover within E-UTRA when the fullConfig is included or in case of handover of handover to E-UTRA; otherwise it is optional present, need ON. |
| DRB-Mod | Modification of already established DRB ID 2 and is used for sending Modify EPS Bearer Context Request message (SRB2 should already be established) |
| SCell_AddMod | Addition or modification of SCell(s) |
| PCell-PATTERN | eICIC Serving Cell Pattern |
| OtherConfig | Other Configuration is configured |
| WLAN-OffloadSetup | When Dedicated WLAN Offload parameters need to be configured in UE |
| WLAN-OffloadRelease | When Dedicated WLAN Offload parameters need to be released in UE |
| PSCell_Mod | Modification of PSCell(s) |
| PSCell_Release | Release of PSCell |
| PSCell_Add_Split_DRB | Add PSCell(s) and setup of Split DRB |
| PSCell_Add_SCG_DRB | Add PSCell(s) and setup of SCG DRB |
| 4Rx_PCC | For 4Rx testcases with more than 2 Layers on PCC |
| MCG_and_SCG | EN-DC with MCG and SCG. |
| MCG_and_split | EN-DC with MCG and split. |
| SCG-DRB(n,m) | EN-DC establishment of additional n x AM DRB:s and m x UM DRB:s in SCG. |
| EN-DC_PSCell_Rel | Release of EN-DC PSCell |
| EN-DC_Split_Rel(bid) | Release of E-UTRA leg of EN-DC Split DRB with bearer identity bid |
| MCG_NR_PDCP | EN-DC configuration of MCG DRB with NR PDCP. |
| SRB_NR_PDCP | EN-DC configuration of SRB1 and SRB2 with NR PDCP. |
| EN-DC_SRB2-DRB | Establishment of SRB2 and DRB combination for EN-DC |
| SRB4 | Establishment of SRB4 |

<Change 1 ends>

<The Unchanged Contents Omitted>

<Change 2 starts>

#### –	RadioResourceConfigDedicated-SRB2-DRB(n,m)

Table 4.6.3-16: RadioResourceConfigDedicated-SRB2-DRB(n,m)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB2-DRB(n, m) ::= SEQUENCE { |  |  | n is the number of AM RLC DRBs (1..N) m is the number of UM RLC DRBs (0..M) |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SEQUENCE { |  | 1 entry |  |  |
| srb-ToAddMod[1] |  | SRB-ToAddMod-DEFAULT using condition SRB2 | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | n+m entries |  |  |
| drb-ToAddMod[k, k=1..n] |  | DRB-ToAddMod-DEFAULT (k) using condition AM | n AM RLC DRBs See subclause 4.8.2 |  |
| drb-ToAddMod[k, k=n+1..n+m] |  | DRB-ToAddMod-DEFAULT (k) using condition UM | m UM RLC DRBs See subclause 4.8.2 | m>0 |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue |  | MAC-MainConfig-RBC using condition DRX_L (note) | See subclause 4.8.2 | m=0 |
|  |  | MAC-MainConfig-RBC using condition DRX_S (note) | See subclause 4.8.2 | m>0 |
| } |  |  |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | PhysicalConfigDedicated-DEFAULT using condition RBC | For signalling test cases see subclause 6.6B.1.1.1. Otherwise, see subclause 4.8.2 |  |
| } |  |  |  |  |
| NOTE: In cases where no RLC-UM bearer is configured, large DRX Cycle length is used. In cases where at least one RLC-UM bearer is configured, small DRX Cycle length is used. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| m=0 | m is equal to zero |
| m>0 | m is greater than zero |

#### –	RadioResourceConfigDedicated-DRB(n,m)

Table 4.6.3-17: RadioResourceConfigDedicated-DRB(n,m)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-DRB(n, m) ::= SEQUENCE { |  |  | n is the number of AM RLC DRBs (0..N) m is the number of UM RLC DRBs (0..M) |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | n+m entries | BID is the total number of established DRBs in the UE, before applying the contents of this IE |  |
| drb-ToAddMod[k, k=BID+1..BID+n] |  | DRB-ToAddMod-DEFAULT (k) using condition AM | n AM RLC DRBs See subclause 4.8.2 | n>0 |
| drb-ToAddMod[k, k= BID+1+n.. BID+n+m] |  | DRB-ToAddMod-DEFAULT (k) using condition UM | m UM RLC DRBs See subclause 4.8.2 | m>0 |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig |  | Not present (note) |  | m=0 |
| mac-MainConfig CHOICE { |  |  |  | m>0 |
| explicitValue |  | MAC-MainConfig-RBC using condition DRX_S (note) | See subclause 4.8.2 |  |
| } |  |  |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | Not present |  |  |
| } |  |  |  |  |
| NOTE: In cases where no RLC-UM bearer is added, existing DRX configuration is used without modification. In cases where at least one RLC-UM bearer is added, small DRX Cycle length is used. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| n>0 | n is greater than zero |
| m=0 | m is equal to zero |
| m>0 | m is greater than zero |

#### –	RadioResourceConfigDedicated-SRB4

Table 4.6.3-17A: RadioResourceConfigDedicated-SRB4

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB4 ::= SEQUENCE { |  |  |  |  |
| srb-ToAddModExtList-r15 SEQUENCE (SIZE (1)) OF SEQUENCE { |  | 1 entry |  |  |
| srb-ToAddMod |  | SRB4-ToAddMod-DEFAULT | See subclause 4.8.2 |  |
| } |  |  |  |  |
| } |  |  |  |  |

<Change2 ends>

<The Unchanged Contents Omitted>

<Change 3 starts>

### 4.8.2	SRB and DRB parameters and combinations

#### 4.8.2.1	SRB and DRB parameters

##### 4.8.2.1.1	SRB configurations

Table 4.8.2.1.1-1: SRB-ToAddMod-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SRB-ToAddMod-DEFAULT ::= SEQUENCE { |  |  |  |  |
| srb-Identity |  | 1 |  | SRB1 |
|  |  | 2 |  | SRB2 |
| rlc-Config CHOICE { |  |  |  |  |
| defaultValue |  |  |  |  |
| } |  |  |  |  |
| logicalChannelConfig CHOICE { |  |  |  |  |
| defaultValue |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.8.2.1.1-2: SRB4-ToAddMod-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SRB-ToAddMod-DEFAULT ::= SEQUENCE { |  |  |  |  |
| rlc-Config CHOICE { |  |  |  |  |
| am SEQUENCE { |  |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |  |
| t-PollRetransmit |  | ms45 |  |  |
| pollPDU |  | infinity |  |  |
| pollByte |  | infinity |  |  |
| maxRetxThreshold |  | t4 |  |  |
| } |  |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |  |
| t-Reordering |  | ms35 |  |  |
| t-StatusProhibit |  | ms0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logicalChannelConfig CHOICE { |  |  |  |  |
| priority |  | 15 |  |  |
| prioritisedBitRate |  | infinity |  |  |
| bucketSizeDuration |  | N/A |  |  |
| logicalChannelGroup |  | 0 |  |  |
| } |  |  |  |  |
| srb-Identity-v1530 |  | 4 |  |  |
| } |  |  |  |  |

<Change 3 ends>