3GPP TSG-RAN5 Meeting #81	R5-187194

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4695 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to LWA TC 8.2.5.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation (UK) Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_WLAN_radio-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Update test case to incorporate generic procedures |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove TP 3; update FFS power level; added new Step 8 in Main Behaviour; updated message contents to reflect newly defined generic procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |

**** Beginning of Revisions *****

#### 8.2.5.7	LWIP / WLAN Release Success / EUTRA RRC_Connected from WLAN (Event W3)

8.2.5.7.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W3 reporting }

ensure that {
  when { UE  detects entering condition for Event W3 is met }

then { UE transmits a measurement report }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and configured with a WLAN mobility set }

ensure that {
  when { UE  receives RRCConnectionReconfiguration including wlanToReleaseList that includes WLAN Identifiers that are part of the current WLAN mobility set }

then { UE releases the WLAN association and sends RRCConnectionReconfigurationComplete  }

}

}

8.2.5.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.13 and 5.6.17.2

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends

[TS 36.331, clause 5.5.4.13]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition W3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition W3-2, as specified below, is fulfilled;

Inequality W3-1 (Entering condition)

Inequality W3-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of WLAN(s) which matches all WLAN identifiers of at least one entry within wlan-MobilitySet in VarWLAN-MobilityConfig, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. w3-Threshold as defined within reportConfigInterRAT for this event).

Ms is expressed in dBm.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

[TS 36.331, clause 5.6.17.2]

The UE shall:

1>	if the received lwip-Configuration is set to release:

2>	release the LWIP configuration, if configured, as described in 5.6.17.3;

1>	else:

2>	if lwip-MobilityConfig is included:

3>	if the received lwip-MobilityConfig includes wlan-ToReleaseList:

4>	for each WLAN-Identifiers included in wlan-ToReleaseList:

5>	remove the WLAN-Identifiers if already part of the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwip-MobilityConfig includes wlan-ToAddList:

4>	for each WLAN-Identifiers included in wlan-ToAddList:

5>	add the WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwip-MobilityConfig includes associationTimer:

4>	start timer T351 with the timer value set according to the value of associationTimer;

3>	if the received lwip-MobilityConfig includes successReportRequested:

4>	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

3>	start WLAN Status Monitoring as described in 5.6.15.4;

2>	if tunnelConfigLWIP is included:

3>	indicate to higher layers to configure the LWIP tunnel according to the received tunnelConfigLWIP [32];

2>	if lwip-Counter is included:

3>	determine the LWIP-PSK based on the KeNB key and received lwip-Counter value, as specified in TS 33.401 [32];

3>	forward the LWIP-PSK to upper layers for LWIP tunnel establishment;

8.2.5.7.3	Test description

8.2.5.7.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1 and  WLAN Cell 27

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.5.7.3.2	 Test procedure sequence

Table 8.2.5.7.3.2-1 illustrates the downlink power levels to be applied for Cell 1 and Cell 27 at various time instants of the test execution. Rows marked "T1", "T2" and “T3” are to be applied in the test procedure. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.5.7.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | WLAN Cell 27 is switched on |
|  | RSRQ | dB | -4.15 | - |  |
|  | BeaconRSSI |  | - | -60 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that condition for entering Event W1 is met |
|  | RSRQ | dB | -4.15 | - |  |
|  | BeaconRSSI |  | - | -50 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that condition for entering Event W3 is met |
|  | RSRQ | dB | -4.15 | - |  |
|  | BeaconRSSI |  | - | -80 |  |

Table 8.2.5.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row "T1" in table 8.2.5.7.3.2-1 | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W1) | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The SS changes Cell parameters according to the row "T2" in table 8.2.5.7.3.2-1. | - | - | - | - |
| 5 | UE transmits a MeasurementReport message to report the Event W1 | --> | MeasurementReport | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 8 | The UE transmits a WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry | --> | WLANConnectionStatusReport | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W3) | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS changes Cell parameters according to the row "T3" in table 8.2.5.7.3.2-1. | - | - | - | - |
| 12 | Check: Does the UE transmit a MeasurementReport message to report the Event W3 | --> | MeasurementReport | 1 | P |
| 13 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToReleaseList | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmits RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | 2 | P |

8.2.5.7.3.3	Specific message contents

Table 8.2.5.7.3.3-1: RRCConnectionReconfiguration (step 2 and 9 Table 8.2.5.7.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { | 1 entry |  |  |
| measConfig | Set as per Table 8.2.5.7.3.3-2 for Step 2. Set as per Table 8.2.5.7.3.3-7 for Step 9. |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-2: MeasConfig (step 2, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | -1 | IdMeasObject |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.7.3.3-3 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| reportConfig[1] | ReportConfigInterRAT-W1-WLAN(-55) | Note: Set as per T2 setting for WLAN RSSI minus 5db in Table 8.2.5.7.3.2-1. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-3: MeasObjectWLAN-GENERIC (step 2, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 (Add Table in test case) | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| ssid-r12[2] | Not Present |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-4: ReportConfigInterRAT-W1-WLAN (step 2, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |  |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |  |
| triggerType CHOICE { |  |  |  |  |
| event SEQUENCE { | 1 entry |  |  |  |
| eventID CHOICE { |  |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |  |
| w1-Threshold-r13 | 46 | ThreshDB+101=46 where ThreshDB = -55dBm. Note: Set as per T2 setting for WLAN RSSI minus 5db in Table 8.2.5.7.3.2-1. |  |  |
| } |  |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |  |
| timeToTrigger | ms5120 | 5120ms |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.5.7.3.3-5: RRCConnectionReconfiguration (step 6 Table 8.2.5.7.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWIP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| measConfig |  | Not Present |  |  |
| mobilityControlInfo |  | Not Present |  |  |
| dedicatedInfoNASList |  | Not Present |  |  |
| radioResourceConfigDedicated |  | Not Present |  |  |
| securityConfigHO |  | Not Present |  |  |
| nonCriticalExtension{ |  |  | v890 |  |
| nonCriticalExtension{ |  |  | v920 |  |
| nonCriticalExtension{ |  |  | v1020 |  |
| nonCriticalExtension{ |  |  | v1130 |  |
| nonCriticalExtension{ |  |  | v1250 |  |
| nonCriticalExtension{ |  |  | v1310 | LWIP |
| Lwip-configuration-r13 |  | LWIP-Configuration-GENERIC(SSID) Set as per Table 8.2.5.7.3.3-6 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.5.7.3.3-6: LWIP-Configuration-GENERIC (step 6 Table 8.2.5.7.3.2-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-33 with condition ADD, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | Not Present |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  |  |
| successReportRequested-r13 | true |  |  |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-7: MeasConfig (step 9, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 | IdMeasObject-f20 |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.7.3.3-8 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W3 |  |
| reportConfig[1] | ReportConfigInterRAT-W3-WLAN(-75) | See Table 8.2.5.7.3.3-9 |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 | IdMeasObject-f20 |  |
| reportConfigId[1] | 1 | IdReportConfig-W3 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-8: MeasObjectWLAN-GENERIC (step 9, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | sValue/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-9: ReportConfigInterRAT-W3-WLAN (step 9, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW3-r13 SEQUENCE { | 1 entry |  |  |
| W3-Threshold-r13 | 26 | ThreshDB+101=26 where ThreshDB = -75dBm. Note: Set as per T3 setting for WLAN RSSI plus 5db in Table 8.2.5.7.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-10: RRCConnectionReconfiguration (step 13 Table 8.2.5.7.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWIP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 | LWIP |
| Lwip-configuration-r13 | LWIP-Configuration-GENERIC(SSID) Set as per Table 8.2.5.7.3.3-11 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-11: LWIP-Configuration-GENERIC (step 13 Table 8.2.5.7.3.2-1)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-33 with condition RELEASE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| LWIP-Config-r13 SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 OF SEQUENCE { | 1 entry |  | RELEASE |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 | Not Present |  |  |
| associationTimer-r13 | Not Present |  |  |
| successReportRequested-r13 | Not Present |  |  |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

**** End of Revisions *****