3GPP TSG-RAN5 Meeting #81	R5-187201

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1220 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of support for LWA/LWIP Test Cases in generic procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation (UK) Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_WLAN_radio-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Generic Procedures need to be updated to support LWA/LWIP test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of Generic Procedures for LWA/LWIP test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LWA/LWIP test cases would not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.8, 4.6.1, 4.6.3, 4.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |

<<< Beginning of Changes >>>>

### 4.4.8	Common parameters for simulated WLAN AP’s

<<Insert after Table 4.4.8-1>>

Table 4.4.8-2: Default parameters for simulated WLAN APs (2nd WLAN)

| Parameter | Value | Condition |
| --- | --- | --- |
| SSID | 3GPP WLAN InterWorking 2 |  |
| Security | [WPA2] |  |
| Hot Spot 2.0 Support | Yes |  |
| Trusted/Un-Trusted non 3GPP Access | Un-Trusted |  |
| EAP-AKA Procedure | CHAP |  |
| Beacon RSSI | 42 (-86 dBm), NOTE 1 | OFFLOAD |
| Beacon RSSI | Configured at the SS according to PIXIT parameter | IMSoWLAN |
| NOTE 1: The power level is specified at each UE Rx antenna. |  |  |

| Condition | Explanation |
| --- | --- |
| OFFLOAD | WLAN offload scenarios |
| IMSoWLAN | IMS over WLAN scenarios |

### 4.6.1	Contents of RRC messages

#### –	RRCConnectionReconfiguration

Table 4.6.1-8: RRCConnectionReconfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier |  |  | RRC-TransactionIdentifier-DL |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |  |
| measConfig |  |  | Not present |  |  |  |  |
|  |  |  | MeasConfig-DEFAULT |  |  |  | MEAS |
| mobilityControlInfo |  |  | Not present |  |  |  |  |
|  |  |  | MobilityControlInfo-HO |  |  |  | HO, HO-TO-EUTRA |
| dedicatedInfoNASList |  |  | Not present |  |  |  |  |
|  |  |  | Set according to specific message content |  |  |  | SRB2-DRB(n, m) DRB(n, m) SRB1-SRB2-DRB(n,m) AM-DRB-ADD(bid) UM-DRB-ADD(bid) DRB-REL(bid) DRB-Mod |
| radioResourceConfigDedicated |  |  | Not present |  |  |  |  |
|  |  |  | RadioResourceConfigDedicated-SRB2-DRB(n, m) |  |  |  | SRB2-DRB(n, m) |
|  |  |  | RadioResourceConfigDedicated-DRB(n, m) |  |  |  | DRB(n, m) |
|  |  |  | RadioResourceConfigDedicated-AM-DRB-ADD(bid) |  |  |  | AM-DRB-ADD(bid) |
|  |  |  | RadioResourceConfigDedicated-UM-DRB-ADD(bid) |  |  |  | UM-DRB-ADD(bid) |
|  |  |  | RadioResourceConfigDedicated-DRB-REL(bid) |  |  |  | DRB-REL(bid) |
|  |  |  | RadioResourceConfigDedicated-HO |  |  |  | HO |
|  |  |  | RadioResourceConfigDedicated-HO-TO-EUTRA(n, m) |  |  |  | HO-TO-EUTRA(n,m) |
|  |  |  | RadioResourceConfigDedicated-DRB-Mod |  |  |  | DRB-Mod |
|  |  |  | RadioResourceConfigDedicated-SCell_AddMod |  |  |  | SCell_AddMod |
|  |  |  | RadioResourceConfigDedicated-PCell-PATTERN |  |  |  | PCell-PATTERN |
|  |  |  | RadioResourceConfigDedicated-EN-DC-Split |  |  |  | MCG_and_split |
|  |  |  | RadioResourceConfigDedicated-EN-DC_Split_DRB_Rel(bid) |  |  |  | EN-DC_Split_Rel(bid) |
| securityConfigHO |  |  | Not present |  |  |  |  |
|  |  |  | SecurityConfigHO-DEFAULT |  |  |  | HO, HO-TO-EUTRA |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |  |
| lateNonCriticalExtension ::= SEQUENCE { |  |  |  |  | 4Rx_PCC |  |
| lateNonCriticalExtension | Not present |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| antennaInfoDedicatedPCell-v10i0 ::= SEQUENCE { |  |  |  |  |  |  |
| maxLayersMIMO-r10 | fourLayers |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| otherConfig-r9 | OtherConfig-r9 |  |  |  | OtherConfig |  |
| fullConfig-r9 | Not present |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |  |  |  |
| sCellToAddModList-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF | The same number of entries as the configured SCell(s) |  | n denotes the index of the entry |  | SCell_AddMod |  |
| sCellToAddMod-r10[n] | SCellToAddMod-r10-DEFAULT |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| systemInfomationBlockType1Dedicated-r11 | Not present |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| wlan-OffloadInfo-r12 CHOICE {} | Not present |  |  |  |  |  |
| wlan-OffloadInfo-r12 CHOICE { |  |  |  |  |  |  |
| release | NULL |  |  |  | WLAN-OffloadRelease |  |
| setup ::= SEQUENCE { |  |  |  |  | WLAN-OffloadSetup |  |
| wlan-OffloadConfigDedicated-r12 | WLAN-OffloadConfig-DEFAULT |  |  |  |  |  |
| t350-r12 | min10 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| scg-Configuration-r12 | Not present |  |  |  |  |  |
| scg-Configuration-r12 | SCG-Configuration-r12-DEFAULT |  |  |  | PSCell_Release, PSCell_Mod, PSCell_Add_Split_DRB, PSCell_Add_SCG_DRB |  |
| sl-SyncTxControl-r12 | Not present |  |  |  |  |  |
| sl-DiscConfig-r12 | Not present |  |  |  |  |  |
| sl-CommConfig-r12 | Not present |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE {} | Not present |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  | LWA |  |
| lwa-configuration-r13 |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  | LWIP |  |
| lwip-configuration-r13 |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  | MCG_and_SCG, MCG_and _split, SCG-DRB(n,m)MCG_NR_PDCP, EN-DC_PSCell_Rel |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |  |  |  |
| release |  |  |  |  | EN-DC_PSCell_Rel |  |
| setup SEQUENCE { |  |  |  |  | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m), MCG_NR_PDCP |  |
| endc-ReleaseAndAdd-r15 | false |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-3 with condition EN-DC. |  |  |  | MCG_and_SCG, |  |
|  | OCTET STRING including the RRCReconfiguration message, the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-3 with condition EN-DC and the IE PDCP-Config according TS 38.508-1 [67], table 4.6.3-74 with condition Split. |  |  |  | MCG_and_split |  |
|  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table [4.8.1-n]. |  |  |  | SCG-DRB(n,m) |  |
| p-MaxEUTRA-r15 | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| sk-Counter-r15 | 0 |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], [table 4.6.3-n] with condition EN-DC. |  |  |  | MCG_and_SCG, MCG_and_split |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table [4.8.1-n]. |  |  |  | SCG-DRB(n,m) |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table [4.8.1-n] with condition MCG_NR_PDCP |  |  |  | MCG_NR_PDCP |  |
| nr-RadioBearerConfig2-r15 | Not present |  |  |  |  |  |
| tdm-PatternConfig-r15 | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB2-DRB(n,m) | Establishment of a SRB and DRB combination with n x AM DRB and m x UM DRB (including establishment of SRB2) |
| DRB(n,m) | Establishment of additional n x AM DRB:s and m x UM DRB:s (SRB2 should already be established) |
| AM-DRB-ADD(bid) | Establishment of a single additional AM DRB with bearer identity bid (SRB2 should already be established) |
| UM-DRB-ADD(bid) | Establishment of a single additional UM DRB with bearer identity bid (SRB2 should already be established) |
| DRB-REL(bid) | Release of the DRB with bearer identity bid |
| HO | Intra LTE handover |
| MEAS | A measurement is configured |
| HO-TO-EUTRA(n,m) | Inter-RAT handover to E-UTRA including the establishment of a SRB1, SRB2 and n x AM DRB plus m x UM DRB |
| nonFullConfig | The field is not present in case of handover within E-UTRA when the fullConfig is included or in case of handover of handover to E-UTRA; otherwise it is optional present, need ON. |
| DRB-Mod | Modification of already established DRB ID 2 and is used for sending Modify EPS Bearer Context Request message (SRB2 should already be established) |
| SCell_AddMod | Addition or modification of SCell(s) |
| PCell-PATTERN | eICIC Serving Cell Pattern |
| OtherConfig | Other Configuration is configured |
| WLAN-OffloadSetup | When Dedicated WLAN Offload parameters need to be configured in UE |
| WLAN-OffloadRelease | When Dedicated WLAN Offload parameters need to be released in UE |
| PSCell_Mod | Modification of PSCell(s) |
| PSCell_Release | Release of PSCell |
| PSCell_Add_Split_DRB | Add PSCell(s) and setup of Split DRB |
| PSCell_Add_SCG_DRB | Add PSCell(s) and setup of SCG DRB |
| 4Rx_PCC | For 4Rx testcases with more than 2 Layers on PCC |
| MCG_and_SCG | EN-DC with MCG and SCG. |
| MCG_and _split | EN-DC with MCG and split. |
| SCG-DRB(n,m) | EN-DC establishment of additional n x AM DRB:s and m x UM DRB:s in SCG. |
| EN-DC_PSCell_Rel | Release of EN-DC PSCell |
| EN-DC_Split_Rel(bid) | Release of E-UTRA leg of EN-DC Split DRB with bearer identity bid |
| MCG_NR_PDCP | Configuration of MCG with NR PDCP version. |
| LWA | LWA is being configured |
| LWIP | LWIP is being configured |

### 4.6.3	Radio resource control information elements

[insert at end of 4.6.3]

#### –	LWIP-Configuration-GENERIC

Table 4.6.3-33: LWIP-Configuration-GENERIC (SSID)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 OF SEQUENCE { | 1 entry |  | RELEASE |
| ssid-r12[1] | SSID |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  | ADD |
| ssid-r12[1] | SSID |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 | 10 seconds | SUCCESS |
| successReportRequested-r13 | true |  | SUCCESS |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| ADD | Add a WLAN entry to the mobility configuration |
| RELEASE | Release a WLAN entry from the mobility configuration |
| SUCCESS | Request a WLAN status report after association |

#### –	LWA-Configuration-GENERIC

Table 4.6.3-34: LWA-Configuration-GENERIC (SSID)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 OF SEQUENCE { | 1 entry |  | RELEASE |
| ssid-r12[1] | SSID |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  | ADD |
| ssid-r12[1] | SSID |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 | 10 seconds | SUCCESS |
| successReportRequested-r13 | true |  | SUCCESS |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| ADD | Add a WLAN entry to the mobility configuration |
| RELEASE | Release a WLAN entry from the mobility configuration |
| SUCCESS | Request a WLAN status report after association |

#### –	LWIP-Configuration-W2-GENERIC

Table 4.6.3-35: LWIP-Configuration-W2-GENERIC (SSID1, SSID2)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | SSID1 |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | SSID2 |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 | 10 seconds | SUCCESS |
| successReportRequested-r13 | true |  | SUCCESS |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUCCESS | Request a WLAN status report after association |

#### –	LWA-Configuration-W2-GENERIC

Table 4.6.3-36: LWA-Configuration-W2-GENERIC (SSID1, SSID2)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | SSID1 |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | SSID2 |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 | 10 seconds | SUCCESS |
| successReportRequested-r13 | true |  | SUCCESS |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUCCESS | Request a WLAN status report after association |

### 4.6.6	Measurement information elements

[insert after MeasObjectUTRA-GENERIC]

#### –	MeasObjectWLAN-GENERIC

Table 4.6.6-3AA: MeasObjectWLAN-GENERIC (Freq, SSID)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Freq | WLAN band and carrier as used in test setup |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | SSID |  |  |
| ssid-r12[2] | Not Present |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

#### –	MeasObjectWLAN-W2-GENERIC

Table 4.6.6-3AB: MeasObjectWLAN-W2-GENERIC (Freq, SSID1, SSID2)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Freq | WLAN band and carrier as used in test setup for 2 entries |  |
| wlan-ToAddModList-r13 OF SEQUENCE { |  | 2 entries |  |
| ssid-r12[1] | SSID1 |  |  |
| ssid-r12[2] | SSID2 |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

[Insert after ReportConfigInterRAT-PERIODICAL]

#### –	ReportConfigInterRAT-W1-WLAN

Table 4.6.6-9A: ReportConfigInterRAT-W1-WLAN (ThreshDB)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | ThreshDB+101 |  |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	ReportConfigInterRAT-W2-WLAN

Table 4.6.6-9B: ReportConfigInterRAT-W2-WLAN (Thresh1DB, Thresh2DB)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W2 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW2-r13 SEQUENCE { | 2 entries |  |  |
| w2-Threshold1-r13 | Thresh1DB+101 |  |  |
| w2-Threshold2-r13 | Thresh2DB+101 |  |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	ReportConfigInterRAT-W3-WLAN

Table 4.6.6-9C: ReportConfigInterRAT-W3-WLAN (ThreshDB)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW3-r13 SEQUENCE { | 1 entry |  |  |
| W3-Threshold-r13 | ThreshDB+101 |  |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |