3GPP TSG-RAN5 Meeting #82	R5-191344

# Athens, Greece,  25 February – 01 March 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR |  | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of FR1 EN-DC non-Contention based random access Test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Non-Contention based random access test in FR1 for NR EN-DC Test case 4.3.2.2.2 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Test case 4.3.2.2.2 Non-Contention based random access test in FR1 for EN-DC. References to TS 38.533 Annexes are aligned with v0.2.0. The Test configuration table follows R5-187995 approved at RAN5#81. Uncertainties for Timing and DL AWGN follow R5-190448 and R5-190582 approved at RAN5 Meeting #4-5G-NR Adhoc in Singapore. Changes to Table F.1.1.2-1 are covered by these CRs and therefore not included here. Testing is carried out under normal conditions only, as the core requirements for power control accuracy apply under normal conditions, as stated in TS 38.101-1 clause 6.3.4.1. The Minimum Conformance requirements are different for contention-based and non-contention based random access, and there is only one EN-DC test case for each scenario. There would be no benefit in separating out the Minimum Conformance requirements to a separate section, so they are kept within each test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-Contention based random access in FR1 for EN-DC would not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2.2.2 (new), Table F.1.1.2-2, Table F.1.3.2-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

# 4	EN-DC with PSCell in FR1

This section contains test scenarios for E-UTRA and NR dual connectivity with E-UTRA as PCell and NR and PSCell. This configuration is also known as NSA Option 3 and 3a. The NR PSCell is in Frequency Range 1.

## 4.1	Void

## 4.2	Void

## 4.3	RRC_CONNECTED state mobility

### 4.3.1	Void

### 4.3.2	RRC connection mobility control

#### 4.3.2.1	Void

#### 4.3.2.2	Random access

##### 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents

- Cell mapping

4.3.2.2.2.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

4.3.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

4.3.2.2.2.3	Minimum conformance requirement

The random access procedure is used when establishing the layer 1 communication between the UE and NG-RAN. The random access is as defined in TS 38.213 [8] clause 7.4 and the control of the RACH transmission is as defined in TS 38.321 [12] clause 5.1.

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula as defined in TS 38.213 [8] clause 7.4 and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as defined in TS 38.101-1 [2] Table 6.3.4.2-1. The relative power applied to additional preambles shall have an accuracy as specified in TS 38.101-1 [2] Table 6.3.4.3-1.

The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in TS 38.321 [12] clause 5.1.4.

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs is configured, with the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs, UE shall have the capability to select the Random Access Preamble corresponding to the selected SSB, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs is configured, with the UE selected CSI-RS with CSI-RSRP above cfra-csirs-DedicatedRACH-Threshold amongst the associated CSI-RSs, UE shall have the capability to select the Random Access Preamble corresponding to the selected CSI-RS, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS, and PRACH occasion shall be randomly selected with equal probability amongst the selected CSI-RS associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

The UE may stop monitoring for Random Access Response(s), if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, unless the random access procedure is initialized for Other SI request from UE.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2 and A.4.3.2.2.2. Non-contention based random access procedure is not initialized for Other SI requested from UE or for beam failure recovery, so the requirements related to those features are omitted.

4.3.2.2.2.4	Test description

4.3.2.2.2.4.1	Initial conditions

This test can be run in the configurations defined in Table 4.3.2.2.2.4.1-1.

Table 4.3.2.2.2.4.1-1: Non-contention based random access test in FR1 for PSCell in EN-DC supported test configurations

| Test Case ID |  | Test Config Index |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| 4.3.2.2.2-1 |  | 1 |  | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD |  |
| 4.3.2.2.2-2 |  | 2 |  | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD |  |
| 4.3.2.2.2-3 | 3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| 4.3.2.2.2-4 | 4 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Configure the test equipment and the DUT according to the parameters in Table 4.3.2.2.2.4.1-2.

Table 4.3.2.2.2.4.1-2: Initial conditions for Non-contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.1-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.3.2.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 4.3.2.2.2.4.3.

2. Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The E-UTRAN PCell power levels and settings are specified in Table A.6.1.1-1. Cell 2 is the NR FR1 PSCell. The connection setup is done according to the settings in Annex C.1.3, with downlink signal levels as per Annex C.1.2. General Test parameters are defined in Table 4.3.2.2.2.5-1.

3. Downlink signals for NR cell are initially set up according to Annex C.2.1.

4.3.2.2.2.4.2	Test procedure

For this test two cells are used, an E-UTRA serving cell (PCell) and an NR FR1 PSCell. For the NR PSCell, the System Simulator shall explicitly assign a random access preamble via dedicated signalling in the downlink. There are two subtests, to test both SSB-based non-contention based random access (subtest 1) and CSI-RS-based non-contention based random access (subtest 2).

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity EN-DC according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 4.3.2.2.2.5-1.

3.	After power-up and cell selection, the UE shall trigger a random access procedure to establish an RRC connection on the NR PSCell.

4.	Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the SSB with index 0, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

5.	Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

5.1. Set the parameters according to Table 4.3.2.2.2.5-1 Subtest 2.

5.2. Repeat steps 1-3

5.3. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the CSI-RS configured, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

6.	Test 3: Correct behaviour when receiving Random Access Response

6.1. Repeat steps 1-3

6.2. The UE shall send preambles to the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

6.3. As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

6.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

6.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

6.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 4.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in 4.3.2.2.2.5.

7.	Test 4: Correct behaviour when not receiving Random Access Response

7.1. Repeat steps 1-3.

7.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 4 preambles.

7.3. As no Random Access Response was received within the RA Response window configured in RACH-ConfigCommon, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

7.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

7.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

7.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 4.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in 4.3.2.2.2.5.

4.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause FFS with the following exceptions:

FFS

4.3.2.2.2.5	Test requirement

Table 4.3.2.2.2.5-1 defines the primary level settings for non-contention based random access test in FR1 for PSCell in EN-DC. Tables 4.3.2.2.2.5-2, 4.3.2.2.2.5-3 and 4.3.2.2.2.5-4 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 4.3.2.2.2.5-1: General test parameters for non-contention based random access test in FR1 for PSCell in EN-DC

| Parameter |  |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 | SSB.1 FR1 | As defined in A.3.1, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 3,4 |  | SSB.2 FR1 | SSB.2 FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | 2 | Different from the definition in A.3.1 |
| SS/PBCH block index |  |  |  | 0,1 | 0,1 | Different from the definition in A.3.1 |
| CSI-RS Configuration |  | Config 1,2 |  | N/A | CSI-RS.1.1 FDD | As defined in A.1.4 |
|  |  | Config 3,4 |  |  | CSI-RS.2.1 TDD |  |
| Duplex Mode for Cell 2 |  | Config 1,2 |  | FDD | FDD |  |
|  |  | Config 3,4 |  | TDD | TDD |  |
| TDD Configuration |  | Config 3,4 |  | TDDConf.1.2 | TDDConf.1.2 |  |
| OCNG Pattern Note 1 |  |  |  | OCNG pattern 1 | OCNG pattern 1 | As defined in A.2.1. |
| PDSCH parameters Note 4 |  | Config 1,2 |  | SR1.1 FDD | SR1.1 FDD | As defined in A.1.1. |
|  |  | Config 3,4 |  | SR2.1 TDD | SR2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |  |
| SSB with index 0 |  |  | dB | 3 | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1,2 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 3,4 |  | -101 | -101 |  |
|  |  |  | dB | 3 | 3 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 | -95 |  |
| SSB with index 1 |  |  | dB | -17 | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1,2 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 3,4 |  | -101 | -101 |  |
|  |  |  | dB | -17 | -17 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 | -115 |  |
| Io Note 2 |  | Config 1,2 | dBm | -65.3/9.36MHz | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3,4 |  | -62.2/38.16MHz | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | PRACH.2 FR1 | PRACH.3 FR1 | As defined in A.7.1. |
| Propagation Condition |  |  | - | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Es/Iot, SS-RSRP and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: Void. Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0.

-	The selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

-	The Random Access Preamble shall have the Preamble Index associated with the CSI-RS configured.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured.

-	the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

Test 3: Correct behaviour when receiving Random Access Response

-	The power of the first preamble shall be -30 dBm within the accuracy specified in Table 4.3.2.2.2.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 4.3.2.2.2.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 4.3.2.2.2.5-4.

Test 4: Correct behaviour when not receiving Random Access Response

-	The power of the first preamble shall be -30 dBm within the accuracy specified in Table 4.3.2.2.2.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 4.3.2.2.2.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 4.3.2.2.2.5-4.

Table 4.3.2.2.2.5-2 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 4.3.2.2.2.5-3 Relative power tolerance Test requirements

| Power step P (Up or down) (dB) | PRACH (dB) |
| --- | --- |
| 2 ≤ ΔP < 3 | ± 3.2 |

Table 4.3.2.2.2.5-4: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 880*Tc |
|  | 30 | 30 | 624*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

## << Unchanged sections omitted >>

### F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tigher measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 shall be handled case by case.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc | dB | ±1.5 |  |
| Ratio of cell X signal / AWGN, Êsx / Noc | dB | ±0.3 | Same as in LTE |
| Fading profile uncertainty* | dB | ±TBD | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.101-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.101-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 |  |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 |  |
| Timing Advance Adjustment accuracy | Tc | ±88 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2: These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. |  |  |  |

The maximum test system uncertainty for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.1.2-2.

The maximum test system uncertainty for the NR SA FR1 test cases in chapter 6 is defined in Table F.1.1.2-3.

Table F.1.1.2-2: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 4.3.2.2.2 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |

Table F.1.1.2-3 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | Same as 6.6.1.1 | Same as 6.6.1.1 |

## << Unchanged sections omitted >>

### F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NR SA FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 4.3.2.2.2 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |

<< End of changes>>