3GPP TSG-RAN5 Meeting #82	R5-19xxxx

# Athens, Greece, 25th Feb 2019 - 1th March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | yyyy | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to connection diagram for 6.6.3F.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current connection diagram for 6.6.3F.1 is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The connection diagram for 6.6.3F.1 is changed from Figure A.3 to Figure A.7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The connection diagram for 6.6.3F.1 remains incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3F.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

### 6.6.3F	Spurious emission for category NB1 and NB2

#### 6.6.3F.1	Transmitter Spurious emissions for category NB1  and NB2

##### 6.6.3F.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

##### 6.6.3F.1.2	Test applicability

This test case applies to all types of UE release 13 and forward of category NB1.

This test case applies to all types of UE release 14 and forward of category NB2.

##### 6.6.3F.1.3	Minimum conformance requirements

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) from the edge of the channel bandwidth. The spurious emission limits in Table 6.6.3F.1.3-1 apply for all transmitter band configurations (NRB) and channel bandwidths.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.6.3F.1.3-1: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |

…

When UE is configured for category NB1  or NB2 uplink transmissions the boundary between category NB1 or NB2 out of band and spurious emission domain shall be FOOB = 1.7 MHz.

The normative reference for this requirement is TS 36.101 [2] subclauses 6.6.3.1 and 6.6.3F.

##### 6.6.3F.1.4	Test description

6.6.3F.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2F. All of these configurations shall be tested with applicable test parameters, and are shown in table 6.6.3F.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.6.3F.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 8.1.1 |  | NC |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Subcarrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | BPSK | 1@0 | 15 |
| 4 |  | BPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.3F.1.4.1-1.

5.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 6.6.3F.1.4.3.

## << End of changes >>