3GPP TSG-RAN5 Meeting #82	R5-19xxxx

# Athens, Greece, 25th Feb 2019 - 1th March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | yyyy | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean up for Inter-band UL CA Tx Test Cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test, LTE_CA_Rel12_2UL-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ・6.2.3A.2 NOTE4 are missing for a few configuration ID. ・6.2.3A.2 The reference numbering is not correct in Test applicability. ・6.3.5A.2.2 Some line feed is missing in test procedure. ・6.5.2A.1.2_1, 6.5.2A.3.2 Some UL allocations are not correct. ・6.6.2.2A.2, 6.6.2.2A.2_1 The reference numbering is not correct in initial condition. ・6.6.3.1A.2 Test requirement title for CA 3A-28A is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ・6.2.3A.2 NOTE4 are added to configuration ID X+22 and X+25. ・6.3.3A.2.2 The reference for 6.3.4A.1.2 is modified. ・6.3.5A.2.2 The line feed is added between step 5.3 and 5.4. ・6.5.2A.1.2_1, 6.5.2A.3.2 UL allocations are modified. ・6.6.2.2A.2, 6.6.2.2A.2_1 The reference numberings are modified in initial conditions. ・6.6.3.1A.2.5 Test requirement Test requirement title for CA 3A-28A is modified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3A.2, 6.3.3A.2.2, 6.3.5A.2.2, 6.5.2A.1.2_1, 6.5.2A.3.2, 6.6.2.2A.2, 6.6.2.2A.2_1, 6.6.3.1A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

#### 6.2.3A.2	Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA)

## << Unchanged sections omitted >>

Table 6.2.3A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |  |
|  | PCC | SCC | PCC & SCC RB allocation | PCC | SCC | NRB_alloc | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |  |
| 1 (NOTE4) | 6 | 25 | N/A for this test | QPSK | QPSK | 2 | 1 | 1 |
| 2 | 6 | 25 |  | QPSK | QPSK | 13 | 5 | 8 |
| 3 | 6 | 25 |  | QPSK | QPSK | 31 | 6 | 25 |
| 4 (NOTE4) | 6 | 25 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 5 | 6 | 25 |  | 16QAM | 16QAM | 13 | 5 | 8 |
| 6 | 6 | 25 |  | 16QAM | 16QAM | 31 | 6 | 25 |
| 7 | 6 | 25 |  | QPSK | 16QAM | 30 | 5 | 25 |
| 8 (NOTE4) | 6 | 50 |  | QPSK | QPSK | 2 | 1 | 1 |
| 9 | 6 | 50 |  | QPSK | QPSK | 17 | 5 | 12 |
| 10 | 6 | 50 |  | QPSK | QPSK | 56 | 6 | 50 |
| 11 (NOTE4) | 6 | 50 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 12 | 6 | 50 |  | 16QAM | 16QAM | 17 | 5 | 12 |
| 13 | 6 | 50 |  | 16QAM | 16QAM | 56 | 6 | 50 |
| 14 | 6 | 50 |  | QPSK | 16QAM | 55 | 5 | 50 |
| 15 (NOTE4) | 25 | 15 |  | QPSK | QPSK | 2 | 1 | 1 |
| 16 | 25 | 15 |  | QPSK | QPSK | 12 | 8 | 4 |
| 17 | 25 | 15 |  | QPSK | QPSK | 40 | 25 | 15 |
| 18 (NOTE4) | 25 | 15 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 19 | 25 | 15 |  | 16QAM | 16QAM | 12 | 8 | 4 |
| 20 | 25 | 15 |  | 16QAM | 16QAM | 40 | 25 | 15 |
| 21 | 25 | 15 |  | QPSK | 16QAM | 23 | 8 | 15 |
| 22 (NOTE4) | 25 | 25 |  | QPSK | QPSK | 2 | 1 | 1 |
| 23 | 25 | 25 |  | QPSK | QPSK | 16 | 8 | 8 |
| 24 | 25 | 25 |  | QPSK | QPSK | 50 | 25 | 25 |
| 25 (NOTE4) | 25 | 25 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 26 | 25 | 25 |  | 16QAM | 16QAM | 16 | 8 | 8 |
| 27 | 25 | 25 |  | 16QAM | 16QAM | 50 | 25 | 25 |
| 28 | 25 | 25 |  | QPSK | 16QAM | 33 | 8 | 25 |
| 29 (NOTE4) | 25 | 50 |  | QPSK | QPSK | 2 | 1 | 1 |
| 30 | 25 | 50 |  | QPSK | QPSK | 16 | 4 | 12 |
| 31 (NOTE4) | 25 | 50 |  | QPSK | QPSK | 20 | 8 | 12 |
| 32 | 25 | 50 |  | QPSK | QPSK | 65 | 15 | 50 |
| 33 (NOTE4) | 25 | 50 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 34 | 25 | 50 |  | 16QAM | 16QAM | 16 | 4 | 12 |
| 35 (NOTE4) | 25 | 50 |  | 16QAM | 16QAM | 20 | 8 | 12 |
| 36 | 25 | 50 |  | 16QAM | 16QAM | 65 | 15 | 50 |
| 37 | 25 | 50 |  | QPSK | 16QAM | 54 | 4 | 50 |
| 38 (NOTE4) | 50 | 25 |  | QPSK | QPSK | 2 | 1 | 1 |
| 39 | 50 | 25 |  | QPSK | QPSK | 20 | 12 | 8 |
| 40 | 50 | 25 |  | QPSK | QPSK | 75 | 50 | 25 |
| 41 (NOTE4) | 50 | 25 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 42 | 50 | 25 |  | 16QAM | 16QAM | 20 | 12 | 8 |
| 43 | 50 | 25 |  | 16QAM | 16QAM | 75 | 50 | 25 |
| 44 | 50 | 25 |  | QPSK | 16QAM | 37 | 12 | 25 |
| 45 (NOTE4) | 50 | 50 |  | QPSK | QPSK | 2 | 1 | 1 |
| 46 | 50 | 50 |  | QPSK | QPSK | 24 | 12 | 12 |
| 47 | 50 | 50 |  | QPSK | QPSK | 100 | 50 | 50 |
| 48 (NOTE4) | 50 | 50 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 49 | 50 | 50 |  | 16QAM | 16QAM | 24 | 12 | 12 |
| 50 | 50 | 50 |  | 16QAM | 16QAM | 100 | 50 | 50 |
| 51 | 50 | 50 |  | QPSK | 16QAM | 62 | 12 | 50 |
| 52 (NOTE4) | 50 | 100 |  | QPSK | QPSK | 2 | 1 | 1 |
| 53 | 50 | 100 |  | QPSK | QPSK | 30 | 12 | 18 |
| 54 | 50 | 100 |  | QPSK | QPSK | 150 | 50 | 100 |
| 55 (NOTE4) | 50 | 100 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 56 | 50 | 100 |  | 16QAM | 16QAM | 30 | 12 | 18 |
| 57 | 50 | 100 |  | 16QAM | 16QAM | 150 | 50 | 100 |
| 58 | 50 | 100 |  | QPSK | 16QAM | 112 | 12 | 100 |
| 59 (NOTE4) | 75 | 75 |  | QPSK | QPSK | 2 | 1 | 1 |
| 60 | 75 | 75 |  | QPSK | QPSK | 32 | 16 | 16 |
| 61 | 75 | 75 |  | QPSK | QPSK | 150 | 75 | 75 |
| 62 (NOTE4) | 75 | 75 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 63 | 75 | 75 |  | 16QAM | 16QAM | 32 | 16 | 16 |
| 64 | 75 | 75 |  | 16QAM | 16QAM | 150 | 75 | 75 |
| 65 | 75 | 75 |  | QPSK | 16QAM | 91 | 16 | 75 |
| 66 (NOTE4) | 100 | 50 |  | QPSK | QPSK | 2 | 1 | 1 |
| 67 | 100 | 50 |  | QPSK | QPSK | 30 | 18 | 12 |
| 68 | 100 | 50 |  | QPSK | QPSK | 150 | 100 | 50 |
| 69 (NOTE4) | 100 | 50 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 70 | 100 | 50 |  | 16QAM | 16QAM | 30 | 18 | 12 |
| 71 | 100 | 50 |  | 16QAM | 16QAM | 150 | 100 | 50 |
| 72 | 100 | 50 |  | QPSK | 16QAM | 68 | 18 | 50 |
| 73 (NOTE4) | 100 | 75 |  | QPSK | QPSK | 2 | 1 | 1 |
| 74 | 100 | 75 |  | QPSK | QPSK | 34 | 18 | 16 |
| 75 | 100 | 75 |  | QPSK | QPSK | 175 | 100 | 75 |
| 76 (NOTE4) | 100 | 75 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 77 | 100 | 75 |  | 16QAM | 16QAM | 34 | 18 | 16 |
| 78 | 100 | 75 |  | 16QAM | 16QAM | 175 | 100 | 75 |
| 79 | 100 | 75 |  | QPSK | 16QAM | 93 | 18 | 75 |
| 80 (NOTE4) | 100 | 100 |  | QPSK | QPSK | 2 | 1 | 1 |
| 81 | 100 | 100 |  | QPSK | QPSK | 36 | 18 | 18 |
| 82 | 100 | 100 |  | QPSK | QPSK | 200 | 100 | 100 |
| 83 (NOTE4) | 100 | 100 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| 84 | 100 | 100 |  | 16QAM | 16QAM | 36 | 18 | 18 |
| 85 | 100 | 100 |  | 16QAM | 16QAM | 200 | 100 | 100 |
| 86 | 100 | 100 |  | QPSK | 16QAM | 118 | 18 | 100 |
| 87 (NOTE5) | 100 | 100 |  | QPSK | QPSK | 11 | 1 | L=1 |
| 88 (NOTE5) | 100 | 100 |  | QPSK | QPSK | 108 | 18 | L=9 |
| 89 (NOTE5) | 100 | 100 |  | 16QAM | 16QAM | 11 | 1 | L=1 |
| 90 (NOTE5) | 100 | 100 |  | 16QAM | 16QAM | 108 | 18 | L=9 |
| X+1 (NOTE4) | 15 | 25 |  | QPSK | QPSK | 2 | 1 | 1 |
| X+2 | 15 | 25 |  | QPSK | QPSK | 12 | 4 | 8 |
| X+3 | 15 | 25 |  | QPSK | QPSK | 40 | 15 | 25 |
| X+4 (NOTE4) | 15 | 25 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| X+5 | 15 | 25 |  | 16QAM | 16QAM | 12 | 4 | 8 |
| X+6 | 15 | 25 |  | 16QAM | 16QAM | 40 | 15 | 25 |
| X+7 | 15 | 25 |  | QPSK | 16QAM | 29 | 4 | 25 |
| X+8 (NOTE4) | 25 | 6 |  | QPSK | QPSK | 2 | 1 | 1 |
| X+9 | 25 | 6 |  | QPSK | QPSK | 13 | 8 | 5 |
| X+10 | 25 | 6 |  | QPSK | QPSK | 31 | 25 | 6 |
| X+11 (NOTE4) | 25 | 6 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| X+12 | 25 | 6 |  | 16QAM | 16QAM | 13 | 8 | 5 |
| X+13 | 25 | 6 |  | 16QAM | 16QAM | 31 | 25 | 6 |
| X+14 | 25 | 6 |  | QPSK | 16QAM | 14 | 8 | 6 |
| X+15 (NOTE4) | 25 | 75 |  | QPSK | QPSK | 2 | 1 | 1 |
| X+16 | 25 | 75 |  | QPSK | QPSK | 24 | 8 | 16 |
| X+17 | 25 | 75 |  | QPSK | QPSK | 100 | 25 | 75 |
| X+18 (NOTE4) | 25 | 75 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| X+19 | 25 | 75 |  | 16QAM | 16QAM | 24 | 8 | 16 |
| X+20 | 25 | 75 |  | 16QAM | 16QAM | 100 | 25 | 75 |
| X+21 | 25 | 75 |  | QPSK | 16QAM | 83 | 8 | 75 |
| X+22 (NOTE4) | 75 | 25 |  | QPSK | QPSK | 2 | 1 | 1 |
| X+23 | 75 | 25 |  | QPSK | QPSK | 24 | 16 | 8 |
| X+24 | 75 | 25 |  | QPSK | QPSK | 100 | 75 | 25 |
| X+25 (NOTE4) | 75 | 25 |  | 16QAM | 16QAM | 2 | 1 | 1 |
| X+26 | 75 | 25 |  | 16QAM | 16QAM | 24 | 16 | 8 |
| X+27 | 75 | 25 |  | 16QAM | 16QAM | 100 | 75 | 25 |
| X+28 | 75 | 25 |  | QPSK | 16QAM | 41 | 16 | 25 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg. only the first of those is tested according to the order on the Test Configuration Table list. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only for 6.6.2.1A.2 Spectrum emission mask for CA (inter-band DL CA and UL CA) Note 5: Void |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

##### 6.3.3A.2.2	Test applicability

The requirements of this test case apply in test cases 6.3.4A.1.2 General ON/OFF time mask for CA and 6.3.4.2 PRACH and SRS time mask to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

## << Unchanged sections omitted >>

6.3.5A.2.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clause 8.3.3.2).

4.	The procedure is separated in various subtests to verify different aspects of relative power control on both PCC and SCC individually. The power patterns of the subtests are shown in figure 6.3.5A.2.2.4.2-1 to 6.3.5A.2.2.4.2-6. In this test case, the term PCMAX_L  – MAX{TL, TLOW(PCMAX_L) } defined in TS 36.101 [2] clause 6.2.5A is used, to ensure the UE is not tested outside its power capability.

Figure 6.3.5A.2.2.4.2-1: FDD ramping up test power patterns

Figure 6.3.5A.2.2.4.2-2: FDD ramping down test power patterns

Figure 6.3.5A.2.2.4.2-3: TDD FS2 ramping up test power patterns

Figure 6.3.5A.2.2.4.2-4: TDD FS2 ramping down test power patterns

Figure 6.3.5A.2.2.4.2-5: TDD FS3 ramping up test power patterns

Figure 6.3.5A.2.2.4.2-6: TDD FS3 ramping down test power patterns

5.	Sub test: ramping up pattern

5.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 or 0B (for LAA SCC) for C_RNTI to schedule the UL RMC according to Table A.2.2.2.1-1 (FDD) or Table A.2.3.2.1-1 (TDD FS2) or Table A.2.5.2.1-1 (TDD FS3) on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC at -36.8dBm +/- 3.2 dB for carrier frequency f ≤ 3.0GHz or at -36.5dBm +/- 3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz or at 36 dBm +/- 4 dB for carrier frequency 4.2GHz < f ≤ 6GHz.

5.2.	Schedule the UE's PUSCH data transmission on each component carrier as described in Figure 6.3.5A.2.2.4.2-1 (FDD pattern A: sub-test is divided in 4 arbitrary radio frames with 10 active uplink sub-frames per radio frame) and Figure 6.3.5A.2.2.4.2-3 (TDD FS2 pattern A: sub-test is divided in 10 arbitrary radio frames with 4 active uplink sub-frames per radio frame) and Figure 6.3.5A.2.2.4.2-5 (TDD FS3 pattern A: sub-test is divided in 10 arbitrary radio frames with 4 active uplink sub-frames per radio frame) with an uplink RB allocation as defined in tables 6.3.5A.2.2.5-1 thru 6.3.5A.2.2.5-14 depending on CC combinations. On the PDCCH format 0 or 0B (for LAA SCC) for the scheduling of the PUSCH the SS will transmit a +1dB TPC command on both PCC and SCC separately. Note that the measurement need not be done continuously, provided that interruptions are whole numbers of frames, and TPC commands of 0dB are sent during the interruption.

5.3.	Measure the power of PUSCH transmissions on both PCC and SCC separately to verify the UE relative power control meet test requirements 6.3.5A.2.2.5. For power transients between subframes, transient periods of 40us between subframes are excluded. For ON/OFF or OFF/ON transients, transient periods of 20 us at the beginning of the subframe are excluded. Calculate the Total uplink power across both CCs. If (PCMAX_L  – MAX{TL, TLOW(PCMAX_L) } - Total uplink power) < 0.7dB, stop ramping, exclude the last measurement and go to step 5.4.

5.4.	Repeat the subtest different pattern B, C to move the RB allocation change at different points in the pattern as described in Table 6.3.5A.2.2.5-1 thru Table 6.3.5A.2.2.5-14 to force bigger UE power steps at various points in the power range.

6.	Sub test: ramping down pattern

6.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 or 0B (for LAA SCC) for C_RNTI to schedule the UL RMC according to Table A.2.2.2.1-1 (FDD) or Table A.2.3.2.1-1 (TDD FS2) or Table A.2.5.2.1-1 (TDD FS3) on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC at +15.0dBm +/- 3.2 dB for carrier frequency f ≤ 3.0GHz or at +14.7dBm +/- 3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz or at 14 dBm +/- 4 dB for carrier frequency 4.2GHz < f ≤ 6GHz.

6.2.	Schedule the UE's PUSCH data transmission as described in Figure 6.3.5A.2.2.4.2-2 (FDD pattern A: sub-test is divided in 4 arbitrary radio frames with 10 active uplink sub-frames per radio frame) and Figure 6.3.5A.2.2.4.2-4 (TDD FS2 pattern A: sub-test is divided in 10 arbitrary radio frames with 4 active uplink sub-frames per radio frame) and Figure 6.3.5A.2.2.4.2-6 (TDD FS3 pattern A: sub-test is divided in 10 arbitrary radio frames with 4 active uplink sub-frames per radio frame) with an uplink RB allocation as defined in tables 6.3.5A.2.2.5-1 thru 6.3.5A.2.2.5-14 depending on CC combinations. On the PDCCH format 0 or 0B (for LAA SCC) for the scheduling of the PUSCH the SS will transmit a -1dB TPC command on both PCC and SCC separately. Note that the measurement need not be done continuously, provided that interruptions are whole numbers of frames, and TPC commands of 0dB are sent during the interruption.

6.3.	Measure the power of PUSCH transmissions on both PCC and SCC separately to verify the UE relative power control meet test requirements 6.3.5.2.5. For power transients between subframes, transient periods of 40us between subframes are excluded. For ON/OFF or OFF/ON transients, transient periods of 20 us at the beginning of the subframe are excluded.

6.4.	Repeat the subtest different pattern B, C to move the RB allocation change at different points in the pattern as described in Table 6.3.5A.2.2.5-1 thru Table 6.3.5A.2.2.5-14 to force bigger UE power steps at various points in the power range.

## << Unchanged sections omitted >>

##### 6.5.2A.1.2_1	Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 64QAM

6.5.2A.1.2_1.1	Test Purpose

For the inter-band carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.2_1.2	Test applicability

This test case applies to all types of E-UTRA UE Release 13 and forward that support inter-band DL CA and UL CA and UL 64QAM.

This test case also applies to all types of E-UTRA UE release 14 and forward that support uplink LAA.

6.5.2A.1.2_1.3	Minimum conformance requirements

The EVM requirement for 64QAM is according to Table 6.5.2A.1.1_1.3-1 if CA is configured in uplink.

Table 6.5.2A.1.2_1.3-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| 64QAM | % | 8 | 8 |

6.5.2A.1.2_1.4	Test description

6.5.2A.1.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.2_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | N/A for this test | 64QAM | 1 | P_1@0 | S_0@0 | - | - |
| 2 | 6 | 25 |  | 64QAM | 1 | P_1@5 | S_0@0 | - | - |
| 3 | 6 | 25 |  | 64QAM | 6 | P_6@0 | S_0@0 | - | - |
| 4 | 6 | 50 |  | 64QAM | 1 | P_1@0 | S_0@0 | - | - |
| 5 | 6 | 50 |  | 64QAM | 1 | P_1@5 | S_0@0 | - | - |
| 6 | 6 | 50 |  | 64QAM | 6 | P_6@0 | S_0@0 | - | - |
| 7 | 15 | 25 |  | 64QAM | 4 | P_4@0 | S_0@0 | - | - |
| 8 | 15 | 25 |  | 64QAM | 4 | P_4@11 | S_0@0 | - | - |
| 9 | 15 | 25 |  | 64QAM | 15 | P_15@0 | S_0@0 | - | - |
| 10 | 25 | 6 |  | 64QAM | 8 | P_8@0 | S_0@0 | - | - |
| 11 | 25 | 6 |  | 64QAM | 8 | P_8@17 | S_0@0 | - | - |
| 12 | 25 | 6 |  | 64QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 25 | 15 |  | 64QAM | 8 | P_8@0 | S_0@0 | - | - |
| 14 | 25 | 15 |  | 64QAM | 8 | P_8@17 | S_0@0 | - | - |
| 15 | 25 | 15 |  | 64QAM | 25 | P_25@0 | S_0@0 | - | - |
| 16 | 25 | 25 |  | 64QAM | 8 | P_8@0 | S_0@0 | - | - |
| 17 | 25 | 25 |  | 64QAM | 8 | P_8@17 | S_0@0 | - | - |
| 18 | 25 | 25 |  | 64QAM | 25 | P_25@0 | S_0@0 | - | - |
| 19 | 25 | 50 |  | 64QAM | 8 | P_8@0 | S_0@0 | - | - |
| 20 | 25 | 50 |  | 64QAM | 8 | P_8@17 | S_0@0 | - | - |
| 21 | 25 | 50 |  | 64QAM | 25 | P_25@0 | S_0@0 | - | - |
| 22 | 50 | 6 |  | 64QAM | 12 | P_12@0 | S_0@0 | - | - |
| 23 | 50 | 6 |  | 64QAM | 12 | P_12@38 | S_0@0 | - | - |
| 24 | 50 | 6 |  | 64QAM | 50 | P_50@0 | S_0@0 | - | - |
| 25 | 50 | 25 |  | 64QAM | 12 | P_12@0 | S_0@0 | - | - |
| 26 | 50 | 25 |  | 64QAM | 12 | P_12@38 | S_0@0 | - | - |
| 27 | 50 | 25 |  | 64QAM | 50 | P_50@0 | S_0@0 | - | - |
| 28 | 50 | 50 |  | 64QAM | 12 | P_12@0 | S_0@0 | - | - |
| 29 | 50 | 50 |  | 64QAM | 12 | P_12@38 | S_0@0 | - | - |
| 30 | 50 | 50 |  | 64QAM | 50 | P_50@0 | S_0@0 | - | - |
| 31 | 50 | 100 |  | 64QAM | 12 | P_12@0 | S_0@0 | - | - |
| 32 | 50 | 100 |  | 64QAM | 12 | P_12@38 | S_0@0 | - | - |
| 33 | 50 | 100 |  | 64QAM | 50 | P_50@0 | S_0@0 | - | - |
| 34 | 75 | 75 |  | 64QAM | 16 | P_16@0 | S_0@0 | - | - |
| 35 | 75 | 75 |  | 64QAM | 16 | P_16@59 | S_0@0 | - | - |
| 36 | 75 | 75 |  | 64QAM | 75 | P_75@0 | S_0@0 | - | - |
| 37 | 75 | 100 |  | 64QAM | 16 | P_16@0 | S_0@0 | - | - |
| 38 | 75 | 100 |  | 64QAM | 16 | P_16@59 | S_0@0 | - | - |
| 39 | 75 | 100 |  | 64QAM | 75 | P_75@0 | S_0@0 | - | - |
| 40 | 100 | 50 |  | 64QAM | 18 | P_18@0 | S_0@0 | - | - |
| 41 | 100 | 50 |  | 64QAM | 18 | P_18@82 | S_0@0 | - | - |
| 42 | 100 | 50 |  | 64QAM | 100 | P_100@0 | S_0@0 | - | - |
| 43 | 100 | 75 |  | 64QAM | 18 | P_18@0 | S_0@0 | - | - |
| 44 | 100 | 75 |  | 64QAM | 18 | P_18@82 | S_0@0 | - | - |
| 45 | 100 | 75 |  | 64QAM | 100 | P_100@0 | S_0@0 | - | - |
| 46 | 100 | 100 |  | 64QAM | 18 | P_18@0 | S_0@0 | - | - |
| 47 | 100 | 100 |  | 64QAM | 18 | P_18@82 | S_0@0 | - | - |
| 48 | 100 | 100 |  | 64QAM | 100 | P_100@0 | S_0@0 | - | - |
| X+1 | 75 | 25 |  | 64QAM | 16 | P_16@0 | S_0@0 | - | - |
| X+2 | 75 | 25 |  | 64QAM | 16 | P_16@59 | S_0@0 | - | - |
| X+3 | 75 | 25 |  | 64QAM | 75 | P_75@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

##### 6.5.2A.3.2	In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA)

6.5.2A.3.2.1	Test Purpose

Same test purpose as in clause 6.5.2A.3.1.1.

6.5.2A.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

This test case also applies to all types of E-UTRA UE release 14 and forward that supports uplink LAA.

6.5.2A.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.3.0.

6.5.2A.3.2.4	Test description

6.5.2A.3.2.4.1	Initial conditions

Same as in clause 6.5.2A.3.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.5.2A.3.1.4.1-1  use Table 6.5.2A.3.2.4.1-1.

-	Instead of clause 6.5.2A.3.1.4.3  use clause 6.5.2A.3.2.4.3.

Table 6.5.2A.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 50 | 6 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 100 | 75 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 75 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. Note 3: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

##### 6.6.2.2A.2	Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA)

6.6.2.2A.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA but do not support UL 64QAM or UL 256QAM.

6.6.2.2A.2.3	Minimum conformance requirements

The minimum requirements are the same as in section 6.6.2.2.3 per component carrier.

6.6.2.2A.2.4	Test description

6.6.2.2A.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For Uplink CA Configurations listed in table 6.6.2.2A.2.4.1-1, Additional Spectrum emission requirements are tested.

Table 6.6.2.2A.2.4.1-1: Additional Spectrum Emission mask for inter-band CA

| Uplink CA Configuration | NS-value per CC (in same order as UL CA Configuration column) |  | Applicable spectrum emission requirements (in same order as UL CA Configuration column) |  |
| --- | --- | --- | --- | --- |
| CA_1A-3A CA_1A-5A CA_1A-7A CA_1A-8A CA_1A-18A CA_1A-26A CA_1A-42A | NS_05 | NS_01 | N/A | N/A |
| CA_1A-19A | NS_05 | NS_08 | N/A | N/A |
| CA_1A-21A | NS_05 | NS_09 | N/A | N/A |
| CA_1A-28A | NS_05 | NS_17 | N/A | N/A |
| CA_2A-4A, CA_2A-66A | NS_03 | NS_03 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_2A-12A, CA_2A-13A, CA_4A-12A, CA_4A-13A, CA_4A-17A | NS_03 | NS_06 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-30A | NS_03 | NS_21 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_3A-5A CA_3A-7A CA_3A-8A CA_5A-7A CA_39A-41A CA_41A-42A | NS_01 | NS_01 | N/A | N/A |
| CA_3A-19A | NS_01 | NS_08 | N/A | N/A |
| CA_3A-20A CA_7A-20A | NS_01 | NS_10 | N/A | N/A |
| CA_2A-13A, CA_4A-13A | NS_03 | NS_07 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-5A CA_2A-7A, CA_4A-5A, CA_4A-7A | NS_03 | NS_01 | Table 6.6.2.2.3.1-1 | N/A |
| CA_5A-66ANote 5 | NS_01 | NS_03 | N/A | Table 6.6.2.2.3.1-1 |
| CA_5A-12A CA_5A-17A | NS_01 | NS_06 | N/A | Table 6.6.2.2.3.3-1 |
| CA_5A-30A | NS_01 | NS_21 | N/A | Table 6.6.2.2.3.1-1 |
| CA_7A-28A CA_18A-28A | NS_01 | NS_17 | N/A | N/A |
| CA_7A-28A | NS_01 | NS_18 | N/A | N/A |
| CA_12A-30A | NS_06 | NS_21 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.3-1 |
| CA_12A-66A | NS_06 | NS_03 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.1-1 |
| CA_19A-21A | NS_08 | NS_09 | N/A | N/A |
| Note 1: For CCs with NS_01, there are no additional requirements defined in 36.101. Only CCs with NS-value different than NS_01 needs to be tested. Note 2: A-MPR values for NS-values in this table are the same as in section 6.2.4.3. Note 3: Selection of CA configurations and NS-values to test is derived in TR 36.905 [19]. Note 4: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column. Note 5: As an exception to note 4, PCC is mapped to the second band of the combination as listed in Uplink CA Configuration column. |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in tables 6.6.2.2A.2.4.1-2 to 6.6.2.2A.2.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

## << Unchanged sections omitted >>

##### 6.6.2.2A.2_1	Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 64QAM

Editor's note: For a transition period until RAN5#83, this test case in version 15.3.1 of 36.521-1 shall be used. This is to ensure no test coverage is lost before the UL 256QAM test case becomes available.

6.6.2.2A.2_1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.2_1.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support inter-band DL CA and UL CA and UL 64QAM but do not support UL 256QAM.

6.6.2.2A.2_1.3	Minimum conformance requirements

The minimum requirements are the same as in section 6.6.2.2A.2.3.

6.6.2.2A.2_1.4	Test description

6.6.2.2A.2_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For Uplink CA Configurations listed in table 6.6.2.2A.2_1.4.1-1, Additional Spectrum emission requirements are tested.

Table 6.6.2.2A.2_1.4.1-1: Additional Spectrum Emission mask for inter-band CA

| Uplink CA Configuration | NS-value per CC (in same order as UL CA Configuration column) |  | Applicable spectrum emission requirements (in same order as UL CA Configuration column) |  |
| --- | --- | --- | --- | --- |
| CA_2A-4A, CA_2A-66A | NS_03 | NS_03 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_2A-12A, CA_2A-13A, CA_4A-12A, CA_4A-13A, CA_4A-17A | NS_03 | NS_06 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-13A, CA_4A-13A | NS_03 | NS_07 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-5A, CA_2A-7A, CA_4A-5A, CA_4A-7A | NS_03 | NS_01 | Table 6.6.2.2.3.1-1 | N/A |
| CA_5A-66ANote5 | NS_01 | NS_03 | N/A | Table 6.6.2.2.3.1-1 |
| CA_2A-30A | NS_03 | NS_21 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_5A-12A | NS_01 | NS_06 | N/A | Table 6.6.2.2.3.3-1 |
| CA_5A-30A | NS_01 | NS_21 | N/A | Table 6.6.2.2.3.1-1 |
| CA_12A-30A | NS_06 | NS_21 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.3-1 |
| CA_1A-3A | NS_05 | NS_01 | N/A | N/A |
| CA_1A-19A | NS_05 | NS_08 | N/A | N/A |
| CA_1A-21A | NS_05 | NS_09 | N/A | N/A |
| CA_3A-19A | NS_01 | NS_08 | N/A | N/A |
| CA_19A-21A | NS_08 | NS_09 | N/A | N/A |
| Note 1: For CCs with NS_01, there are no additional requirements defined in 36.101. Only CCs with NS-value different than NS_01 needs to be tested. Note 2: A-MPR values for NS-values in this table are the same as in section 6.2.4.3. Note 3: Selection of CA configurations and NS-values to test is derived in TR 36.905 [19]. Note 4: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column. Note 5: As an exception to note 4, PCC is mapped to the second band of the combination as listed in Uplink CA Configuration column. |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in tables 6.6.2.2A.2_1.4.1-2 to 6.6.2.2A.2_1.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

## << Unchanged sections omitted >>

##### 6.6.3.1A.2	Transmitter Spurious emissions for CA (inter-band DL CA and UL CA)

<Unchanged part skipped>

6.6.3.1A.2.5	Test requirement

This clause specifies the requirements for the specified E-UTRA band for Transmitter Spurious emissions requirement with frequency range as indicated in table 6.6.3.1A.2.5-1.

For frequencies ΔfOOB greater than FOOB as specified in Table 6.6.3.1A.0-1the measured average power of spurious emission, derived in step 6, shall not exceed the described value in Table 6.6.3.1A.2.5-1.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.6.3.1A.2.5-1: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| Test requirements for CA_1A-3A Configuration |  |  |  |
| 135 MHz  f  270 MHz | -36 dBm | 100 kHz |  |
| 1440 MHz f  1650 MHz 2055 MHz  f  2250 MHz 3630 MHz  f  3765 MHz 5340 MHz  f  5745 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-5A Configuration |  |  |  |
| 222 MHz  f  332 MHz | -36 dBm | 100 kHz |  |
| 1071 MHz f  1156 MHz 2744 MHz  f  2829 MHz 2991 MHz  f  3136 MHz 3568 MHz  f  3678 MHz 4664 MHz  f  4809 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-7A Configuration |  |  |  |
| 520 MHz  f  650 MHz | -36 dBm | 100 kHz |  |
| 1270 MHz f  1460 MHz 3020 MHz  f  3220 MHz 4420 MHz  f  4550 MHz 6340 MHz  f  6530 MHz 6920 MHz  f  7120 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-8A Configuration |  |  |  |
| 90 MHz  f  220 MHz | -36 dBm | 100 kHz |  |
| 1005 MHz f  1100 MHz 2800 MHz  f  2895 MHz 2925 MHz  f  3080 MHz 3680 MHz  f  3810 MHz 4720 MHz  f  4875 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-18A Configuration |  |  |  |
| 260 MHz  f  350 MHz | -36 dBm | 100 kHz |  |
| 1090 MHz  f  1165 MHz 2735 MHz  f  2810 MHz 3010 MHz  f  3145 MHz 3550 MHz  f  3640 MHz 4655 MHz  f  4790 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-19A Configuration |  |  |  |
| 230 MHz  f  320 MHz | -36 dBm | 100 kHz |  |
| 1075 MHz f  1150 MHz 2750 MHz  f  2825 MHz 2995 MHz  f  3130 MHz 3580 MHz  f  3670 MHz 4670 MHz  f  4805 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-21A Configuration |  |  |  |
| 457.1 MHz  f  532.1 MHz 915.8 MHz  f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz f  1005.8 MHz 2377.1 MHz  f  2512.1 MHz 3367.9 MHz  f  3442.9 MHz 4515.8 MHz  f  4905.8 MHz 5287.9 MHz  f  5422.9 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-26A Configuration |  |  |  |
| 222 MHz  f  352 MHz | -36 dBm | 100 kHz |  |
| 1071 MHz f  1166 MHz 2734 MHz  f  2829 MHz 2991 MHz  f  3146 MHz 3548 MHz  f  3678 MHz 4654 MHz  f  4809 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-28A Configuration |  |  |  |
| 424 MHz  f  574 MHz | -36 dBm | 100 kHz |  |
| 1172 MHz f  1277 MHz 2623 MHz  f  2728 MHz 3092 MHz  f  3257 MHz 3326 MHz  f  3476 MHz 4543 MHz  f  4708 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-42A Configuration |  |  |  |
| 240 MHz  f  560 MHz | -36 dBm | 100 kHz |  |
| 1420 MHz f  1680 MHz 4820 MHz  f  5280 MHz 5320 MHz  f  5580 MHz 7240 MHz  f  7560 MHz 8720 MHz  f  9180 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-4A Configuration |  |  |  |
| 95 MHz  f  200 MHz | -36 dBm | 100 kHz |  |
| 1510 MHz f  1660 MHz 1945 MHz  f  2110 MHz 3560 MHz  f  3665 MHz 5270 MHz  f  5575 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-5A Configuration |  |  |  |
| 152 MHz  f  262 MHz | -36 dBm | 100 kHz |  |
| 1001 MHz f  1086 MHz 2674 MHz  f  2759 MHz 2851 MHz  f  2996 MHz 3498 MHz  f  3608 MHz 4524 MHz  f  4669 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-7A Configuration |  |  |  |
| 590 MHz  f  720 MHz | -36 dBm | 100 kHz |  |
| 1130 MHz f  1320 MHz 3090 MHz  f  3290 MHz 4350 MHz  f  4480 MHz 6200 MHz  f  6390 MHz 6850 MHz  f  7050 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-12A Configuration |  |  |  |
| 418 MHz  f  512 MHz | -36 dBm | 100 kHz |  |
| 1134 MHz f  1211 MHz 2549 MHz  f  2626 MHz 2984 MHz  f  3121 MHz 3248 MHz  f  3342 MHz 4399 MHz  f  4536 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-13A Configuration |  |  |  |
| 276 MHz  f  356 MHz | -36 dBm | 100 kHz |  |
| 1063 MHz f  1133 MHz 2627 MHz  f  2697 MHz 2913 MHz  f  3043 MHz 3404 MHz  f  3484 MHz 4477 MHz  f  4607 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-30A Configuration |  |  |  |
| 395 MHz  f  465 MHz | -36 dBm | 100 kHz |  |
| 1385 MHz f  1515 MHz 2700 MHz  f  2780 MHz 4155 MHz  f  4225 MHz 6005 MHz  f  6135 MHz 6460 MHz  f  6540 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-46A Configuration |  |  |  |
| 1330 MHz ≤ f ≤ 2225 MHz 3240 MHz ≤ f ≤ 4075 MHz 7000 MHz ≤ f ≤ 7835 MHz 8390 MHz ≤ f ≤ 10000 MHz 12150 MHz ≤ f ≤ 13760 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-66A Configuration |  |  |  |
| 70 MHz  f  200 MHz | -36 dBm | 100 kHz |  |
| 1510 MHz f  1710 MHz 1920 MHz  f  2110 MHz 3560 MHz  f  3690 MHz 5270 MHz  f  5600 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-5A Configuration |  |  |  |
| 12 MHz  f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz  f  137 MHz 861 MHz  f  961 MHz | -36 dBm | 100 kHz |  |
| 2534 MHz f  2746 MHz 3358 MHz  f  3483 MHz 4244 MHz  f  4419 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-7A Configuration |  |  |  |
| 715 MHz  f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz f  1070 MHz 3215 MHz  f  3430 MHz 4210 MHz  f  4355 MHz 5920 MHz  f  6140 MHz 6710 MHz  f  6925 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-8A Configuration |  |  |  |
| 25 MHz  f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz  f  120 MHz 795 MHz  f  905 MHz | -36 dBm | 100 kHz |  |
| 2505 MHz f  2700 MHz 3470 MHz  f  3615 MHz 4300 MHz  f  4485 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-18A Configuration |  |  |  |
| 50 MHz  f  155 MHz 880 MHz  f  970 MHz | -36 dBm | 100 kHz |  |
| 2525 MHz  f  2755 MHz 3340 MHz  f  3445 MHz 4235 MHz  f  4400 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-19A Configuration |  |  |  |
| 20 MHz  f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz  f  125 MHz 865 MHz  f  955 MHz | -36 dBm | 100 kHz |  |
| 2540 MHz f  2740 MHz 3370 MHz  f  3475 MHz 4250 MHz  f  4415 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-20A Configuration |  |  |  |
| 9 kHz  f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz  f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz  f  121 MHz 848 MHz  f  953 MHz | -36 dBm | 100 kHz |  |
| 2542 MHz f  2738 MHz 3374 MHz  f  3509 MHz 4252 MHz  f  4432 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-26A Configuration |  |  |  |
| 12 MHz  f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz  f  157 MHz | -36 dBm | 100 kHz |  |
| 861 MHz f  971 MHz | -36 dBm | 100 kHz |  |
| 2524 MHz f  2756 MHz 3338 MHz  f  3483 MHz 4234 MHz  f  4419 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-28A Configuration |  |  |  |
| 214 MHz  f  379 MHz 962 MHz  f  1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz  f 1082 MHz 2413 MHz  f  2533 MHz 2672 MHz  f  2867 MHz 3116 MHz  f  3281 MHz 4123 MHz  f  4318 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-41A Configuration |  |  |  |
| 711 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz f 1074 MHz 3207 MHz f 3670 MHz 4206 MHz f 4475 MHz 5916 MHz f 6260 MHz 6702 MHz f 7165 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-42A Configuration |  |  |  |
| 170 MHz f 180 MHz | -36 dBm | 100 kHz |  |
| 1615 MHz f 1890 MHz 5015 MHz f 5490 MHz 6820 MHz f 7170 MHz 8510 MHz f 8985 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_4A-5A Configuration |  |  |  |
| 12 MHz  f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz  f  107 MHz 861 MHz  f  931 MHz | -36 dBm | 100 kHz |  |
| 2534 MHz f  2686 MHz 3358 MHz  f  3453 MHz 4244 MHz  f  4359 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_4A-7A Configuration |  |  |  |
| 745 MHz  f 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz  f 1010 MHz 3245 MHz  f  3430 MHz 4210 MHz  f  4325 MHz 5920 MHz  f  6080 MHz 6710 MHz  f  6895 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_4A-12A Configuration |  |  |  |
| 278 MHz  f  357 MHz 994 MHz  f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz f  1056 MHz 2409 MHz  f  2471 MHz 2704 MHz  f  2811 MHz 3108 MHz  f  3187 MHz 4119 MHz  f  4226 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_4A-13A Configuration |  |  |  |
| 136 MHz  f  201 MHz 923 MHz  f  978 MHz | -36 dBm | 100 kHz |  |
| 2487 MHz f  2542 MHz 2633 MHz  f  2733 MHz 3264 MHz  f  3329 MHz 4197 MHz  f  4297 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_4A-17A Configuration |  |  |  |
| 278 MHz  f  347 MHz 994 MHz  f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz f  1056 MHz 2414 MHz  f  2471 MHz 2704 MHz  f  2806 MHz 3118 MHz  f  3187 MHz 4124 MHz  f  4226 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_5A-7A Configuration |  |  |  |
| 802 MHz  f  922 MHz | -36 dBm | 100 kHz |  |
| 1651 MHz f  1746 MHz 3324 MHz  f  3419 MHz 4148 MHz  f  4316 MHz 5824 MHz  f 5989 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_5A-12A Configuration |  |  |  |
| 108 MHz ≤ f ≤ 150 MHz 549 MHz ≤ f ≤ 608 MHz 932 MHz ≤ f ≤ 999 MHz | -36 dBm | 100 kHz |  |
| 1523 MHz ≤ f ≤ 1565 MHz 2222 MHz ≤ f ≤ 2281 MHz 2347 MHz ≤ f ≤ 2414 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_5A-17A Configuration |  |  |  |
| 108 MHzf  145 MHz 559 MHz  f  608 MHz 932 MHz f  99e4 MHz | -36 dBm | 100 kHz |  |
| 1528 MHzf  1565 MHz 2232 MHz  f  2281 MHz 2352 MHz f  2414 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_5A-30A Configuration |  |  |  |
| 607 MHz  f  667 MHz | -36 dBm | 100 kHz |  |
| 1456 MHz f  1491 MHz 3129 MHz  f  3164 MHz 3761 MHz  f  3806 MHz 3953 MHz  f  4013 MHz 5434 MHz  f 5479 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_5A-66A Configuration |  |  |  |
| 12 MHz  f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz  f  132 MHz 861 MHz  f  956 MHz | -36 dBm | 100 kHz |  |
| 2534 MHz f  2736 MHz 3358 MHz  f  3478 MHz 4244 MHz  f 4409 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_7A-20A Configuration |  |  |  |
| 776 MHz  f  906 MHz | -36 dBm | 100 kHz |  |
| 1638 MHz f  1738 MHz 3332 MHz  f  3432 MHz 4138 MHz  f  4308 MHz 5832 MHz  f 6002 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_7A-28A Configuration |  |  |  |
| 1004 MHz f  1164 MHz 1752 MHz  f  1867 MHz 3203 MHz  f  3318 MHz 3906 MHz  f  4066 MHz 4252 MHz  f  4437 MHz 5703 MHz  f  5888 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_11A-18A Configuration |  |  |  |
| 182.1 MHz  f  232.1 MHz 597.9 MHz  f  632.9 MHz | -36 dBm | 100 kHz |  |
| 2025.8 MHz  f  2080.8 MHz 2242.9 MHz  f  2277.9 MHz 3057.9 MHz  f  3107.9 MHz 3670.8 MHz  f  3725.8 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_12A-30A Configuration |  |  |  |
| 873 MHz  f  917 MHz | -36 dBm | 100 kHz |  |
| 1589 MHz f  1616 MHz 3004 MHz  f  3031 MHz 3703 MHz  f  3747 MHz 3894 MHz  f  3931 MHz 5009 MHz  f 5046 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_18A-28A Configuration |  |  |  |
| 82 MHz f  127 MHz 576 MHz  f  651 MHz 897 MHz  f  957 MHz | -36 dBm | 100 kHz |  |
| 1518 MHz f  1563 MHz 2221 MHz  f  2296 MHz 2333 MHz  f  2393 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_19A-21A Configuration |  |  |  |
| 197.1 MHz  f  242.1 MHz 602.9 MHz  f  632.9 MHz | -36 dBm | 100 kHz |  |
| 2050.8 MHz f  2095.8 MHz 2277.9 MHz  f  2307.9 MHz 3107.9 MHz  f  3152.9 MHz 3725.8 MHz  f  3770.8 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_26A-46A Configuration |  |  |  |
| 3542 MHz ≤ f ≤ 4297 MHz 4301 MHz ≤ f ≤ 5111 MHz 5964 MHz ≤ f ≤ 6774 MHz 6778 MHz ≤ f ≤ 7623 MHz 9541 MHz ≤ f ≤ 11036 MHz 11114 MHz ≤ f ≤ 12699 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_39A-41A Configuration |  |  |  |
| 576 MHz  f  810 MHz | -36 dBm | 100 kHz |  |
| 1070 MHz f  1344 MHz 3072 MHz  f  3500 MHz 4376 MHz  f  4610 MHz 6256 MHz  f  6530 MHz 6872 MHz  f  7300 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_41A-42A Configuration |  |  |  |
| 710 MHz  f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz f  1104 MHz 1392 MHz  f  1980 MHz 4110 MHz  f  4704 MHz 5896 MHz  f  6290 MHz 8392 MHz  f  8980 MHz 9296 MHz  f  9890 MHz | -30 dBm | 1 MHz |  |

## << End of changes >>