3GPP TSG-RAN5 Meeting #82	R5-19xxxx

# Athens, Greece, 25th Feb 2019 - 1th March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | yyyy | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test configurations for NS_05/NS_08 in 6.2.4A.2 A-MPR for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test, LTE_CA_Rel12_2UL-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 6.2.4A.2: Test configuration and test requirements for NS_05/NS_08 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NS_05/NS_08 Test Configuration Table ：Table 6.2.4A.2.4.1-13 - NS_05/NS_08 Test requirement ：Table 6.2.4A.2.5-12 - NS_08 Message contents exceptions ：6.2.4A.2.4.3.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tests for NS_05/NS_08 cannnot be carried out |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

#### 6.2.4A.2	Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA)

Editor's note: For a transition period until RAN5#73, this test case in version 13.1.0 of 36.521-1 shall be used. This is to ensure no test coverage is lost before the UL 64QAM test case becomes available. Based on analysis for CA_3A-19A in TR 36.905 [19] no requirements in additional spurious emissions are required so A-MPR requirements are not tested for this CA configuration in any test case.

##### 6.2.4A.2.1	Test purpose

Additional ACLR, spectrum emission and spurious emission requirements for carrier aggregation can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the CA Power Class as specified in Table 6.2.2A.0-1.

##### 6.2.4A.2.2	Test applicability

The requirements of this test apply in test case 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for network signalled value NS_01/NS_03, NS_03/NS_03, NS_03/NS_06, NS_03/NS_07, NS_03/NS_01, NS_01/NS_06, NS_01/NS_21, NS_03/NS_21, NS_06/NS_21, NS_03/NS_28, NS_03/NS_29, NS_03/NS_30, NS_03/NS_31 to all types of E-UTRA UE release 11 and forward but do not support UL 64QAM or UL 256QAM.

The requirements of this test apply in test case 6.6.3.3A.2 Additional spurious emissions for CA (inter-band DL CA and UL CA) for network signalled values NS_05/NS_01, NS_05/NS_08, NS_03/NS_07, NS_01/NS_12 and NS_01/NS_15, NS_01/NS_21, NS_03/NS_21, NS_06/NS_21, NS_03/NS_28, NS_03/NS_29, NS_03/NS_30, NS_03/NS_31 to all types of E-UTRA UE release 11 and forward but do not support UL 64QAM or UL 256QAM.

NOTE:	As a result TC 6.2.4A.2 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

##### 6.2.4A.2.3	Minimum conformance requirements

If for inter-band carrier aggregation the UE is configured for transmissions on a single serving cell, then subclauses 6.2.3 and 6.2 4 apply with the Network Signalling value indicated by the field additionalSpectrumEmission.

For inter-band carrier aggregation with the UE configured for transmissions on two serving cells the maximum output power reduction specified in Table 6.2.4-1 is allowed for each serving cell of the applicable uplink CA configuration according to the Network Signalling value indicated by the field additionalSprectrumEmission for the PCC and the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10 for the SCC. The value of additionalSpectrumEmissionSCell-r10 is equal to that of additionalSprectrumEmission configured on the SCC. MPR as specified in subclause 6.2.3A is allowed in addition.

For PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth. The normative reference for this requirement is in TS 36.101 [2] clause 6.2.4A.

##### 6.2.4A.2.4	Test description

###### 6.2.4A.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2.4A.2.4.1-1, the power limits specified in subclause 6.2.5A.2.3 apply.

Table 6.2.4A.2.4.1-1: Additional Maximum Power Reduction (A-MPR) for inter-band CA

| Uplink CA Configuration | NS-value per CC (in same order as UL CA Configuration column) |  | Applicable emissions requirements (in same order as UL CA Configuration column) |  |
| --- | --- | --- | --- | --- |
| CA_1A-3A CA_1A-5A CA_1A-7A CA_1A-8A CA_1A-18A CA_1A-26A CA_1A-42A | NS_05 | NS_01 | Table 6.6.3.3.5.2-1 (Note 5) | N/A |
| CA_1A-19A | NS_05 | NS_08 | Table 6.6.3.3.5.2-1 (Note 5) | Table 6.6.3.3.5.4-1 |
| CA_1A-21A | NS_05 | NS_09 | Table 6.6.3.3.5.2-1 (Note 5) | Table 6.6.3.3.5.5-1 (Note 5) |
| CA_2A-4A, CA_2A-66A | NS_03 | NS_03 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_2A-12A, CA_2A-13A, CA_4A-12A, CA_4A-13A, CA_4A-17A | NS_03 | NS_06 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-13A, CA_4A-13A | NS_03 | NS_07 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 Table 6.6.3.3.3.3-1 |
| CA_2A-30A | NS_03 | NS_21 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 Table 6.6.3.3.5.16-1 (Note 8) |
| CA_2A-46A | NS_03 | NS_28 | Table 6.6.2.2.3.1-1 | Table 6.6.3.3.3.22-1 |
| CA_2A-46A | NS_03 | NS_29 | Table 6.6.2.2.3.1-1 | Table 6.6.3.3.3.23-1 |
| CA_2A-46A | NS_03 | NS_30 | Table 6.6.2.2.3.1-1 | Table 6.6.3.3.3.24-1 Table 6.6.3.3.3.24-2 Table 6.6.3.3.3.24-3 |
| CA_2A-46A | NS_03 | NS_31 | Table 6.6.2.2.3.1-1 | Table 6.6.3.3.3.25-1 Table 6.6.3.3.3.25-2 Table 6.6.3.3.3.25-3 Table 6.6.3.3.3.25-4 |
| CA_3A-5A CA_3A-7A CA_3A-8A CA_3A-18A CA_5A-7A CA_11A-18A CA_39A-41A | NS_01 | NS_01 | N/A | N/A |
| CA_3A-19A | NS_01 | NS_08 | N/A | Table 6.6.3.3.5.4-1 |
| CA_3A-20A CA_7A-20A | NS_01 | NS_10 | N/A | N/A (Note 6) |
| CA_3A-26A | NS_01 | NS_12 | N/A | Table 6.6.3.3.3.7-1 (Note 5) |
| CA_3A-26A | NS_01 | NS_13 | N/A | Table 6.6.3.3.3.8-1 (Note 5) |
| CA_3A-26A | NS_01 | NS_14 | N/A | Table 6.6.3.3.3.9-1 (Note 5) |
| CA_3A-26A | NS_01 | NS_15 | N/A | Table 6.6.3.3.3.10-1 (Note 5) |
| CA_2A-5A CA_2A-7A CA_4A-5A CA_4A-7A | NS_03 | NS_01 | Table 6.6.2.2.3.1-1 | N/A |
| CA_5A-66ANote 7 | NS_01 | NS_03 | N/A | Table 6.6.2.2.3.1-1 |
| CA_5A-12A CA_5A-17A | NS_01 | NS_06 | N/A | Table 6.6.2.2.3.3-1 |
| CA_5A-30A | NS_01 | NS_21 | N/A | Table 6.6.2.2.3.1-1 Table 6.6.3.3.5.16-1 (Note 8) |
| 3A-28A CA_7A-28A CA_18A-28A | NS_01 | NS_17 | N/A | Table 6.6.3.3.5.12-1 (Note 5) |
| CA_3A-28A CA_7A-28A | NS_01 | NS_18 | N/A | Table 6.6.3.3.5.13-1 (Note 5) |
| CA_12A-30A | NS_06 | NS_21 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.3-1 Table 6.6.3.3.5.16-1 (Note 8) |
| CA_12A-66A | NS_06 | NS_03 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.1-1 |
| CA_19A-21A | NS_08 | NS_09 | Table 6.6.3.3.5.4-1 (Note 5) | Table 6.6.3.3.5.5-1 (Note 5) |
| CA_1A-28A | NS_05 | NS_17 | Table 6.6.3.3.5.2-1 | Table 6.6.3.3.5.12-1 (Note 5) |
| Note 1: For CCs with NS_01, there are no additional requirements defined in 36.101. Only CCs with NS-value different than NS_01 needs to be tested. Note 2: A-MPR values for NS-values in this table are the same as in section 6.2.4.3. Note 3: Selection of CA configurations and NS-values to test is derived in TR 36.905 [19] Note 4: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column Note 5: No test required since only A-Spur requirements apply, and the frequency range to test is unaffected by intermodulation products as described in section 4.3 in TR 36.905 [19] and refer to Table 4.1.3-1 in TR 36.905. Note 6: No test required since no A_SEM and/or A-Spur requirements apply Note 7: As an exception to note 4, PCC is mapped to the second band of the combination as listed in Uplink CA Configuration column. Note 8: Only A_SEM requirements apply, as the frequency range to test for A-Spur requirements is unaffected by intermodulation products as described in section 4.3 in TR 36.905 [19] and refer to Table 4.1.3-1 in TR 36.905. |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.2.4A.2.4.1-2 to 6.2.4A.1.4.1-18. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4A.2.4.1-2: Test Configuration Table (network signalled value NS_01/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 15 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 15 | 15 |  | 16QAM | 16QAM | 25 | 9 |
| 3 | 15 | 15 |  | QPSK | 16QAM | 6 | 9 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 15 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 15 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-3: Test Configuration Table (network signalled value NS_01/NS_12)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for PCC SCC: (Note 3) For 5 MHz Channel Bandwidth: UL 816.7 MHz (NUL = 26717) For 10 MHz Channel Bandwidth: UL 819.2 MHz (NUL = 26742) For 15 MHz Channel Bandwidth: UL 821.7 MHz (NUL = 26767) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_12) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations (LCRB @ RBstart)(Note 4) |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8@7 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 10 | 8 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 8 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12@16 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 15 | 18 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 18 |
| 7 | 100 | 75 |  | QPSK | QPSK | 15 | 16@31 |
| 8 | 100 | 75 |  | 16QAM | 16QAM | 20 | 30 |
| 9 | 100 | 75 |  | QPSK | 16QAM | 15 | 30 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1 Note 4: Test point with max A-MPR in B26 selected in region A with largest allocation giving max A-MPR at RBstart0; test point with min A-MPR in B26 selected in region A with same allocation but with shifted RBstart to give A-MPR=0; |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-4: Test Configuration Table (network signalled value NS_01/NS_15)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | High range for both PCC and SCC(Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_15) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations (LCRB @ RBstart)(Note 4) |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 6@14 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 10 | 10@15 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 10@15 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12@25 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 15 | 15@35 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 15@35 |
| 7 | 100 | 75 |  | QPSK | QPSK | 18 | 18@19 |
| 8 | 100 | 75 |  | 16QAM | 16QAM | 20 | 20@55 |
| 9 | 100 | 75 |  | QPSK | 16QAM | 18 | 20@55 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 4: Test point with max A-MPR in B26 selected in region C with allocation giving max MPR+A-MPR+deltaTc at max RBend; Test point with min A-MPR in B26 selected in region B with max allocation and RBend giving A-MPR=0. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-5: Test Configuration Table (network signalled value NS_01/NS_17)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_17) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 5 | 5 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 5 | 5 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 5 | 5 |  | 16QAM | QPSK | 25 | 6 |
| 4 | 5 | 10 |  | QPSK | QPSK | 6 | 12 |
| 5 | 5 | 10 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 5 | 10 |  | 16QAM | QPSK | 25 | 12 |
| 7 | 10 | 10 |  | QPSK | QPSK | 12 | 15 |
| 8 | 10 | 10 |  | 16QAM | 16QAM | 50 | 25 |
| 9 | 10 | 10 |  | 16QAM | QPSK | 50 | 12 |
| 10 | 15 | 10 |  | QPSK | QPSK | 16 | 12 |
| 11 | 15 | 10 |  | 16QAM | 16QAM | 50 | 25 |
| 12 | 15 | 10 |  | 16QAM | QPSK | 50 | 12 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-6: Test Configuration Table (network signalled value NS_01/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 25 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 16QAM | 16QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 16QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-7: Test Configuration Table (network signalled value NS_03/NS_01 or network signalled value NS_01/NS_03)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and high range for PCC Mid range for SCC(Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | NS_03 CC | NS_01 CC | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 7 | 50 | 100 |  | QPSK | QPSK | 6 | 18 |
| 8 | 50 | 100 |  | 16QAM | 16QAM | 50 | 100 |
| 9 | 50 | 100 |  | 16QAM | QPSK | 50 | 18 |
| 10 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 11 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 12 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| 13 | 100 | 100 |  | QPSK | QPSK | 10 | 18 |
| 14 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 15 | 100 | 100 |  | 16QAM | QPSK | 100 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. For Mid range place test point with RBstart=0 and RBstart=RBmax-LCRB+1. Note 4: Mapping of NS-value per CC and PCC/SCC selection are specified in Table 6.2.4A.2.4.1-1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-8: Test Configuration Table (network signalled value NS_03/NS_03)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_03) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 6 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 6 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 7 | 50 | 50 |  | QPSK | QPSK | 6 | 6 |
| 8 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 9 | 50 | 50 |  | 16QAM | QPSK | 50 | 6 |
| 10 | 100 | 100 |  | QPSK | QPSK | 10 | 10 |
| 11 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 12 | 100 | 100 |  | 16QAM | QPSK | 100 | 10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-9: Test Configuration Table (network signalled value NS_03/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 8 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 8 |
| 4 | 25 | 15 |  | QPSK | QPSK | 6 | 4 |
| 5 | 25 | 15 |  | 16QAM | 16QAM | 25 | 15 |
| 6 | 25 | 15 |  | 16QAM | QPSK | 25 | 4 |
| 7 | 25 | 25 |  | QPSK | QPSK | 6 | 8 |
| 8 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 9 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 10 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 11 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 12 | 25 | 50 |  | 16QAM | QPSK | 25 | 12 |
| 13 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 14 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 15 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 16 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 17 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 18 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| X+1 | 75 | 25 |  | QPSK | QPSK | 8 | 8 |
| X+2 | 75 | 25 |  | 16QAM | 16QAM | 75 | 25 |
| X+3 | 75 | 25 |  | 16QAM | QPSK | 75 | 8 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-10: Test Configuration Table (network signalled value NS_03/NS_07)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 50 | N/A for this test | QPSK | QPSK | 6 | 7@13 |
| 2 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 3 | 25 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 7@13 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 7 | 100 | 50 |  | QPSK | QPSK | 10 | 7@13 |
| 8 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 9 | 100 | 50 |  | QPSK | 16QAM | 10 | 50 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-11: Test Configuration Table (network signalled value NS_03/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 100 | 50 |  | QPSK | QPSK | 10 | 12@7 |
| 5 (Note 5) | 100 | 50 |  | QPSK | QPSK | 10 | 12@31 |
| 6 | 100 | 50 |  | 16QAM | 16QAM | 100 | 1 |
| 7 | 100 | 50 |  | QPSK | 16QAM | 10 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-12: Test Configuration Table (network signalled value NS_05/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low range, Mid range for PCC In case of Low range: - For 5MHz Channel Bandwidth: 1927.2MHz (NUL = 18072) - For 10 MHz Channel Bandwidth: 1934.7 MHz (NUL = 18147) - For 20 MHz Channel Bandwidth: 1930 MHz (NUL = 18100) Mid range for SCC(Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_05) | SCC (NS_01) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 15 | N/A for this test | QPSK | QPSK | 6 | 4 |
| 2 | 25 | 15 |  | 16QAM | 16QAM | 25 | 6 |
| 3 | 25 | 15 |  | 16QAM | QPSK | 25 | 4 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 7 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 8 | 25 | 50 |  | 16QAM | 16QAM | 25 | 25 |
| 9 | 25 | 50 |  | 16QAM | QPSK | 25 | 12 |
| 10 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 11 | 50 | 50 |  | 16QAM | 16QAM | 50 | 25 |
| 12 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 13 | 100 | 50 |  | QPSK | QPSK | 18 | 12 |
| 14 | 100 | 50 |  | 16QAM | 16QAM | 50 | 25 |
| 15 | 100 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 16 | 100 | 75 |  | QPSK | QPSK | 18 | 16 |
| 17 | 100 | 75 |  | 16QAM | 16QAM | 50 | 25 |
| 18 | 100 | 75 |  | 16QAM | QPSK | 50 | 16 |
| 19 | 100 | 100 |  | QPSK | QPSK | 18 | 18 |
| 20 | 100 | 100 |  | 16QAM | 16QAM | 50 | 25 |
| 21 | 100 | 100 |  | 16QAM | QPSK | 50 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB PCC is tested. Note 3: For Low range place test point with RBstart=0. For Mid range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-13: Test Configuration Table (network signalled value NS_05/NS_08)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low range, Mid range for PCC(Note 3) In case of Low range: - For 5MHz Channel Bandwidth: 1927.2MHz (NUL = 18072) - For 10 MHz Channel Bandwidth: 1934.7 MHz (NUL = 18147) - For 20 MHz Channel Bandwidth: 1930 MHz (NUL = 18100) Mid range for SCC(Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_05) | SCC (NS_08) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 | 100 | 75 |  | QPSK | QPSK | 18 | 16 |
| 5 | 100 | 75 |  | 16QAM | 16QAM | 50 | 75 |
| 6 | 100 | 75 |  | 16QAM | QPSK | 50 | 16 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB PCC is tested. Note 3: For Low range place test point with RBstart=0. For Mid range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-14: Test Configuration Table (network signalled value NS_06/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 16QAM | 16QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 16QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-15: Test Configuration Table (network signalled value NS_03/NS_28)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Mid range for PCC. For SCC a. 5340 MHz for config ID 1-4 b. 5520 MHz for config ID 5-8 |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 100 | N/A for this test | QPSK | QPSK | 6 | L=1 |
| 2 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 3 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 4 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 5 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 6 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 7 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 8 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-16: Test Configuration Table (network signalled value NS_03/NS_29)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Mid range for PCC. For SCC, for configuration ID: a. 5180 MHz for config ID 1-4 b. 5240 MHz for config ID 5-8 c. 5260 MHz for config ID 9-12 d. 5320 MHz for config ID 13-16 e. 5500 MHz for config ID 17-20 f. 5700 MHz for config ID 21-24 |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 100 | N/A for this test | QPSK | QPSK | 6 | L=1 |
| 2 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 3 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 4 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 5 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 6 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 7 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 8 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 9 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 10 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 11 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 12 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 13 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 14 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 15 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 16 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 17 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 18 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 19 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 20 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 21 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 22 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 23 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 24 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-17: Test Configuration Table (network signalled value NS_03/NS_30)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for PCC. For SCC, for configuration ID: a. 5160 MHz for config ID 1-4 b. 5320 MHz for config ID 5-8 c. 5480 MHz for config ID 9-12 d. 5700 MHz for config ID 13-16 e. 5180 MHz for config ID 17-20 |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 100 | N/A for this test | QPSK | QPSK | 6 | L=1 |
| 2 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 3 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 4 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 5 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 6 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 7 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 8 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 9 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 10 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 11 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 12 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 13 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 14 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 15 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 16 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 17 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 18 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 19 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 20 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-18: Test Configuration Table (network signalled value NS_03/NS_31)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for PCC. For SCC, for configuration ID: a. 5160 MHz for config ID 1-4 b. 5260 MHz for config ID 5-8 c. 5700 MHz for config ID 9-12 d. 5745 MHz for config ID 13-16 e. 5600 MHz for config ID 17-20 |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 100 | N/A for this test | QPSK | QPSK | 6 | L=1 |
| 2 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 3 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 4 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 5 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 6 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 7 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 8 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 9 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 10 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 11 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 12 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 13 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 14 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 15 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 16 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 17 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 18 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 19 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 20 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to the applicable table from Table 6.2.4A.2.4.1-2 to 6.2.4A.2.4.1-18.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4A.2.4.3.

###### 6.2.4A.2.4.2	Test procedure

1.	 Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.2.4A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to the applicable from Table 6.2.4A.2.4.1-2 to Table 6.2.4A.2.4.1-18 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

6.	Measure the mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

###### 6.2.4A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. The following exceptions apply for each network signalled value.

6.2.4A.2.4.3.1	Message contents exceptions (network signalled value " NS_03" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_03 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

6.2.4A.2.4.3.2	Message contents exceptions (network signalled value " NS_03" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_03 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.2-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_03 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.2-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_03" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 3 (NS_03) |  |  |

6.2.4A.2.4.3.3	Message contents exceptions (network signalled value " NS_05" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_05 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.3-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_05" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_05) |  |  |

6.2.4A.2.4.3.4	Message contents exceptions (network signalled value " NS_06" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_06 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.4-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_06" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 6 (NS_06) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_06 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.4-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_06" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 6 (NS_06) |  |  |

6.2.4A.2.4.3.5	Message contents exceptions (network signalled value " NS_07" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_07 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.5-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_07" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 7 (NS_07) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_07 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.5-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_07" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 7 (NS_07) |  |  |

6.2.4A.2.4.3.6	Message contents exceptions (network signalled value " NS_08" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_08 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.6-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_08" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 8 (NS_08) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_08 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.6-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_08" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 8 (NS_08) |  |  |

6.2.4A.2.4.3.7	Message contents exceptions (network signalled value " NS_12" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_12 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.7-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_12" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 12 (NS_12) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_12 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.7-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_12" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 12 (NS_12) |  |  |

6.2.4A.2.4.3.8	Message contents exceptions (network signalled value " NS_15" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_15 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.8-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_15" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 15 (NS_15) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_15 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.8-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_15" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 15 (NS_15) |  |  |

6.2.4A.2.4.3.9	Message contents exceptions (network signalled value " NS_21" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_21 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.9-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_21" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 21 (NS_21) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_21 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.9-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_21" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 21 (NS_21) |  |  |

6.2.4A.2.4.3.10	Message contents exceptions (network signalled value " NS_28" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_28 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.10-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_28" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 28 (NS_28) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_28 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.10-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_28" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 28 (NS_28) |  |  |

6.2.4A.2.4.3.11	Message contents exceptions (network signalled value " NS_29" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_29 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.11-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_29" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 29 (NS_29) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_29 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.11-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_29" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 29 (NS_29) |  |  |

6.2.4A.2.4.3.12	Message contents exceptions (network signalled value " NS_30" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_30 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.12-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_30" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 30 (NS_30) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_30 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.12-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_30" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 30 (NS_30) |  |  |

6.2.4A.2.4.3.13	Message contents exceptions (network signalled value " NS_31" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_31 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.13-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_31" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 31 (NS_31) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_31 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.13-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_31" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 31 (NS_31) |  |  |

##### 6.2.4A.2.5	Test requirements

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4A.2.5-1 to Table 6.2.4A.2.5-17. For the UE maximum output power modified by A-MPR specified in sub-clause 6.2.4A.2.3, the power limits specified in Table 6.2.5A.3.3-1 apply.

Table 6.2.4A.2.5-1: Test requirement (network signalled value NS_01/NS_03 or network signalled value NS_03/NS_01)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 13 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 14 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 15 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-2: Test requirement (network signalled value NS_01/NS_06)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 1.5 | 23 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 1.5 | 21 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 0 | 2 | 0 | 0 | 0 | 1.5 | 23 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 1.5 | 23 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 0 | 0 | 1.5 | 21 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 0 | 2 | 0 | 0 | 0 | 1.5 | 23 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-3: Test requirement (network signalled value NS_01/NS_12)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 1 | 0 | 5 | 1.5 | 1.5 | 19.5 | 15.5 | 21.0 | 5 | 25.7 | 15.3 |
| 3 | 0 | 1 | 0 | 5 | 1.5 | 1.5 | 21.5 | 15.5 | 22.5 | 5 | 25.7 | 16.8 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 4 | 1.5 | 1.5 | 19.5 | 15.5 | 21.0 | 5 | 25.7 | 15.3 |
| 6 | 0 | 2 | 0 | 4 | 1.5 | 1.5 | 21.5 | 15.5 | 22.5 | 5 | 25.7 | 16.8 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 0 | 4 | 0 | 0 | 21 | 17 | 22.5 | 5 | 25.7 | 16.8 |
| 9 | 0 | 2 | 0 | 4 | 1.5 | 0 | 21.5 | 17 | 22.8 | 5 | 25.7 | 17.1 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-4: Test requirement (network signalled value NS_01/NS_15)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 9 | 1.5 | 1.5 | 19.5 | 10.5 | 20.0 | 6 | 25.7 | 13.3 |
| 3 | 0 | 2 | 0 | 9 | 1.5 | 1.5 | 21.5 | 10.5 | 21.8 | 5 | 25.7 | 16.1 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 9 | 1.5 | 1.5 | 19.5 | 10.5 | 20.0 | 6 | 25.7 | 13.3 |
| 6 | 0 | 2 | 0 | 9 | 1.5 | 1.5 | 21.5 | 10.5 | 21.8 | 5 | 25.7 | 16.1 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 0 | 9 | 0 | 0 | 21 | 12 | 21.5 | 5 | 25.7 | 15.8 |
| 9 | 0 | 2 | 0 | 9 | 0 | 0 | 21.5 | 12 | 23 | 5 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-5: Test requirement (network signalled value NS_01/NS_17)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-6: Test requirement (network signalled value NS_01/NS_21)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3.0 | 25.7 | 19.3 |
| 3 | 0 | 2 | 0 | 0 | 0 | 0 | 23 | 21 | 23.0 | 3.0 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 5 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 6 | 2 | 1 | 0 | 4 | 0 | 0 | 21 | 18 | 22.8 | 5.0 | 25.7 | 17.1 |
| 7 | 0 | 1 | 0 | 4 | 0 | 0 | 23 | 18 | 23.0 | 3.0 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-7: Test requirement (network signalled value NS_03/NS_03)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 1 | 1.5 | 0 | 18.5 | 20 | 22.3 | 5 | 25.7 | 16.6 |
| 3 | 2 | 0 | 1 | 0 | 1.5 | 0 | 18.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 1 | 0 | 1.5 | 0 | 18.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-8: Test requirement (network signalled value NS_03/NS_06)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 0 | 1.5 | 0 | 18.5 | 21 | 22.9 | 5 | 25.7 | 17.2 |
| 3 | 2 | 0 | 1 | 0 | 1.5 | 1.5 | 18.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 13 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 14 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 15 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 16 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 17 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 18 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| X+1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| X+2 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| X+3 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-9: Test requirement (network signalled value NS_03/NS_07)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| CA_2A-13A |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 3 | 0 | 2 | 0 | 12 | 1.5 | 0 | 21.5 | 9 | 21.7 | 5 | 25.7 | 16.0 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 6 | 0 | 2 | 0 | 12 | 1.5 | 0 | 21.5 | 9 | 21.7 | 5 | 25.7 | 16.0 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 9 | 0 | 2 | 0 | 12 | 1.5 | 0 | 21.5 | 9 | 21.7 | 5 | 25.7 | 16.0 |
| CA_4A-13A |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 3 | 0 | 2 | 0 | 12 | 0 | 0 | 23 | 9 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 6 | 0 | 2 | 0 | 12 | 0 | 0 | 23 | 9 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 9 | 0 | 2 | 0 | 12 | 0 | 0 | 23 | 9 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-10: Test requirement (network signalled value NS_03/NS_21)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3.0 | 25.7 | 19.3 |
| 3 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 5 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 6 | 2 | 1 | 1 | 4 | 0 | 0 | 20 | 18 | 22.1 | 5.0 | 25.7 | 16.4 |
| 7 | 0 | 1 | 0 | 4 | 1.5 | 0 | 21.5 | 18 | 23.0 | 3.0 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-11: Test requirement (network signalled value NS_05/NS_01)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 13 | 0 | 0 | 11 | 0 | 0 | 0 | 12 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 14 | 2 | 2 | 11 | 0 | 0 | 0 | 10 | 21 | 21.3 | 5 | 25.7 | 15.6 Note 3 |
| 15 | 2 | 0 | 11 | 0 | 0 | 0 | 10 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 16 | 0 | 0 | 11 | 0 | 0 | 0 | 12 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 17 | 2 | 2 | 11 | 0 | 0 | 0 | 10 | 21 | 21.3 | 5 | 25.7 | 15.6 Note 3 |
| 18 | 2 | 0 | 11 | 0 | 0 | 0 | 10 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 19 | 0 | 0 | 11 | 0 | 0 | 0 | 12 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 20 | 2 | 2 | 11 | 0 | 0 | 0 | 10 | 21 | 21.3 | 5 | 25.7 | 15.6 Note 3 |
| 21 | 2 | 0 | 11 | 0 | 0 | 0 | 10 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: Void Note 3: For PCC in mid frequency range the lower limit is 19.3 dBm. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-12: Test requirement (network signalled value NS_05/NS_08)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.00 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.00 | 3 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.00 | 3 | 25.7 | 19.3 |
| 4 (Note3) | 0 | 0 | 11 | 0 | 0 | 0 | 12 | 23 | 23.00 | 3 | 25.7 | 19.3 |
| 4 (Note4) | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.00 | 3 | 25.7 | 19.3 |
| 5 (Note3) | 2 | 2 | 11 | 3 | 0 | 0 | 10 | 18 | 18.64 | 5 | 25.7 | 12.9 |
| 5 (Note4) | 2 | 2 | 1 | 3 | 0 | 0 | 20 | 18 | 22.12 | 5 | 25.7 | 16.4 |
| 6 (Note3) | 2 | 0 | 11 | 0 | 0 | 0 | 10 | 23 | 23.00 | 3 | 25.7 | 19.3 |
| 6 (Note4) | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.00 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points. Note 3: Applicable only to Low range frequency for PCC testing. Note 4: Applicable only to Mid range frequency for PCC testing. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-13: Test requirement (network signalled value NS_06/NS_21)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3.0 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 5 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 6 | 2 | 1 | 0 | 4 | 0 | 0 | 21 | 18 | 22.8 | 5.0 | 25.7 | 17.1 |
| 7 | 0 | 1 | 0 | 4 | 1.5 | 0 | 21.5 | 18 | 23.0 | 3.0 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-14: Test requirement (network signalled value NS_03/NS_28)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 2.5 | 0 | 8 | 0 | 0 | 23 | 12.5 | 23.0 | 3 | 25.7 | 18.5 |
| 2 | 2 | 2 | 1 | 9 | 0 | 0 | 20 | 12 | 20.6 | 6 | 25.7 | 13.1 |
| 3 | 0 | 2.5 | 0 | 8 | 0 | 0 | 23 | 12.5 | 23.0 | 3 | 25.7 | 18.5 |
| 4 | 2 | 2 | 1 | 9 | 0 | 0 | 20 | 12 | 20.6 | 6 | 25.7 | 13.1 |
| 5 | 0 | 2.5 | 0 | 2 | 0 | 0 | 23 | 18.5 | 23.0 | 3 | 25.7 | 18.5 |
| 6 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 7 | 0 | 2.5 | 0 | 2 | 0 | 0 | 23 | 18.5 | 23.0 | 3 | 25.7 | 18.5 |
| 8 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-15: Test requirement (network signalled value NS_03/NS_29)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 2 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 3 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 4 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 5 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 6 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 7 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 8 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 9 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 10 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 11 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 12 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 13 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 14 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 15 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 16 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 17 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 18 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 19 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 20 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 21 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 22 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 23 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 24 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-16: Test requirement (network signalled value NS_03/NS_30)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 2.5 | 0 | 16 | 0 | 0 | 23 | 4.5 | 23.0 | 3 | 25.7 | 18.5 |
| 2 | 2 | 2 | 1 | 15 | 0 | 0 | 20 | 6 | 20.2 | 6 | 25.7 | 12.7 |
| 3 | 0 | 2.5 | 0 | 16 | 0 | 0 | 23 | 4.5 | 23.0 | 3 | 25.7 | 18.5 |
| 4 | 2 | 2 | 1 | 15 | 0 | 0 | 20 | 6 | 20.2 | 6 | 25.7 | 12.7 |
| 5 | 0 | 2.5 | 0 | 3 | 0 | 0 | 23 | 17.5 | 23.0 | 3 | 25.7 | 18.5 |
| 6 | 2 | 2 | 1 | 3 | 0 | 0 | 20 | 18 | 22.1 | 5 | 25.7 | 15.6 |
| 7 | 0 | 2.5 | 0 | 3 | 0 | 0 | 23 | 17.5 | 23.0 | 3 | 25.7 | 18.5 |
| 8 | 2 | 2 | 1 | 3 | 0 | 0 | 20 | 18 | 22.1 | 5 | 25.7 | 15.6 |
| 9 | 0 | 2.5 | 0 | 11 | 0 | 0 | 23 | 9.5 | 23.0 | 3 | 25.7 | 18.5 |
| 10 | 2 | 2 | 1 | 11 | 0 | 0 | 20 | 10 | 20.4 | 6 | 25.7 | 12.9 |
| 11 | 0 | 2.5 | 0 | 11 | 0 | 0 | 23 | 9.5 | 23.0 | 3 | 25.7 | 18.5 |
| 12 | 2 | 2 | 1 | 11 | 0 | 0 | 20 | 10 | 20.4 | 6 | 25.7 | 12.9 |
| 13 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 14 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 15 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 16 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 17 | 0 | 2.5 | 0 | NA | 0 | 0 | 23 | NA | NA | NA | NA | NA |
| 18 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23.0 | 3 | 25.7 | 18.5 |
| 19 | 0 | 2.5 | 0 | NA | 0 | 0 | 23 | NA | NA | NA | NA | NA |
| 20 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23.0 | 3 | 25.7 | 18.5 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-17: Test requirement (network signalled value NS_03/NS_31)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 2.5 | 0 | 7 | 0 | 0 | 23 | 13.5 | 23.0 | 3 | 25.7 | 18.5 |
| 2 | 2 | 2 | 1 | 7 | 0 | 0 | 20 | 14 | 21.0 | 5 | 25.7 | 14.5 |
| 3 | 0 | 2.5 | 0 | 7 | 0 | 0 | 23 | 13.5 | 23.0 | 3 | 25.7 | 18.5 |
| 4 | 2 | 2 | 1 | 7 | 0 | 0 | 20 | 14 | 21.0 | 5 | 25.7 | 14.5 |
| 5 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 6 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 7 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 8 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 9 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 10 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 11 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 12 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 13 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 14 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 15 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 16 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 17 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 18 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 19 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 20 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

## << End of changes >>