3GPP TSG-RAN5 Meeting #82	R5-191769

# Athens, Greece, 25th Feb 2019 – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1242 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test frequency for CA_40C-40C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS36.508 4.3.1.2.8A is missing the test frequencies for CA_40C-40C though they are already tested in 36.521-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TS36.508 4.3.1.2.8A : Test frequency for CA_40C-40C is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test frequency for CA_40C-40C remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2.8A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

##### 4.3.1.2.8A	TDD reference test frequencies for CA in operating band 40

Table 4.3.1.2.9A-0: Test frequencies for CA_40A-40A

| Test Frequency ID |  | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB] |  | NUL/DL | fUL/DL [MHz] |
| Max WGap | 50+50 | 50 | 38700 | 2305 | 80 |  | 50 | 39600 | 2395 |
|  | 50+75 | 50 | 38700 | 2305 | 75 |  | 75 | 39575 | 2392.5 |
|  |  | 75 | 38725 | 2307.5 | 75 |  | 50 | 39600 | 2395 |
|  | 50+100 | 50 | 38700 | 2305 | 70 |  | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 70 |  | 50 | 39600 | 2395 |
|  | 75+75 | 75 | 38725 | 2307.5 | 70 |  | 75 | 39575 | 2392.5 |
|  | 75+100 | 75 | 38725 | 2307.5 | 65 |  | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 65 |  | 75 | 39575 | 2392.5 |
|  | 100+100 | 100 | 38750 | 2310 | 60 |  | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-1: Test frequencies for CA_40C

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 50+100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 |
|  | 75+75 | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 |
|  | 75+100 | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 |
|  | 100+100 | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 |
| Mid | 50+100 | 50 | 39056 | 2340.6 | 100 | 39200 | 2355.0 |
|  |  | 100 | 39101 | 2345.1 | 50 | 39245 | 2359.5 |
|  | 75+75 | 75 | 39075 | 2342.5 | 75 | 39225 | 2357.5 |
|  | 75+100 | 75 | 39053 | 2340.3 | 100 | 39224 | 2357.4 |
|  |  | 100 | 39076 | 2342.6 | 75 | 39247 | 2359.7 |
|  | 100+100 | 100 | 39051 | 2340.1 | 100 | 39249 | 2359.9 |
| High | 50+100 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 75+75 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  | 75+100 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  | 100+100 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-2: Test frequencies for CA_40D

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |  | CC3 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 50+100+100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 100 | 39047 | 2339.7 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 100 | 39038 | 2338.8 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 50 | 39092 | 2344.2 |
|  | 75+100+100 | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 100 | 39097 | 2344.7 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 100 | 39092 | 2344.2 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 75 | 39119 | 2346.9 |
|  | 100+100+100 | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 100 | 39146 | 2349.6 |
| Mid | 50+100+100 | 50 | 38957 | 2330.7 | 100 | 39101 | 2345.1 | 100 | 39299 | 2364.9 |
|  |  | 100 | 39006 | 2335.6 | 50 | 39150 | 2350 | 100 | 39294 | 2364.4 |
|  |  | 100 | 39002 | 2335.2 | 100 | 39200 | 2355 | 50 | 39344 | 2369.4 |
|  | 75+100+100 | 75 | 38954 | 2330.4 | 100 | 39125 | 2347.5 | 100 | 39323 | 2367.3 |
|  |  | 100 | 38979 | 2332.9 | 75 | 39150 | 2350 | 100 | 39321 | 2367.1 |
|  |  | 100 | 38977 | 2332.7 | 100 | 39175 | 2352.5 | 75 | 39346 | 2369.6 |
|  | 100+100+100 | 100 | 38952 | 2330.2 | 100 | 39150 | 2350 | 100 | 39348 | 2369.8 |
| High | 50+100+100 | 50 | 39208 | 2355.8 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 39262 | 2361.2 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 39253 | 2360.3 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 75+100+100 | 75 | 39181 | 2353.1 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 39208 | 2355.8 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 39203 | 2355.3 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  | 100+100+100 | 100 | 39154 | 2350.4 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-3: Test frequencies for CA_40E

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |  | CC3 Note1 |  |  | CC4 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 75+75+75+100 | 75 | 38728 | 2307.8 | 75 | 38878 | 2322.8 | 75 | 39028 | 2337.8 | 100 | 39199 | 2354.9 |
|  | 75+75+100+100 | 75 | 38728 | 2307.8 | 75 | 38878 | 2322.8 | 100 | 39049 | 2339.9 | 100 | 39247 | 2359.7 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 75 | 39070 | 2342 | 100 | 39241 | 2359.1 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 75 | 39071 | 2342.1 | 100 | 39242 | 2359.2 |
|  | 75+100+100+100 | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 100 | 39097 | 2344.7 | 100 | 39295 | 2364.5 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 100 | 39092 | 2344.2 | 100 | 39290 | 2364 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 75 | 39119 | 2346.9 | 100 | 39290 | 2364 |
|  | 100+100+100+100 | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 100 | 39146 | 2349.6 | 100 | 39344 | 2369.4 |
| Mid | 75+75+75+100 | 75 | 38903 | 2325.3 | 75 | 39053 | 2340.3 | 75 | 39203 | 2355.3 | 100 | 39374 | 2372.4 |
|  | 75+75+100+100 | 75 | 38879 | 2322.9 | 75 | 39029 | 2337.9 | 100 | 39200 | 2355 | 100 | 39398 | 2374.8 |
|  |  | 75 | 38882 | 2323.2 | 100 | 39053 | 2340.3 | 75 | 39224 | 2357.4 | 100 | 39395 | 2374.5 |
|  |  | 100 | 38904 | 2325.4 | 75 | 39075 | 2342.5 | 75 | 39225 | 2357.5 | 100 | 39396 | 2374.6 |
|  | 75+100+100+100 | 75 | 38855 | 2320.5 | 100 | 39026 | 2337.6 | 100 | 39224 | 2357.4 | 100 | 39422 | 2377.2 |
|  |  | 100 | 38880 | 2323 | 75 | 39051 | 2340.1 | 100 | 39222 | 2357.2 | 100 | 39420 | 2377 |
|  |  | 100 | 38880 | 2323 | 100 | 39078 | 2342.8 | 75 | 39249 | 2359.9 | 100 | 39420 | 2377 |
|  | 100+100+100+100 | 100 | 38853 | 2320.3 | 100 | 39051 | 2340.1 | 100 | 39249 | 2359.9 | 100 | 39447 | 2379.7 |
| High | 75+75+75+100 | 75 | 39079 | 2342.9 | 75 | 39229 | 2357.9 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  | 75+75+100+100 | 75 | 39031 | 2338.1 | 75 | 39181 | 2353.1 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 75 | 39037 | 2338.7 | 100 | 39208 | 2355.8 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 39058 | 2340.8 | 75 | 39229 | 2357.9 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  | 75+100+100+100 | 75 | 38983 | 2333.3 | 100 | 39154 | 2350.4 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 39010 | 2336 | 75 | 39181 | 2353.1 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 39010 | 2336 | 100 | 39208 | 2355.8 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  | 100+100+100+100 | 100 | 38956 | 2330.6 | 100 | 39154 | 2350.4 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-4: Test frequencies for CA_40A-40A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 50+50 | 50 | 38700 | 2305 | 80 | 50 | 39600 | 2395 |
|  | 50+75 | 50 | 38700 | 2305 | 75 | 75 | 39575 | 2392.5 |
|  |  | 75 | 38725 | 2307.5 | 75 | 50 | 39600 | 2395 |
|  | 50+100 | 50 | 38700 | 2305 | 70 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 70 | 50 | 39600 | 2395 |
|  | 75+75 | 75 | 38725 | 2307.5 | 70 | 75 | 39575 | 2392.5 |
|  | 75+100 | 75 | 38725 | 2307.5 | 65 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 65 | 75 | 39575 | 2392.5 |
|  | 100+100 | 100 | 38750 | 2310 | 60 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-5: Test frequencies for CA_40A-40C

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  | CC2 Note1 |  |  | CC3 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 100+50+100 | 100 | 38750 | 2310 | 50.6 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 50.1 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 100+75+75 | 100 | 38750 | 2310 | 50 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  | 100+75+100 | 100 | 38750 | 2310 | 45.4 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 45.1 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  | 100+100+100 | 100 | 38750 | 2310 | 40.2 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-6: Test frequencies for CA_40C-40C

| Test frequency ID | CC-Combo / |  | CC1 |  |  | CC2 |  | Wgap |  | CC3 |  |  | CC4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  | Note1 |  |  | Note1 |  | [MHz] |  | Note1 |  |  | Note1 |  |
|  | [RB] | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |  | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |
|  |  | [RB] |  | [MHz] | [RB] |  | [MHz] |  | [RB |  | [MHz] | [RB] |  | [MHz] |
| Max Wgap | 50+50+ 100+100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 40.7 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 40.7 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  |  | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 40.2 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 41.2 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  | 50+75+75 100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 40.1 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 40.6 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  |  | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 40.6 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 40.1 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 75+75+ 75+75 | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 40 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  | 75+75+ 75+100 | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 35.4 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 35.1 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 35.1 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 35.4 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  | 50+75+ 100+100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 35.5 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 35.2 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 35.7 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 35.2 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 36 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 35.7 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 36 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 35.5 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 50+100+ 100+100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 30.3 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 30.8 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 30.8 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 30.3 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 75+75+ 100+100 | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 30.2 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 30.2 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 30.5 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 30.2 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 30.8 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 30.5 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  | 75+100+ 100+100 | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 25.3 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 25.6 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 25.6 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 25.3 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  | 100+100+ 100+100 | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 20.4 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.2.9	TDD reference test frequencies for Operating Band 41

## << End of changes >>