3GPP TSG-RAN WG5 Meeting # 82	R5-191840

# Athens, Greece,  25 February - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2403 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 4 DL CA Activation and Deactivation with generic duplex modes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel13-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following items are incomplete. - Editor’s note - Applicability spec - Test tolerlance including Annex F |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following items are updated 1) Remove Editor’s note 2) Update to applicability spec 3) Test tolerlance including Annex F 4) Annex E for Cell configuration mapping |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case are incomplete and the WI will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16.89, 8.16.90, Annex E, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-2 CR 0821 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

8.16.89.1	Test Purpose

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in TS 36.133 [4] section 7.7 for UE configured with three downlink SCells, when the SCell is known by the UE at the time of activation.

8.16.89.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE that support E-UTRA FDD and/or E-UTRA TDD and 4DL CA. If the generic duplex-mode test case 8.16.89 is tested with FDD PCell, it will be only applicable to UEs supporting FDD as PCell. If the generic duplex-mode test case 8.16.89 is tested with TDD PCell, it will only be applicable to UEs supporting TDD as PCell. Applicability requires support for FGI bit 25.

Note on the applicability: If the generic duplex-mode test case 8.16.89 is tested with FDD PCell, the specific duplex-mode test cases 8.16.57 and 8.16.59 do not need to be tested. If the generic duplex-mode test case 8.16.89 is tested with TDD PCell, the specific duplex-mode test cases 8.16.58 and 8.16.60 do not need to be tested.

8.16.89.3	Minimum conformance requirements

While activating a SCell if the UE does not receive any command to activate, deactivate, configure or deconfigure any other SCell during the SCell activation delay then the UE shall meet the SCell activation delay requirements specified in TS 36.133 [4] section 7.7.2.

While activating a SCell if any other SCell is activated, deactivated, configured or deconfigured by the UE then the UE shall meet the SCell activation delay requirements (Tactivate_total) according to the following expression:

Where:

Tactivate_total is the total time to activate a SCell and is expressed in subframes.

Tactivate_basic is the SCell activation delay specified in TS 36.133 [4] section 7.7.2;

Ki (0 ≤ Ki ≤ [3]) is the number of times the other ith SCell is activated, deactivated, configured or deconfigured while the SCell is being activated.

N (2≤N≤4) is the maximum number of SCells supported by the UE.

While activating a SCell:

-	The interruption on the PCell and/or on the activated SCell due to the SCell activation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+11 if:

-	the PCell and/or the activated SCell being interrupted and the SCell being activated belong to E-UTRA TDD, or

-	the activated SCell being interrupted and the SCell being activated belong to E-UTRA FDD and the PCell belongs to E-UTRA TDD.

-	Otherwise, the interruption on PCell and/or on the activated SCell due to the SCell activation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+9.

Starting from the subframe specified in section 4.3 of TS 36.213 [8] and until the UE has completed the SCell activation, the UE shall report CQI index = 0 (out of range) if the UE has available uplink resources to report CQI for an SCell.

The UE shall deactivate a SCell and meet the SCell deactivation delay requirements specified in TS 36.133 [4] section 7.7.3 regardless of whether the other SCell is activated, deactivated, configured or deconfigured or not by the UE during the SCell deactivation delay

While deactivating a SCell:

-	The interruption on the PCell and/or on the activated SCell due to the SCell deactivation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+11 if:

-	the PCell and/or the activated SCell being interrupted and the SCell being deactivated belong to E-UTRA TDD or

-	the activated SCell being interrupted and the SCell being deactivated belong to E-UTRA FDD and the PCell belongs to E-UTRA TDD.

-	Otherwise, the interruption on PCell and/or the activated SCell due to the SCell deactivation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+9.

The normative reference for this requirement is TS 36.133 [4] clause 7.7 and A.8.16.89.

8.16.89.4	Test description

8.16.89.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-2 and TS 36.508 [7] clauses 4.4.2 and 4.3.1 for different CA bandwidth classes.

Channel Bandwidth to be tested: The largest aggregated bandwidth combination supported by UE from Table 8.16.89.5-1 as defined in TS 36.508 [7] clause 4.3.1, for different CA configurations as defined in TS 36.521-1 [10] clause 5.4.2A.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as follows:

- For UEs supporting only 2Rx in all the bands under test, use TS 36.508 [7] Annex A, Figure group A.63 as appropriate.

- For UEs supporting 4Rx in any of the bands under test use TS 36.508 [7] Annex A, Figure A.90, without intra-frequency neighbours. For PCell, use the 4Rx module if the UE supports 4Rx for the corresponding band and the 2Rx module otherwise. For all other cells, the 2Rx module can be used.

2.	The general test parameter settings are set up according to Table 8.16.89.4.1-1.

3.	Propagation conditions are set according to Annex B clauses B.0.

4.	Message contents are defined in clause 8.16.89.4.3.

5.	Cell 1 is the PCell on the primary component carrier, Cell 2 is SCell on RF channel number 2, Cell 3 is the deactivated SCell on RF channel number 3 and Cell 4 is the deactivated SCell on RF channel number 4. Cell 1 is the cell used for connection setup with the power levels set according to Annex C.0 and C.1 for this test. Cell 2, Cell 3 and Cell 4 shall be powered OFF.

Table 8.16.89.4.1-1: General test parameters for 4DL CA Activation and Deactivation of Known SCell in non-DRX with generic duplex modes

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, and SCell3. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, and SCell3. |

8.16.89.4.2	Test procedure

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. All cells have constant signal levels throughout the test.

Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2) and Cell 4 (deactivated SCell3) on radio channel 4 (SCC3) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2 and SCC3. The UE shall be continuously scheduled in the PCell throughout the whole test.

There are four carriers, each with one cell. Cell 1, Cell 2, Cell 3 and Cell 4 may operate in either FDD or TDD duplex mode according to test configuration.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3.

2.	Configure SCC2 and SCC3 according to Annex C.0, C.1 for all downlink physical channels.

3.	The SS shall configure SCell2 (Cell 3) on the SCC2 and SCell3 (Cell 4) on the SCC3 as per TS 36.508 [7] clause 5.2A.4 with the message content exceptions defined in clause 8.16.89.4.3.

4.	The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

5.	Set the parameters according to T1 in Table 8.16.89.5-1. Propagation conditions are set according to Annex B clauses B.1.1. T1 starts. The SS shall configure SCell1 (Cell 2) on the SCC1 as per TS 36.508 [7] clause 5.2A.4 with the message content exceptions defined in clause 8.16.89.4.3.

6.	The SS activates SCell1 by sending the activation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8) in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell. If the SS receives ACK for MAC-CE sent by the UE, T2 starts in subframe m, and the test proceeds to step 7, otherwise go to step 12.

7.	The SS activates SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell and SCell 3 in subframe (m+20) for FDD PCell and (m+15) for TDD PCell by sending the activation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8).

8.	The UE shall start sending CSI reports for SCell1 and the SS shall monitor CSI reports for SCell1 sent from the UE and ACK/NACK sent in PCell during SCells.

8a.	For all intra-band PCell FDD case

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+34),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+34)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+13), the subframes (m+15) to (m+23) and the subframes (m+25) to (m+33) up to the end of T2

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

8b.	For the case where the band relation between PCC(FDD) and SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+34),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+34)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+13), the subframes (m+15) to (m+23) and the subframes (m+25) to (m+33) up to the end of T2,

-	and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+5) to (m+13), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+15) to (m+23) and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+25) to (m+33) up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

8c.	For the case where the band relation between PCC(FDD) and SCell1 is intra-band and the others SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+34),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+34)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+13), the subframes (m+15) to (m+23) and the subframes (m+25) to (m+33) up to the end of T2,

-	and ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (m+15) to (m+23) and ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (m+25) to (m+33) up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

8d.	For all intra-band PCell TDD case

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+9) if the subframe (m+8) was subject to interruption,

-	or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+34),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+34).

-	and DTX is not observed by the SS outside the subframes (m+5) to (m+9) and (m+13) and the subframes (m+20) to (m+24) and (m+28), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+6) to (m+10) and (m+13) to (m+14) and the subframes (m+21) to (m+25) and (m+28) to (m+29), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+7) to (m+11) and (m+13) to (m+14) and (m+18) and the subframes (m+22) to (m+26) and (m+28) to (m+29) and (m+33), up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

8e.	For the case where the band relation between PCC(TDD) and SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+9) if the subframe (m+8) was subject to interruption,

-	or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to (m+34),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+34),

-	and DTX is not observed by the SS outside the subframes (m+5), (m+9), (m+20) and (m+24), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+6), (m+13), (m+21) and (m+28), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+7), (m+13), (m+22) and (m+28), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+8) and (m+23), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+9) and (m+24), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+10), (m+14), (m+25) and (m+29), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+11), (m+18), (m+26) and (m+33), up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

8f.	For the case where the band relation between PCC(TDD) and SCell1 is intra-band and the others SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframes (m+8),

-	 or (m+9) if the subframe (m+8) was subject to interruption,

-	or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+34),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+34),

-	and DTX is not observed by the SS outside the subframes (m+5) to (m+9) and (m+13),

-	or DTX is not observed by the SS outside the subframes (m+6) to (m+10) and (m+13) to (m+14),

-	or DTX is not observed by the SS outside the subframes (m+7) to (m+11) and (m+13) to (m+14) and (m+18),

-	and DTX is not observed by the SS outside the subframes (m+20) and (m+24), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+21) and (m+28), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+22) and (m+28), up to the end of T2,

-	or DTX is not observed by the SS outside the subframe (m+23), up to the end of T2,

-	or DTX is not observed by the SS outside the subframe (m+24), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+25) and (m+29), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+26) and (m+33), up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

9.	When T2 expires, the SS deactivate SCell1 by sending the deactivation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8) in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell. If the SS receives ACK for MAC-CE sent by the UE, T3 starts in subframe n, and the test proceeds to step 10, otherwise go to step 12.

10.	The SS deactivates SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell and SCell 3 in subframe (n+20) for FDD PCell and (n+30) for TDD PCell by sending the deactivation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8).

11.	The UE shall stop sending CSI reports for SCell1 and the SS shall monitor CSI reports for SCell1 sent from the UE and ACK/NACK sent in PCell during SCell1 deactivation.

11a.	For all intra-band PCell FDD case

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+13), the subframes (n+15) to (n+23) and the subframes (n+25) to (n+33) up to the end of T3

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11b.	For the case where the band relation between PCC(FDD) and SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+13), the subframes (n+15) to (n+23) and the subframes (n+25) to (n+33) up to the end of T3,

-	and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+5) to (n+13), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+15) to (n+23) and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+25) to (n+33) up to the end of T3

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11c.	For the case where the band relation between PCC(FDD) and SCell1 is intra-band and the others SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS outside the subframes (n+5) to (n+13), the subframes (n+15) to (n+23) and the subframes (n+25) to (n+33) up to the end of T3,

-	and ≤2 non-consecutive DTX subframes are observed by the SS the subframes (n+15) to (n+23) and ≤2 non-consecutive DTX subframes are observed by the SS the subframes (n+25) to (n+33) up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11d.	For all intra-band PCell TDD case

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS outside the subframes (n+5) to (n+9) and (n+13), the subframes (n+20) to (n+24) and (n+28) and the subframes (n+35) to (n+39) and (n+43), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+6) to (n+10) and (n+13) to (n+14), the subframes (n+21) to (n+25) and (n+28) to (n+29) and the subframe (n+36) to (n+40) and (n+43), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+7) to (n+11) and (n+13) to (n+14) and (n+18), the subframes (n+22) to (n+26) and (n+28) to (n+29) and (n+33) and the subframes (n+37) to (n+41) and (n+43), up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11e.	For the case where the band relation between PCC(TDD) and SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS outside the subframes (n+5), (n+9), (n+20), (n+24), (n+35) and (n+39), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+6), (n+13), (n+21), (n+28), (n+36) and (n+43) up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+7), (n+13), (n+22), (n+28), (n+37) and (n+43), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+8) and (n+23), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+9) and (n+24), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+10), (n+14), (n+25), (n+29), (n+40) and (n+44), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+11), (n+18), (n+26), (n+33), (n+41) and (n+48) up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11f.	For the case where the band relation between PCC(TDD) and SCell1 is intra-band and the others SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS outside the subframes (n+5) to (n+9) and (n+13)

-	or DTX is not observed by the SS outside the subframes (n+6) to (n+10) and (n+13) to (n+14),

-	or DTX is not observed by the SS outside the subframes (n+7) to (n+11) and (n+13) to (n+14) and (n+18),

-	and DTX is not observed by the SS outside the subframes (n+20) and (n+24), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+21) and (n+28), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+22) and (n+28), up to the end of T3,

-	or DTX is not observed by the SS outside the subframe (n+23), up to the end of T3,

-	or DTX is not observed by the SS outside the subframe (n+24), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+25) and (n+29), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+26) and (n+33), up to the end of T3,

-	and DTX is not observed by the SS outside the subframes (n+35) and (n+39), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+36) and (n+43), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+37) and (n+43), up to the end of T3,

-	or DTX is not observed by the SS outside the subframe (n+38), up to the end of T3,

-	or DTX is not observed by the SS outside the subframe (n+39), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+40) and (n+44), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+41) and (n+48), up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

12.	When T3 expires, or Activation in step 6 was not acknowledged, or a fail was counted for the event “Activation” in step 8, or Deactivation in step 9 was not acknowledged, the SS shall transmit a RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

13.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

14.	After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [7] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3),
or
- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3.

15.	Repeat steps 2-14 until a test verdict has been achieved.

Each of the events “Activation” and “Deactivation” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict.
If all events pass, the test passes. If one event fails, the test fails.

8.16.89.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions:

Table 8.16.89.4.3-1: Common Exception messages for 4DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |

Table 8.16.89.4.3-2: PhysicalConfigDedicated-DEFAULT: Additional 4DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 4.8.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportConfig | CQI-ReportConfig-DEFAULT using condition CQI_PERIODIC |  | RBC |
| cqi-ReportConfig-r10 | Not present |  | SCell_AddMod |
| } |  |  |  |

Table 8.16.89.4.3-3: CQI-ReportConfig-DEFAULT: Additional 4DL CA Activation and Deactivation of Known SCell in non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { |  |  | CQI_PERIODIC |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 158 |  |  |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL |  |  |
| simultaneousAckNackAndCQI | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.89.4.3-4: PhysicalConfigDedicatedSCell-r10-DEFAULT: Additional 4DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [4] clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfoUL-r10 | Not present |  |  |
| pusch-ConfigDedicatedSCell-r10 | Not present |  |  |
| uplinkPowerControlDedicatedSCell-r10 | Not present |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  | SCell1 |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.89.4.3-5: CQI-ReportConfigSCell-r10-DEFAULT: Additional 4DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

| Derivation Path:TS 36.508 [7] clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 |  |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  | CQI_PERIODIC |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.16.89.4.3-6: CQI-ReportPeriodic-r10-DEFAULT: Additional 4DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportPeriodic-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 0 |  | Cell2 |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| ri-ConfigIndex | 484 |  | Cell2 |
| simultaneousAckNackAndCQI | TRUE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.89.4.3-7: QuantityConfig-DEFAULT: Additional 4DL CA Activation and Deactivation of Known SCell in non-DRX with generic duplex modes

| Derivation Path: TS 36.508 clause 4.6.6, Table 4.6.6-3A: QuantityConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.89.4.3-8: MeasObjectEUTRA-GENERIC: Additional 4DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 4.6.6 Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| cellsToAddModList SEQUENCE (SIZE (1..maxCellMeas)) { } | Not present |  |  |
| measCycleSCell-r10 | sf320 |  | Cell 2, Cell 3, Cell 4 |
| } |  |  |  |

Table 8.16.89.4.3-9: MeasObjectEUTRA-GENERIC: Additional 4DL CA Activation and Deactivation of Known SCell in non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 4.6.6, Table 4.6.6-2 MeasObjectEUTRA-GENERIC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| neighCellConfig | ‘00’B (Not all neighbour cells have the same MBSFN subframe allocation as serving cell) |  | Cell 1, Cell 2, Cell 3, Cell 4 |
| } |  |  |  |

Table 8.16.89.4.3-10: SystemInformationBlockType2: Additional 4DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 7.2.1.3, Table 7.2.1.3-1 SystemInformationBlockType2 exceptions |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| mbsfn-SubframeConfig | Not present |  | Cell 1 |
| } |  |  |  |

Table 8.16.89.4.3-11: SystemInformationBlockType3: Additional 4DL CA Activation and Deactivation of Known SCell in non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 7.2.1.3, Table 7.2.1.3-2 SystemInformationBlockType3 exceptions |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| neighCellConfig | ‘00’B (Not all neighbour cells have the same MBSFN subframe allocation as serving cell) |  | Cell 1, Cell 2, Cell 3, Cell 4 |
| } |  |  |  |

Table 8.16.89.4.3-12: SystemInformationBlockType5: Additional 4DL CA Activation and Deactivation of Known SCell in non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 7.2.1.3, Table 7.2.1.3-3 SystemInformationBlockType5 exceptions |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| neighCellConfig | ‘00’B (Not all neighbour cells have the same MBSFN subframe allocation as serving cell) |  | Cell 1, Cell 2, Cell 3, Cell 4 |
| } |  |  |  |

8.16.89.5	Test Requirements

Table 8.16.89.5-1 and 8.16.89.5-2 define the Cell specific test parameters for known SCell1 activation.

Table 8.16.89.5-1: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD Special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table 8.16.89.5-2: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  |  |  |  |  |  |  |
| TDD Special subframe configuration |  | 6 |  |  |  |  |  |  |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

During T2 the UE shall send the first CSI report for SCell1 in subframe (m+8), or in subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+34).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19) and (m+25) to (m+29) for FDD PCell, and not outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19) and (n+25) to (n+29) for FDD PCell, and not outside subframes (n+5) to (n+11), (n+20) to (n+26) and (n+35) to (n+41) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

8.16.90.1	Test Purpose

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in TS 36.133 [4] section 7.7 for UE configured with three downlink SCells, when the SCell is unknown by the UE at the time of activation.

8.16.90.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE that support E-UTRA FDD and/or E-UTRA TDD and 4DL CA. If the generic duplex-mode test case 8.16.90 is tested with FDD PCell, it will be only applicable to UEs supporting FDD as PCell. If the generic duplex-mode test case 8.16.90 is tested with TDD PCell, it will only be applicable to UEs supporting TDD as PCell. Applicability requires support for FGI bit 25.

Note on the applicability: If the generic duplex-mode test case 8.16.90 is tested with FDD PCell, the specific duplex-mode test cases 8.16.61 and 8.16.63 do not need to be tested. If the generic duplex-mode test case 8.16.90 is tested with TDD PCell, the specific duplex-mode test cases 8.16.62 and 8.16.64 do not need to be tested.

8.16.90.3	Minimum conformance requirements

While activating a SCell if the UE does not receive any command to activate, deactivate, configure or deconfigure any other SCell during the SCell activation delay then the UE shall meet the SCell activation delay requirements specified in TS 36.133 [4] section 7.7.2.

While activating a SCell if any other SCell is activated, deactivated, configured or deconfigured by the UE then the UE shall meet the SCell activation delay requirements (Tactivate_total) according to the following expression:

Where:

Tactivate_total is the total time to activate a SCell and is expressed in subframes.

Tactivate_basic is the SCell activation delay specified in TS 36.133 [4] section 7.7.2;

Ki (0 ≤ Ki ≤ [3]) is the number of times the other ith SCell is activated, deactivated, configured or deconfigured while the SCell is being activated.

N (2≤N≤4) is the maximum number of SCells supported by the UE.

While activating a SCell:

-	The interruption on the PCell and/or on the activated SCell due to the SCell activation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+11 if:

-	the PCell and/or the activated SCell being interrupted and the SCell being activated belong to E-UTRA TDD, or

-	the activated SCell being interrupted and the SCell being activated belong to E-UTRA FDD and the PCell belongs to E-UTRA TDD.

-	Otherwise, the interruption on PCell and/or on the activated SCell due to the SCell activation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+9.

Starting from the subframe specified in section 4.3 of TS 36.213 [8] and until the UE has completed the SCell activation, the UE shall report CQI index = 0 (out of range) if the UE has available uplink resources to report CQI for an SCell.

The UE shall deactivate a SCell and meet the SCell deactivation delay requirements specified in TS 36.133 [4] section 7.7.3 regardless of whether the other SCell is activated, deactivated, configured or deconfigured or not by the UE during the SCell deactivation delay

While deactivating a SCell:

-	The interruption on the PCell and/or on the activated SCell due to the SCell deactivation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+11 if:

-	the PCell and/or the activated SCell being interrupted and the SCell being deactivated belong to E-UTRA TDD or

-	the activated SCell being interrupted and the SCell being deactivated belong to E-UTRA FDD and the PCell belongs to E-UTRA TDD.

-	Otherwise, the interruption on PCell and/or the activated SCell due to the SCell deactivation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+9.

The normative reference for this requirement is TS 36.133 [4] clause 7.7 and A.8.16.90.

8.16.90.4	Test description

8.16.90.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-2 and TS 36.508 [7] clauses 4.4.2 and 4.3.1 for different CA bandwidth classes.

Channel Bandwidth to be tested: The largest aggregated bandwidth combination supported by UE from Table 8.16.90.5-1 as defined in TS 36.508 [7] clause 4.3.1, for different CA configurations as defined in TS 36.521-1 [10] clause 5.4.2A.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as follows:

- For UEs supporting only 2Rx in all the bands under test, use TS 36.508 [7] Annex A, Figure group A.63 as appropriate.

- For UEs supporting 4Rx in any of the bands under test use TS 36.508 [7] Annex A, Figure A.90, without intra-frequency neighbours. For PCell, use the 4Rx module if the UE supports 4Rx for the corresponding band and the 2Rx module otherwise. For all other cells, the 2Rx module can be used.

2.	The general test parameter settings are set up according to Table 8.16.90.4.1-1.

3.	Propagation conditions are set according to Annex B clauses B.0.

4.	Message contents are defined in clause 8.16.90.4.3.

5.	Cell 1 is the PCell on the primary component carrier, Cell 2 is SCell on RF channel number 2, Cell 3 is the deactivated SCell on RF channel number 3 and Cell 4 is the deactivated SCell on RF channel number 4. Cell 1 is the cell used for connection setup with the power levels set according to Annex C.0 and C.1 for this test. Cell 2, Cell 3 and Cell 4 shall be powered OFF.

Table 8.16.90.4.1-1: General test parameters for 4DL CA activation and deactivation of unknown SCell in non-DRX with generic duplex modes

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, and SCell3. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, and SCell3. |

8.16.90.4.2	Test procedure

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. All cells have constant signal levels throughout the test.

Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2) and Cell 4 (deactivated SCell3) on radio channel 4 (SCC3) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2 and SCC3. The UE shall be continuously scheduled in the PCell throughout the whole test.

There are four carriers, each with one cell. Cell 1, Cell 2, Cell 3 and Cell 4 may operate in either FDD or TDD duplex mode according to test configuration.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3.

2.	Configure SCC2 and SCC3 according to Annex C.0, C.1 for all downlink physical channels.

3.	The SS shall configure SCell2 (Cell 3) on the SCC2 and SCell3 (Cell 4) on the SCC3 as per TS 36.508 [7] clause 5.2A.4 with the message content exceptions defined in clause 8.16.90.4.3.

4.	The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

5.	Set the parameters according to T1 in Table 8.16.90.5-1. Propagation conditions are set according to Annex B clauses B.1.1. T1 starts. The SS shall configure SCell1 (Cell 2) on the SCC1 as per TS 36.508 [7] clause 5.2A.4 with the message content exceptions defined in clause 8.16.90.4.3.

6.	The SS activates SCell1 by sending the activation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8) in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell. If the SS receives ACK for MAC-CE sent by the UE, T2 starts in subframe m, and the test proceeds to step 7, otherwise go to step 12.

7.	The SS activates SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell and SCell 3 in subframe (m+20) for FDD PCell and (m+30) for TDD PCell by sending the activation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8).

8.	The UE shall start sending CSI reports for SCell1 and the SS shall monitor CSI reports for SCell1 sent from the UE and ACK/NACK sent in PCell during SCells.

8a.	For all intra-band PCell FDD case

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+44),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+44)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+13), the subframes (m+15) to (m+23) and the subframes (m+25) to (m+33) up to the end of T2

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

8b.	For the case where the band relation between PCC(FDD) and SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+44),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+44)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+13), the subframes (m+15) to (m+23) and the subframes (m+25) to (m+33) up to the end of T2,

-	and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+5) to (m+13), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+15) to (m+23) and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+25) to (m+33) up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

8c.	For the case where the band relation between PCC(FDD) and SCell1 is intra-band and the others SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+44),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+44)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+13), the subframes (m+15) to (m+23) and the subframes (m+25) to (m+33) up to the end of T2,

-	and ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (m+15) to (m+23) and ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (m+25) to (m+33) up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

8d.	For all intra-band PCell TDD case

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+9) if the subframe (m+8) was subject to interruption,

-	or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+44),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+44).

-	and DTX is not observed by the SS outside the subframes (m+5) to (m+9) and (m+13) and the subframes (m+20) to (m+24) and (m+28) and the subframes (m+35) to (m+39) and (m+43) up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+6) to (m+10) and (m+13) to (m+14) and the subframes (m+21) to (m+25) and (m+28) to (m+29) and the subframe (m+36) to (m+40) and (m+43), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+7) to (m+11) and (m+13) to (m+14) and (m+18), the subframes (m+22) to (m+26) and (m+28) to (m+29) and (m+33) and the subframe (m+37) to (m+41) and (m+43), up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

8e.	For the case where the band relation between PCC(TDD) and SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+9) if the subframe (m+8) was subject to interruption,

-	or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to (m+44),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+44),

-	and DTX is not observed by the SS outside the subframes (m+5), (m+9), (m+20), (m+24), (m+35) and (m+39), up to the end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+6), (m+13), (m+21), (m+28), (m+36) and (m+43), up to the end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+7), (m+13), (m+22), (m+28), (m+37) and (m+43), up to the end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+8) and (m+23) up to the end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+9) and (m+24), up to the end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+10), (m+14), (m+25), (m+29) and (m+40) up to the end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+11), (m+18), (m+26), (m+33), (m+41) and (m+43), up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

8f.	For the case where the band relation between PCC(TDD) and SCell1 is intra-band and the others SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframes (m+8),

-	 or (m+9) if the subframe (m+8) was subject to interruption,

-	or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+44),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+44),

-	and DTX is not observed by the SS outside the subframes (m+5) to (m+9) and (m+13),

-	or DTX is not observed by the SS outside the subframes (m+6) to (m+10) and (m+13) to (m+14),

-	or DTX is not observed by the SS outside the subframes (m+7) to (m+11) and (m+13) to (m+14) and (m+18),

-	and DTX is not observed by the SS outside the subframes (m+20) and (m+24), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+21) and (m+28), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+22) and (m+28), up to the end of T2,

-	or DTX is not observed by the SS outside the subframe (m+23), up to the end of T2,

-	or DTX is not observed by the SS outside the subframe (m+24), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+25) and (m+29), up to the end of T2,

-	or DTX is not observed by the SS outside the subframes (m+26) and (m+33), up to the end of T2,

-	and DTX is not observed by the SS anywhere outside the subframes (m+35) and (m+39), up to end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+36) and (m+43), up to end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+37) and (m+43), up to end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+38), up to end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+39), up to end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+40), up to end of T2,

-	or DTX is not observed by the SS anywhere outside the subframes (m+41), up to end of T2,

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 12.

9.	When T2 expires, the SS deactivate SCell1 by sending the deactivation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8) in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell. If the SS receives ACK for MAC-CE sent by the UE, T3 starts in subframe n, and the test proceeds to step 10, otherwise go to step 12.

10.	The SS deactivates SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell and SCell 3 in subframe (n+20) for FDD PCell and (n+30) for TDD PCell by sending the deactivation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8).

11.	The UE shall stop sending CSI reports for SCell1 and the SS shall monitor CSI reports for SCell1 sent from the UE and ACK/NACK sent in PCell during SCell1 deactivation.

11a.	For all intra-band PCell FDD case

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+13), the subframes (n+15) to (n+23) and the subframes (n+25) to (n+33) up to the end of T3

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11b.	For the case where the band relation between PCC(FDD) and SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+13), the subframes (n+15) to (n+23) and the subframes (n+25) to (n+33) up to the end of T3,

-	and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+5) to (n+13), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+15) to (n+23) and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+25) to (n+33) up to the end of T3

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11c.	For the case where the band relation between PCC(FDD) and SCell1 is intra-band and the others SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS outside the subframes (n+5) to (n+13), the subframes (n+15) to (n+23) and the subframes (n+25) to (n+33) up to the end of T3,

-	and ≤2 non-consecutive DTX subframes are observed by the SS the subframes (n+15) to (n+23) and ≤2 non-consecutive DTX subframes are observed by the SS the subframes (n+25) to (n+33) up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11d.	For all intra-band PCell TDD case

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS outside the subframes (n+5) to (n+9) and (n+13), the subframes (n+20) to (n+24) and (n+28) and the subframes (n+35) to (n+39) and (n+43), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+6) to (n+10) and (n+13) to (n+14), the subframes (n+21) to (n+25) and (n+28) to (n+29) and the subframe (n+36) to (n+40) and (n+43), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+7) to (n+11) and (n+13) to (n+14) and (n+18), the subframes (n+22) to (n+26) and (n+28) to (n+29) and (n+33) and the subframes (n+37) to (n+41) and (n+43), up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11e.	For the case where the band relation between PCC(TDD) and SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS outside the subframes (n+5), (n+9), (n+20), (n+24), (n+35) and (n+39), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+6), (n+13), (n+21), (n+28), (n+36) and (n+43) up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+7), (n+13), (n+22), (n+28), (n+37) and (n+43), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+8) and (n+23), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+9) and (n+24), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+10), (n+14), (n+25), (n+29), (n+40) and (n+44), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+11), (n+18), (n+26), (n+33), (n+41) and (n+48) up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11f.	For the case where the band relation between PCC(TDD) and SCell1 is intra-band and the others SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS outside the subframes (n+5) to (n+9) and (n+13)

-	or DTX is not observed by the SS outside the subframes (n+6) to (n+10) and (n+13) to (n+14),

-	or DTX is not observed by the SS outside the subframes (n+7) to (n+11) and (n+13) to (n+14) and (n+18),

-	and DTX is not observed by the SS outside the subframes (n+20) and (n+24), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+21) and (n+28), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+22) and (n+28), up to the end of T3,

-	or DTX is not observed by the SS outside the subframe (n+23), up to the end of T3,

-	or DTX is not observed by the SS outside the subframe (n+24), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+25) and (n+29), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+26) and (n+33), up to the end of T3,

-	and DTX is not observed by the SS outside the subframes (n+35) and (n+39), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+36) and (n+43), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+37) and (n+43), up to the end of T3,

-	or DTX is not observed by the SS outside the subframe (n+38), up to the end of T3,

-	or DTX is not observed by the SS outside the subframe (n+39), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+40) and (n+44), up to the end of T3,

-	or DTX is not observed by the SS outside the subframes (n+41) and (n+48), up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

12.	When T3 expires, or Activation in step 6 was not acknowledged, or a fail was counted for the event “Activation” in step 8, or Deactivation in step 9 was not acknowledged, the SS shall transmit a RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

13.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

14.	After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [7] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3),
or
- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3.

15.	Repeat steps 2-14 until a test verdict has been achieved.

Each of the events “Activation” and “Deactivation” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict.
If all events pass, the test passes. If one event fails, the test fails.

8.16.90.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions:

Table 8.16.90.4.3-1: Common Exception messages for 4DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |

Table 8.16.90.4.3-2: PhysicalConfigDedicated-DEFAULT: Additional 4DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 4.8.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportConfig | CQI-ReportConfig-DEFAULT using condition CQI_PERIODIC |  | RBC |
| cqi-ReportConfig-r10 | Not present |  | SCell_AddMod |
| } |  |  |  |

Table 8.16.90.4.3-3: CQI-ReportConfig-DEFAULT: Additional 4DL CA Activation and Deactivation of Unknown SCell in non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { |  |  | CQI_PERIODIC |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 158 |  |  |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL |  |  |
| simultaneousAckNackAndCQI | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.90.4.3-4: PhysicalConfigDedicatedSCell-r10-DEFAULT: Additional 4DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [4] clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfoUL-r10 | Not present |  |  |
| pusch-ConfigDedicatedSCell-r10 | Not present |  |  |
| uplinkPowerControlDedicatedSCell-r10 | Not present |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  | SCell1 |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.90.4.3-5: CQI-ReportConfigSCell-r10-DEFAULT: Additional 4DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

| Derivation Path:TS 36.508 [7] clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 |  |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  | CQI_PERIODIC |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.16.90.4.3-6: CQI-ReportPeriodic-r10-DEFAULT: Additional 4DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportPeriodic-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 0 |  | Cell2 |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| ri-ConfigIndex | 484 |  | Cell2 |
| simultaneousAckNackAndCQI | TRUE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.90.4.3-7: QuantityConfig-DEFAULT: Additional 4DL CA Activation and Deactivation of Unknown SCell in non-DRX with generic duplex modes

| Derivation Path: TS 36.508 clause 4.6.6, Table 4.6.6-3A: QuantityConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.90.4.3-8: MeasObjectEUTRA-GENERIC: Additional 4DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 4.6.6 Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| cellsToAddModList SEQUENCE (SIZE (1..maxCellMeas)) { } | Not present |  |  |
| measCycleSCell-r10 | sf320 |  | Cell 2, Cell 3, Cell 4 |
| } |  |  |  |

Table 8.16.90.4.3-9: MeasObjectEUTRA-GENERIC: Additional 4DL CA Activation and Deactivation of Unknown SCell in non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 4.6.6, Table 4.6.6-2 MeasObjectEUTRA-GENERIC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| neighCellConfig | ‘00’B (Not all neighbour cells have the same MBSFN subframe allocation as serving cell) |  | Cell 1, Cell 2, Cell 3, Cell 4 |
| } |  |  |  |

Table 8.16.90.4.3-10: SystemInformationBlockType2: Additional 4DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 7.2.1.3, Table 7.2.1.3-1 SystemInformationBlockType2 exceptions |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| mbsfn-SubframeConfig | Not present |  | Cell 1 |
| } |  |  |  |

Table 8.16.90.4.3-11: SystemInformationBlockType3: Additional 4DL CA Activation and Deactivation of Unknown SCell in non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 7.2.1.3, Table 7.2.1.3-2 SystemInformationBlockType3 exceptions |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| neighCellConfig | ‘00’B (Not all neighbour cells have the same MBSFN subframe allocation as serving cell) |  | Cell 1, Cell 2, Cell 3, Cell 4 |
| } |  |  |  |

Table 8.16.90.4.3-12: SystemInformationBlockType5: Additional 4DL CA Activation and Deactivation of Unknown SCell in non-DRX with generic duplex modes

| Derivation Path: TS 36.508 [7] clause 7.2.1.3, Table 7.2.1.3-3 SystemInformationBlockType5 exceptions |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| neighCellConfig | ‘00’B (Not all neighbour cells have the same MBSFN subframe allocation as serving cell) |  | Cell 1, Cell 2, Cell 3, Cell 4 |
| } |  |  |  |

8.16.90.5	Test Requirements

Table 8.16.90.5-1 and 8.16.90.5-2 define the Cell specific test parameters for unknown SCell1 activation.

Table 8.16.90.5-1: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD Special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table 8.16.90.5-2: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  |  |  |  |  |  |  |
| TDD Special subframe configuration |  | 6 |  |  |  |  |  |  |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+44).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19) and (m+25) to (m+29) for FDD PCell, and not outside subframes (m+5) to (m+11), (m+20) to (m+26) and (m+35) to (m+41) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19) and (n+25) to (n+29) for FDD PCell, and not outside subframes (n+5) to (n+11), (n+20) to (n+26) and (n+35) to (n+41) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+44) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

<< Unchanged sections omitted >>

Annex E (normative):
Cell configuration mapping

The cells used in TS 36.521-3 do not correspond to the cells defined in TS 36.508 [7] section 4.4.2. Tables E-1, E-2 and E-3 describes the mapping between cells described in TS 36.521-3 and those defined in TS 36.508 [7]. Table E-4 describe the mapping between Ncells described in TS 36.521-3 and those defined in TS 36.508 [7] For each test case the cells as defined in TS 36.508 [7] section 4.4.2 and section 8.1.4.2 are listed in one row. The test case shall apply the RF parameters as defined in TS 36.521-3 according to the column heading.

NOTE:	For example if the second cell in a test case is an inter-frequency cell then Cell3 from TS 36.508 [7] section 4.4.2 is used with the radio parameters as defined for Cell2 in TS 36.521-3.

Table E-5: Cell configuration mapping for 4DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  | 36.521-3 Cell3 |  | 36.521-3 Cell4 |  | 36.521-3 Cell5 |  | 36.521-3 Cell6 |  | 36.521-3 Cell7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.16.51 8.16.52 8.16.53 8.16.54 (Pcell in TDD) 8.16.55 8.16.56 | Default Test Settings for a CA_XA-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XD-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | Z | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XE |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 |  |  |  |  |
| 9.1.44 9.1.45 (Pcell in TDD) 9.1.46Note 3 9.1.47Note 3 9.1.69(generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | R | 10 | R | 11 |
|  | Default Test Settings for a CA_XA-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 |
|  | Default Test Settings for a CA_XD-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | Y | 10 | Y | 11 |
|  | Default Test Settings for a CA_XC-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 |
|  | Default Test Settings for a CA_XA-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 10 | Z | 11 |
|  | Default Test Settings for a CA_XA-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 |
|  | Default Test Settings for a CA_XC-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 |
|  | Default Test Settings for a CA_XE |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | X | 10 | X | 11 |
| 8.16.57 Note 5 8.16.58 Note 5 8.16.59 Note 5 8.16.60 (Pcell in TDD) Note 5 8.16.61 Note 5 8.16.62 Note 5 8.16.63 Note 5 8.16.64 (Pcell in TDD) Note 5 8.16.89(generic duplex modes) Note 5 8.16.90(generic duplex modes) Note 5 9.2.45 | Default Test Settings for a CA_XA-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Z | 3 | Z | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 |  |  |  |  |  |  |
| Note 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. Note 2: X, Y, Z and R correspond to the bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_2A-2A-4A-5A, X=2, Y=2, Z=4, R=5. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_2A-2A-42A-66A, X=2, Y=2, Z=66, R=42. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_2A-2A-42A-66A, X=42, Y=2, Z=2, R=66. Note 3: These test cases require to swap the bands so that every band in the CA Configuration is used as PCC. In the most generic case XA-YA-ZA-RA, the test case shall be run four times: CA_XA-YA-ZA-RA, CA_YA-ZA-RA-XA, CA_ZA-RA- XA-YA and CA_RA- XA-YA-ZA. Note 4: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB-YA-ZA, the entry for CA_XC-YA-ZA shall be used. Note 5: For intra band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

F.1.2	Measurement of RRM requirements

Table F.1.2-1: Maximum Test System Uncertainty for RRM Requirements

| Subclause |  | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4.2.1 E-UTRA FDD - FDD cell re-selection intra frequency |  | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |  |
| 4.2.2 E-UTRA TDD - TDD cell re-selection intra frequency |  | Same as 4.2.1 |  |  |  |  |
| 4.2.3 E-UTRA FDD - FDD cell re-selection inter frequency |  | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |  |
| 4.2.4 E-UTRA FDD - TDD cell re-selection inter frequency |  | Same as 4.2.3 |  |  |  |  |
| 4.2.5 E-UTRA TDD - FDD cell re-selection inter frequency |  | Same as 4.2.3 |  |  |  |  |
| 4.2.6 E-UTRA TDD - TDD cell re-selection inter frequency |  | Same as 4.2.3 |  |  |  |  |
| 4.2.7 E-UTRA FDD Inter frequency re-selection in the existence of non-allowed CSG cell |  | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês3 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês3 / Noc1 is the ratio of cell 3 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |  |
| <<Many rows skipped>> |  |  |  |  |  |  |
| 8.16.57 4DL FDD CA Activation and Deactivation of Known SCell in Non-DRX |  | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |  |  |  |
| 8.16.58 4DL TDD CA Activation and Deactivation of Known SCell in Non-DRX |  | Same as 8.16.57 |  | Same as 8.16.57 |  |  |  |
| 8.16.59 4DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX |  | Same as 8.16.57 |  | Same as 8.16.57 |  |  |  |
| 8.16.60 4DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX |  | Same as 8.16.57 |  | Same as 8.16.57 |  |  |  |
| 8.16.61 4DL FDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  | Same as 8.16.57 |  | Same as 8.16.57 |  |  |  |
| 8.16.62 4DL TDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  | Same as 8.16.57 |  | Same as 8.16.57 |  |  |  |
| 8.16.63 4DL PCell in FDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  | Same as 8.16.57 |  | Same as 8.16.57 |  |  |  |
| 8.16.64 4DL PCell in TDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  | Same as 8.16.57 |  | Same as 8.16.57 |  |  |  |
| 8.16.65 5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX |  | Same as 8.16.71 |  | Same as 8.16.71 |  |  |  |
| 8.16.66 5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX |  | Same as 8.16.71 |  | Same as 8.16.71 |  |  |  |
| 8.16.67 5DL Pcell in FDD CA activation and deactivation of unknown SCell in non-DRX |  | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |  |  |  |
| 8.16.68 5DL Pcell in TDD CA activation and deactivation of unknown SCell in non-DRX |  | Same as 8.16.67 |  | Same as 8.16.67 |  |  |  |
| 8.16.69 5DL FDD CA activation and deactivation of unknown SCell in non-DRX |  | Same as 8.16.67 |  | Same as 8.16.67 |  |  |  |
| 8.16.70 5DL TDD CA activation and deactivation of unknown SCell in non-DRX |  | Same as 8.16.67 |  | Same as 8.16.67 |  |  |  |
| 8.16.71 5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX |  | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.6 dB averaged over BWConfig Ês6 / Noc5 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Ês6 / Noc5 is the ratio of cell 6 signal / AWGN Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5 uncertainty and Ês6 / Noc5 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |  |  |  |
| 8.16.72 5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX |  | Same as 8.16.71 |  | Same as 8.16.71 |  |  |  |
| 8.16.73 5DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  | Same as 8.16.71 |  | Same as 8.16.71 |  |  |  |
| 8.16.74 5DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  | Same as 8.16.71 |  | Same as 8.16.71 |  |  |  |
| 8.16.75 5DL PCell in FDD CA activation and deactivation of known SCell in non-DRX |  | Same as 8.16.67 |  | Same as 8.16.67 |  |  |  |
| 8.16.76 5DL PCell in TDD CA activation and deactivation of known SCell in non-DRX |  | Same as 8.16.67 |  | Same as 8.16.67 |  |  |  |
| 8.16.77 5DL FDD CA activation and deactivation of known SCell in non-DRX |  | Same as 8.16.67 |  | Same as 8.16.67 |  |  |  |
| 8.16.78 E-UTRAN TDD 5DL CA activation and deactivation of known SCell in non-DRX |  | Same as 8.16.67 |  | Same as 8.16.67 |  |  |  |
| 8.16.83 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  | Same as 8.16.27 |  | Same as 8.16.27 |  |  |  |
| 8.16.84 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  | Same as 8.16.31 |  | Same as 8.16.31 |  |  |  |
| 8.16.85 3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  | Same as 8.16.35 |  | Same as 8.16.35 |  |  |  |
| 8.16.86 3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  | Same as 8.16.39 |  | Same as 8.16.39 |  |  |  |
| 8.16.89 4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  | Same as 8.16.57 |  | Same as 8.16.57 |  |  |  |
| 8.16.90 4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  | Same as 8.16.61 |  | Same as 8.16.61 |  |  |  |
| 8.20.1 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells |  | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 3 frequency Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês1 / Noc1 uncertainty, Ês2 / Noc2 uncertainty, Ês3 / Noc3 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |  |  |
| 8.20.2 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells |  | Same as 8.20.1 |  | Same as 8.20.1 |  |  |
| 8.20.2A UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth |  | Same as 8.20.1 |  | Same as 8.20.1 |  |  |
| 8.20.2B UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth |  |  | Same as 8.20.1 |  | Same as 8.20.1 |  |  |
| <<Many rows skipped>> |  |  |  |  |  |  |  |
| 11.1 V2V UE Transmission Timing Accuracy Test |  |  | ±3Ts Uplink signal transmit timing relative to GNSS synchronization source Absolute GNSS signal level, ±1 dB |  |  | Note: TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] Same value as the A-GNSS Time Assistance tests in section 5 of TS 37.571-1 |
| 11.2 Interruptions due to V2V sidelink communication |  |  | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB Absolute GNSS signal level, ±1 dB |  |  | Note: Ês / Noc is the ratio of cell 1 signal / AWGN Same value as the A-GNSS Time Assistance tests in section 5 of TS 37.571-1 |
| 12.1.1 V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference |  |  | synchronization source Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to GNSS |  |  | Note: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 12.1.2 V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference |  |  | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink |  |  | Note: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 12.2.1 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference |  |  | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig |  |  | Note: Ês / Noc is the ratio of cell 1 signal / AWGN |
| 12.2.2 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference |  |  | Noc ±1.5 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig |  |  | Note: Ês / Noc is the ratio of SyncRef UE signal / AWGN |
| 12.4 Congestion Control Measurement Test for V2X UE |  |  | Noc ±1.5 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig |  |  | Note: Ês / Noc is the ratio of Active UE signal / AWGN |
| In addition, the following Test System uncertainties and related constraints apply. Any additional constraints are defined in the specific tests. |  |  |  |  |  |  |
| AWGN Bandwidth |  |  |  |  |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |
| AWGN absolute power uncertainty |  |  |  |  |  | Test-specific |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  |  |  |  |  | ±2 dB |
| AWGN peak to average ratio |  |  |  |  |  | ≥10 dB @0.001% |
| Signal-to noise ratio uncertainty |  |  |  |  |  | Test-specific |
| Fading profile power uncertainty |  |  |  |  |  | ±0.5 dB |
| Fading profile delay uncertainty, relative to frame timing |  |  |  |  |  | ±5 ns (excludes absolute errors related to baseband timing) |

<< Unchanged sections omitted >>

F.3.2	Measurement of RRM requirements

Because the relationships between the Test system uncertainties and the Test Tolerances are often complex, it is not always possible to give a simple derivation of the Test Requirement in this document. The analysis is recorded in 3GPP TR 36 903 [20].

Table F.3.2-1: Derivation of Test Requirements (RRM tests)

| Test |  |  | Minimum Requirement in TS 36.133 |  |  |  | Test Tolerance (TT) |  |  | Test Requirement in TS 36.521-3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2.1 E-UTRA FDD – FDD cell re-selection intra frequency |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: +13.00dB |  |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13.00dB |  |
| 4.2.2 E-UTRA TDD – TDD cell re-selection intra frequency |  |  | Same as 4.2.1 |  |  |  | Same as 4.2.1 |  |  | Same as 4.2.1 |  |
| 4.2.3 E-UTRA FDD - FDD cell re-selection inter frequency |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: -4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +14.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4.00dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12.00dB |  |  |  | During T0: -1.35dB +0.3dB -1.35dB +0.4dB During T1: -1.35dB +2.4dB -1.35dB +0.3dB During T2: -1.35dB +2.4dB -1.35dB 0dB During T3: -1.35dB +2.4dB -1.35dB +2.4dB |  |  | During T0: Noc1: -99.35dBm/15kHz Ês1 / Noc1: -3.70dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: +14.40dB During T1: Noc1: -99.35dBm/15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: -3.70dB During T2: Noc1: -99.35dBm/15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -99.35dBm /15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: +14.40dB |  |
| 4.2.4 E-UTRA FDD - TDD cell re-selection inter frequency |  |  | Same as 4.2.3 |  |  |  | Same as 4.2.3 |  |  | Same as 4.2.3 |  |
| 4.2.5 E-UTRA TDD - FDD cell re-selection inter frequency |  |  | Same as 4.2.3 |  |  |  | Same as 4.2.3 |  |  | Same as 4.2.3 |  |
| 4.2.6 E-UTRA TDD - TDD cell re-selection inter frequency |  |  | Same as 4.2.3 |  |  |  | Same as 4.2.3 |  |  | Same as 4.2.3 |  |
| 4.2.7 E-UTRA FDD Inter frequency re-selection in the existence of non-allowed CSG cell |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: -3.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Ês3 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Ês3 / Noc1: +13.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +13.00dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: +38.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB |  |  |  | During T0: 0dB 0dB 0dB 0dB 0dB During T1: 0dB -0.2dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: -infinitydB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +7.80dB Ês3 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Ês3 / Noc1: +13.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +13.00dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: +38.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB |  |
| <<Many rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |
| 8.16.57 4DL FDD CA Activation and Deactivation of Known SCell in Non-DRX |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |  |  | During T1,T2,T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |  |
| 8.16.58 4DL TDD CA Activation and Deactivation of Known SCell in Non-DRX |  | Same as 8.16.57 |  |  |  | Same as 8.16.57 |  |  | Same as 8.16.57 |  |  |
| 8.16.59 4DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX |  | Same as 8.16.57 |  |  |  | Same as 8.16.57 |  |  | Same as 8.16.57 |  |  |
| 8.16.60 4DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX |  | Same as 8.16.57 |  |  |  | Same as 8.16.57 |  |  | Same as 8.16.57 |  |  |
| 8.16.61 4DL FDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2, T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |  |
| 8.16.62 4DL TDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  | Same as 8.16.61 |  |  |  | Same as 8.16.61 |  |  | Same as 8.16.61 |  |  |
| 8.16.63 4DL PCell in FDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  | Same as 8.16.61 |  |  |  | Same as 8.16.61 |  |  | Same as 8.16.61 |  |  |
| 8.16.64 4DL PCell in TDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  | Same as 8.16.61 |  |  |  | Same as 8.16.61 |  |  | Same as 8.16.61 |  |  |
| 8.16.65 5 DL PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB E-UTRA Cell 6 Ês6 / Noc5: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: -3dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: -3dB E-UTRA Cell 6 Ês6 / Noc5: -3dB |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB +0.2dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T2: 0dB 0dB 0dB +0.2dB 0dB +0.2dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T3: 0dB 0dB 0dB -0.2dB 0dB -0.2dB 0dB -0.2dB -0.5dB -0.7dB -0.7dB |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17.2dB E-UTRA Cell 4 Noc4: -104.5dBm/15kHz Ês4 / Noc4: 17.7dB E-UTRA Cell 5 Noc5: -104.5dBm/15kHz Ês5 / Noc5: 17.7dB E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17.2dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17.2dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17.2dB E-UTRA Cell 5 Noc5: -104.5dBm/15kHz Ês5 / Noc5: 17.7dB E-UTRA Cell 6 Ês6 / Noc5: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.2dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3.2dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: -3.2dB E-UTRA Cell 5 Noc5: -104.5dBm/15kHz Ês5 / Noc5: -3.7dB E-UTRA Cell 6 Ês6 / Noc5: -3.7dB |  |  |
| 8.16.66 5 DL PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX |  | Same as 8.16.65 |  |  |  | Same as 8.16.65 |  |  | Same as 8.16.65 |  |  |
| 8.16.67 5DL Pcell in FDD CA activation and deactivation of unknown SCell in non-DRX |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2, T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |  |
| 8.16.68 5DL Pcell in TDD CA activation and deactivation of unknown SCell in non-DRX |  | Same as 8.16.67 |  |  |  | Same as 8.16.67 |  |  | Same as 8.16.67 |  |  |
| 8.16.69 5DL FDD CA activation and deactivation of unknown SCell in non-DRX |  | Same as 8.16.67 |  |  |  | Same as 8.16.67 |  |  | Same as 8.16.67 |  |  |
| 8.16.70 5DL TDD CA activation and deactivation of unknown SCell in non-DRX |  | Same as 8.16.67 |  |  |  | Same as 8.16.67 |  |  | Same as 8.16.67 |  |  |
| 8.16.71 5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX |  | Same as 8.16.65 |  |  |  | Same as 8.16.65 |  |  | Same as 8.16.65 |  |  |
| 8.16.72 5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX |  | Same as 8.16.65 |  |  |  | Same as 8.16.65 |  |  | Same as 8.16.65 |  |  |
| 8.16.73 5DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  | During T1, T2, T3 and T4: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Noc5: -101dBm/15kHz Ês5 / Noc5: 16dB During T1 and T3: E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2 and T4: E-UTRA Cell 6 Ês6 / Noc5: 16dB |  |  |  | During T1, T2, T3 and T4: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T1 and T3: 0dB During T2 and T2: 0dB |  |  | During T1, T2, T3 and T4: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Noc5: -101dBm/15kHz Ês5 / Noc5: 16dB During T1 and T3: E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2 and T4: E-UTRA Cell 6 Ês6 / Noc5: 16dB |  |  |
| 8.16.74 5DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  | Same as 8.16.73 |  |  |  | Same as 8.16.73 |  |  | Same as 8.16.73 |  |  |
| 8.16.75 5DL PCell in FDD CA activation and deactivation of known SCell in non-DRX |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |  | During T1,T2,T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |  |
| 8.16.76 5DL PCell in TDD CA activation and deactivation of known SCell in non-DRX |  | Same as 8.16.75 |  |  | Same as 8.16.75 |  |  |  | Same as 8.16.75 |  |  |
| 8.16.77 5DL FDD CA activation and deactivation of known SCell in non-DRX |  | Same as 8.16.75 |  |  | Same as 8.16.75 |  |  |  | Same as 8.16.75 |  |  |
| 8.16.83 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  | Same as 8.16.27 |  |  | Same as 8.16.27 |  |  |  | Same as 8.16.27 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.27, and no separate analysis is needed. |  |  |
| 8.16.84 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  | Same as 8.16.31 |  |  | Same as 8.16.31 |  |  |  | Same as 8.16.31 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.31, and no separate analysis is needed. |  |  |
| 8.16.85 3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  | Same as 8.16.35 |  |  | Same as 8.16.35 |  |  |  | Same as 8.16.35 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.35, and no separate analysis is needed. |  |  |
| 8.16.86 3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  | Same as 8.16.39 |  |  | Same as 8.16.39 |  |  |  | Same as 8.16.39 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |  |
| 8.16.89 4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  | Same as 8.16.57 |  |  | Same as 8.16.57 |  |  |  | Same as 8.16.57 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.35, and no separate analysis is needed. |  |  |
| 8.16.90 4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  | Same as 8.16.61 |  |  | Same as 8.16.61 |  |  |  | Same as 8.16.61 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |  |
| 8.20.1 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: 7dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.7dB 0dB 0dB |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: 7.7dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB |  |  |
| 8.20.2 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells |  | Same as 8.20.1 |  |  | Same as 8.20.1 |  |  |  | Same as 8.20.1 |  |  |
| 8.20.2A UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth |  | Same as 8.20.1 |  |  | Same as 8.20.1 |  |  |  | Same as 8.20.1 |  |  |
| 8.20.2B UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth |  | Same as 8.20.1 |  |  | Same as 8.20.1 |  |  |  | Same as 8.20.1 |  |  |
| 8.20.3 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  |
| 8.20.4 E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: 5.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: 5.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  |
| 8.20.4A E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 20 MHz bandwidth |  | Same as 8.20.4 |  |  | Same as 8.20.4 |  |  |  | Same as 8.20.4 |  |  |
| 8.20.4B E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 10 MHz bandwidth |  | Same as 8.20.4 |  |  | Same as 8.20.4 |  |  |  | Same as 8.20.4 |  |  |
| <<Many rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |
| 12.1.1 V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference | Uplink timing: ±12Ts GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |  |  | ±3Ts 0dB 0dB 0dB 0dB |  |  |  | Uplink timing: ±15Ts GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |  |  |
| 12.1.2 V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference | Uplink timing: ±12Ts |  |  | ±3Ts |  |  |  | Uplink timing: ±15Ts |  |  |
| 12.2.1 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference | During T1: Noc: -110dBm/15kHz Ês / Noc: +4.5dB During T2: Noc: -110dBm/15kHz Ês / Noc: -4.5dB During T3: Noc: -110dBm/15kHz Ês / Noc: +4.5dB |  |  | During T1: 0dB 1.1dB During T2: 0dB -1.1dB During T3: 0dB 1.1dB |  |  |  | During T1: Noc: -110dBm/15kHz Ês / Noc: +5.6dB During T2: Noc: -110dBm/15kHz Ês / Noc: -5.6dB During T3: Noc: -110dBm/15kHz Ês / Noc: +5.6dB |  |  |
| 12.2.2 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference | During T1: Noc: -101dBm/15kHz Ês / Noc: +5.5dB During T2: Noc: -101dBm/15kHz Ês / Noc: -3.5dB During T3: Noc: -101dBm/15kHz Ês / Noc: +5.5dB |  |  | During T1: -1.6dB 3.2dB During T2: -1.6dB 0dB During T3: -1.6dB 3.2dB |  |  |  | During T1: Noc: -102.6dBm/15kHz Ês / Noc: +8.7dB During T2: Noc: -102.6dBm/15kHz Ês / Noc: -3.5dB During T3: Noc: -102.6dBm/15kHz Ês / Noc: +8.7dB |  |  |
| 12.4 Congestion Control Measurement Test for V2X UE | During T1: Noc: -106dBm/15kHz PUCCH Ês / Noc: +7.19dB During T2: Noc: -106dBm/15kHz PUCCH Ês / Noc: +13.19dB |  |  | During T1: 0dB -2.3dB During T2: 0dB 1.7dB |  |  |  | During T1: Noc: -106dBm/15kHz PUCCH Ês / Noc: +4.89dB During T2: Noc: -106dBm/15kHz PUCCH Ês / Noc: +14.89dB |  |  |

<< End of changes >>