3GPP TSG-RAN WG4 Meeting #82

Athens, GR, 25 Feb - 1 Mar, 2019

Source:        Samsung

Title:          TP analysis on FR1 6.5A.2.4.2.1 UTRA ACLR for CA(2UL CA)

# Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 CA NR ACLR, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for UTRA ACLR of 2UL inter-band CA case.

# Discussion

In LTE specification [4], the test configuration for UTRA ACLR and E-UTRA ACLR is the same. Hence the proposals in [3] are applicable for UTRA ACLR test point selection.

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above background.

## Test Environment

In LTE, UTRA ACLR for CA is tested under NC, TL/VL, TL/VH, TH/VL, TH/VH. According to baseline endorsed in [1], it’s proposed to reuse NC, TL/VL, TL/VH, TH/VL, TH/VH for UTRA ACLR of inter-band UL CA.

Proposal 1: Define test environment as NC, TL/VL, TL/VH, TH/VL, TH/VH for UTRA ACLR in FR1 for UL inter-band CA.

## Test Subcarrier Spacing

In [3] the smallest and highest SCS are selected for NR ACLR verification. It is proposed to reuse the same way for UTRA ACLR.

Proposal 2: Select smallest and highest SCS for UTRA ACLR in FR1 for inter-band UL CA.

## Test Frequencies

In LTE, NR ACLR for Inter-band CA is tested for both Low range and High range. The same test coverage as for LTE would be enough to ensure that the ACLR obeys the test requirement at low range and high range.

Proposal 3: Test UTRA ACLR for Low range and High range frequencies in FR1 for inter-band UL CA.

## Test Channel Bandwidth

In LTE, UTRA ACLR for inter-band CA is tested for Lowest and Highest NRB_agg. Hence it proposed to reuse the LTE method for UTRA ACLR test for inter-band UL CA.

Proposal 4: Test UTRA ACLR for Lowest and Highest NRB_agg in FR1 for UL inter-band CA.

## Test Modulations and waveforms

For TP analysis presented in [3] both DFT-s-OFDM and CP-OFDM with all modulation permutation are selected for testing. For CA UTRA ACLR case it’s proposed to reuse this configuration.

Proposal 5: Test UTRA ACLR for CA with DFT-s-OFDM and CP-OFDM with all modulation permutation.

## RB allocation

Same to analysis in [3] considering RB allocation defined in LTE CA and NR SC mode, it is proposed to consider RB allocation as below:

Proposal 6: Test UTRA ACLR for CA as RB allocation as table 1 .

Table 1: PRB allocation

| Modulation (NOTE 3) | RB allocation (NOTE 1) |  |
| --- | --- | --- |
|  | PCC | SCC |
| DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left | Outer_1RB_Left |
| DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right | Outer_1RB_Right |
| DFT-s-OFDM PI/2 BPSK | Outer_Full | Outer_Full |
| DFT-s-OFDM PI/2 BPSK | Inner_Full | Inner_Full |
| DFT-s-OFDM QPSK | Outer_1RB_Left | Outer_1RB_Left |
| DFT-s-OFDM QPSK | Outer_1RB_Right | Outer_1RB_Right |
| DFT-s-OFDM QPSK | Outer_Full | Outer_Full |
| DFT-s-OFDM QPSK | Inner_Full | Inner_Full |
| DFT-s-OFDM 16 QAM | Inner_Full | Inner_Full |
| DFT-s-OFDM 16 QAM | Outer_1RB_Left | Outer_1RB_Left |
| DFT-s-OFDM 16 QAM | Outer_1RB_Right | Outer_1RB_Right |
| DFT-s-OFDM 16 QAM | Outer_Full | Outer_Full |
| DFT-s-OFDM 64 QAM | Outer_1RB_Left | Outer_1RB_Left |
| DFT-s-OFDM 64 QAM | Outer_1RB_Right | Outer_1RB_Right |
| DFT-s-OFDM 64 QAM | Outer_Full | Outer_Full |
| DFT-s-OFDM 256 QAM | Outer_Full | Outer_Full |
| CP-OFDM QPSK | Inner_Full | Inner_Full |
| CP-OFDM QPSK | Outer_1RB_Left | Outer_1RB_Left |
| CP-OFDM QPSK | Outer_1RB_Right | Outer_1RB_Right |
| CP-OFDM QPSK | Outer_Full | Outer_Full |
| CP-OFDM 16 QAM | Inner_Full | Inner_Full |
| CP-OFDM 16 QAM | Outer_1RB_Left | Outer_1RB_Left |
| CP-OFDM 16 QAM | Outer_1RB_Right | Outer_1RB_Right |
| CP-OFDM 16 QAM | Outer_Full | Outer_Full |
| CP-OFDM 64 QAM | Outer_1RB_Left | Outer_1RB_Left |
| CP-OFDM 64 QAM | Outer_1RB_Right | Outer_1RB_Right |
| CP-OFDM 64 QAM | Outer_Full | Outer_Full |
| CP-OFDM 256 QAM | Outer_Full | Outer_Full |

## Number of test points for CA UTRA ACLR

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 2 | 2 for Outer/Inner allocation 1 for Edge allocation | 14 for Outer/Inner allocation 14 for Edge allocation | 840 |

# Conclusion

For inter-band UL CA NR ACLR requirement, the test condition is proposed as below

Proposal 1: Define test environment as NC, TL/VL, TL/VH, TH/VL, TH/VH for UTRA ACLR in FR1 for UL inter-band CA.

Proposal 2: Select smallest and highest SCS for UTRA ACLR in FR1 for inter-band UL CA.

Proposal 3: Test UTRA ACLR for Low range and High range frequencies in FR1 for inter-band UL CA.

Proposal 4: Test UTRA ACLR for Lowest and Highest NRB_agg in FR1 for UL inter-band CA.

Proposal 5: Test UTRA ACLR for CA with DFT-s-OFDM and CP-OFDM with all modulation permutation.

Proposal 6: Test UTRA ACLR for CA as RB allocation as table 1 .

# Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-19xxxx, "Discussion on test point selection for NR ACLR in FR1"

[4] 3GPP TS 36.521-1 v15.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 v15.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15).

[6] 3GPP TS 38.101-1 v15.4.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15).