3GPP TSG-RAN WG4 Meeting #82

Athens, GR, 25 Feb - 1 Mar, 2019

Source:        Samsung

Title:         TP analysis on FR1 6.5A.3.2.1 Spurious emission for UE co-existence for CA (2UL CA)

# Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 SC NR ACLR, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for NR ACLR of 2UL inter-band CA case.

# Discussion

In LTE specification [4], the test configuration for UL and DL inter-band CA is defined for QPSK and 16QAM as table below.

Table 6.6.3.2A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  | CA Configuration / NRB_agg (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  |  |  |  |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |  |  |  |
|  |  |  | CA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC NRB |  |  |  |  |  | SCC NRB |  |  |  |  | CC MOD |  |  |  |  | PCC & SCC RB allocation |  |  |  |  |  | CC MOD |  |  |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |
|  |  |  | PCC |  |  |  |  |  |  |  |  |  |  | SCC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC |  | SCC |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Band |  |  |  |  |  | Range |  |  |  |  | Band |  |  |  |  | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  |  |  |  | Low |  |  |  |  | Y |  |  |  |  | Low |  |  |  |  | Highest NRB |  |  |  |  |  | Highest NRB |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  |  | 1@0 |  |  |  |  | 1@0 |  |
| 2 | X |  |  |  |  | High |  |  |  |  | Y |  |  |  |  | High |  |  |  |  | Highest NRB |  |  |  |  |  | Highest NRB |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  |  | 1@RBmax |  |  |  |  | 1@RBmax |  |
| Test Settings for CA_1A-3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  |  |  |  | Low |  |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |  |
| 2 |  | 1 |  |  |  |  | Low |  |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |  |
| 3 |  | 1 |  |  |  |  | High |  |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |  |
| 4 |  | 1 |  |  |  |  | High |  |  |  |  | 3 |  |  |  |  | Note 7 |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |  |
| Test Settings for CA_1A-7A Configuration(Note 12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |  |
| 2 |  | 1 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |  |
| Test Settings for CA_1A-8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |  |
| 2 |  | 1 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |  |
| Test Settings for CA_1A-18A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  | 1 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@74 |  |
| Test Settings for CA_1A-19A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |  |
| 2 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 25@0 |  |  | 25@0 |  |
| 3 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |  |
| 4 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 100@0 |  |  | 75@0 |  |
| Test Settings for CA_1A-21A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@24 |  |  | 1@24 |  |
| 2 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@24 |  |  | 1@12 |  |
| 3 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 25@0 |  |  | 25@0 |  |
| 4 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@74 |  |
| 5 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@37 |  |
| 6 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 100@0 |  |  | 75@0 |  |
| Test Settings for CA_1A-28A Configuration (Note 16) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@49 |
| 2 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 3 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 100 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@47 |
| 4 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 50 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@49 |
| 5 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Low |  |  |  |  |  |  | 50 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 6 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 50 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@49 |  |  | 1@47 |
| Test Settings for CA_1A-42A Configuration (Note 13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 2 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@99 |
| 3 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 42 |  |  |  |  | Mid |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@24 |
| 4 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@49 |  |  | 1@0 |
| Test Settings for CA_2A-4A Configuration (Note 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 3 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 4 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |  |
| 5 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 6 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 7 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 8 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 9 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| 10 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| Test Settings for CA_2A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| Test Settings for CA_3A-5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 3 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 4 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 5 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 6 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| Test Settings for CA_3A-7A Configuration (Note 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 3 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@75 |  |  |  | 1@99 |  |
| 4 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 5 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 6 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 7 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| Test Settings for CA_3A-8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 3 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 4 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 5 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 6 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 7 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 8 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 9 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| Test Settings for CA_3A-18A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@74 |  |
| Test Settings for CA_3A-19A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |  |
| 2 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@24 |  |  | 1@0 |  |
| 3 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 25@0 |  |  | 25@0 |  |
| 4 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |  |
| 5 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@0 |  |
| 6 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 100@0 |  |  | 75@0 |  |
| Test Settings for CA_3A-20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 3 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| 4 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 5 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 6 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| Test Settings for CA_3A-26A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 3 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 4 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |  |
| 5 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 6 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@74 |  |
| 7 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@74 |  |
| 8 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| Test Settings for CA_3A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | High |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  | 50 |  |  |  |  |  | High |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 2 |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  | 50 |  |  |  |  |  | Low |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| Test Settings for CA_4A-5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 3 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 4 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 5 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 6 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| Test Settings for CA_4A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 3 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |  |
| 4 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@99 |  |
| 5 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |  |
| 6 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 7 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 8 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| 9 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 10 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 11 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 12 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| Test Settings for CA_5A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@99 |  |
| 2 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@24 |  |  |  | 1@0 |  |
| 3 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |  |
| Test Settings for CA_5A-12A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 12 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 2 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 12 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| Test Settings for CA_5A-17A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 17 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| Test Settings for CA_7A-20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 2 |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 3 |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 4 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 5 |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| Test Settings for CA_7A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@74 |  |
| 2 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| Test Settings for CA_18A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Note 15 |  |  |  |  | 75 |  |  |  |  |  |  | 50 |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@2 |  |
| 2 |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Note 15 |  |  |  |  | 75 |  |  |  |  |  |  | 50 |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@74 |  |  |  | 1@47 |  |
| Test Settings for CA_39A-41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 39 |  |  |  |  | High |  |  |  |  | 41 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| Test Settings for CA_41A-42A Configuration (Note 14) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 41 |  |  |  |  | Note 8 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 2 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | Note 11 |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@22 |  |  |  | 1@99 |  |
| 3 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@44 |  |  |  | 1@99 |  |
| 4 |  |  |  | 41 |  |  |  |  | Note 8 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 5 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 6 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@44 |  |  |  | 1@99 |  |
| 7 |  |  |  | 41 |  |  |  |  | Note 10 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 8 |  |  |  | 41 |  |  |  |  | Note 8 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| 9 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 10 |  |  |  | 41 |  |  |  |  | Low |  |  |  |  | 42 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_1A-19A,X=1,Y=19. Note 4: For CA configurations CA_1A-18A, CA_1A-19A, CA_1A-21A , CA_3A-19A and CA_18A-28A each test point shall be tested. For all other CA configurations only test points with highest NRB_agg , according to UE supported BCS shall be tested. Note 5: Test Point ID 7 for CA_3A-7A is applicable only for a Rel-13 and forward UE. Note 6: Test Point ID 3 and Test Point ID 8 for CA_2A-4A are applicable only for a Rel-14 and forward UE. Note 7: Test Point ID 4 for CA_1A-3A has the centre carrier frequency 1849.9 MHz, i.e. DL E-ARFCN is 1649. Note 8: Test Point ID 1, 4 and 8 for CA_41A-42A have the centre carrier frequency of 2635 MHz in Band 41 (EARFCN=41040). Note 9: Test Point ID 2, 3, 5, 6 and 9 for CA_41A-42A has the centre carrier frequency of 2555 MHz in Band 41 (EARFCN=40240). Note 10: Test Point ID 7 for CA_41A-42A has the centre carrier frequency of 2674 MHz in Band 41 (EARFCN=41430). Note 11: Test Point ID 2 for CA_41A-42A has the centre carrier frequency of 3421 MHz in Band 42 (EARFCN=41800). Note 12: Test Point ID 2 for CA_1A-7A is applicable only for a Rel-15 and forward UE. Note 13: Test Point ID 4 for CA_1A-42A is applicable only for a Rel-15 and forward UE. Note 14: Test Point ID 10 for CA_41A-42A is applicable only for a Rel-15 and forward UE. Note 15: Test Point ID 1 and 2 for CA_18A-28A have the centre carrier frequency of 778 MHz in Band 28 (EARFCN=9410). Note 16: As an exception to Note 4, Test Point ID 3 for CA_1A-28A shall be tested for a UE supporting bandwidth combination set 0, even though it has not highest NRB_agg for this UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for spurious emission for UE co-existence for single carrier mode is defined as table below.

Table 6.5.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest supported SCS per test channel BW, Highest supported SCS per test channel BW |  |
| Test Parameters |  |  |  |
| Ch BW | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 5 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 10 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 15 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 20 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 25 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 30 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 40 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 50 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 60 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 80 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 90 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 100 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration. |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

## Test Environment

In LTE, Spurious emission for UE coexistence of CA is tested under NC only. According to baseline endorsed in [1], it’s proposed to reuse NC for Spurious emission of inter-band UL CA.

Proposal 1: Define test environment as NC for spurious emission for UE coexistence in FR1 for UL inter-band CA.

## Test Subcarrier Spacing

In [3] the smallest and highest SCS are selected for Single Carrier Spurious emission for UE coexistence verification. It is proposed to reuse the same way for inter-band UL CA.

Proposal 2: Select smallest and highest SCS for Spurious emission for UE coexistence in FR1 for inter-band UL CA.

## Test Frequencies

In LTE, Spurious emission for UE coexistence of inter-band CA is tested for band combination specific condition. Hence it proposed to reuse the LTE method for NR inter-band UL CA.

Proposal 3: Reuse the band combination specific condition for spurious emission for UE coexistence of UL inter-band CA regarding frequency range under testing.

## Test Channel Bandwidth

In LTE, Spurious emission for UE coexistence of inter-band CA is tested for highest BW for both PCC and SCC. Hence it proposed to reuse the LTE method for NR inter-band UL CA.

Proposal 4: Test with highest BW for spurious emission for UE coexistence of UL inter-band CA.

## Test Modulations and waveforms

For single carrier Spurious emission for UE coexistence TC presented in [3] only CP-OFDM with OPSK is selected for testing. For CA case it’s proposed to reuse this configuration.

Proposal 5: Test Spurious emission for UE coexistence for CA with CP-OFDM QPSK.

## RB allocation

Considering RB allocation defined in LTE CA and NR SC for Spurious emission, the Edge_1RB_Left and Edge_1RB_Right are proposed for NR CA spurious emissions for UE co-existence test.

Proposal 6: Test Spurious emission for UE coexistence of UL inter-band CA with Edge_1RB_Left and Edge_1RB_Right

## Number of test points

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 1 | 2 | 2 | 8 |

# Conclusion

For inter-band UL CA Spurious emission for UE coeixstence requirement, the test condition is proposed as below:

Proposal 1: Define test environment as NC for spurious emission for UE coexistence in FR1 for UL inter-band CA.

Proposal 2: Select smallest and highest SCS for Spurious emission for UE coexistence in FR1 for inter-band UL CA.

Proposal 3: Reuse the band combination specific condition for spurious emission for UE coexistence of UL inter-band CA regarding frequency range under testing.

Proposal 4: Test with highest BW for spurious emission for UE coexistence of UL inter-band CA.

Proposal 5: Test Spurious emission for UE coexistence for CA with CP-OFDM QPSK.

Proposal 6: Test Spurious emission for UE coexistence of UL inter-band CA with Edge_1RB_Left and Edge_1RB_Right

# Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-182019, "Discussion of NR FR1 Test Point for TX Spurious Emission test cases "

[4] 3GPP TS 36.521-1 v15.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 v15.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15).

[6] 3GPP TS 38.101-1 v15.4.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15).