3GPP TSG RAN WG5#82 	R5-192081

# Athens, Greece, 25th Feb – 1st Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4761 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of RRC TC QoE Measurement Collection in 8.3.5.1 and 8.3.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_QMC_Streaming-UEConTest |  |  |  |  |  |  | Date: |  | 2018-10-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 has revised the description for measReportAppLayerContainer to distinguish QoE Measurement Collection Functionalities for MTSI services and Streaming service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align the Conformance requirements with the core spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.5.1, 8.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The associated R2 CR R2-1900993 |  |  |  |  |  |  |  |  |

8.3.5	Application layer measurement reporting

8.3.5.1	RRC connection reconfiguration/ QoE Measurement Collection /QoE measurement setup and report and release

8.3.5.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state without SRB4 setup and it is not configured to provide measurement report for QoE}

ensure that {

when { UE is caused to send MeasReportAppLayer message }

then { the UE does not transmit a MeasReportQoE message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state with SRB4 setup and it is not configured to provide measurement report for QoE}

ensure that {

when { UE is caused to send MeasReportAppLayer message }

then { the UE does not transmit a MeasReportAppLayer message }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state with SRB4 setup and it is configured to provide measurement report for QoE }

ensure that {

when { UE is caused to send MeasReportAppLayer message }

then { the UE does transmit a MeasReportAppLayer message with measReportAppLayerContainer set to the value of the measurement report information of QoE from the upper layer}

}

(4)

with { UE in E-UTRA RRC_CONNECTED state and it is configured to provide measurement report for QoE}

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing a otherConfig that includes the MeasReportAppLayer set to Release }

then { the UE does not transmit any more MeasReportQoE message }

8.3.5.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.10.9, 5.6.19, and 6.3.6. Unless otherwise stated these are Rel-15 requirements.

[TS 36.331, clause 5.3.10.9]

The UE shall:

...

1>	if the received otherConfig includes the measConfigAppLayer:

2>	if measConfigAppLayer is set to setup:

3>	forward measConfigAppLayerContainer to upper layers considering the serviceType;

3>	consider itself to be configured to send application layer measurement report in accordance with 5.6.19;

2>	else:

3>	inform upper layers to clear the stored application layer measurement configuration;

3>	discard received application layer measurement report information from upper layers;

3>	consider itself not to be configured to send application layer measurement report.

TS 36.331, clause 5.6.19

5.6.19.1	General

Figure 5.6.19.1-1: Application layer measurement reporting

The purpose of this procedure is to inform E-UTRAN about application layer measurement report.

5.6.19.2	Initiation

A UE capable of application layer measurement reporting in RRC_CONNECTED may initiate the procedure when configured with application layer measurement, i.e. when measConfigAppLayer has been configured by E-UTRAN.

Upon initiating the procedure, the UE shall:

1>	if configured with application layer measurement, and SRB4 is configured, and the UE has received application layer measurement report information from upper layers:

2>	set the measReportAppLayerContainer in the MeasReportAppLayer message to the value of the application layer measurement report information;

2>	submit the MeasReportAppLayer message to lower layers for transmission via SRB4.

[TS 36.331, clause 6.3.2]

MeasReportAppLayer field descriptions

measReportAppLayerContainer The field contains container of application layer measurements, see Annex L (normative) in TS 26.247 [90] and 16.5 in TS 26.114 [x]

serviceType Indicates the type of application layer measurement. Value qoe indicates Quality of Experience Measurement Collection for streaming services, value qoemtsi indicates Quality of Experience Measurement Collection for MTSI.

8.3.5.1.3	Test Description

8.3.5.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.3.5.1.3.2	Test procedure sequence

Table 8.3.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause UE upper layer to provide APPLICATION LAYER MEASUREMENT REPORT with 1000 bytes "Container for application layer measurement reporting" and the service type is qoe (Note1) |  |  | - | - |
| 2 | Check: Does the UE send a MeasReportAppLayer message within the next 20s? | --> |  | 1 | F |
| 3 | The SS transmits an RRCConnectionReconfiguration message containing srb-ToAddModList to setup SRB4 | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete |  |  |
| 5 | Cause UE upper layer to provide APPLICATION LAYER MEASUREMENT REPORT with 1000 bytes "Container for application layer measurement reporting" and the service type is qoe (Note1) |  |  |  |  |
| 6 | SS waits 5s after step 5 |  |  |  |  |
| 7 | Check: Does the SS receive a MeasReportAppLayer corresponding to the previously transmitted Generate Application Layer Measurement Report command? | --> |  | 2 | F |
| 8 | The SS transmits an RRCConnectionReconfiguration message containing a otherConfig includes the measConfigAppLayer set to Setup and the service type set to qoe. | <-- | RRCConnectionReconfiguration |  |  |
| 9 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 10 | Cause UE upper layer to provide APPLICATION LAYER MEASUREMENT REPORT with 1000 bytes "Container for application layer measurement reporting" and the service type is qoe (Note1) |  |  |  |  |
| 11 | (SS wait 100ms from step 9) |  |  |  |  |
| 12 | Cause UE upper layer to provide APPLICATION LAYER MEASUREMENT REPORT with 4000 bytes "Container for application layer measurement reporting" and the service type is qoe (Note1) |  |  |  |  |
| 13 | (SS wait 100ms from step 11) |  |  |  |  |
| 14 | Cause UE upper layer to provide APPLICATION LAYER MEASUREMENT REPORT with 8000 bytes "Container for application layer measurement reporting" and the service type is qoe (Note1) |  |  |  |  |
| 15 | (SS wait 10s from step 9) |  |  |  |  |
| 16 | Check: Does the SS receive 3 Application measurement reports each corresponding to previously transmitted Generate Application Layer Measurement Report command, and all the MEASUREMENT REPORTs must be transmitted on SRB4 | --> |  | 3 | P |
| 17 | The SS transmits an RRCConnectionReconfiguration message containing srb-ToReleaseList to release SRB4 | <-- | RRCConnectionReconfiguration |  |  |
| 18 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete |  |  |
| 19 | Cause UE upper layer to provide APPLICATION LAYER MEASUREMENT REPORT with 1000 bytes "Container for application layer measurement reporting" and the service type is qoe (Note1) |  |  |  |  |
| 20 | SS wait 5s after Step 19 |  |  |  |  |
| 21 | Check: Does the UE send a MeasReportAppLayer | --> |  | 2 | F |
| 22 | The SS transmits an RRCConnectionReconfiguration message containing srb-ToAddModList to setup SRB4 | <-- | RRCConnectionReconfiguration |  |  |
| 23 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete |  |  |
| 24 | The SS transmits an RRCConnectionReconfiguration message containing otherConfig includes the set to release | <-- | RRCConnectionReconfiguration |  |  |
| 25 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete |  |  |
| 26 | Cause UE upper layer to provide APPLICATION LAYER MEASUREMENT REPORT with 8000 bytes "Container for application layer measurement reporting" and the service type is qoe (Note1) |  |  |  |  |
| 27 | (SS wait 5s from step 26) |  |  |  |  |
| 28 | Check: Does the SS receive a MeasReportAppLayer message? | --> |  | 4 | F |
| Note1: This indication may be triggered with the AT command +CAPPLEVMR specified in TS 27.007 clause 8.79, or any alternative way |  |  |  |  |  |

8.3.5.1.3.3	Specific message contents

Table 8.3.5.1.3.3-1 RRCConnectionReconfiguration (steps 3, and 22, Table 8.3.5.1.3.2-1)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition SRB4

Table 8.3.5.1.3.3-2 RRCConnectionReconfiguration (step 8 and 24, Table 8.3.5.1.3.2-1)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition otherConfig

Table 8.3.5.2.3.3-3 OtherConfig-r9 (Table 8.3.5.1.3.3-2)

Derivation Path: 36.508 Table 4.6.3-29 with conditions QMC and setup

Table 8.3.5.1.3.3-4 MeasReportAppLayer (step 16 Table 8.3.5.1.3.2-1)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasReportAppLayer-r15 ::= SEQUENCE { |  |  |  |
| criticalExtensions ::= CHOICE { |  |  |  |
| MeasReportAppLayer-r15-IEs SEQUENCE { |  |  |  |
| measReportAppLayerContainer-r15 | Octets string (1…8000) | the exact length in each step equals to the length of the container in the previous step |  |
| serviceType | qoe |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.5.1.3.3-5 RRCConnectionReconfiguration (step 17 Table 8.3.5.1.3.2-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| radioResourceConfigDedicated ::= SEQUENCE { |  |  |  |
| srb-ToReleaseExtList-r15 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.3.5.2	RRC connection reconfiguration/ Qoemtsi Measurement Collection /QoE measurement setup and report and release

8.3.5.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state without SRB4 setup or it is not configured to provide measurement report for Qoemtsi }

ensure that {

when { UE is caused to send MeasReportAppLayer message }

then { the UE does not transmit a MeasReportAppLayer message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state with SRB4 setup and it is not configured to provide measurement report for Qoemtsi }

ensure that {

when { UE is caused to send MeasReportAppLayer message }

then { the UE does not transmit a MeasReportAppLayer message }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state with SRB4 setup and it is configured to provide measurement report for Qoemtsi }

ensure that {

when { UE is caused to send MeasReportAppLayer message }

then { the UE does transmit a MeasReportAppLayer message with measReportAppLayerContainer set to the value of the measurement report information for the service type Qoemtsi from the upper layer}

}

(4)

with { UE in E-UTRA RRC_CONNECTED state and it is configured to provide measurement report for Qoemtsi}

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing a otherConfig that includes the measConfigAppLayer set to Release }

then { the UE does not transmit any more MeasReportAppLayer message }

8.3.5.2.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.10.9, 5.6.19, and 6.3.6. Unless otherwise stated these are Rel-15 requirements.

[TS 36.331, clause 5.3.10.9]

The UE shall:

...

1>	if the received otherConfig includes the measConfigAppLayer:

2>	if measConfigAppLayer is set to setup:

3>	forward measConfigAppLayerContainer to upper layers considering the serviceType;

3>	consider itself to be configured to send application layer measurement report in accordance with 5.6.19;

2>	else:

3>	inform upper layers to clear the stored application layer measurement configuration;

3>	discard received application layer measurement report information from upper layers;

3>	consider itself not to be configured to send application layer measurement report.

TS 36.331, clause 5.6.19

5.6.19.1	General

Figure 5.6.19.1-1: Application layer measurement reporting

The purpose of this procedure is to inform E-UTRAN about application layer measurement report.

5.6.19.2	Initiation

A UE capable of application layer measurement reporting in RRC_CONNECTED may initiate the procedure when configured with application layer measurement, i.e. when measConfigAppLayer has been configured by E-UTRAN.

Upon initiating the procedure, the UE shall:

1>	if configured with application layer measurement, and SRB4 is configured, and the UE has received application layer measurement report information from upper layers:

2>	set the measReportAppLayerContainer in the MeasReportAppLayer message to the value of the application layer measurement report information;

2>	submit the MeasReportAppLayer message to lower layers for transmission via SRB4.

[TS 36.331, clause 6.3.2]

MeasReportAppLayer field descriptions

measReportAppLayerContainer The field contains container of application layer measurements, see Annex L (normative) in TS 26.247 [90] and 16.5 in TS 26.114 [x]

serviceType Indicates the type of application layer measurement. Value qoe indicates Quality of Experience Measurement Collection for streaming services, value qoemtsi indicates Quality of Experience Measurement Collection for MTSI.

8.3.5.2.3	Test Description

8.3.5.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.3.5.2.3.2	Test procedure sequence

The same test procedure as specified in clause 8.3.5.2.3.2 with the exception in steps 1, 5, 8, 10, 12, 14, 19 and 26:

The service type is qoemtsi instead of qoe.

8.3.5.2.3.3	Specific message contents

Table 8.3.5.2.3.3-1 RRCConnectionReconfiguration (steps 3, and 22, Table 8.3.5.1.3.2-1)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition SRB4

Table 8.3.5.2.3.3-2 RRCConnectionReconfiguration (step 8 and 24, Table 8.3.5.1.3.2-1)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition otherConfig

Table 8.3.5.2.3.3-3 OtherConfig-r9 (Table 8.3.5.2.3.2-2)

| Derivation Path: 36.508 Table 4.6.3-29 with condition QMC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OtherConfig-r9 ::= SEQUENCE { |  |  |  |
| measConfigAppLayer-r15 :: = CHOICE { | Not present |  | QMC |
| setup SEQUENCE { |  |  |  |
| measConfigAppLayerContainer-r15 | Octets string (1…1000) |  |  |
| serviceType | qoemtsi |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.5.2.3.3-4 MeasReportAppLayer (step 16 Table 8.3.5.1.3.2-1)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasReportAppLayer-r15 ::= SEQUENCE { |  |  |  |
| criticalExtensions ::= CHOICE { |  |  |  |
| MeasReportAppLayer-r15-IEs SEQUENCE { |  |  |  |
| measReportAppLayerContainer-r15 | Octets string (1…8000) | the exact length in each step equals to the length of the container in the previous step |  |
| serviceType | qoemtsi |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.5.2.3.3-5 RRCConnectionReconfiguration (step 17 Table 8.3.5.1.3.2-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| radioResourceConfigDedicated ::= SEQUENCE { |  |  |  |
| srb-ToReleaseExtList-r15 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |